| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0072 |
| --- | --- |

| Title: | AHG8: Stretching ratio based adaptive quantization for 360 video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yule Sun, Lu Yu Zhejiang University Hangzhou, Zhejiang 310027,China | Tel: Email: | +86 57187953107 sunyule@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

To improve the coding efficiency of 360 video, stretching ratio based adaptive quantization for 360 video is proposed. Different positions in images are assigned with different quantization according to the stretching ratio. The experimental results show that this proposed method achieves average gain is -2.15%, -2.22% for E2E WS-PSNR Y and WS-PSNR (encoder) Y respectively.

Stretching ratio based adaptive quantization

In common test condition, QP for each coded frame is fixed. Assuming the texture is uniform and CTUs are independent, each CTU has the same coding error D1 (MSE) and rate R1 (bit/pixel) when QP is fixed for one frame. For this case, the distortion in sphere is also equal to D1 and rate is R1. What is more, the rate in spherical space is uniform. For example, using fixed QP for one frame in ERP, rate (bit/area) near the pole is larger than that near the equator while distortion (SSE for uniform distributed pixel in sphere) is uniform.

For compressing 360 video, (1) is our goal and wi is the weight of WS-PSNR [1].

minwiDi          Ri≤Rc             (1)

By controlling total coding distortion Di in coding texture is equal, which results in similar average rate, we can distribute larger distortion (MSE) in the CTUs where weights are smaller to reduce the total coding distortion wiDi in sphere.

To generate coding CTUs with different distortion, adaptive QP is used in CTU-level.

As a special case, we distribute distortion according to stretching ratio from projection formats to sphere. Stretching ratio can be obtained from weight map of WS-PSNR. For a CTU with average weight WK, assigning D'WK (MSE) to it. And D' is the MSE coded using fixed QP. Finally, normalize the D'WK to keep total distortion in projection equal. To note that this distribution method can improve coding efficiency, but may not be the best distribution method. Details are shown below.

Assuming source probability distribution is uniform, the relationship between Qstep and distortion D (MSE) is:
D=Qstep212                (1)

And the relationship between Qstep and QP in HEVC is:

Qstep=2QP-46        (2).

So, the distortion D (MSE) can be obtained by:

D=2(QP-4)/312                (3)

The new distortion (MSE) for CTU with weight Wi is D’, and D’=1/Wi*D, each CTU needs to adjust the baseQP:

1Wi=2(deltaQP)/3       (4)

And QP is integer, deltaQP=int（3log21w+0.5).

To normalize the weights which change the quality in different CTUs, deltaQP_ave is calculated by average weights of all CTUs: deltaQP_ave=int（3log21w_ave+0.5). Finaly, QP_offset for each CTU can be obtained:

QP_offset=deltaQP-deltaQPave     (5).

For ERP, Weights of CTU can be approximately calculated by:

Wj=cos32+j*64-a2-12*πa              (6)

where W(j) is the weight of CTU in j-th row. (The resolution of equirectangular format is defined as 2a*a).

For an image of 3328*1664 in ERP, QP_Offset and ideal ratio of D(MSE) in different CTU row are show below.

Figure.1 QP_offset in each CTU row

Figure.2 ratio of MSE in each CTU row

Experimental results

The tests are conducted according to the JVET common test conditions and on HM 16.15-360Lib with RA

| Proposed vs. ERP | SPSNR-NN (End to End) |  |  | SPSNR-I (End to End) |  |  | CPP-PSNR (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | 0.6% | 2.2% | 1.3% | 0.6% | 2.2% | 1.4% | 0.6% | 2.2% | 1.4% | 0.6% | 2.2% | 1.4% |
| GasLamp | 0.7% | 2.0% | 3.8% | 0.7% | 2.0% | 3.8% | 0.7% | 2.0% | 3.8% | 0.7% | 2.0% | 3.8% |
| Skateboarding_in_lot | -4.9% | -7.0% | -4.7% | -4.9% | -7.0% | -4.7% | -4.9% | -7.0% | -4.8% | -4.9% | -6.9% | -4.8% |
| Chairlift | -4.9% | -1.5% | -0.1% | -4.9% | -1.5% | -0.1% | -4.8% | -1.4% | -0.1% | -4.8% | -1.4% | -0.1% |
| KiteFlite | -1.2% | 1.1% | 0.5% | -1.2% | 1.1% | 0.5% | -1.3% | 1.0% | 0.4% | -1.3% | 1.0% | 0.5% |
| Harbor | 0.7% | 3.1% | 3.1% | 0.8% | 3.1% | 3.1% | 0.7% | 3.1% | 3.2% | 0.7% | 3.1% | 3.2% |
| PoleVault | -5.4% | -2.1% | -1.8% | -5.4% | -2.0% | -1.7% | -5.4% | -2.0% | -1.7% | -5.3% | -2.0% | -1.8% |
| AerialCity | -2.3% | 0.8% | 0.4% | -2.3% | 0.8% | 0.4% | -2.2% | 0.9% | 0.4% | -2.2% | 0.8% | 0.4% |
| DrivingInCity | 0.6% | 6.8% | 6.0% | 0.6% | 6.8% | 6.0% | 0.6% | 6.8% | 6.0% | 0.6% | 6.8% | 6.0% |
| DrivingInCountry | -5.5% | -4.4% | -3.9% | -5.4% | -4.5% | -4.0% | -5.5% | -4.6% | -4.0% | -5.5% | -4.4% | -3.9% |
| Overall | -2.2% | 0.1% | 0.5% | -2.2% | 0.1% | 0.5% | -2.2% | 0.1% | 0.5% | -2.1% | 0.1% | 0.5% |

| WS-PSNR (codec) |  |  | PSNR (codec) |  |  |
| --- | --- | --- | --- | --- | --- |
| Y | U | V | Y | U | V |
| 0.6% | 2.3% | 1.5% | 3.2% | 6.1% | 3.3% |
| 0.6% | 2.0% | 3.8% | 4.1% | 5.8% | 9.2% |
| -5.2% | -6.8% | -4.7% | 9.0% | -1.9% | -3.8% |
| -5.0% | -1.3% | 0.0% | 13.9% | 17.3% | 9.9% |
| -1.3% | 1.1% | 0.5% | 6.7% | 7.4% | 6.3% |
| 0.8% | 3.1% | 3.2% | 5.5% | 7.1% | 7.2% |
| -5.5% | -2.2% | -1.9% | 5.8% | 11.7% | 9.9% |
| -2.3% | 1.0% | 0.6% | 5.8% | 7.4% | 7.8% |
| 0.6% | 6.8% | 6.1% | 3.4% | 11.5% | 12.2% |
| -5.6% | -4.4% | -3.9% | 8.0% | 2.5% | 2.9% |
| -2.2% | 0.2% | 0.5% | 6.5% | 7.5% | 6.5% |

Conclusion

Stretching ratio based adaptive quantization for 360 video is proposed to improve the coding efficiency of 360 video. However this makes the quality in sphere different. As an example, ERP is tested in this proposal. The experimental results report that the proposed method achieves -2.15%, -2.22% for E2E WS-PSNR Y and WS-PSNR (encoder) Y respectively. And other formats can also be applied to adjust quantization according to weight map derived in WS-PSNR.

References

[1] Y. Sun, A. Lu, L. Yu, “AHG8: WS-PSNR for 360 video objective quality evaluation,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0040, Oct. 2016, Chengdu, China.

Patent rights declaration(s)

Zhejiang university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).