| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Hobart, AU, 31 March – 7 April 2017 | Document: JVET-F0089 |
| --- | --- |

| Title: | AHG7: Comments on metrics for extended colour volume content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | E. Francois, C. Chevance, F. Hiron (Technicolor) D. Rusanovskyy, A.K. Ramasubramonian (Qualcomm) | Email: | edouard.francois@technicolor.com dmytror@qti.qualcomm.com |
| Source: | Technicolor, Qualcomm |  |  |

_____________________________

# Abstract

This contribution discusses about the metrics specified in JEM Common Test Conditions on HDR (JVET-E1020) for evaluating coding performance on extended colour volume materials and addressed in AHG7. It reports observations made on the metrics currently considered in the AHG7, and makes several suggestions on the potential usage of the wPSNR metric.

# Introduction

In the context of AHG7, relating to the coding of extended colour volume materials, it is specified in the current JVET CTC on HDR (JVET-E1020) that several metrics are considered, including a metric wPSNR, previously proposed in documents JVET-C0095 and JVET-D0124.

In contribution JVET-E0067, it was suggested to further study this specific metric. The current contribution aims to report some observations about wPSNR, and to raise several points that need to be addressed for future exploration work on HDR/WCG coding in JVET and for the draft CfE document (JVET-E1002) in case it includes extended colour volume content.

# Anchors generation for extended colour volume content

JVET CTC on HDR and AHG7 consider two types of HDR/WCG content, “true” HDR content represented in BT.2100 PQ 10-bit 4:2:0 Y’CbCr container (that are also used by JCT-VC), and native SDR BT.709 content converted into BT.2100 PQ 10-bit 4:2:0 Y’CbCr container (PQ2100 representation). For generating the HM an JEM anchors of these content, the same luma-based local QP adaptation algorithm as used for JCT-VC HDR anchors (cf JCTVC-Z1020) is applied. In JVET work, two different dQP tables (one for native HDR content and another for native SDR content) are currently used to address different natures of the test video included in CTC, as plotted in Figure 1.

Figure 1. dQP tables used for HDR and SDR content.

# wPSNR description

JVET CTC on HDR/WCG coding current specify use of weighted PSNR metrics (wPSNR), where the distortion is weighted for each sample by a scaling factor depending on the expected dQP applying to the block in which the sample belongs. The weight is derived in the same way as the lambda adjustment achieved in JEM when QP is modified by (–dQP):

In other words, wPSNR gives more importance to samples coded with a higher quality, due to deltaQP table used in encoder, than to samples coded with a lower quality.

Practically, the implementation of wPSNR in HM-16.6-JEM-4.0-EE6-Anchor, and in new anchors from AHG7 generated at this meeting using modified JEM5.0.1, is using continuous and derivable weighting functions defined by the following pseudo-code (also plotted in Figure 2):

- for native HDR content in PQ2100 representation

y = max(-3, min(6, 0.015*x - 1.5 – 6 ) )

weight[y] = pow(2.0, y/3.0)

- for native SDR content in PQ2100 representation

y = max(0, min(12, 0.03*x - 3.0 ) )

weight[y] = pow(2.0, y/3.0)

Note that the weighted distortion is also used in the RDO, which impacts the encoder behavior.

Figure 2. weighting functions used for HDR and SDR content.

# Study of wPSNR in case of SDR content

In contribution JVET-E0067, it was suggested to further study this specific metric. In particular, a comparison with an end-to-end PSNR Y,U,V of the native SDR content in BT.709 container was suggested (cf Figure 3).

Figure 3. Suggested end-to-end metrics (in red font) for AHG7 experiments.

The following table reports results from these different metrics on the SDR content, 1 picture, coded in AI configuration. The reference uses the JEM5.0.1 without using the local QP adaptation algorithm, and the tested solution is the modified JEM5.0.1 applying the local QP adaptation algorithm (with a depth for dQP setting set to 0).

More results on full sequences and on inter configurations are expected to be delivered before the end of the meeting.

Table 1. Metrics in AI configuration, 1 picture coded.

The following observations can be made from these results:

- comparison in SDR case of usual PSNR in BT.709 domain versus wPSNR in BT.2100 PQ domain shows similar trend; it can be noted that for chroma components, metrics are very close;

- DE100 and PSNRL100 also show similar trends as these 2 metrics to measure the luminance and color distortions;

- tPSNRY does not seems to be well correlated to the other metrics.

As a conclusion, wPSNR approach seems to show similar trends than the usual PSNR Y,U,V used for SDR content in BT.709 domain. We think that this metric could be considered as rather relevant to evaluate objectively HDR content, as we usually do for SDR BT.709 content using PSNR Y,U,V. DE100 and PSNRL100 are also relevant, while tPSNRY is questionable.

# Further comments about weighted PSNR

There are nevertheless several points that need to be discussed before deciding to use this metric for further experiments and performance evaluations. The points are listed below:

- wPSNR is not really a metric as it is not symmetric; one possible proposal would be to symmetrize it by computing PSNR (or weighted MSE, noted below wMSE) in both ways:

  - wPSNR_sym(src,test) =  ( wPSNR(src,test) + wPSNR (test, src) ) / 2        or

  - wMSE_sym(src,test) =  ( wMSE(src,test) + wMSE(test, src) ) / 2

one concern with such symetrization is that the weighting becomes dependent on reconstructed samples; we recommend to further discuss this point in the meeting

- wPSNR should not depend on the content, but only on the container (e.g. BT.2100 PQ 10-bits Y’CbCr for current CTC);

otherwise, it could be considered that the usage of content-dependent different weighting (e.g. different weightings for dark content and for bright content) could even been introduced; 
also, changing the metric depending on the content means changing the RDO behavior, which is problematic for comparison of the compression efficiency of different tools or solutions.

- Therefore, we recommend to use one single wPSNR metric for extended colour volume content represented in BT.2100 PQ 10-bits Y’CbCr format, independent from the “nature” of the content (being true HDR or native SDR); and our recommendation is to use the HDR one, that was built from some subjective optimization on the JCT-VC / MPEG true HDR material.

- We also suggest to still report DE100 and PSNR_L100 metrics.

# Reference

- M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki, “Preliminary Joint Call for Evidence on Video Compression with Capability beyond HEVC,” Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-E1002, Jan. 2017.

- “JVET common test conditions and evaluation procedures for HDR/WCG video”, Joint Video Exploration Team (JVET) of ITU-T VCEG (Q6/16) and ISO/IEC MPEG (JTC 1/SC 29/WG 11), 5th Meeting, Geneva, January 2017, Doc. JVET-E1020.