| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13–21 July 2017 | Document: JVET-G_Notes_d1 |
| --- | --- |

| Title: | Meeting Report of the 7th meeting of the Joint Video Exploration Team (JVET), Torino, IT, 13–21 July 2017 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from responsible coordinators of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Responsible coordinators |  |  |

_____________________________

- Summary

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its seventh meeting during 13–21 July 2017 at the Politecnico di Torino, Torino, IT. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations. For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section  of this document.

The JVET meeting sessions began at approximately 0900 hours on Thursday 13 July 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 21 July 2017. Approximately XXX people attended the JVET meeting, and approximately XX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. As a primary goal, the JVET meeting performed an evaluation of compression technology designs proposed in this area, which had been received in response to the Call for Evidence (CfE) as issued by the previous meeting.

Another important goal of the meeting was to review the work that was performed in the interim period since the sixth JVET meeting in producing the Joint Exploration Test Model 6 (JEM6). Video coding results produced with JEM6 had also been included as anchor in the CfE. Furthermore, results from four exploration experiments conducted in the JEM6 framework were reviewed, and other technical input was considered. On this basis, modifications towards JEM7 were planned.

The JVET produced 7 output documents from the meeting (update):

- Algorithm description of Joint Exploration Test Model 6 (JEM6)

- Joint Call for Evidence on video compression with capability beyond HEVC

- Algorithm descriptions of projection format conversion and video quality metrics in 360Lib

- Description of Exploration Experiments on coding tools

- Common test conditions and evaluation procedures for HDR/WCG video

- Common test conditions and evaluation procedures for 360° video

- Subjective testing method for 360° Video projection formats using HEVC

For the organization and planning of its future work, the JVET established X "ad hoc groups" (AHGs) to progress the work on particular subject areas. X Exploration Experiments (EE) were defined on particular subject areas of coding tool testing. The next four JVET meetings are planned for Wed. 18. – Wed. 25 Oct. 2017 under ITU-T auspices in Macao, CN, during Fri. 19 – Fri. 26 Jan. 2018 under WG 11 auspices in Gwangju, KR, during 13-20 April 2018 in San Diego, US, and during XX – XX July 2018 under ITU-T auspices in Ljubljana, SVN.

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Exploration Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its seventh meeting during 13–21 July 2017 at the Politecnico di Torino, Torino, IT. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations.

## Meeting logistics

The JVET meeting sessions began at approximately 0900 hours on Thursday 13 July 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 21 July 2017. Approximately XXX people attended the JVET meeting, and approximately XX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. The JVET meeting also performed an evaluation of compression technology designs proposed in this area, in particular responses to the Call for Evidence were evaluated in this context.

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2017_07_G_Torino/.

## Primary goals

As a primary goal, the JVET meeting performed an evaluation of compression technology designs proposed in this area, which had been received in response to the Call for Evidence (CfE) as issued by the previous meeting. Another important goal of the meeting was to review the work that was performed in the interim period since the sixth JVET meeting in producing the Joint Exploration Test Model 6 (JEM6). Video coding results produced with JEM6 had also been included as anchor in the CfE. Furthermore, results from four exploration experiments conducted in the JEM6 framework were reviewed, and other technical input was considered. On this basis, modifications towards JEM7 were planned.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the JEM software but have no normative effect are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the JEM description (an error, oversight, or messiness) or in the software are marked by the string "Decision (BF):".

This meeting report is based primarily on notes taken by the responsible leaders. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Wednesday, 5 July 2017. Any documents uploaded after 2359 hours Paris/Geneva time on Thursday 24 March were considered "officially late", giving a grace period of 24 hrs to those living in different time zones of the world.

All contribution documents with registration numbers JVET-G0113 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "F0113+" range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- JVET-G0XXX (a proposal on …), uploaded 07-XX,

- …

The following technical design proposal contributions were both registered late and uploaded late:

- JVET-G0XXX (a proposal on …), uploaded 07-XX,

- …

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JVET-G0XXX (an information document on on …), uploaded 07-XX,

- ...

The following cross-verification reports were registered on time but were uploaded late: JVET-G0XXX [uploaded 07-XX], … .

(Documents that were both registered late and uploaded late, other than technical proposal documents, are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-G0XXX, ….

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable rejected in the document management system. The initial uploads of the following contribution documents were rejected as "placeholders" and were not corrected until after the upload deadline: (This case did not happen at the current meeting).

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, EE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly the meeting report JVET-F1000, JEM6 algorithm description JVET-F1001, the Joint Call for Evidence JVET-F1002, the algorithm descriptions of projection format conversion and video quality metrics in 360Lib JVET-F1003, the document “Subjective testing method for comparison of 360° video projection formats using HEVC” JVET-F1004, the description of exploration experiments JVET-F1011, the JVET common test conditions and evaluation procedures for HDR/WCG video JVET-F1020, and the JVET common test conditions and evaluation procedures for 360° video JVET-F1030, were approved. The JEM6 software implementation (version 6.0), and the 360Lib software implementation (version 3.0) were also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Review of results of previous meeting

- Review of AHG reports

- Review of inputs to the Call for Evidence, subjective testing of submitted material, and analysis of results

- Reports of exploration experiments

- Consideration of contributions and communications on project guidance

- Consideration of video technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the JEM software uses the HEVC reference software copyright license header language is the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the JEM software, and for evaluating proposals for technology to be included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

Note: This applies also to the 360Lib video conversion software as well as the JEM and HM.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It is reminded to send notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers was 741.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT: Adaptive multi-core transform.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP: Advanced temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BIO: Bi-directional optical flow.

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EE: Exploration Experiment – a coordinated experiment conducted toward assessment of coding technology.

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ECV: Extended Colour Volume (up to WCG).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- HDR: High dynamic range.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MDNSST: Mode dependent non-separable secondary transform.

- MMLM: Multi-model (cross component) linear mode.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- QTBT: Quadtree plus binary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- UCBDS: Unrestricted center-biased diamond search.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in JEM:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in P/B slice, and one CTB per luma CTU and two CTBs per chroma CTU in I slice.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size to a CU.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size to a CU.

## Opening remarks

- Reviewed logistics, agenda, working practices, policies, document allocation

- Results of previous meeting: JEM, meeting report, etc.

- Goals of the meeting: Evaluation of the results of the joint Call for Evidence (CfE), produce a new version of the JEM algorithm description and software, evaluation of status progress in EEs and new proposals, selection of test sequences and common test conditions for evaluation testing, expert viewing assessment of JEM status, improved 360Lib software, define new EEs.

- Pending adequate results of the joint CfE, produce preliminary joint Call for Proposals (to be issued by parent bodies)

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Thu. 13 Jul, 1st day

  - 0900-1300 Opening, AHG reports (chaired by JRO and GJS)

  - …

- …

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized (final number counts tbd)

- AHG reports (9) (section 2)

- Analysis, development and improvement of JEM (3) (section 3)

- Test material (7) (section 4)

- Call for Evidence (12) (section 5)

- Exploration experiments (45) (section 6)

  - EE1 and related: Intra Prediction (24)

  - EE2 and related: Decoder-side motion vector derivation (6)

  - EE3 and related: Adaptive QP for 360o video (7)

  - EE4 and related: 360o projection modifications and padding (7)

- Non-EE technology proposals (12) (section 7)

- Extended colour volume coding (7) (section 8)

- Coding of 360o video projection formats (23) (section 9)

- Complexity analysis (1) (section 10)

- Encoder optimization (2) (section 11)

- Metrics and evaluation criteria (0) (section 12)

- Withdrawn (2) (section 13)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 14)

- Project planning (section 15)

- Output documents, AHGs (section 16)

# AHG reports (9)

These reports were discussed Thursday 13 July 1000–1215 (chaired by GJS and JRO).

JVET-G0001 JVET AHG report: Tool evaluation (AHG1) [M. Karczewicz, E. Alshina]

This document reports the work of the JVET ad hoc group on Tool evaluation (AHG1) between the 6h JVET meeting at Hobart, Australia (31 March – 7 April 2017) and the 7th Meeting at Turin, Italy (13– 21 July 2017).

A total of 50+ e-mails related to AhG1 and EE activities were sent to the JVET reflector including EE tests scheduling and EE summary discussion

Algorithms included into JEM6.0 are described in JVET-F1001. There is a list of tools below. Tools modified at the JVCT-F meeting are marked as bold. The biggest change is addition of division-free bilateral filter after inverse transform.

JEM6.0 tools:

- Block structure

  - Larger Coding Tree Unit (up to 256x256) and transforms (up to 64x64)

  - Quadtree plus binary tree (QTBT) block structure

- Intra prediction improvements

  - 65 intra prediction directions

  - 4-tap interpolation filter for intra prediction

  - Boundary filter applied to other directions in addition to horizontal and vertical ones

  - Cross-component linear model (CCLM) prediction

  - Position dependent intra prediction combination (PDPC)

  - Adaptive reference sample smoothing

- Inter prediction improvements

  - Sub-PU level motion vector prediction

  - Locally adaptive motion vector resolution (AMVR)

  - 1/16 pel motion vector storage accuracy

  - Overlapped block motion compensation (OBMC)

  - Local illumination compensation (LIC)

  - Affine motion prediction

  - Pattern matched motion vector derivation modified in JEM6.0

  - Bi-directional optical flow (BIO)  modified in JEM6.0

  - Decoder-Side Motion Vector Refinement (DMVR)

- Transform

  - Explicit multiple core transform

  - Mode dependent non-separable secondary transforms Signal dependent transform (SDT)  disabled by default

- In-loop filter

  - Bilateral filter

  - Adaptive loop filter (ALF)

  - Content adaptive clipping

- Enhanced CABAC design

  - Context model selection for transform coefficient levels

  - Multi-hypothesis probability estimation

  - Initialization for context models

Performance progress for JEM (HM-KTA) in terms of BD-rate gain vs. encoder time increase in random access test configuration is demonstrated on Figure 1. Results are based on Software Development AHG reports. Some encoder run-time reduction is observed for JEM6.0 compared to JEM5.0, but still encoder run time is much higher compared to HM (>10).

Screen content coding tools were enabled for HEVC at the last meeting for class F (screen content) which is optional ( not included to the averaging) It needs to be noticed that SCM 16.15 outperforms JEM in “all-intra” (19%) and random access (7%) configurations even JEM’s encoder is much slower.

The progress of JEM performance in RA test configuration.

Coding performance compared to HEVC summary.

JEM6.0 (6th meeting)

| Test configuration | BD-rate |  |  | Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. | Dec. |
| All Intra | −20% | −28% | −28% | 63 | 2 |
| Random Access | −29% | −36% | −35% | 12 | 10 |
| Low Delay-B | −22% | −28% | −29% | 10 | 8 |
| Low Delay-B | −26% | −31% | −32% | 7 | 5 |

JEM5.0 (5rd meeting)

| Test configuration | BD-rate |  |  | Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. | Dec. |
| All Intra | −20% | −28% | −28% | 63 | 2 |
| Random Access | −29% | −35% | −34% | 12 | 10 |
| Low Delay-B | −22% | −29% | −29% | 10 | 8 |
| Low Delay-B | −26% | −31% | −32% | 7 | 5 |

Significant gain is observed in both three color components. In random access test highest gain over HEVC is observed for DaylightRoad test sequence (39.6%), lowest gain JEM shows for ToddlerFountain video (15.2% only).

JVET Common Test Conditions use integer QP settings Qp=22, 27, 32, 37 for all videos in test set. For CfE bit-matching was performed. The table below shows BD-rate performance JEM6.0 for video test set from CfE.

JEM5.0.1 vs HM under CfE test conditions (RA conditions)

| Resolution | Seq. | BD-rate |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| 4K | CrossWalk1 | −37.9% | −43.9% | −47.6% |
|  | FoodMarket3 | −34.9% | −46.5% | −48.9% |
|  | Tango1 | −36.4% | −55.0% | −49.5% |
|  | CatRobot1 | −40.2% | −52.4% | −45.5% |
|  | DaylightRoad1 | −40.7% | −53.5% | −38.1% |
|  | BuildingHall1 | −33.3% | −41.4% | −46.5% |
|  | ParkRunning2 | −31.6% | −26.2% | −29.3% |
|  | CampfireParty | −37.9% | −35.7% | −56.6% |
| 2K | BQTerrace | −30.4% | −50.5% | −61.2% |
|  | RitualDance | −27.8% | −37.9% | −41.7% |
|  | TimeLapse | −26.8% | −61.3% | −67.1% |
|  | BasketballDrive | −32.1% | −46.9% | −43.3% |
|  | Cactus | −36.0% | −49.1% | −45.1% |
| 4K All |  | −36.6% | −44.3% | −45.2% |
| 2K All |  | −30.6% | −49.1% | −51.7% |
| All |  | −34.3% | −46.2% | −47.7% |

In the SDR category (most relevant to this AhG) 2 CfE responses were submitted, even higher performance than the JEM was demonstrated.

At the 2nd JVET meeting Exploration Experiments practice was established. In 6th JVET meeting 4 EEs were created. For each new coding tool under consideration special SW branch was created. After implementation of each tool announcement via JVET reflector was done. For all 4 EEs input contribution for this meeting were submitted. Summary of exploration experiments is provided in JVET-F1001.

In total 20 contributions on coding tools were noted to have been submitted in following categories:

- Structure (0)

- Intra (13),

- Inter (4),

- Transform (0),

- Entropy coding (1),

- In-loop filter (2).

The AHG recommended to:

- Consider encoder complexity as one of the criteria when evaluating the tools. Encourage further encoder and decoder complexity reduction.

- Review all the related contribution.

- Continue the Exploration Experiments practice.

In the discussion, it was remarked that the primary difference between the results with the CfE test conditions (34%) and the usual JEM CTC measured performance (29%) appears to be due to the bit rate range that was tested and the selection of test sequences (2K and 4K only, without screen content, and not including the most difficult test sequence).

JVET-G0002 JVET AHG report: JEM algorithm description editing (AHG2) [J. Chen, E. Alshina, J. Boyce]

This document reports the work of the JVET ad hoc group on JEM algorithm description editing (AHG2) between the 6th meeting at Hobart, AU (31 March – 7 April 2017) and the 7th JVET meeting at Torino, IT, (July 13-21, 2017).

During the editing period, on top of JVET-E1001 Algorithm Description of Joint Exploration Test Model 5, the editors worked on the following two aspects to produce the final version of JVET-F1001 Algorithm Description of Joint Exploration Test Model 6.

- Integrate the following adoptions, which change the encoding or decoding process, at the 6th JVET meeting

  - JVET-F0028, BIO without block extension

  - JVET-F0031, removal of redundant syntax signalling for transform skip

  - JVET-F0032, enhanced FRUC Template Matching Mode, aspect 2

  - JVET-F0096, division-free bilateral in-loop filter

- Editorial improvements by editors

The document also incorporated the fix of text and software bug related to transformed coefficients zero-out for large transform (Ticket#44) which was confirmed at the Hobart JVET meeting.

Currently the document containts the algorithm description as well as encoding logic description for all  new coding features in JEM6.0 beyond HEVC. Compared to HEVC, the following new coding features are included in JEM6.

- Block structure

  - Quadtree plus binary tree (QTBT) block structure with larger CTUs (software supports 256×256, CTC use 128×128)

- Intra prediction

  - 65 intra prediction directions with improved intra mode coding

  - 4-tap interpolation filter for intra prediction

  - Boundary filter applied to other directions in addition to horizontal and vertical ones

  - Cross-component linear model (CCLM) prediction

  - Position dependent intra prediction combination (PDPC)

  - Adaptive reference sample smoothing

- Inter prediction

  - Sub-PU level motion vector prediction

  - Locally adaptive motion vector resolution (AMVR)

  - 1/16 pel motion vector storage accuracy

  - Overlapped block motion compensation (OBMC)

  - Local illumination compensation (LIC)

  - Affine motion prediction

  - Pattern matched motion vector derivation

  - Bi-directional optical flow (BIO)

  - Decoder-side motion vector refinement

- Transform

  - Large block-size transforms with high-frequency zeroing

  - Adaptive multiple core transform

  - Mode dependent non-separable secondary transforms

  - Signal dependent transform (SDT, disabled by default)

- Loop fillter

  - Bilateral filter

  - Adaptive loop filter (ALF)

  - Content adaptive clipping

- Enhanced CABAC design

  - Context model selection for transform coefficient levels

  - Multi-hypothesis probability estimation

  - Initialization for context models

Among all of them, the bilateral filter was newly adopted at the 6th JVET metting. The pattern matched motion vector derivation method was enhanced by adding the switching feature between uni-prediction and bi-prediction in template matching merge mode. BIO was modified in a way that the additional memory access is eliminated.

The AHG recommended to:

- Continue to edit the algorithm description of Joint Exploration Model document to ensure that all agreed elements of JEM are described

- Continue to improve the editorial quality of the algorithm description of Joint Exploration Model document and address issues relating to mismatches between software and text.

- Identify and improve the algorithm description for critically important parts of JEM design for better understanding of complexity.

JVET-G0003 JVET AHG report: JEM software development (AHG3) [X. Li, K. Suehring]

This report summarizes the activities of the AhG3 on JEM software development that has taken place between the 6th and 7th JVET meetings.

Software development was continued based on the HM-16.6-JEM-5.1 version. A branch was created in the software repository to implement the JEM-6 tools based on the decisions noted in section 12.4 in the notes of 6th JVET meeting. All integrated tools were included in macros to highlight the changes in the software related to that specific tool.

HM-16.6-JEM-6.0 was released on Apr. 27th, 2017.

Several branches were created for exploration experiments on top of HM-16.6-JEM-6.0. Note that these branches are maintained by the proponents of exploration experiments.

During the 6th JVET meeting, it was requested to clean bug fix related macros in JEM. Such cleanings have been made in branch HM-16.6-JEM-6.0-dev. A list of macros that have been removed was provided in the report.

The JEM software is developed using a subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/

The implementation of JEM-6 tools has been performed on the branch

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/HM-16.6-JEM-5.1-dev

The released version of HM-16.6-JEM-6.0 can be found at

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-6.0

The branches of exploration experiments can be found at

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates

The performance of HM-16.6-JEM-6.0 over HM-16.6-JEM-5.0.1 and HM-16.15 under test conditions defined in JVET-B1010 is summarized as follows. As agreed in 6th JVET meeting, HM-16.15-SCM-8.4 is used as HM anchor for class F sequences. Note that 8-bit internal bit depth was used for HM-16.15-SCM-8.4 when testing class F.

[Add table from report - paste behaving strangely]

The JEM bug tracker is located at

https://hevc.hhi.fraunhofer.de/trac/jem

It uses the same accounts as the HM software bug tracker. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Please file all issues related to JEM into the bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

The AHG recommends

- To continue software development on the HM-16.6 based version

- Encourage people to test JEM software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bitstreams/test cases that trigger bugs in the JEM.

- Clarify the internal bit-depth settings for HM-16.15-SCM-8.4 when testing class F.

It was noted that the prior JCT-VC SCC CTCs use 8 bit encoding and the Class F test sequences are 8 bit. The coding performance difference was small, it was agreed to use 10 bit encoding always for consistency in comparisons.

SCC and 4:4:4 remain key issues. No progress was reported on SCC support.

JVET-G0004 JVET AHG report: Test material (AHG4) [T. Suzuki, V. Baroncini, J. Chen, J. Boyce, A. Norkin]

The test sequences used for CfE (JVET-E1002) were available on ftp://jvet@ftp.ient.rwth-aachen.de in directory “/jvet-cfe” (please contact the JCT-VC chairs for login information).

HM/JEM anchors (defined in JVET-F1002) were generated and verified by cross checker.

HM anchors:

ftp://jvet@ftp.ient.rwth-aachen.de/jvet-cfe/anchors-hm

JEM anchors:

ftp://jvet@ftp.ient.rwth-aachen.de/jvet-cfe/anchors-jem

Additional 4 rate points for HM were discussed and defined as in the following table (columns with non-integer bit rate identifiers).

SDR target bit rates

|  | Target bit rates [kbps] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequences | Rate 1 | 1.5 | Rate 2 | 2.5 | Rate 3 | 4.5 | Rate 4 | 4.5 |
| UHD1, UHD2 | 1000 | 1250 | 1500 | 1950 | 2400 | 3200 | 4000 | 5200 |
| UHD3, UHD4, UHD5 | 1500 | 1950 | 2400 | 3200 | 4000 | 5500 | 7000 | 9100 |
| UHD6 | 800 | 1000 | 1200 | 1600 | 2000 | 2650 | 3300 | 4300 |
| UHD7, UHD8 | 2000 | 2650 | 3300 | 4650 | 6000 | 8000 | 10000 | 13000 |
| HD1 | 400 | 500 | 600 | 800 | 1000 | 1350 | 1700 | 2300 |
| HD2 | 900 | 1200 | 1500 | 2050 | 2600 | 2950 | 4300 | 5600 |
| HD3 | 180 | 230 | 280 | 380 | 480 | 640 | 800 | 1050 |
| HD4 | 800 | 1000 | 1200 | 1600 | 2000 | 2750 | 3500 | 4600 |
| HD5 | 500 | 650 | 800 | 1000 | 1200 | 1600 | 2000 | 2750 |

Colour primary and matrix conversion issue

The sequences provided by SJTU were converted to YUV assuming SMPTE ST 170 (ITU-R BT.601). When ffmpeg is used with default setting, YUV file converted from RGB will be in SMPTE ST 170 or ST 240. Those needed to be revised. SJTU is working to revise the test sequences.

BCOM test sequence (CatRobot) was converted by ffmpeg by default setting (JCTVC-V0086). It means the sequence should be revised ? (4:4:4 -> 4:2:0 conversion was done by HDR tools)

Other test sequences for SDR are BT.709 container, not BT.2020 (Some of test sequence owner said they captured as RAW of Sony F65).

Chroma sampling position issue

The default setting of ffmpeg chroma subsampling assumes chroma location type 1 for 4:2:0 data. (the sampled chroma data have a placement right in the middle of 4 corresponding luma samples.) However, most of the current HDTV systems use chroma location type 0 (horizontally co-sited, placed in the middle vertically) or chroma location 2 (co-sited with the top left luma sample). BT.2020 and BT.2100 specs even mandate the use of location 2 where as most software assume type 0 or type 2 depending on colour primaries when converting back to 4:4:4.

The Huawei test sequences seem to be converted by ffmpeg with default setting (chroma location is centered for the surrounding luma, i.e., type 1). Those should be revised.

Full range issue

Cactus uses full range of video signal. If original 4:4:4 file is available, it is better to re-generate.

Contributions

Relevant contributions to this meeting included submissions on CfE anchor generation, a new test sequence, studies for the current test materials, and non-normative encoding techniques for adaptive quantization and denoising pre-processing.

Conclusions and discussions

The AHG recommended:

- To review all related contributions.

- To evaluate visual quality of HM/JEM anchors.

- To perform viewing of new test sequences

- To discuss further actions to select new test materials for JVET activity, toward CfP.

In the discussion, the use of an assumption of BT.601 for conversion to YUV should be fixed. BT.709 seems OK to use.

It was commented that if we modify a test sequence to fix the chroma location or narrow range, we should use a different name for the sequence to avoid confusion over which version to use.

It was commented that there may be other problems with colour conversion relating to the use of ffmpeg as a colour conversion tool. HDRtools includes format conversion support, and using it should help avoid such problems.

JVET-G0005 JVET AHG report: Memory bandwidth consumption of coding tools (AHG5) [X. Li, E. Alshina, T. Ikai, H. Yang]

The document summarizes activities of AhG on memory bandwidth consumption of coding tools between the 6th and the 7th JVET meetings.

One relevant contribution was noted (JVET-G0061).

JVET-G0006 JVET AHG Report: 360° video conversion software development (AHG6) [Y. He, V. Zakharchenko]

The document summarizes activities on 360-degree video content conversion software development between the 6th and the 7th JVET meetings.

The 360Lib-3.0 software package integrated all adoptions about projection format and metrics calculation:

- Metrics:

  - Improved cross-format S-PSNR-NN (JVET-F0042);

- Projection formats and frame packing:

  - Adjusted cubemap projection (JVET-F0025);

  - Rotated sphere projection (JVET-F0036);

  - Compact octahedron projection with padding (JVET-F0053);

- Software:

  - Platform independent floating point to integer conversion (JVET-F0041);

  - Modification for spherical rotation (JVET-F0056);

  - Fix for bug ticket #47, #49

- 360Lib-3.0 related release:

  - 360Lib-3.0rc1 with support of HM-16.15 and JEM-6.0rc1 was released on Apr. 21th, 2017;

  - 360Lib-3.0 with support of HM-16.15 and JEM-6.0 was released on May. 1st, 2017;

  - 360Lib-3.0 based HM-16.15 testing results was released on May. 1st, 2017;

  - 360Lib-3.0 based JEM-6.0 testing results was released on May. 16th, 2017.

- Bug fix after 360Lib-3.0 release:

  - Ticket #51 and #52 were submitted after 360Lib-3.0 release. They do not affect CTC and EE tests. The fixes were provided in the development branch (360Lib-3.1-dev).

The 360Lib software is developed using a Subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

The released version of 360Lib_3.0 can be found at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-3.0/

360Lib-3.0 testing results can be found at:

ftp.ient.rwth-aachen.de/testresults/360Lib-3.0

360Lib bug tracker

https://hevc.hhi.fraunhofer.de/trac/jem/newticket?component=360Lib

The tables below measure performance characteristics for random access encoding comparisons.

The table below lists the HM-16.15 based coding performance with different projection formats according to 360o video CTC (JVET-F1030) compared to ERP coding.

HM-16.15-360Lib-3.0 testing (HM ERP coding as anchor)

| Projection format | E2E WS-PSNR |  |  | E2E WS-PSNR for 8K sequences |  |  | E2E WS-PSNR for 4K sequences |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| CMP | -3.5% | -2.5% | -2.5% | -4.6% | -2.7% | -3.2% | -1.8% | -0.9% | -0.2% |
| EAP | 11.5% | -2.3% | -3.2% | 14.1% | -2.1% | -2.6% | 5.4% | -1.1% | -3.1% |
| OHP | -2.2% | 1.5% | 0.4% | -4.7% | 0.2% | -0.9% | 3.3% | 6.8% | 5.3% |
| ISP | -5.2% | -0.4% | -1.3% | -7.2% | -1.1% | -2.4% | -0.9% | 3.3% | 2.8% |
| SSP | -9.7% | -2.9% | -3.5% | -10.7% | -3.8% | -4.2% | -7.6% | 1.5% | 0.1% |
| ACP | -11.0% | -6.1% | -6.3% | -12.0% | -6.0% | -6.1% | -9.5% | -5.3% | -5.5% |
| RSP | -9.9% | -5.1% | -5.2% | -11.3% | -5.9% | -6.3% | -7.3% | -1.5% | -1.5% |

The table below lists the JEM-6.0 based coding performance with different projection formats compared to ERP coding

JEM-6.0-360Lib-3.0 testing (JEM ERP coding as anchor)

| Projection format | E2E WS-PSNR |  |  | E2E WS-PSNR for 8K sequences |  |  | E2E WS-PSNR for 4K sequences |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| CMP | -4.2% | -1.6% | -2.5% | -5.5% | -2.2% | -3.4% | -1.8% | 1.0% | 0.2% |
| EAP | 13.1% | -5.3% | -7.6% | 16.1% | -4.2% | -6.9% | 6.6% | -5.7% | -8.4% |
| OHP | -3.0% | 3.6% | 0.4% | -5.7% | 0.7% | -1.2% | 3.2% | 11.8% | 5.4% |
| ISP | -5.6% | 1.4% | -1.5% | -8.1% | -0.5% | -2.5% | -0.3% | 7.0% | 2.2% |
| SSP | -11.6% | -4.4% | -6.6% | -11.9% | -4.4% | -6.0% | -10.4% | 0.1% | -4.3% |
| ACP | -12.6% | -7.3% | -8.8% | -13.7% | -7.3% | -8.5% | -10.7% | -6.0% | -8.7% |
| RSP | -11.8% | -8.2% | -9.0% | -12.5% | -8.4% | -9.1% | -10.4% | -4.9% | -6.5% |

The table below compares the JEM-6.0 ERP coding with HM-16.15 ERP coding.

JEM-ERP vs HM-ERP coding (HM ERP coding as anchor)

| JEM ERP vs. HM ERP | E2E SPSNR-NN |  |  | E2E SPSNR-I |  |  | E2E CPP-PSNR |  |  | E2E WS-PSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Trolley | -15.6% | -31.2% | -33.7% | -15.6% | -31.0% | -33.5% | -15.7% | -30.9% | -33.5% | -15.7% | -31.2% | -33.8% |
| GasLamp | -20.7% | -45.2% | -42.2% | -20.7% | -45.0% | -41.9% | -20.7% | -45.0% | -41.9% | -20.6% | -45.3% | -42.2% |
| Sb_in_lot | -25.0% | -34.3% | -44.8% | -25.0% | -34.1% | -44.6% | -25.0% | -34.1% | -44.7% | -25.0% | -34.3% | -44.8% |
| Chairlift | -30.3% | -48.7% | -45.8% | -30.3% | -48.6% | -45.7% | -30.3% | -48.5% | -45.8% | -30.3% | -48.7% | -45.9% |
| KiteFlite | -16.8% | -35.8% | -39.1% | -16.8% | -35.4% | -38.9% | -16.8% | -35.4% | -38.8% | -16.8% | -35.7% | -39.1% |
| Harbor | -19.4% | -42.1% | -42.4% | -19.4% | -41.9% | -42.1% | -19.5% | -41.9% | -42.2% | -19.5% | -42.2% | -42.5% |
| PoleVault | -18.2% | -19.1% | -20.8% | -18.1% | -18.3% | -20.2% | -18.1% | -18.4% | -20.3% | -18.2% | -19.1% | -21.0% |
| AerialCity | -25.3% | -45.2% | -31.3% | -25.2% | -44.5% | -30.5% | -25.2% | -44.5% | -30.5% | -25.3% | -45.3% | -31.3% |
| DrivingInCity | -29.0% | -42.8% | -37.6% | -28.8% | -42.0% | -36.9% | -28.8% | -41.8% | -36.8% | -29.0% | -42.8% | -37.6% |
| DrivingInCountry | -28.8% | -35.5% | -39.5% | -28.7% | -34.8% | -39.0% | -28.7% | -34.8% | -39.0% | -28.8% | -35.5% | -39.5% |
| Overall | -22.9% | -38.0% | -37.7% | -22.9% | -37.6% | -37.3% | -22.9% | -37.5% | -37.3% | -22.9% | -38.0% | -37.8% |
| 8K | -21.3% | -39.6% | -41.3% | -21.3% | -39.3% | -41.1% | -21.3% | -39.3% | -41.1% | -21.3% | -39.6% | -41.4% |
| 4K | -27.7% | -41.2% | -36.1% | -27.6% | -40.4% | -35.5% | -27.6% | -40.4% | -35.4% | -27.7% | -41.2% | -36.1% |

The table below lists the conversion-only end-to-end WS-PSNR for different projection formats compared to that of the ERP format.

Conversion only results (ERP format as anchor)

| Projection format | E2E WS-PSNR |  |  | E2E WS-PSNR for 8K sequences |  |  | E2E WS-PSNR for 4K sequences |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| ERP | 45.63 | 56.69 | 56.52 | 45.60 | 58.22 | 57.81 | 45.68 | 54.39 | 54.59 |
| CMP | 0.68 | 0.34 | 0.31 | 0.81 | 0.46 | 0.46 | 0.47 | 0.15 | 0.08 |
| EAP | -1.49 | -0.34 | -0.09 | -1.89 | -0.48 | -0.09 | -0.90 | -0.14 | -0.09 |
| OHP | 1.29 | 0.52 | 0.51 | 1.49 | 0.56 | 0.54 | 0.99 | 0.46 | 0.45 |
| ISP | 2.17 | 0.72 | 0.69 | 2.25 | 0.63 | 0.60 | 2.05 | 0.86 | 0.82 |
| SSP | 3.80 | 2.22 | 2.17 | 2.19 | 0.64 | 0.62 | 6.22 | 4.60 | 4.50 |
| ACP | 2.92 | 0.92 | 0.89 | 2.83 | 0.86 | 0.85 | 3.06 | 1.01 | 0.95 |
| RSP | 3.19 | 1.67 | 1.64 | 2.28 | 0.66 | 0.64 | 4.56 | 3.18 | 3.13 |

The AHG recommended:

- To continue software development of the 360Lib software package.

JVET-G0007 JEM coding of HDR/WCG material (AHG7) [A. Segall, E. Francois, D. Rusanovskyy]

This document summarizes the activity of AHG7: JEM Coding of HDR/WCG Material between the  6th meeting in Hobart, AU (31 March – 7 April 2017) and the 7th meeting in Torino, IT (13 – 21 July 2017).

Accomplishments by the AhG reportedly included:

- HM anchor bit-streams for the CfE HDR category were prepared and made available

- JEM anchor bit-streams for the CfE HDR category were prepared and made available

- HM and JEM rate points and bit-streams for the HLG test material were prepared and made available

Note that more information of these activities is provided in the input documents (listed below).

Twelve contributions related to the AHG were identified, including two CfE responses, a document about the JEM anchor for HDR, a contribution of a new test sequence, and eight other contributions (on QP delta, HLG issues, HDR metrics, SDR content in HDR containers).

The AHG recommended the following:

- Review all input contributions

- Review responses to the HDR CfE category and prepare the Summary of Respones as appropriate

- Review new HDR test material and rate point information; discuss if sequences and configurations for the HDR CTC or CfP should be modified.

- Prepare the HDR video section of the Draft Call for Proposals

In the discussion, the presenter noted that a BoG recommendation at the previous meeting was to reduce the numer of metrics used for HDR evaluation. It was commented that the various metrics tend to perform roughly consistently for SDR but inconsistenly for HDR.

JVET-G0008 JVET AHG report: 360° video coding tools and test conditions [J. Boyce, A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye]

This document summarizes the activity of AHG8: 360° video coding tools and test conditions between the 6th meeting in Hobart, AU (31 March – 7 April 2017) and the 7th meeting in Torino, IT (13 – 21 July 2017).

Output documents JVET-F1003, JVET-F1004, and JVET-F1030 were made available.

HM anchors for the CfE 360 video category were prepared by InterDigital, cross-checked by Samsung and Qualcomm, and made available on May 19. JEM anchors for the CfE 360 video category were prepared by Samsung, cross-checked by InterDigital, and made available on June 10. Dynamic viewport trajectories for CfE subjective viewing were selected by Mathias Wien, Minhua Zhou and Jill Boyce, and documented in JVET-G0066.

Anchors were prepared for the OMAF projections subjective viewing (by Samsung, cross-checked by Intel), and provided with a reporting template in an update version of JVET-F1004 on May 18.

Email activity on the reflector was limited to the kickoff message and announcements about availability of anchors.

There were 37 contributions noted as related to 360º video coding, which were classified as follows, with non-final document counts as listed.

- Video CfE responses (4)

- EE3 and related (7)

- EE4 and related: 360 projection modifications and padding (7)

- Quality assessment and metrics (4)

- Projection formats and padding (9)

- Coding tools (1)

- Use cases (2)

- Test content (3)

The AHG recommends the following:

- Review input contributions

- Review 360 video CfE responses, and contribute towards preparation of Summary of Responses to CfE document

- Assist MPEG OMAF activity with subjective evaluation of projection formats

- Review new 360 video test material, and consider adding or replacing test sequences for common test conditions and/or CfP

- Refine common test conditions for 360° video, including objective metrics

- Prepare a 360° video section of a preliminary Call for Proposals

JVET-G0009 JVET AHG report: 4:4:4 support in JEM (AHG9) [A. M. Tourapis, X. Li]

The document summarizes activities of the AhG on 4:4:4 support in the JEM software that took place between the 6th and the 7th JVET meetings.

One relevant contribution was noted: JVET-G0148.

An email initiating the activities for this AHG was circulated on May 16th. The email reintroduced the mandates of the activity and provided a test plan for performing the evaluation of the software. In particular, the following plan was suggested:

- First, identify a few sequences for running our tests with. We would suggest using very low resolution sequences (e.g. QCIF and/or CIF) since the intent is not to run comparison and true video coding tests (at least at this stage) but instead examine whether the software supports (or not) 4:4:4 coding. Low resolution sequences could permit us to perform more and faster tests.

- Sequences should be split into two groups, one sequence (e.g. the 4:4:4 QCIF foreman sequence) for conducting an early fail/pass test, and a second group with more (very short) sequences for which we will need to run a more thorough test, only and only if the first test passes. The fail/pass test would include a simple encoding run, and if that passes (without a crash) a decoding run that is required to fully match the output of the encoder. If any of these steps fail, then the test would fail. We are not at all concerned about RD performance here. If the fail/pass test passes, then the secondary test is performed so as to determine whether there are other problems that are not immediately identifiable. Again the result of this would be a fail or pass.

- Given these tests, create an experiment that would help us understand the state of the software. The following was suggested:

  - Identify all macros in the software. Then perform the fail/pass test on the following two cases:

    - all macros enabled

    - all macros disabled

    - If the second case passes, then also run the longer test to see if the second case also passes that. if the second case fails, then we would have a more serious problem and we may wish to sit down and identify what is broken in the software. A more thorough analysis experiment would need to be considered at that stage. If any tool passes, then combination of tools could be considered. A possible solution would be to first test if enabling all would pass the test since then we would avoid having to try all possible combinations.

  - If the second case passes both tests then start enabling one by one all macros in the software and run the fail/pass test on all of them independently. If any of them again pass the fail/pass test, the more thorough test could be run. Here we may wish to identify which tools we should be enabling first based on some type of priority, or whether we should just enable tools based on some predefined order (e.g. alphabetic or order defined in the software).

- An encoding/decoding pass btw does not mean that everything works fine. We should also compare (if all goes well) RD performance vs the HM. RD performance also vs 4:2:0 JEM could be of interest. But likely we will not have much time to do this at this stage.

One additional company (Mediatek) showed interest in participating and assisting with these efforts. Several new, low resolution 4:4:4 sequences, at a resolution of 320x180, were generated and circulated amongst the interested parties. These sequences were used for the above tests as well as some of the existing 4:4:4 sequences. As part of the tests, a few bugs were identified and resolved.

Given the performed tests, it has been reported that the software seems to currently operate in terms of 4:4:4 coding, with, however all RExt tools of HEVC disabled (e.g. the cross-component transform). It is also not known whether the JEM coding tools also behave as intended; however, no mismatches were reported when encoding and decoding.

The AHG recommended

- To review all related contributions.

# Analysis, development and improvement of JEM (3)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0090 Unified adaptive search range setting in JEM and HM [T. Ikai, Y. Yasugi (Sharp)]

JVET-G0144 Cross-check of Unified adaptive search range setting in JEM and HM (JVET-G0090) [Y.-H. Ju, P.-H. Lin, C.-C. Lin, C.-L. Lin (ITRI)] [miss] [late]

JVET-G0101 On internal QP increase for bitrate matching [P. Hanhart, Y. He, Y. Ye (InterDigital), X. Ma, H. Chen, H. Yang, M. Sychev (Huawei)]

# Test material (7)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0053 Test Sequences for Virtual Reality Video Coding from LetinVR [R. Guo, W. Sun (LetinVR)]

JVET-G0055 Test Sequences for Virtual Reality Video Coding from InterDigital [E. Asbun, Y. He, P. Hanhart, Y. Ye (InterDigital)] [miss] [late]

JVET-G0063 New Test Sequence of 4K Hybrid Log-Gamma [T. Tsukuba, M. Ikeda, T.Suzuki (Sony)] [miss] [late]

JVET-G0072 AHG7: Analysis of currently proposed HLG content [E. François, F. Le Leannec, F. Galpin (Technicolor)]

JVET-G0096 AhG4: Evaluation on drone test sequences [X. Zheng, W. Li (DJI)] [miss] [late]

JVET-G0145 AHG4: Evaluation report of drone test sequences [Y.-H. Ju, C.-C. Lin, P.-H. Lin, C.-L. Lin (ITRI)] [miss] [late]

JVET-G0147 New Test Sequences for Spherical Video Coding from GoPro [Adeel Abbas, David Newman] [miss] [late]

# Call for Evidence (12)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0021 FastVDO Response to JVET CfE for HDR [P. Topiwala, M. Krishnan, W. Dai (FastVDO)]

JVET-G0022 CfE response to the HDR category from Technicolor [E. François, F. Le Leannec (Technicolor)]

Discussed Thursday 12:20 (GJS & JRO).

This contribution describes the Technicolor response to the Joint Call for Evidence on Video Compression with Capability beyond HEVC, for the HDR category. The proposed technology is directly based on the Exploratory Test Model mapping (a.k.a. “reshaping” with luma-dependent chroma scaling) previously tested in MPEG and JCT-VC. The proposed mapping, applying directly to the input Y′CbCr 4:2:0 BT.2100 PQ content, is based on a single scaling table used both for luma and chroma. The scaling table is used to build a mapping 1D (piece-wise linear) look-up-table applied to the luma component, and to perform a luma-based cross-component scaling of the chroma components. One scaling table per shot was used in the reported experiments.

The described scheme is something not supported by the existing CRI SEI message.

In previous work, a non-normative encoding approach using adaptive QP selection based on luma level was developed, and a version of this has been used in the anchors. (As a non-normative approach, this requires overhead bits for signalling the QP).

The described scheme has a somewhat similar spirit to the non-normative technique (but on a sample-wise basis rather than block-wise basis). It involves sending information to specify the mapping function. The encoder chooses the function parameters based on an analysis of the video content. This could be done based on statistics of IRAP pictures, for example, or by pre-analysis (and possibly shot-by-shot analysis of video scenes). As tested, the encoder considered only the first frame of each test sequence for setting the parameter values.

The encoder process for deciding what mapping function to send to the decoder was not precisely described in the contribution.

The JEM was used for the core encoding-decoding process (with QP adaptation disabled).

HDR metrics using JEM coding with HM anchors as reference

HDR metrics using JEM coding with JEM anchors as reference

Note: for wPSNRV of Cosmos1, the RD curves do not overlap.

As discussed in previous work, this brings up the question of whether to consider this as “normative” or as a pre-/post-processing technique. The proponent remarked that the pictures are not really “reasonably viewable” if the technique is not applied at the decoder side.

With out-of-order decoding, one may need to store two copies of each picture or to incorporate the post-processing within a display/output process.

The gain shown in the objective metrics is not especially large when considering the effect of the proposed feature individually, depending on the metric. The proponent acknowledged that the visual effect is also not especially large.

In previous work, a similar process was shown to be possible to use to provide a form of SDR backward compatibility, where the non-remapped signal is interpreted as an SDR picture. Such a usage is not really relevant in the context of a new coding design as primarily considered in JVET.

For JVET purposes, it also does not seem to especially matter whether something is affecting the core decoding process or not, since the entire core decoding process would presumably be new.

This proposal raises the question of whether this sort of processing is intended to be allowed in the CfE/CfP or not, and whether sequence-by-sequence customization is intended to be allowed. The proponent said they considered this technique to be part of the decoding process, although it lies outside the prediction loop (as it is a process that is necessary for producing properly viewable pictures if the corresponding pre-processing has been applied at the encoder side). We need to consider and clarify this point.

(For that matter, the deblocking and SAO filtering of a non-reference picture, and even other aspects of the decoding process of a non-reference picture, is also outside of the prediction loop.)

It was commented that the CfE is silent on the question of multi-pass and hand-tuning encoding techniques. We need to consider and clarify this point.

Some further consideration of quantization and luma-dependent processing issues may also be desirable for finalizing the CfP.

JVET-G0023 Qualcomm’s response to Joint CfE in 360-degree video category [M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

Discussed Thursday 15:00 (GJS & JRO).

The interior of the encoding/decoding processes is not modified. This contribution is only about a projection mapping.

An adjusted cubemap projection (ACP) with padding is submitted to JVET’s Call for Evidence on 360-degree video coding. ACP was previously described in F0025 but without padding. The padded ACP reportedly offers efficient compression of 360-degree video and reduced seam artefacts in the rendered viewports. ACP is an enhancement of CMP by adjusting the sampling on the cube faces to be nearly uniform. With the proposed padding scheme for ACP, the RA coding gain is reportedly 10.7% (e2e WS-PSNR) compared with the equirectangular projection in accordance with the JVET common test conditions for 360-degree video, while for CfE conditions, the reported coding gain is reportedly 8.1% (e2e WS-PSNR).

The scheme is included in 360Lib version 3.

F0025 had slightly higher objective coding gain.

A 3x2 layout of the cubemap is used, with “padding” around 3x1 regions. The “padding” is done by extending the rectangular faces, not by duplicating samples from spatially adjacent faces.

The “padding” was 4 luma samples wide.

Blending is not used for the duplicated regions. The “padding” areas are discarded after decoding.

JVET-G0024 InterDigital’s Response to the 360° Video Category in Joint Call for Evidence on Video Compression with Capability beyond HEVC [P. Hanhart, X. Xiu, F. Duanmu, Y. He, Y. Ye (InterDigital)]

Discussed Thursday 15:30 (GJS & JRO).

This proposal is the response to the joint Call for Evidence (CfE) on video compression with capability beyond HEVC by InterDigital in the 360° video category.

Four technologies are highlighted:

- “Hybrid cubemap projection” (similar in concept to ACP, with a generalization of the warping parameters) without “padding”, with a 3x2 layout slightly different than 360Lib default in order to facilitate prediction order

- Intra prediction using face continuity

- Inter prediction with geometry padding (which would involve limiting the MV range to prevent excessive padding unless implemented on the fly in the decoder – currently allowing half the face width/height of off-picture region)

- In-loop filtering with face continuity

The table below reports the BD-rate of the response compared to the HM 360° anchors.

Comparison of the response with the HM 360° anchors in terms of coding performance.

| Sequence | E2E WS-PSNR Y | E2E WS-PSNR U | E2E WS-PSNR V |
| --- | --- | --- | --- |
| SkateboardInLot | −39.64% | −63.18% | −70.08% |
| Chairlift | −45.95% | −66.27% | −60.73% |
| KiteFlite | −23.02% | −56.60% | −63.16% |
| Harbor | −25.92% | −56.11% | −56.50% |
| Trolley | −21.56% | −44.27% | −50.52% |
| Average | −31.22% | −57.29% | −60.20% |

- Comparison with the JEM-based 360° anchors

The table below reports the BD-rate of the response compared to the JEM 360° anchors. As reported in the table, the response can effectively improve the coding performance of the current JEM by providing an average BD-rate savings of 11.5% based on E2E WS-PSNR Y. The maximum bit rate savings of the response can reach 22% (sequence Chairlift).

Comparison of the response with the JEM 360° anchors in terms of coding performance.

| Sequence | E2E WS-PSNR Y | E2E WS-PSNR U | E2E WS-PSNR V |
| --- | --- | --- | --- |
| SkateboardInLot | −15.38% | −25.27% | −24.29% |
| Chairlift | −22.06% | −17.77% | −17.60% |
| KiteFlite | −6.35% | −9.09% | −14.69% |
| Harbor | −7.41% | −12.71% | −12.91% |
| Trolley | −6.41% | −6.88% | −11.25% |
| Average | −11.52% | −14.35% | −16.15% |

ACP is a special case of the HCP scheme.

The parameters of the warping were calculated from the first picture of each IRAP segment using an optimization to minimize the conversion error.

In the discussion, it was commented that the interaction with tiles should be considered, as the alignment of tile boundaries with the faces may be desirable.

JVET-G0025 Samsung's response to Joint CfE on Video Compression with Capability beyond HEVC (360° category) [E. Alshina, K. Choi, V. Zakharchenko, S. N. Akula, A. Dsouza, C. Pujara, K. K. Ramkumaar, A. Singh] [late]

Discussed Thursday 16:15 (GJS & JRO).

The interior of the encoding/decoding processes is replaced by a general-purpose coding engine described in JVET-G0029 (a CfE response in the SDR category). This contribution focuses on the projection mapping.

The contribution uses a new layout for compact icosahedral projection and doesn’t use any 360°-specific changes at the block level.

The projection mapping has two aspects

- A new layout for Compact Icosahedral Projection (ISP), with padding of 8 luma samples on vertical & horizontal discontinuous boundaries and 12 luma samples on diagonal discontinuous boundaries. No special padding was used around the exterior picture boundaries.

- Sphere rotation prior to conversion to ISP.

On, average 30% (Y), 43% (Cb) and 47% (Cr) BD-rate gain over the HEVC CfE anchor was reported (as a mixture of the effects of the projection mapping and the general-purpose coding engine modifications).

Compared to the JEM CfE anchor, the benefit was estimated roughly to be 7%, split roughly into 2% from the coding engine and 5% from the projection mapping changes.

BD-rate performance relatively to HM 360° CfE anchor

| Test sequence | E2EWS-PSNR BD-rate |  |  |
| --- | --- | --- | --- |
|  | Y | Cb | Cr |
| Trolley | −21.2% | −28.0% | −38.7% |
| Skateboarding_in_lot | −36.4% | −45.5% | −55.9% |
| Chairlift | −41.9% | −51.3% | −43.0% |
| KiteFlite | −22.8% | −42.1% | −52.0% |
| Harbor | −26.5% | −45.9% | −46.9% |
| Average | −29.8% | −42.6% | −47.3% |

JVET-G0026 Polyphase subsampling applied to 360-degree video sequences in the context of the Joint Call for Evidence on Video Compression [A. Gabriel, E. Thomas (TNO)]

Discussed Thursday 16:45 (GJS & JRO).

Bitstreams were not submitted for this contribution. The proponent said that the results were varying substantially from sequence to sequence, so there is some lack of maturity in the technology at this point.

This contribution presents an analysis of the compression efficiency of 360° video sequences transformed via the polyphase subsampling technique presented in JVET-B0043. This may be conceptually similar to the H.261 Annex D graphics sampling scheme.

This proposed technique decomposes the input video signal (luma and chroma components) by subsampling into lower resolution descriptions using a polyphase subsampling. The multiple lower resolution versions of the signal are then encoded in the same video bitstream via a temporal multiplexing. This enables the decoder to select and to decode the appropriate number of resolution components for the desired output resolution. The polyphase subsampling technique without low-pass filtering is known to be potentially create artefacts such as aliasing when high spatial frequency is present in the signal. The first reported experiment is the comparison of 360 4K video sequences encoded with HM against the polyphase subsampled sequences encoded with HM. The results reportedly show that the best Y-PSNR BD-rate impact is −67.7% for the sequence SkateboardingInLot while the worst is 86.9% for the sequence Trolley. The average Y-PSNR BD-rate impact over all the sequences is 14.3%. The second experiment reported is a simulcast comparison, that is the comparison of 360 4K video sequences encoded with HM plus the 360 video sequences downscaled in HD encoded with HM the against the polyphase subsampled sequences encoded with HM. The results reportedly show that the best Y-PSNR BD-rate impact is −76.50% for the sequence SkateboardingInLot while the worst is 31.30% for the sequence Trolley. The average Y-PSNR BD-rate impact over all the sequences is reportedly −19.10%.

It was remarked that some of the inconsistency in the results may be from bit rate matching effects.

A participant highlighted that the measured performance is reported from measurements made directly in the ERP domain, not considering the spherical domain.

It was commented that since the I frame refresh is not altered to compensate for the 4 coded pictures used per high-resolution frame, the I frame frequency is effectively four times higher than for the anchors, which could adversely effect the results. The QP variation would also affect different spatial positions differently.

The contributor remarked that the encoding time was faster with the scheme than without it. A participant commented that this may be due to the motion search early termination. Each MV value is effectively doubled.

It was commented that the aliasing in the sampling structure might have subjective quality issues.

JVET-G0028 InterDigital’s Response to the SDR Category in Joint Call for Evidence on Video Compression with Capability beyond HEVC [X. Xiu, Y. He, Y. Ye (InterDigital)]

Discussed Thursday 17:45 (GJS & JRO).

This is basically a simplification proposal relative to the JEM.

The goal of this response is to provide evidence on compression efficiency improvement over HEVC Main 10 Profile for the standard dynamic range (SDR) content. The response is based on the Joint Exploration Model (JEM) 6.0 reference platform with a couple of simplifications on the bi-directional optical flow (BIO) coding tool. Compared to the HM-based SDR anchors, it reportedly achieves on average 34.23% BD-rate reduction with 9.9 and 8.7 times increase in encoding and decoding time. Compared to the JEM-based SDR anchors, it reportedly shows 0.14% BD-rate increase with 8% and 25% reductions in encoding and decoding time, respectively.

- Comparison with the HM-based SDR anchors

The table below shows the BD-rate and complexity performance of the proposed solution compared to the JEM-based SDR anchors.

Comparison of the proposal with the HM-based SDR anchors in terms of both coding performance and encoding/decoding complexity

| Sequence |  | Y | U | V | Encoding | Decoding |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | Crosswalk1 | -37.80% | -43.84% | -47.46% | 1120% | 1098% |
|  | FoodMarket3 | -34.86% | -46.58% | -48.91% | 883% | 963% |
|  | Tango1 | -36.28% | -55.06% | -49.63% | 1302% | 1088% |
|  | CatRobot1 | -40.01% | -52.31% | -45.41% | 1045% | 1096% |
|  | DaylightRoad1 | -40.64% | -53.53% | -38.09% | 1123% | 991% |
|  | BuildingHall1 | -33.29% | -41.42% | -46.50% | 654% | 865% |
|  | ParkRunning2 | -31.54% | -26.14% | -29.23% | 1193% | 1068% |
|  | CampfireParty | -37.90% | -35.74% | -56.57% | 1888% | 765% |
| Class B | BQTerrace | -30.37% | -50.58% | -61.19% | 759% | 925% |
|  | RitualDance | -27.66% | -38.00% | -41.68% | 1692% | 907% |
|  | Timelapse | -26.70% | -61.40% | -67.03% | 720% | 939% |
|  | BasketballDrive | -32.04% | -46.87% | -43.39% | 1468% | 996% |
|  | Cactus | -35.87% | -49.10% | -45.11% | 1140% | 1010% |
| Average |  | -34.23% | -46.20% | -47.71% | 1091% | 971% |

- Comparison with the JEM-based SDR anchors

The table below shows the BD-rate and complexity performance of the proposed solution compared to the JEM-based SDR anchors. As shown in the table below, the proposed solution can effectively reduce the complexity of the current JEM by providing 8% and 25% reductions for the encoding and decoding time, respectively. The maximum reduction of the decoding time can reach 40% (sequence Timelapse). When it comes to the compression performance, the proposed solution has a minor impact on the overall BD-rate with an average 0.14% BD-rate increase.

Comparison of the proposal with the JEM-based SDR anchors in terms of both coding performance and encoding/decoding complexity

| Sequence |  | Y | U | V | Encoding | Decoding |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | Crosswalk1 | 0.17% | 0.20% | 0.22% | 93% | 75% |
|  | FoodMarket3 | 0.06% | 0.02% | 0.13% | 92% | 76% |
|  | Tango1 | 0.14% | -0.06% | -0.07% | 93% | 73% |
|  | CatRobot1 | 0.43% | 0.17% | 0.14% | 91% | 69% |
|  | DaylightRoad1 | 0.17% | -0.04% | -0.01% | 91% | 82% |
|  | BuildingHall1 | 0.08% | -0.01% | -0.05% | 92% | 66% |
|  | ParkRunning2 | 0.04% | 0.03% | 0.03% | 89% | 81% |
|  | CampfireParty | 0.04% | 0.00% | 0.01% | 93% | 83% |
| Class B | BQTerrace | 0.07% | -0.14% | 0.00% | 91% | 79% |
|  | RitualDance | 0.14% | -0.16% | -0.04% | 92% | 84% |
|  | Timelapse | 0.13% | -0.19% | -0.76% | 92% | 60% |
|  | BasketballDrive | 0.15% | 0.02% | -0.08% | 91% | 82% |
|  | Cactus | 0.15% | -0.06% | -0.09% | 90% | 64% |
| Average |  | 0.14% | -0.02% | -0.04% | 92% | 75% |

JVET-G0029 Samsung's response to Joint CfE on Video Compression with Capability beyond HEVC (SDR category) [E. Alshina, K. Choi (Samsung)]

Discussed Thursday 18:00 (GJS & JRO).

This is basically a simplification proposal relative to the JEM.

The primary goal of this contribution is to demonstrate the possibility of achieving higher performance gain using smaller number of coding tools compared to JEM 6.0. In average 36% (Y), 42% (Cb) and 44% (Cr) BD-rate gain over CfE anchor was achieved.

[Clean up below]

For this CfE response some JEM tools were disabled, another were replaced or modified. Changes are relatively small since the goal of this contribution just to illustrate video codec design principles Samsung applies preparing CfP response. Choosing tools to replace or modify we tried to resolve several issues which make JEM unfriendly and impractical. In particular:

- Initialization for context models (2.6.3 in [3])

  - Initial probability states of context models for inter-coded slices instead of slice-type and QP determined initialization can be (not always) initialized by copying states from previously coded pictures. The map of contexts which will be alternatively initialized is signalled in slice header. Encoder should decide about context initialization. Overhead associated with signalling the map of context initialized alternatively is not critical under JVET test condition (slice size is equal to picture) but it becomes quite heavy for typical sizes of slices. Those aspects make this particular part of JEM impractical.

- Bi-directional optical flow (2.3.8 in [3])

  - Motion refinement on a sample level is overkill for the majority of high resolution videos (2K, 4K and higher). Making motion vector refinement for 44 cluster of samples actually leads to performance improvement for high resolution classes (A1, A2, B), but for low resolution (classes C, D in JVET ctc performance drop is 0.3…0.4% (EE2 results) [4]. Clustering of samples in BIO leads to reduction for the number of calculations. We support this reasonable approach, and demonstrated an option of clustering w/o performance degradation [5].

- Four-tap intra interpolation filter, Position dependent intra prediction combination, Adaptive reference sample smoothing (2.2.2, 2.2.5 and 2.2.6 in [3])

  - Suppose one uses 3 taps smoothing filter for intra prediction reference and then applies bi-linear filter for fractional position calculation. Two those operations can be effectively combined and represented as 4-taps interpolation filter for Intra [6]. By doing that we would reduce overall latency in Intra prediction and make Intra design cleaner. If multiple 4-taps interpolation filters is the part of JEM design anyway then it is reasonable to merge reference samples smoothing with Intra interpolation filter. Taking into account only the subset of reference smoothing filters from ARSS and PDPC design one can maintain reasonable memory size for interpolation filter coefficients storage.

- “Small” tools

  - Some tools in JEM6.0 have negligible performance impact under CfE test conditions (tools under horizontal line in Fig.1). Functionalities associated with those tools were not included to this CfE response. By disabling weak tools we would not claim those tools are useless, we just express preference of having reasonable number (at most 20) of strong tools in a standard rather than including multiple weak tools with duplicated functionality.

CfE response performance vs CfE anchor (matched bit rates).

| Resolution | Seq. | BD-rate |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| 4K | CrossWalk1 | −38.8% | −34.5% | −38.8% |
|  | FoodMarket3 | −35.9% | −42.3% | −44.8% |
|  | Tango1 | −37.6% | −49.4% | −43.9% |
|  | CatRobot1 | −41.6% | −53.6% | −45.5% |
|  | DaylightRoad1 | −42.2% | −51.5% | −33.8% |
|  | BuildingHall1 | −35.1% | −39.4% | −45.5% |
|  | ParkRunning2 | −35.3% | −20.6% | −24.1% |
|  | CampfireParty | −43.4% | −33.2% | −58.4% |
| 2K | BQTerrace | −31.9% | −42.5% | −54.3% |
|  | RitualDance | −28.7% | −34.3% | −37.9% |
|  | TimeLapse | −29.2% | −61.8% | −65.7% |
|  | BasketballDrive | −33.7% | −43.6% | −39.1% |
|  | Cactus | −37.3% | −44.5% | −40.4% |
| 4K All |  | −38.8% | −40.6% | −41.8% |
| 2K All |  | −32.2% | −45.3% | −47.5% |
| All |  | −36.2% | −42.4% | −44.0% |

In the discussion, a participant

In the discussion, a participant commented that we should think about what we call a “tool”, and consider that small things can sometimes add up. Simplicity and straightforward design is a goal, and sometimes one “tool” can have many complicated little “elements”, etc.

JVET-G0066 Viewpaths for the CfE VR sequences [M. Wien (RWTH), J. Boyce (Intel), M. Zhou (Broadcom)] [miss] [late]

JVET-G0085 Information on CfE anchor generation for HDR content [A. K. Ramasubramonian, D. Rusanovskyy (Qualcomm), E. François (Technicolor), F. Hiron, J. Zhao, A. Segall (Sharp)]

JVET-G0093 AHG4: SDR anchor generation for Joint Call for Evidence by Qualcomm [H.-C. Chuang, J. Chen, M. Karczewicz]

JVET-G0117 AHG4: SDR anchor generation for Joint Call for Evidence by Samsung [K. Choi, E. Alshina (Samsung)] [miss] [late]

# Exploration experiments (45)

## General (1)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0010 Exploration Experiments on Coding Tools Report [E. Alshina, L. Zhang] [miss]

## EE1: Intra prediction (24)

Contributions in this category were discussed Saturday 1200-1300 and 1430-1545 (chaired by JRO).

### Primary (14)

JVET-G0077 EE1: UWP&UW66 with PDPC for other intra mode and ARSS off (Test2) [H. M. Jang, J. Lim, S.-H. Kim (LGE), K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris)]

JVET-G0078 EE1 Test 1: UWP+UW66, PDPC off, ARSS on [K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris)]

JVET-G0079 EE1 Test 10: UWP+UW66, PDPC on for other modes, ARSS constrained as in F0024 but explicit signaling as in F0055 [K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris), H. M. Jang, J. Lim, S.-H. Kim (LGE)]

JVET-G0080 Additional EE1 Tests (Test 2.1: UWP+UW66, PDPC for other modes, PDPC-L, ARSS off, and Test 10.1: UWP+UW66, PDPC on for other modes, PDPC-L, ARSS constrained as in F0024 but explicit signalling as in F0055) [K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris)]

JVET-G0104 EE1: Alternative setting for PDPC mode and explicit ARSS flag (tests 3-7) [M. Karczewicz, V. Seregin, A. Said, N. Hu, X. Zhao (Qualcomm)]

JVET-G0113 EE1 (Tests 8 and 9) Performance of RASS and PDPC in presence of other tools [E. Alshina (Samsung)] [miss] [late]

JVET-G0069 EE1: Crosscheck of tests 6, 8 and 9 [V. Drugeon (Panasonic)]

JVET-G0086 EE1: Cross-check of test4 and test7 [J. Lee, H. Lee, J. Kang (ETRI)] [miss] [late]

JVET-G0087 EE1: Cross-check of test10 [H. Ko, S.-C Lim, J. Kang (ETRI)] [miss] [late]

JVET-G0091 EE1: Crosscheck of Additional EE1 Tests (Test 2.1 and Test 10.1) (JVET-G0080) [T. Ikai, Y. Yasugi (Sharp)]

JVET-G0114 EE1 Cross-check for Test 1 [E. Alshina (Samsung)] [miss] [late]

JVET-G0115 EE1 Cross-check for Test 3 [E. Alshina (Samsung)] [miss] [late]

JVET-G0141 EE1: Cross-check of JVET-G0104, test 5 [F. Racape, E. François, F. Le Léannec (Technicolor)] [miss] [late]

JVET-G0142 EE1: Cross-check of JVET-G0077, test 2 [F. Racape, E. François, F. Le Léannec (Technicolor)] [miss] [late]

### Related (10)

JVET-G0062 EE1-Related: Harmonization UW Prediction method with improved PDPC [H. M. Jang, J. Lim, S.-H. Kim (LGE)]

JVET-G0126 EE1-Related: Crosscheck of JVET-G0062 on UW prediction fix [T. Ikai, Y. Yasugi (Sharp)] [miss] [late]

JVET-G0068 Non EE1: Unified-PDPC: unification of intra filters [M. Philippe, K. Sharman (Sony Europe)]

JVET-G0134 Non-EE1: Cross-check of JVET-G0068 on unification of intra filters (test 3.1.2) [V. Seregin (Qualcomm)] [miss] [late]

JVET-G0081 Comparisons between UWP, W66 and Planar, Angular mode 66 under the same coding conditions [K. Panusopone, S. Hong, Y. Yu, L. Wang (Arris)]

JVET-G0122 Crosscheck of JVET-G0081 "Comparisons between UWP, W66 and Planar, Angular mode 66 under the same coding conditions" [Alexey Filippov, Vasily Rufitskiy (Huawei)] [miss] [late]

JVET-G0107 Non-EE1: PDPC without a mode flag [V. Seregin, M. Karczewicz, A. Said, X. Zhao (Qualcomm)]

JVET-G0108 Non-EE1: Fix for strong intra smoothing filtering [V. Seregin, X. Zhao, M. Karczewicz (Qualcomm)]

JVET-G0136 Non-EE1: Crosscheck of G0108 on strong intra smoothing filtering [T. Ikai (Sharp)] [miss] [late]

JVET-G0146 EE1: Additional tests comparing UWP/UW66 with P-PDPC in EE1 tests [M. Karczewicz, V. Seregin, A. Said, N. Hu, X. Zhao (Qualcomm)] [late]

## EE2: Decoder-side motion vector derivation (6)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

### Primary (3)

JVET-G0082 EE2: A block-based design for Bi-directional optical flow (BIO) [H.-C. Chuang, J. Chen, X. Li, Y.-W. Chen, M. Karczewicz, W.-J. Chien (Qualcomm)]

JVET-G0094 EE2: Cross-check of EE2 test1 (JVET-G0082) [H. Lee, J. Kang (ETRI)] [miss] [late]

JVET-G0116 EE2: Cross-check for block-based BIO design [E. Alshina (Samsung)] [miss] [late]

### Related (3)

JVET-G0083 EE2-related: A simplified gradient filter for Bi-directional optical flow (BIO) [H.-C. Chuang, J. Chen, K. Zhang, M. Karczewicz (Qualcomm)]

JVET-G0105 EE2-related: Crosscheck of A simplified gradient filter for Bi-directional optical flow (JVET-G0083) [M. Ikeda (Sony)] [miss] [late]

JVET-G0137 EE2-related: Crosscheck of JVET-G0083 on gradient filter modification [T. Ikai (Sharp)] [miss] [late]

## EE3: Adaptive QP for 360° video (7)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

### Primary (6)

JVET-G0070 EE3-JVET-F0049/F0038 Adaptive QP for ERP videos [Hendry, M. Coban (Qualcomm), F. Racape, F. Galpin (Technicolor)]

JVET-G0121 EE3 Test1 and 1.1: Cross-check of JVET-G0070 [K. Kawamura, S. Naito (KDDI)] [late]

JVET-G0106 EE3: Adaptive QP for 360° video [Y. Sun, L. Yu (Zhejiang University)]

JVET-G0124 EE3: Cross-check for JVET-G0106 (Test 5 -- Adaptive QP for CMP in HM-360Lib) [Hendry (Qualcomm)] [miss] [late]

JVET-G0125 EE3: Cross-check for JVET-G0106 Test2 [X. Xiu, Y. He, Y. Ye (InterDigital)] [miss] [late]

JVET-G0140 EE3: Cross-check of JVET-G0106 (Test 2.1 -- Adaptive QP with F0072 weighting for rotated ERP in HM-360Lib) [F. Racape, F. Galpin (Technicolor)] [miss] [late]

### Related (1)

JVET-G0089 EE3 Related: Adaptive quantization for JEM-based 360-degree video coding [X. Xiu, Y. He, Y. Ye (InterDigital)]

## EE4: 360° Projection Modifications and Padding (3)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

### Primary (7)

JVET-G0097 EE4: ERP/EAP-based segmented sphere projection with different padding sizes [Y.-H. Lee, H.-C. Lin, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

JVET-G0098	EE4: Padded ERP (PERP) projection format [J. Boyce, Z. Deng (Intel)]

JVET-G0102 EE4: Cross-check of EE4 tests 5-7 (JVET-G0097) [Z. Deng (Intel)] [miss] [late]

JVET-G0129 EE4: Cross-check of EE4 tests 1-4 (JVET-G0097) [Yao Lu, Jisheng Li, Ziyu Wen, Xianyu Meng (Owl Reality)] [miss] [late]

JVET-G0130 EE4 Cross-check for Test 8-10 (JVET-G0128) [Y.-H. Lee, J.-L. Lin (MediaTek)] [miss] [late]

JVET-G0131 EE4 Cross-check for Test 11-14 (JVET-G0098) [Y.-H. Lee, J.-L. Lin (MediaTek)] [miss] [late]

### Related (1)

JVET-G0128 EE4: Padding method for Segmented Sphere Projection [Yao Lu, Jisheng Li, Ziyu Wen, Xianyu Meng (Owl Reality)] [late]

# Non-EE Technology proposals (12)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

## Intra coding (2)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0060 Improvements for Intra Prediction Mode Coding [Y. Han, J. An, J. Zheng (HiSilicon)]

JVET-G0092 Crosscheck of Improvements for Intra Prediction Mode Coding (JVET-G0060) [Y. Yasugi, T. Ikai (Sharp)]

## Inter coding (2)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0065 Simplification and improvements on FRUC [J. Seo, J. Lee, S.-H. Kim, H.M. Jang, J. Lim (LGE)]

JVET-G0119 Cross-check of simplification and improvements on FRUC (JVET-G0065) [K. Choi, E. Alshina (Samsung)] [miss] [late]

## Loop filters (3)

JVET-G0076 Bilateral filter simplification [R. Vanam, Y. He, Y. Ye (InterDigital)]

JVET-G0095 Unified Adaptive Loop Filter for Luma and Chroma [J. An, J. Zheng (HiSilicon)]

JVET-G0118 Cross-check of Unified Adaptive Loop Filter for Luma and Chroma (JVET-G0095) [K. Choi, E. Alshina (Samsung)] [miss] [late]

## Other (5)

JVET-G0067 Chroma Adjustment for SDR Video [J. Ström, P. Wennersten, K. Andersson, R. Sjöberg (Ericsson)]

JVET-G0073 Adaptive quantization and denoising for future video coding CfP and CTC [R. Sjöberg, K. Andersson, P. Wennersten (Ericsson)]

JVET-G0112 Arithmetic coding with context-dependent double-window adaptation response [A. Said, M. Karczewicz, L. Zhang, V. Seregin (Qualcomm)]

JVET-G0143 Cross-check of JVET-G0112 - Arithmetic coding with context-dependent double-window adaptation response [K. Sharman, M. Philippe (Sony)] [late]

JVET-G0148 AHG9: encoding/decoding capability of JEM6 for 4:4:4 colour format [J. Kim, X. Xu, S. Lei (Mediatek)]

# Extended colour volume coding (7)

Contributions in this category were discussed XXday XXth XXXX-XXXX (chaired by XXX).

## Test conditions and evaluation (2)

JVET-G0103 AHG7: Candidate rate points of HLG material for anchor generation [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

JVET-G0120 AHG7 Cross-check of anchor generation of HLG content in JVET-G0103 [K. Kawamura, S. Naito (KDDI)] [late]

## Tools (5)

JVET-G0054 Mapping SDR content into HDR containers [C. Fogg (MovieLabs)]

JVET-G0059 AHG7: On the need of luma delta QP for BT.2100 HLG content [S. Iwamura, S. Nemoto, A. Ichigaya (NHK), M. Naccari (BBC)]

JVET-G0084 Luma/Chroma QP Adaptation for Hybrid Log-Gamma Sequences Encoding [K. Kawamura, S. Naito (KDDI)]

JVET-G0138 Cross-check of JVET-G0084 on luma/chroma QP adaptation for HLG material [S. Iwamura, S. Nemoto, A. Ichigaya(NHK)] [miss] [late]

JVET-G0123 AHG7: Experiments on using local QP adaptation in the context of an HLG container [E. Francois, F. Hiron (Technicolor)] [late]

# Coding of 360O video projection formats (16)

## Conversion tools, 360lib (0)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

## Packing and Projection formats (6)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0051 AHG8: A study on quality impact of line re-sampling rate in EAP [M. Zhou (Broadcom)]

JVET-G0111 AHG8: Crosscheck of JVET-G0051 “A study on quality impact of line re-sampling rate in EAP” [Y. He, X. Xiu, P. Hanhart, F. Duamnu, Y. Ye (InterDigital)]

JVET-G0056 AHG8: A study on Equi-Angular Cubemap projection (EAC) [M. Zhou (Broadcom)]

JVET-G0110 AHG8: Crosscheck of JVET-G0056 “A study on Equi-Angular Cubemap projection” [F. Duanmu, X. Xiu, P. Hanhart, Y. Ye, Y. He (InterDigital)]

JVET-G0064 Stereoscopic 360 video compression with the next generation video codec [F. Henry, J. Jung, A. Ouach, B. Ray, P. Schwellenbach (Orange)]

JVET-G0100 AHG8: A study of 360Lib projections on global motion sequences [M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

## Quality assessment and metrics (4)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0052 AHG8: A study on quality impact of coded picture resolution in 360 video coding [M. Zhou (Broadcom)]

JVET-G0057 AHG8: Influence of coding size on objective gain in 360-degree video CTC [G. Van der Auwera, M. Coban, M. Karczewicz (Qualcomm)]

JVET-G0075 AHG8: On reliability of S-PSNR-NN and S-PSNR-I as quality metrics for 360-degree video [Y. Ye, Y. He (InterDigital)]

JVET-G0088 AHG8: On the derivation of weighted to spherically uniform PSNR (WS-PSNR) for adjusted cubemap projection (ACP) format [X. Xiu, Y. He, Y. Ye (InterDigital)]

## Coding tools (0)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

## Padding (6)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0058 AHG8: Reference picture extension of ACP format 360-degree video [M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-G0071 AHG8: ACP with padding for 360-degree video [G. Van der Auwera, M. Coban, M. Karczewicz (Qualcomm)]

JVET-G0132 AHG8: Crosscheck of JVET-G0071 “ACP with padding for 360-degree video” [Y. He (InterDigital)] [miss] [late]

JVET-G0074 AHG8: ECP with padding for 360-degree video [G. Van der Auwera, M. Coban, M. Karczewicz (Qualcomm)]

JVET-G0133 AHG8: Crosscheck of JVET-G0074 “ECP with padding for 360-degree video” [Y. He (InterDigital)] [miss] [late]

JVET-G0099 Padded ERP (PERP) projection format for OMAF subjective test [J. Boyce, Z. Deng (Intel)]

## HL syntax (0)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

# Complexity analysis (1)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0061 AHG5 External Memory Access Evaluation with the Consideration of Cache [X. Li, T. Hsieh, J. Chen, M.Karczewicz (Qualcomm)]

# Encoder optimization (2)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

JVET-G0109 A modification of fast algorithm in intra mode selection [P.-H. Lin, C.-L. Lin, C.-C Lin (ITRI)]

JVET-G0127 Crosscheck of a modification of fast algorithm in intra mode selection (JVET-G0109) [Y. Yasugi, T. Ikai (Sharp)] [miss] [late]

# Metrics and evaluation criteria (0)

Contributions in this category were discussed XXX XX00-XX00 (chaired by …).

# Withdrawn (2)

See under 1.4.2.

JVET-G0135 Withdrawn

JVET-G0139 Withdrawn

# Joint Meetings, BoG Reports, and Summary of Actions Taken

## Exploration Experiments (update)

The setup of Exploration Experiments was discussed, and an initial draft of the EE document was reviewed in the plenary (chaired by JRO). This included the list of all tools that are intended to be investigated in EEs during the subsequent meeting cycle:

EE1: Intra prediction (continue)

JVET-F0033

JVET-F0054

JVET-F0055

JVET-F0104

The 10 aspects listed in the EE document are the minimum set expected; if additional information is brought about better combinations of the tools investigated in the EE, such combinations should be considered as well, but add-on elements not known so far shall be considered like new contributions.

EE2: Decoder Side Motion Vector Derivation (continue)

JVET-F0022

EE3: Adaptive QP for 360 video

JVET-F0038

JVET-F0049

JVET-F0072

(add rotation)

EE4: 360 Projection Modifications and Padding (HM only)

JVET-F0037

JVET-F0052

JVET-F0108

(only HM)

Li Zhang is mandated to compile the EE document with remote assistance by Elena Alshina, to be circulated by Thursday and reviewed Friday.

It was agreed to give the editors the discretion to finalize the document during the two weeks after the meeting, and circulate/discuss it on the reflector appropriately.

## Joint meetings

- Joint meeting Wednesday 1600–1730 on CfE preparation

In preliminary perceptual testing, in some cases, JEM with up to 35% lower data rate performed visually better than HEVC HM.

An update draft of the CfE was reviewed. One aspect was to request bitstreams to be submitted by 6 July. Additionally, JEM 6.0 anchors will be provided by June 10:

- Samsung and Qualcomm will provide anchors for SDR and perform mutual cross-check.

- Samsung will provide the anchors for 360, InterDigital cross-checks them. Furthermore, HM anchors for 360 need modification of adaptive search range (InterDigital).

- Sharp, Qualcomm and Technicolor will provides HM and JEM anchors for HDR, and will mutually perform crosschecks.

Small changes were made in the test conditions. An SDR bit rate point was modified. A different part of one sequence was used. Some rates were adjusted for HDR. For 360° video, one sequence was added and some rate points were changed.

JVET-F0102 Industry recommendation for Future Video Coding Standard Development [E. Alshina (Samsung), J. Zhou (Huawei), S. Lei (MediaTek), C. Luo (China Telecom), A. Hinds (CableLabs)] [late]

This document describes recommendations for Future Video Coding standard development and concrete suggestions form industry representatives regarding the standardization practice. This contribution is intended to serve as a starting point for discussing improvements for an improved future video coding standard process that meets market requirements in an efficient and timely matter and enables widely distributed and affordable video data types for consumers.

Reviewed in Joint Meeting.

In JVET-F0102, a late contribution commenting on process, planning, and requirements, was presented. Regarding HEVC development, it praised the test model methodology and advocated for “one tool, one functionality”, the principle that clear design and sufficient gain need to be shown to add something to the design, and the incorporation of strong complexity analysis. It advocated holding to a firm two-year timeline upon adoption of the first test model and “freezing” the low-level design 6 months prior to completion. It advocated having a large and diverse CTC test set, perhaps changing the selected sequences frequently, and ensuring that testing bit rates are aligned to application requirements. It requested emphasis on simpler coding features with modest complexity needs, and evaluation of complexity on GPU and MPU platforms as well as reference software.

Some experts expressed concern that they did not have sufficient chance for studying this document before it was presented, It was also mentioned that some of the aspects may be premature to discuss, while other aspects are probably valid and reflect the best practice of past standardization efforts.

## BoGs

## List of actions taken affecting JEM6

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the JEM6 or 360Lib3.0 description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

Was presented and confirmed to be complete Fri morning in the JVET plenary.

### Encoder only or software changes

JVET-F0064 MS-SSIM as an additional mandatory metric to PSNR for future video coding [M. Pettersson, R. Sjöberg, P. Wennersten, K. Andersson, J. Ström, J. Enhorn (Ericsson)]

Adopt into JEM software, but not CTC (optional, off by default). No reference in Excel template.

under BoG report JVET-F0107

Include the HDR anchor software (encoder optimization of quantization) in JEM 6.0. By default off in SDR and 360 CTC, by default on in HDR CTC

### Syntax/semantics/decoding process changes

JVET-F0028 EE3: BIO w/o block extension [A. Alshin, E. Alshina (Samsung)]

JVET-F0031 Reduction redundant syntax signaling for transform skip [H. Jang, J. Lim, J. Nam, S.-H. Kim (LGE)]

JVET-F0032 EE3: Enhanced FRUC Template Matching Mode [Y. Lin, X. Chen, J. An, J. Zheng (HiSilicon)]

Adopt F0032 Aspect 2. However, the multiplication by factor 1.1 should be implemented in fixed point precision.

JVET-F0096 EE2-JVET related: Division-free bilateral filter [J. Ström, K. Andersson, P. Wennersten, M. Pettersson, J. Enhorn, R. Sjöberg (Ericsson)]

### Changes in 360lib

JVET-F0025 AHG8: Adjusted cubemap projection for 360-degree video [M. Coban, G. Van der Auwera, M. Karczewicz (Qualcomm)]

Include in 360Lib.

JVET-F0036 AHG8: Rotated Sphere Projection for 360 Video [A. Abbas, D. Newman (GoPro)]

Include in 360Lib.

JVET-F0041 AHG8: Platform independent floating point to integer conversion for 360Lib [Y. He, P. Hanhart, Y. Ye (InterDigital)]

JVET-F0042 AHG8: On cross-format S-PSNR-NN [Y. He, X. Xiu, Y. Ye (InterDigital)]

Replace the previous cross format S-PSNR-NN by the new method.

JVET-F0053 AHG8: An improvement on compact octahedron projection with padding [Y.-H. Lee, H.-C. Lin, J.-L. Lin, S.-K. Chang, C.-C. Ju (MediaTek)]

Change the COHP1 design in the 360Lib software and update the CTC accordingly.

JVET-F0065 360Lib modifications for spherical rotation [Q. Xu, J. Boyce (Intel), Y. He, Y. Ye (InterDigital)]

# Project planning

## JEM description drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- JEM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XXday XX April 2017.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for Experiments

Note: This section was drafted during the second JVET meeting, and is kept here for information about the EE procedure.

Group coordinated experiments have been planned. These may generally fall into one category:

- "Exploration experiments" (EEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the main branch of JEM by the next meeting.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. (E. Alshina will edit the document based on input from the proponents, review is performed in the plenary)

- Software for tools investigated in EE is provided in a separate branch of the software repository

- During the experiment, further improvements can be made

- By the next meeting it is expected that at least one independent party will report a detailed analysis about the tool, confirms that the implementation is correct, and gives reasons to include the tool in JEM

- As part of the experiment description, it should be captured whether performance relative to JEM as well as HM (with all other tools of JEM disabled) should be reported by the next meeting.

It is possible to define sub-experiments within particular EEs, for example designated as EEX.a, EEX.b, etc., where X is the basic EE number.

As a general rule, it was agreed that each EE should be run under the same testing conditions using one software codebase, which should be based on the JEM software codebase. An experiment is not to be established as a EE unless there is access given to the participants in (any part of) the TE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-B1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to EE work should identify individuals in addition to company names.

EE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the EE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish an EE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to EE activities were established as follows:

- Only qualified JVET members can participate in an EE.

- Participation in an EE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the EE should be announced and made available to all EE participants in a timely manner.

A separate branch under the experimental section will be created for each new tool include in the EE. The proponent of that tool is the gatekeeper for that separate software branch. (This differs from the main branch of the JEM, which is maintained by the software coordinators.)

New branches may be created which combine two or more tools included in the EE document or the JEM. Requests for new branches should be made to the software coordinators.

Don’t need to formally name cross-checkers in the EE document. To promote the tool to the JEM at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

Timeline:

T1 = JEM5.0 SW release + 4 weeks: Integration of all tools into separate EE branch of JEM is completed and announced to JVET reflector.

Initial study by cross-checkers can begin.

Proponents may continue to modify the software in this branch until T2

3rd parties encouraged to study and make contributions to the next meeting with proposed changes

T2: JVET-F meeting start – 3 weeks: Any changes to the exploration branch software must be frozen, so the cross-checkers can know exactly what they are cross-checking. An SVN tag should be created at this time and announced on the JVET reflector.

This procedure was again confirmed during the closing plenary of the third JVET meeting. It was further confirmed that the Common Test Conditions of JVET-B1010 are still valid, however the CTC encoder setting will be reflected in the config file that is attached to the JEM4.0 package.

## Software development and anchor generation

Software coordinators will work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- JEM6.0 including all adoptions from section 14.4 will be released by 2017-04-21.

- The results about coding performance of JEM6.0 will be reported by 2017-05-02.

- Further versions may be released for additional bug fixing, as appropriate

Timeline of 360Lib3.0: 2 weeks after the meeting (2017-04-21).

Further versions may be released as appropriate for bug fixing.

Timelines and volunteers for CfE anchors:

- For SDR: See under …

- For 360: See under …

- For HDR: See under …

New HM anchors will be generated using HM 16.15. JEM anchors will be based on JEM 6.0.

# Output documents and AHGs

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JVET-F1000 Meeting Report of the 6th JVET Meeting [G. J. Sullivan, J.-R. Ohm] [2017-07-14] (near next meeting)

Intermediate versions of the meeting notes (d0 … d6) were made available on a daily basis during the meeting.

JVET-F1001 Algorithm description of Joint Exploration Test Model 6 (JEM6) [J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce] [2017-04-28] (MPEG N16887)

See list of new adoptions under 14.4. During the closing plenary, no complaints were made about the accuracy of that list.

JVET-F1002 Joint Call for Evidence on Video Compression with Capability beyond HEVC [M. Wien, V. Baroncini, J. Boyce, A. Segall, T. Suzuki] [2017-04-14] (MPEG N16886)

Draft was discussed Wed 1400 and in joint meeting with parent bodies Wed 1600. Was again presented during closing plenary on Friday.

The companies responsible for providing the HM and JEM anchors will also provide the corresponding Excel templates for the cases of SDR, HDR and 360.

(see responsibilities under XXX)

JVET-F1003 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib [Y. Ye, E. Alshina, J. Boyce] [2017-04-28] (MPEG N16888)

See list of new adoptions under 14.4.3. During the closing plenary, no complaints were made about the accuracy of that list.

JVET-F1004 Subjective testing method for 3600 video projection formats using HEVC [J. Boyce, E. Alshina, Z. Deng] [2017-04-21] (MPEG N16892)

JVET-B1010 JVET common test conditions and software reference configurations [K. Suehring, X. Li]

remains valid (from 2nd meeting).

Note: Encoder settings reflected in the config file related to CTC in JEM5 (see changes under 14.4)

A directory had been installed in the ftp for bitstreams and results of anchors, but in the previous meeting cycle this had not yet been used. It is planned to upload bitstreams and Excel sheets after completion of anchors.

JVET-F1011 Description of Exploration Experiments on coding tools [E. Alshina, L. Zhang] [2017-04-28] (MPEG N16889)

Initial version was presented in the closing plenary on Friday 20th Jan, and minor changes were made: Bilateral filter test3 (CTU based on/off) and investigation on normalization was removed from the initial draft.

See list of EEs under 14.1.

JVET-F1020 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. Francois, D. Rusanovskyy] [2017-04-21] (MPEG N16890)

JVET-F1030 JVET common test conditions and evaluation procedures for 3600 video [E. Alshina, J. Boyce, A. Abbas, Y. Ye] [2017-04-21] (MPEG N16891)

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header should be generated. This version should be sent to GJS and JRO for upload.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Tool evaluation (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate the exploration experiments. - Investigate interaction of tools in JEM and exploration experiment branches. - Discuss and evaluate methodologies and criteria to assess the benefit of tools, and how to ease the assessment of single tools in terms of encoder runtime. - Study and summarize new technology proposals. | E. Alshina, M. Karczewicz (co-chairs) | N |
| JEM algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-F1001 Algorithm Description of Joint Exploration Test Model 6. - Gather and address comments for refinement of the document. - Coordinate with the JEM software development AHG to address issues relating to mismatches between software and text. | J. Chen (chair), E. Alshina, J. Boyce (vice chairs) | N |
| JEM software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of the JEM6.0 software packages and their distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver JEM6.0 software version and the reference configuration encodings according to JVET-B1010 common conditions. - Coordinate with AHG on JEM model editing and errata reporting to identify any mismatches between software and text, and make further updates to the software as appropriate. - Investigate the implementation of SCC coding tools in JEM. - Coordinate with AHG6 for integration of 360 video software. | X. Li, K. Suehring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of future video coding standards. - Identify and recommend appropriate test materials and corresponding test conditions for use in the development of future video coding standards. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Discuss and prepare HM anchors at 4 additional rate points for the Call for Evidence. - Evaluate new test sequences, e.g. sequences proposed in JVET-F0062, and prepare for the visual assessment in the next meeting. - Make logistic arrangements for the forthcoming Call for Evidence. | V. Baroncini, T. Suzuki (co-chairs), J. Chen, J. Boyce, A. Norkin (vice chairs) | N |
| Memory bandwidth consumption of coding tools (AHG5) (jvet@lists.rwth-aachen.de) - Study the methodology of measuring decoder memory bandwidth consumption, including cache models. - Develop software tools for measuring both average and worst case of memory bandwidth. - Make analysis for examples of JEM coding tools. - Study the impact of memory bandwidth on specific application cases. | X. Li, E. Alshina, T. Ikai, H. Yang | N |
| 3600 video conversion software development (AHG6) (jvet@lists.rwth-aachen.de)  Prepare and deliver 360Lib-3.0 software version and common test condition configuration files according to JVET-F1030.  Generate CTC anchors and a reporting template for the common test conditions.  Produce documentation of software usage for distribution with the software.  Coordinate with JEM software coordinators for integration of 360Lib-3.0 within most recent versions of JEM software packages. - Review integration framework and refine the interface to minimize the needed software changes in JEM. | Y. He, V. Zakharchenko (co-chairs) | N |
| JEM coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Study and refine test conditions and anchors for the JEM coding of HDR/WCG content. - Coordinate generation of HM and JEM anchors for the CfE. - Study and evaluate available HDR/WCG test content. - Study and evaluate the application of QP adaptation in the context of an HLG container. - Study and evaluate visual quality assessment methods including rendering content with a peak brightness larger than available displays. - Study objective metrics for quality assessment of HDR/WCG material. - Study methods for increased coding efficiency of HDR/WCG material. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. Francois, D. Rusanovskyy (vice chairs) | N |
| 3600 video coding tools and test conditions (AHG8) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Study consistency of and potential improvements to the objective quality metrics in CTC. - Coordinate effort to prepare for CfE testing, including anchor generation, in collaboration with AHG5. - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Produce and finalize JVET-F1003 algorithm descriptions of projection format conversion process and objective quality metrics in 360Lib. - Produce and finalize JVET-F1004 Subjective testing method for comparison of 360 video projection formats using HEVC. - Produce and finalize JVET-F1030 JVET common test conditions and evaluation procedures for 360 video. - Study coding tools dedicated to 360 video, and their impact on compression. | J. Boyce (chair), A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye (vice chairs) | N |
| 4:4:4 support in JEM (AHG9) (jvet@lists.rwth-aachen.de) - Evaluate JEM6.0 software in terms of 4:4:4 chroma sampling support: Identify the tools in JEM6.0 that are not able to support 4:4:4 chroma sampling appropriately. Also identify whether this is due to implementation limitations or tool characteristics. | A. Tourapis (chair), X. Li (vice chair) | N |

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Wednesday or Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–7.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 7.5 meeting days).

In cases where high workload is expected for a meeting, an earlier starting date may be defined.

Some specific future meeting plans (to be confirmed) were established as follows:

- Wed. 18 – Wed. 25 Oct. 2017, 8th meeting under ITU-T auspices in Macao, CN.

- Fri. 19 Jan. – Fri. 26 Jan. 2018, 9th meeting under WG 11 auspices in Gwangju, KR.

- Fri. 13 Apr. – Fri. 20 Apr. 2018, 10th meeting under WG 11 auspices in San Diego, US.

- XX – XX July 2018, 11th meeting under ITU-T auspices in Ljubljana, SVN.

The agreed document deadline for the 8th JVET meeting is XXday XX Oct. 2017. Plans for scheduling of agenda items within that meeting remain TBA.

XXX was thanked for the excellent hosting of the 7th meeting of the JVET. NHK and GBTech were thanked for providing viewing equipment. Vittorio Baroncini was thanked for conducting visual tests, Teruhiko Suzuki, Edouard François, and Yuwen He were thanked for their great help in organizing the tests. The participants in the expert viewing were also thanked.

The 7th JVET meeting was closed at approximately XXXX hours on Friday 21 July 2017.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the seventh meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately XXX people in total), were as follows:

- …