| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 7th Meeting: Torino, IT, 13–21 July 2017 | Document: JVET-G0067 |
| --- | --- |

| Title: | Chroma Adjustment for SDR Video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström, Per Wennersten, Kenneth Andersson and Rickard Sjöberg Torshamnsgatan 23 164 80 Stockholm Sweden | Tel: Email: | +46702672192 jacob.strom (at) ericsson.com, per.wennersten (at) ericsson.com, kenneth.r.andersson (at) ericsson.com, rickard.sjoberg (at) ericsson.com |
| Source: | Ericsson Research |  |  |

_____________________________

# Abstract

This contribution discusses an alternative way of subsampling 4:4:4 sequences to 4:2:0 called chroma adjustment. This method was originally created for HDR content as reported in JCTVC-Z0022, but this document asserts that it may be beneficial also for certain SDR sequences with saturated colors such as CampfireParty. Compared to the version of CampfireParty currently used in JVET, the contribution claims that the chroma adjustment version can avoid amplification of noise in some areas, avoid suppression of noise in other areas and avoid a blurring of some edges. The contribution measures the increase in luminance quality from the chroma adjustment method by measuring PSNR on a gamma corrected version of luminance. It is reported that this results in a BD rate change of   -21% for CampfireParty for random access over the current 4:2:0 sequence. The contribution however claims that the perceptual effect seems smaller than this figure suggests, perhaps due to the sequence being noisy. The contribution asserts that non-saturated sequences do not benefit as much from the processing, and results for a non-saturated sequence (TrafficFlow) are also reported, -1%. Furthermore, the contribution notes that the chroma positions in the current 4:2:0 sequence are incorrect. The contribution proposes to that the CampfireParty sequence should be replaced by the one obtained using the proposed technique, and that 4:4:4 representations of all current test sequences should be made available.

# Problem Statement

While working with the JVET test set, we have noticed a strange characteristic of the noise in the CampfireParty sequence. In some areas, the noise in the sequence seems to be following the contours of colors. As an example, the brighter parts of the flag are much noisier than the surrounding parts, as seen in the right most picture of Figure 1.

Figure 1: Left: Thumbnail showing zoomed-in area. Middle: Original 4:4:4 sequence Right: Current JVET 4:2:0 sequence. Notice how the noise in the yellowish area has been amplified while the noise in the red area has been attenuated in the 4:2:0 version compared to the original.

Since the 4:4:4 version of this sequence is available at http://medialab.sjtu.edu.cn/web4k/index.html, this has been used as a comparison (Figure 1, middle). It is clear that in the 4:2:0 sequence, the noise in the yellowish region has been amplified, while the noise in the red part has been suppressed, compared to the 4:4:4 sequence. To see whether it is the downsampling that causes this issue, we have downsampled the 4:4:4 sequence ourselves using a simple {1, 2, 1}/4 downsampling filter. This has resulted in the same type of artifacts, showing that the subsampling process is indeed to blame.

From the work in HDR done in MPEG, it has been noticed that chroma subsampling can amplify noise for certain colors, see for instance Figure 2 in MPEG-m36184 [1]. As an example, a small noise in the green channel close to the black level will get amplified due to the non-linearity of the transfer function, resulting in large changes in the Y’CbCr representation. When Cb and Cr get subsampled but not the luma (Y’), the luminance Y will change although the luma Y’ stays the same (chroma leakage). Please see [2] for a more detailed explanation.

It thus appears as if the same noise amplifying effect is visible also for SDR, although it is not as strong, given that the gamma transfer function specified in ITU-R BT.709 is less non-linear than (is not as steep as) the PQ or HLG transfer functions used in HDR.

Apart from noise amplification, the subsampling seems to also have caused a slight blurring in CampfireParty. This can be seen in Figure 2, where the pupils, the bottom of the glasses and the corners of the mouth have been blurred due to the subsampling.

Figure 2: Left: Original 4:4:4 sequence. Right: Current JVET 4:2:0 test sequence (uncompressed). Note that the subsampled version is blurrier, especially in the pupils, the lower parts of the glasses and the corners of the mouth. Also note the amplified noise in the forehead and cheek regions.

This blurring is due to the fact that for some saturated colors, part of the luminance is carried in the chroma channel, which is subsampled.

To allow subsampling of HDR video without these artifacts, luma adjustment [1-2] was introduced at the Warsaw meeting, and the MPEG and JCT-VC communities have since experimented with various implementations and variants thereof [1-13]. In short, the idea is to change the luma, Y’, to reduce the error in luminance, Y, which is much more relevant to the human visual system.

While luma adjustment only changes the luma channel, Y', the related approach called chroma adjustment may also change the chroma components, albeit indirectly [14]. It works as a preprocessing step in linear RGB 4:4:4, where every pixel color is replaced by a different one that is similar to the original one in terms of chromaticity (in CIE u' v' coordinates) while keeping the luminance intact.  By making neighboring RGB pixels more similar to each other, large movements in Y'CbCr due to RGB noise are suppressed.  This greatly reduces the luminance artifacts. The final part of the chroma adjustment method is a step of luma adjustment, eliminating the remaining artifacts. For a more detailed description, see JCTVC-Z0022 [14].

In this contribution we have adapted the chroma adjustment method from JCTVC-Z0022 to work with SDR data, and we investigate whether it can help with the subsampling of CampfireParty. We have also applied it to TrafficFlow, a mostly gray sequence that should not suffer from any subsampling artifacts.

In the process of this work it was also (re-) discovered that the current Campfire sequence used in JVET has its chroma sample positions incorrectly placed. While they should be situated in the same place as luma samples for BT.709 (cosited chroma), they are instead situated between luma samples (JPEG-style chroma). This results in a slight shift in the image compared to the 4:4:4 sequence if cosited chroma is assumed. Therefore we have used JPEG style upsampling filters when processing the JVET test sequence in order to get rid of the shift.

# Adaptation of chroma adjustment to SDR

The chroma adjustment method from JCTVC-Z0022 can be applied to SDR by exchanging the PQ EOTF with a gamma function. In our work we have used a simple power function

L = gEOTFx=x1/0.45,                         (1)

and the inverse of the EOTF is thus

x=gEOTF-1L=L0.45.                         (2)

To get the original linear RGB 4:4:4 data, the Y'CbCr4:4:4 data is first converted to R'G'B' using the inverse color transformation matrix,

R'G'B'=101.574801-0.18733-0.4681311.855630Y'Cb'Cr',                         (3)

and the resulting data is then converted to linear RGB using

R=gEOTFR'                         (4)

and ditto for green and blue. This is the input to the chroma adjustment method. For the objective measurement we also calculate the luminance Y of the pixel using

Y=wRR+wGG+wBB,                         (5)

where wR= 0.212639, wG=0.715169 and wB=0.072192. Finally, the gamma corrected luminance g-1(Y) is calculated as gEOTF-1Y.

For the chroma adjustment step, the parameters ϕ=2/410 and θ=2/876 are used.

The output of the chroma adjustment is a new Y'CbCr4:2:0 file that is compressed using JEM 6.0.

# Subjective Results

## Effect on amplified noise

Since the traditional subsampling amplifies some noise and suppresses other noise, we have investigated how this compares to subsampling using chroma adjustment. As can be seen in Figure 3, in the chroma adjustment version the noise level is more similar to the original both in the yellow area, where no noise amplification has occurred, as well as in the red area, where no low pass filtering of the noise has taken place.

Figure 3: Left: Original 4:4:4 sequence. Middle: Current JVET 4:2:0 sequence. Right: Chroma adjustment 4:2:0 sequence. All sequences are uncompressed. Note how the chroma adjustment version avoids the noise amplification in the yellow area and the noise suppression in the red area.

The example in Figure 3 was on uncompressed data. In Figure 4 we have an example from data compressed with QP 22.

Figure 4: Left: Original 4:4:4. Middle: Current 4:2:0 sequence. Right: Chroma adjustment. Note how in the 4:2:0 sequence, the noise on the jacket is supressed while the noise in the faces is amplified. No such strong suppression or amplification is found in the chroma adjusted version.

Note that in the jacket area, noise is suppressed in the current sequence, while in the face area, noise is amplified. For the chroma adjusted version, no such strong amplification or suppression is seen.

## Blurring artifacts.

Figure 5 shows the face example from Figure 2 again, but with the chroma adjusted version added.

Figure 5: Left: Original 4:4:4 sequence. Middle: Current JVET 4:2:0 test sequence (uncompressed). Right: Chroma adjusted 4:2:0 sequence (uncompressed). Note that the middle version is blurrier, especially in the pupils, the lower parts of the glasses and the corners of the mouth. Also note the amplified noise in the forehead and cheek regions.

Note that the sharpness is back in the iris, glasses and the mouth corners. For this sequence the extra sharpness did not survive compression using QP 22.

# Objective evaluation

Normally, an objective evaluation would measure BD rate where the distortion would be measured in the luma domain. However, in this case the target Y'CbCr will be different for the 4:2:0 representation currently in JVET and the chroma adjustment 4:2:0 version. Therefore we have chosen to measure the distortion in luminance Y instead of luma Y', or rather in the gamma corrected g-1(Y) version of luminance. The reasoning behind measuring g-1(Y) rather than Y is that it is more similar to a traditional luma BD rate measurement. As an example, if the image was completely gray scale, i.e., Cb = Cr = 0, then g-1Y would equal luma exactly.

The reference value for g-1(Y) is calculated from the original Y'CbCr 4:4:4 representation using Equations (3) through (5) above, followed by application of g-1∙, giving g-1Yorig.

For the current JVET 4:2:0 anchor, the decoded Y'CbCr 4:2:0 file is first upsampled to 4:4:4 using the filters -16, 216, 64, -8/256 and -8, 64, 216, -16/256 since JPEG chroma positions were used when creating the sequence. This results in a Y'CbCr 4:4:4 representation, and then Equations (3) through (5) are used, followed by application of g-1∙, giving g-1(YJVET).

For the chroma adjustment 4:2:0 representation, co-sited chroma positioning was used during downsampling, so co-sited filters need to be used in the upsampling too. We have used -16, 144, 144, -16/256 and 0,0,256,0/256. This results in a Y'CbCr 4:4:4 representation, and then Equations (3) through (5) are used, followed by application of g-1∙, giving g-1YCA.

The mean square error is calculated as MSE=1wh1023*g-1YJVET-1023*g-1Yorig2, where w is the width and h is the height of the image. The PSNR score for each frame is calculated as

PSNR=10ln10232MSE/ln10,

and the PSNR figure for the entire sequence is calculated as the average of the PSNR scores for each frame. The PSNR score for the chroma adjusted sequence is calculated analogously.

The diagram in Figure 6 shows a rate distortion chart of the chroma adjustment method compared to the current anchor for random access.

Figure 6. Rate distortion diagram of compressed material using QP 22, 27, 32 and 37 for random access. The distortion is measured on the gamma corrected luminance.

Note that the difference between the two methods increase with increasing rate. This makes sense, since lossless compression would give a very close to perfect luminance score for chroma adjustment. For the current anchor however, even lossless compression will suffer from luminance artifacts, as we have seen in Figures 1 and 2 above.

The BD-rate difference between the two curves is very high; -21%. One would assume that such a big difference in BD-rate would bring about a huge advantage in perceptual quality. However, when watching the video, the difference seems to be smaller than what a BD-rate difference of 21% would suggest. One reason may be that the bits that are saved from coding amplified noise are instead spent on coding the noise that the current sequence had low-pass filtered away. One example is in the jacked of the right person in Figure 4, which is much more faithfully preserved. While reproducing this noise helps boost PSNR figures substantially it affects perceptual quality less.

The BD rate was also measured on a mostly gray sequence, TrafficFlow. For perfectly gray colors, luma Y’ and gamma corrected luminance g-1(Y) are equivalent. Hence most of the luminance signal should be carried in the luma component for TrafficFlow, and a great difference between using traditional subsampling and subsampling using chroma adjustment is not expected, averaged across the image surface. The result is in line with this expectation; a BD rate change of only -1% was measured for the TrafficFlow sequence.

The distortion above only takes luminance into consideration. Just as in the case with luma, it is not helpful to measure PSNR in Cb and Cr since they differ between the 4:2:0 version currently in JVET and the uncompressed version created by chroma adjustment. Also, as is reported in JCTVC-Z0022, for the saturated problem colors, very different Y'CbCr triplets can represent colors that are indistinguishable to the human visual system. Thus, measuring using Cb and Cr does not work well for sequences like CampfireParty which have a lot of these colors. A better way to measure chrominance would be of interest, but is out of the scope of this contribution.

# Proposals

This contribution proposes to recreate the CampfireParty 4:2:0 sequence using the proper chroma positions. This contribution further proposes the use of chroma adjustment to create the 4:2:0 sequence for CampfireParty.

Another proposal of this contribution is that 4:4:4 versions of all test sequences should be made available so that it will be possible to see if these effects are occurring in other parts of the test set as well.

# References

[1] J. Ström, J. Samuelsson, M. Pettersson, K. Andersson, P. Wennersten, R. Sjöberg, “Ericsson's response to CfE for HDR and WCG”, m36184, the 112th MPEG Meeting in Warsaw, June 2015.

[2] J. Ström, J. Samuelsson and K. Dovstam, ”Luma Adjustment for High Dynamic Range Video”, Proceedings of the IEEE Data Compression Conference (DCC), Snowbird, March 2016.

[3] A. M. Tourapis, Y. Su, D. Singer, “Enhanced Luma Adjustment Methods”, JCT-VC JCTVC-W0052, San Diego, Feb 2016.

[4]	C. Rosewarne and V Kolesnikov, “CE1-related: LUT-based luma sample adjustment”, JCT-VC, JCTVC-W0056, San Diego 2016.

[5] A. Norkin, “Closed form HDR 4:2:0 chroma subsampling (HDR CE1 and AHG5 related)”, JCT-VC JCTVC-W0107, San Diego, Feb 2016.

[6]	J. Ström, J. Samuelsson and K. Dovstam, ”Luma Adjustment for High Dynamic Range Video”, Proceedings of the IEEE Data Compression Conference (DCC), Snowbird, March 2016.

[7] A. Norkin, “Fast algorithm for HDR color conversion”, Proceedings of the IEEE Data Compression Conference (DCC), Snowbird, March 2016.

[8] J. Ström, J. Samuelsson, K. Andersson, P. Hermansson, “Modified Linearization of Luma Adjustment”, JCT-VC, JCTVC-X0036, Geneva, May 2016.

[9] C. Rosewarne and V Kolesnikov, “AHG13: Further results for LUT-based luma sample adjustment”, JCT-VC, JCTVC-X0054, Geneva, May 2016.

[10] A. Norkin, “On closed form HDR 4:2:0 chroma subsampling (AHG13 related)”, JCT-VC, JCTVC-X0072, Geneva, May 2016.

[11] J. Ström, “AhG-13 related: Multi-LUT Luma Adjustment Implementation”, JCT-VC, JCTVC-Y0030, Chengdu, Oct 2016.

[12] A. Norkin, “On HDR 4:2:0 chroma subsampling (AHG13 related)”, JCT-VC, JCTVC-Y0033, Chengdu, Oct 2016.

[13] C. Rosewarne and V Kolesnikov, “AHG13: Further results for luma sample adjustment”, JCT-VC, JCTVC-Y0034, Chengdu, Oct 2016.

[14] J. Ström, P. Wennersten, K. Andersson, R. Sjöberg, M. Pettersson, “AhG on HDR and WCG: Chroma Adjustment for HDR Video”, JCT-VC, JCTVC-Z0022, Geneva, January 2017.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).