| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macao, CN, 18–24 Oct.2017 | Document: JVET-H_Notes_d0 |
| --- | --- |

| Title: | Meeting Report of the 8th meeting of the Joint Video Exploration Team (JVET), Macao, CN, 18–24 Oct.2017 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from responsible coordinators of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Responsible coordinators |  |  |

# Summary

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its eighth meeting during 18–24 Oct. 2017 at the Holiday Inn Macao Cotai Central (Sands Cotai Central Cotai Strip, Macau, CN). The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations. For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section  of this document.

The JVET meeting sessions began at approximately 0900 hours on Wednesday 18 Oct. 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Tuesday 24 Oct. 2017. Approximately XXX people attended the JVET meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. As a primary goal, the JVET meeting finalized the Call for Proposals (CfP), for which a draft had been issued by the previous meeting, refining the testing and evaluation methodology of compression technology designs that would be expected to be proposed in responses to the CfP.

Another important goal of the meeting was to review the work that was performed in the interim period since the seventh JVET meeting in producing the Joint Exploration Test Model 7 (JEM7). Video coding results produced with JEM7 will also be included as anchor in the CfP. Furthermore, results from three exploration experiments conducted in the JEM7 framework were reviewed, and other technical input was considered. On this basis, modifications towards JEM8 were planned.

The JVET produced X output documents from the meeting (update):

- Algorithm description of Joint Exploration Test Model 7 (JEM7)

- Draft Joint Call for Proposals on video compression with capability beyond HEVC

- Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 4

- Results of the Call for Evidence on Video Compression with Capability beyond HEVC

- Description of Exploration Experiments on coding tools

- Common test conditions and evaluation procedures for HDR/WCG video

- Common test conditions and evaluation procedures for 360° video

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. X Exploration Experiments (EE) were defined on particular subject areas of coding tool testing. The next four JVET meetings are planned for Fri. 19 – Fri. 26 Jan. 2018 under WG 11 auspices in Gwangju, KR, during 11 – 20 April 2018 in San Diego, US, during 10 – 18 July 2018 under ITU-T auspices in Ljubljana, SI, and during XX – XX Oct. 2018 under WG11 auspices in XX, XX.

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Exploration Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its seventh meeting during 13–21 July 2017 at the Politecnico di Torino, Torino, IT. The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2017_10_H_Macao/.

## Primary goals

As a primary goal, the JVET meeting finalized the Call for Proposals (CfP), for which a draft had been issued by the previous meeting, refining the testing and evaluation methodology of compression technology designs that would be expected to be proposed in responses to the CfP.

Another important goal of the meeting was to review the work that was performed in the interim period since the seventh JVET meeting in producing the Joint Exploration Test Model 7 (JEM7). Video coding results produced with JEM7 will also be included as anchor in the CfP. Furthermore, results from three exploration experiments conducted in the JEM7 framework were reviewed, and other technical input was considered. On this basis, modifications towards JEM8 were planned.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect the JEM software but have no normative effect are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in the JEM description (an error, oversight, or messiness) or in the software are marked by the string “Decision (BF):”.

This meeting report is based primarily on notes taken by the responsible leaders. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 10 Oct. 2017. Any documents uploaded after 2359 hours Paris/Geneva time on Wednesday 11 Oct. were considered “officially late”, giving a grace period of 24 hrs to those living in different time zones of the world.

All contribution documents with registration numbers JVET-H0072 and higher were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “F0072+” range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contribution was registered on time but was uploaded late:

- JVET-H00XX (a proposal on …), uploaded 10-XX.

The following technical design proposal contributions were both registered late and uploaded late:

- JVET-H00XX (a proposal on …), uploaded 10-XX.

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JVET-H00XX (an information document on …), uploaded 10-XX.

The following cross-verification reports were registered on time but were uploaded late: JVET-H00XX [uploaded 10-XX], ….

(Documents that were both registered late and uploaded late, other than technical proposal documents, are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-X00XX, ….

“Placeholder” contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable rejected in the document management system. The initial uploads of the following contribution document was rejected as “placeholder” and were not corrected until after the upload deadline: JVET-H0043.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, EE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly the meeting report JVET-G1000, JEM7 algorithm description JVET-G1001, the Joint Call for Proposals JVET-G1002, the algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 4 JVET-G1003, the Results of the Call for Evidence on Video Compression with Capability beyond HEVC JVET-G1004, the description of exploration experiments JVET-G1011, the JVET common test conditions and evaluation procedures for HDR/WCG video JVET-G1020, and the JVET common test conditions and evaluation procedures for 360° video JVET-G1030, were approved. The JEM7 software implementation (version 7.0), and the 360Lib software implementation (version 4.0) were also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Review of AHG reports

- Reports of exploration experiments

- Review of results of previous meeting

- Planning of the Call for Proposals

- Consideration of contributions and communications on project guidance

- Consideration of video technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director’s AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the JEM software uses the HEVC reference software copyright license header language is the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the JEM software, and for evaluating proposals for technology to be included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

Note: This applies also to the 360Lib video conversion software as well as the JEM and HM.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It is reminded to send notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers was 741.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT: Adaptive multi-core transform.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP: Advanced temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BIO: Bi-directional optical flow.

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EE: Exploration Experiment – a coordinated experiment conducted toward assessment of coding technology.

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ECV: Extended Colour Volume (up to WCG).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- HDR: High dynamic range.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MDNSST: Mode dependent non-separable secondary transform.

- MMLM: Multi-model (cross component) linear mode.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- QTBT: Quadtree plus binary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- UCBDS: Unrestricted center-biased diamond search.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in JEM:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in P/B slice, and one CTB per luma CTU and two CTBs per chroma CTU in I slice.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size to a CU.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size to a CU.

## Opening remarks

- Reviewed logistics, agenda, working practices, policies, document allocation

- Results of previous meeting: JEM, meeting report, etc.

- Goals of the meeting: Finalization of the joint Call for Proposals (CfE), produce a new version of the JEM algorithm description and software (which version as anchor in CfP?), evaluation of status progress in EEs and new proposals, selection of test sequences and common test conditions for evaluation testing, expert viewing assessment of JEM status, improved 360Lib software, define new EEs (?).

- Discuss further planning of CfP and beyond with by parent bodies

- Meeting ends by Tuesday Oct. 24

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Wed. 18 Oct., 1st day

  - 0900-1300 Opening, AHG reports (chaired by GJS)

  - …

- Thu. 19 Oct., 2nd day

  - …

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized

- AHG reports (10) (section 3)

- Analysis, development and improvement of JEM (1) (section 4)

- Test material (4) (section 5)

- Call for Proposals preparation (4) (section 6)

- Exploration experiments (32) (section )

  - EE Summary (1) (section 7.1)

  - EE1 and related: Intra Prediction and mode coding (17) (section 7.2)

  - EE2 and related: Entropy coding (4) (section 7.3)

  - EE3 and related: Adaptive loop filter (10) (section 7.4)

- Non-EE technology proposals (12) (section 8)

- Extended colour volume coding (0) (section 9)

- Coding of 360o video projection formats (6) (section 10)

- Complexity analysis (2) (section 11)

- Encoder optimization (1) (section 12)

- Metrics and evaluation criteria (2) (section )

- Withdrawn (4) (section 14)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 15)

- Project planning (section 16)

- Output documents, AHGs (section 17)

# AHG reports (10)

These reports were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

JVET-H0001 JVET AHG report: Tool evaluation (AHG1) [M. Karczewicz, E. Alshina]

JVET-H0002 JVET AHG report: JEM algorithm description editing (AHG2) [J. Chen, E. Alshina, J. Boyce] [miss]

JVET-H0003 JVET AHG report: JEM software development (AHG3) [X. Li, K. Suehring]

JVET-H0004 JVET AHG report: Test material (AHG4) [T. Suzuki, V. Baroncini, J. Chen, J. Boyce, A. Norkin]

JVET-H0005 JVET AHG report: Memory bandwidth consumption of coding tools (AHG5) [X. Li, E. Alshina, R. Hashimoto, T. Ikai, H. Yang]

JVET-H0006 JVET AHG Report: 360 video conversion software development (AHG6) [Y. He, V. Zakharchenko]

JVET-H0007 JVET AHG report: JEM coding of HDR/WCG material (AHG7) [A. Segall (chair), E. François, D. Rusanovskyy] [miss]

JVET-H0008 JVET AHG report: 360º video coding tools and test conditions (AHG8) [J. Boyce, A. Abbas, E. Alshina, G. van der Auwera, Y. Ye] [miss]

JVET-H0009 JVET AHG report: 4:4:4 support in JEM (AHG9) [A. Tourapis, X. Li, X. Xu] [miss]

JVET-H0010 JVET AHG report: Denoising and adaptive quantisation [R. Sjöberg, E. Alshina, S. Ikonin, A. Norkin, T. Wiegand]

# Analysis, development and improvement of JEM (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

JVET-H0084 NextSoftware: An alternative implementation the Joint Exploration Model (JEM) [Adam Wieckowski, Tobias Hinz, Valeri George, Jens Brandenburg, Jackie Ma, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Wiegand (Fraunhofer HHI)] [late]

# Test material (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

JVET-H0021 AHG8: Test Sequences for Spherical Video Coding from GoPro [A. Abbas (GoPro)]

JVET-H0022 AHG8: New Sequences for Virtual Reality Video Coding from InterDigital [Y. He, P. Hanhart, E. Asbun, Y. He, Y. Ye (InterDigital)]

JVET-H0038 AHG7: Test Sequences of 4K Hybrid Log-Gamma [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)]

JVET-H0083 AHG7: Cross-check of test sequences of 4K Hybrid Log-Gamma transfer characteristics (JVET-H0038) [S. Nemoto, S. Iwamura, A. Ichigaya (NHK)]

# Call for Proposals preparation (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

## General discussions

JVET-H0082 Comments on HDR category test conditions of draft CfP on video compression beyond HEVC [D. Rusanovskyy, M. Karczewicz (Qualcomm Inc.), E. Francois, M. Kerdranvat (Technicolor)] [late]

## Anchor preparation

JVET-H0060 AHG4: SDR anchor generation for the Draft Joint Call for Proposals [H.-C. Chuang, J. Chen, M. Karczewicz (Qualcomm), K. Choi, E. Alshina (Samsung)]

## Definition of test cases

JVET-H0068 AHG7: HLG rate points for CfP anchor generation [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

## Evaluation methodology

JVET-H0055 Analysis of the HDR Objective Metrics Utility in Draft CfP on FVC [P. Yin, T. Chen, W. Husak, F. Pu, T. Lu (Dolby)]

# Exploration experiments (47)

## General (1)

JVET-H0011 Exploration Experiments on Coding Tools Report [E. Alshina, L. Zhang]

## EE1: Intra prediction (X)

### General

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

### Primary (X)

JVET-H0024 EE1: Improvements for Intra Prediction Mode Coding [Y. Han, J. An, J. Zheng (Hisilicon)]

JVET-H0027 EE1: PDPC without a mode flag [V. Seregin, M. Karczewicz, A. Said, X. Zhao (Qualcomm)]

JVET-H0028 EE1: Fix for strong intra smoothing filtering [V. Seregin, X. Zhao, M. Karczewicz (Qualcomm)]

JVET-H0029 EE1: Secondary MPM list [V. Seregin, W.-J. Chien, M. Karczewicz, N. Hu, X. Zhao (Qualcomm)]

JVET-H0033 EE1: Crosscheck of tests 8 and 9 [Y. Han, J. An, J. Zheng (Hisilicon)]

JVET-H0042 EE1: Cross-check of test 6 [H. Ko, H. Lee, J. Kang (ETRI)]

JVET-H0052 EE1 Tests 2 and 3: Replace Planar mode in P-PDPC with UWP and Enable UW66 on top of the above test [S. Hong, K. Panusopone, Y. Yu, L. Wang (ARRIS), H. M. Jang, J. Lim, S.-H. Kim (LGE)]

JVET-H0062 EE1: Cross-check of Test 7 [D. Jun, H. Lee, J. Kang (ETRI)]

JVET-H0076 EE1: Crosscheck of test 1 [Virginie Drugeon (Panasonic)] [late]

JVET-H0080 EE1: Cross-check of Test 5 [E. Mora (Ateme)] [late]

JVET-H0088 EE1: Crosscheck of tests 2 and 3 [A. Filippov, V. Rufitskiy, T. Solovyev (Huawei)] [late] [miss]

### Related (X)

JVET-H0051 Non-EE1: Priority List Based Intra Mode Coding with 5 MPM [Y. Yu, L. Wang, K. Panusopone (ARRIS)]

JVET-H0075 Non-EE1: Crosscheck of Priority List Based Intra Mode Coding with 5 MPM (JVET-H0051) [T. Ikai (Sharp)] [late]

JVET-H0057 EE1 related: Simplification and extension of PDPC [X. Zhao, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

JVET-H0074 EE1-related: Cross-check of Simplification and extension of PDPC (JVET-H0057) [H. Lee, H. Ko, J. Kang (ETRI)] [late]

JVET-H0059 EE1-related: Two unifications for PDPC [H. Jang, J. Nam, J. Lim, S.-H. Kim (LGE)]

JVET-H0078 Cross-check of JVET-H0059 [K. Choi (Samsung)] [late] [miss]

## EE2: Entropy coding (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

### General

### Primary (X)

JVET-H0061 EE2: Arithmetic coding with context-dependent double-window adaptation response [A. Said, M. Karczewicz, V. Seregin, H. Egilmez, L. Zhang, X. Zhao (Qualcomm)]

JVET-H0069 EE2: Cross-check for Arithmetic coding with context-dependent double-window adaptation response [E. Alshina (Samsung)] [late]

JVET-H0085 EE2: Cross-check for JVET-G00112 Test 3.2 and 3.3 [M. Sychev (Huawei)] [late]

### Related (X)

JVET-H0067 EE2 related: Arithmetic coding with progressive context-dependent double-window adaptation response [A. Said, H. Egilmez, M. Karczewicz, V. Seregin, L. Zhang, X. Zhao (Qualcomm)]

## EE3: Adaptive loop filter (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

### General

### Primary (X)

JVET-H0037 EE3.7: chroma ALF on/off flag [T. Ikai (Sharp), J. An, J. Zheng (HiSilicon)]

JVET-H0041 EE3 Test1-6: Unified Adaptive Loop Filter for Luma and Chroma [J. An, J. Zheng (HiSilicon)]

JVET-H0045 EE3: Test on Chroma QP offset [S. Ikonin, R. Chernyak, T. Solovyev, S. Liu (Huawei)]

JVET-H0046 EE3: Cross-check of Test 6 (JVET-H0041) [L. Zhang, W.-J. Chien (Qualcomm)] [late]

JVET-H0054 EE3: Cross-check of Test 7 additional results (JVET-H0037) [L. Zhang (Qualcomm)] [late]

JVET-H0063 EE3: Cross-check of Test 3 [D. Jun, H. Lee, J. Kang (ETRI)]

JVET-H0064 Crosscheck of EE3 Test 2 of JVET-H0041 [X. Zhang (Kingsoft Cloud)] [late]

JVET-H0065 EE3: Crosscheck of Test7 [J. An (HiSilicon)] [late]

JVET-H0070 EE3: Cross-check for Chroma QP offset 0 and 2 (tests 1 and1.1) [E. Alshina (Samsung)] [late]

JVET-H0073 EE3: Cross-check of Test4 and 5 [Ryuichi Kanoh] [late]

### Related (X)

# Non-EE Technology proposals (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

## Intra coding (X)

JVET-H0036 Condition modification of Strong Intra Smoothing Filter [Y. Kidani, K. Kawamura, S. Naito (KDDI)]

JVET-H0086 Cross-check of JVET-H0036 on strong intra smoothing condition [V. Seregin (Qualcomm)] [late] [miss]

JVET-H0053 Weighted Angular Mode 2 and Adaptive Angular Mode 66 [K. Panusopone, Y. Yu, S. Hong, L. Wang (ARRIS)]

JVET-H0071 Redundant flag removal on chroma intra mode coding [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

## Inter coding (X)

JVET-H0031 Inter Prediction using Estimation and Explicit Coding of Affine Parameters [C. Heithausen, J.-R. Ohm (RWTH Aachen Univ.)]

JVET-H0058 Unified search range for FRUC [H. Jang, J. Nam, J. Lim, S.-H. Kim (LGE)]

JVET-H0077 Cross-check of JVET-H0058 [K. Choi (Samsung)] [late] [miss]

## Loop filters (X)

JVET-H0034 A simplified method to sao_type_idx coding of sample adaptive offset (SAO) [X. Zhang, Z. Zhu, J. Fan, X. Jin, E. Zhang (Kingsoft Could)]

JVET-H0066 Crosscheck of JVET-H0034 A simplified method to sao_type_idx coding of SAO [J. An (HiSilicon)] [late]

## Other (5)

JVET-H0072 Modification of merge candidate derivation for binary split CUs [Y. Ahn, H. Ryu, D. Sim (Digital Insights), J. Lim, D. Jeon (Kaonmedia)] [late]

JVET-H0081 Cross-check of JVET-H0072 “Modification of merge candidate derivation for binary split CUs” [Y. Uk, D. Hyun, J.-G. Kim (??)] [late]

JVET-H0087 Diagonal motion partitions on top of QTBT block structure [Y. Ahn, H. Ryu, D. Sim (Digital Insights)] [late]

# Extended colour volume coding (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

## Test conditions and evaluation (X)

## Tools (X)

# Coding of 360° video projection formats (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

## Conversion tools, 360lib (X)

## Packing and Projection formats (X)

JVET-H0056 AHG8: An Update on RSP Projection [Adeel Abbas, David Newman (??)]

JVET-H0079 Cross-check of JVET-H0056 on an update on RSP Projection [S.N. Akula, C. Pujara (Samsung)] [late]

## Quality assessment and metrics (X)

JVET-H0035 AHG8: The guidance of WS-PSNR weights derivation for different formats [Y. Sun, L. Yu (Zhejiang University)]

JVET-H0048 AhG8: Dynamic viewport based subjective evaluation of Balboa and Landing [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-H0049 AhG8: On viewport size and field of view [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-H0050 AhG8: Measurement of User Exploration Behavior for Omnidirectional (360°) Videos with Head Mounted Display [A. Singla, A. Raake, S. Fremerey, P. List, B. Feiten (??)] [late]

## Coding tools (X)

## Padding (X)

## HL syntax (X)

# Complexity analysis (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

JVET-H0039 AHG5: On worst case memory bandwidth [T. Ikai (Sharp), R. Hashimoto, S. Mochizuki (Renesas)] [late]

JVET-H0043 AHG5: How to measure memory bandwidth [R. Hashimoto, S. Mochizuki (Renesas), T. Ikai (Sharp)] [placehold] [late]

# Encoder optimization (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

JVET-H0023 Implementation and design aspects of xvc [J. Samuelsson, P. Hermansson (Divideon)]

- not related to JEM

# Metrics and evaluation criteria (X)

Contributions in this category were discussed XXday XX Oct. XXXX–XXXX (chaired by XXX).

JVET-H0030 BD-Rate/BD-PSNR Excel extensions [A.M. Tourapis, D. Singer, Y. Su, K. Mammou (Apple Inc)]

JVET-H0047 Perceptually optimized QP adaptation and associated distortion measure [S. Bosse, C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)]

# Withdrawn

See under 2.4.2.

JVET-H0025 Withdrawn

JVET-H0026 Withdrawn

JVET-H0032 Withdrawn

JVET-H0040 Withdrawn

# Joint Meetings, BoG Reports, and Summary of Actions Taken

## Joint meetings

JVET-H0044 Do we need multiview profiles for future video coding generations? [J. Samelak, O. Stankiewicz, M. Domanski (Poznan Univ.)] [late]

## BoGs

## List of actions taken affecting JEM8 and 360lib5 (update)

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the JEM7 or 360Lib4.0 description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

Was presented and confirmed to be complete Tuesday 24th in the JVET plenary.

### Encoder only or CTC/software changes

…

General: It was agreed to ordinarily report only one digit past the decimal point of percentage BD impacts.

### Syntax/semantics/decoding process changes

…

### Changes in 360lib

…

General: It was agreed that the list of projection formats included in the CTC & 360Lib will not grow further, to avoid having so many that we can’t properly study them. If we want to add one, we need a decision to remove one. Anchors for projection formats to be made available only with HM and ERP for JEM.

# Project planning

## Exploration Experiment planning (update)

The setup of Exploration Experiments was discussed, and an initial draft of the EE document was reviewed in the plenary (chaired by JRO). This included the list of all tools that are intended to be investigated in EEs during the subsequent meeting cycle:

EEX: XXX

JVET-H00XX

…

It was agreed to give the editors the discretion to finalize the document during the two weeks after the meeting, and circulate/discuss it on the reflector appropriately.

## JEM description drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- JEM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XXday XX Jan. 2018.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for Experiments

Note: This section was drafted during the second JVET meeting, and is kept here for information about the EE procedure.

Group coordinated experiments have been planned. These may generally fall into one category:

- “Exploration experiments” (EEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the main branch of JEM by the next meeting.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. (E. Alshina will edit the document based on input from the proponents, review is performed in the plenary)

- Software for tools investigated in EE is provided in a separate branch of the software repository

- During the experiment, further improvements can be made

- By the next meeting it is expected that at least one independent party will report a detailed analysis about the tool, confirms that the implementation is correct, and gives reasons to include the tool in JEM

- As part of the experiment description, it should be captured whether performance relative to JEM as well as HM (with all other tools of JEM disabled) should be reported by the next meeting.

It is possible to define sub-experiments within particular EEs, for example designated as EEX.a, EEX.b, etc., where X is the basic EE number.

As a general rule, it was agreed that each EE should be run under the same testing conditions using one software codebase, which should be based on the JEM software codebase. An experiment is not to be established as a EE unless there is access given to the participants in (any part of) the TE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-G1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a company proponent perspective – e.g. referring to methods as “improved”, “optimized” etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to EE work should identify individuals in addition to company names.

EE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the EE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish an EE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to EE activities were established as follows:

- Only qualified JVET members can participate in an EE.

- Participation in an EE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the EE should be announced and made available to all EE participants in a timely manner.

A separate branch under the experimental section will be created for each new tool include in the EE. The proponent of that tool is the gatekeeper for that separate software branch. (This differs from the main branch of the JEM, which is maintained by the software coordinators.)

New branches may be created which combine two or more tools included in the EE document or the JEM. Requests for new branches should be made to the software coordinators.

Don’t need to formally name cross-checkers in the EE document. To promote the tool to the JEM at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

Timeline:

T1 = JEM8.0 SW release + 4 weeks: Integration of all tools into separate EE branch of JEM is completed and announced to JVET reflector.

Initial study by cross-checkers can begin.

##### Proponents may continue to modify the software in this branch until T2

3rd parties encouraged to study and make contributions to the next meeting with proposed changes

T2: JVET-I meeting start – 3 weeks: Any changes to the exploration branch software must be frozen, so the cross-checkers can know exactly what they are cross-checking. An SVN tag should be created at this time and announced on the JVET reflector.

This procedure was again confirmed during the closing plenary of the third JVET meeting. It was further confirmed that the Common Test Conditions of JVET-G1010 are still valid, however the CTC encoder setting will be reflected in the config file that is attached to the JEM4.0 package.

## Software development and anchor generation (update)

Software coordinators will work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- JEM7.0 including all adoptions from section 15.3 will be released by 2017-08-04.

- The results about coding performance of JEM7.0 will be reported by 2017-08-11.

- Further versions may be released for additional bug fixing, as appropriate

Timeline of 360lib4.0: 2 weeks after the meeting (2017-08-04).

- Further versions may be released as appropriate for bug fixing.

Timelines and volunteers for CfP anchors:

Also action seems necessary for rate tuning in the following sequences:

- Cat Robot rates 3 and 4 lower

- Daylight road all rates lower

- BQ terrace: Highest rate lower

- Ritual Dance rates 3 and 4 lower

- Market rates 1 and 2 lower

- Show Girl rates 2-4 lower

- Starting rates 3 and 4 lower

- The change from 4096 to 3840 width for some sequences also requires proportional lowering of rates.

New rate points are determined regarding the HM anchor quality. JRO will propose new rate points by 08-04, and if agreed by 08-11, anchor generation can start.

HM 16.16 anchors by 09-01

JEM 7.0 anchors by 09-29

- For SDR: HD/RA, HD/LD, UHD: Samsung/Qualcomm

- For 360: InterDigital/Samsung

- For HDR: Technicolor/Qualcomm

New HM anchors will be generated using HM 16.16. JEM anchors will be based on JEM 7.0.

- responsibilities for updating sequences: Original contributors except Campfire (Alexis Tourapis)

New sequences needed by 08-01.

Investigation and generating anchors for objective testing:

- CTC set (UHD, HD) plus MountainBay

- Identify more sequences via AHG4

- Discuss by next meeting how to formulate PSNR matching

Excel sheets to be attached to CfP (Sept. 29), to be exercised with JEM vs. HM:

- SDR: J. Chen

- HDR: E. François

- 360: Y. He

Note: MD5 checksums shall not be included in anchor bitstreams.

# Output documents and AHGs (update)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JVET-G1000 Meeting Report of the 7th JVET Meeting [G. J. Sullivan, J.-R. Ohm] [2017-10-15] (near next meeting)

Intermediate versions of the meeting notes (d0 … d8) were made available on a daily basis during the meeting.

JVET-G1001 Algorithm description of Joint Exploration Test Model 7 (JEM7) [J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce] [2017-08-18] (MPEG N17055)

See list of new adoptions under 15.3. During the closing plenary, no complaints were made about the accuracy of that list.

JVET-G1002 Draft Joint Call for Proposals on Video Compression with Capability beyond HEVC [A. Segall, M. Wien, V. Baroncini, J. Boyce, T. Suzuki] [2017-08-11] (MPEG N17053)

Draft was discussed Wed 1400 and in joint meeting with parent bodies Wed 1600. Was again presented during closing plenary on Friday.

The companies responsible for providing the HM and JEM anchors will also provide the corresponding Excel templates for the cases of SDR, HDR and 360.

(see responsibilities under 16.5)

JVET-G1003 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 4 [Y. Ye, E. Alshina, J. Boyce] [2017-08-18] (MPEG N17056)

See list of new adoptions under 15.3.3. During the closing plenary, no complaints were made about the accuracy of that list.

JVET-G1004 Results of the Call for Evidence on Video Compression with Capability beyond HEVC [M. Wien, V. Baroncini, P. Hanhart, J. Boyce, A. Segall] [2017-08-25] (MPEG N17054)

Was presented. Some updates: Make proposal results anonymous; identify confidence intervals by same colours; distinguish between half and full MOS.

JVET-G1010 JVET common test conditions and software reference configurations [K. Suehring, X. Li] [2017-08-01]

Reflects updates of test sequences.

JVET-G1011 Description of Exploration Experiments on coding tools [E. Alshina, L. Zhang] [2017-08-11] (MPEG N17057)

Initial version was presented in the closing plenary on Friday 20th Jan. Additional tests were proposed in the initial version related to JVET-G0146. This was removed, as it had non been agreed before, and the reported benefit seems to be rather low (variation compared to EE1 test 7 clearly less than 0.1%).

See list of EEs under 16.1.

JVET-G1020 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, D. Rusanovskyy] [2017-07-28]

JVET-G1030 JVET common test conditions and evaluation procedures for 360° video [E. Alshina, J. Boyce, A. Abbas, Y. Ye] [2017-07-28]

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header should be generated. This version should be sent to GJS and JRO for upload.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Tool evaluation (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate the exploration experiments. - Investigate interaction of tools in JEM and exploration experiment branches. - Discuss and evaluate methodologies and criteria to assess the benefit of tools, and how to ease the assessment of single tools in terms of encoder runtime. - Study and summarize new technology proposals. - Discuss methodologies for objective comparison in the forthcoming Call for Proposals. | E. Alshina, M. Karczewicz (co-chairs) | N |
| JEM algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-G1001 Algorithm Description of Joint Exploration Test Model 7. - Gather and address comments for refinement of the document. - Coordinate with the JEM software development AHG to address issues relating to mismatches between software and text. | J. Chen (chair), E. Alshina, J. Boyce (vice chairs) | N |
| JEM software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of the JEM7.0 software packages and their distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver JEM7.0 software version and the reference configuration encodings according to JVET-G1010 common conditions. - Coordinate with AHG on JEM model editing and errata reporting to identify any mismatches between software and text, and make further updates and cleanup to the software as appropriate. - Investigate the implementation of SCC coding tools in JEM. - Coordinate with AHG6 for integration of 360 video software. - Coordinate with AHG6 for integration of 360 video software. | X. Li, K. Suehring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of future video coding standards. - Identify and recommend appropriate test materials and corresponding test conditions for use in the development of future video coding standards. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Discuss and prepare HM anchors at additional rate points for the Call for Proposals. - Evaluate new test sequences, and prepare for the visual assessment in the next meeting. - Discuss subjective comparison methodologies, and make logistic arrangements for the forthcoming Call for Proposals. - Prepare viewing equipment arrangements for the upcoming meeting. | V. Baroncini, T. Suzuki (co-chairs), J. Chen, J. Boyce, A. Norkin (vice chairs) | N |
| Memory bandwidth consumption of coding tools (AHG5) (jvet@lists.rwth-aachen.de) - Study the methodology of measuring decoder memory bandwidth consumption, including cache models. - Develop software tools for measuring both average and worst case of memory bandwidth. - Make analysis for examples of JEM coding tools. - Study the impact of memory bandwidth on specific application cases. | X. Li (chair), E. Alshina, R. Hashimoto, T. Ikai, H. Yang (vice chairs) | N |
| 360° video conversion software development (AHG6) (jvet@lists.rwth-aachen.de) - Prepare and deliver 360Lib-4.0 software version and common test condition configuration files according to JVET-G1030. - Generate CTC HM anchors for all projection formats, CTC JEM anchors for the ERP projection format, and a reporting template for the common test conditions. - Produce documentation of software usage for distribution with the software. | Y. He, V. Zakharchenko (co-chairs) | N |
| JEM coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Coordinate generation of HM and JEM anchors for the draft CfP. - Study and evaluate available HDR/WCG test content. - Study objective metrics for quality assessment of HDR/WCG material. - Evaluate transfer function conversion methods, including methods that may be standardized by BT.[HDR-OPS] - Study and refine test conditions and anchors for the JEM coding of HDR/WCG content. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. François, D. Rusanovskyy (vice chairs) | N |
| 360° video coding tools and test conditions (AHG8) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Study consistency of and potential improvements to the objective quality metrics in CTC. - Coordinate effort to prepare for CfP testing, including anchor generation, in collaboration with AHG5. - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Produce and finalize JVET-G1003 algorithm descriptions of projection format conversion process and objective quality metrics in 360Lib. - Produce and finalize JVET-G1030 JVET common test conditions and evaluation procedures for 360 video. - Study coding tools dedicated to 360 video, and their impact on compression. - Study the effect of viewport resolution, field of view, and viewport speed/direction on visual comfort. - Study the impact of coding resolution vs original ERP resolution on coding efficiency. | J. Boyce (chair), A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye (vice chairs) | Y (Phone) |
| 4:4:4 support in JEM (AHG9) (jvet@lists.rwth-aachen.de) - Evaluate JEM6.0 software in terms of 4:4:4 chroma sampling support: Identify the tools in JEM6.0 that are not able to support 4:4:4 chroma sampling appropriately. Also identify whether this is due to implementation limitations or tool characteristics. - Further investigate the problems associated with RExt tools, and perform an investigation with JEM inter coding tools. | A. Tourapis (chair), X. Li, X. Xu (vice chairs) | N |
| Denoising and adaptive quantisation (AHG10) (jvet@lists.rwth-aachen.de) - Investigate the impact of using denoising filters on input sequences before encoding with HM and JEM. - Study the impact of using adaptive quantization in context of HM and JEM SDR coding. - Perform visual quality assessment for cases using denoising filters, renoising, and adaptive quantisation. - Study objective error metrics for measuring small subjective compression efficiency improvements when adaptive quantisation is used. - Solicit input contributions demonstrating subjective benefits over the JEM 7 anchor. | R. Sjöberg (chair), E. Alshina, S. Ikonin, A. Norkin, T. Wiegand (vice chairs) | N |

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Tuesday or Wednesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–7.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Thursday or Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 8.5 meeting days).

In cases where high workload is expected for a meeting, an earlier starting date may be defined.

Some specific future meeting plans (to be confirmed) were established as follows:

- Fri. 19 Jan. – Fri. 26 Jan. 2018, 9th meeting under WG 11 auspices in Gwangju, KR.

- Wed. 11 Apr. – Fri. 20 Apr. 2018, 10th meeting under WG 11 auspices in San Diego, US.

- Tue. 10 – Wed. 18 July 2018, 11th meeting under ITU-T auspices in Ljubljana, SI.

- XX – XX Oct. 2018, 12th meeting under WG 11 auspices in XX, XX.

The agreed document deadline for the 9th JVET meeting is XXday XX Jan. 2018. Plans for scheduling of agenda items within that meeting remain TBA.

XXXX was thanked for the excellent hosting of the 8th meeting of the JVET. NHK and GBTech were thanked for providing viewing equipment. Vittorio Baroncini, Philippe Hanhart, Atsuro Ichigaya, Shunsuke Iwamura, Shimpei Nemoto, and Mathias Wien were thanked for conducting the visual tests in the context of the Call for Evidence. The participants in the expert viewing were also thanked. CRAN/CNRS, GoPro, InterDigital, LetinVR, and Sony were thanked for offering new test sequences.

The 8th JVET meeting was closed at approximately XXXX hours on Tuesday 24 Oct. 2017.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the eighth meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately XXX people in total), were as follows: