| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macao, CN, 18–24 Oct. 2017 | Document: JVET-H_Notes_dA |
| --- | --- |

| Title: | Meeting Report of the 8th meeting of the Joint Video Exploration Team (JVET), Macao, CN, 18–24 Oct. 2017 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from responsible coordinators of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Responsible coordinators |  |  |

# Summary

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its eighth meeting during 18–24 Oct. 2017 at the Holiday Inn Macao Cotai Central (Sands Cotai Central Cotai Strip, Macau, CN). The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations. For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.13 of this document.

The JVET meeting sessions began at approximately 0900 hours on Wednesday 18 Oct. 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1246 hours on Tuesday 24 Oct. 2017. Approximately 195 people attended the JVET meeting, and approximately 75 input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. As a primary goal, the JVET meeting finalized the Call for Proposals (CfP), for which a draft had been issued by the previous meeting, refining the testing and evaluation methodology of compression technology designs that would be expected to be proposed in responses to the CfP.

Another important goal of the meeting was to review the work that was performed in the interim period since the seventh JVET meeting in producing the Joint Exploration Test Model 7 (JEM7). Video coding results produced with JEM7 will also be included as anchor in the CfP. Furthermore, results from three exploration experiments conducted in the JEM7 framework were reviewed, and other technical input was considered. On this basis, it was decided that JEM7 should be frozen at this moment, as it will be used as an anchor in the CfP testing; therefore, beyond bug fixing, no modifications such as adding tools were planned.

The JVET produced 6 output documents from the meeting:

- Joint Call for Proposals on video compression with capability beyond HEVC

- Draft template for proposal description documents

- Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 5

- JVET common test conditions and software reference configurations

- Common test conditions and evaluation procedures for HDR/WCG video

- Common test conditions and evaluation procedures for 360° video

For the organization and planning of its future work, the JVET established 10 “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, no Exploration Experiments (EE) were defined. The next four JVET meetings were planned for Sat. 20 – Fri. 26 Jan. 2018 under WG 11 auspices in Gwangju, KR, 11–20 April 2018 in San Diego, US, 10–18 July 2018 under ITU-T auspices in Ljubljana, SI, and 4–12 Oct. 2018 under WG11 auspices in Macao, CN.

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Exploration Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its eighth meeting during 18–24 Oct. 2017 at the Holiday Inn Macao Cotai Central (Sands Cotai Central Cotai Strip, Macau, CN). The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as the responsible coordinators of the two organizations.

It is noted that at their current meeting, the parent bodies established a plan to modify the unabbreviated name of JVET to “Joint Video Experts Team” when entering the phase of formal standard development (expected in April 2018), pending the outcome of the Call for Proposals (CfP) issued at the current meeting.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2017_10_H_Macao/.

## Primary goals

As a primary goal, the JVET meeting finalized the Call for Proposals (CfP), for which a draft had been issued by the previous meeting, in particular refining the testing and evaluation methodology of compression technology designs that would be expected to be proposed in responses to the CfP.

Another important goal of the meeting was to review the work that was performed in the interim period since the seventh JVET meeting in producing the Joint Exploration Test Model 7 (JEM7). Video coding results produced with JEM7 will also be included as anchor in the CfP. Furthermore, results from three exploration experiments conducted in the JEM7 framework were reviewed, and other technical input was considered. On this basis, it was decided that JEM7 should be frozen at this moment, as it will be used as an anchor in the CfP testing; therefore, beyond bug fixing, no modifications such as adding tools were planned.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect the JEM software but have no normative effect are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in the JEM description (an error, oversight, or messiness) or in the software are marked by the string “Decision (BF):”.

This meeting report is based primarily on notes taken by the responsible leaders. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 10 Oct. 2017. Any documents uploaded after 2359 hours Paris/Geneva time on Wednesday 11 Oct. were considered “officially late”, giving a grace period of 24 hours to accommodate those living in different time zones of the world.

All contribution documents with registration numbers JVET-H0072 and higher were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “H0072+” range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

There were no technical design proposal contributions that were registered on time but uploaded late for the current meeting.

The following technical design proposal contributions were both registered late and uploaded late:

- JVET-H0072 (a proposal on modified merge candidate derivation in binary split), uploaded 10-12.

- JVET-H0087 (a proposal on diagonal motion partitions in QTBT), uploaded 10-17.

The following other documents not proposing normative technical content, but with some need for consideration were uploaded late:

- JVET-H0039 (an information document on worst case memory bandwidth analysis), uploaded 10-13.

- JVET-H0044 (a information document on using modified HEVC screen content coding tools for the coding of depth maps), uploaded 10-15.

- JVET-H0050 (an information document on user behaviour in the usage of 360° video), uploaded 10-12.

- JVET-H0082 (an information document on CfP test conditions for HDR), uploaded 10-14.

- JVET-H0084 (an information document on an alternative software implementation of the JEM), uploaded 10-16.

- JVET-H0091 (a proposal of a non-normative encoding technique for fast encoding for the JEM), uploaded 10-18.

- JVET-H0093 (an information document to provide information regarding the 360-degree video anchor generation for the Draft Joint Call for Proposals), uploaded 10-19.

- JVET-H0094 (an information document on analysis of HLG content), uploaded 10-20.

- JVET-H0099 (an information document studying the effect of luma/chroma QP adaptation for HLG video content), uploaded 10-21.

The following cross-verification reports were registered on time but were uploaded late: JVET-H0046 [uploaded 10-15], JVET-H0054 [uploaded 10-14], JVET-H0064 [uploaded 10-16], JVET-H0065 [uploaded 10-16], JVET-H0066 [uploaded 10-17], JVET-H0069 [uploaded 10-13], JVET-H0070 [uploaded 10-13].

(The cross-verification documents that were both registered late and uploaded late are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-H0025, JVET-H0026, JVET-H0032, JVET-H0040, JVET-H0092.

“Placeholder” contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and rejected in the document management system. The initial upload of the following contribution document was rejected as a “placeholder” and was not corrected until after the upload deadline: JVET-H0043 (an information document on memory bandwidth analysis). The initial upload was reported to just have been an accidental uploading of the wrong version, and a new version was provided on 10-13.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, EE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly the meeting report JVET-G1000, JEM7 algorithm description JVET-G1001, the Joint Call for Proposals JVET-G1002, the algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 4 JVET-G1003, the Results of the Call for Evidence on Video Compression with Capability beyond HEVC JVET-G1004, the description of exploration experiments JVET-G1011, the JVET common test conditions and evaluation procedures for HDR/WCG video JVET-G1020, and the JVET common test conditions and evaluation procedures for 360° video JVET-G1030, were approved. The JEM7 software implementation (version 7.0), and the 360Lib software implementation (version 4.0) were also approved.

The group had initially been asked to review the meeting report of the previous meeting for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Review of AHG activities

- Reports of exploration experiments

- Review of results of previous meeting

- Finalization and planning related to the Call for Proposals

- Consideration of contributions and communications on project guidance

- Consideration of video technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director’s AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the JEM software uses the HEVC reference software copyright license header language is the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the JEM software, and for evaluating proposals for technology to be included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

Note: This applies also to the 360Lib video conversion software as well as the JEM and HM.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It is reminded to send notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers was 771.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT: Adaptive multi-core transform.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP: Advanced temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BIO: Bi-directional optical flow.

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EE: Exploration Experiment – a coordinated experiment conducted toward assessment of coding technology.

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ECV: Extended Colour Volume (up to WCG).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- HDR: High dynamic range.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MDNSST: Mode dependent non-separable secondary transform.

- MMLM: Multi-model (cross component) linear mode.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- QTBT: Quadtree plus binary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- UCBDS: Unrestricted center-biased diamond search.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in JEM:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in P/B slice, and one CTB per luma CTU and two CTBs per chroma CTU in I slice.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size to a CU.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size to a CU.

## Opening remarks

- Reviewed logistics, agenda, working practices, policies, document allocation

- Results of previous meeting: JEM, meeting report, etc.

- Goals of the meeting: Finalization of the joint Call for Proposals (CfE), potentially produce a new version of the JEM algorithm description and software (and identify which version as anchor in CfP), evaluation of status progress in EEs and new proposals, selection of test sequences and common test conditions for evaluation testing, expert viewing assessment of JEM status, improved 360Lib software, potentially define new EEs.

- Discuss further planning of CfP and beyond with parent bodies

- Meeting ending on Tuesday Oct. 24.

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Wed. 18 Oct., 1st day

  - 0900–1230 Opening plenary, AHG reports (chaired by GJS)

  - 1430– Exploration experiments

- Thu. 19 Oct., 2nd day

  - 1115 CfP prep on test conditions and sequences for SDR, sequences for 360° video

  - 1400–1915 EE and EE related (7), non-EE (8), complexity analysis (11) (chaired by JRO)

  - 1400–1830 BoG on HDR (chaired by A. Segall)

- Fri. 20 Oct., 3rd day

  - 0900– 360° video, metrics

  - 0900– BoG on HDR (chaired by A. Segall)

  - 1400–1530 remaining doc review

  - 1400– BoG on CfP questionnaire (chaired by M. Zhou)

- Sat. 21 Oct., 4th day

  - 0900–1330 Review of BoG activities, Update of CfP, further planning

  - 1500–1700 BoG on HDR (chaired by A. Segall & BoG on CfP questionnaire (chaired by M. Zhou)

  - 1730 HDR viewing session

- Sun. 22 Oct., 5th day

  - 0900–1200 Review remaining input docs

  - 1200 HDR viewing

  - 1400–1800 BoG results, revisits, further planning

  - 1800–1900 BoG on CfP questionnaire (chaired by M. Zhou)

- Mon. 23 Oct., 6th day

  - 1630–1800 Joint meeting with parent bodies on CfP

- Tue. 24 Oct., 7th day

  - 1000–1300 JVET closing plenary

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized

- AHG reports (10) (section 3)

- Analysis, development and improvement of JEM (1) (section 4)

- Test material (4) (section 5)

- Call for Proposals preparation (10) (section 6)

- Exploration experiments (33) (section )

  - EE Summary (1) (section 7.1)

  - EE1 and related: Intra Prediction and mode coding (18) (section 7.2)

  - EE2 and related: Entropy coding (4) (section 7.3)

  - EE3 and related: Adaptive loop filter (10) (section 7.4)

- Non-EE technology proposals (15) (section 8)

- Coding of 360o video projection formats (6) (section 10)

- Complexity analysis (2) (section 11)

- Encoder optimization (1) (section 12)

- Metrics and evaluation criteria (2) (section 13)

- Joint meetings, plenary discussions, BoG reports (2), Summary of actions (section 14)

- Project planning (section 15)

- Output documents, AHGs (section 16)

# AHG reports (10)

These reports were discussed Wednesday 18 Oct. 0945–1230 (chaired by GJS).

JVET-H0001 JVET AHG report: Tool evaluation (AHG1) [M. Karczewicz, E. Alshina]

This document reports the work of the JVET ad hoc group on Tool evaluation (AHG1) between the 7h JVET meeting at Turin, Italy (13– 21 July 2017) and the 8th Meeting at Macau, China (18– 24 October 2017).

Altogether, 30+ e-mails related to AhG1 and EE activities were sent to the JVET reflector including EE tests scheduling and EE summary discussion.

Algorithms included into JEM7.0 are described in JVET-G1001. There is a list of tools below. Tools modified at the JVET-F meeting are marked as bold. The biggest change is addition of division-free bilateral filter after inverse transform.

JEM6.0 tools:

- Block structure

    - Larger Coding Tree Unit (up to 256x256) and transforms (up to 64x64)

    - Quadtree plus binary tree (QTBT) block structure

- Intra prediction improvements

    - 65 intra prediction directions

    - 4-tap interpolation filter for intra prediction

    - Boundary filter applied to other directions in addition to horizontal and vertical ones

    - Cross-component linear model (CCLM) prediction

    - Position dependent intra prediction combination (PDPC)  modified in JEM7.0

    - (Note: Adaptive reference sample smoothing is no longer included)

- Inter prediction improvements

    - Sub-PU level motion vector prediction

    - Locally adaptive motion vector resolution (AMVR)

    - 1/16 pel motion vector storage accuracy

    - Overlapped block motion compensation (OBMC)  modified in JEM7.0

    - Local illumination compensation (LIC)

    - Affine motion prediction

    - Pattern matched motion vector derivation

    - Bi-directional optical flow (BIO)  modified in JEM7.0

    - Decoder-Side Motion Vector Refinement (DMVR)

- Transform

    - Explicit multiple core transform

    - Mode dependent non-separable secondary transforms Signal dependent transform (SDT)  disabled by default

- In-loop filter

    - Bilateral filter

    - Adaptive loop filter (ALF)

    - Content adaptive clipping

- Enhanced CABAC design

    - Context model selection for transform coefficient levels

    - Multi-hypothesis probability estimation

    - Initialization for context models

Performance progress for JEM (HM-KTA) in terms of BD-rate gain vs. encoder time increase in random access test configuration is demonstrated on Figure below. Results are based on Software Development AHG reports. Some encoder run-time reduction is observed for JEM7.0 compared to JEM6.0, eventually encoder run time in random access test case is <10 time of HM. Encoder run time of JEM in all intra configuration is ×36 of HM (was ×63 for JEM6.0).

Screen content coding tools were enabled for HEVC at the last meeting for class F (screen content) which is optional (not included to the averaging) Due to this SCM 16.16 outperforms JEM in “all-intra” (19%) and random access (7%) configurations even JEM’s encoder is much slower.

|  | Encoding run time relatively to HEVC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | KTA1 | KTA2 | JEM1 | JEM2 | JEM3 | JEM4 | JEM5 | JEM6 | JEM7 |

Figure: The progress of JEM performance in RA test configuration.

JEM7.0 (8th meeting)

| Test configuration | BD-rate |  |  | Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. | Dec. |
| All Intra | −20% | −28% | −27% | 36 | 2 |
| Random Access | −29% | −35% | −34% | 10 | 7 |
| Low Delay-B | −22% | −28% | −29% | 9 | 7 |
| Low Delay-P | −26% | −31% | −32% | 7 | 5 |

JEM6.0 (7th meeting)

| Test configuration | BD-rate |  |  | Time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc. | Dec. |
| All Intra | −20% | −28% | −28% | 63 | 2 |
| Random Access | −29% | −36% | −35% | 12 | 10 |
| Low Delay-B | −22% | −28% | −29% | 10 | 8 |
| Low Delay-P | −26% | −31% | −32% | 7 | 5 |

Significant gain is observed in all three colour components. In random access testing, the highest gain over HEVC was observed for DaylightRoad test sequence (38.9%), lowest gain JEM shows for ToddlerFountain video (14.7% only).

At the 2nd JVET meeting, the Exploration Experiments practice was established. At the 7th JVET meeting, 3 EEs were created. For each new coding tool under consideration, a special SW branch was created. After implementation of each tool announcement via JVET reflector was done. For all 3 EEs input contribution for this meeting were submitted. A summary of exploration experiments is provided in JVET-H1011.

The number of technical contribution was observed. In total, 20 contributions on coding tools were identified in following categories:

- Entire codec (1)

- Intra (11)

- Inter (3)

- Transform (0)

- Entropy (2)

- In-loop filter (3)

There are some suggestions regarding testing methodology in contribution listed in the below.

| JVET-H0030 | BD-Rate/BD-PSNR Excel extensions | A.M. Tourapis, D. Singer, Y. Su, K. Mammou (Apple Inc) |
| --- | --- | --- |
| JVET-H0047 | Perceptually optimized QP adaptation and associated distortion measure | Sebastian Bosse, Christian Helmrich, Heiko Schwarz, Detlev Marpe, Thomas Wiegand |

Some discussion about balance between Luma and Chroma BD-rate gain happened in EE3.

| JVET-H0041 | EE3 Test1-6: Unified Adaptive Loop Filter for Luma and Chroma | J. An, J. Zheng (HiSilicon) |
| --- | --- | --- |
| JVET-H0045 | EE3: Test on Chroma QP offset | Sergey Ikonin, Roman Chernyak, Timophey Solovyev, Shan Liu (Huawei) |

The SSIM metric was part of AhG10 discussion.

The AHG recommended:

- Consider encoder complexity as one of the criteria when evaluating the tools. Encourage further encoder and decoder complexity reduction.

- Review all the related contribution.

- Continue the Exploration Experiments practice.

- Spend some time remaining in the CfP preparation process on more extensive discussion of objective metrics

JVET-H0002 JVET AHG report: JEM algorithm description editing (AHG2) [J. Chen, E. Alshina, J. Boyce]

This document reports the work of the JVET ad hoc group on JEM algorithm description editing (AHG2) between the 7th JVET meeting at Torino, IT (July 13–21, 2017) and the 8th meeting at Macao, CN (October 18–24, 2017).

Currently the document contains the algorithm description as well as encoding logic description for all new coding features in JEM6.0 beyond HEVC.

During the editing period, on top of JVET-F1001 Algorithm Description of Joint Exploration Test Model 6, the editors worked on the following two aspects to produce the final version of the JVET-G1001 Algorithm Description of Joint Exploration Test Model 7:

- Integrate the following adoptions, which change the encoding or decoding process, at the 7th JVET meeting

  - JVET-G0104: Removal of ARSS; PDPC always applied to planar mode and no PDPC for all other modes; MDIS is applied for a block with non-zero NSST index; and strong intra smoothing is disabled

  - JVET-G0082: Block-based design for Bi-directional optical flow (BIO)

- Editorial improvements by editors and solving of mismatch between text and software

  - Sub-block size derivation in affine motion compensation process

  - Miscellaneous editorial improvements

At the 7th JVET meeting, the intra prediction of the JEM was significantly simplified. With the removal of ARSS, and PDPC applied only for planar mode, the encoder running time of the JEM for all-intra configurations is greatly reduced. BIO is modified in a way that the motion vector offset is derived for a group of 4x4 luma samples instead of each luma sample, and it is not applied during the OBMC process.

The AHG recommended to:

- Continue to edit the Algorithm Description of Joint Exploration Test Model document to ensure that all agreed elements of JEM are described

- Continue to improve the editorial quality of the Algorithm Description of Joint Exploration Test Model document and address issues relating to mismatches between software and text.

- Identify and improve algorithm description for critically important parts of JEM design for better understanding of complexity.

JVET-H0003 JVET AHG report: JEM software development (AHG3) [X. Li, K. Sühring]

This report summarizes the activities of AhG3 on JEM software development that took place between the 7th and 8th JVET meetings.

Software development was continued based on the HM-16.6-JEM-6.1 version. A branch was created in the software repository to implement the JEM-7 tools based on the decisions noted in section 15.4 in the notes of 7th JVET meeting. All integrated tools were included in macros to highlight the changes in the software related to that specific tool.

HM-16.6-JEM-7.0 was released on Aug. 21st, 2017.

Several branches were created for exploration experiments on top of HM-16.6-JEM-7.0. Note that these branches are maintained by the proponents of exploration experiments.

The JEM software is developed using a Subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/

The implementation of JEM-7 tools has been performed on the branch

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/HM-16.6-JEM-6.1-dev

The released version of HM-16.6-JEM-7.0 can be found at

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.0

The branches of exploration experiments can be found at

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates

The performance of HM-16.6-JEM-7.0 over HM-16.6-JEM-6.0 and HM-16.16 under test conditions defined in JVET-G1010 is summarized as follows. As agreed in 7th JVET meeting, HM-16.15-SCM-8.4 is used as HM anchor for class F sequences. Note that 10-bit internal bit depth was used for HM-16.15-SCM-8.4 when testing class F.

It is noted that in contrast to the table above, the previous meeting had made the recommendation of only reporting one digit after the decimal point. In the context of the presentation of JVET-H0057, the issue of only reporting one digit after the decimal point was again discussed. The current Excel template uses rounding which makes 0.05% appear as 0.1%, etc. It was initially agreed that this should be modified as follows: Compute BD rate averages over all sequences of each class and globally with high precision, but in the summary show truncated average values, i.e. anything in the range −0.09 … +0.09 goes to 0. However, this decision was later revised (Sunday morning), and it was agreed that in future reports (also in CfP templates) we should go back to using two digits after the decimal point, as this gives more information.

The JEM bug tracker is located at

https://hevc.hhi.fraunhofer.de/trac/jem

It uses the same accounts as the HM software bug tracker. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Please file all issues related to JEM into the bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

The AHG recommended

- To continue software development on the HM-16.6 based version

- Encourage people to test JEM software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bitstreams/test cases that trigger bugs in JEM.

- Make decision on whether to clean QTBT macro in JEM.

Regarding the question of whether to remove the QTBT macro (and 64-direction intra), there have been problems maintaining the path with that feature disabled. The software sometimes does not function with QTBT off, and there is a lot of code associated with having both modes. There were mixed opinions about that, where some people wanted to “clean” the software, while others thought it better to try to maintain both modes as much as possible – e.g., to enable study and serve as a historical reference.

It was remarked that a contribution JVET-H0084 provides other software, and that software does function with QTBT off. However, it was decided that no action would be taken from JVET-H0084.

JEM is an exploration software, and for the purpose of investigating tools by people who want to use it for making proposals, so it is desirable to keep it configurable in various ways.

A new version was reportedly planned shortly after the meeting, containing fixing of known bugs (not related to QTBT). The benefit of generating another branch with the QTBT macro disabled is not obvious.

It is not known if there are tools that cause problems with the QTBT macro disabled. It is highly encouraged to report such problems if they are found.

JVET-H0004 JVET AHG report: Test material (AHG4) [T. Suzuki, V. Baroncini, J. Chen, J. Boyce, A. Norkin]

The test sequences used for the CfP (JVET-G1002) are available on ftp://jvet@ftp.ient.rwth-aachen.de in directory “/jvet-cfp” (accredited members and potential CfP respondents may contact the JCT-VC chairs for login information).

HM/JEM anchors (defined in JVET-G1002) were generated and verified by at least one cross checker.

HM anchors:

ftp://jvet@ftp.ient.rwth-aachen.de/jvet-cfp/anchors-hm

JEM anchors:

ftp://jvet@ftp.ient.rwth-aachen.de/jvet-cfp/anchors-jem

Relevant contributions to this meeting were noted as follows. (Most of the contributions are submitted for AHG7 and 8)

- CfP anchor generation

  - JVET-H0060 “AHG4: SDR anchor generation for the Draft Joint Call for Proposals", H.-C. Chuang, J. Chen, M. Karczewicz (Qualcomm), K. Choi, E. Alshina (Samsung).

  - JVET-H0068 “AHG7: HLG rate points for CfP anchor generation", S. Iwamura, S. Nemoto, A. Ichigaya(NHK).

- New test materials

  - JVET-H0021 “AHG8: Test Sequences for Spherical Video Coding from GoPro", Adeel Abbas (GoPro).

  - JVET-H0022 “AHG8: New Sequences for Virtual Reality Video Coding from InterDigital", Y. He, P. Hanhart, E. Asbun, Y. He, Y. Ye.

- Study of the current test materials

  - JVET-H0038 “AHG7: Test Sequences of 4K Hybrid Log-Gamma", T. Tsukuba, M. Ikeda, T. Suzuki (Sony).

  - JVET-H0048 “AhG8: Dynamic viewport based subjective evaluation of Balboa and Landing", P. Hanhart, Y. He, Y. Ye (InterDigital).

  - JVET-H0068 “AHG7: HLG rate points for CfP anchor generation”, S. Iwamura, S. Nemoto, A. Ichigaya (NHK).

  - JVET-H0083 “AHG7: Cross-check of test sequences of 4K Hybrid Log-Gamma transfer characteristics (JVET-H0038)”, S. Nemoto, S. Iwamura, A. Ichigaya (NHK).

- New BD-rate/PSNR calculation

  - JVET-H0030 “BD-Rate/BD-PSNR Excel extensions”, A.M. Tourapis, D. Singer, Y. Su, K. Mammou (Apple Inc.).

The AHG recommended:

- To review all related contributions.

- To evaluate visual quality of HM/JEM anchors.

- To discuss further actions to select new test materials, and to revise the current test materials for CfP and to discuss the test conditions of objective quality evaluation for CfP.

In the discussion, it was noted that for the CfP we need to finalize the selection of the test sequences and anchors (including bit rates and objective-only testing sequences and conditions). For cases where candidate anchor encodings are already available, it may be better to use those than to delay availability of anchors in order to use a more recent JEM or HM version.

JVET-H0005 JVET AHG report: Memory bandwidth consumption of coding tools (AHG5) [X. Li, E. Alshina, R. Hashimoto, T. Ikai, H. Yang]

This document summarizes activities of AhG on memory bandwidth consumption of coding tools between the 7th and the 8th JVET meetings.

Two related contributions were noted:

- JVET-H0039 AHG5: On worst case memory bandwidth [T. Ikai (Sharp), R. Hashimoto, S. Mochizuki (Renesas)]

- JVET-H0043 AHG5: How to measure memory bandwidth [R. Hashimoto, S. Mochizuki (Renesas), T. Ikai (Sharp)]

The AHG recommended to review the related contributions.

JVET-H0006 JVET AHG Report: 360 video conversion software development (AHG6) [Y. He, V. Zakharchenko]

This document summarizes activities on 360-degree video content conversion software development between the 7th and the 8th JVET meetings.

The 360Lib-4.0 software package integrated all adoptions about projection format and metrics calculation:

- Metrics:

  - (1)	WS-PSNR calculation for ACP (JVET-G0088);

  - (2)	Add codec S-PSNR-NN

- Projection formats and frame packing:

  - (3)	Adjusted equal-area projection (AEP) (JVET-G0051);

  - (4)	Equi-angular cubemap (EAC) projection (JVET-G0056);

  - (5)	Equatorial cylindrical projection (ECP) with padding (JVET-G0074);

  - (6)	EAP-based segmented sphere projection (SSP) with padding (JVET-G0097);

  - (7)	Padded ERP (PERP) projection format (JVET-G0098);

- Software:

  - (8)	Fix for bug ticket #51, #52, #54

360Lib-4.0 related releases and bug fixes were summarized as follows:

- 360Lib-4.0rc1 with support of HM-16.16 and JEM-7.0rc1 was released on Aug. 4th, 2017;

- 360Lib-4.0 with support of HM-16.16 and JEM-7.0 was released on Aug. 23rd, 2017;

- 360Lib-4.0 based HM-16.16 and JEM-7.0 testing results was released on Aug. 23rd, 2017;

- Ticket #56 was submitted after 360Lib-4.0 release. It only affected codec WS-PSNR calculation for ECP projection format. The fix was provided in the development branch (360Lib-4.1-dev).

The 360Lib software is developed using a Subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

The released version of 360Lib-4.0 can be found at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-4.0/

360Lib-4.0 testing results can be found at:

ftp.ient.rwth-aachen.de/testresults/360Lib-4.0

360Lib bug tracker

https://hevc.hhi.fraunhofer.de/trac/jem/newticket?component=360Lib

360Lib-4.0 results are shown as follows

The table below lists the HM-16.16 based coding performance with different projection formats according to 360° video CTC (JVET-G1030) compared to PERP coding.

HM-16.16-360Lib-4.0 testing (HM PERP coding as anchor)

| Projection forma | E2E WS-PSNR for all sequences |  |  | E2E WS-PSNR for 8K sequences |  |  | E2E WS-PSNR for 6K sequences |  |  | E2E WS-PSNR for 4K sequences |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| CMP | −2.4% | −0.4% | −0.5% | −5.1% | −3.1% | −3.6% | 0.0% | 4.1% | 4.6% | 0.5% | −1.5% | −1.2% |
| AEP | −4.3% | −3.0% | −3.1% | −4.7% | −3.5% | −3.3% | −3.7% | −1.5% | −2.0% | −4.3% | −4.8% | −4.9% |
| COHP | −1.4% | 2.1% | 1.2% | −5.0% | −0.1% | −1.1% | 3.6% | 6.9% | 5.9% | −0.4% | −1.0% | −1.2% |
| CISP | −3.3% | 1.2% | 0.4% | −7.5% | −1.5% | −2.6% | 2.2% | 6.8% | 6.2% | −1.9% | −1.9% | −2.2% |
| EAP−SSP | −9.5% | −5.4% | −5.7% | −11.9% | −6.4% | −6.5% | −7.0% | −3.0% | −3.8% | −7.6% | −6.9% | −7.2% |
| ACP | −9.9% | −5.3% | −5.5% | −12.4% | −6.3% | −6.4% | −7.6% | −3.3% | −3.9% | −6.7% | −6.3% | −6.3% |
| RSP | −8.3% | −4.3% | −4.6% | −11.8% | −6.2% | −6.5% | −4.6% | −1.3% | −1.9% | −5.3% | −4.9% | −4.4% |
| ECP | −9.2% | −5.1% | −5.7% | −12.3% | −6.8% | −7.2% | −5.2% | −1.3% | −2.6% | −7.8% | −7.6% | −7.4% |
| EAC | −9.9% | −5.3% | −5.6% | −12.5% | −6.4% | −6.5% | −8.0% | −3.4% | −3.9% | −6.1% | −6.2% | −6.1% |

The table below compares the JEM-7.0 PERP coding with HM-16.16 PERP coding. JEM has the higher BD rate saving for 6K sequences (−28.1%) compared to 8K (−20.8%) and 4K (−21.3%) sequences because 6K sequences were captured by moving cameras.

JEM-PERP vs HM-PERP coding (HM PERP coding as anchor)

| anchor: HM-PERP | SPSNR-NN (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| test: JEM-PERP | Y | U | V | Y | U | V |
| Trolley | −15.0% | −30.9% | −32.7% | −14.9% | −30.8% | −32.8% |
| GasLamp | −20.3% | −44.6% | −41.5% | −20.2% | −44.6% | −41.5% |
| Skateboarding_in_lot | −24.4% | −33.0% | −42.9% | −24.4% | −33.0% | −42.9% |
| Chairlift | −29.8% | −48.6% | −45.6% | −29.9% | −48.6% | −45.7% |
| KiteFlite | −16.5% | −35.3% | −38.6% | −16.4% | −35.2% | −38.5% |
| Harbor | −19.0% | −41.7% | −42.2% | −19.0% | −41.7% | −42.2% |
| Balboa | −33.1% | −36.5% | −40.9% | −33.1% | −36.5% | −40.9% |
| Broadway | −32.9% | −38.5% | −41.5% | −32.9% | −38.5% | −41.5% |
| Landing | −23.0% | −15.5% | −17.9% | −23.0% | −15.5% | −17.9% |
| BranCastle | −23.3% | −15.5% | −31.6% | −23.3% | −15.4% | −31.5% |
| PoleVault | −17.7% | −19.0% | −20.4% | −17.8% | −19.0% | −20.6% |
| AerialCity | −24.8% | −44.7% | −31.0% | −24.9% | −44.8% | −30.9% |
| Overall | −23.3% | −33.7% | −35.6% | −23.3% | −33.6% | −35.6% |
| 8K | −20.8% | −39.0% | −40.6% | −20.8% | −39.0% | −40.6% |
| 6K | −28.1% | −26.5% | −33.0% | −28.1% | −26.5% | −32.9% |
| 4K | −21.3% | −31.9% | −25.7% | −21.3% | −31.9% | −25.7% |

The AHG recommended

to continue software development of the 360Lib software package.

JVET-H0007 JVET AhG report: JEM coding of HDR/WCG material (AHG7) [A. Segall, E. François, D. Rusanovskyy]

This document summarizes the activity of AHG7: JEM Coding of HDR/WCG Material between the 7th meeting in Torino, IT (13–21 July 2017) and the 8th meeting in Macao, CN (18–25 Oct. 2017).

Accomplishments by the AhG included:

- HM and JEM anchor bit-streams for the draft CfP HDR category were prepared and made available

- HM and JEM rate points and bit-streams for the HLG test material were prepared and made available

The AHG delivered both HM16.16 and JEM7.0 HDR CfP anchors as part of its work. The HM16.16 anchors were delivered on September 4, 2017, and made available at ftp://ftp.ient.rwth-aachen.de/jvet-cfp/anchors-hm/HDR. The JEM 7.0 anchors were delivered on September 28, 2017, and made available at ftp://ftp.ient.rwth-aachen.de/jvet-cfp/anchors-jem/HDR.

The performance of the anchors is summarized below. More detailed information is available in the Excel spreadsheet included in the AHG report.

|  | JEM7.0 over HM16.16 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPSNR Y | wPSNR U | wPSNR V | PSNR Y | PSNR U | PSNR V |
| Market | −38.5% | −24.9% | −25.5% | −45.4% | −40.2% | −23.8% | −38.3% | −31.7% |
| ShowGirl2 | −29.4% | −26.7% | −27.9% | −64.1% | −33.7% | −26.0% | −60.4% | −29.1% |
| Hurdles | −49.3% | −35.9% | −34.5% | −59.6% | −54.4% | −31.4% | −54.6% | −50.2% |
| Starting | −35.2% | −31.4% | −30.8% | −36.5% | −37.2% | −29.1% | −33.8% | −23.0% |
| Cosmos1 | −30.6% | −24.2% | −24.3% | −66.1% | −79.5% | −22.8% | −65.3% | −79.3% |
| Overall | −36.6% | −28.6% | −28.6% | −54.3% | −49.0% | −26.6% | −50.5% | −42.6% |

It was noted during the creation of the anchors that the Summary worksheet should be reviewed prior to inclusion in the CfP.

HDR metrics for the HLG sequences included in the HDR CTC at the Torino meeting (H09_FlyingBirds and H10_SunsetBeach) were delivered on September 13, 2017, and made available on the JVET reflector. A cross check of these results was reported on September 20, 2017.

The results are also available as an attached Excel spreadsheet with this report.

At the previous meeting, it was agreed to further study additional HLG sequences for potential inclusion in the HDR CfP and/or CTC evaluations. During the interim period, bitstreams and candidate rate points were made available. This was reported on the JVET reflector on October 11, 2017, and made available at ftp://ftp.ient.rwth-aachen.de/ahg/candidates/JVET-H0038/.

It had been expected that Sim2 monitors would be used for the CfP response evaluation.

There was some interest expressed on how to view HDR content on the Sony BVM X300. At recent meetings, we have had this type of monitor and not had a Sim2, and had performed a tone mapping to adjust the dynamic range of the sequences for viewing on the BVM X300 display (to avoid the clipping that would otherwise have occurred when displaying such content on that display). Specifically, this discussion considered the problem of rendering content with a peak brightness up to 4,000 cd/m2 and a spatial resolution of 1920x1080 on a display capable of rendering a peak brightness of 1,000 cd/m2 and a spatial resolution of 3840x2160. The BVM X300 has these display characteristics.

It was further commented that the candidate HLG sequences may not be suitable for display on a Sim2, because they have higher resolution than what the Sim2 is capable of displaying. It is thus necessary to determine how the sequences will be displayed if HLG sequences are used in the CfP. The BVM X300 display is suggested to be appropriate as a way to display the available HLG sequences.

As it is expected that the activity is of interest to many AhG members, it was recommended to establish a BoG at the Macao meeting to review and discuss these issues. In the initial discussion of the AHG report, it was suggested that using a Sim2 for the PQ sequences and a BVM X300 for the HLG sequences in the CfP evaluation may be an adequate approach. JVET-H0095 contains a report of such a BoG.

There were 5 contributions identified as related to HDR video coding:

- JVET-H0038 AHG7: Test Sequences of 4K Hybrid Log-Gamma [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)]

- JVET-H0068 AHG7: HLG rate points for CfP anchor generation [S. Iwamura, S. Nemoto, A. Ichigaya(NHK)]

- JVET-H0083 AHG7: Cross-check of test sequences of 4K Hybrid Log-Gamma transfer characteristics (JVET-H0038) [S. Nemoto, S. Iwamura, A. Ichigaya (NHK)]

- JVET-H0055 Analysis of the HDR Objective Metrics Utility in Draft CfP on FVC [P. Yin, T. Chen, W. Husak, F. Pu, T. Lu (Dolby)]

- JVET-H0082 Comments on HDR category test conditions of draft CfP on video compression beyond HEVC [D. Rusanovskyy, M. Karczewicz (Qualcomm Inc.), E. François, M. Kerdranvat (Technicolor)]

The AHG recommended the following:

- Review all input contributions

- Review new HDR test material and rate point information; discuss if sequences and configurations for the HDR CTC or CfP should be modified

- Review and discuss results for visualizing HDR content on the BVM X300

- Prepare the HDR video section of the Call for Proposal

JVET-H0008 JVET AHG report: 360º video coding tools and test conditions (AHG8) [J. Boyce, A. Abbas, E. Alshina, G. van der Auwera, Y. Ye]

This document summarizes the activity of AHG8: 360º video coding tools and test conditions between the 7th meeting in Torino, IT (13–21 July 2017) and the 8th meeting in Macao, CN, 18–24 Oct. 2017.

There were 7 contributions noted to be related to 360º video coding, which are classified below

- Projection formats and padding

  - JVET-H0056 AHG8: An Update on RSP Projection [	Adeel Abbas (GoPro)]

- Test Conditions and Evaluation

  - JVET-H0048 AhG8: Dynamic viewport based subjective evaluation of Balboa and Landing [	P. Hanhart, Y. He, Y. Ye (InterDigital)]

  - JVET-H0050 AhG8: Measurement of User Exploration Behavior for Omnidirectional (360°) Videos with Head Mounted Display [	A. Singla, S. Fremerey, A. Raake, P. List, B. Feiten]

  - JVET-H0049 AhG8: On viewport size and field of view	 [P. Hanhart, Y. He, Y. Ye (InterDigital)]

  - JVET-H0035 AHG8: The guidance of WS-PSNR weights derivation for different formats [Y. Sun, L. Yu (Zhejiang University)]

- Content

  - JVET-H0021 AHG8: Test Sequences for Spherical Video Coding from GoPro	 [A. Abbas (GoPro)]

  - JVET-H0022 AHG8: New Sequences for Virtual Reality Video Coding from InterDigital	 [Y. He, P. Hanhart, E. Asbun, Y. He, Y. Ye]

The AHG recommended the following:

- Review input contributions

- Review new 360° video test material, and consider adding or replacing test sequences for common test conditions and/or CfP

- Refine common test conditions for 360° video, including objective metrics and viewports

- Finalize the 360° video section of the Call for Proposals

JVET-H0009 JVET AHG report: 4:4:4 support in JEM (AHG9) [A. M. Tourapis, X. Li]

This AHG report was discussed verbally prior to upload.

No activity was reported. 4:4:4 encoding and decoding is verbally reported to be working for intra, but not inter.

It was remarked that no progress was also apparent on integration of screen content coding tools into the JEM either.

JVET-H0010 JVET AHG report: Denoising and adaptive quantization [R. Sjöberg, E. Alshina, S. Ikonin, A. Norkin, T. Wiegand]

The document summarizes the activities of the AHG on Denoising and adaptive quantization between the 7th and the 8th JVET meetings.

Two related contributions were noted

- H0047	 Perceptually optimized QP adaptation and associated distortion measure:

- This document describes an adaptive quantization algorithm and a corresponding distortion metric.

- H0055 	Analysis of the HDR Objective Metrics Utility in Draft CfP on FVC:

- This contribution reviews the HDR objective metrics listed in the draft CfP and compares the correlation between subjective and objective results done previously.

An investigation of denoising filters was done by Ericsson and results were shared on the e-mail reflector.

Four encoding settings for AI, RA, LDB and LDP were run:

- HM 16.15 on anchor test set

- JEM 6.0 on anchor test set

- HM 16.15 on denoised test set

- JEM 6.0 on denoised test set

The anchor test set consist of the SDR CTC sequences. The denoised test set consist of the same sequences but after applying denoising. The Avisynth tool MC_Spudsmod was used with the following settings: (frames=4, strength=1, sharpp=0, chroma=true). Further information can be found in the JVET-G0073 contribution from Torino.

The HM encodings were done using the HM-16.6-JEM-6.0\cfg\hm-16.13 configurations. Those give identical results as the HM anchor reported in JEM4-vs-JEM5-gcc62.xlsm. The BD-rate savings of JEM over HM were calculated. When comparing encoding 3 and 4 the denoised source was used for computing PSNR and MS-SSIM values.

The results can be found in tables in the AHG report. Full results can be found at http://phenix.int-evry.fr/jvet/doc_end_user/documents/7_Torino/comments/comment-2-JEM-vs-HEVC-on-denoised-sequences.zip. The differences in JEM-over-HM BD-rate savings between coding original sequences and coding denoised sequences were reported to be fairly small. The results are tabulated in the AHG report.

In the review of the AHG report, it was noted that the processing done for these experiments involved an 8-bit “bottleneck”, so that a readily available software package could be used as-is.

A suggestion to include anchors outside of competition to the CfP subjective tests for SDR-HD1 or SDR-UHD1 using adaptive quantization and denoising was made on the reflector. The motivation for that was that it would quantify the MOS improvements from using adaptive QP, denoising and renoising. Also, it would provide MOS scores from encoders that are rather different compared to the anchors which can be used for a later evaluation of different quality metrics.

No conclusion on the suggestion was reached by the AHG.

In the draft CfP, denoising is prohibited in all cases. QP adaptation related to average local brightness is allowed for HDR content (and is used with a specific brightness adaptation rule in the anchors). QP adaptation as a function of geometric position (not customized to the content) is allowed for the 360° cases.

# Analysis, development and improvement of JEM (1)

Contributions in this category were discussed Sunday 22 Oct. 1100–1130 (chaired by JRO & GJS).

JVET-H0084 NextSoftware: An alternative implementation the Joint Exploration Model (JEM) [Adam Wieckowski, Tobias Hinz, Valeri George, Jens Brandenburg, Jackie Ma, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Wiegand (Fraunhofer HHI)] [late]

This document presents an alternative implementation of the Joint Exploration Model (JEM): The "NextSoftware" codebase. The main objectives during the development of this software were reported to have been code modernization, unification and simplification. This contribution was reportedly intended to provide an alternative implementation to JEM for the development of future coding tools. In comparison to the JEM software, the authors report the following advantages: (1) Better code readability, (2) Easier integration of new coding tools, (3) A design by which all coding tools can be enabled/disabled at run time (per configuration file or command line parameters), no modification of compiler flags is required, (4) Fast encoder control yielding a 25% encoder speed-up for random access, (5) Improved debugging capabilities, (6) Integration of perceptually optimized QP adaptation and the associated WPSNR measure (JVET-H0047).

The s

oftware licensing status was requested to be clarified.

The encoding time is faster (75%) in RA fast mode, almost identical in full mode; and the decoding time similar or slightly higher as for the JEM

Encoding results are not identical to JEM; about 0.5% worse in fast mode, almost identical in full mode.

This document presents an alternative implementation of the Joint Exploration Model (JEM): The “NextSoftware” implementation. The main objectives during the development of this software were reported to be code modernization, unification and simplification. This contribution is suggested to provide an alternative to the JEM implementation for the development of future coding tools. In comparison to the JEM software, the authors claim the following advantages: 1) Better code readability, 2) Easier integration of new coding tools, 3) A design by which all coding tools can be enabled/disabled at run time (per configuration file or command line parameters), with no modification of compiler flags required, 4) Fast encoder control yielding a 25% encoder speed-up for the random access configuration, 5) Improved debugging capabilities, 6) Integration of perceptually optimized QP adaptation and the associated WPSNR measure (JVET-H0047).

It also supports HEVC encoding and decoding, and can be configured to emulate the HM behaviour.

There may be some aspects that are slightly different from the JEM. A few coding tools are not included: some method of mode signalling for 65 intra modes (JVET-B0051), adaptive reference sample smoothing (COM16-C983 RSAF), enhanced motion cost estimation (JVET-D0123) and KLT (VCEG-AZ08).

For about the same speed, there is a reported benefit of about 0.3% in BD BR. For about the same coding performance, there is a speed-up of about 26% for the random access configuration.

4:2:2 and 4:4:4 are reportedly supported.

The contribution document provided a link to the software. It was reported that the copyright header for the software may need some adjustment to be appropriate for our use. The proponent said this would be fixed.

The software was derived from the HM.

It was commented that a different name may be appropriate, as the same name had been used for other recently reported software (see the JVET C meeting).

The proponent suggested using this software for the study of the WPSNR scheme discussed in JVET-H0047, since it already includes that capability. This seemed appropriate, and is encouraged.

The software may be very helpful as a basis for responses to the CfP and perhaps for other future uses as well – e.g., in further work beyond the CfP (although it has not yet been studied by our group). Study and experimentation with this software is encouraged.

No specific group action was taken or requested at this stage; if useful, it could be used in AHG10 study.

# Test material (4)

Contributions in this category were discussed Friday 20 Oct. 0900–1200 (chaired by JRO).

JVET-H0021 AHG8: Test Sequences for Spherical Video Coding from GoPro [A. Abbas (GoPro)]

This contribution provides new versions of two 360 test sequences namely BranCastle and Landing. The new versions have improved image quality over previously shared versions in JVET-G0147, and a minor bug-fix over sequences that were shared on September 19th. Furthermore, a new video clip called SkateboardAtBridge is shared.

It was suggested to investigate the two new versions of Landing, and decide which to use for replacing the old version for CfP after viewing them.

Bran Castle is still not a good sequence for subjective tests due to motion jerkiness. The new version shows higher rate at high to medium QP, and approximately the same rate at QP22. This is likely due to the improved texture. It was agreed to replace the old version by this, and it should get a new name e.g. BranCastle2.

It was agreed to make SkateboardAtBridge available as a candidate for further investigation to be potentially included into CTC in the future (in the development of a new standard).

JVET-H0022 AHG8: New Sequences for Virtual Reality Video Coding from InterDigital [Y. He, P. Hanhart, E. Asbun, Y. He, Y. Ye (InterDigital)]

This contribution makes two 360-degree video sequences available to the JVET. The video sequences are provided in 8K (8192x4096) resolution.

The new sequences have approximately the same content as previous ones but use moving cameras: they are “HarbourBiking” and “KiteFliteWalking”.

They have improved colours compared to the old version.

Some visible stitching artefacts are noted in cases of close objects.

KiteFliteWalking has some motion jerkiness, but less than BranCastle.

It was agreed to make both sequences available as candidate for further investigation to be potentially included into CTC in the future (in the development of a new standard).

Informative viewing to get an impression about the visibility of compression at QP32/37 for the two sequences and SkateborardAtBridge was requested be organized later in the week.

JVET-H0038 AHG7: Test Sequences of 4K Hybrid Log-Gamma [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)]

This was presented in the BoG JVET-H0095 on HDR and WCG material on 19 October.

This contribution provides (i) candidate rate points of 4K Hybrid Log-Gamma test sequences (DayStreet, PeopleInShoppingCenter) that are matched between both HM16.16 and JEM-7.0, and (ii) information on dynamic range and colour gamut of those sequences. It was reported that this information was requested by the BoG on HDR and WCG material that met during the Torino meeting

Six rate points were studied and proposed. Additionally, a study of the sequence characteristics was provided. A visualization of sample locations that correspond to values outside of the BT.709 gamut was shown.

It was reported that a peak value of 1,000 cd/m2 was used to calculate the PSNR-L100 and deltaE100 metrics, as was done in JVET-H0068. (See the discussion of this issue in the notes on JVET-H0068.)

One participant noted that the gain of JEM 7.0 relative to the HM is lower for the sequences described in this document than the sequences in JVET-H0068. It was suggested that this may mean that the sequences are not challenging for compression.

It was suggested to have a viewing session at 1700 on 19 October. The goal is two-fold. First to determine what sequences are candidates for inclusion in the CfP; second what are appropriate target rate points for those sequences. This was agreed.

See further conclusions under JVET-H0095.

JVET-H0083 AHG7: Cross-check of test sequences of 4K Hybrid Log-Gamma transfer characteristics (JVET-H0038) [S. Nemoto, S. Iwamura, A. Ichigaya (NHK)]

This was reviewed in the BoG JVET-H0095.

# Call for Proposals preparation (10)

## General discussions (1)

JVET-H0082 Comments on HDR category test conditions of draft CfP on video compression beyond HEVC [D. Rusanovskyy, M. Karczewicz (Qualcomm Inc.), E. François, M. Kerdranvat (Technicolor)] [late]

This contribution was discussed Wednesday 18 October 1700 (chaired by GJS).

This contribution presents comments related to HDR category of the draft CfP document JVET-G1002, and several detailed proposals for evolving respective sections of the CfP document.

- In comments on the HDR test set, section 4.2.1 of G1002, it is suggested to preserve the native high peak brightness BT.2100 PQ content (peak luminance up-to 4000cd/m2) at its original quality, and thus avoid its re-grading with the goal of limiting it to 1000 cd/m2. (Adapting content to 1000 cd/m2, if needed, could be something that occurs either before encoding or after decoding.)

- It is proposed to establish an AhG to address the rendering issue of such content for the evaluation tests. It is also suggested to extend the HDR test set in CfP with HLG test sequences. Comments are also provided on the HDR coding conditions, section 4.2.2 of G1002.

- It is suggested to allow CfP responses to the HDR category to use certain type of pre- and post-processing, limiting these to colour volume transforms conducted on the input video data representation.

It was remarked that 4000 cd/m2 content is becoming more common, and non-mobile devices are expected to continue to increase in dynamic range, while mobile devices may have more limited range for some time to come. It is expected that a Sim2 (or equivalent) will be used for the testing of the PQ HD content.

Regarding the third aspect, in spirit we are willing to allow out-of-loop remapping to take place, but need to work out exactly what to say about that for the CfP.

A BoG (coordinated by A. Segall) was established.

- to select HLG sequences

- to work out exactly what to say about out-of-loop remapping for the CfP

- to review, but not for planned action in relation to the CfP, experiment results of tone mapping

- to conduct an initial review of JVET-H0055 objective metrics for HDR

This contribution was further discussed in the BoG for HDR and WCG content on 19 October at 1506 and again on 20 October at 0900.

The group identified the following concerns that should be considered in the text. The concerns were:

- Do we allow a spatially varying colour volume transform?

- Do we allow temporal variation?

- Question about bit-rate allocation?

- It would be good to better define the meaning of colour volume transform.

- It should be considered if the constraint allows denoising.

- Discuss of 4:2:0 sample domain statement.

It was commented by a participant that the limitation to the YCbCr sample domain may be unnecessarily limiting.

The group then edited the text in an attempt to address these issues. The result of this editing provided the following constraint.

A colour volume transform in the YCbCr 4:2:0 sample domain may be performed on the input video signal prior to coding, and an inverse of the colour volume transform may be performed on the signal after decoding. A colour volume transform modifies a luma or chroma sample at a relative location in the picture only based on the value of the luma or chroma sample and on the value of the other component samples located at the same or closest relative location. The colour volume transform should be static within a frame and sequence. The use of a colour volume transform for the purpose of rate allocation or denoising is discouraged. If a colour volume transform is used, a description of the transform shall be provided in the descriptive document submission. Respondents are additionally asked to provide descriptions of any multi-pass processing and any dynamic change in the colour volume transform.

The BoG recommended to include the constraint in the CfP, and this was confirmed by the JVET plenary.

## Anchor preparation (3)

JVET-H0060 AHG4: SDR anchor generation for the Draft Joint Call for Proposals [H.-C. Chuang, J. Chen, M. Karczewicz (Qualcomm), K. Choi, E. Alshina (Samsung)]

In this document, information regarding the SDR anchor generation for the Draft Joint Call for Proposals is described. On the selected five UHD sequences (FoodMarket4, CatRobot1, DaylightRoad2, ParkRunning3, CampfireParty2, namely Class SDR-UHD1) and five HD sequences (BQTerrace, RitualDance, MarketPlace, BasketballDrive, Cactus, namely Class SDR-HD1), encoding results and comparison of coding performance between HM-16.16 and JEM-7.0 anchors are reported. The matched bit rates of all sequences are less than 1% from the target bit rate, and the averaged absolute deviation from the target bit rate was 0.2% and 0.0% using Constraint Set 1 and Constraint Set 2, respectively.

This was discussed Thursday morning at 11:15.

It was clarified that the anchors should not exceed target bit rates (same as condition for proposals).

It was clarified to use JEM7 anchors in the CfP, such that they can be made available to prospective proponents as early as possible.

Excel sheets for the CfP were prepared by Jianle Chen (SDR), Yuwen He (for 360°, represented by Philippe Hanhart) and Edouard François (for HDR). Side activity was conducted to unify the three Excel sheets and make them available as input document.

Configuration files shall also be made available for all test cases.

Subsequently, the CfP was discussed.

It was further decided that no additional test cases for objective measurements will be defined in the CfP. This was anyway only planned for SDR, where the CfP already has a sufficient variety of test material.

In a follow-up discussion (in the context of presenting JVET-H0030) it was decided to add another, higher rate point for HM, such that it would be possible to compute reasonable BD-rate numbers. See further notes in the section discussing JVET-H0030.

It was further clarified that this does not intend to change the current CTC, and new CTC will have to be established after the CfP anyway.

For 360, Philippe Hanhart prepared viewing sessions. Rate points for Landing and Balboa need to be confirmed.

New sequences were proposed as replacements for Harbour and KiteFlite (with moving cameras) in JVET-H0022. However, encodings only exist for the CTC; these should not be used in the CfP.

A report on anchor generation for 360° video will be provided. It was verbally reported that some of the anchors exceed the target rates, which need to be updated.

If possible, all 6 sequences should be kept in the CFP (see further notes on this under JVET-H0022).

One expert suggested to include MountainBay2 (which was in Draft CfP as a placeholder for objective testing). However, this would mean that it needs to replace one other sequence. Further, though some subjective viewing was done in the last meeting, rate points for this sequence need further clarification.

Further discussion and update was performed Sat. 21st 1130–1300, as well as in the context of presenting the BoG report JVET-H0095. Various text improvements were made without controversial discussion. The reporting template for the SDR test cases was also presented.

It was decided that the templates should compute two comparisons (against the HM and against the JEM). This has the advantage that another anchor exists that is closer to the expected proposal performance, such that mutual comparison of BD performance differences between proposals becomes more reasonable.

Comparison of encoding/decoding time is only mandatory with the HM as reference, and is optional for the JEM.

A template had been made available as input document JVET-H0100, and the final version (3 sheets for SDR, HDR, 360) is to be included with CfP. It was later reported that the HDR and 360° versions are making progress and no specific problems were foreseen.

JVET-H0100 Draft excel template for call for proposals [J. Chen, E. François, P. Hanhart]

This was reviewed and approved in the context of final approval of the CfP (see output doc JVET-H1003).

JVET-H0093 AHG8: 360 anchor generation for the Draft Joint Call for Proposals [Y. He, Y. Ye (InterDigital), C. Pujara, K. Choi (Samsung)]

This document provides information regarding the 360-degree video anchor generation for the Draft Joint Call for Proposals. The matched bit rates of all 6 sequences were within 1% from the target bit rate, and the averaged deviation from the target bit rate is −0.02% using Constraint Set 1 for HM anchors and 0.16% using Constraint Set 1 for JEM anchors.

No detailed presentation was needed.

About half of the anchor bitstreams were slightly exceeding the target rate points and needed to be newly generated.

## Definition of test cases (5)

JVET-H0068 AHG7: HLG rate points for CfP anchor generation [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

This was presented in the BoG JVET-H0095 on HDR and WCG material on 19 October at 1420.

This contribution proposes rate points for 4K Hybrid Log-Gamma (HLG) test sequences for the CfP anchor generation. The proposed rate points were originally recommended by the BoG on extended colour volume and high dynamic range of 7th JVET meeting. During the interim period between the 7th and 8th JVET meetings, anchor bitstreams were generated by HM16.16 and JEM7.0 and available in JVET FTP site.

The proponent further commented on the calculation of the PSNR-L100 and deltaE100 for HLG graded content. Specifically, the peak luminance of these metrics (as defined in the CfP) is set to 10,000 cd/m2. However, that the peak luminance of 1,000 cd/m2 for HLG content would be more appropriate.

A participant commented that changing the value from 10,000 cd/m2 in the PSNR-L100 and deltaE100 calculations will change the PSNR-L100 and deltaE100 values by an offset, and so will not change the computed BD-Rate value. Additionally, by leaving the value unchanged, it may be easier to compare PSNR-L100 and deltaE100 values between coded PQ and HLG content.

Another participant commented that care should be taken when comparing the PSNR-L100 and deltaE100 objective metrics between PQ and HLG content.

The proponent reported that the reported results use a peak luminance of 1,000 cd/m2 for the PSNR-L100 and deltaE100 calculations.

The proponent also commented that currently there is no function to calculate wPSNR between two YUV sequences. It was requested that it would be useful to have this included in the HDRTools software package.

JVET-H0094 AHG7: Analysis of candidate HLG content [E. François (Technicolor)] [late]

This contribution provides some information on the HLG content currently considered as candidate test content for the Call for Proposal.

It was presented in the BoG JVET-H0095 (see notes under JVET-H0038).

JVET-H0097 AHG7 Cross-check of HLG rate points in JVET-H0068 [K. Kawamura, S. Naito (KDDI)] [late]

This was presented in the BoG JVET-H0095.

JVET-H0098 AHG7 Cross-check of Test Sequence of 4K Hybrid Log-Gamma in JVET-H0038 [K. Kawamura, S. Naito (KDDI)] [late]

This was presented in the BoG JVET-H0095.

JVET-H0099 Evaluation of Luma/Chroma QP Adaptation for HLG content [K. Kawamura, S. Naito (KDDI)]

The was presented in the BoG JVET-H0095.

## Evaluation methodology (1)

JVET-H0055 Analysis of the HDR Objective Metrics Utility in Draft CfP on FVC [P. Yin, T. Chen, W. Husak, F. Pu, T. Lu (Dolby)]

This was presented in the BoG on HDR/WCG on 20 October at 1110.

This document asserts that the draft CfP for FVC has two unclear and potentially overlapping bullets addressing subjective and objective testing. It also provides a review of the HDR objective metrics listed in the draft CfP and reports the correlation between subjective and objective results in previous JCT-VC HDR tests. The objective metrics include wPSNR Y/U/V, DE100 and PSNR-L100. It was noted that the metrics do not show perfect correlation with subjective tests.

The proponents recommended limiting the scope of the objective metrics to prevent misleading results being used in making technology selection decisions. They also propose using the HDR subjective evaluations performed as part of the CfP to further determine the correlation between subjective and objective results.

During the presentation, it was commented that the term BD-PSNR may be misleading, and that BD-Distortion may be better.

An overview of the wPSNR metric was then presented. The proponent highlighted three concerns about the wPSNR metric: the weight table was asserted to be derived from empirical tests, (ii) changes in the weight functions may lead to contradictory results, and wPSNR-U and wPSNR-V uses the weight of the collocated luma value.

Results from the PQ-10 verification tests were presented. It was reported that the Anchor 1.0 results included luma adjustment. The Anchor 3.2 results included luma adjustment, luma dependent adaptive quantization, and chroma dependent adaptive quantization.

A second set of results was provided comparing JCTVC-W0084 to Anchor 3.2. It was reported that subjective testing was performed using three rate points.

Regarding the CfP text, the proponent proposed to modify the language such that:

- Submissions for Class HDR-HD will be evaluated by means of a formal subjective assessment.

- Submissions for Class HDR-HD should also include the results of the following metrics: weighted PSNR values (at least the average of frame wPSNR for each video sequence and encoding point, separated by luma and chroma components), deltaE100 and PSNR-L100, as well as the Bjøntegaard Delta-Rate and Delta-PSNR for each metric. Metric definitions are provided in Annex D. The purpose of reporting these results will be to assist in the evaluation of the compression performance of each proposal, but should not replace the subjective evaluation.

This was discussed and resulted in the following edited text:

- Submissions for Class HDR-HD will be evaluated by means of a formal subjective assessment.

- Submissions for Class HDR-HD will be further evaluated by the following metrics: PSNR, weighted PSNR, deltaE100 and PSNR-L100, as well as the Bjøntegaard Delta-Rate and Delta-Distortion for each metric. Metric definitions are provided in Annex D.

There was discussion during the BoG on whether the purpose of the CfP was to assign a priority to the evaluation of the information in the manner requested by the proponents. It was suggested that the CfP document would not reflect that, and instead, it would reflect that the requested information would be used by JVET experts in the April meeting to come to a consensus on the way forward. The language above reflects that direction. However, if the suggestion is not correct, it should be revisited.

This was later discussed in JVET plenary (see under JVET-H0095). The edited text was recommended as appropriate.

# Exploration experiments (33)

## General (1)

JVET-H0011 Exploration Experiments on Coding Tools Report [E. Alshina, L. Zhang]

This report was discussed Wednesday 18 October 1430 (chaired by GJS). Information from this report is integrated into the sub-sections below on each of the three reported EEs.

Three experiments on coding tools were agreed to be carried out between the JVET-G and JVET-H meetings in order to get a better understanding of technologies considered for inclusion to the next version of the JEM, analyze and verify their performance, complexity and interaction with the existing JEM. This report summarizes the status of each experiment.

Key announcement related to EE activities and discussion about EE tools were done using the JVET reflector. EE-SW branches discussion was carried on among software coordinators, EE coordinators and proponents of the proposals studied in the EE.

Exploration experiments were conducted according to the timeline agreed at the JVET-G meeting.

In the discussions of the results of the EEs, as recorded below, some tests seemed promising enough that if we were to take action for some other reason, we would be likely to adopt the associated change. However, stability of the JEM and its anchors seemed a higher priority, given the limited benefits available by taking action.

## EE1: Intra prediction (18)

### General

This EE was discussed Wednesday 18 October 1440 (chaired by GJS).

EE1: Intra prediction

EE1 test results summary (all intra)

| Test | Performance | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1: PDPC without a mode flag (NSST==1  PDPC, NSST==3 MDIS) | AI: −0.2% (Y) −0.3% (U) −0.2% (V) Enc. 98%, Dec. 96% | Qualcomm JVET-H0027 | Panasonic JVET-H0076 |
| 1.1 PDPC without a mode flag (NSST==1  PDPC) | AI: −0.2% (Y) −0.3% (U) −0.1% (V) Enc. 101%, Dec. 99% | Qualcomm JVET-H0027 | Sharp JVET-H0075 |
| 2 Planar replaced by UWP (PDPC is still applied on top) | AI: −0.1% (Y) −0.1% (U) −0.1% (V) Enc. 97%, Dec. 95% | Arris & LGE JVET-H0052 | Huawei JVET-H0088 |
| 3. UW66 on top of Test 2 | Withdrawn |  |  |
| 4. Disable strong intra reference filter completely | Same as JEM7.0 (anchor) | Qualcomm JVET-H0028 |  |
| 5. Strong intra reference filter fix | AI: 0.0% (Y) 0.0% (U) 0.0% (V) Enc. 97%, Dec. 95% | Qualcomm JVET-H0028 | Ateme JVET-H0080 |
| 6. DC is in the back of the above-left mode of MPM list | AI: 0.0% (Y) 0.0% (U) 0.0% (V) Enc. 97%, Dec. 95% | HiSilicon JVET-H0024 | ETRI JVET-H0042 |
| 7. Initial offset = 2 in MPM list | AI: −0.1% (Y) −0.1% (U) −0.1% (V) Enc. 97%, Dec. 95% | HiSilicon JVET-H0024 | ETRI JVET-H0062 |
| 8. Initial offset = 1 in MPM list | AI: −0.1% (Y) −0.1% (U) 0.0% (V) Enc. 97%, Dec. 99% | Qualcomm JVET-H0029 | HiSilicon JVET-H0033 |
| 9. Offset in MPM list depends on the MPM index | AI: −0.1% (Y) −0.1% (U) −0.1% (V) Enc. 97%, Dec. 95% | Qualcomm JVET-H0029 | HiSilicon JVET-H0033 |

It was remarked that combining items 6 and 7 could provide 0.2% gain.

Test 1 had the biggest gains, but has a memory penalty as shown below.

Memory usage of coding tools tested in EE1

| Tools | Memory size |
| --- | --- |
| Test 1: PDPC (all modes) | 8400 bits (=5  35  6  8 bits) |
| UWP | 1260 bits (=126  10 bits) |
| P-PDPC (JEM7.0) | 240 bits (=5  1  6  8 bits) |
| Test 2: UWP+JEM7.0 | 1500 bits |

The primary candidates for action would appear to be:

- The combination of items 6 and 7.

- Item 5 is an overflow bug fix (but of a tool that is disabled in the CTC).

Further discussion was held Thursday afternoon (chaired by JRO) after reviewing all EE related and other intra prediction & mode coding related contributions were discussed (except for JVET-H0071), and the following were agreed:

- No need for continuation of the EE

- No changes to the JEM from EE proposals

### Primary (12)

JVET-H0024 EE1: Improvements for Intra Prediction Mode Coding [Y. Han, J. An, J. Zheng (Hisilicon)]

In the 7th JVET meeting, the contribution JVET-G0060 was discussed and included in the EE1 test. This contribution presents the test results of EE1 test 6 and test 7. For test 6, the MPM list initial order includes the DC mode after the above left modes. It reports an average BD bit rate impact of −0.04% on Y for the common test conditions of the AI case. For test 7, the selected modes are generated from the adjacent directional modes of MPM modes. It reports an average BD bit rate impact of −0.11% on Y for the common test condition of the AI case. The test result of the combination of the two techniques is about −0.2% BD bit rate impact on Y for AI case with almost no change in encoding time and decoding time.

JVET-H0027 EE1: PDPC without a mode flag [V. Seregin, M. Karczewicz, A. Said, X. Zhao (Qualcomm)]

This contribution proposes extending PDPC to all intra modes without a mode flag signalling, in which PDPC use is defined by the NSST index. The controlling of MDIS by the NSST index was studied in this contribution as well. Two tests were performed, where both PDPC and MDIS changes are applied together in the first test, and only PDPC change is applied in the second test. Test results reportedly show 0.2% luma BD rate gain with 100% encode running time on average in the all intra configuration for both tests.

JVET-H0028 EE1: Fix for strong intra smoothing filtering [V. Seregin, X. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes a fix for the strong intra smoothing. In the current JEM implementation, the strong intra smoothing is implemented using a bit shift operation, however a block can have a rectangular shape, so the shift operation is incorrect when the sum of width and height of the block is not a power of 2 value. In this contribution, the strong intra smoothing process is split into two parts, one is associated with a width, and the second one is associated with a height of the block, so the bit shift operation can be kept. Test results reportedly show almost no performance change, as 0.0% BD rate with 100% encoder running time for AI configuration on average.

JVET-H0029 EE1: Secondary MPM list [V. Seregin, W.-J. Chien, M. Karczewicz, N. Hu, X. Zhao (Qualcomm)]

This proposal presents modified intra mode coding considering a secondary most probable mode list. Simulation results reportedly show that proposed method provides 0.1% BD rate saving for luma in all intra configuration on average with 100% encoder running time.

JVET-H0033 EE1: Crosscheck of tests 8 and 9 [Y. Han, J. An, J. Zheng (Hisilicon)]

JVET-H0042 EE1: Cross-check of test 6 [H. Ko, H. Lee, J. Kang (ETRI)]

JVET-H0052 EE1 Tests 2 and 3: Replace Planar mode in P-PDPC with UWP and Enable UW66 on top of the above test [S. Hong, K. Panusopone, Y. Yu, L. Wang (Arris), H. M. Jang, J. Lim, S.-H. Kim (LGE)]

This contribution reports results of EE1 for Test 2 (Replace Planar mode in P-PDPC with UWP) and Test 3 (Enable UW66 on top of Test 2). Test 2 shows the BD rates of −0.1% with the encoding time of 99% for AI, and no significant change in decoding time. Test 3 shows the BD rates of −0.1% with the encoding time of 99% for AI, and no significant change in decoding time.

JVET-H0062 EE1: Cross-check of Test 7 [D. Jun, H. Lee, J. Kang (ETRI)]

JVET-H0076 EE1: Crosscheck of test 1 [V. Drugeon (Panasonic)] [late]

JVET-H0080 EE1: Cross-check of Test 5 [E. Mora (Ateme)] [late]

JVET-H0088 EE1: Crosscheck of tests 2 and 3 [A. Filippov, V. Rufitskiy, T. Solovyev (Huawei)] [late]

JVET-H0090 EE1: Crosscheck of tests 1.1 [T. Ikai (Sharp)] [late]

### Related (6)

JVET-H0051 Non-EE1: Priority List Based Intra Mode Coding with 5 MPM [Y. Yu, L. Wang, K. Panusopone (Arris)]

This contribution proposes a priority list based intra mode coding with five MPM design. Intra modes are ordered according to their priorities. Based on this ordered priority list, the first five modes are considered as most probable modes (MPM) and coded with a truncated unary code. The next sixteen modes are considered as selected modes and coded with fixed length coding. The remaining 46 intra modes are coded with a truncated binary code and the first eighteen modes among these 46 remaining modes are coded with seven bins while other modes are coded with eight bins. Simulation results show that the proposed method can provide 0.1% gain with less context based coding compared to JEM7. When DC mode is moved back after mode of above left block, the average gain can be 0.2% with a little bit faster encoding time and no decoding time change.

This has small gain, and could be interpreted somewhat as a simplification; however, in terms of the JEM which is an exploration software and not a standards development, this is not of substantial benefit.

JVET-H0075 Non-EE1: Crosscheck of Priority List Based Intra Mode Coding with 5 MPM (JVET-H0051) [T. Ikai (Sharp)] [late]

JVET-H0057 EE1 related: Simplification and extension of PDPC [X. Zhao, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

In this contribution, the look-up table for storing PDPC parameters is removed for planar mode and PDPC parameters are derived uniformly for all block sizes. In addition, PDPC with removed look-up table is applied to DC, vertical and horizontal modes. Simulation results reportedly show average −0.2% luma BD-rate gain and −0.1% luma BD-rate impacts for all intra (AI) and random access (RA) configurations, respectively.

There are two aspects of simplification: Avoiding a LUT, and usage of a 5-tap intra smoothing filter in PDPC for blocks larger than 32 luma samples.

In a second version, it is proposed to enable PDPC also with the DC, horizontal and vertical modes (which is not a simplification), which gives approx. 0.2% average.

Neither of these affects runtime significantly

This seemed to be only small tweaks, so no action for was taken for JEM or EE.

JVET-H0074 EE1-related: Cross-check of Simplification and extension of PDPC (JVET-H0057) [H. Lee, H. Ko, J. Kang (ETRI)] [late]

JVET-H0059 EE1-related: Two unifications for PDPC [H. Jang, J. Nam, J. Lim, S.-H. Kim (LGE)]

This contribution proposes three modification to simplify the planar mode, where PDPC is always applied, in JEM 7.0. The first modification introduces unified reference sample filtering for planar mode as with other conventional intra modes. The second modification includes using a unified reference buffer during PDPC processing. The third modification considers unification of the planar predictor generation process between PDPC and UWP. Three combination of these three modifications had been investigated and test results reportedly show BD-rate difference found in between 0.0% and 0.1% in the AI configuration.

This was probably a useful simplification (in particular, avoiding switching of filters depending on block size in PDPC); however, for the JEM which is an exploration software and not a standards development project itself, this is not of substantial benefit.

JVET-H0078 Cross-check of JVET-H0059 [K. Choi (Samsung)] [late]

## EE2: Entropy coding (4)

This EE was discussed Wednesday 18 October at 1510 (chaired by GJS).

EE2 test results summary

| Test | Performance | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1. Slice type independent context initialization | AI: 0.0% (Y) 0.3% (U) 0.3% (V) Enc. 99%, Dec. 99% RA: 0.0% (Y), 0.2% (U), 0.3% (V) Enc. 100%, Dec. 101% | Qualcomm JVET-H0061 | Samsung JVET-H0069 |
| 2. Multi-hypothesis CABAC window size for each context | AI: −0.1%(Y) −0.4% (U) −0.4% (V) Enc. 101%, Dec. 99% RA: −0.3% (Y) 0.1% (U) 0.0% (V) Enc. 101%, Dec. 99% | Qualcomm JVET-H0061 | Samsung JVET-H0069 |
| 3. Test 2 with switching QP < 30 | AI: −0.1% (Y) −0.3% (U) −0.4% (V) Enc. 99%, Dec. 99% RA: −0.3% (Y) 0.1% (U) 0.1% (V) Enc. 101%, Dec. 99% | Qualcomm JVET-H0061 | Samsung JVET-H0069 |
| 3.1. Test 3 with additional QP range (low QP) | AI: −0.0% (Y) −0.1% (U) −0.2% (V) Enc. 102%, Dec. 100% RA: −0.2% (Y) 0.5% (U) 0.4% (V) Enc. 101%, Dec. 100% | Qualcomm JVET-H0061 | Huawei/ Samsung JVET-H0069 |
| 3.2. Test 3 with additional test sequences | AI: −0.2% (Y) −0.1% (U) −0.1% (V) Enc. 102%, Dec. 100% RA: −0.3% (Y) 0.4% (U) 0.6% (V) Enc. 101%, Dec. 100% | Qualcomm JVET-H0061 | Huawei/ Samsung JVET-H0069 |
| 3.3. (Supplementary) Test 3.1 with additional test sequences and additional QP range | AI: −0.1% (Y) −0.1% (U) −0.1% (V) Enc. 102%, Dec. 100% RA: −0.2% (Y) 0.4% (U) 0.4% (V) Enc. 101%, Dec. 100% | Qualcomm JVET-H0061 Huawei JVET-H0085 | Huawei JVET-H0085 |

There was some discussion of extra testing done (reported as 3.3 supplemental information above) for additional test sequences that was announced to proponents late in the process. This data is considered supplemental. One participant commented that the purpose of this was to check for potential overtraining. Some others expressed the opinion that it was unanticipated extra work and unjustified on that basis. Another participant commented that having extra data provided has happened before and isn’t necessarily a problem if providing it is voluntary and the extra data is identified as supplemental and can be cross-checked.

The primary candidates in this test would be

- Test 1, since that is simpler than the current JEM scheme in terms of the initialization (not dependent on slice type)

- Test 2, since that provides as much gain as any others and reduces a table size. tests 2 and 3 require an additional 6 bits storage per context model for the counter, whereas test3 requires two different initialization tables depending on QP

Follow-up discussion was conducted Thursday afternoon (chaired by JRO):

- It seems there is still more headroom for improvement (as shown in JVET-H0067), though with additional complexity

- The reason for the drop in chroma (in particular for RA and LDB) is not well understood; one aspect may be that the chroma QP offset is not suitable for the modified entropy coding

No action was taken for this for the JEM.

It was agreed to continue study in a new AHG9 on entropy coding tradeoffs (A. Said), studying the impact on complexity, the impact on RD decisions, etc.

### General

### Primary (3)

JVET-H0061 EE2: Arithmetic coding with context-dependent double-window adaptation response [A. Said, M. Karczewicz, V. Seregin, H. Egilmez, L. Zhang, X. Zhao (Qualcomm)]

This contribution reports test results for the CABAC modification method proposed in JVET-G0112. It contains CTC results, which confirm previous results, and tests on additional sequences and QP values, which show similar coding gains. For the CTC, 0.1%, 0.3%, 0.1% and 0.1% BD-rate reduction is achieved for AI, RA, LDB and LDP configurations, respectively.

JVET-H0069 EE2: Cross-check for Arithmetic coding with context-dependent double-window adaptation response [E. Alshina (Samsung)] [late]

JVET-H0085 EE2: Cross-check for JVET-G00112 Test 3.2 and 3.3 [M. Sychev (Huawei)] [late]

### Related (1)

JVET-H0067 EE2 related: Arithmetic coding with progressive context-dependent double-window adaptation response [A. Said, H. Egilmez, M. Karczewicz, V. Seregin, L. Zhang, X. Zhao (Qualcomm)]

This proposal extends the context-based double CABAC adaptation window selection of JVET-G0112, adding a bin counter to define per-context change of double windows, but only allowing parameter changes at the start of a CTU. Simulation results reportedly show that, without an encoding or decoding time increase, the proposed method produces luma BD-rate coding gains of 0.2% for All Intra, 0.4% for Random Access, 0.2% for Low Delay B, and 0.2% for Low Delay P.

This builds on top of test 2. It requires another 6 bits per context model (in total 12).

It is also generally more complex than EE2 methods due to using multiple windows, but gives additional gain.

## EE3: Adaptive loop filter (10)

This EE was discussed Wednesday 18 October 1600 (chaired by GJS).

EE3: Adaptive loop filter

EE3 test results summary

| Test | Performance | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1. Chroma QP offset set to 0 in JEM7.0 (no other changes) | AI:1.6% (Y) −12.4% (U) −12.2% (V) Enc. 101%, Dec. 101% RA: 0.9% (Y) −10.6% (U) −10.2% (V) Enc. 100%, Dec. 100% | HiSilicon JVET-H0041 | Samsung JVET-H0070 |
| 1.1 Chroma QP offset set to 2 in JEM7.0 | AI: −1.2% (Y) 13.8% (U) 13.3% (V) Enc. 100%, Dec. 100% RA: −1.1% (Y) 16.0%(U) 15.9%(V) Enc. 100%, Dec. 100% | Huawei JVET-H0045 | Samsung JVET-H0070 |
| 2. New lambda settings for Chroma on top of Test 1 (w/o ALF modification) | AI: −0.9% (Y) 9.7% (U) 10.8%(V) Enc. 99%, Dec. 99% RA: −0.9% (Y) 4.9% (U) 4.6%(V) Enc. 99%, Dec. 100% | HiSilicon JVET-H0041 | Kingsoft Cloud JVET-H0064 |
| 3. Luma samples classification granularity in ALF 44 | AI: 0.1% (Y) 0.0% (U) 0.0% (V) Enc. 100%, Dec. 100% RA: 0.1% (Y) 0.2% (U) 0.3% (V) Enc. 100%, Dec. 101% | HiSilicon JVET-H0041 | ETRI JVET-H0063 |
| 4. Chroma ALF borrows samples classification from Luma; filter coefficients are also derived from Luma ALF on top of Test 3 | AI: 0.1% (Y) −0.5% (U) −0.5% (V) Enc. 100%, Dec. 101% RA: 0.1% (Y) −1.7% (U) −1.4% (V) Enc. 100%, Dec. 101% | HiSilicon JVET-H0041 | Panasonic JVET-H0000 |
| 5. UALF is switched to JEM7.0 (separate) ALF by picture level flag on top of Test 4 | AI: 0.1% (Y) −0.8% (U) −0.9% (V) Enc. 100%, Dec. 101% RA: 0.1% (Y) −3.0% (U) −2.6% (V) Enc. 100%, Dec. 101% | HiSilicon JVET-H0041 | Panasonic JVET-H0000 |
| 6. Chroma ALF CTU level control for separate ALF on top of Test 5 | AI: 0.1% (Y) −0.8% (U) −0.9% (V) Enc. 100%, Dec. 101% RA: 0.1% (Y) −3.2% (U) −2.9% (V) Enc. 100%, Dec. 101% | HiSilicon JVET-H0041 | Qualcomm JVET-H0046 |
| 7. CTU level on/off control for chroma ALF on top of Test3 | AI: 0.1% (Y) 0.0% (U) 0.0% (V) Enc. 100%, Dec. 99% RA: 0.1% (Y) −0.5% (U) −0.7% (V) Enc. 100%, Dec. 99% LB: 0.1% (Y) −1.6% (U) −1.2% (V) Enc. 100%, Dec. 99% | Sharp JVET-H0037 | HiSilicon JVET-H0065 Qualcomm JVET-H0054 |

Information provided in response to the questions aske when setting up the EE is given below.

[Q]: Do the proposed lambda settings for Chroma provide a better trade-off compared to Chroma QP offset?

[A]: A table below shows the BD-rate between Cb component (horizontal axis) and Y component (vertical axis) for RA configurations, respectively. The relationship between Cr and Y is similar.

It is noted that EE3 Test 2 involves different weights for different configuations (i.e, AI, RA and LD are using different weights). JEM7.0 sets the chroma QP offset to 1 for AI and RA, and 0 for LD.

The table below shows the shifting ratio from BD gains of chroma to BD gains of luma with different setting for JEM7.0 for RA, i.e., how much chroma gain is equivalent to 1% luma gain.

Luma-chroma gain shifting ratio table:

|  | luma | (Cb+Cr)/2 | Chroma/luma |
| --- | --- | --- | --- |
| CbCrQPOffset = 1 | 0.9 | −10.4 | 11.6 |
| CbCrQPOffset = 2 | −1.1 | 16 | 14.5 |
| CbCrQPOffset =1.5 | −0.4 | 6 | 15 |
| CbCrQPOffset =1.75 | −0.7 | 9.3 | 13.3 |
| EE3.2 | −0.9 | 4.7 | 5.2 |

It was commented by a cross-checker that the BD rate measure for EE3.2 may not be reliable due to curves crossing each other.

[Q]: What is the performance effect of changing granularity for Luma samples classification in ALF to 44?

[A]: An average 0.1% Luma BD-rate loss is observed.

[Q]: What is the performance benefits of multiple filters for Chroma (with samples classification from Luma)?

[A]: No explicit results were available to answer this query.

Comparing Test 4 with Test 3, it shows the gain of chroma ALF with 4x4 luma block classification + multiple filters for chroma + on/off control inheritance from luma as a replacement of the current JEM chroma ALF design. On average, coding gains of 0.0%/0.5%, 0.0%/1.6%, 0.0%/2.6% and −0.1%/3.7% for luma/chroma under AI, RA, LDB, LDP, respectively, were observed.

When comparing Test 5 to Test 3, it shows the gain of chroma ALF with 4x4 luma block classification + multiple filters for chroma + on/off control inheritance as an additional chroma mode relative to the current JEM. On average, coding gains of 0.0%/0.9%, 0.0%/2.8%, 0.0%/3.9% and −0.1%/4.8% for luma/chroma under AI, RA, LDB, LDP, respectively, were observed.

[Q]: What is the performance benefit of CTU-level control of Chroma ALF?

[A]: The Test 7 was built on top of the Test 3 (4x4 luma classification), and the CTU level control for chroma ALF was added. The chroma gain of Test7 compared to Test 3 is 0.0%, 0.6% and 1.4% for AI, RA and LDB, respectively. The test 7 additional test, which is built on top of JEM7.0 (2x2 luma classification) also confirmed the same conclusion.

The chroma gain of Test 6 compared to Test 5 is, 0.0%, 0.3%, 0.7% for AI, RA, and LD, respectively.

[Q]: Whether similar gain could be achieved without cross-colour dependency (i.e., Chroma filter is signalled as in JEM7, with either LCU-level control (explicitly signalled) or block-level control inherited from Luma (implicitly signalled) depending on the separate mode flag)?

[A]: Not tested in this EE.

Comments on the luma-chroma dependency in ALF variant studied in EE3 were discussed on the JVET reflector and details are available in JVET-H0041.

Proponents were requested to prepare a subjective quality demonstration.

The luma and chroma BD rates were plotted relative to each other as shown below.

Test 5 and test 6 require the decoder to switch the chroma filtering coefficients on a 2x2 basis instead of the current whole-picture filtering approach. Between test 5 and test 6, test 6 is suggested to be better.

The proponent of test 6 asserted that there was no subjective impact, but a cross-checker said that some subjective degradation can be observed on still frames.

It was asked what would be the impact of using 8x8 luma granularity, since test 3 shows no degradation for 4x4 granularity. It was suggested that this would likely have some degradation of coding performance, although the precise impact is not known.

As primary candidates for action:

- Test 3 seems to be a useful simplification (with little apparent impact on quality – about 0.1% for RA and LB)

- Test 6 would be a complexity increase (at least relative to test 3, as it introduces 2x2 switching for chroma), for a 3–5% chroma gain (aside from subjective effects, which seem to be a degradation)

If we would take action to change the JEM, we would take action on test 3.

Follow-up discussion wsa held Thursday afternoon, chaired by JRO.

Test 3 is probably a useful simplification that would be useful in a standard under development. For the JEM, there is no urgency of action on this, so no action was taken.

No continuation of EE3 was planned.

### General

### Primary (10)

JVET-H0037 EE3.7: chroma ALF on/off flag [T. Ikai (Sharp), J. An, J. Zheng (HiSilicon)]

This contribution presents the results of the EE3 Test7 about the CTU level on/off control for chroma ALF. It is reported that 0.0%, 0.6%, 1.4% BD rate reduction was observed on average for Cb and Cr in AI, RA, and LB configurations, respectively, with no run time change.

JVET-H0041 EE3 Test1-6: Unified Adaptive Loop Filter for Luma and Chroma [J. An, J. Zheng (HiSilicon)]

This contribution presents the results of the EE3 Test1-6 about the unified adaptive loop filter (UALF) for luma and chroma. In the proposed unified ALF mode, the chroma sample classification and on/off decision directly re-use the results of the collocated luma sample so that there is no complexity increase for the chroma classification. The chroma ALF coefficients also re-use those of luma but with a constant 5x5 diamond tap region of support to minimize the low complexity of chroma filtering. In the original separate ALF mode, a CTU-level on/off control for chroma ALF was proposed.

JVET-H0045 EE3: Test on Chroma QP offset [S. Ikonin, R. Chernyak, T. Solovyev, S. Liu (Huawei)]

This contribution provides test results for EE3 test 1.1 with chroma QP offset set to 2. In addition, experimental results with a non-integer chroma QP offset are included.

JVET-H0046 EE3: Cross-check of Test 6 (JVET-H0041) [L. Zhang, W.-J. Chien (Qualcomm)] [late]

JVET-H0054 EE3: Cross-check of Test 7 additional results (JVET-H0037) [L. Zhang (Qualcomm)] [late]

JVET-H0063 EE3: Cross-check of Test 3 [D. Jun, H. Lee, J. Kang (ETRI)]

JVET-H0064 Crosscheck of EE3 Test 2 of JVET-H0041 [X. Zhang (Kingsoft Cloud)] [late]

JVET-H0065 EE3: Crosscheck of Test7 [J. An (HiSilicon)] [late]

JVET-H0070 EE3: Cross-check for Chroma QP offset 0 and 2 (tests 1 and1.1) [E. Alshina (Samsung)] [late]

JVET-H0073 EE3: Cross-check of Test4 and 5 [Ryuichi Kanoh] [late]

### Related (0)

# Non-EE Technology proposals (15)

## Intra coding (6)

Contributions in this category were discussed Thursday 19 October in the afternoon (chaired by JRO) except as otherwise noted.

JVET-H0036 Condition modification of Strong Intra Smoothing Filter [Y. Kidani, K. Kawamura, S. Naito (KDDI)]

This contribution proposes application conditions of a strong intra smoothing (SIS) filter for non-square blocks. The SIS filter in the JEM-7.0 is disabled as an anchor condition. In the EE1 Test #5, a "correction method" of the SIS filter for non-square blocks is tested. The applicable condition is, however, still said to be incorrect, as a comparison pixel position is not at the center of the end pixels but on the corner of a non-square block. The proposed method is implemented on the top of the EE1 Test #5 software. Although no BD-rate impact was observed, the asserted bug-fix was suggested to be included in the future JEM software.

Currently, the strong filter is disabled, and the conclusion from EE1 was to keep it like that. There seems to be no visual quality problem due to that. Therefore

no action was taken on the proposal.

JVET-H0086 Cross-check of JVET-H0036 on strong intra smoothing condition [V. Seregin (Qualcomm)] [late]

JVET-H0044 Do we need multiview profiles for future video coding generations? [J. Samelak, O. Stankiewicz, M. Domanski (Poznan Univ.)] [late]

Some existing video coding standards, like MPEG-2, AVC, HEVC, provide multiview profiles. It was demonstrated that application of the multiview coding technology provides some gains over simulcast video coding of multiple views. Nevertheless, the multiview video coding technology was adopted by industry in the limited number of applications only. On the other hand, the frame compatible approach to compress stereoscopic video was quite commonly used recently. Moreover, a new technology of Screen Content Coding has been adopted recently, and this technology seems to be successful in real-word applications. The Screen Content Coding extensions provides a tool known as current picture referencing (CPR) or intra block copy (IBC). This contribution shows that this tool may be efficiently used for multiview video coding, when the quarter-pel accuracy of vectors is applied (unlike in the SCC extensions, where the feature is restricted to integer-pell accuracy). The contribution demonstrates that intraframe coding efficiency of a standard intraframe HEVC codec augmented with the CPR tool is similar to that of Multiview HEVC under the assumption that the accuracy of vectors is the same for both codecs. The HEVC codec augmented with the CPR tool has the simple single-loop architecture that is compliant with that of the widely used single-view codecs.

This was presented Tuesday 24 October by JRO in the absence of the proponents (the proponents had been informed, but were not present.

A comment was made that it is an interesting idea to make frame packing multi-view coding more efficient by using current picture referencing (unlike in HEVC, the proponents use quarter-pel accuracy). Compression performance of the modified HEVC-SCC comparable or sometimes better than MV-HEVC.

No specific action item was identified for JVET, although the contribution provided interesting information.

JVET-H0053 Weighted Angular Mode 2 and Adaptive Angular Mode 66 [K. Panusopone, Y. Yu, S. Hong, L. Wang (ARRIS)]

This contribution proposes to replace angular mode 2 with weighted angular prediction. Additionally, angular mode 66 is also modified to adaptively select predictor based on CU size. The boundary filter is disabled when the proposed weighted angular mode 2 is chosen and modified for the adaptive angular mode 66. Simulation results reportedly show that the proposed method, in combination with UWP, provides luma BD-rate gain of 0.2% over the JEM-7.0 anchor. The coding gain is 0.3% for class A test sequences.

The contribution proposes different additional elements (not a simplification) to intra prediction.

The benefit was not obvious enough to prefer this change versus keeping the JEM stable.

JVET-H0071 Redundant flag removal on chroma intra mode coding [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

This contribution proposes redundancy removal on chroma intra mode coding. In the current JEM7.0 implementation, an intra prediction mode of chroma CU is signalled with one redundant CABAC-coded bit when the last candidate of the DM list derived from collocated luma block and neighbouring chroma block is applied. The removal of the redundant signalling reportedly brings a slight gain on chroma components without increase of encoding and decoding time.

This was presented Friday 20 October afternoon (chaired by JRO).

The aspect pointed out in this contribution is valid without doubt. However, considering the fact that the benefit of redundancy removal is practically almost invisible w.r.t. rate saving, it is not worthwhile to have this as the only element of JEM8. As a current conclusion, no new version of the JEM would be defined from the current meeting. If it would be decided later to produce JEM8, the change suggested in JVET-H0071 should be included.

JVET-H0089 Crosscheck of JVET-H0071 Redundant flag removal on chroma intra mode coding [P.-H. Lin, C.-L. Lin, C.-C. Lin (ITRI)] [late]

## Inter coding (5)

Contributions in this category were discussed Thursday 19 Oct. afternoon (chaired by JRO) except as otherwise noted.

JVET-H0031 Inter Prediction using Estimation and Explicit Coding of Affine Parameters [C. Heithausen, J.-R. Ohm (RWTH Aachen Univ.)]

This contribution was discussed Friday October 20 at 1400 (chaired by GJS).

This document proposes a higher order motion compensation scheme which includes the estimation, prediction and direct coding of affine parameters, which is suggested to be more precise then the affine mode in JEM. The estimation algorithm derives block-wise affine parameters from local gradients of all samples contained in the considered block. The focus however lies on an efficient prediction and coding of estimated affine motion parameters. Since inheriting affine parameters from a neighbouring block would entail a translational aberration, a change-of-basis transformation method is introduced to adapt affine motion parameters from one block to another one. By this, the affine motion model provides compression advantages despite the additional parameters it requires, in particular for prediction modes where it is not necessary to transmit a motion vector difference. For affine parameter prediction in the case of different reference picture distances, a novel distance scaling scheme is introduced which corrects the fact that in case of zoom the translation grows exponentially. The higher order motion compensation scheme resulting from these approaches reportedly provides an overall average bit-rate reduction of about 8%/16%/19% for RA/LDB/LDP relative to HEVC+64×64CTU&TU when integrated into an HEVC encoder and tested on video sequences of mainly non-translational motion content (a separate test set, not the common conditions test sequences). It should however be emphasized that the experiments have not been based on integrating the developed methods in the context of JEM so far. No specific action was suggested for the current meeting.

This proposes a mode with higher-order motion compensation using 1/16th pel interpolation (per SHVC filtering) on a per-sample basis.

Affine parameters are coded directly (with some prediction of parameters), unlike the scheme in the JM that uses control point translation parameters.

A translational shift of the parameters is incorporated to produce a uniform motion field across block region boundaries.

A summary of the results is provided below. A four-parameter scheme supporting zoom & rotation (sometimes called the “similarity model”) does a little better in the experiments than the affine scheme. The four-parameter scheme is similar to what in the JEM called affine motion prediction in that context. The JEM scheme uses 4x4 blocks rather than sample-by-sample position computation.

The proponent reported results that were typically 1–2% worse for 4x4 blocks.

It was asked whether the mode is used only in single-list prediction or also for biprediction. This wasn’t clear.

Further study and integration into JEM would be needed to further assess the potential.

JVET-H0058 Unified search range for FRUC [H. Jang, J. Nam, J. Lim, S.-H. Kim (LGE)]

This contribution proposes a motion vector (MV) refinement process based on unified search range for the Frame Rate-Up Conversion (FRUC). In the current JEM, motion derivation process in FRUC merge mode has two steps; a CU level motion search is first performed, and then followed by a sub-CU level MV refinement. Since a sub-CU level MV refinement is performed based on the motion of a selected sub-CU candidate as a starting point, an independent motion refinement (i.e. search) is performed for each sub-CU. Here, setting its own search range at each sub-CU reportedly needs a frequent block memory patching from corresponding reference picture and also can cause critical memory bandwidth demanding. In this proposal, a unified search range between the CU-level and a sub-CU level is proposed so that the MV refinement process in a sub-CU level is always performed within a CU level search range. Test results reportedly showed that 0.05%, 0.03% and 0.01% BD rate increases had been observed in RA, LDB, and LDP configurations, respectively.

The proposal likely has advantages in reducing memory bandwidth, which would be highly desirable in the development of a standard. For the JEM, this is not of high importance; encoder/decoder run times are not affected significantly.

Further study was recommended.

JVET-H0077 Cross-check of JVET-H0058 [K. Choi (Samsung)] [late]

JVET-H0072 Modification of merge candidate derivation for binary split CUs [Y. Ahn, H. Ryu, D. Sim (Digital Insights), J. Lim, D. Jeon (Kaonmedia)] [late]

In this contribution, modification of merge candidate derivation for a binary split CU is proposed to prevent merging two binary split CUs. The merging two binary split CUs having the same motion data and block size can be alternatively expressed by a non-split CU, and it corresponds to redundant syntax. To derivate merge candidate list for the second binary split CU, the proposed modification provides the partitioning redundancy check among spatially neighbouring blocks on top of quadtree plus binary tree (QTBT) structure. When a current CU is a second binary split CU, left CU or above CU which has a same block size of the current CU is set to unavailable for the merge candidate depending on the split type. With the proposed modifications, 0.0% and 0.1% BD-rate reductions over JEM-7.0 were reportedly achieved with same encoder and decoder complexities for random access and low-delay B configurations, respectively.

Some additional condition checks are necessary for the scheme.

No obvious benefit was evident.

Some doubt was expressed that the redundancy that is claimed really exists; for example, it may be desirable to have two adjacent blocks have the same motion vector if they split in order for the transform size to be selected.

No action was taken on this.

JVET-H0081 Cross-check of JVET-H0072 “Modification of merge candidate derivation for binary split CUs” [Y. Uk, D. Hyun, J.-G. Kim (KAU)] [late]

## Loop filters (2)

Contributions in this category were discussed Thursday 19 October in the afternoon (chaired by JRO), except as otherwise noted.

JVET-H0034 A simplified method to sao_type_idx coding of sample adaptive offset (SAO) [X. Zhang, Z. Zhu, J. Fan, X. Jin, E. Zhang (Kingsoft Cloud)]

In the current JEM, sao_type_idx_luma and sao_type_idx_chroma are coded with up to 2 bins and one context model to represent the Sample Adaptive Offset (SAO) modes of SKIP, Band offset (BO) or Edge Offset (EO). This contribution proposes to (1) code up to 2 bins of sao_type_idx syntax in equal probability without any context model, (2) code EO mode with the shortest binarization. It is reported that this method contributes to a computational complexity saving to the decoder due to the removing of the context model, together with a slight bit rate saving.

Some losses occur in class E for LDB and LDP modes. It might be interesting to investigate the performance when only taking the second change, but still using a context model.

No action was taken for the JEM – further study was recommended.

As a general remark, SAO was historically developed for 8 bit content (as class E is) and might not be optimum in all details for newer 10 bit material.

JVET-H0066 Crosscheck of JVET-H0034 A simplified method to sao_type_idx coding of SAO [J. An (HiSilicon)] [late]

## Partitioning and related (2)

Contributions in this category were discussed Thursday 19 October in the afternoon (chaired by JRO), except as otherwise noted.

JVET-H0087 Diagonal motion partitions on top of QTBT block structure [Y. Ahn, H. Ryu, D. Sim (Digital Insights)] [late]

In this contribution, diagonal motion partitions (DMPs) for inter prediction are proposed on top of the quadtree plus binary tree (QTBT) block structure. In the proposed partitioning method, a coding unit (CU) is split into two diagonal motion partitions. The proposed method includes only two diagonal directions, but it can represent various arbitrary partitions on top of QTBT block structure. The proposed DMPs can reportedly achieve 0.15% BD-rate reduction over JEM-7.0 for the random access configurations.

The signalling might be further improved.

Results reported are only for one intra period, and only for RA.

For class D, an almost 10% increase of decoder runtime was reported, though it is probably not used very often.

Further study was recommended, with no direct action taken.

JVET-H0023 Implementation and design aspects of xvc [J. Samuelsson, P. Hermansson (Divideon)]

This document presents information related to implementation aspects and design aspects of a publicly available video encoding and decoding software package called xvc. The includes some of the technologies that are under study in JVET, but the software is an independent implementation, in particular a quad-tree, binary-tree split structure is included. The xvc software is asserted to have an efficiently implemented decoder which makes it possible to decode full HD video in real time on mobile devices. The software is available for use in research and standardization activities.

The presentation concentrated on the differences of the QTBT in this scheme relative to the JEM version:

  - The default binary split depth is 2 instead of 3. A binary split depth of 3 is only used for a specific speed level.

  - It does not perform 5 of the shortcuts/speedups that the JEM version of QTBT does (corresponding to LCUFast, JVET_C0024_AMAX_BT, JVET_C0024_PBINTRA_FAST and 2 speed-ups without any #define tags related to early termination based on horizontal splits and re-using motion vectors from previous blocks).

  - It performs the “do not evaluate quad-split if the best evaluated mode so far had no further splits”-speedup only if the best binary split had no further splits. It was also reported that the way of doing it like in JEM was found to lead to artefacts

The contribution was noted – no specific action was taken.

# Coding of 360° video projection formats (6)

Contributions in this category were discussed Friday 20 October during 0900–1200 (chaired by JRO).

Packing and Projection formats (2)

JVET-H0056 AHG8: An Update on RSP Projection [A. Abbas, D. Newman (GoPro)]

This contribution offers an modification of the previously proposed Rotated Sphere Projection (RSP) scheme. The proposed method draws inactive region on a 16x16 grid, resulting in better coding efficiency for block based coding. Additionally, a fix to WS-PSNR calculation method was proposed. Overall, a RA coding gain of 0.7% for Luma and 1.0% for Chroma is reportedly achieved over the RSP anchor (using WS-PSNR-E2E metric).

Compared to the previous RSP, the arc (circular shape) is drawn more precisely, which firstly increases the number of inactive pixels. As a second step, the arc is extended such that it is fully fitting into a 16x16 block (a kind of padding). This effectively increases the number of active pixels, but they are easier to code by a block based coder.

Decision (360lib): Replace the current RSP scheme by the method from JVET-H0056.

JVET-H0079 Cross-check of JVET-H0056 on an update on RSP Projection [S. N. Akula, C. Pujara (Samsung)] [late]

The cross-checker confirmed the validity of results and also reported that the code is clean.

## Quality assessment and metrics (4)

JVET-H0035 AHG8: The guidance of WS-PSNR weights derivation for different formats [Y. Sun, L. Yu (Zhejiang University)]

WS-PSNR was proposed to measure the coding and end-to-end distortion for 360° video. To measure objective quality of omnidirectional video in the observation space, the error at each pixel position in the representation space is weighted according to a corresponding mapped spherical area in the spherical observation space. This proposal is for guidance of WS-PSNR weights derivation for different formats of 360° video.

The contribution explains steps to compute WS-PSNR individually for projection formats. In the meantime, WS-PSNR computation has been made available for all projection formats included in 360lib.

JVET-H0048 AhG8: Dynamic viewport based subjective evaluation of Balboa and Landing [P. Hanhart, Y. He, Y. Ye (InterDigital)]

This document reports results of a subjective evaluation comparing dynamic viewports for the newly adopted Balboa and Landing sequences. The HM and JEM CfP anchors were used in the evaluation, as well as the updated Landing sequence (JVET-H0021), which was encoded with HM-16.16 and JEM-7.0 in the padded equirectangular projection (PERP) format under CfP coding conditions for the current Landing sequence. The double stimulus impairment scale (DSIS) method was used to assess the subjective quality of these sequences coded using HM and JEM. Results show that JEM achieves some visual quality improvements over HM across the selected target bit rates. Finally, some comments regarding the Landing sequence are made based on the feedback from the viewers.

Experts viewing was performed (9 participants), using the 5 grade MOS scale.

For Balboa, the range of rates reportedly seems OK, differentiation between JEM and HM with non-overlapping confidence intervals was reported up to the highest bit rate.

Landing1 is rated at highest rate with a MOS of 3.x. Viewport 1 seems to be inappropriate, as high camera motion in the end may be disturbing. It was pointed out in the discussion that for 360° video with a high amount of camera motion that too much additional variation in dynamic viewport should be avoided.

Landing 2 is reportedly somewhat better. It was however observed that in the case of both JEM and HM coding the variation over rates 1–4 is not large for viewport 2 (looking up to sky). Nevertheless, differences between HM and JEM are still visible.

If we make a reduction to 5 sequences, Landing is probably a candidate for being dropped.

It was later decided that Landing will not be included in the CfP.

JVET-H0049 AhG8: On viewport size and field of view [P. Hanhart, Y. He, Y. Ye (InterDigital)]

This document investigates suitable viewport field of view (FOV) parameters for rendering of high definition resolutions viewports. Two viewport configurations were investigated and compared to the CTC conditions (i.e., 75×75 FOV and 1816×1816 resolution): 78.1×49.1 FOV and 116.7×84.8 FOV. Informal viewing was conducted to evaluate the suitability of both configurations. Observations made during the viewing are reported.

78.1x49.1 requires upsampling (approx. by 1.5) to be displayed on a full HD display.

It was agreed to organize a viewing session to judge (1) if compression artefacts are equivalently visible for the non-square viewports and (2) if yes, which non-square configuration is more suitable.

After the viewing session, it was decided to use the viewport size 78.1×49.1 FOV in the CfP.

Sentence(s) were added in the CfP to inform proponents about the viewport size, spatial resolution and bit depth, the interpolation filters that are used for generating it, and otherwise the same subjective viewing methodology will be used as described in annex C of the CfP.

Furthermore, annex E of the CfP was aligned concerning the reduction of number of metrics.

Philippe Hanhart was requested to take action on making these text modifications and give them to Andrew Segall for inclusion.

JVET-H0050 AhG8: Measurement of User Exploration Behavior for Omnidirectional (360°) Videos with Head Mounted Display [A. Singla, A. Raake, S. Fremerey (TU Ilmenau), P. List, B. Feiten (Deutsche Telekom)] [late]

This contribution reports on the exploration behaviour of users while watching omnidirectional (360°) video content. A subjective test was carried out in order to find favourable yaw and pitch ranges, velocities and viewports for different 360° video sequences. Results reportedly show that users explore 360° video more in yaw than in the pitch direction.

This was presented by P. Hanhart.

In the case of pitch, it is more common to look up rather than looking down.

Also pitch and yaw movement velocities (degrees/s) were reported.

All sequences were with static cameras (Harbour, KiteFlite, Trolley).

In particular the velocity is important to consider for definition of the dynamic viewports for the CfP response evaluations. It should however be observed that in the case of moving cameras it is likely that even slower head movements would be exercised.

Additional results with moving cameras would be welcome.

## Complexity analysis (2)

Contributions in this category were discussed Thursday 19 October in the afternoon (chaired by JRO), except as otherwise noted.

JVET-H0039 AHG5: On worst case memory bandwidth [T. Ikai (Sharp), R. Hashimoto, S. Mochizuki (Renesas)] [late]

This contribution discusses issues of the worst-case memory bandwidth, and suggests the following recommendations.

- Check the memory size of the constant / table data which should be always used in CTU level

- Calculate HEVC style memory bandwidth for motion compensation related tools

The tool on/off methodology was applied using HEVC MB analysis and the tools valgrind/cachegrind. Some findings reported included that:

- Sometimes “tool off” operation increases cache misses (probably because other more memory-intensive tools are used more often)

- Big tables often do not fit the cache

It was further discussed how relevant the worst case is, and how it could be measured. Basically this would mean that “evil bitstreams” need to be generated.

Another interesting type of information might be variation over different sequences.

JVET-H0043 AHG5: How to measure memory bandwidth [R. Hashimoto, S. Mochizuki (Renesas), T. Ikai (Sharp)] [placeholder] [late]

This contribution suggests how to measure the memory bandwidth in JEM. Focusing on access to reference pictures reportedly makes it easy to model a cache in JEM. The implementation of a cache in the JEM reportedly enables performance checking of the cache without any external tool, and reportedly makes crosscheck easy by removing implementation dependency. The result was expected to be available in the next meeting.

Whereas tools like valgrind rely on the computing hardware and compiler that is used, the suggested approach would make it independent, and could be parametrized for different cache sizes.

Several experts expressed the opinion that such an approach would be highly beneficial, with the ability of defining cache and memory models. This could be beneficial in the development of a new test model after the CfP. Therefore, further study (currently in JEM) was recommended (in the continuing AHG).

In the context of the CfP, measuring memory accesses with tools like valgrind does not seem to be beneficial, as it is too much platform dependent. For initial complexity analysis, the technical description of proposals should be sufficient, and runtime also gives some indication.

One approach could be to formulate questions to proponents, such as

- sample units that require access causing cache misses

- maximum length of filters

A BoG (coordinated by Minhua Zhou) was asked to compile an initial version of this questionnaire. See the notes for JVET-H0096.

# Encoder optimization (1)

Contributions in this category were discussed Friday 20 October during 1215–1230 (chaired by JRO).

JVET-H0091 Restriction of fast intra-mode decision [P.-H. Lin, C.-L. Lin, C.-C. Lin, Y.-T. Tsai (ITRI)]

This contribution proposes to restrict the fast intra-mode decision in JEM7.0. The simulation results reportedly show that significant gain is observed in the class A1 in the RA condition.

The most gain was observed for ToddlerFountain (0.4%) and CampfireParty (0.7%). Additional options for intra were tested, which increases the encoder run time.

Several experts expressed the view that this contribution provides very interesting gain for difficult sequences (which probably use intra mode frequently).

Only RA results were presented – the approach only applies in the case where inter and intra modes are competing. It would, however, be interesting to see results on LDB and LDP cases before taking further action. Further study was recommended.

# Metrics and evaluation criteria (2)

JVET-H0030 BD-Rate/BD-PSNR Excel extensions [A. M. Tourapis, D. Singer, Y. Su, K. Mammou (Apple Inc)]

This contribution was discussed Friday October 20 at 1440 (chaired by GJS & JRO).

(This was also submitted to the JCT-VC as JCTVC-AC0026.)

This contribution provides some extensions to the JCT-VC/JVET Excel template for the computation of BD-rate and BD-PSNR numbers. In particular, the new formulation enables computation of BD-rate and BD-PSNR numbers for data sets larger than 4-points, can report “coverage” percentages between two data sets, and can also provide the ability to dynamically extrapolate the curves so as to guarantee their overlap. The ability to also compute “region of interest” measurements is also provided.

The provided scheme supports extrapolation (as a mode) and any number of test points and “gaps” where some points are missing, and arbitrary ordering of the test points. It also supports “ROI” results – i.e., results within a specific range.

It was agreed to plan to use this in the CfP spreadsheet template.

It was commented that the CfP will be focused on bit rate matching, not quality matching, which may cause difficulties with trying to calculate bit rate savings comparisons.

Decision (CfP): It was agreed to add an additional higher rate point for the HM to increase the overlap coverage range.

JVET-H0047 Perceptually optimized QP adaptation and associated distortion measure [S. Bosse, C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)]

The peak signal-to-noise ratio (PSNR) measure has been reported to show a lower correlation with results of corresponding subjective evaluations than some other measures. Moreover, it is also known that constant quantization parameter (QP) assignments frequently lead to visually suboptimal results. To reduce this divergence between objective measurements and subjective quality assessments, a generalization of PSNR is introduced, called perceptually weighted PSNR (WPSNR) together with a corresponding block-wise adaptive QP selection method that is optimized for subjective quality.

The suggested block-wise QP adaptation at CTU granularity reportedly provides significant improvements in subjective quality compared to a constant QP assignment for each picture as illustrated by a subjective test and examples. These subjective improvements result in PSNR decreases, but in corresponding WPSNR increases. Moreover, WPSNR reportedly shows high correlation with results of subjective quality assessments similar to other perceptually motivated measures such as (MS-)SSIM, VIF, or FSIM.

The weights of the WPSNR measure are derived via the impact of the QP adaptation method on the block-wise Lagrange parameters used in the Lagrangian bit allocation. Hence, as for the combination of a picture-wise constant QP assignment and PSNR, the combination of adaptive QP assignment and the corresponding WPSNR measure lead to operating points on the convex hull of the rate distortion space. Therefore, when due to a coding tool an improvement can be determined for the combination constant-QP/PSNR, a similar improvement can be determined for the combination of adaptive-QP with WPSNR.

It is suggested that JVET considers test conditions for their standardization work that use an adaptive QP assignment (for improved subjective quality) and an associated WPSNR measure.

This contribution was discussed Sunday 22 October at 0910 (chaired by GJS & JRO).

The peak signal-to-noise ratio (PSNR) measure has been reported to show a lower correlation with results of corresponding subjective evaluations than some other measures. It is also known that constant quantization parameter (QP) assignments frequently lead to visually suboptimal results. To reduce this divergence between objective measurements and subjective quality assessments, a generalization of PSNR is described, called perceptually weighted PSNR (WPSNR) together with a corresponding block-wise adaptive QP selection method that is optimized for subjective quality.

The suggested block-wise QP adaptation at CTU granularity reportedly provides significant improvements in subjective quality compared to a constant-QP assignment for each picture as illustrated by a subjective test and examples. These subjective improvements result in PSNR decreases, but in corresponding WPSNR increases. Moreover, WPSNR reportedly shows high correlation with results of subjective quality assessments similar to other perceptually motivated measures such as SSIM/MS-SSIM, VIF, or FSIM.

The weights of the WPSNR measure are derived via the impact of the QP adaptation method on the block-wise Lagrange parameters used in the Lagrangian bit allocation. Hence, as for the combination of a picture-wise constant QP assignment and PSNR, the combination of adaptive QP assignment and the corresponding WPSNR measure lead to operating points on the convex hull of the rate distortion space. Therefore, when, due to a coding tool, an improvement can be determined for the combination constant-QP/PSNR, a similar improvement can be determined for the combination adaptive-QP/WPSNR.

The scheme was described as being compatible overall with our usual RDO encoding processing.

Given the asserted advantages of WPSNR over PSNR and the strong similarity between the two methods (one is the generalization of the other), the contributor suggested that JVET considers test conditions for standardization work that use an adaptive QP assignment (for improved subjective quality) and an associated WPSNR measure.

The presenter indicated that per-frame weighting as a function of the coding structure (e.g. the level in a B hierarchy) can be used with this measure.

The presenter also discussed the difference between pooling PSNR per frame by averaging or by pooling in the MSE domain and then applying the logarithm.

A particular way of computing the weight was described, based on the result of a high-pass filtering.

The overall amount of effect was suggested to be about 10% in terms of visual benefit for optimizing with this domain rather than in the PSNR domain. The proponent said it was sometimes more than 10% (e.g. for BQTerrace).

A “Spearman rank-order correlation” with MOS scores based on still pictures was reported to be similar to that for various other perceptually motivated distortion measures.

In the experiment, QP was adapted based on the typical coupling rule of QP and λ. (Note that changing QP requires some bits. It was also commented that in some uses, QP adaptation may only be applied for the most-referenced pictures in a B hierarchy.)

It was commented that:

- Unlike some other metrics, the optimization of this measure seems straightforward to incorporate into our encoding process.

- It was commented that still picture results may not be appropriate for video. The proponent, however, said testing for video had shown the scheme to be effective (with about a 10% benefit on average).

- It is important that the weighting is only a function only of the input data, not the coding results – i.e., that the metric does not depend on the results of the encoding process (so computing the weight as a function of prediction residual characteristics, for example, would not be appropriate).

- The overall magnitude of the weights affects the magnitude of the result, so it may be more difficult to intuitively compare weighted measure results when the weights are not the same for all points in the comparison. The proponent described an overall scaling factor method of dealing with this.

- Weights for 64x64 areas (excluding boundary regions) were tested. Weight could be computed on smaller block regions.

- It was suggested the weights could even be computed per sample rather than per block and then pooled to the relevant segmentation region at which an encoder is operating.

- The proponent said the coupling with QP/λ is a particularly important aspect of the effectiveness.

- Smooth blocks get minimal weight.

- It was commented that incorporating brightness as well as local variance may be beneficial.

- It was reported that the scheme seems to help most at middle bit rates, rather than very high or low bit rates.

- It was noted that adding noise would tend to increase the weight.

- Chroma can be included. As tested, the chroma was included – but using luma-based weighting.

- It was commented that even without adapting the QP, using a varying λ may provide some benefit. The proponent advocated using the QP adaptation.

- It was asked whether the scheme had been used to test coding tools, e.g., of the sort in JEM. The proponents said this had worked well in their experiments.

- It was suggested that picture characteristics measures could also be used to affect the operation.

- It was asked whether the same method had appeared in previous literature (although this does not necessarily matter in our context).

It was agreed to study this in the metric AHG.

A participant commented that it was desirable to test the scheme on coding tools to see how much of a difference it makes in the evaluated benefit of a coding tool.

It was noted that in the CfP, the SDR encoding is discouraged from using adaptive QP, and in HDR adaptation based on luma and chroma value is allowed but not variance, and in 360°, adaptation based on geometric location is allowed but not based on other characteristics.

It was suggested that this could become a part of future common test conditions if the benefit is validated.

It was suggested to consider having this enabled in the JEM (or HM) as an extra data point in the CfP testing. This could hypothetically just be planned later (e.g. in January), as it is just a collection of more data rather than a change of the call conditions.

It is claimed that the benefit is roughly 10% bit rate saving for same quality.

The approach would require prescribing weighting factors per block per picture.

It is suggested to compute the weights per 64x64 block, based on a local activity criterion (it was verbally reported that in terms of measurement this does not make a difference; however visually it may make a difference).

QP is locally adapted based on the weight.

Chroma is following luma; weights are only based on luma.

It was pointed out that the wPSNR in HDR is similar, but more granular in computing the weight – some concern was expressed on the weighting based on 64x64 areas, which seems to be not granular enough.

It is reportedly mostly effective at low to medium bit rates.

It was asked whether this might have problems with QP cascading in hierarchical B configurations, e.g. causing additional temporal pumping. This remains to be further investigated.

It was asked how is it reacting on noise and whether it differentiates noise from texture. It does not.

It was asked whether the method could also be used in combination with RDOQ, which seemed possible.

It was asked whether the method could use different block sizes depending on a hierarchy level? Potentially, this could be done (although for comparison purposes, the same weighting and thus the same hierarchy tree should be use din both the tested method and the reference), but was not investigated so far.

The measured gain when comparing tools seems similar as with conventional PSNR. In this context, the question was thus also raised why it is the useful to change the metric.

Comparison to visual quality was only performed based on still images. Testing with video would certainly be needed.

Performance may also be highly dependent on the way how QP is signalled, e.g. for a method that signals QP changes more efficiently, finer granularity might clearly be useful.

Further study (in AHG10) was planned.

It was discussed whether to potentially put this as additional test case in CfP (both for the HM and JEM). This would, however, mean that it would violate conditions of CfP that prohibit adaptive QP for the SDR case. This was planned to be further discussed at the next meeting.

# Joint Meetings, BoG Reports, and Summary of Actions Taken

## Joint meetings

The draft CfP was presented in a joint meeting with the parent bodies Monday 23 October during 1630–1800. The proposed terms of reference (ToR) for the establishment of the "Joint Video Experts Team" as a formal joint collaborative team to be tasked with the development of a future video coding standard was also presented. The CfP and ToR were both formally approved by the parent bodies.

## BoGs (2)

JVET-H0095 BoG Report: Extended Colour Volume and High Dynamic Range [A. Segall]

This is a report of the Breakout Group on Extended Colour Volume and High Dynamic Range that met during the 8th meeting. The mandates of the group were as follows:

- Select HLG sequences

- Work out exactly what to say about out-of-loop remapping for the CfP

- Conduct an initial review of JVET-H0055 objective metrics for HDR

- Review (but not for planned action in relation to the CfP), experimental results of tone mapping.

The BoG met on October 19th and October 20th. Some aspects of the BoG discussions are recorded elsewhere in this report.

The BoG reported Sat. Oct. 21st 0900–1000.

On selection of HLG sequences: For subjective testing, 4 would be needed, but only 4 are available, and it was concluded in the last meeting that two of them might not be best suitable for subjective viewing. Further viewing was performed later (see the outcome below).

Options might be:

- only perform objective tests on HLG

- perform subjective tests and see in how far differences are visible (if not, we may still decide only considering sequences where benefit over HEVC is visible)

Regarding the second bullet, there is still uncertainty whether subjective tests on UHD-HDR are possible at all, depending on display availability. For HD-HDR, the Sim2 display is planned to be used, but this is not capable of displaying UHD. It was later clarified that also testing of 4K content using Sony BVM300 will definitely be possible.

The following constraint was recommended to be added to CfP:

A colour volume transform within the YCbCr 4:2:0 sample domain may be performed on the input video signal prior to coding, and an inverse of the colour volume transform may be performed on the signal after decoding. A colour volume transform modifies a luma or chroma sample at a relative location in the picture only based on the value of the luma or chroma sample and on the value of the other component samples located at the same or closest relative location. The colour volume transform should be static within a frame and sequence. The use of a colour volume transform for the purpose of rate allocation or denoising is discouraged. If a colour volume transform is used, a description of the transform shall be provided in the descriptive document submission. Respondents are additionally asked to provide descriptions of any multi-pass processing and any dynamic change in the colour volume transform.

This was confirmed by the JVET plenary.

The BoG suggests the following formulation regarding metrics:

- Submissions for Class HDR-HD will be evaluated by means of a formal subjective assessment.

- Submissions for Class HDR-HD will be further evaluated by the following metrics: PSNR, weighted PSNR, deltaE100 and PSNR-L100, as well as the Bjøntegaard Delta-Rate and Delta-Distortion for each metric. Metric definitions are provided in Annex D of the CfP document.

This was confirmed by JVET plenary, but another formulation was added clarifying that subjective criteria are anticipated to have primary importance, and it is up to committee consensus deciding how criteria are prioritized.

Another viewing session had been held on Sunday where the Birds sequence was compared with and without a chroma QP offset. It was clearly obvious that the chroma artefacts disappeared in the version with QP offset. However, the Birds sequence mostly showed chroma artefacts at low rates, whereas for other sequences also texture artefacts were more obvious. Generally, the benefit of the chroma QP offset for the Birds sequence for subjective testing seems to be lowest among all sequences.

It was decided in the Sunday afternoon JVET plenary that 3 HLG sequences (SunsetBeach, DayStreet and ShoppingMall) with the rate points defined in JVET-H0068 and JVET-H0038 should be included in the CfP. HM anchors with modified Chroma QP offset were requested to be generated by Tuesday, and based on another viewing session the final decision about anchors was made.

For HDR category submissions, both (HD/PQ=B and UHD/HLG=A) classes will be mandatory for submissions.

Classes will be called SDR-A/B, HDR-A/B and 360°.

The BoG was to meet again on some lower-priority items.

The same rules of allowing quantizer adaptation should apply for both HDR classes (though the anchors are not using it for HDR-A, which is due to the fact that previous trials of doing it did not unveil any subjective benefit).

JVET-H0096 Report of BoG on recommended CfP questionnaire [M. Zhou, E. Alshina]

This BoG was mandated to develop a questionnaire that would be helpful for gathering information of coding tools from CfP responses. The recommended questionnaire is not intended to assume any specific encoding architecture; it rather focuses on collecting basic design parameters of coding tools.

The report of the BoG was initially presented and discussed in the JVET plenary on Saturday 21 October during 1000–1050.

Some concern was expressed requiring too much detail of complexity at the tool level. It was confirmed that complexity information should rather be given on the codec level, and some tool-level questions could be answered optionally.

The questionnaire is intended to become the basis of a template document for proposal descriptions, such that better identification and clustering of proposals would be possible from CfP responses.

The BoG met again after this initial review.

It was suggested to put the recommended questionnaire into a template document “Description of video coding technology proposal by [proponent]” instead of in the “CfP text” as an annex. Proponents of CfP responses are to be required to answer the codec-level questionnaire, and are also encouraged to fill out the optional tool-level questionnaire in addition to the complexity analysis of their proposals. This suggestion was generally agreed by the group and later confirmed in the JVET plenary.

The main purpose of the questionnaire is to get a unified and condensed overview of the main properties of proposals.

The encoder and decoder runtime are already part of the Excel templates, and do not appear in the questions.

The following questions were agreed to be included

Codec level questions

- Maximum number of reference frames used by encoder

- Maximum coding unit size (e.g. CTU in HEVC)

- Minimum and maximum transform block size

- Minimum and maximum intra prediction block size

- Minimum and maximum inter uni- and multi-hypothesis prediction block size

- List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools (tool on/off tests, optional)

- List all the tools that require access of the neighbouring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, divisions, sample by sample recursive prediction, and irregular memory access

- List all the tools that use non-rectangular (e.g. triangle) partitions

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

- Indicate whether luma and chroma use separate partitioning structures

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

- Describe the maximum filter tap length (both luma and chroma) used for motion compensation

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

Tool level questions

- Entropy coding

  - Total number of context models

  - Memory storage size for each context model

  - Range table size if table look up is used for range update

  - Initialization table size

  - Other memory associated with context model adaptation

- Decoder side motion vector derivation and refinement

  - Temporal motion data storage granularity

  - Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2)

  - Line buffer size in terms of motion vector block lines (assuming horizontal motion vector block size is 4 luma samples)

  - Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the MV refinements

  - Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the worst search window size

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines

  - List all the filters used for intra prediction

  - Indicate whether there are cross-component dependencies in intra prediction reconstruction

- Inter prediction

  - Motion vector accuracy

  - Storage size for a motion vector component

  - List all the interpolation filters used for Luma/Chroma motion compensation

- Quantization

  - Storage size associated with quantization

- Transforms

- List all the combinations of transform block size and transform type for primary transforms

- List all the combinations of transform block size and transform type for secondary transforms

- Indicate whether transforms can be fully/partially implemented using direct matrix multiply

- Indicate whether transforms need multiple passes

- In-loop filters

- List line buffer size in terms of Luma and Chroma lines (if any)

- List all the filters used for in-loop filtering

- Minimum processing unit size

It was agreed to issue a draft of the template document as an output of this meeting, with the questionnaire as an annex. The BoG met again Sunday 22 October at 1800 to further work on refinement of the draft.

## List of actions taken affecting JEM7 and 360lib5

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the JEM7 or 360Lib4.0 description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

This content was presented and confirmed to be complete Tuesday 24th in the JVET plenary.

### Encoder only or CTC/software changes

The new CTC (JVET-H1010) reflects updates of Excel sheets (as per JVET-H0030).

General: It was agreed to ordinarily report two digits past the decimal point of percentage BD impacts. The basis for revisiting the last meeting’s decision in this regard is that some confusing cases of reports were found when rounding was made.

### Syntax/semantics/decoding process changes

No changes of JEM syntax, semantics, or decoding process were made at this meeting.

### Changes in 360lib

JVET-H0056 AHG8: An Update on RSP Projection [A. Abbas, D. Newman (GoPro)]

Decision (360lib): Replace the current RSP by the method from JVET-H0056.

General: It had been agreed by the 7th JVET meeting that the list of projection formats included in the CTC & 360Lib will not grow further, to avoid having so many that we can’t properly study them. If we want to add one, we need a decision to remove one. Anchors for projection formats are to be made available only with the HM and using ERP for the JEM. The outcome of this meeting is in line with this decision
.

# Project planning

## Exploration Experiment planning

No EEs were established.

## JEM description drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- JEM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be Thursday 11 January 2018.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name (as the focus in our work is on the technology rather than company business considerations).

## General issues for Experiments

Note: This section was drafted during the second JVET meeting, and is kept here for information about the EE procedure. It may become relevant in the future again.

Group coordinated experiments have been planned. These may generally fall into one category:

- “Exploration experiments” (EEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the main branch of JEM by the next meeting.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. (E. Alshina will edit the document based on input from the proponents, review is performed in the plenary)

- Software for tools investigated in EE is provided in a separate branch of the software repository

- During the experiment, further improvements can be made

- By the next meeting it is expected that at least one independent party will report a detailed analysis about the tool, confirms that the implementation is correct, and gives reasons to include the tool in JEM

- As part of the experiment description, it should be captured whether performance relative to JEM as well as HM (with all other tools of JEM disabled) should be reported by the next meeting.

It is possible to define sub-experiments within particular EEs, for example designated as EEX.a, EEX.b, etc., where X is the basic EE number.

As a general rule, it was agreed that each EE should be run under the same testing conditions using one software codebase, which should be based on the JEM software codebase. An experiment is not to be established as a EE unless there is access given to the participants in (any part of) the TE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-G1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a company proponent perspective – e.g. referring to methods as “improved”, “optimized” etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to EE work should identify individuals in addition to company names.

EE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the EE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish an EE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to EE activities were established as follows:

- Only qualified JVET members can participate in an EE.

- Participation in an EE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the EE should be announced and made available to all EE participants in a timely manner.

A separate branch under the experimental section will be created for each new tool include in the EE. The proponent of that tool is the gatekeeper for that separate software branch. (This differs from the main branch of the JEM, which is maintained by the software coordinators.)

New branches may be created which combine two or more tools included in the EE document or the JEM. Requests for new branches should be made to the software coordinators.

It is not necessary to formally name cross-checkers in the EE document. To promote the tool to the JEM at the next meeting, we would want to see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of the value of the tool given any relevant tradeoffs.

## Software development and anchor generation

The planned timeline for software releases was established as follows:

- JEM7.1 will be released by 2017-10-25.

- Further versions may be released for additional bug fixing, as appropriate

Timeline of 360lib5.0: 2 weeks after the meeting (2017-11-10).

- Further versions may be released as appropriate for bug fixing.

CfP anchors will be updated as necessary (assigning the same responsibilities as from the 7th meeting)

HDR: NHK/Sony will provide (and verify) HDR-A anchors (by November 10)

- For SDR: HD/RA, HD/LD, UHD: Samsung/Qualcomm (no update necessary)

- For 360°: InterDigital/Samsung (Nov. 10)

- For HDR-B: Technicolor/Qualcomm (Nov. 10)

New HM anchors will be generated using HM 16.16. JEM anchors will be based on JEM 7.0.

# Output documents and AHGs

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JVET-H1000 Meeting Report of the 8th JVET Meeting [G. J. Sullivan, J.-R. Ohm] [2018-01-18] (near next meeting)

Intermediate versions of the meeting notes (d0 … d5) were made available on a daily basis during the meeting and periodically thereafter.

Remains valid – not re-issued: JVET-G1001 Algorithm description of Joint Exploration Test Model 7 (JEM7) [J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce]

This document remains in force (from the previous meeting).

JVET-H1002 Joint Call for Proposals on Video Compression with Capability beyond HEVC [A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki] [2017-10-24] (MPEG N17195)

In the closing plenary, various additional edits were approved; in particular aspects of the testing fee and payment were clarified.

JVET-H1003 Draft template of proposal description document [M. Zhou, J. Chen, E. François, P. Hanhart] [2017-11-17]

This includes Excel templates for the cases of SDR, HDR and 360° video for data to be provided by proponents. All anchor data will be included by the date of final availability.

JVET-H1004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 5 [Y. Ye, E. Alshina, J. Boyce] [2017-11-17] (MPEG N17197)

The only change is the new version of RSP from JVET-H0056.

JVET-H1010 JVET common test conditions and software reference configurations [K. Sühring, X. Li] [2017-11-10]

This reflects updates of Excel sheets (as per JVET-H0030).

JVET-H1020 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, D. Rusanovskyy] [2017-11-10]

This reflects updates of Excel sheets (as per JVET-H0030) and will

include the four HLG sequences Birds, DayStreet, SunsetBeach, ShoppingMall (these may not be the correct official names of these sequences, but should suffice for purposes of this meeting report).

JVET-H1030 JVET common test conditions and evaluation procedures for 360° video [E. Alshina, J. Boyce, A. Abbas, Y. Ye] [2017-11-17]

This reflects updates of Excel sheets (as per JVET-H0030) and a new RSP version (JVET-H0056) and

replacing the Landing sequence with the Landing2 sequence.

It was reminded that in cases where a JVET document is also made available as an MPEG output document, a version under the MPEG document header should also be generated. This version should be sent to GJS and JRO for upload.

AHGs established

| Title and email reflector | Chairs | Mtg |
| --- | --- | --- |
| CFP preparation (AHG1) (jvet@lists.rwth-aachen.de) - Raise awareness of, and distribute the Call for Proposals. - Coordinate the registration of proposals. - Coordinate distribution of test material and anchors to registered proponents. - Discuss and improve the proposal description template JVET-H1003. - Make logistic arrangements for the Call for Proposals. | J.-R. Ohm, G. Sullivan, V. Baroncini, M. Zhou | N |
| JEM algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Develop and propose improvements to JVET-G1001 Algorithm Description of Joint Exploration Test Model 7. - Gather and address comments for refinement of the document. - Coordinate with the JEM software development AHG to address issues relating to mismatches between software and text. | J. Chen (chair), E. Alshina, J. Boyce (vice chairs) | N |
| JEM software development (AHG3) (jvet@lists.rwth-aachen.de) - Continue development of the JEM software package. - Produce documentation of software usage for distribution with the software. - Prepare and deliver JEM reference configuration encodings according to JVET-H1010 common testing conditions, if necessary in case of bug fix fixing. - Coordinate with AHG on JEM model editing to identify any mismatches between software and text, and make further updates and cleanup to the software as appropriate. - Investigate the implementation of SCC coding tools in JEM. - Coordinate with AHG6 for integration of 360° video software. | X. Li, K. Sühring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of future video coding standards. - Identify and recommend appropriate test materials and corresponding test conditions for use in the development of future video coding standards. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Evaluate new test sequences, and prepare for the visual assessment and availability of viewing equipment in the next meeting. | V. Baroncini, T. Suzuki (co-chairs), J. Chen, J. Boyce, A. Norkin (vice chairs) | N |
| Memory bandwidth consumption of coding tools (AHG5) (jvet@lists.rwth-aachen.de) - Study the methodology of measuring decoder memory bandwidth consumption, including cache models. - Develop software tools for measuring both average and worst case of memory bandwidth. - Make analysis of memory bandwidth needs for examples of JEM coding tools. - Study the impact of memory bandwidth on specific application cases. | X. Li (chair), E. Alshina, R. Hashimoto, T. Ikai, H. Yang (vice chairs) | N |
| 360° video conversion software development (AHG6) (jvet@lists.rwth-aachen.de) - Prepare and deliver 360Lib-5.0 software version and common test condition configuration files according to JVET-H1030. - Generate CTC HM anchors for all projection formats, CTC JEM anchors for the padded ERP format, and finalize the reporting template for the common test conditions. - Produce documentation of software usage for distribution with the software. | Y. He (chair), K. Choi, V. Zakharchenko (vice chairs) | N |
| JEM coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Coordinate generation of HM and JEM anchors for the CfP. - Study and evaluate available HDR/WCG test content, including appropriate displays for subjective evaluation of the content. - Study objective metrics for quality assessment of HDR/WCG material. - Evaluate transfer function conversion methods. - Study and refine test conditions and anchors for the coding of HDR/WCG content. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. François, D. Rusanovskyy (vice chairs) | N |
| 360° video coding tools and test conditions (AHG8) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Study consistency of and potential improvements to the objective quality metrics in CTC. - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Produce and finalize JVET-H1004 algorithm descriptions of projection format conversion process and objective quality metrics in 360Lib. - Produce and finalize JVET-H1030 JVET common test conditions and evaluation procedures for 360° video. - Study coding tools dedicated to 360° video, and their impact on compression. - Study the effect of viewport resolution, field of view, and viewport speed/direction on visual comfort. - Study the impact of coding resolution vs original ERP resolution on coding efficiency. | J. Boyce (chair), A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye (vice chairs) | N |
| Entropy Coding Trade-offs (AHG9) (jvet@lists.rwth-aachen.de) - Study factors that affect computational complexity of probability estimation for entropy coding. - Identify and study new methods for probability estimation. - Provide initial complexity analysis of new methods, and coding gain trade-offs, - Discuss impact of eliminating context initialization dependence on slice type, when new estimation methods are employed. - Discuss potential impact of new entropy coding on current JEM RD decisions. | A. Said (chair), E. Alshina, S.-H. Kim (vice-chair) | N |
| Denoising and adaptive quantisation (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using adaptive quantization in context of HM and JEM coding. - Prepare HM and JEM bitstreams using adaptive QP matching the rates of the CfP, for comparison against the CfP anchors. - Perform visual quality assessment for cases using denoising filters, renoising, and adaptive quantisation. - Study objective error metrics for measuring small subjective compression efficiency improvements when adaptive quantisation is used. - Solicit input contributions demonstrating subjective benefits over the JEM 7 anchor. | R. Sjöberg (chair), E. Alshina, S. Ikonin, A. Norkin, T. Wiegand (vice chairs) | N |

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Tuesday or Wednesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–7.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Thursday or Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 8.5 meeting days).

In cases where high workload is expected for a meeting, an earlier starting date may be defined.

Some specific future meeting plans (to be confirmed) were established as follows:

- Sat. 20 Jan. – Fri. 26 Jan. 2018, 9th meeting under WG 11 auspices in Gwangju, KR.

- Tue. 10 Apr. – Fri. 20 Apr. 2018, 10th meeting under WG 11 auspices in San Diego, US.
(Note: Tue. 10 Apr. is planned only for cross-checking of bitstreams by CfP proponents)

- Tue. 10 – Wed. 18 July 2018, 11th meeting under ITU-T auspices in Ljubljana, SI.

- Thu. 4 – Fri. 12 Oct. 2018, 12th meeting under WG 11 auspices in Macao, CN.

The agreed document deadline for the 9th JVET meeting is Thursday 11 January 2018. Plans for scheduling of agenda items within that meeting remain TBA.

The Macao Convention & Exhibition Association was thanked for the excellent hosting of the 8th meeting of the JVET. NHK and GBTech were thanked for providing viewing equipment. GoPro and InterDigital were thanked for offering new test sequences.

It was clarified that additional viewing of HDR-A anchors will be performed in the afternoon of Tuesday 24 October, with decision about final selection of anchors included under AHG7 mandates.

The 8th JVET meeting was closed at approximately 1246 hours on Tuesday 24 October 2017.

# Annex A to JVET report:
List of documents

| JVET number | MPEG number | Created | First upload | Last upload | Title | Authors |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-H0001 | m41565 | 2017-10-12 14:50:43 | 2017-10-17 02:38:25 | 2017-10-17 02:38:25 | JVET AHG report: Tool evaluation (AHG1) | M. Karczewicz, E. Alshina |
| JVET-H0002 | m41647 | 2017-10-15 05:13:16 | 2017-10-18 00:24:58 | 2017-10-18 00:24:58 | JVET AHG report: JEM algorithm description editing (AHG2) | J. Chen, E. Alshina, J. Boyce |
| JVET-H0003 | m41578 | 2017-10-13 05:50:55 | 2017-10-17 09:19:10 | 2017-10-18 04:59:21 | JVET AHG report: JEM software development (AHG3) | X. Li, K. Sühring |
| JVET-H0004 | m41671 | 2017-10-16 10:21:25 | 2017-10-17 11:10:20 | 2017-10-17 11:10:20 | JVET AHG report: Test material (AHG4) | T. Suzuki, V. Baroncini, J. Chen, J. Boyce, A. Norkin |
| JVET-H0005 | m41579 | 2017-10-13 05:55:41 | 2017-10-17 09:24:37 | 2017-10-17 09:24:37 | JVET AHG report: Memory bandwidth consumption of coding tools (AHG5) | X. Li, E. Alshina, R. Hashimoto, T. Ikai, H. Yang |
| JVET-H0006 | m41664 | 2017-10-16 08:46:04 | 2017-10-16 08:48:01 | 2017-10-16 08:48:01 | JVET AHG Report: 360° video conversion software development (AHG6) | Y. He, V. Zakharchenko |
| JVET-H0007 | m41813 | 2017-10-18 02:24:01 | 2017-10-18 02:24:38 | 2017-10-18 04:44:39 | JVET AhG report: JEM coding of HDR/WCG material (AHG7) | A. Segall, E. François, D. Rusanovskyy |
| JVET-H0008 | m41776 | 2017-10-17 01:06:35 | 2017-10-17 20:30:56 | 2017-10-18 06:05:57 | JVET AHG report: 360° video coding tools and test conditions (AHG8) | J. Boyce, A. Abbas, E. Alshina, G. van der Auwera, Y. Ye |
| JVET-H0009 | m41818 | 2017-10-18 06:16:52 | 2017-10-18 06:17:08 | 2017-10-18 06:17:08 | JVET AHG report: 4:4:4 support in JEM (AHG9) | A. M. Tourapis, X. Li |
| JVET-H0010 | m41542 | 2017-10-11 11:09:14 | 2017-10-12 02:20:29 | 2017-10-17 15:42:18 | JVET AHG report: Denoising and adaptive quantisation | R. Sjöberg, E. Alshina, S. Ikonin, A. Norkin, T. Wiegand |
| JVET-H0011 | m41577 | 2017-10-13 03:17:55 | 2017-10-13 03:18:26 | 2017-10-18 05:12:00 | Exploration Experiments on Coding Tools Report | E. Alshina, L. Zhang |
| JVET-H0021 | m41403 | 2017-09-15 20:55:03 | 2017-09-16 00:21:33 | 2017-10-18 10:18:20 | AHG8: Test Sequences for Spherical Video Coding from GoPro | A. Abbas (GoPro) |
| JVET-H0022 | m41458 | 2017-10-06 00:12:18 | 2017-10-06 19:03:10 | 2017-10-18 04:00:40 | AHG8: New Sequences for Virtual Reality Video Coding from InterDigital | Y. He, P. Hanhart, E. Asbun, Y. He, Y. Ye (InterDigital) |
| JVET-H0023 | m41468 | 2017-10-09 13:42:35 | 2017-10-09 13:45:09 | 2017-10-19 01:13:22 | Implementation and design aspects of xvc | J. Samuelsson, P. Hermansson (Divideon) |
| JVET-H0024 | m41470 | 2017-10-09 14:29:42 | 2017-10-10 01:10:49 | 2017-10-10 01:10:49 | EE1: Improvements for Intra Prediction Mode Coding | Yu Han, Jicheng An, Jianhua Zheng (Hisilicon) |
| JVET-H0025 | m41471 |  |  |  | Withdrawn |  |
| JVET-H0026 | m41472 |  |  |  | Withdrawn |  |
| JVET-H0027 | m41479 | 2017-10-09 20:30:33 | 2017-10-11 05:52:39 | 2017-10-11 05:52:39 | EE1: PDPC without a mode flag | V. Seregin, M. Karczewicz, A. Said, X. Zhao (Qualcomm) |
| JVET-H0028 | m41480 | 2017-10-09 20:30:50 | 2017-10-11 02:52:49 | 2017-10-11 02:52:49 | EE1: Fix for strong intra smoothing filtering | V. Seregin, X. Zhao, M. Karczewicz (Qualcomm) |
| JVET-H0029 | m41481 | 2017-10-09 20:31:25 | 2017-10-11 02:50:25 | 2017-10-11 02:50:25 | EE1: Secondary MPM list | V. Seregin, W.-J. Chien, M. Karczewicz, N. Hu, X. Zhao (Qualcomm) |
| JVET-H0030 | m41482 | 2017-10-09 21:44:52 | 2017-10-11 23:57:00 | 2017-10-12 22:04:29 | BD-Rate/BD-PSNR Excel extensions | A. M. Tourapis, D. Singer, Y. Su, K. Mammou (Apple Inc) |
| JVET-H0031 | m41484 | 2017-10-10 00:32:07 | 2017-10-10 00:34:53 | 2017-10-20 06:04:39 | Inter Prediction using Estimation and Explicit Coding of Affine Parameters | C. Heithausen, J.-R. Ohm |
| JVET-H0032 | m41485 |  |  |  | Withdrawn |  |
| JVET-H0033 | m41486 | 2017-10-10 01:13:40 | 2017-10-10 01:16:05 | 2017-10-10 01:16:05 | EE1: Crosscheck of tests 8 and 9 | Y. Han, J. An, J. Zheng (Hisilicon) |
| JVET-H0034 | m41487 | 2017-10-10 01:49:57 | 2017-10-10 09:22:29 | 2017-10-18 09:43:13 | A simplified method to sao_type_idx coding of sample adaptive offset (SAO) | X. Zhang, Z. Zhu, J. Fan, X. Jin, E. Zhang (Kingsoft Cloud) |
| JVET-H0035 | m41488 | 2017-10-10 04:17:48 | 2017-10-10 04:33:57 | 2017-10-18 03:56:17 | AHG8: The guidance of WS-PSNR weights derivation for different formats | Yule Sun, Lu Yu (Zhejiang Univ.) |
| JVET-H0036 | m41489 | 2017-10-10 04:38:11 | 2017-10-11 02:00:48 | 2017-10-16 01:32:37 | Condition modification of Strong Intra Smoothing Filter | Y. Kidani, K. Kawamura, S. Naito (KDDI) |
| JVET-H0037 | m41490 | 2017-10-10 07:45:10 | 2017-10-10 08:28:55 | 2017-10-10 08:28:55 | EE3.7: chroma ALF on/off flag | T. Ikai (Sharp), J. An, J. Zheng (HiSilicon) |
| JVET-H0038 | m41493 | 2017-10-10 08:18:27 | 2017-10-10 09:12:08 | 2017-10-16 04:13:17 | AHG7: Test Sequences of 4K Hybrid Log-Gamma | T. Tsukuba, M. Ikeda, T. Suzuki (Sony) |
| JVET-H0039 | m41494 | 2017-10-10 10:29:06 | 2017-10-13 07:21:51 | 2017-10-19 13:01:59 | AHG5: On worst case memory bandwidth | T. Ikai (Sharp), R. Hashimoto, S. Mochizuki (Renesas) |
| JVET-H0040 | m41492 |  |  |  | Withdrawn |  |
| JVET-H0041 | m41495 | 2017-10-10 11:48:28 | 2017-10-11 03:22:25 | 2017-10-16 10:51:08 | EE3 Test1-6: Unified Adaptive Loop Filter for Luma and Chroma | J. An, J. Zheng (HiSilicon) |
| JVET-H0042 | m41496 | 2017-10-10 11:57:48 | 2017-10-11 04:10:45 | 2017-10-11 04:10:45 | EE1: Cross-check of test 6 | H. Ko, H. Lee, J. Kang (ETRI) |
| JVET-H0043 | m41498 | 2017-10-10 14:25:55 | 2017-10-10 23:28:17 | 2017-10-13 07:13:01 | AHG5: How to measure memory bandwidth | R. Hashimoto, S. Mochizuki (Renesas), T. Ikai (Sharp) |
| JVET-H0044 | m41499 | 2017-10-10 14:45:28 | 2017-10-15 14:12:02 | 2017-10-15 14:12:02 | Do we need multiview profiles for future video coding generations ? | J. Samelak, O. Stankiewicz, M. Domanski (Poznan Univ.) |
| JVET-H0045 | m41500 | 2017-10-10 15:07:14 | 2017-10-11 00:11:53 | 2017-10-17 12:24:04 | EE3: Test on Chroma QP offset | S. Ikonin, R. Chernyak, T. Solovyev, S. Liu (Huawei) |
| JVET-H0046 | m41509 | 2017-10-10 19:43:05 | 2017-10-15 21:10:39 | 2017-10-17 18:57:43 | EE3: Cross-check of Test 6 (JVET-H0041) | L. Zhang, W.-J. Chien (Qualcomm) |
| JVET-H0047 | m41510 | 2017-10-10 19:44:32 | 2017-10-10 19:58:39 | 2017-12-06 16:37:08 | Perceptually optimized QP adaptation and associated distortion measure | S. Bosse, C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI) |
| JVET-H0048 | m41511 | 2017-10-10 19:57:44 | 2017-10-10 20:05:23 | 2017-10-16 20:08:11 | AhG8: Dynamic viewport based subjective evaluation of Balboa and Landing | P. Hanhart, Y. He, Y. Ye (InterDigital) |
| JVET-H0049 | m41512 | 2017-10-10 19:59:38 | 2017-10-10 20:05:48 | 2017-10-16 20:08:49 | AhG8: On viewport size and field of view | P. Hanhart, Y. He, Y. Ye (InterDigital) |
| JVET-H0050 | m41513 | 2017-10-10 20:27:39 | 2017-10-12 17:58:48 | 2017-10-20 11:14:12 | AhG8: Measurement of User Exploration Behavior for Omnidirectional (360°) Videos with a Head Mounted Display | A. Singla, S. Fremerey, A. Raake (TU Ilmenau), P. List, B. Feiten (Deutsche Telekom) |
| JVET-H0051 | m41517 | 2017-10-10 21:15:39 | 2017-10-10 21:20:24 | 2017-10-16 21:50:59 | Non-EE1: Priority List Based Intra Mode Coding with 5 MPM | Y. Yu, L. Wang, K. Panusopone (ARRIS) |
| JVET-H0052 | m41518 | 2017-10-10 21:22:23 | 2017-10-10 22:15:37 | 2017-10-10 22:27:31 | EE1 Tests 2 and 3: Replace Planar mode in P-PDPC with UWP and Enable UW66 on top of the above test | S. Hong, K. Panusopone, Y. Yu, L. Wang (Arris), H. M. Jang, J. Lim, S.-H. Kim (LGE) |
| JVET-H0053 | m41519 | 2017-10-10 21:24:00 | 2017-10-10 22:15:52 | 2017-10-18 10:58:41 | Weighted Angular Mode 2 and Adaptive Angular Mode 66 | K. Panusopone, Y. Yu, S. Hong, L. Wang (Arris) |
| JVET-H0054 | m41520 | 2017-10-10 22:37:39 | 2017-10-14 01:21:15 | 2017-10-14 01:21:15 | EE3: Cross-check of Test 7 additional results (JVET-H0037) | L. Zhang (Qualcomm) |
| JVET-H0055 | m41521 | 2017-10-10 22:43:40 | 2017-10-10 22:56:16 | 2017-10-21 02:49:18 | Analysis of the HDR Objective Metrics Utility in Draft CfP on FVC | P. Yin, T. Chen, W. Husak, F. Pu, T. Lu (Dolby) |
| JVET-H0056 | m41525 | 2017-10-10 23:49:30 | 2017-10-11 04:16:43 | 2017-10-18 11:56:43 | AHG8: An Update on RSP Projection | A. Abbas (GoPro) |
| JVET-H0057 | m41528 | 2017-10-11 00:13:27 | 2017-10-11 07:03:07 | 2017-10-18 08:43:36 | EE1 related: Simplification and extension of PDPC | X. Zhao, V. Seregin, A. Said, M. Karczewicz (Qualcomm) |
| JVET-H0058 | m41529 | 2017-10-11 02:03:37 | 2017-10-11 03:33:32 | 2017-10-19 09:22:09 | Unified search range for FRUC | H. Jang, J. Nam, J. Lim, S.-H. Kim (LGE) |
| JVET-H0059 | m41530 | 2017-10-11 02:37:14 | 2017-10-11 03:34:06 | 2017-10-18 10:25:55 | EE1-related : Two unifications for PDPC | H. Jang, J. Nam, J. Lim, S.-H. Kim (LGE) |
| JVET-H0060 | m41531 | 2017-10-11 02:38:51 | 2017-10-11 02:48:22 | 2017-10-23 08:42:10 | AHG4: SDR anchor generation for the Draft Joint Call for Proposals | H.-C. Chuang, J. Chen, M. Karczewicz (Qualcomm), K. Choi, E. Alshina (Samsung) |
| JVET-H0061 | m41532 | 2017-10-11 02:47:02 | 2017-10-11 02:57:35 | 2017-10-11 02:57:35 | EE2: Arithmetic coding with context-dependent double-window adaptation response | A. Said, M. Karczewicz, V. Seregin, H. Egilmez, L. Zhang, X. Zhao |
| JVET-H0062 | m41533 | 2017-10-11 02:48:35 | 2017-10-11 08:14:11 | 2017-10-11 08:14:11 | EE1: Cross-check of Test 7 | D. Jun, H. Lee, J. Kang (ETRI) |
| JVET-H0063 | m41534 | 2017-10-11 02:59:17 | 2017-10-11 08:14:35 | 2017-10-11 08:14:35 | EE3: Cross-check of Test 3 | D. Jun, H. Lee, J. Kang (ETRI) |
| JVET-H0064 | m41535 | 2017-10-11 04:01:13 | 2017-10-16 16:23:39 | 2017-10-16 16:23:39 | Crosscheck of EE3 Test 2 of JVET-H0041 | X. Zhang (Kingsoft Cloud) |
| JVET-H0065 | m41536 | 2017-10-11 04:26:15 | 2017-10-16 03:57:26 | 2017-10-16 03:57:26 | EE3: Crosscheck of Test7 | J. An (HiSilicon) |
| JVET-H0066 | m41537 | 2017-10-11 04:27:55 | 2017-10-17 14:17:01 | 2017-10-17 14:17:01 | Crosscheck of JVET-H0034 A simplified method to sao_type_idx coding of SAO | J. An (HiSilicon) |
| JVET-H0067 | m41538 | 2017-10-11 08:02:23 | 2017-10-11 08:06:00 | 2017-10-19 06:03:46 | EE2 related: Arithmetic coding with progressive context-dependent double-window adaptation response | A. Said, H. Egilmez, M. Karczewicz, V. Seregin, L. Zhang, X. Zhao |
| JVET-H0068 | m41543 | 2017-10-11 11:22:13 | 2017-10-11 11:24:14 | 2017-10-11 11:24:14 | AHG7: HLG rate points for CfP anchor generation | S. Iwamura, S. Nemoto, A. Ichigaya (NHK) |
| JVET-H0069 | m41547 | 2017-10-11 12:15:17 | 2017-10-13 06:07:34 | 2017-10-13 06:07:34 | EE2: Cross-check for Arithmetic coding with context-dependent double-window adaptation response | E. Alshina (Samsung) |
| JVET-H0070 | m41548 | 2017-10-11 12:26:11 | 2017-10-13 13:35:32 | 2017-10-17 06:15:48 | EE3: Cross-check for Chroma QP offset 0 and 2 (tests 1 and1.1) | E. Alshina |
| JVET-H0071 | m41549 | 2017-10-11 13:47:13 | 2017-10-11 13:55:38 | 2017-10-19 02:46:36 | Redundant flag removal on chroma intra mode coding | S. Iwamura, S. Nemoto, A. Ichigaya (NHK) |
| JVET-H0072 | m41559 | 2017-10-12 06:30:00 | 2017-10-12 10:04:33 | 2017-10-19 08:52:19 | Modification of merge candidate derivation for binary split CUs | Y. Ahn, H. Ryu, D. Sim (Digital Insights), J. Lim, D. Jeon (Kaonmedia) |
| JVET-H0073 | m41561 | 2017-10-12 08:40:58 | 2017-10-14 11:59:01 | 2017-10-14 11:59:14 | EE3: Cross-check of Test4 and 5 | Ryuichi Kanoh (Panasonic) |
| JVET-H0074 | m41576 | 2017-10-13 02:56:15 | 2017-10-17 12:51:29 | 2017-10-17 12:51:29 | EE1-related: Cross-check of Simplification and extension of PDPC (JVET-H0057) | H. Lee, H. Ko, J. Kang (ETRI) |
| JVET-H0075 | m41596 | 2017-10-13 10:59:20 | 2017-10-16 12:04:21 | 2017-10-18 02:36:07 | Non-EE1: Crosscheck of Priority List Based Intra Mode Coding with 5 MPM (JVET-H0051) | T. Ikai (Sharp) |
| JVET-H0076 | m41601 | 2017-10-13 11:49:07 | 2017-10-13 11:55:22 | 2017-10-13 11:55:22 | EE1: Crosscheck of test 1 | V. Drugeon (Panasonic) |
| JVET-H0077 | m41602 | 2017-10-13 12:00:36 | 2017-10-18 03:29:04 | 2017-10-19 03:43:04 | Cross-check of JVET-H0058 | K. Choi (Samsung) |
| JVET-H0078 | m41603 | 2017-10-13 12:05:05 | 2017-10-18 03:29:38 | 2017-10-19 03:43:49 | Cross-check of JVET-H0059 | K. Choi (Samsung) |
| JVET-H0079 | m41607 | 2017-10-13 13:47:44 | 2017-10-16 08:39:08 | 2017-10-17 06:39:07 | Cross-check of JVET-H0056 on an update on RSP Projection | S. N. Akula, C. Pujara (Samsung) |
| JVET-H0080 | m41598 | 2017-10-13 11:34:20 | 2017-10-13 16:29:31 | 2017-10-13 16:29:31 | EE1: Cross-check of Test 5 | E. Mora (Ateme) |
| JVET-H0081 | m41636 | 2017-10-14 06:40:18 | 2017-10-16 05:58:20 | 2017-10-19 11:13:55 | Cross-check of JVET-H0072 “Modification of merge candidate derivation for binary split CUs” | Y. U. Yoon, D. H. Park, J.-G. Kim (KAU) |
| JVET-H0082 | m41637 | 2017-10-14 07:14:22 | 2017-10-14 07:23:19 | 2017-10-20 13:37:49 | Comments on HDR category test conditions of draft CfP on video compression beyond HEVC | D. Rusanovskyy, M. Karczewicz (Qualcomm Inc.), E. François, M. Kerdranvat (Technicolor), C. Fogg, J. Helman (MovieLabs) |
| JVET-H0083 | m41662 | 2017-10-16 06:13:55 | 2017-10-16 08:16:50 | 2017-10-16 08:16:50 | AHG7: Cross-check of test sequences of 4K Hybrid Log-Gamma transfer characteristics (JVET-H0038) | S. Nemoto, S. Iwamura, A. Ichigaya (NHK) |
| JVET-H0084 | m41687 | 2017-10-16 11:52:40 | 2017-10-16 11:58:06 | 2017-12-06 16:44:43 | NextSoftware: An alternative implementation of the Joint Exploration Model (JEM) | A. Wieckowski, T. Hinz, V. George, J. Brandenburg, J. Ma, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI) |
| JVET-H0085 | m41719 | 2017-10-16 17:40:30 | 2017-10-16 18:11:25 | 2017-10-16 18:11:25 | EE2: Cross-check for JVET-G00112 Test 3.2 and 3.3 | M. Sychev (Huawei) |
| JVET-H0086 | m41743 | 2017-10-16 20:01:35 | 2017-10-18 03:58:30 | 2017-10-18 03:58:30 | Cross-check of JVET-H0036 on strong intra smoothing condition | V. Seregin (Qualcomm) |
| JVET-H0087 | m41778 | 2017-10-17 04:08:13 | 2017-10-17 04:12:52 | 2017-10-19 08:49:51 | Diagonal motion partitions on top of QTBT block structure | Y. Ahn, H. Ryu, D. Sim (Digital Insights) |
| JVET-H0088 | m41797 | 2017-10-17 16:28:51 | 2017-10-17 18:49:50 | 2017-11-24 12:37:40 | EE1: Crosscheck of tests 2 and 3 | A. Filippov, V. Rufitskiy, T. Solovyev (Huawei) |
| JVET-H0089 | m41799 | 2017-10-17 17:48:05 | 2017-10-17 17:57:18 | 2017-10-17 17:57:18 | Crosscheck of JVET-H0071 Redundant flag removal on chroma intra mode coding | P.-H. Lin, C.-L. Lin, C.-C. Lin (ITRI) |
| JVET-H0090 | m41814 | 2017-10-18 02:37:31 | 2017-10-20 15:22:39 | 2017-10-20 15:22:39 | EE1: Crosscheck of tests 1.1 | T. Ikai (Sharp) |
| JVET-H0091 | m41815 | 2017-10-18 03:24:03 | 2017-10-18 03:29:44 | 2017-10-19 10:47:43 | Restriction of fast intra-mode decision | P.-H. Lin, C.-L. Lin, C.-C. Lin, Y.-T. Tsai (ITRI) |
| JVET-H0092 | m41817 |  |  |  | Withdrawn |  |
| JVET-H0093 | m41859 | 2017-10-19 07:51:09 | 2017-10-19 10:02:23 | 2017-10-19 10:02:23 | AHG8: 360° anchor generation for the Draft Joint Call for Proposals | Y. He, Y. Ye (InterDigital), C. Pujara, K. Choi (Samsung) |
| JVET-H0094 | m41869 | 2017-10-20 13:09:07 | 2017-10-20 17:25:08 | 2017-10-21 03:08:25 | AHG7: Analysis of candidate HLG content | E. François, M. Kerdranvat |
| JVET-H0095 | m41872 | 2017-10-21 01:31:02 | 2017-10-21 01:32:35 | 2017-10-21 01:32:35 | BoG Report: Extended Colour Volume and High Dynamic Range | A. Segall |
| JVET-H0096 | m41874 | 2017-10-21 05:45:45 | 2017-10-21 06:00:19 | 2017-10-22 13:11:03 | Report of BoG on recommended CfP questionnaire | M. Zhou, E. Alshina |
| JVET-H0097 | m41877 | 2017-10-21 08:49:25 | 2017-10-21 08:52:56 | 2017-10-21 08:52:56 | AHG7 Cross-check of HLG rate points in JVET-H0068 | K. Kawamura, S. Naito (KDDI) |
| JVET-H0098 | m41878 | 2017-10-21 08:51:12 | 2017-10-21 08:53:22 | 2017-10-21 10:48:04 | AHG7 Cross-check of Test Sequence of 4K Hybrid Log-Gamma in JVET-H0038 | K. Kawamura, S. Naito (KDDI) |
| JVET-H0099 | m41879 | 2017-10-21 12:49:23 | 2017-10-21 12:50:37 | 2017-10-22 04:45:52 | Evaluation of Luma/Chroma QP Adaptation for HLG content | K. Kawamura, S. Naito (KDDI) |
| JVET-H0100 | m41881 | 2017-10-22 04:17:41 | 2017-10-22 06:03:18 | 2017-10-24 03:00:40 | Draft Excel template for Call for Proposals | J. Chen, E. François, P. Hanhart |
| JVET-H1000 | m41924 | 2017-11-08 21:12:40 | (this document) | (this document) | Meeting Report of the 8th JVET Meeting (1824 Oct. 2017, Macao, CN) | G. J. Sullivan, J.-R. Ohm |
| JVET-H1002 | m41646 | 2017-10-15 05:03:03 | 2017-10-23 09:34:52 | 2017-11-08 19:42:08 | Joint Call for Proposals on Video Compression with Capability beyond HEVC | A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki (editors) |
| JVET-H1003 | m41896 | 2017-10-24 18:02:38 | 2017-10-24 19:48:44 | 2017-12-23 19:11:55 | Draft template of proposal description document | M. Zhou, J. Chen, E. François, P. Hanhart (editors) |
| JVET-H1004 | m41925 | 2017-11-08 21:15:00 | 2017-11-14 20:34:47 | 2017-11-18 02:38:45 | Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 5) | Y. Ye, E. Alshina, J. Boyce (editors) |
| JVET-H1010 | m41926 | 2017-11-08 21:16:25 | 2017-12-08 12:52:50 | 2017-12-08 12:52:50 | JVET common test conditions and software reference configurations | K. Sühring, X. Li (editors) |
| JVET-H1020 | m41927 | 2017-11-08 21:33:23 | 2018-01-17 13:51:50 | 2018-01-17 13:51:50 | JVET common test conditions and evaluation procedures for HDR/WCG video | A. Segall, E. François, D. Rusanovskyy (editors) |
| JVET-H1030 | m41892 | 2017-10-24 05:32:29 | 2017-10-24 05:33:00 | 2017-10-24 05:33:00 | JVET common test conditions and evaluation procedures for 360° video | E. Alshina, J. Boyce, A. Abbas, Y. Ye (editors) |

# Annex B to JVET report:
List of meeting participants

The participants of the eighth meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately 195 people in total), were as follows:

- Adeel Abbas (GoPro)

- Yong-Jo Ahn (Digital Insights)

- Sri Nitchith Akula (Samsung)

- Elena Alshina (Samsung Electronics)

- Peter Amon (Siemens AG)

- Jicheng An (HiSilicon)

- Robert Byoungman An (Humax)

- Kenneth Andersson (LM Ericsson)

- Ichiro Ando (Nikon)

- Stefano Andriani (Arri)

- Daehyun Baek (Humax)

- Yukihiro Bandoh (NTT)

- Gun Bang (ETRI)

- Vittorio Baroncini (GBTech)

- Guillaume Barroux (Fujitsu Labs)

- Max Bläser (RWTH Aachen Univ.)

- Frank Bossen

- Jill Boyce (Intel)

- Benjamin Bross (Fraunhofer HHI)

- Wenting Cai (Fujitsu)

- Eric (Chi W.) Chai (Real Commun.)

- Yao-Jen Chang (Foxconn)

- Peisong Chen (Broadcom)

- Qinbo	Chen (DJI)

- Huanbang Chen (Huawei)

- Xu Chen (Huawei Tech.)

- Jianle Chen (Qualcomm)

- Yi-Wen Chen (Qualcomm)

- Jie Chen (Samsung)

- Kiho Choi (Samsung Electronics)

- Takeshi Chujoh (Sharp)

- Gordon Clare (Orange Labs FT)

- Zhipin Deng (Intel)

- Tianyu Dong (Hanyang Univ.)

- Virginie Drugeon (Panasonic)

- Alberto Duenas (ARM)

- David Flynn (Blackberry)

- Edouard François (Technicolor)

- Alexandre Gabriel (TNO)

- Wen Gao (Harmonic)

- Shan Gao (Huawei)

- Xin Wer Gao (Tencent)

- Christophe Gisquet (Canon Research France)

- Dae-hyeok Gwon (Hanbat Univ.)

- Jong-Ki Han (Sejong Univ.)

- Soo-Chul Han (Vidyo)

- Philippe Hanhart (InterDigital Commun.)

- Ryoji Hashimoto (Renesas)

- Tomonori Hashimoto (Sharp)

- Yun He (Tsinghua Univ.)

- Cordula Heithausen (RWTH Aachen Univ.)

- Christian Helmrich (Fraunhofer HHI)

- Christopher Hollman (Ericsson)

- Michael Horowitz (Sharp)

- Chih-Wei Hsu (MediaTek)

- Yu-Wen Huang (MediaTek)

- Xiaozheng Huang (Tencent)

- Cheng Huang (ZTE)

- Walt Husak (Dolby Labs)

- Hyunsu Hwang (Yonsei Univ.)

- Atsuro Ichigaya (NHK Japan Broadcasting Corp.)

- Tomohiro Ikai (Sharp)

- Masaru Ikeda (Sony)

- Sergey Ikonin (Huawei)

- Shunsuke Iwamura (NHK Japan Broadcasting Corp.)

- Hyeongmoon Jang (LG Electronics)

- Byeungwoo Jeon (Sungkyunkwan Univ. SKKU)

- Teresa Jiang (Foxconn)

- Jung Won Kang (Electronics and Telecom Research Institute ETRI)

- Ryuichi Kanoh (Panasonic)

- Jung-Yang Kao (ITRI)

- Kei Kawamura (KDDI)

- Michel Kerdranvat (Technicolor)

- Yoshitaka Kidani (KDDI)

- Jae-Gon Kim (Korea Aerosp. Univ.)

- Seung-Hwan Kim (LG Electronics)

- Hyosong Kim (Pixtree)

- Hyeong-Duck Kim (Pixtree)

- Myung Jun Kim (Sejong Univ.)

- Nam-Uk Kim (Sejong Univ. DMS Lab)

- Dongcheol Kim (Wilus)

- Geonjung Ko (Wilus)

- Krasimir Kolarov (Apple)

- Jin Sam Kwak (Wilus)

- Jani Lainema (Nokia)

- Junghyun Lee (Hanyang Univ.)

- Yongjae Lee (Humax)

- Bae-Keun Lee (KT)

- Sunyoung Lee (Pixtree)

- Jae Yung Lee (Sejong Univ.)

- Yung-Lyul Lee (Sejong Univ.)

- Shawmin Lei (MediaTek)

- Weiram Li (DJI)

- Ling Li (LG Electronics)

- Xiang Li (Qualcomm Tech.)

- Ru Ling Liao (Panasonic)

- Sung-Chang Lim (Electronics and Telecom Research Institute ETRI)

- Jeong Yun Lim (Kaon Media)

- Jaehyun Lim (LG Electronics)

- Chongsoon Lim (Panasonic)

- Jeongyeon Lim (SK Telecom)

- Vincent Lin (DJI)

- Po Han Lin (ITRI Intl.)

- Chun-Lung Lin (ITRI USA)

- Jian-Liang Lin (MediaTek)

- Hung-Chih Lin (MediaTek)

- Shan Liu (Huawei)

- Victorien Lorcy (B-Com)

- Xiao Mu Lu (Huawei)

- Ajay Luthra (Arris)

- Xiang Ma (Huawei)

- Gaëlle Martin-Cocher (Blackberry)

- Akira Minezawa (Mitsubishi Electric)

- Kiran Misra (Sharp)

- Karsten Müller (Fraunhofer HHI)

- Taeyoung Na (SK Telecom)

- Ohji Nakagami (Sony)

- Shimpei Nemoto (NHK)

- Benben Niu (Tsinghua Univ.)

- Andrey Norkin (Netflix)

- Hyun Mook Oh (LG Electronics)

- Jens-Rainer Ohm (RWTH Aachen Univ.)

- Jun Ono (Socionext)

- Jörn Ostermann (Univ. Hannover)

- Krit Panusopone (Arris)

- Dohyeon Park (Korea Aerosp. Univ.)

- Seanae Park (KWU)

- Jae Yoon Park (SKKU Univ.)

- Pierrick Philippe (Orange Labs FT)

- Magali Philippe (Sony Europe)

- Chirag Pujara (Samsung)

- Shouxing Qu (Blackberry)

- Fabien Racape (Technicolor)

- Justin Ridge (Nokia)

- Christopher Rosewarne (CiSRA / Canon)

- Dmytro Rusanovskyy (Qualcomm)

- Hochan Ryu (Digital Insights)

- Amir Said (Qualcomm Tech.)

- Jonatan Samuelsson (Divideon)

- Eiichi Sasaki (Socionext)

- Andrew Segall (Sharp)

- Jiyeong Seok (Humax)

- Vadim Seregin (Qualcomm)

- Karl Sharman (Sony Europe Broad. & Prof. Research Labs)

- Masato Shima (Canon)

- Jae-Seob Shin (Pixtree)

- Rickard Sjöberg (Ericsson)

- Sehoon Son (Pixtree)

- Juhyung Son (Wilus)

- Benno Stabernack (Fraunhofer HHI)

- Shiori Sugimoto (NTT)

- Karsten Sühring (Fraunhofer HHI)

- Gary Sullivan (Microsoft)

- Yule Sun (Zhejiang Univ.)

- Teruhiko Suzuki (Sony)

- Han Boon Teo (Panasonic)

- Herbert Thoma (Fraunhofer IIS)

- Emmanuel Thomas (TNO)

- Tadamasa Toma (Panasonic)

- Pankaj Topiwala (FastVDO)

- Alexandros Tourapis (Apple)

- Yi-Ting Tsai (ITRI)

- Chia-Yang Tsai (Real Networks)

- Yi-Shin Tung (ITRI USA / MStar Semi.)

- Geert Van der Auwera (Qualcomm)

- Glenn Van Wallendael (Univ. Ghent)

- Li Wang (HIK Vision)

- Zhao Wang (Peking Univ.)

- Liqiang Wang (Tsinghua Univ.)

- Stephan Wenger (Vidyo)

- Adam Wieckowski (Fraunhofer HHI)

- Thomas Wiegand (Fraunhofer HHI)

- Dongjae Won (Sejong Univ.)

- Yongjun Wu (Amazon)

- Ping Wu (ZTE UK)

- Zhanglei Xu (Fujitsu)

- Yong Yan (Polycom)

- Haitao Yang (Huawei Tech.)

- Jar-Ferr (Kevin) Yang (National Cheng Kung Univ.)

- Yan Ye (InterDigital Commun.)

- Gunwoo Yim (Kaon Media)

- Peng Yin (Dolby Labs)

- Yong-uk Yoon (Korea Aerosp. Univ.)

- Jongsu Yoon (Yonsei Univ.)

- Lu Yu (Zhejiang Univ.)

- Vladyslav Zakharchenko (Huawei)

- Hong Zhang (Huawei)

- Xianguo Zhang (Kings of Cloud)

- Li Zhang (Qualcomm Tech.)

- Liang Zhao (Hulu)

- Xin Zhao (Qualcomm Tech.)

- Xiaozhen Zheng (DJI)

- Jianhua Zheng (Huawei Tech.)

- Minhua Zhou (Broadcom)

- Jian Qing Zhu (Fujitsu R&D Center)