| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macao, CN, 18–25 Oct. 2017 | Document: JVET-H0021-v2 |
| --- | --- |

| Title: | AHG8: Updated Test Sequences for Spherical Video Coding from GoPro |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adeel Abbas, David Newman | Email: | aabbas@gopro.com |
| Source: | GoPro |  |  |

_____________________________

# Abstract

# This contribution provides new versions of two 360 test sequences namely BranCastle and Landing. The new versions have improved image quality over previously shared versions in JVET-G0147, and a minor bug-fix over sequences that were shared on September 19th. Furthermore, a new video clip called SkateboardAtBridge is shared.

# Introduction

# In Torino meeting, JVET-G0147 provided BranCastle and Landing sequences for 360 video coding. New versions of the same sequences are available with the following improvements/fixes:

## Improvements in Image Development, Stitching and Sharpening

Due to the improvements in image development, colors of the updated sequences look more vivid and natural. New sequences also feature improved stitching and sharpening algorithms.

## Banding Fix in Landing

Landing sequence had banding on the sky, which has been fixed.

## Halo Fix in BranCastle

BranCastle sequence had a halo on the trees towards later part of the sequence. This is also fixed.

# Example Frames

Figure 1: Left: Frame on current Landing; Right: Frame on updated Landing

Figure 2: Left: Frame on current BranCastle; Right: Frame on updated BranCastle

# Difference with version that was shared on September 19

There was a minor bug in sequences that were shared on Sept 19. The image development was adding an extra tint of green. This has also been fixed. Figure 3 shows an example frame from BranCastle that was shared on September 19, compared against new version.

Figure 3: Left: Image crop from BranCastle from September 19. Right: Image crop on new BranCastle

# Landing Update

Some experts shared a concern that Landing has a camera rotation in the end, which may pose a challenge for subjective testing. Therefore, two clips of landing namely LandingClip1 and LandingClip2 are shared. LandingClip1has similar in/out points as before. LandingClip2 has in/out points moved earlier in time to avoid camera rotation in the end.

# SkateboardAtBridge Sequence

A new sequence called SkateboardAtBridge is shared. SkateboardAtBridge features camera panning, translational motion and motion with texture on both north and south poles. The sequence was captured on GoPro Fusion camera and stitched using Fusion Studio software at 6144x3072 resolution. No compression was done after stitching.

Figure 5: Example frame from SkateboardAtBridge sequence

# Sequence Md5Sums

| Sequence |  | Md5Sum |  |
| --- | --- | --- | --- |
| BranCastle_6144x3072_30fps_8bit_420_erp.yuv |  | 9b411717977f6c50aadb458f52b96951 |  |
| LandingClip1_6144x3072_30fps_8bit_420_erp.yuv |  | 1874f18d66422794d6c205c28d3cb9b2 |  |
| LandingClip2_6144x3072_30fps_8bit_420_erp.yuv |  | d1b064b272d82d4de6f853a760c2f75e |  |
| SkateBoardAtBridge_6144x3072_30fps_8bit_420_erp.yuv |  | 3dd5bef4f2ab2c887d4730332ee4d5de |  |

# Conclusion

# It is asked that:

- New clips be used instead of the ones that were shared on September 19

- One Landing sequence be picked between LandingClip1 and LandingClip2

# SkateboardAtBridge be considered for CTC and CFP

# Copyright Information

The sequences provided in this contribution are owned by GoPro.

The following uses are allowed for the proposed sequences:

- Can be published in technical papers, played at technology research and development events.

- Can be used by Standards committees. (e.g., SMPTE, ITU, MPEG, VQEG).

The following uses are NOT allowed for the proposed sequences:

- Do not publish snapshots in product brochures.

- Do not use video for marketing purposes.

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.

- Do not share with press.