| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macao, CN, 18–24 Oct. 2017 | Document: JVET-H0031 |
| --- | --- |

| Title: | Inter Prediction using Estimation and Explicit Coding of Affine Parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cordula Heithausen Jens-Rainer Ohm Melatener Str. 23 52074 Aachen Germany | Tel: Email: | +49 (0)241 80 2 7677 {heithausen,ohm}@ient.rwth-aachen.de |
| Source: | Institute of Communications Engineering, RWTH Aachen University |  |  |

_____________________________

# Abstract

This document introduces a higher order motion compensation scheme which includes the estimation, prediction and coding of affine parameters. The estimation algorithm derives block-wise affine parameters from local gradients of all samples contained in the considered block. The focus however lies on an efficient prediction and coding of estimated affine motion parameters. Since inheriting affine parameters from a neighbouring block would entail a translational aberration, a change-of-basis transformation method is introduced to adapt affine motion parameters from one block to another one. By this, the affine motion model provides compression advantages despite the additional parameters it requires, in particular for prediction modes where it is not necessary to transmit a motion vector difference. For affine parameter prediction in the case of different reference picture distances, the conventional distance scaling scheme is amended by a method of individually scaling the non-translational motion components. The higher order motion compensation scheme resulting from these approaches provides an overall average bit-rate reduction of about 20% when integrated into an HEVC coder and tested on video sequences of mainly non-translational motion content. It should however be emphasized that the experiments have not been based on integrating the developed methods in the context of JEM so far. No specific action is suggested from here.

# Introduction

While all recent video coding standards including HEVC [1], utilize only translational motion representation for motion compensated inter-prediction, employing a more complex motion model than the translational one can be expected to enable an increased motion representation accuracy and thus a decreased prediction residual. Furthermore, as translational approximation of non-translational motion on a granular basis would no longer be necessary, a coarser and therefore less costly partitioning of such content can be expected to be advantageous.
A method of including non-translational motion in the motion prediction procedure was proposed in [2] and then adopted by the Joint Exploration Test Model 4 [3] under the name of Affine motion compensation prediction. It employs a reduced affine motion model with four parameters, only capable to express scaling and rotation, which is also referred to as similarity model or simplified affine model. This model representation is constructed as an intermediate step between spatially neighbouring translational motion vector predictor candidates for providing a translational motion vector predictor of increased accuracy. The additional motion dimensions are derived per 4×4-partition from two control point motion vectors adjacent to the upper block corners. No additional motion parameters are coded and transmitted, just signalling of the mode is necessary.
In contrast to that concept, the approach introduced hereafter includes both estimation and coding of two (for rotation/scaling) or four (affine) parameters in addition the conventional two translational parameters. The estimation is realized by an iterative gradient-based algorithm. The parameter prediction is adapted to the affine motion model representation by converting the motion parameter predictor (i.e. motion vector predictor that is not restricted to translation) from the coordinate system of the neighbouring block to that of the current block by means of a method named Block-to-Block Translational Shift Compensation (BBTSC). Furthermore, a concept of temporal distance scaling for the non-translational motion parameters in the case of variable POC-distances is introduced via Higher Order Distance Scaling (HODS). Both the full affine motion model with six parameters and its reduced version, the simplified affine motion model with four parameters, are supported, which one of these to employ is set beforehand in the respective experiments. The algorithm has been implemented into the KTA1.0 [4] software with an HM14 [5] configuration (i.e. all other additional tools switched off) and is therefore not directly comparable to Affine motion compensation prediction in JEM. However, considering the coding efficiency emerging from such an approach, taking its underlying ideas as a tool for upcoming video coding standards might be advantageous.
While section 2 of this document outlines the employed estimation method, efficient prediction and coding of the affine parameters which provide additional bit-rate reduction are discussed in section 3. Section 4 describes the integration of the affine motion compensation into the KTA framework, and finally, results are presented in section 5.

# Estimation of Simplified Affine or Full Affine Parameters

The algorithm applied to derive simplified or full affine parameters that describe the motion between a current and a reference block, is an iterative approach based on spatial and temporal local gradients. It has been introduced in [6], and has been independently re-implemented within the framework used for the presented approach. This approach is conducted subsequently to the conventional block matching procedure that results in a translational motion vector (∆x,∆y)T describing the horizontal and vertical offset between the current block and its best match. The higher order motion estimation algorithm is outlined in the following, refer to [6,7] for further details.

The sought higher order motion parameters shall compose an affine motion transformation matrix AfullAff or AsimpAff that transforms a homogeneous coordinate vector (xi,yi,1)T to its motion compensated position (xi',yi',1)T via multiplication. Both AsimpAff and AfullAff are 3×3 matrices. In case of the simplified affine motion model, only four parameters, including a0 and a1, need to be estimated to determine the sought motion transformation matrix, due to the non-translational parameters correlation a3=a2 and a4=-a5=:a3'.

xi'yi'1=A___Aff∙xiyi1 with AfullAff=a2a4a0a5a3a1001 or  AsimpAff=a2a3'a0-a3'a2a1001

It is assumed that a currently considered PU within the current video picture at time t = k is composed of Nx samples sk(x,y) at individual luma sample positions xi=(xi, yi)T. For this PU, a P-sized motion parameter set a=(a0,a1,…,aP-1)T is to be determined, with P=4 for the simplified affine motion model and P=6 for a full affine solution. The motion parameter vector a is initialized by the translational motion vector (∆x,∆y)T that is resulting from the previously performed block-matching, i.e. initialized as  ∆a(0)=(∆x,∆y,1,1,0,0)T. The best matching block position, pointed at by (∆x,∆y)T, shall be within a reference picture at time t=k-b. Taking into account all Nx pixel positions, an equation system is set up:

gtx0gtx1⋮gtxNx-1Gt=hTx0hTx1⋮hTxNx-1H∙a0a1⋮aP-1∆a

The two-dimensional vector gtxi=skxi, yi-sk-b(xi, yi) represents the temporal local gradient at luma sample positions xi and Gt results from computing the temporal local gradient at any of the Nx positions. The P-sized vectors hTxi containing the spatial local gradients are derived from the transformation rules of the simplified or full affine motion model, respectively:

hsimpAffT=(gx,gy,gxxk+gyyk,gxyk-gyxk)

hfullAffT=(gx,gy,gxxk,gyyk,gxyk,gyxk)

Once the matrix H is constituted from hTxi at all positions i, 0≤i<Nx, its pseudo-inverse H+=(HT∙H)-1∙HT is calculated in order to solve the equation system for the sought parameters,

∆a=H++Gt.

Due to the video signal’s non-linearity, the estimation approach is performed iteratively, with a final ∆a resulting from the summation of M partial displacements ∆a(iit),

∆a=iit=1M∆a(iit).

An amount of iterations M=4 was empirically found to be sufficient. During the estimation process, the parameters and operations are in float precision. Prior to being stored in the motion vector field, the affine parameters are quantized as outlined in section 3.4.

# Representation and Coding of Parameters

## PU-wise Coordinate Representation

It is crucial that all considered sample positions are expressed consistently during estimation and compensation. For this purpose, the affine motion transformation matrix is referring to the local coordinate system of the current PU1, or more specifically, its centre. This is achieved by initially subtracting the exact PU centre position cPU=width-12,height-12T, which is located between the four central full pixel positions of the PU, from the coordinates xi,yi of all pixels within the PU. After the estimation or compensation process is completed, cPU is added again, in preparation for the RD-based motion model decision or reference sample interpolation, respectively.

1Though it would be an option, it is decided not to use a picture-wise coordinate system, since, for one thing, local motion that occurs relatively far away from the origin of that overall coordinate system might then require large values of translation parameters or be less accurately describable, such that by the proposed approach, the translational component can be better predicted. Beyond that, the prediction of higher-order parameters is not affected by the re-alignment of the coordinate system on a PU basis.

## Parameter Prediction with Block-to-Block Translational Shift Compensation

By adding motion parameters to the block-wise video motion representation and thus enabling the description of non-translational motion, a dependency on the coordinate origin is introduced. As discussed in section 3.1, the motion of each PU is referencing a PU-wise coordinate system. Therefore, predicting the motion of a current PU from the already known motion of a neighboured PU by simply using the provided parameters entails a translational aberration as illustrated below in Figure 1.

Figure 1: Motion transform predictor from neighboured PU (right) requiring additional translation shift (∆a0,∆a1)T in current PU (left).

To compensate this translational shift, the Block-to-Block Translational Shift Compensation (BBTSC) is introduced as an intermediate step, which is applied whenever affine motion is predicted. Its concept is outlined for the full affine motion model in the following and further detailed in [8].

Assuming a motion parameter predictor Apred=a2a4a0a5a3a1001 is provided, the following BBTSC steps are performed to obtain the sought shift-compensated motion parameter predictor. Each step is represented by one of three homogenous transformation matrices, altogether constituting the three factors of a matrix product, which results in the compensated motion parameter predictor Acurr. The following steps are applied:

- Transformation of the pixel positions within the current PU from the current coordinate system into the coordinate system of the neighbouring prediction PU via change-of-basis transformation matrix:
Bcurr→pred=10∆Mx01∆My001
with
∆M=Mcurr-Mpred=Mcurr,x-Mpred,xMcurr,y-Mpred,y=∆Mx∆My
Mcurr defines the current PU centre and Mpred defines the prediction PU centre.

- Application of the affine motion transformation matrix Apred of the neighbouring prediction PU to the previously transformed pixel positions of the current PU.

- Transformation of the motion transformation result (of 2.) from the coordinate system of the neighbouring prediction PU to the current coordinate system via change-of-basis transformation matrix Bpred→curr=Bcurr→pred-1.

Solving this matrix product Acurr=Bpred→curr∙Apred∙Bcurr→pred results in

Acurr=a2a4a0+∆Mx(a2-1)+∆Mya4a5a3a1+∆Mxa5+∆My(a3-1)001

As the only matrix elements of Acurr differing from Apred are the translational shifts, BBTSC can be applied by simply adding ∆a0=∆Mxa2-1+∆Mya4 to a0 and ∆a1=∆Mxa5+∆My(a3-1) to a1.

The flow chart below shows how BBTSC is applied during the motion prediction stage, based on either AMVP or MRG/SKIP mode.

## Higher Order Distance Scaling of Predicted Parameters

Whenever the POC distance of a neighbouring PU used for motion parameter prediction is different from that of the current PU, i.e. if the PU providing the motion parameter predictor references a picture that is closer or further away from it than the current reference picture is from the current PU, motion parameters require distance scaling. While the translational motion parameters a0 and a1 are linearly scaled by the conventional motion vector distance scaling contained in an HEVC-compliant coder, the non-translational parameters ap, p>1, are converted in this approach by Higher Order Distance Scaling (HODS), which was introduced in [9]. The concept of HODS is to decompose the affine motion transformation matrix that is provided by a neighbouring PU and scale the contained motion components individually, according to their temporal behavior. The following six steps are conducted (using the full affine motion model here, but equivalently applicable to the simplified affine model):

- Determination of distance scaling factor D=POCcurr-POCcurrRefPOCpred-POCpredRef=∆POCcurr∆POCpred.

- De-quantization and conversion to float of motion parameters provided by prediction candidate.

- Decomposition2 of non-translational 2×2-sized motion transformation matrix Aaff, yielding two scaling factors sx and sy, a shearing factor k and a rotation angle φ:

Aaff=a2a4a5a3=Arot∙Ashear∙Ascale=sxcosφsykcosφ-sysinφsxsinφsyk sinφ+sycosφ

⟹sx=sgna2∙a22+a52,sy=a2a3-a4a5sx,k=a2a4+a3a5a2a3-a4a5, φ=atan2(a5,a2)

- Distance scaling sx, sy, k and φ using distance scaling factor D, resulting in sx,sy,k, φ:3

AscaleD=sx00syD=sxD00syD  ⟹   sx=sxD

⟹   sy=syD

AshearD=1k01D=1D∙k01 ⟹ k=D∙k

ArotD=cosφ-sinφsinφcosφD=cos(D∙φ)-sin(D∙φ)sin(D∙φ)   cos(D∙φ) ⟹φ=D∙φ

- Composition of distance-scaled non-translational 4×4-sized motion transformation matrix Aaff from sx,sy,k, φ:

Aaff=a2a4a5a3=sxcosφsykcosφ-sysinφsxsinφsyksinφ+sycosφ

- Quantization of distance scaled motion parameters.

For further details on the HODS algorithm, please refer to [9].

## Quantization of Affine Motion Parameters

Once the affine motion parameters have been estimated with float precision, they are quantized via uniform quantization.

ap= sgnap⋅q ⋅apq+12

The translational parameters a0 and a1 are quantized with quantization factor q=4, i.e. quarter-pel precision as in HM14KTA1, while the additional parameters ap,p>1, representing non-translational motion, are quantized with a bigger quantization factor qho=256, considering their higher sensitivity and preserving sufficient precision.

## Coding of Higher Order Motion Parameters

The usage of the optional affine motion model is decided and signalled for each individual PU by a PU-wise higher order motion flag, which is set to true if the provided affine motion model is used. The flag set true indicates that additional motion parameters are to be coded and transmitted, four in the case of the full affine motion model and two in the case of the simplified affine motion model. If the higher order motion flag is set to false, only the conventional (translational) parameters are considered for the current PU, while ap,p>1 are set to the corresponding identity matrix elements and neither coded nor transmitted.
Apart from for the higher order motion flag, the signalling of the non-translational affine motion parameters is inherited from the translational ones. The higher order motion flag is coded by using a different context depending on whether or not the PU above or to the left employs the affine motion model as well. Provided no such neighbouring PU is available, the translational motion model is assumed for the coding context. The transmission of the affine motion parameters ap,p>1 is identical to that of the translational parameters a0 and a1, using separate context models. Skip/Merge and AMVP modes are using the same construction of candidate lists, which can now include candidates using a non-translational motion model.


2 An identical multiplication order of Arot,Ashear,Ascale has to be used at encoder and decoder.

3 φ is derived from Arot via trigonometric double-angle formulas.

# Affine Motion Compensation

In order to apply simplified or full affine motion compensation to a PU, the affine motion transformation that has been determined (section 2) and/or predicted (sections 3.2 and 3.3) is applied to the pixel positions of a PU, using the PU-centered coordinate system (section 3.1). The resulting reference position is then translated to the picture coordinate system and used to acquire sample values from the reference picture. Whenever the affine motion compensated reference position is not at a full pixel position, sub-sample interpolation is applied.

## Sub-Sample Interpolation

Due to the increased sensitivity and high precision of the higher order motion parameters, a finer sub-sample interpolation scheme is required. For this purpose, the interpolation filters from SHVC [SHM] were employed. These are 8-tap filters, providing a 116th-pixel accuracy.

## Pixel-wise or 4×4-Partition-wise Motion Compensation

The full or simplified affine motion transformation matrix can either be multiplied to each PU pixel position individually, which yields a pixel-wise motion compensation, or it can be applied to each 4×4-sub-partition. The latter variant corresponds to affine motion compensation of JEM (i.e. only the parameter derivation and signalling of the here presented approach is different). It is implemented by first multiplying the affine motion transformation matrix to the 4x4 sub-partition center and then using the resulting translational offset as motion vector for the 16 pixel positions.
While pixel-wise motion compensation is more precise, compensating on a 4×4-pixel level can reduce computational cost. Both methods are compared in section 5.

# Results

The introduced algorithm was evaluated on a test set of ten sequences (100 frames each) that mainly contain higher order motion and are therefore part of the target group4 of higher order motion compensation. The CTU size was set to 64×64 pixels, as well as the TU-size. All results are taken in comparison to HM14-KTA1.0 with a standard HEVC-setting, except for the increased TU-size.
The basic higher order motion compensation, using estimation and plain coding of parameters with BBTSC and HODS deactivated, results in an average bit-rate reduction of over 10% and over 13% for the full and the simplified affine motion model, respectively. Another averaged 6.5% and 7.1% are achieved when improving the scheme via BBTSC alone and another averaged 1.2% and 1.0% when improving the scheme via HODS, for the full and the simplified affine motion model, respectively.
Results for the complete higher order motion compensation system, including BBTSC and HODS, are presented in the two tables shown below. Table 1 reveals that the best performance with an average of 19% and 21.3% is clearly achieved in low delay P mode. This is explainable by the gradient-based estimation algorithm working best for small motion displacements. However, also low delay B and random access mode benefit from the introduced higher order motion compensation scheme, with 15.6%/18.4% and 8.3%/9.6%, respectively.

4 As tests on sequences with little to no higher order motion content showed, no video coding performance loss is received by these on average.

Table 1: Rate reduction of complete higher order motion compensation scheme over HM14-KTA1.0, for low delay P, low delay B and random access mode.

## Prediction Mode Occurrences with and without BBTSC

The exemplary pictures from the Spincalendar sequence (POC=2, QP=36), given in Figure 2, illustrate a bit-rate reducing shift in prediction mode occurrences that is owed to BBTSC. While AMVP (highlighted green) is the predominant inter-prediction mode in the left picture that was coded without BBTSC, most inter-prediction is realized via Skip mode (highlighted red) in the right picture that was coded with BBTSC. Also Merge mode (highlighted yellow) is used more often when affine predictors are corrected by BBTSC.

Figure 2: Green area – AMVP, yellow area – MRG, red area – SKIP; left: w/o BBTSC, right: with BBTSC.

## Results for Pixel-wise vs. 4×4-Partition-wise Motion Compensation

As shown in Table 2, a less fine motion compensation granularity of one motion compensation vector per 4×4- sub-partition instead of one per pixel still results in notable bit rate reduction of 17.3% on average and 20% on average for the full and the simplified affine motion model, respectively. These average bit rate reductions are roughly 2% less than for pixel-wise motion compensation while allowing a simplification of motion compensation (equivalent to the motion compensation that is applied in the context of affine mode in JEM).
Table 2: Rate reduction of complete higher order motion compensation scheme over HM14-KTA1.0, for pixel-wise motion compensation and 4×4-pixel-sub-partition-wise motion compensation.

Inspecting some of the test sequences visually, it was found that some blocking artefacts can be observed in case of strong rotation motion of straight lines, when applying the block-wise compensation. An example of such artefacts is given in Figure 3, where a stair-like distortion of the originally straight red line can be observed in the right picture.

Figure 3: Detail of Spincalendar sequence, motion compensated pixel-wise (left) and 4x4-pixel-wise (right).

# Patent Rights Declaration

RWTH Aachen University does not have any current or pending patent rights relating to the technology described in this contribution.

# References

- Joint Collaborative Team on Video Coding. H.265 : High Efficiency Video Coding. http://www.itu.int/rec/T-REC-H.265-201504-I/en.

- S. Lin, H. Chen, H. Zhang, M. Sychew, H. Yang, and J. Zhou: Affine transform prediction for next generation video coding. ITU-T SG 16 (Study Period 2013) Contribution 1016. Sept. 2015.

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, and J. Boyce. Algorithm Description of Joint Exploration Test Model 4. Oct. 2016.

- Joint Collaborative Team on Video Coding. HM 14.0 KTA 1.0 software. https://vceg.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-14.0-KTA- 1.0.

- Joint Collaborative Team on Video Coding. HM Reference software 14.0. https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware.

- M. Narroschke and R. Swoboda. “Extending HEVC by an affine motion model.” In: Proc. of PCS ’13. 2013.

- C. Heithausen and J. H. Vorwerk. “Motion Compensation with Higher Order Motion Models for HEVC.” In: Proc. of ICASSP ’15. IEEE. 2015.

- C. Heithausen, M. Bläser, M. Wien, and J.-R. Ohm. “Improved Higher Order Motion Compensation in HEVC with Block-to-Block Translational Shift Compensation.” In: Proc. of IEEE International Conference on Image Processing ICIP ’16. Phoenix, Arizona, USA, 2016.

- C. Heithausen, M. Bläser, and M. Wien. “Distance Scaling of Higher Order Motion Parameters in an Extension of HEVC.” In: Proc. of PCS ’16. Nuremberg, Germany, 2016.

- Joint Collaborative Team on Video Coding. SHM Reference software 5.0. https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware.