| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macau, CN, 18–25 October 2017 | Document: JVET-H0035 |
| --- | --- |

| Title: | AHG8: The guidance of WS-PSNR weights derivation for different formats |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yule Sun, Lu Yu Zhejiang University Hangzhou, Zhejiang 310027,China | Tel: Email: | +86 57187953107 sunyule@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Introduction

WS-PSNR [1] was proposed to measure the codec and end-to-end distortion for 360 video. To measure objective quality of omnidirectional video in observation space, the error at each pixel position in representation space is weighted according to corresponding mapped spherical area in spherical observation space. This proposal is the guidance of WS-PSNR weights derivation for different formats of 360 video.

Overview

As projection planes are commonly uniformly sampled, weights of pixels can be calculated using stretching ratio of area from projection planes to spherical surface. The weights used for different projection formats are derived through the following two steps.

A. Step 1: Stretching ratio in continuous spatial domain

Assuming the spherical surface and projection planes of a specific format are in continuous spatial domain, the stretching ratio at different locations on projection planes could be determined by area variation in mapping process.

Firstly, the corresponding longitude and latitude (θ,φ) is determined on unit spherical surface (θ-φ surface) which is mapped from point position (x,y) on projection plane (x-y plane). The relationship between (x,y) and (θ,φ) can be represented as follows,

x=fθ,φ

y=gθ,φ.           (1)

Each point (x,y) lies on the center of a little square, which is defined as a micro unit and corresponding area is δP(x,y). Then by mapping the micro unit on projection plane (x-y plane) to unit spherical surface (θ-φ surface), the area turns into δS(θ,φ). The micro unit relationship between dxdy and  dθdφ can be expressed as dxdy=J(θ,φ)dθdφ, where Jθ,φ is Jacobian determinant, which can be derived from (1):

Jθ,φ=∂(x,y)∂(θ,φ)=∂x∂θ∂x∂φ∂y∂θ∂y∂φ.             (2)

Finally, the stretching ratio δS(θ,φ)/δP(x,y) is calculated in continuous spatial domain.  δP(x,y) on projection plane equals to |dxdy| while δSθ,φ on unit spherical surface is equal to cosφ|dθdφ|. The area stretching ratio SRx,y  for each point (x,y) in projection formats can be obtained by

SRx,y=cosφ|dθdφ||dxdy|=cos⁡(φ)J(θ,φ) .        (3)

B. Step 2: Sampled representation of WS-PSNR weight

The weights of WS-PSNR is equal to the area stretching ratio. Since digital image is a sampled representation of a continuous image, the weighting function in step 1 should be discretized to be applied for digital image. According to the relationship between (m, n) and (x, y) for different formats, the WS-PSNR weights for each pixel can be derived.

Detailed information is available in WS-PSNR paper [2].

References

[1] Y. Sun, A. Lu, and L. Yu., “AHG8: WS-PSNR for 360 video objective quality evaluation”, JVET-D0040, Oct 2016.

[2] Y. Sun, A. Lu, and L. Yu. "Weighted-to-Spherically-Uniform Quality Evaluation for Omnidirectional Video." IEEE Signal Processing Letters 24.9(2017):1408-1412.

Patent rights declaration(s)

Zhejiang university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).