| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macao, CN, 18–24 Oct. 2017 | Document: JVET-H0049 |
| --- | --- |

| Title: | AhG8: On viewport size and field of view |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA | Tel: Email: | +1 858 210 4841 Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

This document investigates suitable viewport field of view (FOV) parameters for rendering of high definition resolutions viewports. Two viewport configurations were investigated and compared to the CTC conditions (i.e., 75×75 FOV and 1816×1816 resolution): 78.1×49.1 FOV and 116.7×84.8 FOV. Informal viewing was conducted to evaluate the suitability of both configurations. Observations made during the viewing are reported.

# Introduction

Subjective evaluations of the responses to the Joint Call for Evidence (CfE) on Video Compression with Capability beyond HEVC were conducted at the 7th JVET meeting [1]. For the 360-degree video category, dynamic 2D rectilinear viewports were rendered from the coded sequences using the same field of view (FOV) and resolution as used in CTC [2], i.e., with 75×75 degrees FOV and 1816×1816 luma samples. The viewports were rendered on a 4K display, so padding was necessary to match the display resolution. It was suggested during the 7th meeting that viewports rendered in high definition (HD) resolution, i.e., 1920×1080 luma samples, might be more friendly for the CfP evaluations, as those viewports could be displayed directly on a HD monitor without padding. It was further suggested that the viewport resolution and/or FOV may be defined based on the coded resolution rather than the original equirectangular projection (ERP) resolution, as in the CTC. Specifically, the following mandates were added to AHG8 to investigate these aspects:

- Study the effect of viewport resolution, field of view, and viewport speed/direction on visual comfort.

In response to these AHG8 mandates, and to investigate suitable parameters for the viewport resolution and FOV, we generated viewports with different FOVs that are suitable for display on a HD monitor and conducted an informal viewing for the sequences included in the CfP [3].

# Viewport definition in CTC

The 2D rectilinear viewport generation is controlled by four parameters: the horizontal and vertical viewport FOVs, as well as the horizontal and vertical viewport sizes. In the CTC [2], the viewport FOV is set to 75×75 degrees and the viewport resolution is computed such that, if the FOV was set to 90×90, the resulting viewport corresponds to the face of a cube having the same number of samples as the original ERP sequence. The face size, F, of a cube having the same number of samples as ERP is computed as:

F= W∙H6=W23

where W and H are the ERP width and height, respectively. Then, the viewport size is computed as:

S=FtanFOV2

# Investigated viewports

Two different viewport configurations with HD resolution were investigated and compared to the CTC conditions: a viewport whose FOV is determined based on the original 8K ERP and a viewport whose FOV is determined based on the coded 4K ERP (see Table 1). Figures 1 to 6 depict examples of the resulting viewports for the Balboa and Landing sequences.

Table 1. Viewport parameters.

| Configuration | Viewport FOV | Viewport resolution | Number of samples on a cube |
| --- | --- | --- | --- |
| CTC | 75×75 | 1816×1816 | Same as 8K ERP |
| HD1 | 78.1×49.1 | 1920×1080 | Same as 8K ERP |
| HD2 | 116.7×84.8 | 1920×1080 | Same as 4K ERP |

Figure 1. Balboa - CTC viewport: 1816×1816 resolution and 75×75 FOV.

Figure 2. Balboa - HD1 viewport: 1920×1080 resolution and 78.1×49.1 FOV.

Figure 3. Balboa - HD2 viewport: 1920×1080 resolution and 116.7×84.8 FOV.

Figure 4. Landing - CTC viewport: 1816×1816 resolution and 75×75 FOV.

Figure 5. Landing - HD1 viewport: 1920×1080 resolution and 78.1×49.1 FOV.

Figure 6. Landing - HD2 viewport: 1920×1080 resolution and 116.7×84.8 FOV.

# Discussion

We conducted an informal viewing to evaluate the suitability of both HD configurations. For this purpose, viewports were generated using the 360Lib [4] for each of the sequences included in the draft CfP [3], for the uncompressed sequence, as well as for the HM anchor at the lowest rate point. The viewports generated with the HD1 configuration are very similar to those generated using the CTC conditions. The horizontal FOV is slightly larger, but the difference is not noticeable and the rectilinear distortions are rather small. However, the vertical FOV is reduced, meaning that, for the same viewport path, the vertical coverage on the sphere is lower when compared to the CTC conditions. Viewports generated with the HD1 configuration do not introduce visual discomfort when compared to the CTC viewports. On the other hand, the viewports generated with the HD2 configuration exhibit strong stretching at the left and right sides of the video due to the rectilinear projection using a wide horizontal FOV. Additionally, such stretching causes the objects on the left and right sides of the video to appear to have a higher motion. These two aspects significantly increase visual discomfort for dynamic viewports and moving content. Consequently, this forces the viewer to focus more toward the center of the video and unable to take full advantage of the whole resolution. Therefore, if HD resolution viewports are to be selected for the CfP subjective evaluations, we recommend to select the HD1 configuration, i.e., 78.1×49.1 FOV.

# References

- V. Baroncini, P. Hanhart, M. Wien, J. Boyce, A. Segall, T. Suzuki, “Results of the Call for Evidence on Video Compression with Capability beyond HEVC”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G1004, 7th Meeting, July 2017.

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360° video”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G1030, 7th Meeting, July 2017.

- A. Segall, M. Wien, V. Baroncini, J. Boyce, T. Suzuki, “Draft Joint Call for Proposals on Video Compression with Capability beyond HEVC”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G1002, 7th Meeting, July 2017.

- 360Lib-4.0, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-4.0