| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macao, CN, 18 – 25 Oct. 2017 | Document: JVET-H0052-r1 |
| --- | --- |

| Title: | EE1 Tests 2 and 3: Replace Planar mode in P-PDPC with UWP and Enable UW66 on top of the above test |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Hong, Krit Panusopone, Yue Yu, Limin Wang (ARRIS) Hyeongmun Jang, Jaehyun Lim, Seung-Hwan Kim (LGE) | Tel: Email: | +1-858-404-2671 seungwook.hong@arris.com hm.jang@lge.com |
| Source: | ARRIS, LGE |  |  |

_____________________________

# Abstract

This contribution reports results of EE1 for Test 2 (Replace Planar mode in P-PDPC with UWP) and Test 3 (Enable UW66 on top of Test 2). Test 2 shows the BD rates of -0.1% with the encoding time of 99% for AI, and no significant change in decoding time. Test 3 shows the BD rates of -0.1% with the encoding time of 99% for AI, and no significant change in decoding time.

# Introduction

G0081 [1] proposes to replace P-PDPC with UWP and angular mode 66 with UW66. For UW66, post filtering is disabled. UW-Planar and UW66 improve the coding efficiency by modifying Planar mode and Angular mode 66. To reduce the UWP complexity, its implementation is modified so that one less multiplication is required to compute final UWP predictor. EE1 Test2 [2] evaluates coding performance impact from replacing Planar mode with UWP. EE1 Test3 [2] evaluates coding performance impact from replacing Planar mode and Angular mode 66 with UWP and UW66.

# Modifications to reduce number of multiplication in UWP

Division-free calculation of the final predictor proposed in [3] is shown in Equations (1) - (3). However, number of multiplication required can be reduced due to the fact that combination of horWeight and verWeight paramenters have to be a constant.

| Px,y=H×Phx,y×horWeight+W×Pvx,y×verWeight≫(10+log2W+log2H) | (1) |
| --- | --- |
| horWeight=1024-verWeightwhen x<y(y+1)×S[x+y-1]otherwise | (2) |
| verWeight=(x+1)×S[x+y-1]when x<y1024-horWeightotherwise | (3) |

Where Sn is a weight array, R(x,y) Rx,y denotes the intensity value of the reconstructed neighboring pixel atx,yx,ycoordinate, Wis block width, and His block height. Ph(x,y) and Pv(x,y)are horizontal and vertical predictors, respectively.

The implementation with reduced number of multiplication is shown below.

| left=R-1,y, right=RW,y, top=Rx,-1, bot=Rx,H | (4) |
| --- | --- |
| hPred=y*left+x+1*right | (5) |
| vPred=x*{top+y+1*bot} | (6) |

Multiplications in Equations (5) and (6) could be replaced with additions in a loop and the final predictor calculation could be simplified as shown in Equations (7) - (9):

| Px,y={(hPred≪log2H-vPred≪log2W)*Weight+vPred≪(10+log2W)}≫(10+log2W+log2H) |  |
| --- | --- |
| when, x≥y | (7) |
| Px,y={(vPred≪log2W-hPred≪log2H)*Weight+hPred≪(10+log2H)}≫(10+log2W+log2H) |  |
| when, x<y | (8) |
| Weight=S[x+y-1] | (9) |

# Simulation Results

The software is implemented with the simplified version of UWP on top of HM-16.6-JEM-7.0. Tables 1 and 2 show coding performance of EE Tests 2 and 3, respectively for AI configuration. These tests are cross-checked by Huawei.

Table 1: Coding performance of Test 2

|  | Replace Planar mode in P-PDPC with UWP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.1% | -0.2% | -0.2% | 98% | 100% |
| Class A2 | -0.1% | -0.2% | -0.3% | 99% | 100% |
| Class B | -0.1% | -0.2% | -0.2% | 99% | 100% |
| Class C | -0.0% | 0.0% | 0.1% | 99% | 100% |
| Class D | -0.0% | 0.1% | 0.2% | 99% | 101% |
| Class E | -0.0% | -0.1% | 0.0% | 99% | 100% |
| Overall | -0.1% | -0.1% | -0.1% | 99% | 100% |
| Class F (optional) | 0.0% | 0.3% | 0.0% | 100% | 100% |

Table 2: Coding performance of Test 3

|  | Enable UW66 on top of the above test |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.2% | -0.4% | -0.4% | 99% | 100% |
| Class A2 | -0.2% | -0.2% | -0.3% | 100% | 100% |
| Class B | -0.1% | -0.3% | -0.3% | 101% | 100% |
| Class C | -0.1% | -0.0% | -0.0% | 98% | 100% |
| Class D | -0.0% | 0.0% | 0.1% | 99% | 100% |
| Class E | -0.1% | -0.0% | 0.0% | 98% | 100% |
| Overall | -0.1% | -0.2% | -0.2% | 99% | 100% |
| Class F (optional) | 0.0% | 0.2% | 0.2% | 99% | 99% |

# Conclusions

This contribution reports the EE1 tests 2 and 3 experiments which evaluate coding efficiency of UWP and UW66. The simulation shows a coding gain (-0.1%) and an encoder speed-up (99%) and no significant change in decoding time.

# References

[1] K. Panusopone, S. Hong, Y. Yu and L. Wang, “Comparisons between UWP, W66 and Planar, Angular mode 66 under the same coding conditions,” JVET-G0081, July 2017.

[2] E. Alshina, and L. Zhang, “Description of Exploration Experiments on Coding Tools,” JVET-G1011, July 2017.

[3] K. Panusopone, S. Hong, and L. Wang, “Unequal Weight Planar Prediction and Constrained PDPC,” JVET-E0068, April 2017.

# Patent rights declaration(s)

ARRIS, LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).