| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macao, CN, 18–25 Oct. 2017 | Document: JVET-H0056-v2 |
| --- | --- |

| Title: | AHG8: An Update on RSP Projection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adeel Abbas, David Newman | Email: | aabbas@gopro.com |
| Source: | GoPro |  |  |

_____________________________

# Abstract

# This contribution offers an improvement to the Rotated Sphere Projection (RSP). The proposed method draws inactive region on a 16x16 grid, resulting in better coding efficiency for a block based codec. Additionally, a fix to WS-PSNR calculation method is proposed. Overall, a RA coding gain of 0.7% for Luma and 1.0% for Chroma is achieved over RSP anchor (using WS-PSNR-E2E metric).

# Proposal

RSP was introduced in [3]. The improvement can be explained with the help of a Chart, which is shown in ERP format in Figure 1. Chart is a computer rendered image with longitudes and latitudes drawn as horizontal and vertical lines.

Figure 1: Chart image in ERP format

When converted into anchor RSP format, the chart image is shown in Figure 2. RSP arc is defined as the line that separates inactive and active regions on corners. Anchor RSP uses a circle on a 2D face to draw an arc, which is not optimal. In the next section, an improvement to this arc is described.

Figure 2: Chart image in anchor RSP configuration

## 3D Arc

Anchor RSP draws a circular arc on a 2D face to mark inactive pixels, as shown in Figure 3 (left). It can be seen that this does not align with the 45 line (marked in blue), resulting in more active pixels than needed. For the new 3D arc, a given 2D point on a certain face is mapped to 3D and then this 3D point is mapped back to 2D. If the face index of the new 2D point is same as original 2D point, the point should be marked as active. Otherwise, the point must lie on other face, and hence can be marked as inactive on this face. 3D arc is shown in Figure 3 (center). 3D arc gets closer to 45 line, and hence results in more inactive pixels.

## 3D Arc on 16x16 Grid

Instead of drawing 3D arc as elliptical, as shown in Figure 3 (center), the arc is drawn on a 16x16 grid, as shown in in Figure 3 (right). This aligns inactive/active area border with CU boundaries, resulting in improved coding performance.

Figure 3: Left: Anchor arc; Center: 3D arc; Right: 3D arc on 16x16 grid

## A Fix in WS-PSNR (codec) Computation

In order to determine if a 2D point lies inside a face or not, the codec level WS-PSNR function also uses the same 3D arc that has been described above. This change affects only codec-level WS-PSNR metric.

# CTC Results

Proposed RSP configuration has been implemented in 360Lib-4.0 [4] and tested on HM-16.6 software using common testing conditions [1]. As a refresher, anchor results are shown in Figure 4.

Figure 4: Coding results of anchor RSP

Results of 3D arc on 16x16 grid, along with WS-PSNR (codec) fix are shown in Figure 5.

Figure 5: Coding results of proposed RSP

Conversion-only results are same as anchor and not reported here. The Excel file that accompanies this document contains the full set of results. Results have been cross-checked by Samsung in JVET-H0079 [6].

# Results on JVET-H0021 Sequences

New versions of BranCastle and Landing sequences, along with SkateboardAtBridge has been shared in JVET-H0021 [5]. Proposed RSP is compared against anchor for these four clips, using CTC conditions.

Figure 6: Coding results of anchor RSP on JVET-H0021 Sequences

Figure 6: Coding results of proposed RSP on JVET-H0021 Sequences

# Conclusions

An improvement to RSP is proposed. The proposed method draws inactive regions on a 16x16 grid, resulting in better coding performance. It is suggested to replace this method with anchor RSP implementation for further exploration.

# References

- J. Boyce, E. Alshina, A. Abbas, Y. Ye, “JVET common test conditions and evaluation procedures for 360° video”,  Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G1030, 7th Meeting, July. 2017.

- Y. Ye, E. Alshina, J. Boyce, “AHG8: Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 4”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G1003, 7th Meeting, July. 2017.

- A. Abbas and D. Newman, “AHG8: Rotated Sphere Projection for 360 Video”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-F0036, 6th Meeting, April. 2017.

- 360Lib-4.0: https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-4.0/

- A. Abbas, D. Newman, “Updated Test Sequences for Spherical Video Coding from GoPro”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-H0021, 6th Meeting, April. 2017.

- S.N. Akula and C. Pujara “Cross-check of JVET-H0056 on an update on RSP Projection”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-H0079, 8th Meeting, October 2017.

# Patent rights declaration(s)

GoPro may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).