| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macau, CN, 18–25 October 2017 | Document: JVET-H0080 |
| --- | --- |

| Title: | EE1: Cross-check of Test 5 |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input Document to JVET |  |  |
| Purpose: |  | Report |  |  |
| Author(s) or Contact(s): |  | E. Mora 6 rue Dewoitine 78140 Vélizy-Villacoublay France | Email: | e.mora@ateme.com |
| Source: |  | Ateme |  |  |

____________________________

# Abstract

This contribution is a cross-check of Exploration Experiment 1 (Intra Prediction) test 5: “Fix for strong intra smoothing filtering” as described in JVET-G0108 [1]. The anchor is JEM-7.0 (strong intra smoothing filtering disabled), and the tested software enables strong intra smoothing filtering and implements the fix on top of JEM-7.0. The results in an All Intra configuration show 0% coding gain with 98% encoder and 98% decoder runtime. Results match the proponents’ and the results shared by Samsung on the JVET mailing list on October 10th, 2017.

# Introduction

This contribution is a cross-check of Exploration Experiment 1 (Intra Prediction) test 5: “Fix for strong intra smoothing filtering” as described in JVET-G0108 [1]. In JEM-7.0, the strong intra smoothing is implemented using a bit shift operation. However a block can have a rectangular shape, so the shift operation is incorrect when the sum of width and height of the block is not a power of 2 value. In test 5, the strong intra smoothing process is split into two parts, one is associated with a width, and the second one is associated with a height of the block, so the bit shift operation can be kept. This is done for both left column and upper row reference samples smoothing.

Note that in JEM-7.0 anchors, strong intra smoothing is disabled. In test 5, strong intra smoothing is both enabled and fixed.

# Software inspection

The tests were performed for an All Intra configuration according to the common test conditions described in JVET-G1010 [2] and using the software provided by Qualcomm at the following location:

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/candidates/HM-16.6-JEM-7.0-EE1-G0108

The code was reviewed, it is confirmed that it performs the algorithm described in JVET-G0108.

# Test results

A summary of the results for test 5 is provided below. Full results are available in the attached excel sheet JVET-H0080_EE1Test5.xlsm. These results match the ones sent by Samsung to the JVET mailing list on October 10th, 2017 and the ones provided by the proponent Qualcomm in JVET-H0028 [3].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM-16.6-JEM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,0% | 0,0% | 0,1% | 96% | 96% |
| Class A2 | 0,0% | 0,0% | 0,0% | 94% | 93% |
| Class B | 0,0% | 0,0% | 0,0% | 99% | 100% |
| Class C | 0,0% | 0,0% | 0,0% | 99% | 98% |
| Class D | 0,0% | 0,0% | 0,1% | 102% | 101% |
| Class E | 0,0% | 0,0% | 0,1% | 103% | 102% |
| Overall | 0,0% | 0,0% | 0,0% | 98% | 98% |

As can be seen from the results, enabling and fixing the strong intra smoothing brings no coding gain or loss, but yields a small (2%) encoder and decoder runtime reduction.

# Visual tests

Visual comparisons between the anchor results and the results of test 5 were performed on the encoded CTC sequences. The tests were however inconclusive as no clear trend pertaining to improving or degrading visual quality was noticed when enabling and fixing strong intra smoothing filtering. Indeed, on some parts of the images, test 5 seem to be preserving more details, while on others, either no difference is observed, or some details are lost compared to the anchor reference.

# Conclusions

Test 5 brings 0% coding gain and 98% encoder / decoder runtime over the JEM-7.0 anchors in an All Intra configuration. Furthermore, visual comparisons tests were inconclusive as no clear trend pertaining to improving or degrading visual quality was noticed.

# References

- V. Seregin, X. Zhao, M. Karczewicz, “Non-EE1: Fix for strong intra smoothing filtering“, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G0108, 7th Meeting, July 2017

- K. Sühring, X. Li, “ JVET common test conditions and software reference configurations“, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G1010, 7th Meeting, July 2017

- V. Seregin, X. Zhao, M. Karczewicz, “ EE1: Fix for strong intra smoothing filtering“, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-H0028, 8th Meeting, October 2017