| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Macao, CN, 18–25 Oct. 2017 | Document: JVET-H0087 |
| --- | --- |

| Title: | Diagonal motion partitions on top of QTBT block structure |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Hochan Ryu Donggyu Sim | Tel: Email: | +82-2-941-6470 yjahn@digitalinsights.co.kr hc.ryu@digitalinsights.co.kr dgsim@digitalinsights.co.kr |
| Source: | Digital Insights Inc. |  |  |

_____________________________

# Abstract

In this contribution, diagonal motion partitions (DMPs) for inter prediction are proposed on top of quadtree plus binary tree (QTBT) block structure. In the proposed partitioning method, a coding unit (CU) is split into two diagonal motion partitions. The proposed method includes only two diagonal directions, but it can represent various arbitrary partitions on top of QTBT block structure. The proposed DMPs can achieve 0.15% BD-rate reduction over JEM-7.0 for random access configurations.

# Introduction

To improve prediction performance, several arbitrary block partitioning methods have been proposed [1-5]. Especially, geometry motion partition (GEO) was proposed during JCT-VC activities, and it could achieve 1.9-5.0% BD-rate reduction over TMuC and HM [3-5]. In GEO, a square block is divided into two regions, and the boundary separating the two regions is defined by a straight line. The line is represented by two polar coordinates parameters ( and ). Figure 1 shows an example of arbitrary block partitions with GEO. Even though the meaningful coding gains could be achieved, but it was not adopted in HEVC standard. There were two main concerns that one was signaling overhead and the other was complexity increment.

Figure 1. An example of arbitrary block partitions with GEO

# Technical description

In this contribution, diagonal motion partitions (DMPs) are proposed on top of quadtree plus binary tree (QTBT) block structure. The proposed partitioning method uses only two diagonal partitions, left and right diagonal, for each coding unit (CU), but it can cover the various partitions in geometry motion partition (GEO). The motivation of the proposed partitioning method is that binary split CUs can help represent arbitrary motion partitions. It means that the binary split CUs can make rectangular shape blocks which have different block width and height compared with only using the quadtree (QT) block structure, and the various angle and distance parameters of GEO can be represented by only two diagonal partitions combined with QTBT block structure. Therefore, the proposed DMPs can reduce signaling overhead with QTBT split syntaxes. Figure 2 illustrates an example of the proposed DMPs on top of QTBT block structure. The gray solid lines indicate QT splitting and dotted lines indicate binary tree (BT) splitting. In addition, the red solid lines indicate diagonal splitting a CU with the proposed DMPs.

Figure 2. An example of the proposed diagonal motion partitions on top of QTBT structure

## Representation of diagonally partitioned blocks

Figure 3 represents two diagonal motion partitions in the proposed method. As shown in Fig. 3, the proposed method uses straight lines to split a CU into two diagonal motion partitions. In this contribution, the straight lines can be represented by simple linear equations as below:

- Left diagonal: 							(1)

- Right diagonal:							(2)

where (x,y) is the sample position within a CU, w is CU width, and h is CU height. This contribution assumes that the origin of 2D-coordinate is the top-left position of a CU.

(a) Left diagonal			 (b) Right diagonal

Figure 3. Two diagonal motion partitions in the proposed method

To split a CU into two diagonal partitions, it has a division problem of samples within the straight lines. The problem means that it cannot divide a sample into two regions. To solve the division problem, the proposed method applies mask based motion compensation and overlapping the samples within the straight lines. The mask represents that each sample in a CU is belonged to which diagonal partitions and, it is pre-defined according to block sizes and directions of diagonal partitions. The masks for the partition 0 (P0) and the partition 1 (P1) are evaluated as follows:

(3)

(a) DMP mask for P0			 		(b) DMP mask for P1

Figure 4. An example of right diagonal masks for a 16x8 CU

Figure 4 illustrates an example of right diagonal mask for a 16x8 CU. As shown in equation (3) and Fig. 4, the proposed method applies overlapping the samples within the straight line. The final prediction of the CU is can be stated as:

(4)

where PDMP represents the final prediction with the proposed method, and PP0 and PP1 are the prediction blocks for partition 0 and partition 1, respectively. shift is a shift factor for scaling down, in this case, it is equal to 1.

## Overlapped block motion compensation (OBMC) for DMP

To decrease discontinuities on the prediction boundaries, the proposed method applies overlapped block motion compensation (OBMC). The proposed OBMC for DMP applies additional overlapping the samples nearby the straight line which splits a CU into two diagonal motion partitions. Figure 5 presents an example of right diagonal masks applying the proposed OBMC for DMP.

(a) DMP mask for P0					(b) DMP mask for P1

Figure 5. An example of right diagonal masks applying the proposed OBMC for DMP

In the same manner of OBMC in JEM, OBMC for DMP is always applied for CUs with size larger than 256 luma samples. For a CU with size less than or equal to 256 luma samples, a CU level flag is a signaled to indicate whether OBMC for DMP is applied or not.

## Signaling DMP parameters

To indicate DMP block and its motion data, the proposed DMP parameters as below:

- DMP flag: whether DMP is applied or not for the current CU

- DMP direction: which diagonal direction (left:0 or right:1) when the DMP flag is TRUE

- DMP merge flag: whether the diagonal partition is merged or not

- DMP merge index: which merge candidate when the DMP merge flag is TRUE

- Motion data: inter prediction direction, reference indices, motion vector differences, and AMVP indices when the DMP merge flag is FALSE

Figure 6 represents an example of replacing GEO parameters on top of QTBT block structure. As shown in Fig. 6, a DMP flag and a DMP direction combined with a binary split flag syntax and a binary split type syntax can be used to represent the diagonally partitioned block.

Figure 6. An example of replacing GEO parameters on top of QTBT block structure

## Constraints of harmonization with other tools

In JEM-7.0, there are many different inter prediction tools already adopted. Especially, to improve the performance of inter prediction, sub-block level motion prediction techniques are working well. However, the proposed diagonal motion partitions should perform at the sample level, it cannot be easily harmonized with sub-block level motion prediction techniques. In this reason, the CU coded by the proposed diagonal motion partition cannot apply the sub-block level motion prediction, e.g., ATMVP, STMVP, FRUC, and AFFINE, in the current implementation.

# Performance evaluation

The proposed method was evaluated under the common test conditions (CTC) for JEM [6], but only class B, C, and D of common test sequences were used for the performance evaluation. The proposed method was integrated on JEM-7.0 [7]. Table 1 shows the BD-rate reduction of the proposed diagonal motion partitions on top of QTBT block structure against JEM-7.0. With the proposed method, 0.15% BD-rate reductions are achieved with 53% of encoder and 4% decoder complexity increments for random access configuration. Figure 7 shows an example of the proposed DMP coded blocks in RaceHorsesC.

Table 1. BD-rat reduction of the proposed method against JEM-7.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - |
| Class A2 | - | - | - | - | - |
| Class B | -0.06% | 0.02% | 0.11% | 166% | 102% |
| Class C | -0.20% | -0.23% | 0.04% | 158% | 103% |
| Class D | -0.23% | -0.32% | 0.29% | 134% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -0.18% | 0.15% | 153% | 104% |

Figure 7. An example of the proposed DMP coded blocks in RaceHorsesC (RA, QP: 27, POC: 1)

# Conclusion

In this contribution, diagonal motion partitions on top of quadtree plus binary tree (QTBT) block structure are proposed. The proposed method can achieve 0.15% BD-rate reduction over JEM-7.0 for random access configuration. The current implementation is the initial version, and we need to further improve and harmonize it with other tools in JEM. Further study will be focused on harmonization with BIO and DMVR, modification of DMP parameter signaling, and encoder complexity reduction.

# References

- O. Divorra, P. Yin, C. Gomila, “Geometry-adaptive block partitioning,” ITU-T Q.6/SG16 VCEG, VCEG-AF10, San Jose, US, Apr. 2007.

- J. Chen, S. Lee, K.-H. Lee, and W.-J. Han, “Object boundary based motion partition for video coding,” Proc. of IEEE International Symposium on Circuits and Systems, Nov. 2007.

- P. Chen, W. Chien, R. Panchal, and M. Karczewicz, “Geometry motion partition,” JCTVC-B049, 2nd JCT-VC meeting, Geneva, CH, Jul. 2010.

- P. Chen, L. Guo, and M. Karczewicz, “Geometry motion partition,” JCTVC-C301, 3rd JCT-VC meeting, Guangzhou, CN, Oct. 2010.

- E. Francois, P. Bordes, L. Guo, and M. Karczewicz, “CE2: Simplified geometry block partitioning,” JCTVC-D230, 4th JCT-VC meeting, Daegu, KR, Jan. 2011.

- K. Suehring and X. Li, “JVET common test conditions and software reference configurations,” JVET-G1010, 7th JVET meeting, Torino, IT, Jul. 2017.

- HEVC reference model (HM) 16.6, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.6

# Patent rights declaration(s)

Digital Insights Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).