| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Gwangju, KR, 20–26 Jan. 2018 | Document: JVET-I_Notes_d1 |
| --- | --- |

| Title: | Meeting Report of the 9th meeting of the Joint Video Exploration Team (JVET), Gwangju, KR, 20–26 Jan. 2018 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from responsible coordinators of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Responsible coordinators |  |  |

# Summary

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its ninth meeting during 20–26 Jan. 2018 at the Gwangju Kimdaejung Convention Center (30 Sangmunuriro, Seo-gu, Gwangju, 61958, Republic of Korea). The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as responsible coordinators of the two organizations. For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.13 of this document.

The JVET meeting sessions began at approximately 0900 hours on Saturday 20 Jan. 2018. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 26 Jan. 2018. Approximately XXX people attended the JVET meeting, and approximately XX input documents and 10 AHG reports were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of studying future video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or gives better support regarding the requirements of newly emerging application domains of video coding. As a primary goal, the JVET meeting made further planning related to the Call for Proposals (CfP), which had been issued by the previous meeting, refining the testing and evaluation methodology of compression technology designs that would be expected to be proposed in responses to the CfP.

Another important goal of the meeting was to review the work that was performed in the interim period since the eighth JVET meeting in improving the Joint Exploration Test Model 7 (JEM7). Video coding results produced with JEM7 will also be included as anchor in the CfP. No exploration experiments conducted in the JEM7 framework had been run, but other technical input was considered. No new JEM version was issued.

The JVET produced 6 output documents from the meeting:

- Joint Call for Proposals on video compression with capability beyond HEVC

- Template for proposal description documents

- Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 5

- JVET common test conditions and software reference configurations

- Common test conditions and evaluation procedures for HDR/WCG video

- Common test conditions and evaluation procedures for 360° video

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, no Exploration Experiments (EE) were defined. The next four JVET meetings were planned for 11–20 April 2018 in San Diego, US, 10–18 July 2018 under ITU-T auspices in Ljubljana, SI, 4–12 Oct. 2018 under WG11 auspices in Macao, CN, and 8–16 January 2019 under ITU-T auspices in Marrakesh, MA.

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Exploration Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Exploration Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its ninth meeting during 20–24 Jan. 2018 at the Gwangju Kimdaejung Convention Center (30 Sangmunuriro, Seo-gu, Gwangju, 61958, Republic of Korea). The JVET meeting was held under the leadership of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany) as the responsible coordinators of the two organizations.

It is noted that at their current meeting, the parent bodies established a plan to modify the unabbreviated name of JVET to “Joint Video Experts Team” when entering the phase of formal standard development (expected in April 2018), pending the outcome of the Call for Proposals (CfP) issued at the current meeting.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2018_01_I_Gwangju/.

## Primary goals

As a primary goal, the JVET meeting made further planning related to the Call for Proposals (CfP), which had been issued by the previous meeting, in particular refining the testing and evaluation methodology of compression technology designs that would be expected to be proposed in responses to the CfP.

Another important goal of the meeting was to review the work that was performed in the interim period since the seventh JVET meeting in improving the Joint Exploration Test Model 7 (JEM7). Video coding results produced with JEM7 will also be included as anchor in the CfP. No exploration experiments had been conducted in the JEM7 framework, but other technical input was considered. No new JEM version was issued.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect the JEM software but have no normative effect are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in the JEM description (an error, oversight, or messiness) or in the software are marked by the string “Decision (BF):”.

This meeting report is based primarily on notes taken by the responsible leaders. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Thursday, 11 Jan. 2018. Any documents uploaded after 2359 hours Paris/Geneva time on Friday 12 Jan. were considered “officially late”, giving a grace period of 24 hours to accommodate those living in different time zones of the world.

All contribution documents with registration numbers JVET-I0026 and higher were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “I0026+” range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

There were no technical design proposal contributions that were registered on time but uploaded late for the current meeting.

The following technical design proposal contributions were both registered late and uploaded late:

- JVET-I00xx (a proposal on …), uploaded 01-XX.

- ….

The following other documents not proposing normative technical content, but with some need for consideration were uploaded late:

- JVET-I00xx (an information document on …), uploaded 01-XX.

- …

The following cross-verification reports were registered on time but were uploaded late: JVET-I00xx [uploaded 01-XX], … .

(The cross-verification documents that were both registered late and uploaded late are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-I00xx, … .

“Placeholder” contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and rejected in the document management system. The initial upload of the following contribution document was rejected as “placeholder” and was not corrected until after the upload deadline: JVET-I00xx (an information document on …). A new version was provided on 01-XX.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, EE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly the meeting report JVET-H1000, the Joint Call for Proposals JVET-H1002, the draft template of proposal description document JVET-H1003, the algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 5 JVET-H1004, the JVET common test conditions and reference software configurations JVET-H1010, the JVET common test conditions and evaluation procedures for HDR/WCG video JVET-H1020, and the JVET common test conditions and evaluation procedures for 360° video JVET-H1030, were approved. The JEM7 software implementation (version 7.1), and the 360Lib software implementation (version 5.0) were also approved.

The group had initially been asked to review the meeting report of the previous meeting for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Review of results of previous meeting

- Review of the Joint Call for Proposals that was issued from the October 2017 meeting in Macao, addressing any deficiencies in the Call as necessary, and preparation for consideration of responses to the Call

- Consideration of contributions and communications on project guidance

- Consideration of video coding technology contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director’s AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the JEM software uses the HEVC reference software copyright license header language is the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the JEM software, and for evaluating proposals for technology to be included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

Note: This applies also to the 360Lib video conversion software as well as the JEM and HM.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It is reminded to send notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers was 808.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT: Adaptive multi-core transform.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP: Advanced temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BIO: Bi-directional optical flow.

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EE: Exploration Experiment – a coordinated experiment conducted toward assessment of coding technology.

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ECV: Extended Colour Volume (up to WCG).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- HDR: High dynamic range.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MDNSST: Mode dependent non-separable secondary transform.

- MMLM: Multi-model (cross component) linear mode.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- QTBT: Quadtree plus binary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- UCBDS: Unrestricted center-biased diamond search.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in JEM:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in P/B slice, and one CTB per luma CTU and two CTBs per chroma CTU in I slice.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size to a CU.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size to a CU.

## Opening remarks

- Reviewed logistics, agenda, working practices, policies, document allocation

- Results of previous meeting: JEM, meeting report, etc.

- Goals of the meeting: Refinement of the joint Call for Proposals (CfP) if necessary, evaluation of status progress in CfP preparation, finalization of template document, Review of new technology proposals.

- Discuss further planning of CfP and beyond with parent bodies

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Sat. 20 Jan., 1st day

  - 0900–1030 Opening plenary, AHG reports (chaired by GJS&JRO)

  - 1100–1215 CfP preparation (section 6).

- Sun. 21 Jan., 2nd day

  - 0900–1230 Technical contributions

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized

- AHG reports (10) (section 3)

- Analysis, development and improvement of JEM (0) (section 4)

- Test material (0) (section 5)

- Call for Proposals preparation (3) (section 6)

- Non-EE technology proposals (5) (section 7)

- Extended colour volume coding (0) (section 8)

- Coding of 360o video projection formats (2) (section 9)

- Complexity analysis (0) (section 10)

- Encoder optimization (1) (section 11)

- Metrics and evaluation criteria (0) (section 12)

- Withdrawn (1) (section 13)

- Joint meetings, plenary discussions, BoG reports (2), Summary of actions (section 14)

- Project planning (section 15)

- Output documents, AHGs (section 16)

# AHG reports (10)

These reports were discussed Saturday 20 Jan. 0930–1030 (chaired by GJS and JRO).

JVET-I0001 JVET AHG report: CfP preparation (AHG1) [J.-R. Ohm, G. J. Sullivan, V. Baroncini]

This document reports the work of the JVET ad hoc group on CfP preparation (AHG1) between the 8th JVET meeting at Macao, China (18–24 October 2017) and the 9th Meeting at Gwangju, Korea (20–26 January 2018).

The CfP text was finalized according to plans, and distributed via the JVET and parent bodies’ websites, as well as directly to interested participants. Final anchors in the different categories and refined evaluation sheets were produced. Test sequences and anchor bitstreams were made available to prospective proponents via ftp download. Test labs were clarified, and logistics arrangements for receiving submissions and conducting tests were made.

By the deadline of 2017-12-15, registrations were received from 21 distinct proponent groups. These include a total of 26 companies and institutions who are planning to contribute to the CfP. Three of the proponents or proponent groups are planning to submit two proposals each in all categories, other proponents or proponent groups may submit in either one, two, or three categories. In total, 46 category-specific submissions were registered to be tested as follows:

  - SDR: 22 submissions (8 of which were registered only in this category)

  - HDR: 12 submissions (4 of which were registered only in this category)

  - 360°: 12 submissions (4 of which were registered only in this category)

In addition, HM and JEM anchors will be tested in all three categories, which makes a total of 52 category-specific tests to be performed.

To conduct the high amount of tests that are expected to be conducted, a total of 7 test labs have been contacted who are experienced in their respective fields, who have meanwhile agreed to contribute to the test effort. Part of the tests will be overlapping (to be conducted in at least two labs) such that verification about the validity and comparability of the tests will be possible. In particular, the test labs will be as follows:

For SDR and 360°

  - BBC and WSU (University of Western Scotland), who have participated in the prior HEVC verification testing;

  - University of Padova (particularly expert in video coding editing and shooting);

  - CWI, a Dutch company who participated in MPEG’s 3D Point Cloud CfP test.

For HDR/WCG:

  - RAI and Sisvel who have had experience in HDR since several years;

  - DBW, a small but active production company in the area of Broadcasting that begun shooting and selling HDR video content since 3 years ago.

GBTech and CWI will also collaborate taking care of:

  - Coordinate and take care of all the contacts and administrative matters with the Proponents;

  - Design 56 test sessions (44 SDR/360 + 12 HDR);

  - Receive the disks from the proponents;

  - Visually check all the 1992 received video files;

  - Make a quick negotiation with Proponents when corrupted (totally or partially) disks are received, to recover the good video clips;

  - Group the files to create the test disk to the Test Labs;

  - Create the scripts to download the test sessions files;

  - Create the scripts to run the test sessions;

  - Create the test sessions and the test instructions for the viewers’ training (this will be the same for all the Test Labs);

  - CWI will run 22 test sessions;

  - GBTech will run two verification test sessions for SDR, 360 and HDR;

  - GBTech will act as back-up testing site in the case one or more Labs will have problems in completing their tasks.

In the 360° video category, dynamic viewports will be tested which shall not be known to proponents beforehand according to the CfP. It is to be clarified during the meeting how the dynamic viewports will be determined and who is responsible for defining them.

The submission package (proposal description document and Excel sheets to report the results have to be finalized during the meeting.

Input document JVET-I0025 raises some issues to require clarification relating to the HDR category of the CfP, which needs to be discussed during the meeting; it is to be decided whether an update of the CfP would be necessary or whether it is sufficient to communicate clarifications directly to the proponents in that category.

Other issues that potentially need clarification were mentioned in the discussion as follows:

number of frames (Showgirl and FoodMarket and HDR Market, see also the AHG7 report) and frame rate (a filename said 24 Hz but was for a 25 Hz sequence). For sequences with a copyright frame at the end, that frame is not to be encoded.

Additional material was noted to be under preparation by the test coordinator for presentation later in the meeting.

JVET-I0002 JVET AHG report: JEM algorithm description editing (AHG2) [J. Chen, E. Alshina, J. Boyce]

This document reports the work of the JVET ad hoc group on JEM algorithm description editing (AHG2) between the 8th meeting at Macao, CN (October 18–24, 2017) and the 9th Meeting at Gwangju, KR (January 20–26 2018).

Currently the JEM document contains the algorithm description as well as encoding logic description for all new coding features in JEM7.0 beyond HEVC. The new coding features relative to HEVC that are included in JEM7 were summarized in the AHG report.

The AHG did not have any activities to report for this meeting.

The AHG recommended to:

- Continue to improve the editorial quality of the Algorithm Description of Joint Exploration Test Model document and address issues relating to mismatches between software and text.

- Identify and improve algorithm description for critically important parts of JEM design for better understanding of complexity.

JVET-I0003 JVET AHG report: JEM software development (AHG3) [X. Li, K. Sühring]

This report summarizes the activities of the AhG3 on JEM software development that has taken place between the 8th and 9th JVET meetings.

Software development was continued based on the HM-16.6-JEM-7.0 version. A branch was created in the software repository mainly for bug fix and minor cleaning. HM-16.6-JEM-7.1 was released on Oct. 27th, 2017.

The JEM software is developed using a Subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/

The implementation of JEM-7 bug fixes has been performed on the branch

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/branches/HM-16.6-JEM-7.0-dev

The released version of HM-16.6-JEM-7.1 can be found at

https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.1

The performance of HM-16.6-JEM-7.1 shares the same performance as HM-16.6-JEM-7.0. For convenience, the performance HM-16.6-JEM-7.0 is listed below. As agreed in 7th JVET meeting, HM-16.15-SCM-8.4 is used as HM anchor for class F sequences. Note that 10-bit internal bit depth was used for HM-16.15-SCM-8.4 when testing class F.

It was noted that version 7.1 is compatible with version 7.0 – i.e., it can decode bitstreams encoded with version 7.0

The JEM bug tracker is located at

https://hevc.hhi.fraunhofer.de/trac/jem

It uses the same accounts as the HM software bug tracker. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Contributors were encouraged to please file all issues related to JEM into the bug tracker, and to try to provide all the details that are necessary to reproduce the reported issues. Patches for solving issues and improving the software are always appreciated.

The AHG recommended

- To continue software development on the HM-16.6 based version

- Encourage people to test JEM software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bit-streams/test cases that trigger bugs in JEM.

JVET-I0004 JVET AHG report: Test material (AHG4) [T. Suzuki, V. Baroncini, J. Chen, J. Boyce, A. Norkin]

The test sequences used for CfP (JVET-H1002) are available on ftp://jvet@ftp.ient.rwth-aachen.de in directory “/jvet-cfp” (please contact the JCT-VC chairs for login information).

HM/JEM anchors (defined in JVET-H1002) were generated and verified by cross checker.

HM anchors:

ftp://jvet@ftp.ient.rwth-aachen.de/jvet-cfp/anchors-hm

JEM anchors:

ftp://jvet@ftp.ient.rwth-aachen.de/jvet-cfp/anchors-jem

Relevant contributions to this meeting are as follows. (Most of the contributions are submitted for AHG7 and 8)

CfP anchor generation

- JVET-I0026 “AHG7: HLG anchor generation of S31 and S32 for Joint Call for Proposals on Video Compression with Capability beyond HEVC", T. Tsukuba, M. Ikeda, T. Suzuki(Sony).

Clarification of the test

- JVET-I0025 “Clarifications on HDR Test of CfP on Video Compression beyond HEVC", P. Yin, W. Husak, T. Lu, F. Pu, T. Chen (Dolby), S. Iwamura (NHK).

The AHG recommended to:

- Review all related contributions

- Continue to collect new test sequences available for JVET with licensing statement

JVET-I0005 JVET AHG report: Memory bandwidth consumption of coding tools (AHG5) [X. Li, E. Alshina, R. Hashimoto, T. Ikai, H. Yang]

The document summarizes activities of AhG on memory bandwidth consumption of coding tools between the 8th and the 9th JVET meetings.

There was no related email discussion during this meeting cycle. The desire to provide a memory model software implementation had not yet progressed, but it was reported that this might become available during the meeting.

JVET-I0006 JVET AHG Report: 360 video conversion software development (AHG6) [Y. He, K. Choi, V. Zakharchenko]

The document summarizes activities on 360-degree video content conversion software development between the 8th and the 9th JVET meetings.

The 360Lib-5.0 software package integrated all adoptions about projection format and metrics calculation:

- For projection formats and frame packing:

- Update on Rotated Sphere Projection (RSP) (JVET-H0056);

- For software:

- Fix for bug ticket #56;

- Bug fix for the interpolation weight derivation;

360Lib-5.0 related release:

360Lib-5.0rc1 with support of HM-16.16 and JEM-7.0 was released on Nov. 1st, 2017;

360Lib-5.0 with support of HM-16.16 and JEM-7.0 was released on Nov. 10th, 2017;

The 360Lib software is developed using a Subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

The released version of 360Lib-5.0 can be found at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-45.0/

360Lib-5.0 testing results can be found at:

ftp.ient.rwth-aachen.de/testresults/360Lib-45.0

360Lib bug tracker

https://hevc.hhi.fraunhofer.de/trac/jem/newticket?component=360Lib

Table 1 lists the HM-16.16 based coding performance with different projection formats according to 360o video CTC (JVET-H1030) compared to PERP coding. Table 2 compares the JEM-7.0 PERP coding with HM-16.16 PERP coding. Table 3 lists the conversion only end-to-end WS-PSNR for different projection formats compared to that of PERP format.

Table 1. HM-16.16-360Lib-5.0 testing (HM PERP coding as anchor)

| Projection | E2E WS-PSNR for all sequences |  |  | E2E WS-PSNR for 8K sequences |  |  | E2E WS-PSNR for 6K sequences |  |  | E2E WS-PSNR for 4K sequences |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| CMP | -2.4% | -0.7% | -1.0% | -5.1% | -3.0% | -3.5% | 0.1% | 3.3% | 2.8% | 0.5% | -1.6% | -1.2% |
| AEP | -4.0% | -3.1% | -3.2% | -4.7% | -3.4% | -3.4% | -2.9% | -2.0% | -2.2% | -4.3% | -4.6% | -4.7% |
| COHP | -1.4% | 1.5% | 0.6% | -5.0% | -0.2% | -1.3% | 3.5% | 5.5% | 4.5% | -0.4% | -1.1% | -1.3% |
| CISP | -3.3% | 1.1% | 0.2% | -7.5% | -1.3% | -2.7% | 2.4% | 6.2% | 5.7% | -1.8% | -1.8% | -2.1% |
| EAP-SSP | -9.7% | -5.7% | -6.3% | -11.9% | -6.4% | -6.6% | -7.3% | -4.1% | -5.3% | -7.6% | -6.9% | -7.2% |
| ACP | -9.9% | -5.7% | -6.1% | -12.5% | -6.3% | -6.5% | -7.7% | -4.7% | -5.3% | -6.7% | -6.2% | -6.2% |
| RSP | -9.2% | -5.8% | -6.1% | -12.4% | -7.0% | -7.4% | -6.0% | -3.9% | -4.5% | -6.0% | -5.9% | -5.3% |
| ECP | -9.0% | -5.4% | -6.0% | -12.3% | -6.7% | -7.2% | -4.5% | -2.4% | -3.6% | -7.8% | -7.6% | -7.3% |
| EAC | -9.9% | -5.8% | -6.0% | -12.5% | -6.4% | -6.4% | -8.1% | -4.8% | -5.3% | -6.2% | -6.2% | -6.0% |

Table 2. JEM-PERP vs HM-PERP coding (HM PERP coding as anchor)

| anchor: HM-PERP | SPSNR-NN (End to End) |  |  | WS-PSNR (End to End) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| test: JEM-PERP | Y | U | V | Y | U | V |
| Trolley | -15.0% | -30.9% | -32.7% | -14.9% | -30.8% | -32.8% |
| GasLamp | -20.3% | -44.6% | -41.5% | -20.2% | -44.6% | -41.5% |
| Skateboarding_in_lot | -24.4% | -33.0% | -42.9% | -24.4% | -33.0% | -42.9% |
| Chairlift | -29.8% | -48.6% | -45.6% | -29.9% | -48.6% | -45.7% |
| KiteFlite | -16.5% | -35.3% | -38.6% | -16.4% | -35.2% | -38.5% |
| Harbor | -19.0% | -41.7% | -42.2% | -19.0% | -41.7% | -42.2% |
| Balboa | -33.1% | -36.5% | -40.9% | -33.1% | -36.5% | -40.9% |
| Broadway | -32.9% | -38.5% | -41.5% | -32.9% | -38.5% | -41.5% |
| Landing2 | -24.1% | -24.5% | -21.8% | -24.2% | -24.6% | -21.8% |
| BranCastle2 | -22.0% | -29.0% | -28.7% | -22.0% | -29.0% | -28.6% |
| PoleVault | -17.7% | -19.0% | -20.4% | -17.8% | -19.0% | -20.6% |
| AerialCity | -24.8% | -44.7% | -31.0% | -24.9% | -44.8% | -30.9% |
| Overall | -23.3% | -35.5% | -35.7% | -23.3% | -35.5% | -35.7% |
| 8K | -20.8% | -39.0% | -40.6% | -20.8% | -39.0% | -40.6% |
| 6K | -28.0% | -32.1% | -33.2% | -28.0% | -32.1% | -33.2% |
| 4K | -21.3% | -31.9% | -25.7% | -21.3% | -31.9% | -25.7% |

Table 3. Conversion only results (PERP format as anchor)

| Projection format | E2E WS-PSNR |  |  | E2E WS-PSNR for 8K sequences |  |  | E2E WS-PSNR for 6K sequences |  |  | E2E WS-PSNR for 4K sequences |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| PERP | 45.90 | 56.59 | 56.63 | 45.60 | 58.22 | 57.81 | 46.41 | 56.94 | 57.36 | 45.77 | 50.97 | 51.63 |
| CMP | 0.88 | 0.50 | 0.47 | 0.81 | 0.46 | 0.46 | 1.33 | 0.71 | 0.67 | 0.20 | 0.17 | 0.11 |
| AEP | 0.77 | 0.40 | 0.39 | 0.70 | 0.33 | 0.33 | 0.83 | 0.44 | 0.46 | 0.88 | 0.52 | 0.45 |
| COHP | 1.32 | 0.62 | 0.62 | 1.49 | 0.56 | 0.54 | 1.41 | 0.76 | 0.81 | 0.61 | 0.50 | 0.46 |
| CISP | 2.10 | 0.79 | 0.76 | 2.25 | 0.63 | 0.60 | 2.14 | 0.94 | 0.92 | 1.58 | 1.00 | 0.92 |
| SSP | 2.29 | 1.06 | 1.04 | 2.25 | 0.67 | 0.65 | 1.98 | 0.95 | 0.96 | 3.04 | 2.47 | 2.40 |
| ACP | 2.69 | 1.05 | 1.02 | 2.83 | 0.86 | 0.85 | 2.57 | 1.29 | 1.27 | 2.50 | 1.13 | 1.03 |
| RSP | 2.44 | 1.23 | 1.18 | 2.28 | 0.66 | 0.64 | 2.02 | 0.98 | 0.96 | 3.75 | 3.43 | 3.27 |
| ECP | 2.62 | 1.06 | 1.03 | 2.68 | 0.83 | 0.79 | 2.61 | 1.23 | 1.23 | 2.42 | 1.44 | 1.34 |
| EAC | 2.90 | 1.37 | 1.34 | 2.81 | 0.86 | 0.85 | 2.60 | 1.30 | 1.27 | 3.81 | 3.05 | 2.96 |

The AHG recommended to continue software development of the 360Lib software package.

JVET-I0007 JVET AHG report: JEM coding of HDR/WCG material (AHG7) [A. Segall, E. François, D. Rusanovskyy]

This document summarizes the activity of AHG7: JEM Coding of HDR/WCG Material between the 8th meeting in Macao, CN (18–25 Oct. 2017) and the 9th meeting in Gwangju, KR (20 – 26 Jan. 2018).

HM and JEM anchor bit-streams for the Call for Proposals HDR category were prepared and made available.

The AHG delivered both HM16.16 and JEM7.0 HDR CfP anchors as part of its work.  The HM16.16 anchors were delivered on November 10, 2017, and made available at ftp://ftp.ient.rwth-aachen.de/jvet-cfp/anchors-hm/HDR. The JEM 7.0 anchors were also delivered on November 10, 2017, and made available at ftp://ftp.ient.rwth-aachen.de/jvet-cfp/anchors-jem/HDR.

While not official AhG activity, the editors of the AhG were aware of side discussions related to the HDR category of the Call for Proposals. The topics are listed below to enable further discussion, should the JVET decide to discuss further:

- It has been commented by some interested parties that the Show Girls sequence is approximately 13.56 seconds long (339 frames), instead of the 10 seconds of most sequences.

- It has been commented by some interested parties that the Market3 sequence is 8 seconds long (400 frames), which may be too short for performing a visual assessment.

- It was noted that the configuration files were unclear on what value for the PeakValue parameter should be used for the calculation of objective metrics for the HLG sequence provided. It was clarified that a PeakValue equal to 1,000 was used to compute the objective metrics in the document response template. Configuration files documenting this parameter value were uploaded to the /jvet-cfp/anchors-jem/HDR/hlg/cfgFiles directory on January 10, 2018.

- It was commented that the use of HDRTools v0.15 and v0.17 may result in different md5sums for the resulting EXR files. It was confirmed that this difference was due only to the header data and did not correspond to a change in the image data.

See also the notes for the AHG1 report.

In the discussion, it was remarked that for PSNR calculations there may be some ambiguity over the peak value to be used in the PSNR calculations, (255*4)^2 or 1023^2. This makes a difference of 0.0255 dB in the result. As the HM and JM use the latter, it was agreed that this is what should be used, and this should be communicated to the proponents.

There were 2 contributions identified as related to HDR video coding:

JVET-I0025 Clarifications on HDR Test for CfP on Video Compression beyond HEVC [P. Yin, W. Husak, T. Lu, F. Pu, T. Chen (Dolby), S. Iwamura (NHK)]

JVET-I0026 AHG7: HLG anchor generation of S31 and S32 for Joint Call for Proposals on Video Compression with Capability beyond HEVC [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)]

The AHG recommended to:

•	Review all input contributions

•	Discuss any open issues related to the Call for Proposals

JVET-I0008 JVET AHG report: 360° video coding tools and test conditions (AHG8) [J. Boyce, A. Abbas, E. Alshina, G. van der Auwera, Y. Ye]

This document summarizes the activity of AHG8: 360º  video coding tools and test conditions between the 8th Meeting in Macao, CN, 18–24 Oct. 2017 and the 9th meeting in Gwangju, KR (20–26 Jan 2018).

There was no AHG email activity on the main jvet reflector, jvet@lists.rwth-aachen.de, with an [AHG8] indication on message headers.

The main activity of the AHG was producing output documents. Two versions of output document JVET-H1004 “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 5)” were made available, on Nov 14 and Nov 18. Output document JVET-H1030 “JVET common test conditions and evaluation procedures for 360° video” was made available on Oct 24.

There were 2 contributions related to 360º video coding, which are listed below.

JVET-I0023

Methods of Padding Inactive Regions for Segmented Sphere Projection	Y.-U. Yoon, D.-H. Park, J.-G. Kim (KAU), Y. Ahn, D. Sim (Digital Insights)

JVET-I0024

Efficient Frame Packing Method for Icosahedral Projection	H.-H. Kim, Y.-U. Yoon, D.-H. Park, J.-G. Kim (KAU), Y. Ahn, D. Sim (Digital Insights)

The AHG recommends the following:

- Review input contributions

- Review 360° video test material, and consider adding or replacing test sequences for common test conditions

- Review common test conditions for 360 video, including objective metrics and viewports

JVET-I0009 JVET AHG report: Entropy Coding Trade-offs (AHG9) [A. Said, E. Alshina, S.-H. Kim]

The document summarizes activities of the AhG on Entropy Coding trade-offs in the JEM software, which took place between the 8th and the 9th JVET meetings.

Unfortunately, due to the proximity of the CfP response deadline, there was no reported activity on this topic from the interim period.

The AhG recommended to maintain this AhG until the CfP responses are evaluated during the 10th JVET meeting.

In the discussion, it was remarked that it seems feasible to expect some progress in the next period, at least in the form of input contributions to the next meeting.

JVET-I0010 JVET AHG report: Denoising and adaptive quantization [R. Sjöberg, E. Alshina, S. Ikonin, A. Norkin, T. Wiegand]

The document summarizes the activities of the AHG on Denoising and adaptive quantization between between the 8th meeting at Macao, CN (October 18–24, 2017) and the 9th Meeting at Gwangju, KR (January 20–26 2018).

One message related to AhG 10 activity was sent to the JVET reflector, on 7 December, which concerned an update of the HHI software contributed to the previous meeting. The subject line was "[AHG10] QP adaptation update and faster NextSoftware 1.8.5 released".

The announced revision was uploaded as a revision of the input document to the previous meeting.

There was no further discussion, and so no conclusion on the suggestion was reached by the AHG.

In the review of the AHG report, it was expressed that there is continuing interest in this topic.

# Analysis, development and improvement of the JEM (0)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

# Test material (0)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

# Call for Proposals preparation (3)

Contributions in this category were discussed Saturday 20 Jan. 1100–1215 (chaired by GJS & JRO).

JVET-I0025 Clarifications on HDR Test of CfP on Video Compression beyond HEVC [P. Yin, W. Husak, T. Lu, F. Pu, T. Chen (Dolby), S. Iwamura (NHK)]

Sat 1120 GJS & JRO

This contribution requests a few clarifications regarding the High Dynamic Range Video tests described in JVET-H1002 “Joint Call for Proposals on Video Compression with Capability beyond HEVC”. The clarification relates to two aspects: 1) the input video format to the HDR display for coded test material submission; 2) reference software and configuration to be used to compute HDR objective metrics.

One issue raised is the conversion of the 4:2:0 YUV 10 bit decoded video (obtained from proponents) to the format needed for the display (e.g., 4:2:2 YUV or 4:4:4). It was remarked that it would be desirable to have information about the exact process to be applied, so that a proponent could replicate the subjective display conditions. This could apply not only to the HDR case, but also to the SDR case as well. One test sequence ("Cosmos") was noted to be 1920x856 rather than 1920x1080, so part of that conversion process could include a resolution conversion process, if necessary. Revisit to obtain such information.

The BVM X300 reportedly supports two HLG modes: HLG SG 1.2 (HDR) and HLG SG Variable (HDR) (default system gamma value is 1.2). Since HLG SG Variable (HDR) is reportedly aligned with ITU-R BT 2100, it is suggested to use HLG SG Variable mode for the HDR HLG content test. It was verbally suggested that the system gamma should be set to 1.2 for the variable mode, and it was remarked that the variable mode with system gamma set to 1.2 was used in the evaluation of test sequences for selection. Decision: This was agreed (using the variable mode with the system gamma set to 1.2).

Information about the version of HDRtools used for computing wPSNR was requested. This metric was not supported in HDRtoold v0.15, which is the version prescribed for use in the CfP. The CfP says that the implementation in the JEM software is to be used (although it would be difficult for proponents to use this, since it is built into the JEM and that is not a codebase that proponents would be expected to be using). The contributor suggested using HDRtools v0.17. It was remarked that any difference between what is in the JEM software and the implementation in HDRtools v0.17 is negligible. The measurements used for the anchors were as specified in the CfP. Decision: Proponents may use either the v0.17 HDRtools implementation or the JEM implementation as they so choose, since the difference seems negligible. Revisit to confirm.

The contribution suggested that for the deltaE100 and PSNR-L100 metrics, the peak luminance should be set to 1,000 cd/m2 for HDR-A (HLG) and 10,000 cd/m2 for HDR-B (PQ). It was noted that the CfP says to use 10,000 cd/m2 but also says that configuration files would be provided, and the provided configuration files for the conversion process used 1,000 cd/m2 for the HLG content. Thus the suggestion from the contributor is in line with the config files that have been distributed. Decision: Agreed as suggested. If a proponent uses a different convention, the difference would be obvious and easy to correct by simply adding 10 to all provided measurements (Revisit to confirm that).

JVET-I0026 AHG7: HLG anchor generation of S31 and S32 for Joint Call for Proposals on Video Compression with Capability beyond HEVC [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)] [late]

Sat 1100 GJS & JRO

This contribution provides information regarding HLG anchor generation for the DayStreet (S31) and the PeopleInShoppingCenter (S32) for Joint Call for Proposals on Video Compression with Capability beyond HEVC. Encoding results are reported for HM-16.16 as well as JEM-7.1 at given bit rate points which were identified in last meeting.

The table below provides the target and matched rates with the QP parameters, based QP and QPIncrementFrame (QPIF), for two HLG sequences. For each rate the rate difference is about 0.7% at maximum as shown in the table. A peak luminance of 1,000 cd/m2 was used for HLG graded contents whereas 10,000 cd/m2 for PQ graded contents. The number to be used was requested in JVET-I0025 to be clarified. The results were reportedly crosschecked by NHK.

Bit rate of HM-16.6 and JEM-7.1

| Sequence | Target rate (kbps) | HM-16.16 |  |  |  | JEM-7.1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kbps | baseQP | QPIF | Diff | kbps | baseQP | QPIF | Diff |
| DayStreet | 680 | 679.8 | 46 | 598 | 0.0% | 679.0 | 43 | 254 | −0.1% |
|  | 1100 | 1094.9 | 41 | 64 | −0.5% | 1098.2 | 40 | 512 | −0.2% |
|  | 1750 | 1749.5 | 38 | 280 | 0.0% | 1749.8 | 36 | 64 | 0.0% |
|  | 2800 | 2800.0 | 35 | 532 | 0.0% | 2798.0 | 33 | 176 | −0.1% |
|  | QP32 | 4402.5 | 32 |  |  |  |  |  |  |
| PeopleInShoppingCenter | 640 | 636.3 | 46 | 574 | −0.6% | 635.5 | 44 | 318 | −0.7% |
|  | 960 | 959.9 | 42 | 359 | 0.0% | 959.9 | 41 | 551 | 0.0% |
|  | 1450 | 1449.8 | 39 | 516 | 0.0% | 1450.0 | 37 | 184 | 0.0% |
|  | 2150 | 2148.8 | 35 | 39 | −0.1% | 2141.5 | 34 | 320 | −0.4% |
|  | QP33 | 3094.9 | 33 |  |  |  |  |  |  |

JVET-I0030 Suggested changes to JVET-H1003 Draft template of proposal description document [Y. Ye, Y. He (InterDigital)] [late]

TBP. The contributor was not available to present this on Saturday 19 January morning.

# Non-EE Technology proposals (5)

## Intra coding (0)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

## Inter coding (0)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

## Loop filters (5)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

JVET-I0021 Chroma Artifact Reduction by New Sample Adaptive Offset method for Chroma [E. Sasaki, J. Ono (Socionext Inc.)]

JVET-I0027 Cross-check of JVET-I0021: Chroma Artifact Reduction by New Sample Adaptive Offset method for Chroma [T. Zhou, T. Ikai (Sharp)] [late]

JVET-I0031 Cross-check of JVET-I0021: Chroma Artifact Reduction by New Sample Adaptive Offset method for Chroma [S. Ye, L. Wang, F. Chen, X. Zuo (Hikvision)] [late]

JVET-I0022 Convolutional Neural Network Filter (CNNF) for intra frame [L. Zhou, X. Song, J. Yao, L. Wang, F. Chen (Hikvision)]

JVET-I0032 Cross-check of JVET-I0022: Convolutional Neural Network Filter (CNNF) for intra frame [X. Zhao (Tencent)] [late] [miss]

## Partitioning and related (0)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

# Extended colour volume coding (0)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

# Coding of 360° video projection formats (2)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

JVET-I0023 Methods of Padding Inactive Regions for Segmented Sphere Projection [Y.-U. Yoon, D.-H. Park, J.-G. Kim (KAU), Y. Ahn, D. Sim (Digital Insights)]

JVET-I0024 Efficient Frame Packing Method for Icosahedral Projection [H.-H. Kim, Y.-U. Yoon, D.-H. Park, J.-G. Kim (KAU), Y. Ahn, D. Sim (Digital Insights)]

# Complexity analysis (0)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

# Encoder optimization (1)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

JVET-I0028 Report of H0091 Restriction of fast intra-mode decision [P.-H. Lin, C.-L. Lin, C.-C. Lin (ITRI)] [late]

# Metrics and evaluation criteria (0)

Contributions in this category were discussed XXday XX Jan. XXXX–XXXX (chaired by XXX).

# Withdrawn (1)

JVET-I0029 Withdrawn

# Joint Meetings, BoG Reports, and Summary of Actions Taken

## Joint meetings

The CfP was presented in a joint meeting with the parent bodies Mon 1630–1800. The ToR for the Joint Video Experts Team to be tasked with the development of a future video coding standard was also presented.

## BoGs (0)

## List of actions taken affecting JEM7 and 360lib5 (update or delete)

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the JEM7 or 360Lib5.0 description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

Was presented and confirmed to be complete Tuesday 24th in the JVET plenary.

### Encoder only or CTC/software changes

New CTC (JVET-H1010) reflects updates of Excel sheets (as per JVET-H0030).

General: It was agreed to ordinarily report two digits past the decimal point of percentage BD impacts. The basis for revisiting the last meeting’s decision in this regard is that some confusing cases of reports were found when rounding is made.

### Syntax/semantics/decoding process changes

None.

### Changes in 360lib

JVET-H0056 AHG8: An Update on RSP Projection [A. Abbas, D. Newman (GoPro)]

Decision(360lib): Replace the current RSP by the method from JVET-H0056.

General: It had been agreed by the 7th JVET meeting that the list of projection formats included in the CTC & 360Lib will not grow further, to avoid having so many that we can’t properly study them. If we want to add one, we need a decision to remove one. Anchors for projection formats to be made available only with HM and ERP for JEM. The action taken is in line with this policy

# Project planning

## Exploration Experiment planning (update)

No EEs were established.

## JEM description drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- JEM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be Thursday 11 Jan. 2018.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for Experiments

Note: This section was drafted during the second JVET meeting, and is kept here for information about the EE procedure. It may become relevant in the future again.

Group coordinated experiments have been planned. These may generally fall into one category:

- “Exploration experiments” (EEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the main branch of JEM by the next meeting.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. (E. Alshina will edit the document based on input from the proponents, review is performed in the plenary)

- Software for tools investigated in EE is provided in a separate branch of the software repository

- During the experiment, further improvements can be made

- By the next meeting it is expected that at least one independent party will report a detailed analysis about the tool, confirms that the implementation is correct, and gives reasons to include the tool in JEM

- As part of the experiment description, it should be captured whether performance relative to JEM as well as HM (with all other tools of JEM disabled) should be reported by the next meeting.

It is possible to define sub-experiments within particular EEs, for example designated as EEX.a, EEX.b, etc., where X is the basic EE number.

As a general rule, it was agreed that each EE should be run under the same testing conditions using one software codebase, which should be based on the JEM software codebase. An experiment is not to be established as a EE unless there is access given to the participants in (any part of) the TE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-G1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a company proponent perspective – e.g. referring to methods as “improved”, “optimized” etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to EE work should identify individuals in addition to company names.

EE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the EE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish an EE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to EE activities were established as follows:

- Only qualified JVET members can participate in an EE.

- Participation in an EE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the EE should be announced and made available to all EE participants in a timely manner.

A separate branch under the experimental section will be created for each new tool include in the EE. The proponent of that tool is the gatekeeper for that separate software branch. (This differs from the main branch of the JEM, which is maintained by the software coordinators.)

New branches may be created which combine two or more tools included in the EE document or the JEM. Requests for new branches should be made to the software coordinators.

Don’t need to formally name cross-checkers in the EE document. To promote the tool to the JEM at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

## Software development and anchor generation

The planned timeline for software releases was established as follows:

- JEM7.1 will be released by 2017-25-10.

- Further versions may be released for additional bug fixing, as appropriate

Timeline of 360lib5.0: 2 weeks after the meeting (2017-11-10).

- Further versions may be released as appropriate for bug fixing.

CfP anchors will be updated as necessary (same responsibilities as from 7th meeting)

HDR: NHK/Sony will provide (and verify) HDR-A anchors (Nov. 10)

- For SDR: HD/RA, HD/LD, UHD: Samsung/Qualcomm (no update necessary)

- For 360: InterDigital/Samsung (Nov. 10)

- For HDR-B: Technicolor/Qualcomm (Nov. 10)

New HM anchors will be generated using HM 16.16. JEM anchors will be based on JEM 7.0.

# Output documents and AHGs

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JVET-H1000 Meeting Report of the 8th JVET Meeting [G. J. Sullivan, J.-R. Ohm] [2018-01-18] (near next meeting)

Intermediate versions of the meeting notes (d0 … d5) were made available on a daily basis during the meeting.

Remains valid – not re-issued: JVET-G1001 Algorithm description of Joint Exploration Test Model 7 (JEM7) [J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce]

Remains valid (from previous meeting).

JVET-H1002 Joint Call for Proposals on Video Compression with Capability beyond HEVC [A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki] [2017-10-24] (MPEG N17195)

In the closing plenary, various additional edits were approved; in particular aspects of testing fee and payment were clarified.

JVET-H1003 Draft template of proposal description document [M. Zhou, J. Chen, E. François, P. Hanhart] [2017-11-17]

Includes Excel templates for the cases of SDR, HDR and 360° video for data to be provided by proponents. All anchor data will be included by the date of final availability.

JVET-H1004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 5 [Y. Ye, E. Alshina, J. Boyce] [2017-11-17] (MPEG N17197)

The only change is the new version of RSP from JVET-H0056.

JVET-H1010 JVET common test conditions and software reference configurations [K. Sühring, X. Li] [2017-11-10]

Reflects updates of Excel sheets (as per JVET-H0030).

JVET-H1020 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, D. Rusanovskyy] [2017-11-10]

Reflects updates of Excel sheets (as per JVET-H0030).

+ include the four HLG sequences Birds, DayStreet, SunsetBeach, ShoppingMall (correct names)

JVET-H1030 JVET common test conditions and evaluation procedures for 360° video [E. Alshina, J. Boyce, A. Abbas, Y. Ye] [2017-11-17]

Reflects updates of Excel sheets (as per JVET-H0030) and new RSP version (JVET-H0056).

+ Replace Landing by Landing2

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header should be generated. This version should be sent to GJS and JRO for upload.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| CFP preparation (AHG1) (jvet@lists.rwth-aachen.de) - Raise awareness of, and distribute the Call for Proposals. - Coordinate the registration of proposals. - Coordinate distribution of test material and anchors to registered proponents. - Discuss and improve the proposal description template JVET-H1003. - Make logistic arrangements for the Call for Proposals. | J.-R. Ohm, G. Sullivan, V. Baroncini, M. Zhou | N |
| JEM algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Develop and propose improvements to JVET-G1001 Algorithm Description of Joint Exploration Test Model 7. - Gather and address comments for refinement of the document. - Coordinate with the JEM software development AHG to address issues relating to mismatches between software and text. | J. Chen (chair), E. Alshina, J. Boyce (vice chairs) | N |
| JEM software development (AHG3) (jvet@lists.rwth-aachen.de) - Continue development of the JEM software package. - Produce documentation of software usage for distribution with the software. - Prepare and deliver JEM reference configuration encodings according to JVET-H1010 common testing conditions, if necessary in case of bug fix fixing. - Coordinate with AHG on JEM model editing to identify any mismatches between software and text, and make further updates and cleanup to the software as appropriate. - Investigate the implementation of SCC coding tools in JEM. - Coordinate with AHG6 for integration of 360° video software. | X. Li, K. Sühring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of future video coding standards. - Identify and recommend appropriate test materials and corresponding test conditions for use in the development of future video coding standards. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Evaluate new test sequences, and prepare for the visual assessment and availability of viewing equipment in the next meeting. | V. Baroncini, T. Suzuki (co-chairs), J. Chen, J. Boyce, A. Norkin (vice chairs) | N |
| Memory bandwidth consumption of coding tools (AHG5) (jvet@lists.rwth-aachen.de) - Study the methodology of measuring decoder memory bandwidth consumption, including cache models. - Develop software tools for measuring both average and worst case of memory bandwidth. - Make analysis of memory bandwidth needs for examples of JEM coding tools. - Study the impact of memory bandwidth on specific application cases. | X. Li (chair), E. Alshina, R. Hashimoto, T. Ikai, H. Yang (vice chairs) | N |
| 360° video conversion software development (AHG6) (jvet@lists.rwth-aachen.de) - Prepare and deliver 360Lib-5.0 software version and common test condition configuration files according to JVET-H1030. - Generate CTC HM anchors for all projection formats, CTC JEM anchors for the padded ERP format, and finalize the reporting template for the common test conditions. - Produce documentation of software usage for distribution with the software. | Y. He (chair), K. Choi, V. Zakharchenko (vice chairs) | N |
| JEM coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Coordinate generation of HM and JEM anchors for the CfP. - Study and evaluate available HDR/WCG test content, including appropriate displays for subjective evaluation of the content. - Study objective metrics for quality assessment of HDR/WCG material. - Evaluate transfer function conversion methods. - Study and refine test conditions and anchors for the coding of HDR/WCG content. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. François, D. Rusanovskyy (vice chairs) | N |
| 360° video coding tools and test conditions (AHG8) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Study consistency of and potential improvements to the objective quality metrics in CTC. - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Produce and finalize JVET-H1004 algorithm descriptions of projection format conversion process and objective quality metrics in 360Lib. - Produce and finalize JVET-H1030 JVET common test conditions and evaluation procedures for 360 video. - Study coding tools dedicated to 360° video, and their impact on compression. - Study the effect of viewport resolution, field of view, and viewport speed/direction on visual comfort. - Study the impact of coding resolution vs original ERP resolution on coding efficiency. | J. Boyce (chair), A. Abbas, E. Alshina, G. v. d. Auwera, Y. Ye (vice chairs) | N |
| Entropy Coding Trade-offs (AHG9) (jvet@lists.rwth-aachen.de) - Study factors that affect computational complexity of probability estimation for entropy coding. - Identify and study new methods for probability estimation. - Provide initial complexity analysis of new methods, and coding gain trade-offs, - Discuss impact of eliminating context initialization dependence on slice type, when new estimation methods are employed. - Discuss potential impact of new entropy coding on current JEM RD decisions. | A. Said (chair), E. Alshina, S.-H. Kim (vice-chair) | N |
| Denoising and adaptive quantisation (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using adaptive quantization in context of HM and JEM coding. - Prepare HM and JEM bitstreams using adaptive QP matching the rates of the CfP, for comparison against the CfP anchors. - Perform visual quality assessment for cases using denoising filters, renoising, and adaptive quantisation. - Study objective error metrics for measuring small subjective compression efficiency improvements when adaptive quantisation is used. - Solicit input contributions demonstrating subjective benefits over the JEM 7 anchor. | R. Sjöberg (chair), E. Alshina, S. Ikonin, A. Norkin, T. Wiegand (vice chairs) | N |

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Tuesday or Wednesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–7.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Thursday or Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 8.5 meeting days).

In cases where high workload is expected for a meeting, an earlier starting date may be defined.

Some specific future meeting plans (to be confirmed) were established as follows:

- Tue. 10 Apr. – Fri. 20 Apr. 2018, 10th meeting under WG 11 auspices in San Diego, US.
(Note: Tue. 10 Apr. is planned only for cross-checking of bitstreams by CfP proponents)

- Tue. 10 – Wed. 18 July 2018, 11th meeting under ITU-T auspices in Ljubljana, SI.

- Thu. 4 – Fri. 12 Oct. 2018, 12th meeting under WG 11 auspices in Macao, CN.

- Tue. 8 – Wed. 16 January 2019, 13th meeting under ITU-T auspices in Marrakesh, MA.

The agreed document deadline for the 10th JVET meeting is XXday XX Apr. 2018. Plans for scheduling of agenda items within that meeting remain TBA.

XXX was thanked for the excellent hosting of the 9th meeting of the JVET. XXX were thanked for providing viewing equipment. XXX were thanked for offering new test sequences.

The 9th JVET meeting was closed at approximately XXX hours on Friday 26 Jan. 2018.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the ninth meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately XXX people in total), were as follows: