| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Gwangju, KR, 20–26 Jan. 2018 | Document: JVET-I0021 |
| --- | --- |

| Title: | Chroma Artifact Reduction by New Sample Adaptive Offset method for Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eiichi Sasaki, Jun Ono 2-10-23 Shin-Yokohama, Kohoku-ku, Yokohama, Kanagawa, 222-0033, Japan | Tel: Email: | +81-45-568-1078 sasaki.eiichi@socionext.com ono.jun@socionext.com |
| Source: | Socionext Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a new Sample Adaptive Offset (SAO) method for chroma called Chroma Artifact Reduction SAO (CAR SAO). The main purpose of CAR SAO is to reduce Chroma Artifact that degrades subjective quality and appears especially around colorless area. CAR SAO focuses on reducing chroma distortion around colorless area with less bit consumption than legacy SAO types by decoder side confining the area where offsets are assigned.

Simulation results show that only the proposed SAO type in chroma has cb/cr BD-rate of –13.92/-41.74% in best case without luma degrade and also provides improvement of subjective quality by reducing Chroma Artifact.

Problem Statement

Chroma distortion in colorless areas causes Chroma Artifacts and deteriorations in subjective quality. In YUV format, chroma values of the colorless area are the median of gamut. JCTVC-Z1017[1] reported this problem in HDR/WCG contents and showed an example of Chroma Artifact. Even if chroma distortion is not large, Chroma Artifacts appear around colorless and HDR/WCG contents are more strongly affected than SDR contents. JCTVC-Z1017 showed Chroma QP offset settings for reducing Chroma Artifacts, and simulation results of JEM with Chroma QP offset settings were reported by JVET-G0084[2]. The results show improvement of subjective quality by reducing Chroma Artifacts. However, using Chroma QP offset must degrade the luma quality relatively.

Technology description

This contribution proposes a new Sample Adaptive Offset (SAO) method for chroma called Chroma Artifact Reduction SAO (CAR SAO) to reduce Chroma Artifact without degrading the luma quality.

SAO has been adopted in HEVC[3] as well as current JEM[4]. SAO has two types: Edge Offset (EO) and Band Offset (BO). Regarding the classification for adding offsets in SAO, EO is based on comparison between target pixel and two neighbor pixels defined by EO Classes (0°, 90°, 135° and 45°) and BO has 32 classes based on pixel values, and length of each classes is set as bitdepth >> 5.

The classification of CAR SAO is based on pixel values like BO. Focusing Color Artifact, the length of each classes is set as bitdepth >> 7 and number of classes is only 6 that around median of chroma pixel.  Figure 1 shows classification of BO and CAR SAO.

Figure 1 Classification of BO and CAR SAO

In case of BO, 4 continuous classes are selected by the band-position syntax and the offsets are signaled and assigned for these classes. In case of CAR SAO, 3 classes are selected by counting up the number of pixels that belong to each class in decoding process to reduce bit consumption. Table 1 shows an example of CAR SAO class selection.

Table 1 Example of class selection for CAR SAO

| class index | range of class (in 10bit) | number of pixels in CTB |
| --- | --- | --- |
| 0 | 488 - 495 | 2327 |
| 1 | 496 - 503 | 1134 |
| 2 | 504 - 511 | 134 |
| 3 | 512 - 519 | 158 |
| 4 | 520 - 527 | 0 |
| 5 | 528 - 535 | 0 |

The classes are selected in descending order of the number of pixels.

In case of Table 1, offsets are signaled and assigned for class index 0, 1, 3.

Modify RDCost calculation of CAR SAO for HDR

The proposed CAR SAO improves subjective quality by reducing Chroma Artifacts.

However, according to subjective test of HDR contents, it seems CAR SAO should be applied rather in blight area for further improvements of subjective quality.

We have modified the RDCost calculation of chroma SAO type decision as (1).

|  | (1) |
| --- | --- |

W  is weight of HDR wPSNR[5] and calculated from average of luma pixel values in CTB.

Simulation results

The proposed method has been implemented on top of HM16.6-JEM7.1. Test sequences are the contents reported chroma improvement by JVET-G0084 and results are shown in Table 2. There are two test cases: Test 1 is applied CAR SAO and remove other SAO types (EO, BO) in chroma, and test 2 is applied modified RDCost calculation in section 3 to test1 (It’s for only HDR test).

Table 2 Performance of proposed method on the top of HM16.6-JEM7.1

|  | RA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test1: CAR SAO |  |  |  |  | Test2: CAR SAO with WRDO |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |  |
| Class B | 0.00% | 0.49% | 0.12% | 100% | 100% |  |  |  |  |  |
| Class C | -0.02% | 0.95% | 1.33% | 100% | 100% |  |  |  |  |  |
| Class D | 0.00% | 0.73% | 0.40% | 100% | 100% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |  |

|  | RA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test1: CAR SAO |  |  |  |  | Test2: CAR SAO with WRDO |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| HLG-1 | 0.06% | -0.36% | -2.64% | 100% | 109% | 0.05% | -1.05% | -4.73% | 101% | 108% |
| HLG-4 | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| HLG-5 | 0.02% | -11.75% | -36.54% | 96% | 92% | 0.03% | -13.92% | -41.74% | 98% | 98% |
|  | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |

Regarding results of subjective evaluation, CAR SAO can reduce Chroma Artifact in the contents and test2 has better subjective quality than test1 about HDR contents.

Conclusion

This contribution proposes Chroma Artifact Reduction Sample Adaptive Offset. CAR SAO provides subjective quality improvement by reducing Chroma Artifact. It is suggested to further study proposed method and Chroma Artifact. And it is also suggested to continuously evaluate the contents that has Chroma Artifact problem like HLG-1, HLG-4 and HLG-5.

References

- Jonatan Samuelsson, Chad Fogg, Andrey Norkin, Andrew Segall, Jacob Ström, Gary Sullivan, Pankaj Topiwala, and Alexis Tourapis, “Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics (Draft 4)”, JCTVC-Z1017, Geneva, CH, 12–20 January 2017.

- Kei Kawamura and Sei Naito, “Luma/Chroma QP Adaptation for Hybrid Log-Gamma Sequences Encoding”,JVET-G0084, Torino, IT, 13-21 July 2017.

- C. Fu, E. Alshina, A. Alshin, Y. Huang, C. Chen, Chia. Tsai, C. Hsu, S. Lei, J. Park, W. Han, “Sample adaptive offset in the HEVC standard, EIEEE Trans. Circuits Syst. Video Technol., 22(12): 1755 E764 (2012)

- Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm and Jill Boyce, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, Torino, IT, 13-21 July 2017

- A. Segall, E. François and D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-G1020, Torino, IT, 13-21 July 2017

Patent rights declaration(s)

Socionext Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).