| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Gwangju, KR, 20–26 Jan. 2018 | Document: JVET-I0030 |
| --- | --- |

| Title: | Suggested changes to JVET-H1003 Draft template of proposal description document |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye, Yuwen He 9710 Scranton Rd, #250 San Diego, CA 92121 USA. | Tel: Email: | +18582104803 {yan.ye,yuwen.he}@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

JVET-H1003 “Draft template of proposal description document” provides a template to be used by the proponents to describe their CfP responses. This document suggests some changes, clarifications, and one addition to Section 4 of this template.

# Suggested changes and clarifications

## Suggested changes to encoding and decoding time reporting in JVET-H1003

Section 4 of the JVET-H1003 document provides template for proponents to report the encoding and decoding complexity of their proposal. Most of the content in Section 4 seems to be inherited from the HEVC CfP template. Looking at the HEVC CfP responses, how proponents actually reported their proposal’s complexity seemed to be wide ranging. For example, for encoding and decoding time reporting (Sections 4.1 and 4.2), some HEVC CfP responses provided their proposal’s timing results, but did not compare with the JM anchor’s timing. Other HEVC CfP responses not only provided their timing results, but also did a direct comparison with JM’s timing results. To better understand how different proposals compare to each other and to the anchors, it is suggested to ask proponents to provide direct comparison with the HM and the JEM anchors in terms of encoding time and decoding time ratios. To provide meaningful comparisons, the proponents should collect timing results of the HM and JEM anchors on the same computing platform as their proposal.

Further, for the Random Access configuration, most of the proposals (if not all) are now anticipated to use parallel encoding to generate their response bitstreams. This was not the case in HEVC CfP time. Therefore, it is suggested to provide guidance in JVET-H1003 on how the encoding and decoding time should be measured when parallel encoding and decoding are used. It is suggested to use the same encoding time measurement methodology as what had been used in JVET EE’s.

Specifically, it is suggested to add the highlighted text to 4.1 in JVET-H1003:

4.1 Encoding time and measurement methodology

Proponents are requested to provide relative encoding time ratios of their proposal compared to the HM and JEM anchors.

When parallel encoding is used to generate the bitstreams, the following method should be used to measure the encoding time of the proposal. Encoding log files should be parsed for each IRAP. And total encoding time should be calculated by adding the encoding time of all IRAPs and then subtracting the encoding time of the first picture for all IRAPs except the first IRAP. In other words, encoding time can be measured as

TotalEncTime=i=0NEncTime of IRAP(i)-i=1NEncTime of first picture in IRAP(i)

Given that parallel encoding will generate a number of IRAP bitstreams for each sequence at each rate point, it is suggested to clarify that decoding time should be measured after the IRAP bitstreams are concatenated into one bitstream and the decoding time should be measured by decoding the concatenated bitstreams. Further, in JVET-H1003, in 4.2 on decoding time reporting, proponents are asked to report decoding time with YUV output enabled. This could create problems and timing fluctuations given the specific computer configuration, especially the hard disk read/write speed, I/O bandwidth, etc. This could be especially problematic given the large number of 4K sequences in the CfP test set as writing out large 4K YUV outputs could significantly impact the decoding time. Therefore, it is suggested to request decoding time with YUV output disabled rather than enabled.

Specifically, it is suggested to use the highlighted text in 4.2 in JVET-H1003:

4.2 Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

(Comparison to HM 16.16 should be with YUV output enabled disabled and reference input disabled; additional reporting of speeds with the yuv output disabled enabled is optional encouraged.)

Proponents are requested to provide relative decoding time ratios of their proposal compared to the HM and JEM anchors.

When parallel encoding is used to generate the bitstreams, the proponents are requested to first concatenate the IRAP bistreams and report decoding time of concatenated bitstreams.

## Clarification to reporting encoder and decoder memory usage in JVET-H1003

Looking at the HEVC CfP responses, how proponents actually reported their proposal’s memory usage also seemed to be wide ranging. Some HEVC CfP responses provided specific memory usage numbers in GBs, while other proposals only provided qualitative descriptions such as “comparable to H.264”. In order to better understand how different proposals compare to each other, it is suggested to ask proponents to provide quantitative information on encoder and decoder memory usage. It is further suggested to encourage proponents to provide direct comparison with the HM and the JEM anchors in terms of memory usage.

Encoder and decoder memory usage could not only be impacted by the proposed coding technologies, but also by the content (esp. resolution) and the target bit rate (though the memory usage impact due to target bit rate tends to be small). In order to focus the discussion on memory usage impact due to proposed coding technologies, it is suggested to select one rate point per sequence (e.g. the highest rate point) and report memory usage for that rate point. Further, resolution has a big impact on memory usage. Therefore, it is suggested to report memory usage only for the highest resolution sequence class for a given encoding configuration, e.g., report memory usage only for Class A for Random Access and Class B for Low Delay.

Specifically, it is suggested to use the highlighted text in 4.4 and 4.5 in JVET-H1003:

4.4 Expected memory usage of encoder

Proponents are requested to provide quantitative measurement (e.g. in units of GB) of memory usage of their proposed encoder implementation. Proponents are also encouraged to compare their proposal’s encoder memory usage with that of the HM and JEM anchors.

For each constraint set, encoder memory usage measurement only needs to be reported for the sequence class of the largest resolution at the highest target rate point.

4.5 Expected memory usage of decoder

Proponents are requested to provide quantitative measurement (e.g. in units of GB) of memory usage of their proposed decoder implementation. Proponents are also encouraged to compare their proposal’s decoder memory usage with that of the HM and JEM anchors.

For each constraint set, decoder memory usage measurement only needs to be reported for the sequence class with the largest resolution at the highest target rate point.

## Suggestion to request information on special encoder optimization methods

In JVET-H1003, a variety of encoder information is requested, including encoder motion estimation and motion segmentation selection, encoder intra-frame prediction type selection, encoder transforms and transform type selection, encoder side motion data refinement, encoder in-loop filtering type selection, encoder entropy coding type selection, encoder quantization and quantization type selection, degree of capability for encoder parallel processing, etc. In JVET-H1002 the final Call, certain encoder optimization mechanisms are explicitly discouraged. For example, in JVET-H1002 section 4.1.2:

- Proponents are discouraged from optimizing encoding parameters using non-automatic means as well as optimizing encoding parameters on a per sequence basis within a Class.  When any optimization is employed, it shall be described.

To better match with information requested in the final Call, it is suggested to add a section in JVET-H1003 for proponents to report any special encoder optimization used in their proposal. This could include information explicitly listed in the CfP as discouraged (but not disallowed) encoder optimization, as well as other encoder optimization methods not used by the anchors, for example, adaptive GOP, multi-pass encoding, etc.

4.25 Additional complexity discussion topic #1

4.25 Additional encoder optimization

Proponents are requested to report any additional encoder optimization methods used in their proposal that are not used by the anchors, for example, non-automatic means of encoding parameter optimization, multi-pass encoding, adaptive GOP, etc..

# Patent rights declaration(s)

InterDigital Communications Inc. does not have any current or pending patent rights relating to the technology described in this contribution.