| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0011-v3 |
| --- | --- |

| Title: | Description of SDR video coding technology proposal by DJI and Peking University |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Zhao Wang, Xuewei Meng, Chuanmin Jia, Jing Cui, Suhong Wang, Shanshe Wang, Siwei Ma Peking University Weiran Li, Zexiang Miao, Xiaozhen Zheng DJI | Tel: Email: Tel: Email: | swma@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

This document reports DJI and Peking University’s response to the Call for Proposals (CfP) on video compression with capability beyond HEVC, jointly issued by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG). The response is implemented on top of JEM7.0 software. Four new coding tools are proposed in this document, which includes Non-Local Structure-based Filter, Adaptive Update Long-Term Reference, OBMC Improvement and Multi-hypothesis probability estimation entropy coding.

The document reports that -34.19%/-43.75%/-44.37% and -26.87%/-42.96%/-44.53% Y/Cb/Cr BD rate reduction over HM16.16 anchor for SDR constraint set 1 and 2 respectively. When compared to JEM7.0 anchor, -1.57%/-0.71%/-1.72% and -3.30%/-0.67%/-4.26% Y/Cb/Cr BD rate reduction are observed for SDR constraint set 1 and 2 respectively.

- Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.1.1	Bitstream structure	1

2.1.2	Entropy decoding	1

2.1.3	Conversion process from entropy decoded data to symbols	1

2.1.4	Decoder-side motion refinement	1

2.1.5	Inverse quantization	1

2.1.6	Inverse transforms	2

2.1.7	Motion compensation	2

2.1.8	Intra prediction	2

2.1.9	In-loop filtering	2

2.1.10	Additional tools	2

2.1.11	Additional algorithmic description topic(s)	2

2.2	360° Video	2

2.2.1	Projection format (including padding)	2

2.2.2	Quantization handling	2

2.2.3	360° video specific decoding tools	2

2.3	HDR	2

2.3.1	Post-processing	2

2.3.2	Quantization handling	2

2.3.3	HDR specific decoding tools	2

3	Encoder algorithm description	3

3.1	General algorithm description	3

3.1.1	Rate-distortion optimization	3

3.1.2	Partitioning structure selection	3

3.1.3	Intra prediction mode estimation and type selection	3

3.1.4	Motion estimation and motion segment selection	3

3.1.5	Mode decisions	3

3.1.6	In-loop filtering type selection and parameter estimation	3

3.1.7	Transforms and transform type selection	3

3.1.8	Quantization and quantization type selection	3

3.1.9	Entropy coding	3

3.1.10	Additional tools	3

3.1.11	Additional encoder optimization	4

3.1.12	Additional algorithmic description topic(s)	4

3.2	360°	4

3.2.1	Usage of projection format adaptation	4

3.2.2	360° Video specific encoding tools	4

3.2.3	Additional encoder optimization	4

3.3	HDR	4

3.3.1	Pre-processing	4

3.3.2	HDR specific encoding tools	4

3.3.3	Additional encoder optimization	4

4	Compression performance	4

4.1	SDR: Constraint set 1 configuration relative to HM anchor	4

4.1.1	Class SDR-A (RD-curves)	4

4.1.2	Class SDR-B (RD-curves)	4

4.1.3	Overall (BD-rate summary tables)	5

4.2	SDR: Constraint set 2 configuration relative to HM anchor	5

4.2.1	Class SDR-B	5

4.2.2	Overall	5

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	5

4.3.1	Class SDR-A	5

4.3.2	Class SDR-B	5

4.3.3	Overall	5

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	5

4.4.1	Class SDR-B	5

4.4.2	Overall	5

4.5	HDR: Constraint set 1 configuration relative to HM anchor	5

4.5.1	Class HDR-A	5

4.5.2	Class HDR-B	5

4.5.3	Overall	5

4.6	HDR: Constraint set 1 configuration relative to JEM anchor	6

4.6.1	Class HDR-A	6

4.6.2	Class HDR-B	6

4.6.3	Overall	6

4.7	360° Video: Constraint set 1 configuration relative to HM anchor	6

4.7.1	Class 360	6

4.7.2	Overall	6

4.8	360° Video: Constraint set 1 configuration relative to JEM anchor	6

4.8.1	Class 360	6

4.8.2	Overall	6

5	Decoder complexity analysis	6

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	6

5.2	Description of computing platform used to determine decoding times reported in section 5.1	6

5.3	Memory usage of decoder	7

5.4	Complexity characteristics of decoder entropy decoding operation	7

5.5	Complexity characteristics of decoder inverse quantization	7

5.6	Complexity characteristics of decoder inverse transform operation	7

5.7	Complexity characteristics of decoder-side motion refinement	7

5.8	Complexity characteristics of decoder motion compensation	7

5.9	Complexity characteristics of decoder intra-frame prediction operation	7

5.10	Complexity characteristics of decoder in-loop filtering operation	7

5.11	Complexity characteristics of additional decoder tools	7

5.12	Complexity characteristics of 360° video specific decoding tools	7

5.13	Complexity characteristics of HDR specific decoding tools	7

5.14	Degree of capability for decoder parallel processing	7

6	Encoder complexity analysis	8

6.1	Encoding time and measurement methodology	8

6.2	Description of computing platform used to determine encoding times reported in section 6.1	8

6.3	Memory usage of encoder	8

6.4	Complexity characteristics of encoder intra-frame prediction type selection	8

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	8

6.5.1	General	8

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	8

6.6	Complexity characteristics of encoder transforms and transform type selection	8

6.7	Complexity characteristics of encoder quantization and quantization type selection	8

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	9

6.9	Complexity characteristics of encoder entropy coding type selection	9

6.10	Complexity characteristics of additional encoder tools	9

6.11	Complexity characteristics of 360° video specific encoding tools	9

6.12	Complexity characteristics of HDR specific encoding tools	9

6.13	Degree of capability for encoder parallel processing	9

7	Algorithmic characteristics	9

7.1	Random access characteristics	9

7.2	Delay characteristics	9

7.3	Additional characteristics discussion topic(s)	9

8	Software implementation description	9

9	Highlighted aspects discussion	9

10	Closing remarks	9

11	Patent rights declaration(s)	9

12	Annex 1: Questionnaire	10

12.1	Codec level questions	10

12.2	Tool level questions	11

13	Annex 2: Example of filling questionnaire HM vs. JEM	14

13.1	Codec level Q&A	14

13.2	Tool level Q&A	15

13.2.1	Category 1. Partitioning structure	15

13.2.2	Category 2. Entropy coding	15

13.2.3	Category 3. Control data derivation process	15

13.2.4	Category 4. Decoder-side motion refinement	16

13.2.5	Category 5. Intra prediction	16

13.2.6	Category 6. Inter prediction	17

13.2.7	Category 7. Quantization	17

13.2.8	Category 8. Transforms	17

13.2.9	Category 9. In-loop filtering	18

13.2.10	Category 10. “tool-off” and “tool-on” tests	19

Introduction

This response is based on JEM7.0 platform for submitted SDR category. Compared to JEM7.0, four new coding tools are implemented and proposed. Besides the new coding tools implementation, the other aspects, including encoder optimization methods, are exactly same as JEM7.0. The proposed coding tools are summarized below:

Table 1-1: Summary of coding tools

| Tools | Short description |
| --- | --- |
| NLSF (Non-Local Structure-based Filter) | A noise reduction in-loop filter. |
| ALTR (Adaptive Update Long-Term Reference) | A new long-term reference that allows to be partial updated based on current coding frame. |
| OBMC Improvement | Minor change to OBMC that improve subjective quality. |
| Multi-hypothesis probability estimation entropy coding | Improvement on entropy coding. |

Decoder algorithm description

General algorithm description

Bitstream structure

The basic bitstream structure of J0011 is conceptually same as JEM7.0’s structure. The tool-level flags that switch on / off a tool are added to SPS and PPS and CTU level.

Entropy decoding

In this proposal, a new multi-hypothesis probability estimation method with counter-based updating speed is described to improve the probability estimation method in JEM7.0. The details of the new probability estimation method are described below.

The binary arithmetic decoder is applied with a “multi-hypothesis” probability update model based on two probability estimates P0 and P1 that are associated with each context model. Two probability estimates P0 and P1 are updated independently with different adaptation rates as follows:

where  and  (j=0, 1) represent the probabilities before and after decoding a bin, respectively. The parameters a and b are such that 2 ≤ a ≤ b ≤ 9 which are assigned for each context. Due to their limited range, a and b can be stored using only 4 bits each, and k represents the precision of probabilities (here it is equal to 15). The probability estimate P used for the interval subdivision in the binary arithmetic coder is the average of the estimates from the two hypotheses:

In JEM7.0, parameters a and b control the probability updating speed for each context model, and the default values of a and b are 4 and 8, respectively. To preserve the computational complexity of the current method, the bit-shift values a and b are set only when the context probability is re-initialized (i.e., when probabilities are set to pre-defined values, depending on the quantization parameter QP) at each slice. Since shift parameters and probabilities are changed together during context re-initialization, they were jointly optimized to obtain the largest gains per context.

In the proposed method, instead of having different arrays of context initializers for I, P, and B slices, the same arrays of initializers is used for all slice types, plus two pairs of shifts parameters (a, b) per context. One pair is applied when QP < 30, and the other for QP ≥ 30. With this setup, the total amount of memory required for context initialization is maintained as 3 bytes per context.

To further adjust the probability updating speed for each context adaptively, the bin counter c is assigned for each context as well, which represents the number of bins using the given context within each slice. The counter c is initialized as 0 at the beginning of each slice. Once the counter c is larger than a threshold Thr, the parameters a and b are adjusted as follows:

where (a’, b’) is the current shifts parameters of the current context. Thr is the pre-defined threshold which can be selected from 64, 128, 256 and 512 adaptively based on the characteristic of each context.

Regarding the initialization of context model, it keeps the same process as that in JEM7.0.

Conversion process from entropy decoded data to symbols

Same as JEM7.0’s implementation except for the addition of CTU level control flag for the proposed coding tools.

Intra prediction parameters

Same as JEM7.0’s implementation.

Motion parameters

Same as JEM7.0’s implementation.

Quantization parameters

Same as JEM7.0’s implementation.

Transform coefficients

Same as JEM7.0’s implementation.

In-loop filter parameters

A new in-loop filter, which is named as Non-Local Structure-based Filter (NLSF), is proposed. The related parameters are signaled at SPS, slice and CTU level, which will be described respectively.

SPS-level parameters:

A syntax element nonlocal_structure_filter_enabled_flag is signaled in the Sequence Parameter Set (SPS) to indicate whether NLSF is enabled in the current video sequence. If nonlocal_structure_filter_enabled_flag is true, the parameters which indicate the thresholds of SVD will be further signalled. The SPS signalling structure for NLSF is described below:

| seq_parameter_set_rbsp () { | Descriptor |
| --- | --- |
| …… |  |
| amp_enabled_flag | u(1) |
| nonlocal_structure_filter_enabled_flag | u(1) |
| if (nonlocal_structure_filter_enabled_flag){ |  |
| non-local loop filter I parameter k | ue(v) |
| non-local loop filter I parameter b | ue(v) |
| non-local loop filter B parameter k | ue(v) |
| non-local loop filter B parameter b | ue(v) |
| non-local loop filter B parameter k, Bi-prediction | ue(v) |
| non-local loop filter B parameter b, Bi-prediction | ue(v) |
| } |  |
| sample_adaptive_offset_enabled_flag | u(1) |
| …… |  |
| } |  |

Slice-level parameters:

If nonlocal_structure_filter_enabled_flag is true, two additional syntax elements, slice_nlsf_luma_flag and slice_nlsf_chroma_flag, are signalled in the slice header to indicate whether NLSF are enabled for luma and chroma components, respectively.

| slice_segment_header( ){ | Descriptor |
| --- | --- |
| …… |  |
| if (nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP){ |  |
| …… |  |
| } |  |
| if(nonlocal_structure_filter_enabled_flag){ |  |
| slice_nlsf_luma_flag | u(1) |
| if(slice_nlsf_luma_flag) |  |
| slice_nlsf_chroma_flag | u(1) |
| } |  |
| if(sample_adaptive_offset_enabled_flag){ |  |
| …… | u(1) |
| } |  |
| …… |  |
| } |  |

CTU-level parameter:

If nonlocal_structure_filter_enabled_flag is true and NLSF is enabled for luma component, the on/off flag of each CTU are signalled to decoder side, and the CTU level flag are coded by the bypass mode.

| coding_tree_unit() { | Descriptor |
| --- | --- |
| …… |  |
| if(sample_adaptive_offset_enabled_flag && (slice_sao_luma_flag ¦¦ slice_sao_chroma_flag)) |  |
| sao(xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY) |  |
| if(nonlocal_structure_filter_enabled_flag && slice_nlsf_luma_flag) |  |
| nlsf_ctu_flag | u(1) |
| …… |  |
| } |  |

Decoder-side motion refinement

Same as JEM7.0’s implementation.

Inverse quantization

Same as JEM7.0’s implementation.

Inverse transforms

Same as JEM7.0’s implementation.

Motion compensation

In this proposal, the Overlapped Block Motion Compensation (OBMC) in current JEM7.0 is improved. In the OBMC of JEM-7.0, the fixed weighting factors {1/4, 1/8, 1/16, 1/32} are used for PN and the weighting factors {3/4, 7/8, 15/16, 31/32} are used for PC. In this proposal, the weighting factors according to the size of the current CU are modified.

Usually, a CU with larger size has higher probability to be the background area and hence its MV can easily describe the motion field in the current CU. On the contrast, for the smaller CU, it tends to contain high texture and the boundary pixels may belong to the same object with the neighbouring blocks. Therefore, for the weighting factors used for PN, its value is increased for smaller CU and decreased for larger CU.

In particular, the CU size is evaluated by the CU width when process the left and right columns, and evaluated by CU height when process the top and bottom rows, which is called CU length in this proposal. For the first row/column, the weighting factors {1/4, 3/4} for {PN, PC} are modified to {1/5, 4/5} if the block length is larger than 32, and the weighting factors {1/4, 3/4} for {PN, PC} is modified to {1/3, 2/3} if the block length is less than 16. For the 2nd, 3rd and 4th row/column, considering the impact of OBMC is decreased, the weighting factors are kept as JEM7.0’s setting.

Intra prediction

Same as JEM7.0’s implementation.

In-loop filtering

NLSF (Non-Local Structure-based Filter) is a new in-loop filter that targets at the reduction of noise by using non-local reconstructed pixels. It is employed between Deblocking Filter (DF) and Sample Adaptive Offset (SAO), which includes group construction by block matching and SVD-based filtering processes.

Group construction by block matching for NLSF

As illustrated in Fig 2.1.9.1-1, the input reconstructed frame is firstly divided into K overlapped blocks, and each block serves as the basic loop unit. Let Bs × Bs denote the size of loop unit, which is set to 6 by default. For the current loop unit, no more than c (defaulted to 20) similar blocks are collected within the corresponding search window to construct the block group. As the example of Fig 2.1.9-1, the red square represents the current loop unit, meanwhile the blue dotted square denotes the search window, and the golden dotted square denotes the CTU current block’s location. In this proposal, the size of search window is set to 32x32 by default, and SSD (Sum of Square Differences) is adopted as the matching criterion. Details of block matching process are given bellow.

Fig 2.1.9.1-1 Illustrations for Non-Local Structure-based Filter

Step one: As shown in Fig 2.1.9.1-2, an eight-points diamond template is applied to search the most similar blocks in the search window. The center point of eight-points diamond template is the current loop unit labelled by “0” in Fig 2.1.9.1-2. The search process starts from the center point and successively search the points labelled by “1”, “2”, “4” and “8”. During the searching process, at most T (defaulted to 5) most similar blocks (golden points labelled by “8” in Fig2.1.9.1-3 with SSD smaller than threshold ε are selected and their coordinates are recorded. Estimation of threshold ε is described in section 3.1.6. The number of final selected blocks in step one is denoted as n (n≤T).

Fig 2.1.9.1-2 Block matching template

Step two. The n similar blocks selected in step one serve as the centers of the second round search, as shown in Fig 2.1.9.1-3. The similar blocks selected in step two are denoted by red points in Fig 2.1.9.1-3. After removing the reduplicative similar blocks, the number of selected blocks in the end is no more than c. A parameter m is used to represent the number of similar blocks selected in block matching process.

For each block, it is viewed as a vector xk∈RBs × Bs, k=1,2,…,K. Then, the current loop unit and the selected similar blocks generates a matrix XGk=[xGk⊗1, xGk⊗2, …, xGk⊗m].

Fig 2.1.9-3 Block matching process

SVD-based Filtering for NLSF

For each group XGk=xGk⊗1, xGk⊗2, …, xGk⊗m, k=1,2,…,K, it is decomposed by SVD as follows,

XGk= UGkGkVGkT= i=1mγxGk⊗i(uGk⊗ivGk⊗iT)

where γxGk=[γxGk⊗1;γxGk⊗2;…;γxGk⊗m] is a column vector, Gk=diag(γxGk) is a diagonal matrix with the elements of γxGk on its main diagonal, and uGk⊗i, vGk⊗i are the columns of UGk and VGk, separately. To suppress the compression noise, the hard thresholding operation is applied to the singular values, γxGk

αGk=hard(γxGk,τ),

where hardx,a=x⊙1 (absx-a) denotes the operator of hard thresholding and ⊙ denotes the element-wise product of two vectors. τ is the threshold described in section 3.1.6.1.

The filtered image can be reconstructed from the shrunk singular values αGk, and the reconstruction for group XGk can be formulated as,

XGk= i=1mαGk⊗i(uGk⊗ivGk⊗iT),

This process is applied for all K groups to achieve XGk, k=1,2,…,K. At last, all XGk are averaged to get the frame x.

Additional tools

A new tool which is named as Adaptive Updated Long-Term Reference (ALTR) is proposed. It generates and updates a long-term reference by using information of static areas, which are always regarded as background, among a video scene.

Usually, background areas have few motion in a long temporal window. Therefore the blocks with minor difference between the background and current frame are picked up, and are used to replace the co-located blocks in the long-term reference that targets at the renewal of the background information.

A flag use_altr_flag is use to control ALTR. If the flag is equal to 1, the reference lists of current coding slice will be modified. The long-term reference with POC equal to -1 will be added to the end of both reference list A and reference list B of the current slice.

If current slice is an IDR slice, all the CTUs in the slice will be marked to update the long-term reference, else if current slice is a P slice or a B slice, a syntax named block_lt_update flag will be parsed at CTU level. If the value of block_lt_update is equal to 1, the CTU will be marked to update the long-term reference.

After a slice is decoded and reconstructed completely, the process of updating the long-term reference will be performed. For every CTU marked to update the long-term reference, its Luma and Chroma reconstructed pixels will be used to replace the co-located pixels in the long-term reference, as illustrated in 2.1.10-1.

Fig 2.1.10-1 Illustration of block-level long-term reference update process

In the decoding process, if long-term reference is used as reference, motion vector scaling and decoder motion refine operation that use motion trajectory are invalid because the distance between the long-term reference and the current slice is not available and motion trajectory model doesn’t work for long-term reference. Therefore, the tools like BIO、DMVR、FRUC are set to disable if any of the motion vector is referred to the long-term reference.

Additional algorithmic description topic(s)

N/A

360° Video

Projection format (including padding)

Quantization handling

360° video specific decoding tools

HDR

Post-processing

Quantization handling

HDR specific decoding tools

Encoder algorithm description

General algorithm description

Rate-distortion optimization

Distortion metrics

Same as JEM7.0’s implementation.

Rate-distortion dependency models

Same as JEM7.0’s implementation.

Partitioning structure selection

Same as JEM7.0’s implementation.

Intra prediction mode estimation and type selection

Same as JEM7.0’s implementation.

Motion estimation and motion segment selection

General

Two new proposed tools “OBMC improvement” and “Adaptive Updated Long-Term Reference” have the impact on motion estimation and motion partitions.

OBMC improvement

In the JEM, for a CU with size less than or equal to 256 luma samples, a CU level flag is signalled to indicate whether OBMC is applied or not for the current CU. For the CUs with size larger than 256 luma samples or not coded with AMVP mode, OBMC is applied by default.

At the encoder, when OBMC is applied for a CU, its impact is taken into account during the motion estimation stage. The prediction signal formed by OBMC using motion information of the top neighbouring block and the left neighbouring block is used to compensate the top and left boundaries of the original signal of the current CU, and then the normal motion estimation process is applied.

Let wnand (1-wn) denote the weighting factors used for the neighbouring MV and the current MV respectively. The search template used for motion estimation can be obtained as

pt=((1+wn)∙po-wn∙pn)

where pt is the pixel value of template used for motion estimation, po is the original pixel value and pn is the pixel value compensated by the neighbouring MV.

Adaptive Updated Long-Term Reference

The long-term reference that is updated by ALTR is used as an additional reference frame for motion estimation. It should be pointed out that only lowdelay configuration use ALTR method. Therefore, ALTR doesn’t have any impact on randomaccess configuration in the proposed platform.

Usage of decoder-side motion refinement

Same as JEM7.0’s implementation.

Mode decisions

Same as JEM7.0’s implementation.

In-loop filtering type selection and parameter estimation

Switch on/off control and parameters estimation in the proposed NLSF tool are described as follows

Frame level on/off control

For one frame, there are two flags signaled in slice header, slice_nlsf_luma_flag and slice_nlsf_chroma_flag, determined according to the rate-distortion costs of luma and Cb component respectively w/o NLSF. If the luma component of current frame performs better with NLSF than without, switch on the slice_nlsf_luma_flag and then calculate the rate-diatortion costs of Cb component. If Cb component performs better, switch on the slice_nlsf_luma_flag. Otherwise, switch off it. If luma component of current frame performs worse, switch off the slice_nlsf_luma_flag and don’t need to calculate the rate-distortion costs of Cb component. And the slice_nlsf_chroma_flag will not be signalled.

CTU level on/off control

NLSF is CTU level loop filter tool. The on/off control of each CTU is determined according to the SSE (Sum of Square Error) of luma component between CTU before/after NLSF and its corresponding CTU in original video sequence, i.e. dfSSE and nlsfSSE. If dfSSE > nlsfSSE, switch on the CTU level on/off control. Otherwise, it is switched off.

Parameter ε estimation

As the similarity criterion of similarity evaluation is SSD, the parameter ε is related to the size of loop unit and the bit depth of video. In this proposal, parameter ε is determined as

ε=Bs2×(1≪(2×bitDepth))×λ,

where Bs denotes the size of loop unit, bitDepth represents the bit depth of input sequence, and λ is the parameter obtained according to the statistics of SSD between loop unit and the selected similar blocks.

Parameter τ estimation

Gaussian model is adopted to characterize the quantization noise between the original frame and the reconstructed frame. Here, the estimation of τ is converted to estimate the variance σ2 of the quantization noise. There exists a linear relationship between σ and the quantization step size Qstep, as

σ=a×Qstep+b,

where a and b are parameters trained offline. Qstep can be computed according to the QP as

Qstep= 2QP-46

Moreover, the relation between σ and τ can be expressed as

τ=σ×(Bs2+ c)

Transforms and transform type selection

Same as JEM7.0’s implementation.

Quantization and quantization type selection

Same as JEM7.0’s implementation.

Entropy coding

Multi-hypothesis probability estimation is used to update model based on two probability estimates P0 and P1 that are associated with each context model, which is the same as encoder part. Two probability estimates P0 and P1 are updated independently with different adaptation rates as follows:

where  and  (j=0, 1) represent the probabilities before and after decoding a bin, respectively. The parameters a and b are such that 2 ≤ a ≤ b ≤ 9 which are assigned for each context. Due to their limited range, a and b can be stored using only 4 bits each, and k represents the precision of probabilities (here it is equal to 15). The probability estimate P used for the interval subdivision in the binary arithmetic coder is the average of the estimates from the two hypotheses:

In JEM7.0, the value of the parameter a and b that control the probability updating speed for each context model, where a and b are set as 4 and 8 respectively. To preserve the computational complexity of the current method, the bit-shift values a and b are set only when the context probability is re-initialized (i.e., when probabilities are set to pre-defined values, depending on the quantization parameter QP) at each slice. Since shift parameters and probabilities are changed together during context re-initialization, they were jointly optimized to obtain the largest gains per context.

In the proposed design, instead of having different arrays of context initializers per B, P, and I slice type, this is replaced by the same arrays of initializers for all slice types, plus two pairs of shifts parameters (a, b) per context, with one pair used if QP < 30, and another if QP ≥ 30. With this setup the total amount of memory needed for context initialization is maintained as 3 bytes per context.

To further adjust the probability updating speed for each context adaptively, the bin counter c is assigned for each context as well, which represents the number of bins using the given context within each slice. The counter c is initialized as 0 at the beginning of each slice. Once the counter c is larger than a threshold Thr, the parameter a and b are adjusted as follows:

Where (a’, b’) is the current shifts parameters of the current context. Thr is the pre-defined threshold which can be selected from 64, 128, 256 and 512 adaptively based on the characteristic of each context.

Additional tools

ALTR (Adaptive Updated Long-Term Reference) determines the coding blocks that are used to update long-term reference at the beginning of current frame encoding. It compares current original frame with the last two encoded original frames CTU by CTU. For P slice and B slice, up to half of a coding frame’s CTUs can be used to update the long-term reference. The maximum number of CTU that is used to reference update is determined by a control parameter that is coded in the bitstream.

The metrics that are used to select long-term reference updated CTU are summarized below:

- Differences between the pixels of current block and co-located block at frameN-1, and differences between the pixels of current block and co-located block at frameN-2;

- Number of pixels whose difference is larger than thresholdA, and number of pixels whose difference is larger than thresholdB.

Additional encoder optimization

N/A

Additional algorithmic description topic(s)

N/A

360°

Usage of projection format adaptation

360° Video specific encoding tools

Additional encoder optimization

HDR

Pre-processing

HDR specific encoding tools

Additional encoder optimization

Compression performance

SDR: Constraint set 1 configuration relative to HM anchor

Class SDR-A (RD-curves)

Class SDR-B (RD-curves)

Overall (BD-rate summary tables)

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -33.54% | -43.75% | -46.69% | 1.67 | 1.26 | 1.47 |
| CatRobot1 | -40.10% | -53.03% | -46.28% | 1.57 | 0.97 | 1.31 |
| DaylightRoad2 | -41.15% | -56.22% | -41.43% | 1.15 | 1.19 | 0.76 |
| ParkRunning3 | -31.76% | -25.53% | -27.26% | 1.02 | 0.43 | 0.38 |
| Campfire | -35.19% | -35.78% | -52.07% | 1.03 | 1.35 | 1.62 |
| BQTerrace | -33.14% | -45.29% | -54.78% | 1.00 | 0.82 | 1.03 |
| RitualDance | -28.43% | -37.55% | -40.79% | 1.52 | 1.07 | 1.33 |
| MarketPlace | -29.97% | -47.44% | -44.95% | 1.05 | 0.83 | 0.82 |
| BasketballDrive | -32.69% | -47.86% | -46.46% | 1.18 | 1.07 | 1.43 |
| Cactus | -35.90% | -45.07% | -43.00% | 1.48 | 0.72 | 1.13 |
| Average SDR-A | -36.35% | -42.86% | -42.75% | 1.29 | 1.04 | 1.11 |
| Average SDR-B | -32.03% | -44.64% | -46.00% | 1.25 | 0.90 | 1.15 |
| Average all | -34.19% | -43.75% | -44.37% | 1.27 | 0.97 | 1.13 |
| Enc Time[%] | 765% |  |  |  |  |  |
| Dec Time[%] | 2911% |  |  |  |  |  |
| Max Delta Rate[%] | 0.00% |  |  |  |  |  |
| Min Delta Rate[%] | -0.48% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 2.03 |  | 1.51 |  | 1.87 |  |
| Min Delta PNSR (YUV) | 0.73 |  | 0.42 |  | 0.37 |  |

SDR: Constraint set 2 configuration relative to HM anchor

Class SDR-B

Overall

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -28.58% | -45.59% | -52.38% | 0.86 | 0.53 | 0.62 |
| RitualDance | -22.76% | -33.59% | -35.76% | 1.17 | 0.80 | 0.99 |
| MarketPlace | -21.83% | -43.73% | -42.13% | 0.70 | 0.58 | 0.61 |
| BasketballDrive | -27.47% | -44.89% | -45.76% | 1.04 | 0.96 | 1.37 |
| Cactus | -33.68% | -46.99% | -46.61% | 1.39 | 0.69 | 1.09 |
| Average SDR-B | -26.87% | -42.96% | -44.53% | 1.03 | 0.71 | 0.94 |
| Enc Time[%] | 917% |  |  |  |  |  |
| Dec Time[%] | 3675% |  |  |  |  |  |
| Max Delta Rate | 0.00% |  |  |  |  |  |
| Min Delta Rate | -0.16% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 1.47 |  | 1.02 |  | 1.46 |  |
| Min Delta PNSR (YUV) | 0.61 |  | 0.49 |  | 0.54 |  |

SDR: Constraint set 1 configuration relative to JEM anchor

Class SDR-A

Class SDR-B

Overall

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -0.32% | -0.23% | 0.11% | 0.01 | 0.00 | 0.00 |
| CatRobot1 | -1.36% | -2.04% | -1.25% | 0.04 | 0.02 | 0.02 |
| DaylightRoad2 | -1.39% | 2.86% | -0.34% | 0.03 | -0.03 | 0.00 |
| ParkRunning3 | -0.94% | -0.51% | 0.23% | 0.03 | 0.01 | 0.00 |
| Campfire | -1.29% | -1.75% | -5.13% | 0.03 | 0.06 | 0.10 |
| BQTerrace | -4.00% | -0.17% | -0.41% | 0.09 | 0.00 | 0.00 |
| RitualDance | -1.56% | -0.80% | -0.36% | 0.07 | 0.02 | 0.01 |
| MarketPlace | -1.12% | -0.24% | -0.95% | 0.03 | 0.00 | 0.01 |
| BasketballDrive | -2.28% | -4.59% | -5.51% | 0.06 | 0.07 | 0.12 |
| Cactus | -1.45% | 0.34% | -3.54% | 0.04 | 0.00 | 0.06 |
| Average SDR-A | -1.06% | -0.33% | -1.28% | 0.03 | 0.01 | 0.02 |
| Average SDR-B | -2.08% | -1.09% | -2.15% | 0.06 | 0.02 | 0.04 |
| Average all | -1.57% | -0.71% | -1.72% | 0.04 | 0.01 | 0.03 |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 381% |  |  |  |  |  |
| Max Delta Rate[%] |  |  |  |  |  |  |
| Min Delta Rate[%] |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.13 |  | 0.07 |  | 0.13 |  |
| Min Delta PNSR (YUV) | -0.05 |  | -0.08 |  | -0.08 |  |

SDR: Constraint set 2 configuration relative to JEM anchor

Class SDR-B

Overall

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -3.32% | 4.23% | -2.13% | 0.08 | -0.03 | 0.01 |
| RitualDance | -1.36% | 1.23% | -0.47% | 0.06 | -0.02 | 0.01 |
| MarketPlace | -1.44% | -1.96% | -2.50% | 0.04 | 0.02 | 0.03 |
| BasketballDrive | -2.54% | 1.35% | -3.59% | 0.08 | -0.02 | 0.08 |
| Cactus | -7.83% | -8.19% | -12.61% | 0.26 | 0.08 | 0.21 |
| Average SDR-B | -3.30% | -0.67% | -4.26% | 0.10 | 0.01 | 0.07 |
| Enc Time[%] | 113% |  |  |  |  |  |
| Dec Time[%] | 631% |  |  |  |  |  |
| Max Delta Rate |  |  |  |  |  |  |
| Min Delta Rate |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.32 |  | 0.10 |  | 0.26 |  |
| Min Delta PNSR (YUV) | 0.03 |  | -0.05 |  | -0.01 |  |

HDR: Constraint set 1 configuration relative to HM anchor

Class HDR-A

Class HDR-B

Overall

HDR: Constraint set 1 configuration relative to JEM anchor

Class HDR-A

Class HDR-B

Overall

360° Video: Constraint set 1 configuration relative to HM anchor

Class 360

Overall

360° Video: Constraint set 1 configuration relative to JEM anchor

Class 360

Overall

Decoder complexity analysis

Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

The decoding time of J0011 platform and HM16.16 is measured by the decoding time statistics function of the software. The decoding time for both software includes the output of reconstructed yuv file. The decoding time comparison between J0011 and HM16.16 is summarized below. The decoding times for R1~R4 for a sequence are calculated on average.

Table 5.1-1: decoding time comparison between J0011 and HM16.16

|  | Sequences | Dec Time Ratio (J0011/HM) |
| --- | --- | --- |
| C1 SDR-A | FoodMarket4 | 2559.04% |
|  | CatRobot1 | 2879.93% |
|  | DaylightRoad2 | 2963.22% |
|  | ParkRunning3 | 3273.63% |
|  | CampfireParty2 | 4622.70% |
|  | Average | 3259.71% |
| C1 SDR-B | BQTerrace | 2196.24% |
|  | RitualDance | 3044.98% |
|  | MarketPlace | 2352.56% |
|  | BasketballDrive | 3364.46% |
|  | Cactus | 2490.20% |
|  | Average | 2689.69% |
| C2 SDR-B | BQTerrace | 2977.26% |
|  | RitualDance | 3883.54% |
|  | MarketPlace | 3315.98% |
|  | BasketballDrive | 4549.31% |
|  | Cactus | 3797.71% |
|  | Average | 3704.76% |

Description of computing platform used to determine decoding times reported in section 5.1

The configurations of computing platform used to determine decoding times are summarized as below:

Executable: 64bit file

Clock speeds: 3.00GHz

Use of multicore parallel processing: 23

Memory: 256GB

Hard drive: HDD with 10000RPM

Memory usage of decoder

The comparison of decoder memory usage between J0011 platform, HM16.16 and JEM7.0 is summarized below:

|  | RitualDance_1920x1080_60fps_10bit @ R4 |  |  | CatRobot1_3840x2160_60fps_10bit @ R4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | J0011 | HM16.16 | JEM7.0 | J0011 | HM16.16 | JEM7.0 |
| CS1 | 0.44GB | 0.95GB | 0.34GB | N/A | N/A | N/A |
| CS2 | 0.47GB | 1.01GB | 0.41GB | 1.57GB | 1.96GB | 1.21GB |

Complexity characteristics of decoder entropy decoding operation

Based on JEM7.0, one additional parameter counter c is required for each context in our proposal, and c adds one after encoding one bin when c is smaller than Thr. Therefore, one addition operation and one judgment operation is added for encoding one bin before c reached Thr.

Complexity characteristics of decoder inverse quantization

Same as JEM7.0’s implementation.

Complexity characteristics of decoder inverse transform operation

Same as JEM7.0’s implementation.

Complexity characteristics of decoder-side motion refinement

Same as JEM7.0’s implementation.

Complexity characteristics of decoder motion compensation

The original weighting factors in OBMC are {1/4, 1/8, 1/16, 1/32}, which can be processed by the right shift operator >>. In this proposal, the weighting factors are modified and the right shift operation is demanded to be substituted by division operation. For a motion compensation block, each boundary pixel will be processed twice by OBMC. Let w and h denote the width and height of current CU. The proposal will add 8×(w+h) division operations for a CU compared to the OBMC in JEM.

Complexity characteristics of decoder intra-frame prediction operation

Same as JEM7.0’s implementation.

Complexity characteristics of decoder in-loop filtering operation

Block matching and SVD take the majority of the running time of NLSF. The concrete complexity characteristics of block matching and SVD are as follows.

Complexity characteristics of block matching sub-module

The block matching sub-module takes up about 30%~55% running time of NLSF. The percentage of running time for block matching is more when QP is bigger.

During the block matching process, the most time-consuming part is the calculation of SSD between candidate blocks and the current block. In total, Bs subtraction operations and Bs multiplications are required for each SSD computation, and the number of SSD computation depends on the searching points during the matching process. All calculations in block matching sub-module are integer operations and all variables in this process are 32-bit integers.

Complexity characteristics of SVD sub-module

The SVD sub-module takes up about 35%~60% time of NLSF. The percentage of SVD time decreases as QP increases. During the SVD process, the most time consuming part is the calculation of singular value and matrix UGk and VGk. The maximum number of iterations to obtain a singular value is ten. The maximum number of iterations of the whole SVD process is m (the number of similar blocks selected in block matching process). There are some decimal operations in SVD, including the calculation of singular value and matrix UGk and VGk. Most of the variables during SVD are double type.

Complexity characteristics of additional decoder tools

When ALTR (Adaptive Update Long-Term Reference) is equal to 1, an additional frame buffer is required for long-term reference. The update of the long-term reference is performed after the reconstruction process of a slice. If the slice is an IDR slice, the entire reconstructed pixels of the IDR slice are copied to the long-term reference. If the slice is a P slice or a B slice, the reconstructed pixels of the CTUs with block_lt_update flag equal to 1 are copied to the co-located block in long-term reference.

Bandwidth analysis: an additional reference frame reading is required for a P slice or a B slice decoding. To update the long-term reference, at most a half frame writing is required for a P slice or a B slice, and a whole frame writing is required for an I slice.

Complexity characteristics of 360° video specific decoding tools

Complexity characteristics of HDR specific decoding tools

Degree of capability for decoder parallel processing

Encoder complexity analysis

Encoding time and measurement methodology

The encoding time of J0011 platform and HM16.16 is measured by the decoding time statistics function of the software. The encoding time for both software includes the output of reconstructed yuv file. Regarding J0011 platform, parallel encoding is used. The measurement of J0011 platform’s encoding time follows the instruction of section 6.1 at the document “JVET-I1003”.

The encoding time comparison between J0011 and HM16.16 is summarized below. The encoding times for R1~R4 for a sequence are calculated on average.

Table 6.1-1: decoding time comparison between J0011 and HM16.16

|  | Sequences | Enc Time Ratio (J0011/HM) |
| --- | --- | --- |
| C1 SDR-A | FoodMarket4 | 578.11% |
|  | CatRobot1 | 692.14% |
|  | DaylightRoad2 | 729.92% |
|  | ParkRunning3 | 776.26% |
|  | CampfireParty2 | 1212.75% |
|  | Average | 797.84% |
| C1 SDR-B | BQTerrace | 549.13% |
|  | RitualDance | 1027.34% |
|  | MarketPlace | 715.07% |
|  | BasketballDrive | 895.05% |
|  | Cactus | 764.35% |
|  | Average | 790.19% |
| C2 SDR-B | BQTerrace | 543.22% |
|  | RitualDance | 1303.33% |
|  | MarketPlace | 852.92% |
|  | BasketballDrive | 1171.51% |
|  | Cactus | 958.55% |
|  | Average | 965.91% |

Description of computing platform used to determine encoding times reported in section 6.1

The configurations of computing platform used to determine decoding times are summarized as below:

Executable: 64bit file

Clock speeds: 3.00GHz

Use of multicore parallel processing: 23

Memory: 256GB

Hard drive: HDD with 10000RPM

Memory usage of encoder

(Proponents are requested to provide quantitative measurement (e.g. in units of GB) of memory usage of their proposed encoder implementation. Proponents are also encouraged to compare their proposal’s encoder memory usage with that of the HM and/or JEM anchors.

For each constraint set, encoder memory usage measurement only needs to be reported for the sequence class of the largest resolution at the highest target rate point.)

The comparison of decoder memory usage between J0011 platform, HM16.16 and JEM7.0 is summarized below:

|  | RitualDance_1920x1080 _60fps_10bit @ R4 |  | CatRobot1_3840x2160 _60fps_10bit @R4 |  |
| --- | --- | --- | --- | --- |
|  | J0011 | HM16.16 | J0011 | HM16.16 |
| CS1 | 1.75GB | 0.52GB | 5.89GB | 1.87GB |
| CS2 | 0.86GB | 0.23GB | N/A | N/A |

Complexity characteristics of encoder intra-frame prediction type selection

Same as JEM7.0’s implementation.

Complexity characteristics of encoder motion estimation and motion segmentation selection

General

OBMC improvement has the impact on motion estimation. The original weighting factors in OBMC are {1/4, 1/8, 1/16, 1/32}, which can be processed by the right shift operator >>. In this proposal, the weighting factors are modified and the right shift operation is demanded to be substituted by division operation. For a motion estimation block, each boundary pixel will be processed twice by OBMC. Let w and h denote the width and height of current CU. The proposal will add 8×(w+h) division operations for a CU compared to the OBMC in JEM.

Complexity characteristics of usage of decoder-side motion refinement

Same as JEM7.0’s implementation.

Complexity characteristics of encoder transforms and transform type selection

Same as JEM7.0’s implementation.

Complexity characteristics of encoder quantization and quantization type selection

Same as JEM7.0’s implementation.

Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

The encoder sider of NLSF is the same as decoder except for the Rate Distortion Optimization (RDO) process. Thus, the complexity characteristics of encoder can refer to section 5.10.

Complexity characteristics of encoder entropy coding type selection

Based on JEM7.0, one additional parameter counter c is needed for each context in our proposal, and c increases one after encoding one bin before the value of c is smaller than Thr. Therefore, there are one additional addition operation and one judgment operation when one bin is encoded before c reached Thr.

Complexity characteristics of additional encoder tools

At the encoder side, three additional frame buffers are required to perform ALTR (Adaptive Update Long-Term Reference). One is used for buffering the long-term reference, and another two frames are used for buffering previous two original frames. A pre-processing is performed before each slice encoding, which is described at the section 3.1.4.1, to determine the CTUs that are used to update the long-term reference. The update process for the long-term reference is performed after the reconstruction process of a slice. If the slice is an I slice, the whole reconstructed slice are copied to the long-term reference. If the slice is a P slice or a B slice, the reconstructed pixels of the CTUs with block_lt_update flag equal to 1 are copied to the co-located pixels in long-term reference.

Bandwidth analysis: an additional reference frame reading is required for a P slice or a B slice decoding. To update the long-term reference, at most a half frame writing is required for a P slice or a B slice, and a whole frame writing is required for an I slice.

Complexity characteristics of 360° video specific encoding tools

Complexity characteristics of HDR specific encoding tools

Degree of capability for encoder parallel processing

Algorithmic characteristics

Random access characteristics

The implementation of J0011 platform is based on JEM7.0, and doesn’t add new feature that impacts on JEM7.0’s random access characteristics.

Delay characteristics

The implementation of J0011 platform is based on JEM7.0, and doesn’t add new feature that impacts on JEM7.0’s lowdelay characteristics.

Additional characteristics discussion topic(s)

N/A

Software implementation description

The implementation of J0011 platform is based on JEM7.0. The implementations of new coding tools follow the instruction of the document JCTVC-F688 “HEVC software guidelines”.

Highlighted aspects discussion

This response presents four new coding tools based on JEM7.0 platform. Besides the integration of the proposed tools, the response’s platform is exactly same as JEM7.0. Compared to JEM7.0 encoding time, the response shows relative low complexity for encoder implementation. Regarding SDR constraint set 1, it shows -1.57%/-0.71/-1.72% coding gain for Y/U/V components with no encoding time increase. For SDR constraint set 2, it shows -3.3%/-0.67%/-4.26% coding gain for Y/U/V components with about 12% encoding time increase.

The response has non-negligible decoding time increase compared to JEM7.0. The major increase of decoding time comes from the implementation of NLSF. It can be further simplified by optimization, the use of fast search algorithm and integer SVD method.

Closing remarks

This proposal presents DJI and Peking University’s response to the joint CfP on video compression with capability beyond HEVC by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC1/SC29/WG11 (MPEG). Four coding tools are proposed and show substantial coding quality improvement. The response uses JEM software as the experimental platform. The proposed tools can also be implemented on and extended to other video coding software easily and flexibly.

Patent rights declaration(s)

SZ DJI Technology Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex: Questionnaire

Proponents of CfP responses are required to answer the codec level questions, and are also encouraged to fill out the optional tool level questions in addition to the complexity analysis of their proposals. If some question is not applicable to your proposal, you may express that as well.

Codec level questions

- Maximum number of reference frames used by encoder

Random access: 4 short-term references.

Lowdelay: 4 short-term references + 1 long-term reference.

- Maximum coding unit (e.g. CTU in HEVC)

Same as JEM7.0.

- Minimum and maximum transform block size

Same as JEM7.0.

- Minimum and maximum intra prediction block

Same as JEM7.0.

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

Same as JEM7.0.

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

Based on JEM7.0, additional tools are: NLSF and the improvement of OBMC.

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

Based on JEM7.0, additional tools are: NLSF.

- List all tools that use non-rectangular (e.g. triangle) partitions

N/A

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

No.

- Indicate whether luma and chroma use separate partitioning structures

Same as JEM7.0.

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

Same as JEM7.0.

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

Same as JEM7.0.

Tool level questions

- Partitioning structure

  - Whether the prediction unit partitioning is different from transform unit partitioning: Same as JEM7.0.

  - Maximum tree depth: Same as JEM7.0.

- Entropy coding

  - Total number of context models: Based on JEM7.0, 1 additional context models is added.

  - Memory storage size for each context model: Same as JEM7.0.

  - Range table size if table look up is used for range update: Same as JEM7.0.

  - Initialization table size: Same as JEM7.0.

  - Other memory associated with context model adaptation: Same as JEM7.0.

- Control data derivation process

  - Temporal motion data storage granularity: Same as JEM7.0.

  - Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-predictive reference picture are used, count this picture as 2) : Same as JEM7.0 for random access and 5 reference pictures for lowdelay.

- Decoder-side motion refinement

  - Line buffer size in terms of motion vector block lines (assuming horizontal motion vector block size is 4 luma samples) : Same as JEM7.0.

  - Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement: Same as JEM7.0.

  - Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size: Same as JEM7.0.

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines: Same as JEM7.0.

  - List all the filters used for intra prediction: Same as JEM7.0.

  - Indicate whether there are cross-component dependencies in intra prediction reconstruction: Same as JEM7.0.

- Inter prediction

  - Motion vector accuracy: Same as JEM7.0.

  - Storage size for a motion vector component: Same as JEM7.0.

  - List all the interpolation filters used for luma/chroma motion compensation: Same as JEM7.0.

- Quantization

  - Storage size associated with quantization: Same as JEM7.0.

- Transforms

- List all the combinations of transform block size and transform type for primary transforms: Same as JEM7.0.

- List all the combinations of transform block size and transform type for secondary transforms: Same as JEM7.0.

- List all the non-separable transforms used: Same as JEM7.0.

- Indicate whether transforms can be fully/partially implemented using direct matrix multiply: Same as JEM7.0.

- List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) : Same as JEM7.0.

- In-loop filtering

- List line buffer size in terms of luma and chroma lines (if any): Based on JEM7.0, 16 additional line buffer size for luma and 16 additional line buffer size for chroma are added.

- List all the filters used for in-loop filtering: Based on JEM7.0, a new in-loop filter NLSF is added.

- Minimum processing unit size: Same as JEM7.0.

- “tool-off” and “tool-on” tests

The summary of the tools that bring major objective coding gain in this response is provided at the below tables.

Table 12.2-1: Summary of “tool-off” and “tool-on” tests for J0011vs. JEM7.0 (SDR constrain set 1)

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| NLSF | -1.2% | -0.4% | -1.3% | 101% | 397% | -1.2% | -0.4% | -1.4% | 101% | 401% |
| ALTR | - | - | - | - | - | - | - | - | - | - |
| Entropy improve | -0.3% | -0.2% | -0.4% | 99% | 100% | -0.3% | -0.2% | -0.5% | 99% | 101% |

Table 12.2-2: Summary of “tool-off” and “tool-on” tests for J0011vs. JEM7.0 (SDR constrain set 2)

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| NLSF | -1.9% | 0.4% | -3.2% | 103% | 607% | -2.0% | -0.3% | -2.0% | 99% | 595% |
| ALTR | -0.8% | -1.7% | -2.0% | 112% | 99% | -0.9% | -1.5% | -0.9% | 108% | 97% |
| Entropy improve | -0.3% | 0.3% | -0.1% | 100% | 99% | -0.3% | 0.5% | 0.2% | 100% | 99% |