| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0016 |
| --- | --- |

| Title: | Description of SDR video coding technology proposal by KDDI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-5066-9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents description of SDR video coding technology proposal by KDDI. The proposed coding technology is based on the top-of-JEM software with six addition coding tools.

The six tools are cross-component reference prediction, adaptive inter-residual prediction of chroma components, shrink transform, block-size dependent coefficient scanning, extended deblocking filter, and convolutional neural network based in-loop filtering. The two intra prediction focuses on the performance improvement of chroma component. One transform tool realize large transform block by half-transform and up sampling. Coefficient scanning is also modified based on both intra prediction mode and ratio of block shape. Two in-loop filter improve the subjective quality.

The proposed coding technology provides -33.50% and -24.64% BD-rate gain for constraint set 1 and 2, respectively, compared with HM16.6. It also provides -0.47% and -0.17% BD-rate gain for CS1 and CS2, respectively, compared with JEM7.0. It is noted that two in-loop filtering provide no objective gain.

Running times of both encoding and decoding of CS1 are 8.5 times and 18.6 times, respectively. All tool except CNN-based in-loop filtering requires less additional running time. Although the load of CNN-based in-loop filtering might be heavy in CPU implementation, it is moderate for GPU or dedicated implementation.

- Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.1.1	Bitstream structure	1

2.1.2	Entropy decoding	1

2.1.3	Conversion process from entropy decoded data to symbols	1

2.1.4	Decoder-side motion refinement	1

2.1.5	Inverse quantization	2

2.1.6	Inverse transforms	2

2.1.7	Motion compensation	2

2.1.8	Intra prediction	2

2.1.9	In-loop filtering	2

2.1.10	Additional tools	3

2.1.11	Additional algorithmic description topic(s)	3

3	Encoder algorithm description	3

3.1	General algorithm description	3

3.1.1	Rate-distortion optimization	3

3.1.2	Partitioning structure selection	3

3.1.3	Intra prediction mode estimation and type selection	4

3.1.4	Motion estimation and motion segment selection	4

3.1.5	Mode decisions	4

3.1.6	In-loop filtering type selection and parameter estimation	4

3.1.7	Transforms and transform type selection	4

3.1.8	Quantization and quantization type selection	4

3.1.9	Entropy coding	4

3.1.10	Additional tools	4

3.1.11	Additional encoder optimization	4

3.1.12	Additional algorithmic description topic(s)	5

4	Compression performance	5

4.1	SDR: Constraint set 1 configuration relative to HM anchor	5

4.1.1	Class SDR-A (RD-curves)	5

4.1.2	Class SDR-B (RD-curves)	7

4.1.3	Overall (BD-rate summary tables)	8

4.2	SDR: Constraint set 2 configuration relative to HM anchor	9

4.2.1	Class SDR-B	9

4.2.2	Overall	11

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	11

4.3.1	Class SDR-A	11

4.3.2	Class SDR-B	13

4.3.3	Overall	15

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	16

4.4.1	Class SDR-B	16

4.4.2	Overall	18

5	Decoder complexity analysis	18

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	18

5.2	Description of computing platform used to determine decoding times reported in section 5.1	18

5.3	Memory usage of decoder	18

5.4	Complexity characteristics of decoder entropy decoding operation	18

5.5	Complexity characteristics of decoder inverse quantization	19

5.6	Complexity characteristics of decoder inverse transform operation	19

5.7	Complexity characteristics of decoder-side motion refinement	19

5.8	Complexity characteristics of decoder motion compensation	19

5.9	Complexity characteristics of decoder intra-frame prediction operation	19

5.10	Complexity characteristics of decoder in-loop filtering operation	19

5.11	Complexity characteristics of additional decoder tools	19

5.12	Complexity characteristics of 360° video specific decoding tools	19

5.13	Complexity characteristics of HDR specific decoding tools	19

5.14	Degree of capability for decoder parallel processing	19

6	Encoder complexity analysis	19

6.1	Encoding time and measurement methodology	19

6.2	Description of computing platform used to determine encoding times reported in section 6.1	20

6.3	Memory usage of encoder	20

6.4	Complexity characteristics of encoder intra-frame prediction type selection	20

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	20

6.5.1	General	20

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	20

6.6	Complexity characteristics of encoder transforms and transform type selection	20

6.7	Complexity characteristics of encoder quantization and quantization type selection	20

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	20

6.9	Complexity characteristics of encoder entropy coding type selection	20

6.10	Complexity characteristics of additional encoder tools	20

6.11	Complexity characteristics of 360° video specific encoding tools	21

6.12	Complexity characteristics of HDR specific encoding tools	21

6.13	Degree of capability for encoder parallel processing	21

7	Algorithmic characteristics	21

7.1	Random access characteristics	21

7.2	Delay characteristics	21

7.3	Additional characteristics discussion topic(s)	21

8	Software implementation description	21

9	Highlighted aspects discussion	21

10	Closing remarks	21

11	Patent rights declaration(s)	21

Introduction

This contribution presents description of SDR video coding technology proposal. The proposed coding technology is based on the JEM 7.0 software with six addition coding tools.

Decoder algorithm description

General algorithm description

Bitstream structure

Same as in JEM. When the stream is generated parallel, POC signalling is now broken. The proposed decoder ignore such POC signal violation.

Entropy decoding

Same as in JEM.

Conversion process from entropy decoded data to symbols

Intra prediction parameters

For the cross-component reference prediction, an additional chroma-intra-prediction mode is inserted.

For the adaptive inter-residual prediction of chroma component, an additional flag is inserted.

Motion parameters

Same as in JEM.

Quantization parameters

Same as in JEM.

Transform coefficients

Shrink transform

When the transform block size is equal or larger than 64, the number of coefficients is always half to that in JEM. Otherwise (the transform block size is less than 64), decode of transform coefficients is the same as in JEM.

Coefficient scanning

Coefficient scanning is modified based on the ratio of block size. When the ratio of horizontal and vertical size is equal or larger than four, mode-dependent coefficient scanning is applied. The MDCS itself is the same as in JEM.

JEM and the proposed decision maps are shown in the following figure.

JEM decision map	The proposed decision map

In-loop filter parameters

Same as in JEM.

Decoder-side motion refinement

Same as in JEM.

Inverse quantization

Same as in JEM.

Inverse transforms

A family of inverse transforms are the same as in JEM.

When the transform block size is equal or larger than 64, inverse transform is realized by two steps; inverse transform with half size and up-sampling. The up-sampling ratio is twice with DCT based coefficient, which is the same as in SHVC. Otherwise (the transform block size is less than 64), inverse transform is the same as in JEM.

Motion compensation

Same as in JEM.

Intra prediction

Two chroma intra prediction tools are proposed. One is the cross-component reference prediction and another is the adaptive residual prediction

Inter-color reference prediction

This method is stated as an additional chroma-intra-prediction mode. The reference samples of chroma component is replaced by that of luma component with a linear model. The linear model is borrowed from the existing CCLM tool in JEM. After the refinement of the reference samples, the existing chroma intra-prediction is performed. The prediction mode of chroma is borrowed from the mode of luma.

Adaptive inter-residual prediction

CCLM Cb-to-Cr prediction exists in JEM. The adaptive inter-residual prediction introduces CCLM Cr-to-Cb prediction. Reference component is signaled by an additional flag. The remaining parts like prediction and scaling factor derivation are the same as in JEM.

In-loop filtering

One modified deblocking filter and one new in-loop filtering are proposed. The deblocking filter is exteneded to the large block size. The new in-loop filtering is based on the convolutional neural network.

Extended deblocking filter

This tool is an extension of both deblocking filter decision and process for large block boundary. The extended deblocking filter consists of two parts; edge boundary detection of large intra block and extension of the filtered samples on edge filtering process.

When the following conditions are true, the proposed decision process is continued:

- Edge boundary is located between two blocks of which cross edge sizes are 16 or longer than 16.

- The filter type is strong.

It is noted that a block size is replaced by an edge size. Direction of edge size depends on that of edge boundary like EDGE_VER or EDGE_HOR.

When the above criteria is true, coding mode of blocks are checked. When both blocks are not intra coded, no deblocking filter is applied while the proposed extended deblocking filter is applied for other blocks.

The decision of filter type is extended. JEM derive an edge magnitude from p0, p1, and p2, while the proposed method derive it from p0 to p7.

The JEM filter coefficients of p3-p0 and q0-q3 for p0, p1, and p2 pixels are as follows.

p2 = 2,3,1,1,1,0,0,0
p1 = 0,2,2,2,2,2,0,0
p0 = 0,1,2,2,2,1,0,0

The proposed filter coefficients of p7-p0, and q0-q7 for p0-p6 pixels are as follows.

p6 = 3,5,2,1,1,1,1,1,1,0,0,0,0,0,0,0
p5 = 1,2,5,2,1,1,1,1,1,1,0,0,0,0,0,0
p4 = 0,1,3,3,3,1,1,1,1,1,1,0,0,0,0,0
p3 = 0,1,2,2,2,2,2,1,1,1,1,1,0,0,0,0
p2 = 0,0,1,2,2,2,2,2,2,1,1,1,0,0,0,0
p1 = 0,0,1,1,2,2,2,2,2,2,1,1,0,0,0,0
p0 = 0,0,0,1,2,2,2,2,2,2,2,1,0,0,0,0

As a summary, JEM and the proposed method apply three pixels and seven pixels for each side, respectively. The number of filter taps are also extended from five taps to 11 taps. The coefficients are decided based on the JEM.

Convolutional neural network based filter

This tool is one of a typical convolutional neural network based filter.

The filter structure is shown in the following figure. This filter has four layers with 3x3 taps. The input of sum block is residual signal from the left and reconstructed signal before all in-loop filters from the bottom. The output of sum block is filtered pixels. Actual output is weighted sum of after filtered and before filtered pixels based on the distance of edge.

Filter coefficients are common in both the encoder and the decoder so that additional side information is not required.

Due to the encoder/decoder complexity issue, the CNN-based in-loop filtering is applied only for the intra slice.

Additional tools

No additional tool is proposed.

Additional algorithmic description topic(s)

No additional topic is proposed.

Encoder algorithm description

General algorithm description

Rate-distortion optimization

Distortion metrics

Same as in JEM.

Rate-distortion dependency models

Same as in JEM.

Partitioning structure selection

Same as in JEM.

Intra prediction mode estimation and type selection

For the luma intra coding, encoder algorithm is the same as in JEM. For the chroma intra coding, following adaptations are introduced to consider the proposed two tools; cross-component reference prediction and CCLM Cb-to-Cr/Cr-to-Cb.

Regarding the CCRP, one chroma intra prediction mode is added. One additional RDO is performed in the mode finding loop.

Regarding the CCLM Cb-to-Cr/Cr-to-Cb, additional flag is required to select the reference sample. For a fast encoding, a reduction of the number of RD cost check is proposed. The key idea is to decide the reference chroma component by only one time RD cost check. To realize the idea, the decision is based on the RD cost of DM_Chroma since a probability of DM_Chroma is highest. At the same time, DM_Chroma is moved to the first position in chroma RD-cost-check order. It is noted that a signal processing based decision without RD cost check does not work well in our experiments.

Motion estimation and motion segment selection

General

Same as in JEM.

Usage of decoder-side motion refinement

Same as in JEM.

Mode decisions

Same as in JEM.

In-loop filtering type selection and parameter estimation

Same as in JEM.

It is noted that CNN-based in-loop filter is learned in advance to that encoding. The test set is different sequences are used.

Transforms and transform type selection

For the large block, shrink transform is introduced. In the current JEM configuration, 128x128 transform is enabled in the RDO loop. As we mention the section 2.1.6, inverse transform is performed as 64x64 with zero padding at the high frequency. To realize such the inverse transform, residual signal is firstly shrink to half size, a 64x64 transform is secondly applied, and finally some coefficients in high frequency are removed. i.e. such positions are set as zero.

For other blocks, transform algorithm is the same as in JEM.

Quantization and quantization type selection

Same as in JEM.

Entropy coding

Same as in JEM.

Additional tools

No additional tool is proposed.

Additional encoder optimization

No additional encoder optimization is proposed.

Additional algorithmic description topic(s)

No additional topic is proposed.

Compression performance

SDR: Constraint set 1 configuration relative to HM anchor

Class SDR-A (RD-curves)

FoodMarket4, CS1

CatRobot1, CS1

DaylightRoad2, CS1

ParkRunning, CS1

Campfire, CS1

Class SDR-B (RD-curves)

BQTerrace, CS1

RitualDance, CS1

MarketPlace, CS1

BasketballlDrive, CS1

Cactus, CS1

Overall (BD-rate summary tables)

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -33.65% | -43.17% | -46.82% | 1.67 | 1.25 | 1.47 |
| CatRobot1 | -39.82% | -53.91% | -46.83% | 1.56 | 0.99 | 1.33 |
| DaylightRoad2 | -40.81% | -56.74% | -42.49% | 1.14 | 1.21 | 0.79 |
| ParkRunning3 | -31.13% | -25.87% | -28.72% | 0.99 | 0.44 | 0.41 |
| Campfire | -34.53% | -34.50% | -50.20% | 1.01 | 1.28 | 1.54 |
| BQTerrace | -31.54% | -45.19% | -54.82% | 0.95 | 0.82 | 1.03 |
| RitualDance | -27.32% | -37.73% | -41.41% | 1.45 | 1.08 | 1.36 |
| MarketPlace | -29.39% | -46.49% | -43.07% | 1.03 | 0.81 | 0.78 |
| BasketballDrive | -31.23% | -46.50% | -44.86% | 1.12 | 1.03 | 1.36 |
| Cactus | -35.65% | -45.63% | -42.57% | 1.47 | 0.73 | 1.11 |
| Average SDR-A | -35.99% | -42.84% | -43.01% | 1.28 | 1.03 | 1.11 |
| Average SDR-B | -31.02% | -44.31% | -45.35% | 1.20 | 0.89 | 1.13 |
| Average all | -33.50% | -43.57% | -44.18% | 1.24 | 0.96 | 1.12 |
| Enc Time[%] | 858% |  |  |  |  |  |
| Dec Time[%] | 15868% |  |  |  |  |  |
| Max Delta Rate[%] | 0.00% |  |  |  |  |  |
| Min Delta Rate[%] | -1.11% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 2.04 |  | 1.49 |  | 1.82 |  |
| Min Delta PNSR (YUV) | 0.68 |  | 0.43 |  | 0.40 |  |

SDR: Constraint set 2 configuration relative to HM anchor

Class SDR-B

BQTerrace, CS2

RitualDance, CS2

MarketPlace, CS2

BasketballDrive, CS2

Cactus, CS2

Overall

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over HM16.16 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -26.68% | -46.84% | -51.63% | 0.79 | 0.55 | 0.62 |
| RitualDance | -21.71% | -34.36% | -35.32% | 1.11 | 0.83 | 0.97 |
| MarketPlace | -20.57% | -42.55% | -40.99% | 0.65 | 0.55 | 0.58 |
| BasketballDrive | -25.56% | -46.12% | -44.04% | 0.96 | 0.99 | 1.30 |
| Cactus | -28.68% | -42.22% | -39.71% | 1.15 | 0.60 | 0.89 |
| Average SDR-B | -24.64% | -42.42% | -42.34% | 0.93 | 0.70 | 0.87 |
| Enc Time[%] | 895% |  |  |  |  |  |
| Dec Time[%] | 2121% |  |  |  |  |  |
| Max Delta Rate | -0.01% |  |  |  |  |  |
| Min Delta Rate | -0.15% |  |  |  |  |  |
| Max Delta PSNR (YUV) | 1.26 |  | 1.08 |  | 1.41 |  |
| Min Delta PNSR (YUV) | 0.55 |  | 0.48 |  | 0.49 |  |

SDR: Constraint set 1 configuration relative to JEM anchor

Class SDR-A

FoodMarket4, CS1

CatRobot1, CS1

DaylightRoad2, CS1

ParkRunning, CS1

Campfire, CS1

Class SDR-B

BQTerrace, CS1

RitualDance, CS1

MarketPlace, CS1

BasketballlDrive, CS1

Cactus, CS1

Overall

|  | Constraint Set 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| FoodMarket4 | -0.51% | 0.84% | -0.08% | 0.02 | -0.02 | 0.00 |
| CatRobot1 | -0.92% | -4.15% | -2.38% | 0.02 | 0.04 | 0.04 |
| DaylightRoad2 | -0.88% | 1.01% | -2.51% | 0.02 | -0.01 | 0.03 |
| ParkRunning3 | 0.00% | -0.98% | -1.86% | 0.00 | 0.01 | 0.02 |
| Campfire | -0.26% | 0.15% | -1.35% | 0.01 | 0.00 | 0.03 |
| BQTerrace | -1.73% | 0.47% | -0.23% | 0.04 | -0.01 | 0.00 |
| RitualDance | 0.01% | -1.21% | -1.50% | 0.00 | 0.02 | 0.04 |
| MarketPlace | -0.27% | 1.47% | 2.30% | 0.01 | -0.02 | -0.03 |
| BasketballDrive | -0.08% | -2.00% | -2.65% | 0.00 | 0.03 | 0.06 |
| Cactus | -1.05% | -0.78% | -2.71% | 0.03 | 0.01 | 0.05 |
| Average SDR-A | -0.51% | -0.63% | -1.64% | 0.01 | 0.00 | 0.03 |
| Average SDR-B | -0.62% | -0.41% | -0.96% | 0.02 | 0.01 | 0.02 |
| Average all | -0.57% | -0.52% | -1.30% | 0.01 | 0.01 | 0.02 |
| Enc Time[%] | 108% |  |  |  |  |  |
| Dec Time[%] | 1867% |  |  |  |  |  |
| Max Delta Rate[%] |  |  |  |  |  |  |
| Min Delta Rate[%] |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.05 |  | 0.05 |  | 0.06 |  |
| Min Delta PNSR (YUV) | -0.01 |  | -0.04 |  | -0.06 |  |

SDR: Constraint set 2 configuration relative to JEM anchor

Class SDR-B

BQTerrace, CS2

RitualDance, CS2

MarketPlace, CS2

BasketballDrive, CS2

Cactus, CS2

Overall

|  | Constraint Set 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 |  |  |  |  |  |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-PSNR(Y) | BD-PSNR(U) | BD-PSNR(V) |
| BQTerrace | -0.56% | 1.72% | -1.37% | 0.01 | -0.01 | 0.01 |
| RitualDance | 0.00% | 0.01% | 0.27% | 0.00 | 0.00 | 0.00 |
| MarketPlace | 0.15% | 0.32% | -0.30% | 0.00 | 0.00 | 0.00 |
| BasketballDrive | 0.12% | -0.55% | -0.35% | 0.00 | 0.01 | 0.01 |
| Cactus | -0.54% | 0.41% | -0.21% | 0.02 | 0.00 | 0.00 |
| Average SDR-B | -0.17% | 0.38% | -0.40% | 0.00 | 0.00 | 0.00 |
| Enc Time[%] | 102% |  |  |  |  |  |
| Dec Time[%] | 297% |  |  |  |  |  |
| Max Delta Rate |  |  |  |  |  |  |
| Min Delta Rate |  |  |  |  |  |  |
| Max Delta PSNR (YUV) | 0.02 |  | 0.02 |  | 0.03 |  |
| Min Delta PNSR (YUV) | -0.01 |  | -0.04 |  | -0.03 |  |

Decoder complexity analysis

Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

(Comparison to HM 16.16 should be with YUV output enabled and reference input disabled; additional reporting of speeds with the yuv output disabled is encouraged.)

The decoding time is reported in the tables of section 4. The measurement methodology is following the guidance. The most of decoding time is consumed by the CNN-based in-loop filtering.

Description of computing platform used to determine decoding times reported in section 5.1

(64b versus 32b executables, clock speeds, use of multicore parallel processing, memory, hard drive characteristics, etc.)

The computing platform is CentOS 7.2 64bit. CPU and memory size are Intel Xeon E5-2640 v4 (2.40GHz) with 128GB. Storage is HDD.

Memory usage of decoder

(Proponents are requested to provide quantitative measurement (e.g. in units of GB) of memory usage of their proposed decoder implementation. Proponents are also encouraged to compare their proposal’s decoder memory usage with that of the HM and/or JEM anchors.

For each constraint set, decoder memory usage measurement only needs to be reported for the sequence class of the largest resolution at the highest target rate point.)

The memory usages of the proposed decoder are 8GB and 2GB for constraint set 1 with SDR-A and constraint set 2 with SDR-B, respectively.

Complexity characteristics of decoder entropy decoding operation

Same as in JEM.

Complexity characteristics of decoder inverse quantization

Same as in JEM.

Complexity characteristics of decoder inverse transform operation

Same as in JEM.

Complexity characteristics of decoder-side motion refinement

Same as in JEM.

Complexity characteristics of decoder motion compensation

Same as in JEM.

Complexity characteristics of decoder intra-frame prediction operation

Same as in JEM.

Complexity characteristics of decoder in-loop filtering operation

Same as in JEM.

Complexity characteristics of additional decoder tools

Compared with the JEM, negligible running time is required. The inter-residual prediction of chroma components introduce the dependency of chrome decoding due to the adaptive reference component selection.

Complexity characteristics of 360° video specific decoding tools

No response.

Complexity characteristics of HDR specific decoding tools

No response.

Degree of capability for decoder parallel processing

Same as in JEM.

Encoder complexity analysis

Encoding time and measurement methodology

(Proponents are requested to provide relative encoding time ratios of their proposal compared to the HM anchors.

If parallel encoding has been used to generate the bitstreams, the following method should be used to measure the encoding time of the proposal: encoding log files should be parsed for each IRAP segment (or equivalent parallelism segment), and the total encoding time should be calculated by adding the encoding time of all parallelism segments and then subtracting the encoding times of the first pictures (or equivalent overlapping elements) for all segments except the first.)

The parallel encoding for constraint set 1 is utilized, which is same as the JEM manner. After obtaining the bitstream file for eash rap, we concatenate them into one bitstream file.

The encoding time is reported in the tables of section 4. The measurement methodology is following the guidance. The most of running time is consumed by the CNN-based in-loop filtering.

Description of computing platform used to determine encoding times reported in section 6.1

(64b versus 32b executables, clock speeds, use of multicore parallel processing, memory, hard drive characteristics, etc.)

Same is in section 5.2.

Memory usage of encoder

(Proponents are requested to provide quantitative measurement (e.g. in units of GB) of memory usage of their proposed encoder implementation. Proponents are also encouraged to compare their proposal’s encoder memory usage with that of the HM and/or JEM anchors.

For each constraint set, encoder memory usage measurement only needs to be reported for the sequence class of the largest resolution at the highest target rate point.)

The memory usage of constraint set 1 is roughly 13.5GB. Another usage of constraint set is now checking.

Complexity characteristics of encoder intra-frame prediction type selection

Same as in JEM.

Complexity characteristics of encoder motion estimation and motion segmentation selection

General

Same as in JEM.

Complexity characteristics of usage of decoder-side motion refinement

Same as in JEM.

Complexity characteristics of encoder transforms and transform type selection

Same as in JEM.

Complexity characteristics of encoder quantization and quantization type selection

Same as in JEM.

Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

Same as in JEM.

Complexity characteristics of encoder entropy coding type selection

Same as in JEM.

Complexity characteristics of additional encoder tools

Same as in JEM.

Complexity characteristics of 360° video specific encoding tools

No response.

Complexity characteristics of HDR specific encoding tools

No response.

Degree of capability for encoder parallel processing

Same as in JEM.

Algorithmic characteristics

Random access characteristics

Same as in JEM.

Delay characteristics

Same as in JEM.

Additional characteristics discussion topic(s)

No additional topic is proposed.

Software implementation description

The proposed algorithm is implemented on the top-of-JEM7.0. It is noted that CNN-based filter is also realized not by GPU implementation but by CPU implementation.

Highlighted aspects discussion

CNN based in-loop filtering is strong tool but high complexity in CPU implementation. Running time of such tool is needed to be considered separately. Furthermore, such the filter is also compared with the role of post-filter.

Closing remarks

This contribution presented description of SDR video coding technology proposal. The proposed coding technology was based on the JEM 7.0 software with six addition coding tools.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[end of document]