| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, CA, 10–20 April 2018 | Document: JVET-J0017-v1 |
| --- | --- |

| Title: | Description of SDR video coding technology proposal by LG Electronics |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Call for Proposals |  |  |
| Author(s) or Contact(s): | Moonmo Koo, Jin Heo, Junghak Nam, Naeri Park, Jaeho Lee, Jangwon Choi, Sunmi Yoo, Hyeongmun Jang, Ling Li, Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 {moonmo.koo, jin78.heo, junghak.nam, naeri.park, jhlee.lee, jangwon84.choi, sunmi.yoo, hm.jang, l.li, jaehyun.lim}@lge.com |
|  | Seethal Paluri, Mehdi Salehifar, Seung Hwan Kim 10225 Willow Creek Rd San Diego, CA 92131 USA | Tel: Email: | +1-858-635-5300 {seethal.paluri, mehdi.salehifar, seunghwan3.kim}@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

It is a response of LG Electronics to the Call for Proposals (CfP) issued jointly by ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 Q.6 (VCEG). The proposal contains multiple tools covering several aspects of video compression technology. These include a quad-tree plus binary and ternary trees (QTBTT) block partitioning structure, a linear interpolation intra prediction, a multiple primary transform, a reduced secondary transform, a motion predictor candidate refinement algorithm based on template matching, and a modified affine motion prediction. When all the proposed algorithmic tools are used, it is reported that the average achieved bit-savings are approximately 34.75% and 26.05% compared to HM16.16 in RA and LD configurations, respectively. It is also reported that the average decoding time for the proposed codec is measured to be approximately 6.4 and 5.8 times compared to those of HM16.16 for RA and LD configurations, respectively.

- Contents

Abstract	i

Contents	i

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.1.1	Bitstream structure	1

2.1.2	Entropy decoding	1

2.1.3	Conversion process from entropy decoded data to symbols	2

2.1.4	Decoder-side motion refinement	4

2.1.5	Inverse quantization	6

2.1.6	Inverse transforms	6

2.1.7	Motion compensation	14

2.1.8	Intra prediction	16

2.1.9	In-loop filtering	21

2.1.10	Additional tools	21

2.1.11	Additional algorithmic description topic(s)	21

2.2	360° Video	22

2.2.1	Projection format (including padding)	22

2.2.2	Quantization handling	22

2.2.3	360° video specific decoding tools	22

2.3	HDR	22

2.3.1	Post-processing	22

2.3.2	Quantization handling	22

2.3.3	HDR specific decoding tools	22

3	Encoder algorithm description	22

3.1	General algorithm description	22

3.1.1	Rate-distortion optimization	22

3.1.2	Partitioning structure selection	23

3.1.3	Intra prediction mode estimation and type selection	23

3.1.4	Motion estimation and motion segment selection	24

3.1.5	Mode decisions	24

3.1.6	In-loop filtering type selection and parameter estimation	25

3.1.7	Transforms and transform type selection	25

3.1.8	Quantization and quantization type selection	25

3.1.9	Entropy coding	25

3.1.10	Additional tools	25

3.1.11	Additional encoder optimization	25

3.1.12	Additional algorithmic description topic(s)	25

3.2	360°	25

3.2.1	Usage of projection format adaptation	25

3.2.2	360° Video specific encoding tools	26

3.2.3	Additional encoder optimization	26

3.3	HDR	26

3.3.1	Pre-processing	26

3.3.2	HDR specific encoding tools	26

3.3.3	Additional encoder optimization	26

4	Compression performance	26

4.1	SDR: Constraint set 1 configuration relative to HM anchor	26

4.1.1	Class SDR-A (RD-curves)	27

4.1.2	Class SDR-B (RD-curves)	30

4.1.3	Overall (BD-rate summary tables)	32

4.2	SDR: Constraint set 2 configuration relative to HM anchor	33

4.2.1	Class SDR-B	33

4.2.2	Overall	36

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	36

4.3.1	Class SDR-A	36

4.3.2	Class SDR-B	39

4.3.3	Overall	42

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	42

4.4.1	Class SDR-B	42

4.4.2	Overall	45

4.5	HDR: Constraint set 1 configuration relative to HM anchor	45

4.5.1	Class HDR-A	45

4.5.2	Class HDR-B	45

4.5.3	Overall	45

4.6	HDR: Constraint set 1 configuration relative to JEM anchor	45

4.6.1	Class HDR-A	45

4.6.2	Class HDR-B	45

4.6.3	Overall	45

4.7	360° Video: Constraint set 1 configuration relative to HM anchor	45

4.7.1	Class 360	46

4.7.2	Overall	46

4.8	360° Video: Constraint set 1 configuration relative to JEM anchor	46

4.8.1	Class 360	46

4.8.2	Overall	46

5	Decoder complexity analysis	46

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	46

5.2	Description of computing platform used to determine decoding times reported in section 5.1	49

5.3	Memory usage of decoder	49

5.4	Complexity characteristics of decoder entropy decoding operation	49

5.5	Complexity characteristics of decoder inverse quantization	49

5.6	Complexity characteristics of decoder inverse transform operation	50

5.6.1	JEM NSST	50

5.6.2	RST	50

5.6.3	LGT	51

5.7	Complexity characteristics of decoder-side motion refinement	51

5.8	Complexity characteristics of decoder motion compensation	51

5.9	Complexity characteristics of decoder intra-frame prediction operation	51

5.10	Complexity characteristics of decoder in-loop filtering operation	51

5.11	Complexity characteristics of additional decoder tools	51

5.12	Complexity characteristics of 360° video specific decoding tools	51

5.13	Complexity characteristics of HDR specific decoding tools	52

5.14	Degree of capability for decoder parallel processing	52

6	Encoder complexity analysis	52

6.1	Encoding time and measurement methodology	52

6.2	Description of computing platform used to determine encoding times reported in section 6.1	54

6.3	Memory usage of encoder	54

6.4	Complexity characteristics of encoder intra-frame prediction type selection	54

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	55

6.5.1	General	55

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	55

6.6	Complexity characteristics of encoder transforms and transform type selection	55

6.7	Complexity characteristics of encoder quantization and quantization type selection	55

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	55

6.9	Complexity characteristics of encoder entropy coding type selection	55

6.10	Complexity characteristics of additional encoder tools	55

6.11	Complexity characteristics of 360° video specific encoding tools	55

6.12	Complexity characteristics of HDR specific encoding tools	56

6.13	Degree of capability for encoder parallel processing	56

7	Algorithmic characteristics	56

7.1	Random access characteristics	56

7.2	Delay characteristics	56

7.3	Additional characteristics discussion topic(s)	57

8	Software implementation description	57

9	Highlighted aspects discussion	57

10	Closing remarks	57

11	Patent rights declaration(s)	58

12	Annex 1: Questionnaire	58

12.1	Codec level questions	58

12.2	Tool level questions	59

13	Annex 2: Example of filling questionnaire HM vs. Proposed	60

13.1	Codec level Q&A	60

13.2	Tool level Q&A	61

13.2.1	Category 1. Partitioning structure	61

13.2.2	Category 2. Entropy coding	62

13.2.3	Category 3. Control data derivation process	62

13.2.4	Category 4. Decoder-side motion refinement	62

13.2.5	Category 5. Intra prediction	63

13.2.6	Category 6. Inter prediction	64

13.2.7	Category 7. Quantization	64

13.2.8	Category 8. Transforms	64

13.2.9	Category 9. In-loop filtering	65

13.2.10	Category 10. “tool-off” and “tool-on” tests	66

14	Reference	67

Introduction

This document describes LGE’s response to the call for proposals (CfP) [1], [2] issued jointly by ISO/IEC JTC 1/SC 29/WG 11 (MPEG) and ITU-T SG 16 Question 6 (VCEG). It aims to provide a video compression technology which has significantly higher compression capability than the state-of-the-art HEVC standard. To achieve this goal, a number of algorithmic tools are proposed, covering many aspects of video compression technology.

A quad-tree plus binary and ternary trees (QTBTT) structure based on quad-tree (QT), ternary tree (TT), and binary tree (BT) is proposed. In QTBTT, after a coding tree unit (CTU) is first split by a QT structure as in HEVC, each intermediary square node can be split further using either a BT or TT structure. In the QTBTT structure, a leaf node of the binary or ternary or quad-tree is called a coding unit (CU), and this segmentation is used for prediction and transform processing without any further partitioning.

In order to increase coding performance of intra prediction, a linear interpolation intra prediction (LIP) is applied on top of Joint Exploration Model (JEM) [3] intra prediction. An LIP flag for an intra coded CU is signaled at the CU level to indicate whether LIP intra prediction mode is used or not.

For the primary transform, reduced number of explicit multiple core transform is utilized by adding two transforms (DST7 and DCT8) on top of HEVC, and maximum explicit multiple transform (EMT) size is restricted to 32. For the secondary transform, a 4×4 layered Givens transform (LGT) and an 8×8 reduced secondary transform (RST) are introduced. Unlike the secondary transform (HyGT) in JEM, RST does not require multiple iterations but reduces the number of multiplications required by factor of 3 as compared to the JEM 8×8 HyGT.

With regard to inter prediction, motion predictor candidate reordering and refinement scheme based on a template matching cost is proposed for MERGE/AMVP mode. Also, an adaptive affine motion model based on multiple (i.e. four and six) parameters is proposed to effectively capture the complex nature of the motion information. This proposal also contains several modifications for better coding performance.

When all the proposed algorithmic tools are used, the proposed video codec achieves approximately 34.75% and 26.05% bit-savings for RA and LD configurations on average, respectively, compared to HEVC. Furthermore, the average decoding time for the proposed codec is measured to be approximately 6.4 and 5.8 times compared to those of HM16.16 for RA and LD configurations, respectively.

Decoder algorithm description

General algorithm description

Bitstream structure

This is identical to that in JEM

Entropy decoding

This is identical to that in JEM.

Conversion process from entropy decoded data to symbols

Intra prediction parameters

Linear interpolation intra prediction (LIP)

This subsection discusses LIP that is applied on luma intra coded CU. To specify the use of LIP, a LIP flag for an intra coded CU is signaled at a CU level. If the current block is an intra coded CU, an LIP flag is first parsed to indicate whether LIP is applied to the current block or not. When LIP is applied, a most probably mode (MPM) index is parsed, and the LIP mode is determined in accordance with the MPM index information. If the LIP flag is false, an MPM flag is parsed and an MPM index is parsed when the MPM flag is enabled (i.e. MPM mode). Otherwise, intra mode information is parsed.

Modified LM-Angular prediction (LAP)

This subsection discusses LM-angular prediction (LAP) modification that is applied on chroma intra coded CU. If the current chroma CU is coded with non-CCLM (Cross-component linear model) intra mode (i.e., angular, DC, or Planar), LAP flag is parsed to indicate whether LAP is applied or not. When the LAP flag is on and the CU width plus height is over than 11, then the selected CCLM mode (single model CCLM or multi model CCLM) is parsed with 1 bit flag and applied to the LAP. If the LAP flag is on and the CU width plus height is not over than 11, the default mode (multi model CCLM) is selected to the LAP.

Motion parameters

FRUC syntax elements are removed since the proposed decoder-side motion refinement scheme in 2.1.4 is implemented instead of FRUC. In affine prediction, two additional syntax elements are considered. In affine merge mode, the syntax indicating the position of the affine merge candidate list is parsed when the candidate number is two. In affine inter mode, a flag is pared whether four or six parameter motion model are used when at least one neighboring block is coded as the affine prediction.

Quantization parameters

This is identical to that in JEM.

Transform coefficients

A secondary transform index is parsed only if the number of nonzero coefficients in the region other than the top-left 8×8 is zero. It is further described in 2.1.6.1.1.

In-loop filter parameters

This is identical to that in JEM.

Partitioning structure parameters

This contribution introduces a block partitioning method called quad-tree plus binary and ternary tree (QTBTT) structure. In the proposed QTBTT, a CTU is recursively split into sub-blocks based on a quad-tree structure as HEVC and it can further be split either by binary tree structure or by ternary tree structure. The signaling and splitting of QT structure is identical to HEVC.

After QT splitting, a BTT_split_idx syntax can be signalled to indicate next split type as stated in Table 1. The first bin signals whether a block is partitioned or not. If the block is not split, the second bin is not presented. When the block is split, the second bin signals to either a binary tree or ternary tree.

Table 1 QTBTT syntax

| BTT_split_idx | Split type |
| --- | --- |
| 0 | NO_SPLIT |
| 1 | BINARY_TREE |
| 2 | TERNARY_TREE |

Once split type is chosen by the second bin of BTT_split_idx, a BTT_split_mode syntax is further signalled to indicate split direction (i.e., horizontal or vertical). BTT_split_mode is not signalled when BTT_split_idx is equal to zero, which means no further split. BTT_split_mode equal to 0 specifies a CU is split with the horizontal direction. When BTT_split_mode equal to 1, a CU is split with the vertical direction. This syntax cannot be presented according to BTT_split_idx and block size.

Finally, split mode of a CU is derived based on the current split type and BTT_split_mode as stated inTable 2.

Table 2 Derivation of split mode for a CU

| Split type | BTT_split_mode | Split mode |
| --- | --- | --- |
| BINARY_TREE | 0 | PART_BT_HOR |
|  | 1 | PART_BT_VER |
| TERNARY_TREE | 0 | PART_TT_HOR |
|  | 1 | PART_TT_VER |

The proposed QTBTT structure includes some restrictions regarding the minimum quadtree size, maximum BT size, and minimum BT size. For example, the quadtree splitting process can be iterated until the leaf node size of a quad-tree reaches the minimum allowed quad-tree size (MinQTSize) that is specified in the bitstream. The BTT splitting process cannot be started until the leaf node size reaches the maximum allowed size of the binary or ternary tree (MaxBTTSize) that is specified in the bitstream. The BTT splitting process can be iterated until either the leaf node size of a binary or ternary tree reaches the minimally allowed leaf node size (MinBTTSize), or the depth of the binary or ternary tree reaches the maximum depth (MaxBTTDepth) that is specified in the bitstream. According to QTBTT parameters in Table 3, syntaxes for split type or split direction can be conditionally decoded.

Table 3 QTBTT parameters used for test

| QTBTT parameters | I slices |  | P and B slices |
| --- | --- | --- | --- |
| CTUSize | 128×128 |  | 128×128 |
|  | Luma | Chroma |  |
| MinQTSize | 8×8 | 4×4 | 8×8 |
| MaxBTTSize | 32×32 | 64×64 | 128×128 |
| MinBTTSize | 4 | 4 | 4 |
| MaxBTTDepth | 3 | 3 | 3 |

Decoder-side motion refinement

Motion Predictor Candidate Refinement (MPCR)

In the inter prediction process, a list of motion vector predictor candidates for MERGE/AMVP mode is constructed with motion information of spatial/temporal neighboring blocks in a pre-defined order. However, the scheme used in HEVC does not consider the underlying characteristics of sequence or blocks as the construction of list is fixed. This contribution proposes reordering and refining the candidate list using a template matching scheme at the decoder. The proposed motion predictor candidate refinement (MPCR) scheme offers a chance to change the generation of the candidate list by reordering and allows an additional candidate without any overhead of an additional syntax element. The MPCR scheme is applied to both MERGE and AMVP modes.

Candidate list reordering

The proposed scheme reorders the candidate list based on the template matching method. The candidates in the initial list are derived using the MERGE candidate selection process. Figure 1 depicts an example of the MERGE list candidates that have been reordered based on the template matching cost. The proposed change enables candidates that might have had a lower priority in the MERGE list to have a higher priority in the reordered list without requiring any additional syntax. Additional candidates can also be added without any syntax overhead. This is detailed in the sections below.

Figure 1 Example for candidate list and reordered candidate list

The template matching neighbors used by the current block to compute the cost for reordering the list are illustrated in Figure 2. First, the availability of the above and the left neighbors is checked and then the available blocks are used to compute the cost. The sum of absolute difference (SAD) is calculated between the template in the current picture and the block in the reference picture. This is done for every candidate in the list. The candidates are then reordered in descending order of their SAD cost.

Figure 2 Template matching neighbors used to compute the cost

The cost for the uni-directional and bi-directional candidates are described in below:

- In case of uni-directional candidate:

|  | cost_uni(LX) = costA(LX) + costL(LX) | (1) |
| --- | --- | --- |

- In case of bi-directional candidate:

|  | cost_bi = (cost_uni(L0) + cost_uni(L1)) >> 1 | (2) |
| --- | --- | --- |

where costA(LX) and costL(LX) refer to the template matching cost of the above and left neighbors for the list X where X is either 0 or 1. Similarly, cost_bi is computed as an average of the uni-directional cost for both of the lists.

It should be noted that the template matching distortion measured with the SAD is influenced by the position of the candidate. For example, if the candidate is placed on left neighbor of the current block, the cost of the left template region will be generally lower than other candidates. If the candidate is placed on the above neighbor of the current block, the cost of the above template regions will be generally lower than other candidates. Furthermore, if the template size of the neighbors is different, the cost will strongly dependent on the position of the candidate. These differences are compensated by normalizing the cost using a right shift operation. The memory (i.e., line buffer) for this operation is reduced by using 2*width and 2*height for the above and left template regions respectively.

Candidate refinement and list update

A candidate refinement is performed after the list has been reordered. Note that ATMVP and STMVP candidates are not considered as a reference candidate for the refinement process. The motion vector of the first candidate in the reordered list becomes a starting point for the refinement process. Then a local search around the starting point is performed and the motion vector that results in the minimum cost is taken as the motion vector for the refined candidate. When the refined candidate and the reference candidate have the same motion vector, the list is traversed to the next candidate for consideration of the refinement process. When the refined candidate is available, it is added to the list and the number of candidates in the list is incremented by one. The SAD is computed using the template in the current picture and a block in the reference picture. An example of the updated candidate list with a refined candidate is depicted in Figure 3.

Figure 3 Example for of the refined candidate and candidate list

Interaction with other inter tools

The proposed scheme, which exits in harmony with other inter tools, is executed during the decoding process when the merge flag is TRUE and the affine flag is FALSE. LIC which is adopted into JEM is also applied to the MERGE candidates in the proposed scheme. The LIC flag for the MERGE candidates is derived using the same rules as in JEM. The LIC flag for the proposed motion refined candidate i.e., Refined B1 is set to be equal to that of the corresponding reference candidate. However, in the block matching motion estimation, mean removed SAD (MR-SAD) is used instead of SAD when the LIC flag of any candidate in the candidate list is true.

Extension to AMVP

In this contribution, the MPCR scheme is applied on AMVP mode. In order to facilitate reordering of the candidate list, the buffer size is increased from 3 to 5 candidates. The candidate checking order is also unified to the MERGE method i.e., {A1, B1, B0, A0, B2, TMVP, Zero MVs} with the exception of the ATMVP, STMVP and combined bi-predicted candidates. MPCR for AMVP mode is applied by first constructing the candidate list with the checking order i.e., {A1, B1, B0, A0, B2, TMVP, Zero MVs} and then by reordering and refining the list as described in the sections above. Finally, the first 2 candidates in the list are selected for AMVP mode and this process is iterated for L0 and L1. It should be noted that the syntax element for AMVP is not changed.

Inverse quantization

This is identical to that in JEM.

Inverse transforms

For core transform, simplified explicit multiple core transform is utilized by adding two transforms (DST7 and DCT8) on top of HEVC, and maximum EMT is restricted to a size of 32. For the secondary transform, the 4×4 LGT and the 8×8 RST are introduced. Unlike the secondary transform (HyGT) in JEM, RST does not only require multiple iterations but also reduces the number of multiplications required by factor of 3 compared to JEM 8×8 HyGT. An overview of the proposed structure is illustrated in Figure 4 below.

After inverse quantization, secondary inverse transform and core (primary) inverse transforms are applied to restore the residual block.

Secondary Transforms

A secondary inverse transform is conditionally applied when the following two conditions are satisfied:

- ST ON? :

- Block size is greater than or equal to the given threshold (BW>=4 && BH>=4)

- Transform skip mode flag is equal to zero

- ST_idx: secondary transform index is greater than zero.

There are two kinds of secondary transforms; LGT and RST. If both width (BW) and height (BH) of a transform coefficient block is greater than 4, then the 8×8 RST is applied to the top-left 8×8 region of the transform coefficient block. Otherwise, the 4×4 LGT is applied on the top-left min(8, W) × min(8, H) region of the transform coefficient block.

Figure 4 Details of Secondary Transforms

Reduced Secondary Transform

The main idea of a Reduced Transform (RT) is to map an N dimensional vector to an R dimensional vector in a different space, where R/N (R < N) is the reduction factor.

The RT matrix is an R×N matrix as follows:

|  |  | (3) |
| --- | --- | --- |

where the R rows of the transform are R bases of the N dimensional space. The inverse transform matrix for RT is the transpose of its forward transform. The forward and inverse RT are depicted in Figure 5.

Figure 5 Forward and inverse Reduced Transform

In this contribution, the 8×8 RST with a reduction factor of 1/4 is applied. In other words, the 64×16 inverse RST matrix is used at the decoder side to generate core (primary) transform coefficients in 8×8 lower (top-left) regions. The proposed 8×8 RST reduces computational complexity (e.g. # of required multiplication) by 3 times compared to JEM 8×8 (HyGT) while achieving additional coding performance. Since RST uses 64×16 matrix, it only produces non-zero coefficients in the lower 4×4 region within the given 8×8 region. Based on the above given information, residual part has been further changed to reflect the distribution of non-zero coefficients which gives additional coding performance improvement. Unlike JEM NSST, the proposed RST does not require multiple iterations.

The forward 8x8 RST uses 16×64 matrices so that it produces non-zero coefficients only in the lower 4×4 region within the given 8×8 region. In other words, if RST is applied (i.e. ST_idx is greater than zero) then the lower 8×8 region except the lower 4×4 region (this region is called ROI region) will have to generate only zero coefficients.

As a result, the ST_idx coding method based on the distribution of non-zero coefficients in the ROI region is introduced. Specifically, ST_idx is not coded when RST is used as a secondary transform and any non-zero element is detected within the ROI region because it implies that RST was not applied. In such a case, ST_idx is inferred to be zero.

As depicted in Figure 6, if any non-zero coefficient in scanning position from 17 to 64 is detected, then it would imply that RST was not applied and ST_idx may be inferred to be zero.

Figure 6 Scanning the position 17 to 64 for non-zero element

Therefore, the decoder first decodes the coefficients, and later scans the ROI region. If any non-zero elements are found then, ST_idx is inferred to be zero. If no non-zero elements are detected then it will decode/parse the ST_idx as illustrated in Figure 6. Note that the transform set configuration (e.g. 35 sets, and each consists of 2 or 3 transforms) for the 8×8 RST are identical to JEM NSST.

Layered Givens Transform

For smaller blocks, a Givens rotation based transform called LGT is applied on 4×4 blocks at the top-left 4×4, 4×8, or 8×4 region of a coded block. This consists of 5 Givens rotation layers (GRLs) and 6 permutations. Each GRL is composed of 8 Givens rotations and each permutation shuffles transform inputs or outputs of the previous stage. The overall 4×4 LGT structure is depicted in Figure 7.

Figure 7 Structure of the 4×4 LGT

In Figure 7, the elements xi (i = 0, 1, …, 15) and yj (j = 0, 1, …, 15) are the input and output vectors of the data. Each of these vectors corresponds to a 4×4 block and is rearranged by following the row-first order. Each square box with a cross mark indicates a Givens rotation. The operation of the permutation block in Figure 7 represents a shuffling process where the current elements (e.g. 16 elements) are sorted into an output array using a predetermined (or stored) order.

Note that the transform set configuration (e.g. 35 sets, and each consists of 2 or 3 transforms) and signaling for the 4×4 LGT are identical to JEM NSST.

Primary Transforms and transform type selection

This proposal introduces DST7 and DCT8, which are similar to the Explicit Multiple Transform (EMT) in JEM. The proposed transform differs from the EMT in JEM as it uses a different mapping of the intra prediction modes to the transform pairs as well as fewer trigonometric transforms as detailed in the sections below.

Primary Transform Kernel Selection

The EMT index ranging from 0 to 3, which indicates a transform pair for the horizontal and vertical direction in a mapping table, is not changed from JEM. However, for intra-predicted residual, a uniform mapping with only DST7 and DCT8 is applied to all intra-prediction modes. This is contrary to JEM EMT where each intra prediction mode group uses its own unique mapping. The mapping for inter-predicted residual is identical to that in JEM EMT, which also consists of four combinations of DST7 and DCT8.

The EMT flag, which is signaled for each block, indicates whether EMT is applied or not. The signaling details do not differ to that of JEM. The EMT max size supported in this proposal is 32×32. The 64×64 EMT that exists in JEM was omitted due to its computational complexity. The transform kernel for each size was implemented from a direct matrix computation, which is explained in more details in the following sections. Other transform configurations remain same as that of JEM.

Forward and inverse DST7 transform kernels

Equation (4) below corresponds to the inverse DST7 matrix. Also, the transpose of Equation (4) corresponds to the forward DST7 transform.

|  |  | (4) |
| --- | --- | --- |

DST7 4×4 Transform

This is identical to that in HEVC.

DST7 8×8 Transform

A forward N×N DST7 with 2N + 1 length Discrete Fourier Transform (DFT) is implemented to achieve the 8×8 DST7. Mathematical relationship between the DST7 and DFT can be expressed as follows:

|  | , | (5) |
| --- | --- | --- |
|  | where , | (6) |
|  | , | (7) |
|  | and . | (8) |

where,  in Equation (5) denotes that only the imaginary part of the DFT result is chosen. Equation (5) holds true only if N is even. It should be noted that the authors in [4] describe the conditions when N is either odd or even. Without loss of generality, if N is treated as even (as proposed in the proposed test model), the norm of the transform vectors results in 1, as in JEM and the proposed model. It can be deduced from Equation (5) that the 8×8 DST7 requires a 17-point DFT. In order to reduce the complexity, a 17-point Winograd FFT [5] was adopted as other implementations of FFT algorithms are more computationally complex. In designing the inverse N×N DST7, the following relation between the inverse DST7 and DFT is utilized:

|  | , | (9) |
| --- | --- | --- |
|  | where , | (10) |
|  | , | (11) |
|  | and . | (12) |

Similar to the forward DST7, its inverse can be deduced to be the same as the forward DST6. It should be noted that Equation (9) assumes that N is even. The inverse DST7 of Equation (9) can be expressed with the 2N+1 length DFT. Therefore, the 17-point Winograd FFT in the forward 8×8 DST7 case is reused here.

DST7 16×16 Transform

The derivation of a 16×16 forward DST7 with FFT also exploits Equation (5). However, using a direct implementation of a 33-point FFT does not seem beneficial due to the computational overhead. Although, it is more beneficial to use a 33-point Winograd FFT, it would introduce a significant number of additions and subtractions. Consequently, the 33-point Winograd FFT has been reformulated into an equivalent multi-dimensional FFT consisting of 3-point and 11-point FFTs as illustrated in Figure 8.

Figure 8 16 × 16 DST7 implementation with a 33-point FFT of which equivalent two dimensional FFT consists of 3-point Winograd FFTs and 11-point FFTs

The stages 1, 3 and 5 in Figure 8 indicate the number of permutations of the input, the output of the 3-point FFT stage and the outputs of the 11-point FFT stage, respectively. These are symbolic of the P, Q, and R matrices in Equation (5) and Equation (9), which must be computed for realizing the 1-D DFT.

Designing the inverse 16×16 DST7 according to Equation (9), requires the 33-point FFT which results in a similar computational architecture to Figure 8. However, its permutation blocks in Figure 8 have to be substituted for those of the forward DST7, as the P, Q, and R matrices in Equation (5) and Equation (9) are different.

Furthermore, redundant computations were removed by exploiting the symmetry of inputs that are supplied to the DFT and by utilizing only the imaginary part of the DFT outputs to compute the final DST7 outputs. Consequently, the number of multiplications can be reduced. Table 4 states the required number of element operations for transforming one row or column of length N along with matrix computation counterpart.

Table 4 Number of element operations required for transforming one row or column

| Size | Proposed implementation |  | Matrix computation |  |
| --- | --- | --- | --- | --- |
|  | # of mult. | # of add./sub. | # of mult. | # of add./sub. |
| 8×8 | 21 | 77 | 64 | 56 |
| 16×16 | 42 | 125 | 256 | 240 |
| 32×32 | 87 | 279 | 1024 | 992 |

DST7 32×32 Transform

The forward and inverse 32×32 DST7s are implemented similar to the details outlined in the prior section. Equation (5) and Equation (9) are used for computing the forward and inverse transforms. This necessitates a 65-point FFT design which can be realized using an equivalent multi-dimensional FFT. This multi-dimensional FFT consists of two 5-point FFT stages and a 13-point FFT stage as depicted in Figure 9.

Figure 9 32×32 DST7 implementation with a 65-point FFT of which equivalent two dimensional FFT consists of 5-point Winograd FFTs and 13-point FFTs

Both the forward and inverse 32×32 DST7 illustrated in Figure 9 have a similar architecture to the 16×16 DST7. However, it should be noted that the associated permutations of the 32×32 DST7 and 16×16 DST7 are different. Additionally, a similar scheme to reduce the unnecessary repeated computation is also applied.

The memory requirement for the Winograd FFTs is characterized by storing multiplication coefficients. Subsequently, the memory usage is much less than the case of direct matrix multiplication. Additionally, each set of coefficients in the same length FFT modules can be shared. For example, in Figure 9, storing only one set of coefficients for all the 5-point Winograd FFTs is necessary.

Forward and inverse DCT8 transform kernels

Equation (13) below describes the inverse DCT8 matrix. The transpose of Equation (13), corresponds to the forward DCT8 transform:

|  |  | (13) |
| --- | --- | --- |

If the inverse N×N DST7 is denoted as  and the inverse N×N DCT8 as , the relation between DST7 and DCT8 are described as the following:

|  | , | (14) |
| --- | --- | --- |
|  | where , | (15) |
|  | and | (16) |

In the Equation (14), JN is the matrix is represented by 1s along its anti-diagonal line, and the matrix DN alternates between 1 and -1 on its diagonal line. Therefore, DCT8 can be derived from DST7 with sign changes and reordering just before and after the DST7 computation. Hence, DST7 is reused in this implementation for DCT8. The sign changes and shuffling do not add any additional overhead to DST7, so that the computational complexity of DCT8 is identical to that of DST7. This avoids the usage of any additional memory in DCT8.

Motion compensation

Affine motion prediction & compensation

In JEM, the affine motion prediction is introduced to describe the complex motions such as rotation and zooming efficiently. It includes two affine modes: affine inter (AF_INTER) mode and affine merge (AF_MERGE) mode.

In this contribution, three modifications are proposed for the affine motion prediction as follows:

- Adaptive affine merge candidate selection is proposed for AF_MERGE.

- Construction process of the control point motion vector predictor (CPMVP) pair candidate list is proposed for AF_INTER.

- Adaptive four- and six-parameter motion model is used for AF_INTER.

Adaptive affine candidate selection for Affine merge

In JEM, AF_MERGE determines a control point motion vector (CPMV) pair according to the affine motion model of the neighbouring block coded as the affine motion prediction. The first affine coded neighbouring block in the search order is used for AF_MERGE. A current block can be coded as AF_MERGE when one or more of the neighbouring blocks are coded as affine motion prediction.

The proposed candidate selection can use up to two candidates to determine the CPMV pair for AF_MEREG. The details are descried as follows:

- The scanning order of candidate is A, B, C, D, and E in Figure 10.

- When the number of available (affine mode) candidates in the candidate list (A, B, C, D, and E) is zero, the affine merge mode is skipped for the current block

- When the available candidate number is one (e.g. A in Figure 11), the corresponding candidate is used to derive a CPMV pair (CPMV0 and CPMV1) so that the candidate position index is not required.

- When the available candidate number is more than one, the first two candidates in the scanning order are considered for AF_MERGE and the position of the candidate is signaled.

Figure 10 Scanning order for the affine motion prediction

Figure 11 Example of AF_MERGE

Affine Motion Vector Prediction

In AF_INTER, the construction process of the CPMVP candidate is proposed. Same as AMVP, the number of candidate is two and the index indicating the position of candidate list is signaled.

The construction process of a CPMVP candidate list is as follows:

- Scan the neighbouring blocks to check whether it is coded as the affine motion prediction or not. If the scanned block is coded as the affine prediction, derive the motion vector pair of current block from the affine motion model of the scanned neighbouring block until the number of candidate is two.

- If the number of candidate is less than two, perform the candidate construction process of JEM.

Adaptive four and six parameter motion model

In AF_INTER, a six-parameter motion model is proposed to use in addition to the existing a four parameter motion model in JEM. The six-parameter affine motion model is described in Equation (17).

|  | x'=a*x+b*y+cy'=d*x+e*y+f | (17) |
| --- | --- | --- |

Here, the coefficients a, b, c, d, e, and f are the affine motion parameters and, (x,y) and (x',y') are the co-ordinates of pixel location before and after the transformation of the affine motion model. To use the affine motion model in video coding, if CPMV0, CPMV1 and CPMV2 are the MV for CP0 (left above), CP1 (right above) and CP2 (left bottom), Equation (17) can be described as:

|  | vx=v1x-v0xw*x+v2x-v0xh*y+v0xvy=v1y-v0yw*x-v2y-v0yh*y+v0y | (18) |
| --- | --- | --- |

where CPMV0={v0x,v0y}, CPMV1={v1x,v1y}, CPMV2={v2x,v2y}, and w and h are the width and the height of coding block, respectively. Equation (18) describes the motion vector field (MVF) of a block.

A flag is parsed at the CU level to indicate whether the four-parameter or the six-parameter affine motion model is used when the neighboring blocks are coded as the affine prediction. If no neighboring block is coded as the affine prediction, the flag is skipped and four-parameter model is used for the affine prediction. In other words, the six-parameter model is considered in the condition that one or more of the neighboring blocks are coded as the affine motion model. When it comes to the number of CPMVD, two and three CPMVDs are signaled for the four-parameter and six-parameter affine motion model, respectively.

Modified ATMVP

Basically, the whole process for the temporal motion vector derivation is similar with JEM. However, in the proposed method, the usage of memory is reduced since the reference picture to find the corresponding block is restricted to the collocated picture (designated in the slice segment header), while it is up to 4 reference pictures in the method in JEM. To find the corresponding block, a temporal vector is derived from one of spatial candidates in a scanning order. If the current candidate has the reference picture same with the collocated picture, the search is finished.

Intra prediction

The processes of intra mode coding with 67 intra prediction modes, chroma intra mode coding, the boundary prediction filter, the multiple model CCLM (MMLM), and down-sampling filters in CCLM mode (MFLM) are identical to those in JEM.

Luma intra mode coding

To accommodate the increased number of directional intra modes, an intra mode coding with 6 most probable modes (MPMs) is used in JEM.

The changes proposed in this proposal differ in the order of the neighboring intra modes being considered. Although, the spatial location of the neighbors and the pruning process to remove duplicate candidates remains the same as in merge mode (described in Figure 10), the neighboring intra modes are added to the list as follows:

Case 1: When the intra mode of E is similar to that of A, the order of neighbor intra modes is B, A, Planar, DC, C, E, and D.

Case 2: When the intra mode of E is similar to that of B, the order of neighbor intra modes is A, B, Planar, DC, D, E, and C.

Case 3: If neither of case 1 nor 2 occurs, the order of neighbor intra modes is A, B, Planar, D, C, and E.

A truncated unary binarization is used to code the selected mode from the 6 MPMs. For the remaining 61 non-MPMs, if the mode to be indicated is not the last mode among the 61 non-MPMs, the mode is signalled with a 6-bit fixed-length code. If the mode to be indicated is the last mode, the mode is signalled with a 4-bit fixed-length code.

Reference sample smoothing

The reference sample smoothing is identical to that in JEM for weak filtering. Strong filtering is employed in the proposed model with minor modifications from that in HEVC. The smoothing can be activated when either the width (W) or the height (H) is greater than or equal to 32. This enables its use even when the width and height dimensions are different from one another. In the case when the width and height are not equal, a division free implementation using shift operations is used where the weights are up-scaled to an approximate integer value of 210. This is formulated in Equation (19) to Equation (23).

|  | ptemp-1y=W+H-1*p-1-1+y+1*p-1W+H-1 | (19) |
| --- | --- | --- |
|  | ptempx-1=W+H-1*p-1-1+x+1*p[W+H-1][-1] | (20) |
|  | offset=W+H | (21) |
|  | p-1y=(ptemp-1[y]*scaleW+H+(offset*scaleW+H≫1))≫10 | (22) |
|  | px-1=(ptempx-1*scaleW+H+(offset*scaleW+H≫1))≫10 | (23) |

where

| scale[x] = { 0, 0, 512, 341, 256, 205, 171, 146, 128, 114, 103, 93, 85, 79, 73, 68, 64, 60, 57, 54, 51, 49, 47, 45, 43, 41, 39, 38, 37, 35, 34, 33, 32, 31, 30, 29, 28, 28, 27, 26, 26, 25, 24, 24, 23, 23, 22, 22, 21, 21, 20, 20, 20, 19, 19, 19, 18, 18, 18, 17, 17, 17, 16, 16, 16, 16, 16, 15, 15, 15, 15, 14, 14, 14, 14, 14, 14, 13, 13, 13, 13, 13, 13, 12, 12, 12, 12, 12, 12, 11, 11, 11, 11, 11, 11, 11, 11, 11, 10, 10, 10, 10, 10, 10, 10, 10, 10, 10, 10, 9, 9, 9, 9, 9, 9, 9, 9, 9, 9, 9, 9, 8, 8, 8, 8, 8, 8, 8, 8, 8, 8, 8, 8, 8, 8, 8, 8, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 7, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 6, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4 } | (24) |
| --- | --- |

Four-tap intra interpolation filter

In HEVC, a two-tap linear interpolation filter is used to generate the intra prediction block in the directional prediction modes (i.e., excluding the Planar and DC predictors). In this proposed test model, a four-tap intra interpolation filter is used for directional intra prediction filtering. Two types of four-tap interpolation filters are used: (i) Cubic interpolation filters for blocks with the size smaller than or equal to 64 samples or with the intra modes whose absolute angles are less than 11 (±6 modes from horizontal and vertical modes), and (ii) Gaussian interpolation filters for blocks with the size larger than 64 samples or with the intra modes whose absolute angles are equal to or larger than 11.

Linear interpolation intra prediction (LIP)

The block-based intra prediction generates the predicted samples by extrapolating from the neighboring reconstructed samples along a certain direction. This directional intra prediction is based on the assumption that the texture in a region is directional, which means the sample values have a strong correlation and change smoothly along a specific direction. Thus for smooth region, a directional intra prediction can work well. However, sample values may change drastically for high-detailed regions, and a fixed direction cannot describe them well. This means if the conventional block-based intra prediction is employed, regardless of the direction selected, it is difficult to predict the whole block accurately. Usually the bottom right side samples in the block to be coded are not predicted accurately because they are not in close proximity to the reference samples and consequently, the correlation between them is weaker. In order to solve this problem, the linear interpolation intra prediction (LIP) is proposed in intra coding to increase the accuracy of bottom right region.

The proposed intra prediction method can be classified into two prediction types namely (i) the conventional intra prediction method, and (ii) the LIP method. A CU level flag is signalled to indicate whether LIP is applied or not. LIP flag 0 indicates that JEM intra prediction method is used, while a value of 1 means that the LIP approach is applied. The original intra prediction method uses 67 modes including 65 angular predictions and 2 non-angular (Planar and DC) predictions. However, the LIP prediction method was applied to 66 prediction modes; i.e., 65 angular predictions and LIP planar prediction.

In JEM intra coding, the primary transform is used for residual coding. Then, the secondary transform is applied between the forward core transform and quantization (at the encoder side) and between the de-quantization and inverse core transform (at the decoder side). In the proposed LIP intra coding/prediction, if the current block is coded in the conventional intra prediction mode both the primary and secondary transforms are still used. Whereas, if the current block is encoded in the LIP mode, the secondary transform is skipped.

The proposed LIP intra prediction method first computes the intensity value of the bottom right (BR) neighboring sample, as described in Equation (25). Then, it generates the intensity values of the bottom row (Pb(x,y)) and the right column (Pr(x,y)) of the neighboring samples as depicted in Figure 12. These samples serve as references on the opposite side of the CU and are derived using the intensity values of the bottom right neighboring sample and the intensity values of other corner neighboring samples, i.e., the top right neighboring sample, the bottom left neighboring sample, as described in Equation (26) and Equation (27), respectively.

|  | BRW,H= W×R2W-1,-1+H×(-1, 2H-1)(W+H) | (25) |
| --- | --- | --- |

|  | Pbx, H= (W-1-x×R-1, H +(x+1)×BR(W, H)W | (26) |
| --- | --- | --- |

|  | PrW, y= (H-1-y×RW, -1 +(y+1)×BR(W, H)H | (27) |
| --- | --- | --- |

Figure 12 BR, bottom row, and right column reference pixels

where, R(x, y) denotes the intensity value of the reconstructed neighboring sample at (x, y) coordinate, W and H are the block width and height, Pb(x,y) and Pr(x,y) are the intensity values of the bottom row and right column, respectively.

Interpolation is performed linearly using the horizontal and vertical predictions for the LIP planar predictor. The LIP planar predictor calculation is described in Equation (28).

|  | Px, y= (H×Phx, y×y+1+W×Pvx, y×x+1)(H×W×(x+y+2) | (28) |
| --- | --- | --- |

The horizontal prediction Ph(x,y) and the vertical prediction Pv(x,y) are calculated as follows:

|  | Phx,y=W-1-x×R-1,y+x+1×PrW,y | (29) |
| --- | --- | --- |

|  | Pvx,y=H-1-y×Rx,-1+y+1×Pbx,H | (30) |
| --- | --- | --- |

Figure 13 illustrates the sample prediction process using LIP for an arbitrary angular prediction mode. The red line represents the prediction direction. The points ‘A’ and ‘B’ represent the samples at the integer sample positions and the point ‘P’ indicates the sample at 1/32 sample position in the reference sample array. The point ‘A´’ and ‘B´’ represent the samples at the integer sample positions and the point ‘P´’ indicates the sample at 1/32 sample position in the opposite side reference sample array.

Figure 13 Sample prediction process for LIP

The first stage of the intra angular mode prediction involves extrapolating samples from a projected reference sample location based on a given direction. The projected reference sample location is computed with four-tap intra interpolation filter which is used to obtain the value of the projected reference sample using four closest reference samples located at integer position.

The second stage in the prediction process requires an identical approach to generate another predicted sample value at the same sample location. The second predicted sample value is obtained by projecting the location of the sample onto the opposite side of the reference sample array by applying the selected prediction direction and interpolating the value to 1/32 sample position accuracy. The bilinear interpolation is performed linearly using the two closest opposite side reference samples in the direction of prediction.

The LIP takes advantage of the difference in accuracy between the input intensities in the final interpolation process. Specifically, larger weights are applied to the positions that are closer to more reliable neighboring positions. Final predicted sample value is calculated by using Equation (31).

|  | C= (W1×P+W2×P')(W1+W2) | (31) |
| --- | --- | --- |

where, C is the predicted sample value, P and P’ are the predictors generated from the reference sample location and the opposite side reference sample location, respectively. W1 and W2 are weighting factors determined according to the distance between the current sample position and the reference (the opposite side reference) sample position.

Cross-component linear model prediction (CCLM)

To reduce the cross-component redundancy, a cross-component linear model (CCLM) prediction mode is used, for which the chroma samples are predicted based on the reconstructed luma samples of the same CU by using a linear model.

Although it is similar to JEM, the change proposed is that the CCLM Cb-to-Cr prediction is also used in the CCLM luma-to-chroma intra prediction modes.

Multiple model CCLM (MMLM)

The process of MMLM is identical to that in JEM.

Downsampling filters in CCLM mode (MFLM)

The process of MFLM is identical to that in JEM.

Modified LM-Angular prediction in chroma intra prediction (LAP)

With LAP (LM-Angular prediction), which was introduced in JVET-E0077, two prediction patterns named P1 and P2 are generated for the chroma block first, and then they are combined together as follows:

|  | P=0.5×P1+0.5×P2 | (32) |
| --- | --- | --- |

where, P1 is generated with the chroma intra modes other than the CCLM modes (including angular, DC and Planar mode), and P2 is generated with the CCLM mode, as described in 2.1.8.5. Unlike the original LAP algorithm, which generates P2 with the multiple model CCLM mode only, two CCLM modes (single model CCLM mode or multiple model CCLM mode) are used for P2 prediction in the proposed test model.

If the current chroma block is not coded with the CCLM mode, i.e., it is coded the angular, DC and Planar intra modes, a flag is followed to indicate whether LAP is applied or not with this mode. The encoder chooses the CCLM mode of P2 (single model CCLM or multiple model LM) in the RDO process and signals the selected CCLM mode of P2 if the LAP flag is on.

In-loop filtering

This is identical to that in JEM.

Additional tools

N/A

Additional algorithmic description topic(s)

N/A

360° Video

Projection format (including padding)

N/A

Quantization handling

N/A

360° video specific decoding tools

N/A

HDR

Post-processing

N/A

Quantization handling

N/A

HDR specific decoding tools

N/A

Encoder algorithm description

General algorithm description

Rate-distortion optimization

Distortion metrics

This is identical to that in JEM.

Rate-distortion dependency models

N/A

Partitioning structure selection

In HEVC, a CTU is recursively split into CUs based on a quadtree structure. In the JEM, the QTBT structure is used to split a CTU into CUs based on the quad-tree structure and binary tree structure.

This contribution proposes a quad-tree plus binary and ternary tree (QTBTT) structure. In QTBTT, a CTU is split by a quad-tree structure as in HEVC. After the quad-tree partitioning, each intermediary partition has a square shape. Each intermediary square node can be split further using either a binary tree or ternary tree structure. If the intermediary node is split using the binary tree, it can be split either horizontally or vertically as depicted in Figure 14 (a-b). If the intermediary node is split using a ternary tree, it can also be split either horizontally i.e., 2N×N/2, 2N×N, and 2N×N/2 or vertically i.e., N/2×2N, N×2N, and N/2×2N as represented in Figure 14 (c-d). In the QTBTT structure, a leaf node of the binary or ternary tree is called CU, and this segmentation is used for prediction and transform processing without any further partitioning. This means that the CU, PU and TU have the same block size.

(a)               (b)               (c)               (d)

Figure 14 (a) Horizontal binary partitioning, (b) Vertical binary partitioning, (c) Horizontal ternary partitioning, (d) Vertical ternary partitioning

Intra prediction mode estimation and type selection

At the encoder a two-stage intra mode decision process similar to that in JEM is used.

N intra prediction modes are first selected based on a sum of absolute transform difference (SATD) cost. Then, the best intra prediction mode is chosen based on the full rate distortion optimization (Full RDO) cost. In order to reduce the complexity, LIP is only applied to the 6 MPM candidate modes. The final intra mode decision process is summarized as follows.

Table 5 Summary of the intra mode decision process

|  | Candidate list update | Matching criteria |
| --- | --- | --- |
| Step 1: Rough mode decision | Select N candidate modes from even modes in JEM | SATD +λ*mode information |
| Step 2: Rough mode update | Update N candidates modes from - Previously chosen N candidates - Neighboring modes of previously selected N modes | SATD +λ*mode information |
| Step 3: Final mode decision | Select the final mode from the following three groups - N candidates - M MPM modes after pruning check - 6 MPM modes based on LIP | Full RDO |

In the rough mode decision, the number N depends on intra prediction block size. Usually N is set to three. For Full RDO decision, M MPM modes are added to candidate list after the pruning check. Usually M is found to be two or three. The proposed intra mode decision process is depicted in Figure 15.

Figure 15 Encoder process

Motion estimation and motion segment selection

General

In the proposed contribution, motion estimation process and the method of motion segment selection are used as similar to that in JEM. Two tools are proposed, one of them is about decoder-side motion refinement and another one is about affine motion prediction. Further details are described in 3.1.4.2 and 3.1.4.3.

Usage of decoder-side motion refinement

There is no difference compared with usage at the decoder.

Affine motion prediction & estimation

In the affine prediction, three modifications are proposed as follows:

- Adaptive candidate selection for affine merge mode

- Construction process of the CPMVP candidate list for affine inter mode

- Adaptive four and six parameter motion model for affine inter mode

In affine merge mode, the additional RDO is performed to select the candidate when the candidate number is two. In affine inter mode, additional affine motion estimations for CP0, CP1, and CP3 are performed for the six parameter motion model.

Mode decisions

This is identical to that in JEM.

In-loop filtering type selection and parameter estimation

This is identical to that in JEM.

Transforms and transform type selection

Primary Transforms and transformation type selection

For primary transform, the encoding procedure for determining one of four possible combinations is the same as JEM except that possible transform types of both horizontal and vertical directions are fixed to be DST7 and DCT8 among all intra prediction modes.

Secondary transforms and transform type selection

For secondary transform, the encoding procedure for determining secondary transform index is the same as that in JEM. Also transform type (kernel) selection for secondary transform part is not changed.

Quantization and quantization type selection

This is identical to that in JEM.

Entropy coding

This is identical to that in JEM.

Additional tools

N/A

Additional encoder optimization

This is identical to that in JEM.

Additional algorithmic description topic(s)

N/A

360°

Usage of projection format adaptation

N/A

360° Video specific encoding tools

N/A

Additional encoder optimization

N/A

HDR

Pre-processing

N/A

HDR specific encoding tools

N/A

Additional encoder optimization

N/A

Compression performance

SDR: Constraint set 1 configuration relative to HM anchor

Class SDR-A (RD-curves)

Figure 16 RD curves for FoodMarket4 (Constraint Set 1, compared to HM 16.16)

Figure 17 RD curves for CatRobot1 (Constraint Set 1, compared to HM 16.16)

Figure 18 RD curves for DaylightRoad2 (Constraint Set 1, compared to HM 16.16)

Figure 19 RD curves for ParkRunning3 (Constraint Set 1, compared to HM 16.16)

Figure 20 RD curves for CampfireParty2 (Constraint Set 1, compared to HM 16.16)

Class SDR-B (RD-curves)

Figure 21 RD curves for BQTerrace (Constraint Set 1, compared to HM 16.16)

Figure 22 RD curves for RitualDance (Constraint Set 1, compared to HM 16.16)

Figure 23 RD curves for MarketPlace (Constraint Set 1, compared to HM 16.16)

Figure 24 RD curves for BasketballDrive (Constraint Set 1, compared to HM 16.16)

Figure 25 RD curves for Cactus (Constraint Set 1, compared to HM 16.16)

Overall (BD-rate summary tables)

Table 6 Experimental result of the proposed model for Constraint Set 1(compared to HM16.16)

SDR: Constraint set 2 configuration relative to HM anchor

Class SDR-B

Figure 26 RD curves for BQTerrace (Constraint Set 2, compared to HM 16.16)

Figure 27 RD curves for RitualDance (Constraint Set 2, compared to HM 16.16)

Figure 28 RD curves for MarketPlace (Constraint Set 2, compared to HM 16.16)

Figure 29 RD curves for BasketballDrive (Constraint Set 2, compared to HM 16.16)

Figure 30 RD curves for Cactus (Constraint Set 2, compared to HM 16.16)

Overall

Table 7 Experimental result of the proposed model (Constraint Set 2)

SDR: Constraint set 1 configuration relative to JEM anchor

Class SDR-A

Figure 31 RD curves for FoodMarket4 (Constraint Set 1, compared to JEM 7.0)

Figure 32 RD curves for CatRobot1 (Constraint Set 1, compared to JEM 7.0)

Figure 33 RD curves for DaylightRoad2 (Constraint Set 1, compared to JEM 7.0)

Figure 34 RD curves for ParkRunning3 (Constraint Set 1, compared to JEM 7.0)

Figure 35 RD curves for Campfire (Constraint Set 1, compared to JEM 7.0)

Class SDR-B

Figure 36 RD curves for BQTerrace (Constraint Set 1, compared to JEM 7.0)

Figure 37 RD curves for RitualDance (Constraint Set 1, compared to JEM 7.0)

Figure 38 RD curves for MarketPlace (Constraint Set 1, compared to JEM 7.0)

Figure 39 RD curves for BasketballDrive (Constraint Set 1, compared to JEM 7.0)

Figure 40 RD curves for Cactus (Constraint Set 1, compared to JEM 7.0)

Overall

Table 8 Experimental result of the proposed model (Constraint Set 1, compared to JEM 7.0)

SDR: Constraint set 2 configuration relative to JEM anchor

Class SDR-B

Figure 41 RD Curves of BQTerrace (Constraint Set 2, compared to JEM 7.0)

Figure 42 RD Curves of RitualDance (Constraint Set 2, compared to JEM 7.0)

Figure 43 RD Curves of MarketPlace (Constraint Set 2, compared to JEM 7.0)

Figure 44 RD Curves of BasketballDrive (Constraint Set 2, compared to JEM 7.0)

Figure 45 RD Curves of Cactus (Constraint Set 2, compared to JEM 7.0)

Overall

Table 9 Experimental result of the proposed model (Constraint Set 2, compared to JEM 7.0)

HDR: Constraint set 1 configuration relative to HM anchor

N/A

Class HDR-A

N/A

Class HDR-B

N/A

Overall

N/A

HDR: Constraint set 1 configuration relative to JEM anchor

N/A

Class HDR-A

N/A

Class HDR-B

N/A

Overall

N/A

360° Video: Constraint set 1 configuration relative to HM anchor

Class 360

N/A

Overall

N/A

360° Video: Constraint set 1 configuration relative to JEM anchor

N/A

Class 360

N/A

Overall

N/A

Decoder complexity analysis

Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

The decoding time with YUV output for the proposed codec is reportedly shown in Table 10. It is measured to be about 6.4 and 5.8 times compared to those of HM16.16 for RA and LD configurations, respectively. Additionally, the decoding time without YUV output is reported in Table 11. It is measured to be approximately 8.0 and 6.7 times for RA and LD configurations, respectively.

Table 10 Decoding time with YUV output enabled

Table 11 Decoding time with YUV output disabled

Description of computing platform used to determine decoding times reported in section 5.1

In this proposal, a 64-bit executable is used for the simulations on Linux. The proposed model is tested taking advantage of multicore parallel processing. A brief hardware specification of the computing machine is like below.

- CPU : Intel Xeon E5-2699v3 2.3GHz

- RAM : 768 GB

- HDD : Network attached storage

Memory usage of decoder

Memory usage of the proposed decoder is provided in following tables. For both RA and LD configurations, it reports that the proposed model utilizes less memory than that of JEM.

Table 12 Decoder memory usage measurement for RA (in units of GB)

| Sequence | QP | Proposed | JEM | HM |
| --- | --- | --- | --- | --- |
| FoodMarket4 | 34 | 1.02 | 1.25 | 0.98 |
| CatRobot1 | 31 | 1.02 | 1.25 | 0.98 |
| DaylgithRoad2 | 31 | 1.02 | 1.25 | 0.98 |
| ParkRunning3 | 37 | 1.01 | 1.25 | 0.98 |
| CampfireParty2 | 32 | 1.01 | 1.25 | 0.98 |

Table 13 Decoder memory usage measurement for LD (in units of GB)

| Sequence | QP | Proposed | JEM | HM |
| --- | --- | --- | --- | --- |
| BQTerrace | 32 | 0.32 | 0.35 | 0.23 |
| RitualDance | 29 | 0.32 | 0.35 | 0.23 |
| MarketPlace | 34 | 0.32 | 0.35 | 0.23 |
| BasketballDrive | 30 | 0.31 | 0.35 | 0.23 |
| Cactus | 32 | 0.31 | 0.35 | 0.23 |

Complexity characteristics of decoder entropy decoding operation

N/A

Complexity characteristics of decoder inverse quantization

N/A

Complexity characteristics of decoder inverse transform operation

JEM NSST

L×log2N×N2      for storing θ                                                                                                                                                      N2                           for storing permutation matrix (where we need log2N2 bits for storing each elements

Multiplication Complexity: 4×L×log2N×N2

L is the number of the iterations which is 4 for N=8 in current JEM software.

An 8x8 NSST in JEM consists of 4 iterations, where each iteration it has 6 passes. Each of these passes has 128 multiplications. Applying the 8x8 NSST requires a total of 3072 multiplications.

RST

Memory Complexity:  R×N

Multiplication Complexity:  R×N

Where, N = 64 (8×8) and R = 16 (N×1/4)

In the proposed method RST, reduction factor of 1/4 was considered.

Table 14 Summary for N=8×8

|  | Memory Complexity (Bytes) | # of Multiplication | Multiple iterations |
| --- | --- | --- | --- |
| JEM NSST | 816 | 3072 | Yes (4-iterations) |
| RST | 1152 | 1152 | No |

* Multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output)

In additional to the memory requirements characterized by multiplications, it should be noted that JEM NSST needs to process 4×6 = 24 sequential steps to get the final results, whereas RST can get the output in one pass.

For CfP sequences in [1], following is the comparison result for RST with JEM anchor:

Table 15  Comparison result for RST with JEM anchor

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A | -0.15% | 0.05% | -0.24% |
| Class B | -0.03% | -0.59% | -0.39% |
| All | -0.09% | -0.27% | -0.32% |

The 8×8 RST outperforms JEM NSST as shown in Table 15 in spite that the required number of multiplications for 8×8 RST is much less than that for JEM NSST.

LGT

The number of multiplications required to process one 4×4 block with LGT amounts to (5 GRLs)×8×4 = 160. Each Givens rotation is performed with 4 multiplications. As JEM NSST consists of 8 GRLs, the proposed 4×4 LGT has 37.5% less computational load than JEM NSST.

Additional memory usage incurred by the 4×4 LGT includes information of both Givens rotations and permutations. Each permutation array element requires 4 bits, and each rotation θ is indexed with 8 bits, because θ is quantized value, (2πk/256), k = 0, 1, … , 255.

Table 16 Summary for N = 4×4

| Transform | Givens rotation | Permutation | Total |
| --- | --- | --- | --- |
| 4×4 JEM NSST | 512 bits | 64 bits | 576 bits |
| 4×4 LGT | 320 bits | 384 bits | 704 bits |

The increased memory usage of the 4×4 LGT can be regarded as the cost for reduction of computation.

Complexity characteristics of decoder-side motion refinement

In this section, the complexity characteristics of the MPCR scheme which incorporates the reordering and refinement of the candidate list is described. MPCR requires template matching at the decoder. And for motion compensation (MC) of template region, it is used 2-tap bilinear interpolation filter same as FRUC on JEM. Also, template region is restricted by 2 to reduce the memory (i.e., line buffer).

Complexity characteristics of decoder motion compensation

The complexity of the motion compensation in the proposed model is identical to that in JEM, as the 8-tap filter is used at 1/16 pixel accuracy. The number of pictures for the corresponding block of ATMVP is 1, while the one in JEM is used up to 4.

Complexity characteristics of decoder intra-frame prediction operation

In this section, the decoder complexity characteristics of linear interpolation intra prediction (LIP) is described. The proposed LIP method requires two additional processes; the generation of the opposite side reference sample array and the prediction from the array.

Complexity characteristics of decoder in-loop filtering operation

N/A

Complexity characteristics of additional decoder tools

N/A

Complexity characteristics of 360° video specific decoding tools

N/A

Complexity characteristics of HDR specific decoding tools

N/A

Degree of capability for decoder parallel processing

N/A

Encoder complexity analysis

Encoding time and measurement methodology

In the proposed model, parallel encoding is used to speed up the simulation. The encoding time is calculated by adding the encoding time of all parallelism segment and the subtracting the encoding times of the first I pictures which are overlapping in the previous segments. The encoding times for each sequence and each target bitrate are reported in Table 17. The encoding time for the proposed codec is measured to be about 15.2 and 19.1 times compared to those of HM16.16 for RA and LD configurations, respectively.

Table 17 Encoding time of the proposal compared to HM 16.16

Description of computing platform used to determine encoding times reported in section 6.1

In this proposal, a 64-bit executable is used for the simulations on Linux. The proposed model is tested taking advantage of multicore parallel processing. A brief hardware specification of the computing machine is like below.

- CPU : Intel Xeon E5-2699v3 2.3GHz

- RAM : 768 GB

- HDD : Network attached storage

Memory usage of encoder

Memory usage of the proposed encoder is provided in following tables. For both RA and LD configurations, it reportedly shows that the proposed model utilizes less memory than that of JEM.

Table 18 Encoder memory usage measurement for RA (in units of GB)

| Sequence | QP | Proposed | JEM | HM |
| --- | --- | --- | --- | --- |
| FoodMarket4 | 34 | 4.86 | 6.04 | 1.89 |
| CatRobot1 | 31 | 4.86 | 6.04 | 1.88 |
| DaylgithRoad2 | 31 | 4.86 | 6.04 | 1.88 |
| ParkRunning3 | 37 | 4.86 | 6.04 | 1.86 |
| CampfireParty2 | 32 | 4.86 | 6.04 | 1.85 |

Table 19 Encoder memory usage measurement for LD (in units of GB)

| Sequence | QP | Proposed | JEM | HM |
| --- | --- | --- | --- | --- |
| BQTerrace | 32 | 0.71 | 0.92 | 0.26 |
| RitualDance | 29 | 0.71 | 0.92 | 0.26 |
| MarketPlace | 34 | 0.71 | 0.92 | 0.26 |
| BasketballDrive | 30 | 0.71 | 0.92 | 0.26 |
| Cactus | 32 | 0.71 | 0.92 | 0.26 |

Complexity characteristics of encoder intra-frame prediction type selection

In this section, the encoder complexity characteristics of linear interpolation intra prediction (LIP) is described. In the rough mode decision (RMD) stage, first N intra prediction modes are selected. Then, 6 MPM candidate modes based on LIP are added to the candidate list for the full rate distortion optimization (Full RDO) stage.

Complexity characteristics of encoder motion estimation and motion segmentation selection

General

In the affine motion vector prediction, affine motion estimation is added for the six-parameter affine motion model. In the affine merge mode, an additional RDO process is performed to select a candidate when the number of neighboring affine coded blocks is more than one. For the other motion prediction tools in the proposed model, they are identical to those in JEM.

Complexity characteristics of usage of decoder-side motion refinement

Complexity of MPCR on encoder is same as that at the decoder.

Complexity characteristics of encoder transforms and transform type selection

There is no meaningful change in terms of the encoding time for transform part compared to the encoding time in JEM.

Complexity characteristics of encoder quantization and quantization type selection

N/A

Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

N/A

Complexity characteristics of encoder entropy coding type selection

N/A

Complexity characteristics of additional encoder tools

In this contribution, to represent ternary tree in the QTBTT structure is added to two partitioning types compared to QTBT structure in JEM. In encoder, it is required to conduct RDO process for the added partitioning types to determine the final block partitioning type. It yields increment of encoding run-time.

Complexity characteristics of 360° video specific encoding tools

N/A

Complexity characteristics of HDR specific encoding tools

N/A

Degree of capability for encoder parallel processing

N/A

Algorithmic characteristics

Random access characteristics

In order to meet target bitrates in [1], the following coding conditions are used to generate bitstreams.

Bit-depth : 10 bits

I period : 64 (60 fps), 48 (50 fps), 32 (30 fps)

In order to generate bitstreams, parallel coding scheme [6] is employed when using Constraint Set 1 to speed up the process. Also, a three-step scheme is employed to match the target bitrate. The first step is to encode the selected sequences with a range of integer QP values to find the nearest two integer QP values which covers the target bitrate. As described in [1], a one-time change of the quantization parameter from value QP to value QP+1 is applied to achieve better accuracy. A rough search (step 2) followed by a finer search (step 3) is then performed to match the target bitrate. The integer QP values and the frame numbers of each bitrate and each sequence for the proposed model are listed in Table 20. In Table 20, the frame numbers written as N/A mean that there is no QP increment to meet the target bitrate.

Table 20 Integer QP and QP increment frame of each sequence and target rate (Constraint 1)

|  | QP, Frame number for QP+1 |  |  |  |
| --- | --- | --- | --- | --- |
| Sequence | R1 | R2 | R3 | R4 |
| FoodMarket4 | 43, 192 | 40, 320 | 36, 38 | 32, 256 |
| CatRobot1 | 39, 140 | 35, 64 | 32, 412 | 29, N/A |
| DaylightRoad2 | 39, 256 | 36, 544 | 32, 108 | 29, 120 |
| ParkRunning3 | 46, 352 | 42, 140 | 39, 352 | 36, 418 |
| CampfireParty2 | 42, 264 | 38, 18 | 34, 61 | 30, 45 |
| BQTerrace | 41, 4 | 39, N/A | 35, 320 | 33, N/A |
| RitualDance | 39, 242 | 35, 210 | 32, 384 | 28, 329 |
| MarketPlace | 41, 128 | 39, 535 | 36, 449 | 33, 476 |
| BasketballDrive | 39, 318 | 36, 370 | 32, 208 | 28, 104 |
| Cactus | 42, 494 | 38, 192 | 35, 208 | 32, N/A |

Delay characteristics

In order to meet target bitrates in [1], the following coding conditions are used to generate bitstreams.

Bit-depth : 10 bits

I period : One (at the first) intra slice in bitstreams

Sequential coding is applied for Constraint Set 2. Also, as described in 7.1, the three-step scheme is employed for matching the target bitrate. The integer QP values and the frame numbers of each bitrate and each sequence for the proposed model are listed in Table 21.

Table 21 Integer QP and QP increment frame of each sequence and target rate (Constraint 2)

|  | QP, Frame number for QP+1 |  |  |  |
| --- | --- | --- | --- | --- |
| Sequence | R1 | R2 | R3 | R4 |
| BQTerrace | 40, 544 | 37, 106 | 34, 24 | 32, 90 |
| RitualDance | 41, 475 | 37, 454 | 33, 144 | 29, 137 |
| MarketPlace | 42, 279 | 40, 519 | 37, 371 | 34, 450 |
| BasketballDrive | 41, 471 | 38, 475 | 34, 442 | 30, 488 |
| Cactus | 42, 435 | 38, 89 | 35, 8 | 32, 367 |

Additional characteristics discussion topic(s)

N/A

Software implementation description

The integrated software is written in C++ programing language, without any additional assembly code and is single-threaded. It is scratched from JEM code (JEM 7.0) with removing irrelevant variables and functions, and all tools that are described above are then integrated into this code base, including an implementation of MPCR in replacement of FRUC.

Highlighted aspects discussion

No specific comment.

Closing remarks

This proposal introduces a video coding system that achieves higher compression efficiency minimizing complexity increase compared to the state-of-the-art HEVC standard. The major novelties of the proposal are in the area of block partitioning including ternary tree, linear intra prediction, simplified primary transform and secondary transform, modified affine motion model, and modified motion predictor candidate refinement scheme. We assert that the balance of complexity and compression efficiency of the proposed system makes it well suited to be the baseline for developing the next generation video coding standard with the JVET.

Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex 1: Questionnaire

Proponents of CfP responses are required to answer the codec level questions, and are also encouraged to fill out the optional tool level questions in addition to the complexity analysis of their proposals. If some question is not applicable to your proposal, you may express that as well.

Codec level questions

- Maximum number of reference frames used by encoder

- Maximum coding unit (e.g. CTU in HEVC)

- Minimum and maximum transform block size

- Minimum and maximum intra prediction block

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

- List all tools that use non-rectangular (e.g. triangle) partitions

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

- Indicate whether luma and chroma use separate partitioning structures

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

Tool level questions

- Partitioning structure

  - Whether the prediction unit partitioning is different from transform unit partitioning

  - Maximum tree depth

- Entropy coding

  - Total number of context models

  - Memory storage size for each context model

  - Range table size if table look up is used for range update

  - Initialization table size

  - Other memory associated with context model adaptation

- Control data derivation process

  - Temporal motion data storage granularity

  - Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-predictive reference picture are used, count this picture as 2)

- Decoder-side motion refinement

  - Line buffer size in terms of motion vector block lines (assuming horizontal motion vector block size is 4 luma samples)

  - Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement

  - Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines

  - List all the filters used for intra prediction

  - Indicate whether there are cross-component dependencies in intra prediction reconstruction

- Inter prediction

  - Motion vector accuracy

  - Storage size for a motion vector component

  - List all the interpolation filters used for luma/chroma motion compensation

- Quantization

  - Storage size associated with quantization

- Transforms

- List all the combinations of transform block size and transform type for primary transforms

- List all the combinations of transform block size and transform type for secondary transforms

- List all the non-separable transforms used

- Indicate whether transforms can be fully/partially implemented using direct matrix multiply

- List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output)

- In-loop filtering

- List line buffer size in terms of luma and chroma lines (if any)

- List all the filters used for in-loop filtering

- Minimum processing unit size

- “tool-off” and “tool-on” tests

- List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools (see Table 22 below for test procedure)

Table 22 Test procedure of “tool-off” and “tool-on” tests for a CfP response

| step | “tool-off” test | “tool-on” test |
| --- | --- | --- |
| 1 | Identify what you consider major tools in a CfP response that contribute to compression efficiency gain, and create a test candidates list |  |
| 2 | Turn on all the tools in the test candidates list, run the CfP response software and compute PSNRs, bit rates, and run times | Turn off all the tools in the test candidates list, run the CfP response software, compute PSNRs, bit rates, and run times, and compute the BD-rate and run-time numbers relative to the HM anchor |
| 3 | Turn off a tool in the test candidates list, and keep all the other tools in the test candidates list on, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools on | Turn on a tool in the test candidates list, and keep all the other tools in the test candidates list off, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools of the test candidates list off (i.e. step 2) |
| 4 | Repeat step 3 until all the tools in the test candidates list are tested | Repeat step 3 until all the tools in the test candidates list are tested |
| 5 | Compose the test data |  |

Annex 2: Example of filling questionnaire HM vs. Proposed

This annex is meant as an example how the questions could be answered. If some of the aspects are not applicable to an algorithm, the corresponding lines may not need to be filled. You may also add lines at the end of the table to compare additional aspects of a proposal versus HM.

Codec level Q&A

| Codec Level Q&A |  | HM | Proposed |
| --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/128×128 chroma 2×2/64×64 |
| 4 | Minimum and maximum intra prediction block | luma 4×4 chroma 4×4 | luma 4×4 chroma 2×2 |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | template matching in MPCR, LIC |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | n/a | 64-bit arithmetic for LIC, BIO and LMC |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | no | Yes, in I-slices |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation |

Tool level Q&A

Category 1. Partitioning structure

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 1. Partitioning structure |  | HM | Proposed |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | Yes, PU and TU partitioning for a CU can differ | No |
| 1.b | Maximum tree depth | CU maximum depth 4 TU maximum depth 3 PU maximum depth 1 | QT maximum depth 5 BTT maximum depth 3 |

Category 2. Entropy coding

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 2. Entropy coding |  | HM | Proposed |
| 2.a | Total number of context models | 186 | 339 |
| 2.b | Memory storage size for each context model | 8-bit | 30-bit |
| 2.c | Range table size if table look up is used for range update | LPSTable[64×4×8bits]; RenormTable[32× 3bits] | LPSTable[512×64×9bits]; RenormTable[128×3bits] |
| 2.d | Initialization table size | 3×186×8 bits | 3×339×8 + 339×15×5 bits |
| 2.e | Other memory associated with context model adaptation | LPS transition table: 64*6 bits MPS transition table: 64*6 bits | n/a |

Category 3. Control data derivation process

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 3. Control data derivation process |  | HM | Proposed |
| 3.a | Temporal motion data storage granularity | 16×16 | 4×4 |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 1 | 1 |

Category 4. Decoder-side motion refinement

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 4. Decoder-side motion refinement |  | HM | Proposed |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | n/a | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - MPCR : SAD between template region of the current block and template region of the L0/L1 reference block. Minimum processing unit is 4×4 as CU not sub-CU. The number of SADs per processing unit is not constrained. Bi-linear interpolation filter for MC on template matching is used. Line buffer size is 2. |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | n/a | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - MPCR not constrained |

Category 5. Intra prediction

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 5. Intra prediction |  | HM | Proposed |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 1, chroma 1 | luma 4, chroma 2 |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, Strong intra smoothing filter: 2-tap bilinear filter (only for 32×32 luma TU) Interpolation: 2×32×5 bits (bi-linear 32 phases) | Reference sample smoothing: {1,2,1} Strong intra smoothing filter: 2-tap bilinear filter (for larger than 32×32 luma block) Boundary filter:2×2×4 bits (2 taps, 2 sets (modes 2 and 66)) 3×16×4 bits (3 taps, 16 sets (from mode 3 to mode 10 and from mode 58 to mode 65)) Interpolation for conventional intra prediction : 2×4×32×8 bits (4 taps, 2 sets, 32 phases) Interpolation for LIP 2×32×5 bits (bi-linear 32 phases) |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | n/a | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) LAP (Cb dependes on Y, and Cr depends on Cb&Y) |

Category 6. Inter prediction

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 6. Inter prediction |  | HM | Proposed |
| 6.a | Motion vector accuracy | luma ¼ pel chroma 1/8 pel | luma 1/16 pel chroma 1/32 pel |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits |

Category 7. Quantization

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 7. Quantization |  | HM | Proposed |
| 7.a | Storage size associated with quantization | n/a | n/a |

Category 8. Transforms

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 8. Transforms |  | HM | Proposed |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | - 8-bit 4×4, 8×8, 16×16 and 32×32 DCT-II, - 8-bit 4×4 DST VII | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) blocks for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) blocks for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) blocks for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-VIII), only applied to CTUs up to 32×32, transform size combinations: - (4, 8, 16, 32) × (4, 8, 16, 32) blocks for luma CTUs |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | n/a | (2×2+33×3) 4×4 non-separable layered Givens Transform (LGT), (2×2+33×3) 8×8 non-separable Reduced Secondary Transform (RST) |
| 8.c | List all the non-separable transforms used | n/a | 4×4 LGT / 8×8 RST |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | partial factorization equivalent to matrix multiplication | EMT : factorized form not equivalent to matrix multiplication 4×4 LGT: factorized form not equivalent to matrix multiplication 8×8 RST: fully implemented using direct non-square matrix multiply |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | n/a | n/a |

Category 9. In-loop filtering

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 9. In-loop filtering |  | HM | Proposed |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma |
| 9.c | Minimum processing unit size | Deblocking (luma: 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 4×4 filter decision) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) |

Category 10. “tool-off” and “tool-on” tests

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 10. “tool-off” and “tool-on” tests |  | HM | Proposed |
| 10.a | List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools | None | QTBTT, MPCR, AFFINE, LIP, EMT, 4×4 LGT + 8×8 RST See Table 2 below for an example of “tool-off” and “tool-on” tests |

Table 23 Summary of “tool-off” and “tool-on” tests for the proposed model (Random Access)

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| QTBTT | 2.4% | 4.0% | 3.5% | 50% | 98% | -2.3% | -3.2% | -3.9% | 195% | 103% |
| LIP | 0.6% | 0.0% | 0.6% | 95% | 96% | -0.6% | -1.0% | -1.1% | 105% | 101% |
| AFFINE | 2.6% | 2.0% | 1.6% | 89% | 97% | -3.0% | -2.1% | -2.2% | 120% | 108% |
| MPCR | 2.3% | 2.0% | 2.2% | 93% | 83% | -2.7% | -2.0% | -2.3% | 106% | 121% |
| EMT | 0.7% | -0.8% | −0.8% | 96% | 100% | −1.7% | 0.4% | −0.5% | 104% | 100% |
| 4×4 LGT + 8×8 RST | 1.7% | 4.2% | 4.4% | 92% | 100% | −2.3% | −4.7% | −5.1% | 106% | 100% |

Table 24 Summary of “tool-off” and “tool-on” tests for the proposed model (Low Delay)

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| QTBTT | 2.4% | 4.6% | 3.8% | 39% | 97% | -2.6% | -4.2% | -3.5% | 250% | 105% |
| LIP | 0.3% | -0.4% | -0.3% | 98% | 100% | -0.2% | -0.7% | -0.5% | 102% | 100% |
| AFFINE | 3.5% | 2.0% | 2.3% | 82% | 91% | -3.5% | -2.8% | -2.7% | 142% | 119% |
| MPCR | 1.7% | 1.6% | 1.4% | 79% | 81% | -1.9% | -1.9% | -1.7% | 133% | 134% |
| EMT | 0.5% | -1.9% | -1.5% | 98% | 101% | -1.1% | -0.2% | 0.8% | 105% | 101% |
| 4×4 LGT + 8×8 RST | 1.3% | 3.8% | 4.1% | 95% | 101% | -1.7% | -5.2% | -5.2% | 106% | 101% |

Reference

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-H1002, 8th meeting, Macao, CN, 18-24 Oct. 2017.

- H.-C. Chuang, J. Chen, M. Karczewicz, K. Choi, E. Alshina, “AHG4: SDR anchor generation for the Draft Joint Call for Proposals,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-H0060, 8th meeting, Macao, CN, 18-24 Oct. 2017.

- JEM Software, https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.0.

- Ravi K. Chivukula, and Yuriy A. Reznik, “Fast computing of discrete cosine and sine transforms of types VI and VII,” SPIE Optical Engineering+ Applications, 2011, pp. 813505-313505

- S. Winograd, “On the multiplicative complexity of the discrete Fourier transform,” Adv. Math., 1979, pp. 83-117

- X. Ma, H. Chen, H. Yang, “Simplification of the common test condition for fast simulation,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-B0036, 2nd Meeting: San Diego, USA, 20–26 Feb. 2016.

[end of document]