| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0033 |
| --- | --- |

| Title: | Description of 360° video coding technology proposal by Zhejiang University |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yule Sun, Xuchang Huangfu, Ruidi Zheng, Bin Wang, Lu Yu Hangzhou, Zhejiang 310027,China | Tel: Email: | +86 57187953107 sunyule@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

This proposal describes the Zhejiang University’s response to the joint Call for Proposal (CfP) on video compression with capability beyond HEVC in the 360º video category. A new projection format with padding called parallel-to-axis uniform cubemap projection (PAU) is proposed and the format related information are described as SEI message. The proposed format is integrated into 360Lib-5.0 and JEM 7.0, and the coding technology directly uses the algorithm of JEM. Compared with HM and JEM anchor (PERP coded with HM 16.16 and JEM 7.0), this proposed format based on JEM reduces 30.6% and 9.0% (E2E WS-PSNR) bit rate for Y component respectively.

- Contents

Abstract	i

Contents	ii

1	Introduction	1

2	Decoder algorithm description	1

2.1	General algorithm description	1

2.1.1	Bitstream structure	1

2.1.2	Entropy decoding	1

2.1.3	Conversion process from entropy decoded data to symbols	1

2.1.4	Decoder-side motion refinement	1

2.1.5	Inverse quantization	1

2.1.6	Inverse transforms	2

2.1.7	Motion compensation	2

2.1.8	Intra prediction	2

2.1.9	In-loop filtering	2

2.1.10	Additional tools	2

2.1.11	Additional algorithmic description topic(s)	2

2.2	360° Video	2

2.2.1	Projection format (including padding)	2

2.2.2	Quantization handling	2

2.2.3	360° video specific decoding tools	2

2.3	HDR	2

2.3.1	Post-processing	2

2.3.2	Quantization handling	2

2.3.3	HDR specific decoding tools	2

3	Encoder algorithm description	3

3.1	General algorithm description	3

3.1.1	Rate-distortion optimization	3

3.1.2	Partitioning structure selection	3

3.1.3	Intra prediction mode estimation and type selection	3

3.1.4	Motion estimation and motion segment selection	3

3.1.5	Mode decisions	3

3.1.6	In-loop filtering type selection and parameter estimation	3

3.1.7	Transforms and transform type selection	3

3.1.8	Quantization and quantization type selection	3

3.1.9	Entropy coding	3

3.1.10	Additional tools	3

3.1.11	Additional encoder optimization	4

3.1.12	Additional algorithmic description topic(s)	4

3.2	360°	4

3.2.1	Usage of projection format adaptation	4

3.2.2	360° Video specific encoding tools	4

3.2.3	Additional encoder optimization	4

3.3	HDR	4

3.3.1	Pre-processing	4

3.3.2	HDR specific encoding tools	4

3.3.3	Additional encoder optimization	4

4	Compression performance	4

4.1	SDR: Constraint set 1 configuration relative to HM anchor	4

4.1.1	Class SDR-A (RD-curves)	4

4.1.2	Class SDR-B (RD-curves)	4

4.1.3	Overall (BD-rate summary tables)	5

4.2	SDR: Constraint set 2 configuration relative to HM anchor	5

4.2.1	Class SDR-B	5

4.2.2	Overall	5

4.3	SDR: Constraint set 1 configuration relative to JEM anchor	5

4.3.1	Class SDR-A	5

4.3.2	Class SDR-B	5

4.3.3	Overall	5

4.4	SDR: Constraint set 2 configuration relative to JEM anchor	5

4.4.1	Class SDR-B	5

4.4.2	Overall	5

4.5	HDR: Constraint set 1 configuration relative to HM anchor	5

4.5.1	Class HDR-A	5

4.5.2	Class HDR-B	5

4.5.3	Overall	5

4.6	HDR: Constraint set 1 configuration relative to JEM anchor	6

4.6.1	Class HDR-A	6

4.6.2	Class HDR-B	6

4.6.3	Overall	6

4.7	360° Video: Constraint set 1 configuration relative to HM anchor	6

4.7.1	Class 360	6

4.7.2	Overall	6

4.8	360° Video: Constraint set 1 configuration relative to JEM anchor	6

4.8.1	Class 360	6

4.8.2	Overall	6

5	Decoder complexity analysis	6

5.1	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16	6

5.2	Description of computing platform used to determine decoding times reported in section 5.1	6

5.3	Memory usage of decoder	7

5.4	Complexity characteristics of decoder entropy decoding operation	7

5.5	Complexity characteristics of decoder inverse quantization	7

5.6	Complexity characteristics of decoder inverse transform operation	7

5.7	Complexity characteristics of decoder-side motion refinement	7

5.8	Complexity characteristics of decoder motion compensation	7

5.9	Complexity characteristics of decoder intra-frame prediction operation	7

5.10	Complexity characteristics of decoder in-loop filtering operation	7

5.11	Complexity characteristics of additional decoder tools	7

5.12	Complexity characteristics of 360° video specific decoding tools	7

5.13	Complexity characteristics of HDR specific decoding tools	7

5.14	Degree of capability for decoder parallel processing	7

6	Encoder complexity analysis	8

6.1	Encoding time and measurement methodology	8

6.2	Description of computing platform used to determine encoding times reported in section 6.1	8

6.3	Memory usage of encoder	8

6.4	Complexity characteristics of encoder intra-frame prediction type selection	8

6.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	8

6.5.1	General	8

6.5.2	Complexity characteristics of usage of decoder-side motion refinement	8

6.6	Complexity characteristics of encoder transforms and transform type selection	8

6.7	Complexity characteristics of encoder quantization and quantization type selection	8

6.8	Complexity characteristics of encoder in-loop filtering type selection and parameter estimation	9

6.9	Complexity characteristics of encoder entropy coding type selection	9

6.10	Complexity characteristics of additional encoder tools	9

6.11	Complexity characteristics of 360° video specific encoding tools	9

6.12	Complexity characteristics of HDR specific encoding tools	9

6.13	Degree of capability for encoder parallel processing	9

7	Algorithmic characteristics	9

7.1	Random access characteristics	9

7.2	Delay characteristics	9

7.3	Additional characteristics discussion topic(s)	9

8	Software implementation description	9

9	Highlighted aspects discussion	9

10	Closing remarks	9

11	Patent rights declaration(s)	9

12	Annex 1: Questionnaire	10

12.1	Codec level questions	10

12.2	Tool level questions	11

13	Annex 2: Example of filling questionnaire HM vs. JEM	14

13.1	Codec level Q&A	14

13.2	Tool level Q&A	15

13.2.1	Category 1. Partitioning structure	15

13.2.2	Category 2. Entropy coding	15

13.2.3	Category 3. Control data derivation process	15

13.2.4	Category 4. Decoder-side motion refinement	16

13.2.5	Category 5. Intra prediction	16

13.2.6	Category 6. Inter prediction	17

13.2.7	Category 7. Quantization	17

13.2.8	Category 8. Transforms	17

13.2.9	Category 9. In-loop filtering	18

13.2.10	Category 10. “tool-off” and “tool-on” tests	19

Introduction

A Call for Proposals (CfP) was issued jointly by ITU-T SG16 Q.6 (VCEG) and ISO/IEC JTC 1/SC 29/W G11 (MPEG) [1].This is the description of 360 video coding technology proposal by Zhejiang University. Figure 1 shows the simplified processing chain for 360° video. High fidelity ERP is converted to low resolution projection format, which is processed by codec and then converted back to high fidelity ERP. The end-to-end quality is measured between two high fidelity ERP. In this contribution, a new projection format with padding called parallel-to-axis uniform cubemap projection (PAU) is proposed and the format related information are described in SEI message. The codec engine is based on JEM 7.0 and the only difference is that format related SEI messages are added in this proposal.

Figure 1 – Simplified processing chain for 360° video

Decoder algorithm description

General algorithm description

Bitstream structure

A new SEI Payload Type item is defined for this new PAU-CMP format. Additional to that, two flags are defined to respectively indicate each row of 3-faces either being horizontally parallel-to-axis uniform or vertically. And number of padding pixels between the two rows of faces are also indicated in the bit stream. All these information are not used for decoding.

Entropy decoding

The same as JEM.

Conversion process from entropy decoded data to symbols

The same as JEM.

Intra prediction parameters

Motion parameters

Quantization parameters

Transform coefficients

In-loop filter parameters

Decoder-side motion refinement

The same as JEM.

Inverse quantization

The same as JEM.

Inverse transforms

The same as JEM.

Motion compensation

The same as JEM.

Intra prediction

The same as JEM.

In-loop filtering

The same as JEM.

Additional tools

The same as JEM.

Additional algorithmic description topic(s)

The same as JEM.

360° Video

Projection format (including padding)

Cube map projection (CMP) is one typical projection format as its simple representation of sphere and efficient compatibility of existing codec for each face. However the sampling density near the face center and corner still differs a lot, which restricts the representation efficiency of CMP. To make the sample density more uniform, adjusted cubemap projection (ACP), equal-angular cubemap (EAC) [2] and Hybrid cubemap projection (HCP) [3] were proposed by adjusting the sampling density on each cube face. All ACP, EAC and HCP have the same adjusting function of sampling rate in horizontal and vertical directions, which improve the coding efficiency a lot compared with CMP.

This response proposes a PAU, which is similar to ACP, EAC and HCP. The difference is that the sampling rate adjusting function of PAU in horizontal and vertical directions are different, which can make the sampling distribution on cube face more uniform. For PAU, one direction of cube face is first resampled to achieve approximately equal-angle sampling. Then another orthometric direction is equal-angle sampling based on the resampling position in first direction. In this response, vertical direction is first determined. Detailed mapping adjustments are described below.

The PAU has the same face definition as CMP and frame packing is shown in the left figure of Figure 2. Assuming (u’, v’) is defined in a CMP face, and (u, v) is the corresponding point in a PAU face. Each face is defined in the range of [-1, 1]. The equations (1)(2) and (3) are the forward adjustment function from CMP to PAU. Vertical direction is first adjusted by (1). For the horizontal direction, sampling adjustments of u are related to vertical sampling v, which is expressed in (2) and (3).

v=tanv'π4.0                                                             (1)

α=tan-1v                                                                (2)

u=1cos⁡(α)*tanu'*tan-1cosα                (3)

For the backward adjustment function, the equations (4)(5) and (6) are used to map PAU to CMP.

v'=4.0π*tan-1v                                            (4)

β=tan-1v                                                      (5)

u'=tan-1u*cosβtan-1cosβ                                    (6)

Figure 2. Frame packing of PAU with (left) and without padding (right).

To avoid obvious seam artifact near the discontinuous face boundary, padding is performed on the vertical face boundary, which is shown in the right figure of Figure 2. The gray areas are the padding pixels. In this proposal, padding width is 3 and the value of pixel are copied from the nearest pixel in vertical direction inside each face. The content of each face is scaled on an area of (face width) x (face height-3). So [0, face height-3] is scaled to [-1, 1] in vertical direction when padding width is 3.

Quantization handling

Quantization settings is kept static except that a one-time change of the quantization parameter from value QP to value QP+1 is used to meet the target bit rate.

360° video specific decoding tools

No special decoding tools.

HDR

n/a

Post-processing

Quantization handling

HDR specific decoding tools

Encoder algorithm description

General algorithm description

The same as JEM.

Rate-distortion optimization

Distortion metrics

Rate-distortion dependency models

Partitioning structure selection

Intra prediction mode estimation and type selection

Motion estimation and motion segment selection

General

Usage of decoder-side motion refinement

Mode decisions

In-loop filtering type selection and parameter estimation

Transforms and transform type selection

Quantization and quantization type selection

Entropy coding

Additional tools

Additional encoder optimization

Additional algorithmic description topic(s)

360°

Usage of projection format adaptation

No adaptation of projection format.

360° Video specific encoding tools

No specific encoding tools.

Additional encoder optimization

No additional encoder optimization.

HDR

n/a

Pre-processing

HDR specific encoding tools

Additional encoder optimization

Compression performance

SDR: Constraint set 1 configuration relative to HM anchor

n/a

Class SDR-A (RD-curves)

Class SDR-B (RD-curves)

Overall (BD-rate summary tables)

SDR: Constraint set 2 configuration relative to HM anchor

n/a

Class SDR-B

Overall

SDR: Constraint set 1 configuration relative to JEM anchor

n/a

Class SDR-A

Class SDR-B

Overall

SDR: Constraint set 2 configuration relative to JEM anchor

n/a

Class SDR-B

Overall

HDR: Constraint set 1 configuration relative to HM anchor

n/a

Class HDR-A

Class HDR-B

Overall

HDR: Constraint set 1 configuration relative to JEM anchor

n/a

Class HDR-A

Class HDR-B

Overall

360° Video: Constraint set 1 configuration relative to HM anchor

Class 360

Overall

|  | E2E WS-PSNR |  |  | E2E SPSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Balboa | -39.5% | -47.0% | -51.4% | -39.5% | -47.1% | -51.4% |
| Chairlift | -43.9% | -56.8% | -52.9% | -43.8% | -56.8% | -52.8% |
| KiteFlite | -23.1% | -51.5% | -59.4% | -23.2% | -51.4% | -59.4% |
| Harbor | -25.9% | -52.1% | -52.6% | -25.7% | -52.1% | -52.7% |
| Trolley | -21.0% | -39.4% | -45.9% | -21.0% | -39.5% | -45.8% |
| Average 360 | -30.7% | -49.4% | -52.4% | -30.6% | -49.4% | -52.4% |

360° Video: Constraint set 1 configuration relative to JEM anchor

Class 360

Overall

|  | E2E WS-PSNR |  |  | E2E SPSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Balboa | -6.3% | -8.0% | -7.9% | -6.3% | -8.1% | -7.8% |
| Chairlift | -19.2% | -11.2% | -12.7% | -19.1% | -11.1% | -12.7% |
| KiteFlite | -6.8% | -7.3% | -15.3% | -6.7% | -7.2% | -15.2% |
| Harbor | -6.9% | -11.8% | -11.2% | -6.7% | -11.9% | -11.5% |
| Trolley | -6.0% | -5.9% | -12.4% | -5.9% | -5.9% | -12.4% |
| Average 360 | -9.0% | -8.8% | -11.9% | -9.0% | -8.8% | -11.9% |

Decoder complexity analysis

Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

| Average Dec Time[%] | 885% |
| --- | --- |

Description of computing platform used to determine decoding times reported in section 5.1

64b executables

Intel (R) Xeon (R) CPU E5-2667 V4 @ 3.2G Hz 3.2G Hz (2 CPU)

Multicore parallel processing

RAM: 128G

Hard drive characteristics: Dell MD36xxf SCSI Disk Device (2T)

Memory usage of decoder

Decoder only: 1.35G

Convert only: 1.04G

Decoder and Convert (Combine them together): 1.83G

Complexity characteristics of decoder entropy decoding operation

The same as JEM.

Complexity characteristics of decoder inverse quantization

The same as JEM.

Complexity characteristics of decoder inverse transform operation

The same as JEM.

Complexity characteristics of decoder-side motion refinement

The same as JEM.

Complexity characteristics of decoder motion compensation

The same as JEM.

Complexity characteristics of decoder intra-frame prediction operation

The same as JEM.

Complexity characteristics of decoder in-loop filtering operation

The same as JEM.

Complexity characteristics of additional decoder tools

The same as JEM.

Complexity characteristics of 360° video specific decoding tools

The same as JEM.

Complexity characteristics of HDR specific decoding tools

The same as JEM.

Degree of capability for decoder parallel processing

The same as JEM.

Encoder complexity analysis

Encoding time and measurement methodology

Compared to HM16.16:

| Average Enc Time[%] | 508% |
| --- | --- |

Description of computing platform used to determine encoding times reported in section 6.1

64b executables

Intel (R) Xeon (R) CPU E5-2667 V4 @ 3.2G Hz 3.2G Hz (2 CPU)

Multicore parallel processing

RAM: 128G

Hard drive characteristics: Dell MD36xxf SCSI Disk Device (2 T)

Memory usage of encoder

Encoder: 8.6G

Complexity characteristics of encoder intra-frame prediction type selection

The same as JEM.

Complexity characteristics of encoder motion estimation and motion segmentation selection

The same as JEM.

General

Complexity characteristics of usage of decoder-side motion refinement

Complexity characteristics of encoder transforms and transform type selection

The same as JEM.

Complexity characteristics of encoder quantization and quantization type selection

The same as JEM.

Complexity characteristics of encoder in-loop filtering type selection and parameter estimation

The same as JEM.

Complexity characteristics of encoder entropy coding type selection

The same as JEM.

Complexity characteristics of additional encoder tools

The same as JEM.

Complexity characteristics of 360° video specific encoding tools

The same as JEM.

Complexity characteristics of HDR specific encoding tools

The same as JEM.

Degree of capability for encoder parallel processing

The same as JEM.

Algorithmic characteristics

Random access characteristics

The same as JEM.

Delay characteristics

The same as JEM.

Additional characteristics discussion topic(s)

The same as JEM.

Software implementation description

The software is written in C++ and based on 360Lib 5.0 and JEM 7.0.

Highlighted aspects discussion

n/a

Closing remarks

Parallel-to-axis uniform cubemap projection (PAU) with padding is proposed and the format related information are described as SEI messages. The proposed format is integrated into 360Lib-5.0 and JEM 7.0, and the coding technology directly uses the algorithm of JEM. Compared with HM and JEM anchor (PERP coded with HM 16.16 and JEM 7.0), this proposed format based on JEM achieves 30.6% and 9.0% (E2E WS-PSNR) average bit rate saving for Y component respectively.

Reference

[1] A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, JVET H1002, Oct.2017, Macao, CN.

[2] Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib”, JVET H1004, Oct.2017, Macao, CN.

[3] P. Hanhart, X Xiu, F. Duanmu, Y, He, Y, Ye, “InterDigital’s Response to the 360º Video Category in Joint Call for Evidence on Video Compression with Capability beyond HEVC”, JVET-G0024, July. 2017, Torino, IT.

Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex 1: Questionnaire

Proponents of CfP responses are required to answer the codec level questions, and are also encouraged to fill out the optional tool level questions in addition to the complexity analysis of their proposals. If some question is not applicable to your proposal, you may express that as well.

Codec level questions

- Maximum number of reference frames used by encoder

4

- Maximum coding unit (e.g. CTU in HEVC)

luma 128×128

chroma 64×64

- Minimum and maximum transform block size

luma 4×4/128×128

chroma 2×2/64×64

- Minimum and maximum intra prediction block

luma 4×4

chroma 2×2

- Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length

luma 4×4 (uni/bi-pred), 8-tap

Chroma 2×2 (uni/bi-pred), 4-tap

- List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category)

template matching in FRUC, LIC

- List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples)

64-bit arithmetic for LIC, BIO and LMC

- List all tools that use non-rectangular (e.g. triangle) partitions

n/a

- Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa)

n/a

- Indicate whether luma and chroma use separate partitioning structures

Yes, in I-slices

- Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes)

Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation

- If the coded picture resolution is different from the input picture resolution, list the coded picture resolution for those sequences (e.g. 360° video, input is 8K ERP, coded video is in 4K CMP converted from the 8K input)

360 category: the resolution of all coded sequence is 3552x2368 (4K).

Tool level questions

The same as JEM.

- Partitioning structure

  - Whether the prediction unit partitioning is different from transform unit partitioning

  - Maximum tree depth

- Entropy coding

  - Total number of context models

  - Memory storage size for each context model

  - Range table size if table look up is used for range update

  - Initialization table size

  - Other memory associated with context model adaptation

- Control data derivation process

  - Temporal motion data storage granularity

  - Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-predictive reference picture are used, count this picture as 2)

- Decoder-side motion refinement

  - Line buffer size in terms of motion vector block lines (assuming horizontal motion vector block size is 4 luma samples)

  - Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement

  - Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size

- Intra prediction

  - Line buffer size in terms of luma and chroma sample lines

  - List all the filters used for intra prediction

  - Indicate whether there are cross-component dependencies in intra prediction reconstruction

- Inter prediction

  - Motion vector accuracy

  - Storage size for a motion vector component

  - List all the interpolation filters used for luma/chroma motion compensation

- Quantization

  - Storage size associated with quantization

- Transforms

- List all the combinations of transform block size and transform type for primary transforms

- List all the combinations of transform block size and transform type for secondary transforms

- List all the non-separable transforms used

- Indicate whether transforms can be fully/partially implemented using direct matrix multiply

- List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output)

- In-loop filtering

- List line buffer size in terms of luma and chroma lines (if any)

- List all the filters used for in-loop filtering

- Minimum processing unit size

- “tool-off” and “tool-on” tests

- List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools (see Table 1 below for test procedure)

Table 1. Test procedure of “tool-off” and “tool-on” tests for a CfP response

| step | “tool-off” test | “tool-on” test |
| --- | --- | --- |
| 1 | Identify what you consider major tools in a CfP response that contribute to compression efficiency gain, and create a test candidates list |  |
| 2 | Turn on all the tools in the test candidates list, run the CfP response software and compute PSNRs, bit rates, and run times | Turn off all the tools in the test candidates list, run the CfP response software, compute PSNRs, bit rates, and run times, and compute the BD-rate and run-time numbers relative to the HM anchor |
| 3 | Turn off a tool in the test candidates list, and keep all the other tools in the test candidates list on, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools on | Turn on a tool in the test candidates list, and keep all the other tools in the test candidates list off, run the CfP response software and compute the BD-rate and run-time numbers relative to the CfP response software with all tools of the test candidates list off (i.e. step 2) |
| 4 | Repeat step 3 until all the tools in the test candidates list are tested | Repeat step 3 until all the tools in the test candidates list are tested |
| 5 | Compose the test data |  |

Annex 2: Example of filling questionnaire HM vs. JEM

This annex is meant as an example how the questions could be answered. If some of the aspects are not applicable to an algorithm, the corresponding lines may not need to be filled. You may also add lines at the end of the table to compare additional aspects of a proposal versus HM.

Codec level Q&A

| Codec Level Q&A |  | HM | JEM |
| --- | --- | --- | --- |
| 1 | Maximum number of reference frames used by encoder | 4 | 4 |
| 2 | Maximum coding unit (e.g. CTU in HEVC) | luma 64×64 chroma 32×32 | luma 128×128 chroma 64×64 |
| 3 | Minimum and maximum transform block size | luma 4×4/32×32 chroma 4×4/16×16 | luma 4×4/128×128 chroma 2×2/64×64 |
| 4 | Minimum and maximum intra prediction block | luma 4×4 chroma 4×4 | luma 4×4 chroma 2×2 |
| 5 | Minimum inter uni- and multi-hypothesis prediction block size and associated filter tap length | luma 4×8/8×4 (uni-pred) and 8×8 (bi-pred), 8-tap Chroma 2×4/4×2 (uni-pred) and 4×4 bi-pred, 4-tap | luma 4×4 (uni/bi-pred), 8-tap Chroma 2×2 (uni/bi-pred), 4-tap |
| 6 | List all the tools that require access of the neighboring reconstructed block samples (before in-loop filtering) for the reconstruction of the current block (excluding tools in intra-prediction category) | n/a | template matching in FRUC, LIC |
| 7 | List all the tools that require unconventional operation and/or memory access for video codec such as floating point operations, operations exceeding 32-bit arithmetic, divisions, sample-by-sample recursion, and irregular memory access (e.g. isolated samples, position unforeseeable) | n/a | 64-bit arithmetic for LIC, BIO and LMC |
| 8 | List all the tools that use non-rectangular (e.g. triangle) partitions | n/a | n/a |
| 9 | Indicate whether reference frame resampling is used (e.g. the current picture is in 1080p, while the reference picture is in 720p, and vice versa) | n/a | n/a |
| 10 | Indicate whether luma and chroma use separate partitioning structures | no | Yes, in I-slices |
| 11 | Indicate whether there are parsing dependencies (e.g. depending on motion vector reconstruction, reconstructed samples and other complex reconstruction processes) | Entropy parsing depends on intra prediction mode reconstruction, which requires an intra MPM list derivation | Entropy parsing depends on intra prediction mode reconstruction, which requires a more advanced (complex) intra MPM list derivation |

Tool level Q&A

Category 1. Partitioning structure

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 1. Partitioning structure |  | HM | JEM |
| 1.a | Whether the prediction unit partitioning is different from transform unit partitioning | Yes, PU and TU partitioning for a CU can differ | No |
| 1.b | Maximum tree depth | CU maximum depth 4 TU maximum depth 3 PU maximum depth 1 | QT maximum depth 5 BT maximum depth 3 |

Category 2. Entropy coding

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 2. Entropy coding |  | HM | JEM |
| 2.a | Total number of context models | 186 | 329 |
| 2.b | Memory storage size for each context model | 8-bit | 30-bit |
| 2.c | Range table size if table look up is used for range update | LPSTable[64×4×8bits]; RenormTable[32× 3bits] | LPSTable[512×64×9bits]; RenormTable[128×3bits] |
| 2.d | Initialization table size | 3×186×8 bits | 3×329×8 + 329×15×5 bits |
| 2.e | Other memory associated with context model adaptation | LPS transition table: 64*6 bits MPS transition table: 64*6 bits | n/a |

Category 3. Control data derivation process

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 3. Control data derivation process |  | HM | JEM |
| 3.a | Temporal motion data storage granularity | 16×16 | 4×4 |
| 3.b | Worst case number of reference pictures which provide temporal motion data for temporal motion vector derivation (if both list0 and list1 motion data from a bi-directional reference picture are used, count this picture as 2) | 1 | 4 |

Category 4. Decoder-side motion refinement

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 4. Decoder-side motion refinement |  | HM | JEM |
| 4.a | Operation type (e.g. SAD, block gradient), minimum processing unit, worst case number of operations (e.g. number of SADs per processing unit) and interpolation filter associated with the motion refinement. | n/a | - BIO 2×2D separable 6 taps filter (for gradients), mults per sample in refinement calculation 5+5*4 +1/8+2, minimum processing unit same as regular bi-pred, no SAD calculation - FRUC minimum processing unit 44, sequential PU followed by sub-PU level refinement, number of SAD calculations and additional MC interpolations per unit for MV refinement is not constrained (refinement is terminated when cost no longer decreases); bi-linear filter at MV refinement stage |
| 4.b | Indicate whether additional reference samples are needed in addition to reference blocks used for regular motion compensation. If yes, describe the largest search window size. | n/a | - DMVR needs to access (H+7+2)×(W+7+2) MC instead of (H+7)×(W+7) in HEVC MC - FRUC not constrained |

Category 5. Intra prediction

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 5. Intra prediction |  | HM | JEM |
| 5.a | Line buffer size in terms of luma and chroma sample lines | luma 1, chroma 1 | luma 4, chroma 2 |
| 5.b | List all the filters used for intra prediction | Reference sample smoothing: {1,2,1}, Strong intra smoothing filter: 2-tap bilinear filter (only for 32×32 luma TU) Interpolation: 2×32×5 bits (bi-linear 32 phases) | Reference sample smoothing: {1,2,1}, {3,7,14,16,14,7,3}, {4,9,12,14,12,9,4} Interpolation: 2×4×32×8 bits (4 taps, 2 sets, 32 phase) |
| 5.c | Indicate whether there are cross-component dependencies in intra prediction reconstruction | n/a | LM-CHROMA (Cb depends on Y, and Cr depends on Cb&Y) |

Category 6. Inter prediction

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 6. Inter prediction |  | HM | JEM |
| 6.a | Motion vector accuracy | luma ¼ pel chroma 1/8 pel | luma 1/16 pel chroma 1/32 pel |
| 6.b | Storage size for a motion vector component | 16-bit | 16-bit |
| 6.c | List all the interpolation filters used for luma/chroma motion compensation | luma 8-tap, 8×4×6 bits chroma 4-tap, 4×8×6 bits | luma 8-tap, 8×16×6 bits chroma 4-tap, 4×32×6 bits |

Category 7. Quantization

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 7. Quantization |  | HM | JEM |
| 7.a | Storage size associated with quantization | n/a | n/a |

Category 8. Transforms

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 8. Transforms |  | HM | JEM |
| 8.a | List all the combinations of transform block size and transform type for primary transforms | - 8-bit 4×4, 8×8, 16×16 and 32×32 DCT-II, - 8-bit 4×4 DST VII | HEVC transform mode (10-bit DCT-II), transform size combinations: - (4, 8, 16, 32, 64, 128) × (4, 8, 16, 32, 64, 128) TUs for luma CTUs - (2, 4, 8, 16, 32, 64) × (2, 4, 8, 16, 32, 64) TUs for inter-coded chroma CTUs - (2, 4, 8, 16, 32, 64,) × (2, 4, 8, 16, 32, 64) TUs for intra-coded chroma CTUs EMT transform mode (10-bit DST VII, DCT-V, DCT-VIII, DST-I), only applied to CTUs up to 64×64, transform size combinations: - (4, 8, 16, 32, 64) × (4, 8, 16, 32, 64) TUs for luma CTUs |
| 8.b | List all the combinations of transform block size and transform type for secondary transforms | n/a | 35×3 4×4 and 8×8 non-separable Hypercube-Givens Transform (HyGT) |
| 8.c | List all the non-separable transforms used | n/a | 4×4/8×8 HyGT |
| 8.d | Indicate whether transforms can be fully/partially implemented using direct matrix multiply | partial factorization equivalent to matrix multiplication | factorized form not equivalent to matrix multiplication |
| 8.e | List all the transforms that need multiple iterations (where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output) | n/a | 4×4 HyGT 2-interations, 8×8 HyGT 4-interations |

Category 9. In-loop filtering

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 9. In-loop filtering |  | HM | JEM |
| 9.a | List line buffer size in terms of luma and chroma lines (if any) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) | Deblocking (luma 4 & chroma 2) SAO (luma 1 & chroma 1) ALF (luma 8, chroma 4) |
| 9.b | List all the filters used for in-loop filtering | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. | Deblocking: For luma: (3 7 9 −3)/16; (8 19 −1 9 −3)/32; (1 2 2 2 1)/8, (1 1 1 1)/4, and (2 3 1 1 1)/8; For chroma: (1 4 4 –1)/8. ALF: 9×9/7×7/5×5 diamond filter support for luma; and 5×5 diamond filter support for chroma |
| 9.c | Minimum processing unit size | Deblocking (luma: 8×8 grid with 4×4 filter decision, chroma 8×8 grid with 4×4 filter decision) | Deblocking (luma 4×4 grid with 4×4 filter decision, chroma 8×8 grid with 2×2 filter decision) ALF: for luma, filter decision is based on 2×2 block Bilateral filter: sample level filter switch (luma only) |

Category 10. “tool-off” and “tool-on” tests

| Tool Level Q&A |  |  |  |
| --- | --- | --- | --- |
| Category 10. “tool-off” and “tool-on” tests |  | HM | JEM |
| 10.a | List major tools that contribute to compression efficiency gain relative to HEVC, and provide information about encoder/decoder runtime and coding gain of those individual tools | None | QTBT, ALF, FRUC, BIO, DMVR, EMT, … See Table 2 below for an example of “tool-off” and “tool-on” tests |

Table 2. Summary of “tool-off” and “tool-on” tests for JEM6.0 vs. HM

| Tested tool name | “tool-off” test results |  |  |  |  | “tool-on” test results |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Runtime |  | BD-rate |  |  | Runtime |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| AFFINE | −1.2% | −1.0% | −1.1% | 105% | 101% | −2.7% | −2.2% | −2.2% | 135% | 107% |
| ALF | −5.0% | −2.8% | −2.0% | 101% | 115% | −5.8% | −2.3% | −1.7% | 111% | 175% |
| BIO | −1.1% | −0.2% | −0.1% | 115% | 147% | −2.3% | −0.8% | −0.5% | 113% | 187% |
| DMVR | −0.3% | −0.4% | −0.4% | 101% | 100% | −2.6% | −2.5% | −2.5% | 105% | 128% |
| EMT | −2.6% | −0.6% | −0.9% | 130% | 99% | −3.0% | −0.7% | −0.5% | 135% | 99% |
| FRUC | −3.8% | −3.8% | −4.1% | 125% | 130% | −6.1% | −5.8% | −5.9% | 119% | 180% |
| IMV | −1.0% | −1.4% | −1.5% | 111% | 97% | −2.2% | −2.7% | −2.9% | 117% | 95% |
| LM−CHROMA | −1.7% | −24.1% | −29.2% | 100% | 98% | −2.8% | −5.8% | −6.9% | 101% | 98% |
| NSST | −1.7% | −3.6% | −4.0% | 119% | 99% | −2.5% | −5.1% | −5.4% | 111% | 100% |

[end of document]