| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0037-v1 |
| --- | --- |

| Title: | Intra prediction modes based on neural networks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Philipp Helle, Dominique Maniry, Stephan Kaltenstadler, Björn Stallenberger, Philipp Merkle, Mischa Siekmann, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany | Tel: Email: | +49 30 31002-835 jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute. |  |  |

_____________________________

# Abstract

In this document, intra prediction modes to generate an intra-picture prediction signal on a rectangular block in a future video codec are proposed. These intra prediction modes perform the following two main steps: First, a set of features is extracted from the decoded samples. Second, these features are used to select an affine linear combination of predefined image patterns as the prediction signal. Also, a specific signalization scheme for the intra-prediction modes is proposed. Since the proposed predictors are non-linear, they can neither be represented by the angular-prediction nor by the DC- or Planar-prediction modes of the HEVC resp. JEM reference software.

The proposed intra-prediction modes are based on fully connected neural networks with several layers. Such networks come with an additional computational complexity compared to the traditional intra prediction modes. The proposed predictors have the following properties to deal with this complexity: For a given block shape, all predictors share all but the last layer in the neural network. Moreover, as a further development of the proposed technique, it is proposed that for large blocks, the target space of the neural network based prediction signal is the frequency domain where a lot of frequency components of the prediction signal are constrained to a constant value.

It is also proposed that for each of the above intra prediction modes a set of non-separable orthogonal transforms is available which are secondary transforms for large blocks. It is proposed that these transforms can be applied in the transform-coding of the prediction residual of the corresponding mode.

# Description

## Proposed predictions

### Generation of the proposed prediction signal

On a given MxN block with M≤32, N≤32, the generation of a proposed luma prediction signal pred is performed by processing a set of reference samples r through a neural network. The reference samples r consist of K rows of size N+K above and K columns of size M left of the block, see Figure 1. The number K may depend on M and N. At the moment, K is set to 2 for all M and N. A modification is described in section 1.1.7.

Fig. 1: Prediction of MxN intra block from reconstructed samples using a neural network

The proposed neural network first extracts a vector ftr of features from the reconstructed samples r as follows. If d0=K*(N +M+K) denotes the number of samples of r, then r is regarded as a vector in the real vector space of dimension d0. For fixed integral square-matrices A1 and A2 which have d0 rows resp. columns and for fixed integral bias vectors b1 and b2 of dimension d0 one first computes

t1=ρ(A1∙r+b1 ).

Here, ∙ denotes the ordinary matrix-vector product. Moreover, the function ρ is an integer-approximation of the ELU function ρ0, where the latter function is defined on a p-dimensional vector v by putting

ρ0vi=vi, if vi>0expvi-1, else,

where ρ0vi and vi denote the i-th component of the vectors. One applies similar operations to t1 and computes

t2=ρ(A2∙t1+b2).

Finally, for a fixed integer d1 with 0≤d1≤d0, there is a predefined integral matrix A3 with d1 rows and d0 columns and there is a predefined integral bias vector b3 of dimension d1 such that one computes the feature vector ftr as

ftr=ρ(A3∙t2+b3).

The value of d1 depends on M and N. At the moment, one puts d1= d0 . A modification is described in section 1.1.7.

Out of the feature vector ftr, the final prediction signal pred is generated using an affine linear map followed by the standard Clipping operation  Clip that depends on the bit-depth. Thus, there is a predefined matrix A4 with M*N rows and d1 columns and a predefined bias vector b4 of dimension M*N such that one computes

pred=ClipA4∙ftr+b4 .

The above proposed generation of the prediction signal pred is depicted at the right hand side of Figure 3 below.

### Signalization of the specific proposed intra mode

In the present document, n different intra prediction modes are proposed, where n is set to 35 for max(M,N)<32 and to 11 else. Thus, an index predmode with 0≤predmode<n is to be signaled by an encoder and to be parsed by a decoder and the following syntax is proposed. One has

n=3+2k,

where k=3 if max(M,N)=32 and k=5, else. In a first step, an index predIdx with 0≤predIdx<n is signaled using the following code. First, one bin encodes whether predIdx<3 or not. If predIdx<3, a second bin encodes if predIdx=0 or not, and, if predIdx≠0, another bin encodes whether predIdx is equal to 1 or 2. If predIdx ≥3 then the value of predIdx is signaled in the canonical way using k bins. This syntax for the signalization of the index predIdx is delineated in Table 1.   	
From the index predIdx, the actual index predmode is derived using a fully connected neural network with one hidden layer that has the reconstructed samples r' on the two rows of size N+2 above and the two columns of size M left of the block as input, see Figure 2.

Fig. 2: Prediction of mode probabilities from reconstructed samples using a neural network

The reconstructed samples r' are regarded as a vector in the real vector space of dimension 2*(M+N+2). There is a fixed square-matrix A1ʹ which has 2*(M+N+2) rows resp. columns and there is a fixed bias vector b1ʹ in the real vector space of dimension 2*(M+N+2) such that one computes

t1'=ρ(A1'∙r'+b1').

Next, there exists a matrix A2ʹ which has n rows and 2*(M+N+2) columns and there is a fixed bias vector b2ʹ in the real vector space of dimension n such that one computes

lgt=A2'∙t1'+b2' .

The index predmode is now derived as being the position of the predIdx-th largest component of lgt. Here, if two components (lgt)k and  (lgt)l  are equal for k≠l , (lgt)k is regarded as larger than (lgt)l  if k<l and (lgt)l  is regarded as larger than (lgt)k, else.	
To explain the above procedure, it is remarked that the i-th entry of the vector lgt is to be interpreted, up to normalization, as the logarithm (‘logit’) of the conditional probability of the i-th proposed intra prediction mode given the reference samples r'and thus the index predIdx indicates that the predIdx-th most probable mode is selected. 	
The derivation of the index predmode proposed in this section is depicted on the left hand side of Figure 3 below.

|  |  | flag | unary_max |  | fixed_length |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | predIdx | CTX | EP |  | LSB |  | EP | MSB |  |
| MPM | 0 | 1 | 0 |  |  |  |  |  |  |
|  | 1 | 1 | 1 | 0 |  |  |  |  |  |
|  | 2 | 1 | 1 | 1 |  |  |  |  |  |
|  | 3 | 0 |  |  | 0 | 0 | 0 | 0 | 0 |
|  | 4 | 0 |  |  | 1 | 0 | 0 | 0 | 0 |
|  | 5 | 0 |  |  | 0 | 1 | 0 | 0 | 0 |
|  | 6 | 0 |  |  | 1 | 1 | 0 | 0 | 0 |
|  | 7 | 0 |  |  | 0 | 0 | 1 | 0 | 0 |
|  | 8 | 0 |  |  | 1 | 0 | 1 | 0 | 0 |
|  | 9 | 0 |  |  | 0 | 1 | 1 | 0 | 0 |
|  | 10 | 0 |  |  | 1 | 1 | 1 | 0 | 0 |
| if width < 32 and height < 32 | 11 | 0 |  |  | 0 | 0 | 0 | 1 | 0 |
|  | 12 | 0 |  |  | 1 | 0 | 0 | 1 | 0 |
|  | 13 | 0 |  |  | 0 | 1 | 0 | 1 | 0 |
|  | 14 | 0 |  |  | 1 | 1 | 0 | 1 | 0 |
|  | 15 | 0 |  |  | 0 | 0 | 1 | 1 | 0 |
|  | 16 | 0 |  |  | 1 | 0 | 1 | 1 | 0 |
|  | 17 | 0 |  |  | 0 | 1 | 1 | 1 | 0 |
|  | 18 | 0 |  |  | 1 | 1 | 1 | 1 | 0 |
|  | 19 | 0 |  |  | 0 | 0 | 0 | 0 | 1 |
|  | 20 | 0 |  |  | 1 | 0 | 0 | 0 | 1 |
|  | 21 | 0 |  |  | 0 | 1 | 0 | 0 | 1 |
|  | 22 | 0 |  |  | 1 | 1 | 0 | 0 | 1 |
|  | 23 | 0 |  |  | 0 | 0 | 1 | 0 | 1 |
|  | 24 | 0 |  |  | 1 | 0 | 1 | 0 | 1 |
|  | 25 | 0 |  |  | 0 | 1 | 1 | 0 | 1 |
|  | 26 | 0 |  |  | 1 | 1 | 1 | 0 | 1 |
|  | 27 | 0 |  |  | 0 | 0 | 0 | 1 | 1 |
|  | 28 | 0 |  |  | 1 | 0 | 0 | 1 | 1 |
|  | 29 | 0 |  |  | 0 | 1 | 0 | 1 | 1 |
|  | 30 | 0 |  |  | 1 | 1 | 0 | 1 | 1 |
|  | 31 | 0 |  |  | 0 | 0 | 1 | 1 | 1 |
|  | 32 | 0 |  |  | 1 | 0 | 1 | 1 | 1 |
|  | 33 | 0 |  |  | 0 | 1 | 1 | 1 | 1 |
|  | 34 | 0 |  |  | 1 | 1 | 1 | 1 | 1 |

Table 1: Binarization of the index j used to generate the prediction mode index predIdx

### Extension to larger blocks

For blocks of size 64x64, the proposed intra prediction modes for blocks of size 32x32 are reused. For that purpose, for K=2 one first takes 2K rows of size 64+2K above the block and 2K columns of size 64 left of the block with reconstructed samples r. Then, one applies a normalized one-two-one-filter in each direction followed by a factor two downsampling in each direction. This yields reconstructed samples rds that can be regarded as an input for the previously proposed prediction modes on blocks of size 32x32. Let predsmall be a prediction signal of size 32x32 generated by the proposed method using the input rds. Then it is proposed that predsmall is upsampled by bilinear interpolation to obtain a prediction signal pred on the actual block of size 64x64. Using a similar operation, the signalization of the proposed intra prediction modes for blocks of size 32x32 induces a signalization for the proposed intra prediction modes for blocks of size 64x64. 	
A similar procedure to reuse the proposed intra prediction modes for blocks of size 16x16 to generate prediction signals on blocks of size 32x32 is proposed.

### Integration into the present codec: Decoder

At the parsing stage, the following two steps are proposed to be carried out by a decoder. First, for each luma Coding Unit (CU) in intra mode, a flag is parsed that determines whether a proposed intra prediction mode is to be applied for the present CU or not. If a proposed intra mode is to be applied, a split of the CU into transform units (TU) is prohibited and the CU consist of one TU only. If the present CU is of size 32x32 and if a proposed intra mode is to be applied, another flag is parsed that determines whether the proposed intra mode is to be generated using the method described in section 1.1.3. Second, if a proposed intra mode is to be applied, an index predIdx is parsed according to Table 1. 	
At the reconstruction stage, it is proposed that the decoder carries out the following two steps. First, the logits lgt are computed as described in section 1.1.2 resp. 1.1.3. Using the index predIdx, the index predmode  and thus the specific proposed prediction mode is determined according to section 1.1.2. resp. 1.1.3. Second, the prediction signal pred corresponding to predmode is obtained as described in section 1.1.1 resp. 1.1.3. The proposed steps to be carried out at the reconstruction stage by a decoder are depicted in Figure 3.

Fig. 3: Block diagram of decoder-side reconstruction process (predIdx is sent in bit stream)

The proposed intra prediction modes are applied to the luma component only. If the chroma components are to be predicted in DM-chroma mode on a given CU and if the luma prediction signal is generated by the proposed modes, the chroma prediction signals are given by the planar prediction signals. If on a CU or on a TU the conventional intra prediction is to be performed at the luma component and the list of most probable modes is to be generated, all neighboring, already reconstructed CUs or TUs in intra mode that have the proposed intra prediction modes are regarded as having the planar prediction mode

### Integration into the present codec: Encoder

The following steps are proposed to be carried out by an encoder. Whenever an intra search for the luma component is conducted, first the best conventional intra prediction mode (DC-, Planar- or Angular-Prediction-Mode) with corresponding rate distortion (RD) costs is determined as in the underlying encoder implementation. Then, the best proposed intra prediction mode with corresponding RD-costs is determined as described below. If these costs are lower than the costs of the best conventional intra prediction mode, a flag indicating that a proposed intra prediction mode is to be used is encoded with value true. Otherwise, it is encoded with value false. 	
The following RD-search among the proposed intra prediction modes is proposed. First, the encoder computes the feature vector ftr as described in section 1.1.1 resp. section 1.1.3. It is pointed out that this computation needs to be carried out only once since all different proposed intra prediction modes use the same feature vector ftr. Second, the encoder computes the logits lgt as described in section 1.1.2. resp. 1.1.3 and thus, according to that sections, the signalization costs for each proposed intra prediction mode are known to the encoder. Out of the feature vector ftr, the encoder computes each proposed prediction signal pred as described in section 1.1.1 resp. 1.1.3. For each proposed prediction signal, the encoder computes costs that are set to be the weighted sum of the L1 norm of the Hadamard-Transform of the prediction residual and the signalization costs of the mode. The encoder ranks all proposed intra prediction modes according to the latter costs and performs a full RD search for the k best proposed intra modes in that ranking. Here, the number k depends on the blocksize and has the same value as the number of modes for the full RD-search in the conventional intra search with Hadamard-Fast Search. It is proposed that always the three modes for which the logits lgt are the largest are included in the list for the full RD search, which possibly augments k. This is analogous to the inclusion of the most probable modes (MPMs) into the list for the full RD search in the conventional intra search with Hadamard-Fast Search. 	
If the encoder has decided that a proposed intra prediction mode with index predmode is to be chosen at the current CU and if in the vector lgt, computed previously by the encoder, the entry at position predmode is the predIdx-th largest among all entries of  lgt, it is proposed that the encoder encodes the index predIdx according to Table 1.    	
The proposed estimation process for an encoder just described is depicted in Figure 4.

Fig. 4 Block diagram of encoder-side estimation process (difference to decoder-side processing is highlighted blue, predIdx is sent in bit stream)

If the block is of size 32x32, the encoder conducts the above search both for the proposed modes corresponding to size 32x32 and for the proposed modes induced from blocks of size 16x16 as in section 1.1.3 and chooses the best result among the two candidates. Moreover, if a proposed intra mode is selected, a second flag indicating which of the two options is to be chosen at the present block of size 32x32 is encoded.

### Simplification of the proposed predictors by predicting into the frequency domain

In order to facilitate the computation of the prediction signal pred out of the feature vector ftr from section 1.1.1, as a further development of the aforementioned technique it is proposed that the modes predict into the frequency domain instead of the sample domain. Thus, keeping the notations from section 1.1.1, it is proposed that there is a predefined matrix A4,fr with M*N rows and d1 columns and a predefined bias vector b4,fr of dimension M*N such that one computes

predfr=A4,fr∙ftr+b4,fr .

If W-1 is the matrix of the integral approximation of the Inverse Discrete Cosine Transform (DCT II) that is already present in the codec, it is proposed that the actual prediction signal pred is then obtained as

pred=W-1(predfr).

It is proposed that this procedure to generate the prediction signal is to be applied to all block shapes except blocks of size 4x4. If the proposed prediction predicts into the frequency domain, it is proposed that most of the frequency components of the prediction signal predfr are constrained to a constant value. If the latter constraint is true for a specific frequency component, then the row of the matrix A4,fr corresponding to that frequency component is identically zero and no multiplications need to be performed for the corresponding frequency component. The latter effect is illustrated by the fact that for example on square blocks of size 32x32, it is reported that at least 813 and up to 927 rows of the 1024 rows of the matrices A4,fr are identically zero for the 11 proposed intra prediction modes. 
Optionally, it is proposed that the Inverse Discrete Cosine Transform is applied to the sum of the dequantized prediction residual and the proposed prediction signal predfr to obtain the reconstruction signal directly if the residual is transform-coded using the Discrete Cosine Transform.

### Modifications of the network architecture

The following modifications of the parameters K and d1 (see section 1.1.1) depending on the blocksizes M and N were carried out. First, one puts K=2 for either max(M,N)=32 and min(M,N)>4 or max(M,N)<32 and M≠N and K=4, else. Second, one puts d1=M+N+1 for max(M,N)=32 and d1= 2*(M+N+2), else.

## Proposed Transforms

### Description

For each proposed intra prediction mode, an additional set of four transforms is proposed. If the block on which the proposed intra prediction is defined is of size MxN, then the proposed transform is a primary transform if maxM,N≤8 and a secondary transform, else. The proposed secondary transforms are applied to the minM,8×min⁡(N,8) lowest frequencies of the DCT-II transform coefficients of the prediction residual. Thus, if n denotes the number of proposed intra prediction modes on the MxN block, for each predmode with  0≤predmode<n and each tridx with 0≤tridx<4, there exists a transform Tpredmode,tridx with inverse transform Tpredmode,tridx-1 that operates on blocks of size minM,8×min⁡(N,8).

### Integration into the present codec: Decoder

At the parsing stage, the following steps are proposed for a decoder on a given luma transform unit (TU) for which the coding unit (CU) that contains the TU is in intra mode. Assume that, according to the previous description, the decoder has already parsed the information that indicates whether for the CU that contains the present TU one of the proposed intra prediction modes is to be applied. Moreover, assume that the decoder has already parsed the transform coefficients for the luma prediction residual. If a proposed prediction modes is to be applied and if k or more of these transform coefficients are different from zero, the decoder parses a flag that indicates if a proposed transform for the present TU is to be applied or not. Here, k is equal to two if the Chroma-Dual tree is used and to three, else. Otherwise, no proposed transform can be applied. If the newly proposed transform is to be applied, the decoder parses context modeled bins to determine the index tridx of the transform.	
At the reconstruction stage, the following steps are proposed if at the present TU a proposed intra prediction mode predmode and a newly proposed transform with index tridx are to be used. If maxM,N≤8, the decoder applies the inverse transform  Tpredmode,tridx-1 to the transform coefficients. If maxM,N>8, the decoder applies  Tpredmode,tridx-1 to the first minM,8×min⁡(N,8) transform coefficients and then applies the inverse of the DCT-II to the canonical concatenation of the result and the remaining transform coefficients.

### Integration into the present codec: Encoder

The following extension of the encoder procedure described in section 1.1.5 is proposed. Whenever an intra search for the luma component is conducted on a CU, the encoder first carries out the same search steps as in section 1.1.5. In particular, the encoder has determined a proposed prediction mode candidate predmode0 among all proposed prediction modes and a set of proposed intra prediction modes that are to be checked by a full RD-search. For each of these proposed intra prediction modes, the encoder computes the RD-costs for each of the proposed transforms. Excluding pairs for which strictly less than k transform coefficients are different from zero (k as in the previous section), the encoder chooses predmode1 and tridx as the pair with minimal RD-costs. If these RD costs are larger than the RD costs of predmode0 determined previously, the encoder proceeds as in section 1.1.5 with the extension that if predmode0 is to be used on the present CU and if the prediction residual of predmode0 has k or more nonzero transform-coefficients, also the flag that none of the proposed transforms is to be used for predmode0 is encoded. Else, if these RD costs are also smaller than the RD-costs of the best conventional intra prediction modes, the encoder encodes the information leading to the proposed prediction mode predmode1 as described in section 1.1.5. Moreover, after having coded the prediction residual, the encoder encodes the flag that a proposed transform is to be used. Then, the encoder encodes the index tridx.

# Experimental Results

The following results have been tested on a JEM like reference software using a HEVC configuration with QTGBS partitioning structure (for a description of the latter partitioning structure, proposed by HHI, see Document JVET-J0035).

Table 1 shows the results for the intra predictors described in section 1.1 without the simplifications and modifications of 1.1.6 and 1.1.7. Table 2 shows the results for the intra predictors of section 1.1 including the simplifications of 1.1.6 and 1.1.7. Table 3 shows the results for the intra predictors of section 1.1 without the simplifications of 1.1.6 and 1.1.7 if four additional transforms per proposed mode are included, as described in section 1.2. Table 4 shows the results for the intra predictors of section 1.1 including the simplifications of 1.1.6 and 1.1.7 if four additional transforms are included per proposed mode, as described in section 1.2. Table 5 shows the same results as Table 4 with the difference that in both the tested configuration and the reference configuration, the tool SOT (‘set of transforms’, an extension of the NSST and EMT tools of JEM proposed by HHI in Document JVET-J0040) is switched on.

Table 1: Results for intra predictors (section 1.1) without simplifications.

| Intra Only |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| JVET A1 | -3.07% | -1.22% | -1.38% | 278% | 270% | -3.11% | -0.74% | -0.58% | 296% | 303% |
| JVET A2 | -3.03% | -1.36% | -1.36% | 286% | 250% | -2.99% | -0.67% | -0.60% | 304% | 285% |
| JVET B | -2.97% | -1.28% | -0.97% | 265% | 236% | -3.06% | -0.93% | -0.53% | 288% | 266% |
| JVET C | -1.54% | -0.01% | 0.28% | 225% | 164% | -1.70% | 0.79% | 0.95% | 240% | 184% |
| JVET D | -2.14% | -0.89% | -0.13% | 220% | 148% | -2.27% | -0.94% | 0.04% | 237% | 163% |
| JVET E | -4.26% | -1.45% | -2.33% | 273% | 221% | -4.16% | -0.28% | -1.70% | 293% | 248% |
| JVET F | -0.85% | 0.17% | -0.09% | 265% | 153% | -1.02% | 0.53% | -0.05% | 270% | 159% |
| CfP 360 | -3.49% | -1.55% | -1.25% | 327% | 257% | -3.35% | -0.95% | -0.52% | 361% | 279% |
| CfP SDR-A | -2.78% | -1.26% | -1.34% | 266% | 239% | -2.75% | -0.78% | -0.82% | 281% | 274% |
| CfP SDR-B | -3.05% | -1.14% | -0.53% | 268% | 239% | -3.13% | -0.65% | 0.32% | 291% | 268% |
| CfP HDR-A | -3.87% | -2.17% | -1.94% | 292% | 286% | -3.84% | -1.76% | -1.39% | 321% | 317% |
| CfP HDR-B | -2.17% | 0.57% | 1.58% | 248% | 205% | -1.74% | 1.53% | 1.92% | 254% | 230% |
| JVET all | -2.51% | -0.86% | -0.81% | 257% | 201% | -2.58% | -0.34% | -0.31% | 274% | 223% |
| CfP all | -3.00% | -1.02% | -0.59% | 278% | 240% | -2.89% | -0.42% | 0.02% | 298% | 269% |
| JVET + CfP | -2.81% | -0.99% | -0.75% | 268% | 218% | -2.81% | -0.44% | -0.22% | 286% | 241% |
| Random Access |  |  |  |  |  |  |  |  |  |  |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| JVET A1 | -2.04% | 2.82% | 1.18% | 180% | 153% | -1.86% | 3.52% | 2.50% | 175% | 148% |
| JVET A2 | -1.63% | 0.86% | 0.90% | 161% | 117% | -1.65% | 1.10% | 1.03% | 160% | 117% |
| JVET B | -1.76% | 0.35% | -0.04% | 167% | 123% | -1.96% | 0.27% | 0.20% | 172% | 128% |
| JVET C | -1.22% | 0.41% | 0.80% | 165% | 123% | -1.37% | 0.81% | 1.65% | 166% | 121% |
| JVET D | -1.30% | -0.14% | 0.41% | 158% | 117% | -1.39% | -0.19% | 1.92% | 162% | 120% |
| JVET E | -2.85% | -0.24% | -0.74% | 160% | 114% | -3.24% | 0.17% | 0.73% | 160% | 114% |
| JVET F | -0.68% | 0.01% | 0.19% | 184% | 118% | -0.78% | 0.55% | -0.25% | 184% | 119% |
| CfP 360 | -2.42% | 0.63% | 1.26% | 174% | 120% | -2.50% | 1.12% | 2.15% | 183% | 123% |
| CfP SDR-A | -1.37% | 0.55% | 0.36% | 164% | 125% | -1.34% | 0.77% | 0.64% | 162% | 124% |
| CfP SDR-B | -1.88% | 0.97% | 0.62% | 164% | 125% | -1.87% | 1.32% | 1.36% | 164% | 128% |
| CfP HDR-A | -2.22% | 0.32% | 1.17% | 167% | 128% | -2.28% | 0.79% | 2.34% | 166% | 126% |
| CfP HDR-B | -1.51% | 1.57% | 2.01% | 171% | 122% | -1.33% | 1.98% | 1.58% | 177% | 125% |
| JVET all | -1.60% | 0.60% | 0.41% | 168% | 123% | -1.70% | 0.89% | 1.09% | 169% | 124% |
| CfP all | -1.85% | 0.85% | 1.08% | 168% | 124% | -1.83% | 1.23% | 1.55% | 170% | 125% |
| JVET + CfP | -1.75% | 0.64% | 0.71% | 168% | 124% | -1.81% | 1.02% | 1.33% | 170% | 125% |
| Low Delay |  |  |  |  |  |  |  |  |  |  |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| Cactus | -1.14% | 0.42% | 0.08% | 161% | 116% | -1.08% | 1.83% | 0.56% | 162% | 119% |
| BasketballDrive | -0.99% | -0.51% | -0.84% | 173% | 117% | -0.89% | -0.57% | -0.68% | 176% | 128% |
| BQTerrace | -0.89% | 0.38% | 2.63% | 143% | 107% | -1.54% | 0.87% | 3.65% | 149% | 110% |
| RitualDance | -2.14% | -0.21% | -0.30% | 194% | 136% | -1.81% | 1.18% | 1.37% | 187% | 134% |
| MarketPlace | -1.08% | 0.08% | 1.02% | 151% | 114% | -1.14% | -0.01% | 1.52% | 150% | 117% |
| All | -1.25% | 0.03% | 0.52% | 163% | 118% | -1.29% | 0.66% | 1.28% | 164% | 121% |

Table 2: Results for intra predictors (section 1.1) including simplifications (sections 1.1.6 and 1.1.7).

| Intra Only |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| JVET A1 | -2.27% | -0.93% | -0.98% | 176% | 144% | -2.41% | -0.81% | -0.58% | 178% | 150% |
| JVET A2 | -2.23% | -0.90% | -1.03% | 179% | 136% | -2.30% | -0.60% | -0.76% | 181% | 142% |
| JVET B | -2.41% | -0.96% | -0.81% | 167% | 136% | -2.51% | -0.81% | -0.64% | 174% | 143% |
| JVET C | -1.42% | -0.16% | 0.03% | 162% | 123% | -1.58% | 0.32% | 0.53% | 163% | 129% |
| JVET D | -1.76% | -0.91% | -0.46% | 157% | 119% | -1.97% | -1.08% | 0.05% | 165% | 127% |
| JVET E | -3.80% | -2.12% | -2.68% | 167% | 133% | -3.80% | -1.47% | -2.20% | 172% | 140% |
| JVET F | -0.87% | -0.10% | -0.11% | 166% | 113% | -1.02% | -0.25% | -0.16% | 170% | 118% |
| CfP 360 | -3.03% | -1.51% | -1.46% | 193% | 145% | -2.96% | -1.10% | -0.99% | 202% | 148% |
| CfP SDR-A | -1.77% | -0.77% | -0.82% | 166% | 130% | -1.99% | -0.61% | -0.62% | 169% | 137% |
| CfP SDR-B | -2.45% | -0.82% | -0.74% | 168% | 132% | -2.54% | -0.57% | -0.38% | 174% | 139% |
| CfP HDR-A | -2.83% | -1.52% | -1.24% | 179% | 140% | -2.96% | -1.30% | -0.76% | 186% | 150% |
| CfP HDR-B | -2.06% | -0.07% | 0.29% | 161% | 127% | -1.75% | 0.65% | 0.88% | 161% | 132% |
| JVET all | -2.06% | -0.83% | -0.80% | 168% | 129% | -2.18% | -0.65% | -0.48% | 172% | 135% |
| CfP all | -2.39% | -0.89% | -0.76% | 172% | 134% | -2.39% | -0.52% | -0.34% | 177% | 141% |
| JVET + CfP | -2.26% | -0.92% | -0.83% | 170% | 131% | -2.35% | -0.67% | -0.47% | 174% | 138% |
| Random Access |  |  |  |  |  |  |  |  |  |  |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| JVET A1 | -1.42% | 2.20% | 0.85% | 131% | 111% | -1.30% | 2.50% | 1.92% | 129% | 109% |
| JVET A2 | -1.06% | 0.47% | 0.67% | 122% | 98% | -1.20% | 0.71% | 0.75% | 120% | 98% |
| JVET B | -1.36% | 0.14% | 0.06% | 124% | 102% | -1.57% | 0.10% | 0.17% | 125% | 105% |
| JVET C | -1.01% | 0.38% | 0.40% | 129% | 108% | -1.03% | 0.22% | 1.20% | 126% | 103% |
| JVET D | -1.09% | 0.70% | -0.31% | 125% | 108% | -1.15% | 1.50% | 0.73% | 124% | 107% |
| JVET E | -2.31% | -0.40% | -0.57% | 122% | 104% | -2.67% | 0.54% | -0.08% | 122% | 105% |
| JVET F | -0.58% | 0.29% | -0.03% | 133% | 105% | -0.73% | 0.48% | 0.48% | 134% | 107% |
| CfP 360 | -2.09% | 0.07% | 0.59% | 126% | 101% | -2.22% | 0.58% | 0.94% | 130% | 102% |
| CfP SDR-A | -0.83% | 0.13% | 0.37% | 123% | 102% | -0.88% | 0.26% | 0.61% | 121% | 102% |
| CfP SDR-B | -1.39% | 0.43% | 0.41% | 124% | 103% | -1.49% | 0.82% | 1.14% | 123% | 105% |
| CfP HDR-A | -1.57% | 0.01% | 0.75% | 125% | 105% | -1.71% | 0.39% | 1.75% | 124% | 104% |
| CfP HDR-B | -1.38% | 0.49% | 1.17% | 125% | 103% | -1.37% | 0.67% | 0.45% | 127% | 105% |
| JVET all | -1.23% | 0.56% | 0.17% | 127% | 105% | -1.34% | 0.85% | 0.75% | 126% | 105% |
| CfP all | -1.44% | 0.24% | 0.65% | 125% | 102% | -1.52% | 0.56% | 0.91% | 125% | 103% |
| JVET + CfP | -1.36% | 0.39% | 0.35% | 126% | 104% | -1.46% | 0.73% | 0.82% | 126% | 104% |
| Low Delay |  |  |  |  |  |  |  |  |  |  |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| Cactus | -0.91% | 0.53% | 0.37% | 122% | 104% | -0.84% | 1.42% | 0.69% | 122% | 106% |
| BasketballDrive | -0.69% | -0.31% | -0.46% | 125% | 98% | -0.52% | -0.68% | -0.34% | 127% | 108% |
| BQTerrace | -0.89% | 0.48% | 2.41% | 117% | 106% | -1.60% | -0.07% | 3.48% | 120% | 106% |
| RitualDance | -1.80% | 0.08% | -0.75% | 132% | 108% | -1.52% | 1.11% | 0.85% | 128% | 108% |
| MarketPlace | -0.78% | -0.07% | -0.24% | 116% | 104% | -0.92% | 0.30% | 0.33% | 116% | 105% |
| All | -1.01% | 0.14% | 0.27% | 122% | 104% | -1.08% | 0.42% | 1.00% | 122% | 106% |

Table 3: Results for intra predictors (section 1.1) without simplifications and with four additional transforms per proposed mode (section 1.2).

| Intra Only |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| JVET A1 | -7.06% | -2.89% | -2.98% | 442% | 306% | -6.86% | -2.17% | -1.91% | 436% | 333% |
| JVET A2 | -7.38% | -2.90% | -2.86% | 440% | 295% | -6.70% | -2.14% | -1.93% | 430% | 324% |
| JVET B | -6.53% | -2.73% | -2.47% | 439% | 267% | -6.64% | -1.99% | -1.59% | 440% | 291% |
| JVET C | -4.09% | 0.35% | 0.37% | 414% | 191% | -4.36% | 2.25% | 2.05% | 410% | 208% |
| JVET D | -4.93% | -1.16% | -0.80% | 356% | 156% | -5.26% | -0.68% | 0.49% | 360% | 170% |
| JVET E | -8.09% | -3.18% | -3.90% | 412% | 237% | -7.40% | -1.36% | -2.47% | 412% | 264% |
| JVET F | -2.09% | 0.47% | 0.09% | 394% | 163% | -2.47% | 1.05% | 0.52% | 399% | 175% |
| CfP 360 | -7.77% | -2.70% | -1.95% | 553% | 301% | -7.22% | -1.32% | -0.17% | 568% | 318% |
| CfP SDR-A | -6.07% | -2.49% | -2.54% | 408% | 273% | -5.78% | -1.90% | -1.93% | 398% | 300% |
| CfP SDR-B | -6.84% | -2.40% | -1.85% | 427% | 265% | -6.68% | -1.16% | -0.39% | 428% | 288% |
| CfP HDR-A | -8.21% | -4.11% | -3.63% | 476% | 327% | -7.41% | -2.86% | -2.12% | 487% | 350% |
| CfP HDR-B | -5.42% | 0.99% | 2.71% | 386% | 233% | -4.67% | 2.30% | 2.80% | 367% | 257% |
| JVET all | -5.68% | -1.70% | -1.74% | 414% | 224% | -5.64% | -0.74% | -0.66% | 413% | 245% |
| CfP all | -6.74% | -1.97% | -1.26% | 444% | 274% | -6.26% | -0.82% | -0.21% | 441% | 297% |
| JVET + CfP | -6.28% | -1.93% | -1.59% | 430% | 246% | -6.05% | -0.86% | -0.49% | 428% | 266% |
| Random Access |  |  |  |  |  |  |  |  |  |  |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| JVET A1 | -2.67% | 2.61% | 1.19% | 212% | 156% | -2.57% | 4.03% | 2.97% | 210% | 150% |
| JVET A2 | -3.38% | 0.89% | 0.85% | 193% | 121% | -3.47% | 1.10% | 0.94% | 189% | 120% |
| JVET B | -3.14% | -0.37% | -0.37% | 202% | 126% | -3.73% | -0.16% | -0.05% | 211% | 131% |
| JVET C | -1.95% | 0.86% | 0.76% | 193% | 125% | -2.26% | 1.65% | 1.73% | 197% | 124% |
| JVET D | -1.95% | 0.01% | 0.39% | 214% | 128% | -2.19% | 0.06% | 1.26% | 197% | 121% |
| JVET E | -5.48% | -0.63% | -1.50% | 194% | 114% | -5.63% | 0.77% | -0.93% | 196% | 116% |
| JVET F | -1.35% | 0.42% | 0.22% | 233% | 116% | -1.65% | 0.73% | 0.44% | 236% | 121% |
| CfP 360 | -5.06% | 0.08% | 1.24% | 231% | 123% | -5.23% | 1.18% | 2.85% | 241% | 124% |
| CfP SDR-A | -2.13% | 0.62% | 0.39% | 197% | 129% | -2.25% | 0.82% | 0.64% | 194% | 127% |
| CfP SDR-B | -2.96% | 0.40% | 0.43% | 194% | 128% | -3.26% | 1.26% | 1.71% | 198% | 130% |
| CfP HDR-A | -4.05% | -0.25% | 0.63% | 204% | 133% | -3.98% | 0.61% | 2.19% | 205% | 132% |
| CfP HDR-B | -3.32% | 2.04% | 2.74% | 213% | 123% | -3.08% | 2.37% | 2.29% | 219% | 127% |
| JVET all | -2.76% | 0.55% | 0.26% | 206% | 127% | -3.00% | 1.14% | 0.94% | 205% | 126% |
| CfP all | -3.46% | 0.65% | 1.13% | 208% | 127% | -3.52% | 1.30% | 1.91% | 211% | 128% |
| JVET + CfP | -3.16% | 0.52% | 0.63% | 208% | 127% | -3.30% | 1.19% | 1.40% | 209% | 127% |
| Low Delay |  |  |  |  |  |  |  |  |  |  |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| Cactus | -2.48% | 0.45% | -0.04% | 181% | 118% | -2.52% | 1.67% | 0.55% | 188% | 122% |
| BasketballDrive | -0.97% | -0.26% | -0.59% | 196% | 115% | -0.66% | -0.36% | 0.12% | 205% | 128% |
| BQTerrace | -2.43% | 1.64% | 3.79% | 166% | 110% | -4.39% | 2.49% | 5.42% | 179% | 112% |
| RitualDance | -2.44% | -0.32% | -0.34% | 221% | 135% | -2.14% | 0.93% | 1.23% | 217% | 134% |
| MarketPlace | -1.89% | 0.23% | 0.61% | 169% | 116% | -2.00% | 1.01% | 2.41% | 170% | 118% |
| All | -2.04% | 0.35% | 0.69% | 185% | 119% | -2.34% | 1.15% | 1.95% | 191% | 123% |

Table 4: Results for intra predictors (section 1.1) including simplifications (sections 1.1.6 and 1.1.7) and with four additional transforms per proposed mode (section 1.2).

| Intra Only |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| JVET A1 | -5.75% | -2.38% | -2.45% | 285% | 168% | -5.74% | -2.01% | -1.80% | 264% | 169% |
| JVET A2 | -5.56% | -2.14% | -2.13% | 277% | 155% | -5.22% | -1.79% | -1.67% | 251% | 159% |
| JVET B | -5.50% | -2.18% | -1.90% | 290% | 157% | -5.72% | -1.87% | -1.60% | 274% | 160% |
| JVET C | -3.18% | 0.20% | 0.20% | 287% | 137% | -3.46% | 1.54% | 1.04% | 266% | 141% |
| JVET D | -3.66% | -0.32% | -0.19% | 279% | 127% | -4.01% | -0.14% | 0.60% | 275% | 137% |
| JVET E | -6.44% | -2.78% | -3.43% | 268% | 142% | -6.10% | -1.49% | -2.78% | 253% | 147% |
| JVET F | -1.74% | 0.15% | -0.38% | 273% | 121% | -2.16% | 0.45% | -0.37% | 263% | 125% |
| CfP 360 | -6.58% | -2.11% | -1.70% | 358% | 175% | -6.29% | -1.29% | -0.50% | 342% | 173% |
| CfP SDR-A | -4.35% | -1.87% | -1.92% | 260% | 147% | -4.50% | -1.73% | -1.79% | 239% | 153% |
| CfP SDR-B | -5.52% | -1.87% | -1.42% | 282% | 152% | -5.46% | -1.21% | -0.62% | 265% | 154% |
| CfP HDR-A | -6.34% | -2.89% | -2.62% | 299% | 167% | -5.99% | -2.19% | -1.52% | 285% | 171% |
| CfP HDR-B | -4.64% | 0.02% | 1.16% | 262% | 142% | -4.18% | 0.99% | 1.44% | 235% | 146% |
| JVET all | -4.51% | -1.33% | -1.41% | 281% | 144% | -4.62% | -0.77% | -0.90% | 264% | 148% |
| CfP all | -5.41% | -1.64% | -1.19% | 289% | 155% | -5.22% | -0.99% | -0.52% | 269% | 158% |
| JVET + CfP | -5.01% | -1.56% | -1.38% | 286% | 149% | -5.00% | -0.95% | -0.79% | 268% | 153% |
| Random Access |  |  |  |  |  |  |  |  |  |  |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| JVET A1 | -1.98% | 2.48% | 0.97% | 147% | 115% | -1.95% | 3.87% | 2.50% | 141% | 113% |
| JVET A2 | -2.28% | 0.37% | 0.71% | 143% | 102% | -2.49% | 0.40% | 0.79% | 137% | 101% |
| JVET B | -2.49% | -0.20% | -0.53% | 149% | 152% | -3.11% | -0.01% | -0.41% | 152% | 154% |
| JVET C | -1.48% | 0.57% | 0.23% | 150% | 111% | -1.71% | 0.91% | 0.72% | 149% | 108% |
| JVET D | -1.67% | -0.18% | 0.17% | 149% | 109% | -1.85% | -0.49% | 1.80% | 148% | 109% |
| JVET E | -4.05% | -1.47% | -2.20% | 148% | 107% | -4.42% | -0.75% | -1.39% | 146% | 108% |
| JVET F | -1.10% | 0.14% | 0.02% | 171% | 108% | -1.42% | 0.31% | 0.02% | 169% | 110% |
| CfP 360 | -4.20% | -0.16% | 0.61% | 172% | 108% | -4.49% | 0.46% | 1.62% | 174% | 108% |
| CfP SDR-A | -1.39% | 0.31% | 0.29% | 141% | 105% | -1.63% | 0.23% | 0.43% | 136% | 104% |
| CfP SDR-B | -2.20% | 0.24% | 0.18% | 144% | 128% | -2.52% | 1.19% | 1.25% | 143% | 128% |
| CfP HDR-A | -2.99% | -0.15% | 0.81% | 147% | 108% | -3.06% | 0.60% | 1.90% | 144% | 107% |
| CfP HDR-B | -2.82% | 0.86% | 2.10% | 162% | 132% | -2.77% | 1.33% | 1.07% | 160% | 133% |
| JVET all | -2.09% | 0.29% | -0.03% | 151% | 115% | -2.37% | 0.63% | 0.61% | 149% | 115% |
| CfP all | -2.70% | 0.25% | 0.80% | 153% | 116% | -2.88% | 0.78% | 1.20% | 151% | 116% |
| JVET + CfP | -2.44% | 0.20% | 0.31% | 152% | 115% | -2.66% | 0.65% | 0.88% | 151% | 115% |
| Low Delay |  |  |  |  |  |  |  |  |  |  |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| Cactus | -1.73% | 0.85% | -0.27% | 134% | 106% | -1.74% | 2.14% | 0.66% | 133% | 106% |
| BasketballDrive | -0.73% | 0.36% | -0.39% | 135% | 98% | -0.50% | 0.56% | 0.00% | 136% | 106% |
| BQTerrace | -1.84% | 0.48% | 3.82% | 134% | 106% | -3.54% | 0.83% | 3.60% | 140% | 106% |
| RitualDance | -1.97% | -0.29% | -0.61% | 144% | 109% | -1.65% | 0.40% | 0.92% | 139% | 108% |
| MarketPlace | -1.43% | -0.14% | 1.10% | 127% | 105% | -1.60% | -0.01% | 2.08% | 126% | 106% |
| All | -1.54% | 0.25% | 0.73% | 135% | 105% | -1.81% | 0.78% | 1.45% | 135% | 107% |

Table 5: Results for intra predictors (section 1.1) including simplifications (sections 1.1.6 and 1.1.7) and with four additional transforms per proposed mode (section 1.2) on top of SOT.

| Intra Only |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| JVET A1 | -2.45% | -1.18% | -1.06% | 224% | 135% | -2.32% | -0.92% | -0.69% | 229% | 134% |
| JVET A2 | -2.03% | -0.71% | -0.77% | 232% | 129% | -1.64% | -0.47% | -0.42% | 235% | 129% |
| JVET B | -2.66% | -1.15% | -0.90% | 224% | 137% | -2.55% | -0.99% | -0.84% | 229% | 137% |
| JVET C | -1.24% | -0.16% | -0.20% | 237% | 133% | -1.26% | 0.26% | 0.13% | 252% | 143% |
| JVET D | -1.63% | -0.31% | -0.39% | 213% | 120% | -1.73% | -0.22% | -0.09% | 216% | 122% |
| JVET E | -2.90% | -1.67% | -1.70% | 220% | 121% | -2.64% | -1.12% | -0.98% | 238% | 132% |
| JVET F | -0.88% | -0.21% | -0.31% | 230% | 113% | -1.03% | -0.08% | -0.40% | 236% | 117% |
| CfP 360 | -3.11% | -1.49% | -1.14% | 248% | 145% | -2.82% | -1.10% | -0.60% | 252% | 143% |
| CfP SDR-A | -1.77% | -0.89% | -0.86% | 221% | 125% | -1.69% | -0.75% | -0.81% | 223% | 126% |
| CfP SDR-B | -2.45% | -0.77% | -0.53% | 230% | 134% | -2.25% | -0.45% | -0.21% | 234% | 136% |
| CfP HDR-A | -2.73% | -1.17% | -1.11% | 208% | 122% | -2.53% | -0.75% | -0.65% | 214% | 124% |
| CfP HDR-B | -2.11% | 0.24% | 0.53% | 224% | 128% | -1.65% | 0.85% | 0.90% | 225% | 131% |
| JVET all | -1.96% | -0.75% | -0.73% | 226% | 127% | -1.88% | -0.50% | -0.47% | 233% | 131% |
| CfP all | -2.41% | -0.79% | -0.58% | 227% | 131% | -2.16% | -0.41% | -0.24% | 231% | 133% |
| JVET + CfP | -2.21% | -0.82% | -0.71% | 226% | 129% | -2.07% | -0.51% | -0.40% | 232% | 132% |
| Random Access |  |  |  |  |  |  |  |  |  |  |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| JVET A1 | -1.55% | 1.90% | 0.99% | 137% | 112% | -1.39% | 2.95% | 2.35% | 133% | 108% |
| JVET A2 | -0.83% | 1.07% | 0.82% | 133% | 104% | -0.80% | 1.03% | 0.85% | 133% | 104% |
| JVET B | -1.39% | -0.08% | 0.08% | 141% | 110% | -1.55% | 0.15% | 0.49% | 145% | 111% |
| JVET C | -0.72% | -0.06% | -0.08% | 131% | 107% | -0.74% | 0.61% | 0.49% | 133% | 108% |
| JVET D | -0.65% | -0.76% | -0.16% | 142% | 107% | -0.84% | -1.00% | -0.10% | 142% | 107% |
| JVET E | -1.72% | -0.07% | 0.28% | 140% | 106% | -1.94% | 1.17% | 0.59% | 143% | 106% |
| JVET F | -0.54% | -0.49% | -0.27% | 154% | 104% | -0.72% | -0.38% | -0.51% | 155% | 104% |
| CfP 360 | -2.04% | -0.43% | 0.57% | 151% | 108% | -2.10% | 0.14% | 1.53% | 154% | 106% |
| CfP SDR-A | -0.78% | 0.50% | 0.27% | 132% | 106% | -0.84% | 0.54% | 0.48% | 130% | 106% |
| CfP SDR-B | -1.30% | 0.32% | 0.46% | 137% | 110% | -1.30% | 0.60% | 0.88% | 139% | 110% |
| CfP HDR-A | -1.49% | 0.24% | 0.66% | 137% | 108% | -1.48% | 0.62% | 1.22% | 137% | 107% |
| CfP HDR-B | -1.38% | 0.99% | 1.69% | 152% | 109% | -1.29% | 0.98% | 0.89% | 154% | 107% |
| JVET all | -1.05% | 0.22% | 0.23% | 140% | 107% | -1.13% | 0.61% | 0.59% | 140% | 107% |
| CfP all | -1.39% | 0.33% | 0.74% | 142% | 108% | -1.40% | 0.57% | 0.98% | 143% | 107% |
| JVET + CfP | -1.23% | 0.21% | 0.41% | 141% | 108% | -1.28% | 0.58% | 0.72% | 142% | 107% |
| Low Delay |  |  |  |  |  |  |  |  |  |  |
|  | QP values 22, 27, 32, 37 |  |  |  |  | QP values 27, 32, 37, 42 |  |  |  |  |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| Cactus | -0.75% | 0.05% | 0.13% | 129% | 104% | -0.70% | 0.45% | -0.12% | 130% | 106% |
| BasketballDrive | -0.52% | -0.32% | -0.52% | 134% | 106% | -0.35% | -0.24% | -0.15% | 133% | 106% |
| BQTerrace | -0.52% | 1.68% | 0.81% | 126% | 104% | -1.11% | 1.50% | 2.29% | 133% | 105% |
| RitualDance | -1.34% | 0.45% | 0.14% | 136% | 107% | -1.28% | 0.79% | 0.73% | 133% | 108% |
| MarketPlace | -0.65% | 0.70% | -0.04% | 120% | 101% | -0.46% | 0.73% | 1.76% | 122% | 102% |
| All | -0.75% | 0.51% | 0.11% | 129% | 104% | -0.78% | 0.64% | 0.90% | 130% | 105% |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).