| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0038-v1 |
| --- | --- |

| Title: | Signal Adaptive Diffusion Filters For Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Jennifer Rasch, Michael Schäfer, Heiko Schwarz, Martin Winken, Anastasia Henkel, Mischa Siekmann, Detlev Marpe, Thomas Wiegand Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany | Tel: Email: | +49 30 31002-727 Jennifer.Rasch@hhi.fraunhofer.de, Jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute. |  |  |

_____________________________

# Abstract

In this document, diffusion filters are introduced that may be applied to the prediction signal generated by a hybrid video codec. Two types of diffusion filters are proposed: Linear and nonlinear diffusion filters. The linear diffusion filters correlate the extended prediction signal n times using a symmetric filter mask. The nonlinear diffusion filters use the input prediction signal to identify structures of the underlying signal and diffuse along edges rather than perpendicular to them. It is reported that the proposed diffusion filters lead to a coding gain of up to -1.45% in All Intra and -2.00% in Random Access configuration.

# Decoder algorithm description

## General algorithm description

It is proposed that the diffusion filters are applied to the prediction signal pred generated by the underlying codec. The filter may be applied for intra as well as inter predictions. Two types of diffusion filters are proposed: Linear and nonlinear diffusion filters.

### Linear Diffusion Filter

Mathematically, the linear diffusion filters are motivated by the following partial differential equation

∂∂tux,t=div∇ ux,t,

ux,0=pred(x),

where u denotes the filtered prediction and parameter t time which corresponds to iterations in a discrete setting. The linear diffusion filters are implemented as a symmetric filter mask

h =    14 010101010

Note that whenever possible, all 2D filter masks are implemented as 1D filters. The filtered prediction un is calculated by correlating n times the prediction signal pred  with the filter mask h, i.e., by calculating

un=h*…*hn times*pred

Here * denotes the application of the filter mask to the image, also called correlation. At each correlation step, at the right and lower boundary symmetric boundary conditions are assumed while at the top and left boundary equality with the reconstructed samples is assumed as shown in Figure 1.

prediction block

reconstructed samples

reflected samples

Figure 1 Extended prediction signal: the blue boundary denotes the boundary of reconstructed samples, the white block is the given prediction signal, the red boundary are the reflected samples.

It is proposed that the number n of iterations is equal to 5 or 15 iteration steps, in case of Inter and to 5 or 35 in case of Intra prediction signals as depicted in Table 3 below. The more iteration steps are carried out, the smoother the modified prediction will be. In the end, the filtered prediction is used as the modified prediction block.

### Nonlinear Diffusion Filter

While the linear diffusion filters smooth edges and smoother areas alike, the idea of the nonlinear diffusion filters is to locate edges using the given prediction signal and reduce the magnitude of diffusion at those locations. The implementation of the nonlinear diffusion filters is motivated by the following nonlinear differential equation

∂∂tu=divfJρ∇predx ∇u

with initial and boundary conditions as described above. Here, edges and their directions are detected  in terms of the gradient of the initial prediction signal. Therefore, the proposed nonlinear diffusion filter performs n correlations of the prediction signal pred with a mask h(x) that is different at each position x ϵR2 and is computed out of the prediction signal pred  as follows.

First, in order to identify edges, a Gaussian-smoothed diffusion tensor

Jρ∇pred  :=K* ∇pred  ∇pred T

is constructed, with a Gaussian kernel K. The discrete gradient ∇pred=EW, NST   is calculated using finite differences, namely by

EW≔ -000101000*pred   and   NS≔ 0100000-10*pred

using boundary conditions as described above. That means the gradient components EW and NS are given by simple differences of two sample values.

Then, for each sample location x ϵR2 inside the prediction block area, the 2x2 diffusion tensor is given by

4 ∇pred (x) ∇pred (x)T=EW(x)∙EWxEW(x)∙NSxEW(x)∙NSxNS(x)∙NSx ,

where the multiplication dot is understood to be pointwise. The Gaussian kernel is chosen as

K =    14 010121010

The Gaussian kernel is applied on the gradient matrices EW∙EW, EW∙NS and NS∙NS. This way, we receive a symmetric 2x2 matrix

Jρ∇pred (x)=14 gxxgxygxygyy(x)=14 ((K*(EW∙EW))x((K*(EW∙NS))x((K*(EW∙NS))x((K*(NS∙NS))x

for every x ϵR2 inside the prediction block area.

Secondly, the matrices Jρ∇pred (x) are diagonalized such that

λ1(x)00λ2(x)=cos⁡(θ)-sin⁡(θ)sin⁡(θ)cos⁡(θ) Jρ∇pred(x) cos⁡(θ)sin⁡(θ)-sin⁡(θ)cos⁡(θ),

where λ1(x), λ2(x) denote the eigenvalues of Jρ∇pred (x) and θ represents the associated rotation angle in the coordinate system. Using trigonometric formulas, it can be shown that

λ1(x)=14(gxx(x)-gxy(x)tanθ)

λ2(x)=14(gyy(x)-gxy(x)tanθ),

where 
tan⁡(2θ) =2gxy(x)gyy(x)-gxx(x)

The determination of the eigenvalues λ1,  λ2 is implemented using integer look-up tables. For that purpose, with a and b representing the rounded integers

a =256⋅2gxygyy-gxx       and      b =256⋅gyy-gxx2gxy

the following expressions are tabulated for a = 0..256 and b = 0..256

tab1 a =256⋅cos 0.5⋅atana256 2

tab2 a =256⋅cos 0.5⋅atana256 ⋅sin 0.5⋅atana256

tab3 a =256⋅sin 0.5⋅atana256 2

tab4 b =256⋅cos 0.5⋅atan256b 2

tab5 b =256⋅cos 0.5⋅atan256b ⋅sin 0.5⋅atan256b

tab6 b =256⋅sin 0.5⋅atan256b 2

where the square brackets [ ] specify rounding to the next integer.

Next, a function f  is applied to the eigenvalues λ1, λ2 of Jρ∇pred which serves as an edge detector: if eigenvalues λ1, λ2 are large, i.e. an edge is indicated, function f will be small. Function f is chosen in case of intra as an approximation of fs=exp⁡(-sμ), and in case of inter as fs=1 / (1+sμ) with parameter μ>0. In the implementation, the factor 14  can be incorporated into function f. The Parameter μ is a fixed value given by

μ=200,  &for intra prediction64,  &for inter prediction and QP<33550,  &for inter prediction and QP≥33

The two functions fs are tabulated using integer tables.

After applying the function f, the matrix is retransformed

fJρ∇predx:=cosθsinθ-sinθcosθfλ100f(λ2)cosθ-sinθsinθcosθ=: axxaxyaxyayy  (x)

Using this sample-wise definition, three functions axx, axy,ayy are defined with an entry for each sample location x ϵR2 inside the prediction block area. Similarly, as described above the rotation of the 2×2 matrix is implemented using the look-up tables given above.

Given the arrays axx(x), axy(x),ayy(x) for each sample location xϵR2, the following 8 arrays are derived using a simple averaging of two neighboring samples:

wN≔12010010000*ayy

wS≔12000010010*ayy

wW≔12000110000*axx

wE≔12000011000*axx

wO1≔14010001000* axy

wO2≔14000001010* axy

wO3≔14010100000* axy

wO4≔14000100010* axy

The integer weighting matrices wN, wS, wW, wE, wO1, wO2, wO3, wO4 are calculated at the beginning of the non-linear diffusion filter process. These matrices are used for the actual filtering in the iteration steps. For summarizing the approach, the integer weighting matrices are derived by the following ordered steps:

- Calculate discrete samples differences EW(x) and NS(x)

- Calculate products (EW⋅EW)(x), EW⋅NSx , (NS⋅NS)(x)

- Filter the three products with a 5-tap filter kernel for obtaining the gradient arrays gxx(x), gxy(x), and gyy(x)

- Determine the eigenvalues λ1(x), λ2(x) using look-up tables

- Apply a function f(s) to the eigenvalues using pre-defined look-up tables

- Derive the integer arrays axx(x), axy(x), and ayy(x) using look-up tables

- Derive the integer weighting arrays wN, wS, wW, wE, wO1, wO2, wO3, and wO4 using sample averages.

Given the arrays wN, wS, wW, wE, wO1, wO2, wO3, and wO4 for the sample locations xϵR2, the iterative filtering proceeds as follows. Let u0=pred represent the initial luma prediction signal. In each iteration step, the signal u is modified according to

ut+1=ut+14(wO1∙001000000*ut-wO2∙000000001*ut-wO3∙100000000*ut+wO4∙000000100*ut+wN∙0100-10000*ut+wW∙0001-10000*ut+wE∙0000-11000*ut+wS∙(0000-10010*ut))

Figure 2 Left: Original Intra Prediction, Middle: Linearly Filtered Intra Prediction, Right: Nonlinearly Filtered Intra Prediction

where the multiplication dot and the summation is to be understood pointwise. Mapping ut+1denotes the filtered prediction after t+1 iteration steps. This way, diffusion along edges is preferred over diffusion perpendicular to them. The nonlinear filter is applied to the prediction for two different amounts of iteration steps which corresponds to two nonlinear diffusion filter modes from which the encoder can choose. In case of Intra, it is proposed that the encoder can choose between 5 and 20 iteration steps and for Inter between 4 and 8 as depicted in Table 3 below. In the end, the filtered prediction u is used as the modified prediction block.

Figure 2 shows a comparison for the application of the linear and the nonlinear filter on an angular intra prediction signal. It is visible that while the linear filter blurres the edges of the underlying prediction, the nonlinear filter preserves them.

### Bitstream structure

For every CU, one flag is sent to indicate if the diffusion filter is enabled or not. If it is enabled, the index of the diffusion filter mode (in range 0 to 4) is sent. These bits are written into the bitstream after sending the intra prediction modes, reference indexes and motion vector information, respectively.

In the skip case, the diffusion filter is not allowed, thus no bits are sent. Additionally, the diffusion filter is not allowed in case the current CU lies on the left or top picture border. In case the intra prediction mode is DC or Planar, the diffusion is not allowed either. In case of merge, the diffusion filter can be merged according to the chosen candidate or tested on top of the merged prediction block. In the latter case, the diffusion filter information has to be sent additionally.
There are several options to constrain the usage of diffusion filters in order to reduce encoder run time and signaling:

- Restriction to larger blocks: The Diffusion filters are not allowed, if the area of CU is smaller or equal than 32.

- Restriction to larger blocks: The Diffusion filters are not allowed, if at least one side of CU is equal to 4 or 128 (except for 128 x 128).

- Reduction of tested diffusion filters: Only one linear and two non-linear filters are allowed for CUs with at least one side equal to 8.

The implemented restrictions a) – c) depending on block width and height are depicted in the table below. For each case the index number of the tested filters is given. Index 0 stands for the diffusion filter being disabled. The Indices 1 and 2 stand for the linear filter options, indices 3 and 4 for the nonlinear filter options.

| BlkSizes | 4 | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | 0 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0 |
| 16 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 32 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 64 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 128 | 0 | 0 | 0 | 0 | 0 | 0,1,2,3,4 |

Table 1 Restricted number of tested Filters ordered by block sizes, width versus height

- Temporal layer restriction: In inter case the amount of possible diffusion filters for non-reference frames (more specifically for temporal layer 4) is reduced.

### Entropy decoding

If the diffusion filter is enabled, the diffusion filter index in the range within 0 and 4 is binarized sending one bit to distinguish between linear and nonlinear filtering and another bit to determine which one of the two filter modes was chosen. This is depicted in the table below.

| diffFilterIdx | Bits |
| --- | --- |
| 0 | 0 |
| 1 | 100 |
| 2 | 101 |
| 3 | 110 |
| 4 | 111 |

Table 2 Binarization of the diffusion filter index

The syntax element diffFilterIdx indicates the filtering mode as well as the number of applied iterations as specified in the table below.

| cu_diffusion_filter_idx | filter type | number of iterations n |  |
| --- | --- | --- | --- |
|  |  | intra | inter |
| 0 | no filter | - | - |
| 1 | linear | 5 | 5 |
| 8 | linear | 15 | 35 |
| 3 | non-linear | 5 | 4 |
| 4 | non-linear | 20 | 8 |

Table 3 Interpretation of the syntax element diffFilterIdx

The restrictions are masked the following way. The config parameter FastInterDiffusionFilter can be set to 1 which enables restriction a). Another bit is send for restrictions b) - d) which results in setting FastInterDiffusionFilter to 3. An additional bit is send to indicate restriction d) which can be signaled by setting FastInterDiffusionFilter to 7. The detailed settings can be seen in the table below.

| FastInterDiffusionFilter | Restrictions | Bits |
| --- | --- | --- |
| 0 | no restrictions | 0 |
| 1 | restriction a) | 001 |
| 2 | restriction b) | 010 |
| 3 | restriction a), b) | 011 |
| 4 | restriction c) | 100 |
| 5 | restriction a), c) | 101 |
| 6 | restriction b), c) | 110 |
| 7 | restriction a) - c) | 111 |

Table 4 Binarization of the implemented restrictions

### Conversion process from entropy decoded data to symbols

There is one context for the enabling flag of the diffusion filter in case the CU is intra, is inter or inter-merge. The same distinction is done for the contexts used to determine if the filter is linear or nonlinear and which one of the filters it is. The latter bit has an additional set of contexts (intra, inter and inter-merge) for the linear and nonlinear filter case.

#### Intra prediction parameters

The diffusion filter is indicated by a CU parameter called diffFilterIdx. Its range is within 0 and 4, where diffFilterIdx >0 indicates that the filter is enabled and which one of the four modes was chosen.

#### Motion parameters

The diffusion filter is indicated by a CU parameter called diffFilterIdx. Its range is within 0 and 4, where diffFilterIdx >0 indicates that the filter is enabled and which one of the four modes was chosen.

# Encoder algorithm description

The diffusion filter is tested in the Intra case on top of the angular intra prediction modes that were chosen for a full RD cost check. All possible combinations of the intra prediction modes and diffusion filter modes are tested using a fast cost check and then those with minimal costs are chosen for a full RD cost check. In case that PbIntraFast is enabled, the list of options for the best diffusion filter combinations is reduced in the same way as the intra mode list. A full RD cost check is performed for the best option with linear and for the best option with nonlinear filter (if applicable). Only if one of these two is better than the best non-diffused option, further diffusion filter combinations are checked.

In case the non-diffused option resulted in a cbf equal to zero, there are no diffusion filter modes checked.

In case of inter, the diffusion filter is applied on the motion compensated prediction. In case of merge, it can be chosen between merging the diffusion filter candidate or testing it on top of the merged prediction and sending additional filter information. If the config parameter FastInterDiffusionFilter is enabled, there is a fast cost check performed and only the best diffusion filter mode is chosen for a full RD cost check. When performing the fast cost check, first only one linear and one nonlinear filter is tried. Only if the first filter of either kind performs better, the other one is tested.

# Experimental Results

The following results have been tested on a JEM like reference software using a HEVC configuration with QTGBS partitioning structure.

## All Intra, With Restrictions a)-c)

In the table below, the results for All Intra with restrictions 1.1.3 a) – c) (restriction d) has no impact on Intra) are depicted.

|  |  | All Intra Main10 (High Tier) |  |  |  |  |  | All Intra Main10 (Main Tier) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  | Y BD-rate | U BD-rate | V BD-rate |
| JVET A1 |  | -1.37% |  | -0.98% |  | -0.94% |  | -1.21% | -0.96% | -0.75% |
| JVET A2 |  | -1.21% |  | -0.93% |  | -0.92% |  | -1.07% | -0.72% | -0.74% |
| A3 |  | -1.45% |  | -0.75% |  | -0.84% |  | -1.41% | -0.72% | -0.66% |
| JVET B |  | -0.65% |  | -0.26% |  | -0.40% |  | -0.57% | -0.25% | -0.33% |
| B1 |  | -1.00% |  | -0.64% |  | -0.60% |  | -0.78% | -0.87% | -0.69% |
| CfP SDR-A |  | -0.98% |  | -0.70% |  | -0.69% |  | -0.91% | -0.53% | -0.47% |
| CfP SDR-B |  | -0.77% |  | -0.33% |  | -0.39% |  | -0.62% | -0.39% | -0.32% |
| CfP HDR-A |  | -0.91% |  | -0.59% |  | -0.52% |  | -0.80% | -0.55% | -0.35% |
| CfP HDR-B |  | -0.49% |  | -0.48% |  | -0.31% |  | -0.41% | -0.53% | -0.18% |
| Overall |  | -0.98% |  | -0.63% |  | -0.62% |  | -0.86% | -0.61% | -0.50% |
| Enc Time [%] |  | 118% |  |  |  |  |  | 117% |  |  |
| Dec Time [%] |  | 104% |  |  |  |  |  | 104% |  |  |

The detailed results, the sequence composition of the classes and other classes can be found enclosed in JVET-J0038_RestrFilters_TempLayer0.

## Random Access, No Additional Test In Merge Case, With Restrictions a)-d)

The results in the table below in the merge case, the diffusion filter is merged with the merge candidate. Restrictions 1.1.3 a) – d) are employed.

|  |  | Random Access Main10 (High Tier) |  |  |  |  |  | Random Access Main10 (Main Tier) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  | Y BD-rate | U BD-rate | V BD-rate |
| JVET A1 |  | -1.50% |  | -2.92% |  | -1.79% |  | -0.97% | -2.77% | -1.61% |
| JVET A2 |  | -1.50% |  | -1.86% |  | -1.35% |  | -0.78% | -0.93% | -0.53% |
| A3 |  | -0.74% |  | -0.44% |  | -0.41% |  | -0.64% | -0.22% | -0.21% |
| JVET B |  | -0.97% |  | -0.91% |  | -1.72% |  | -0.52% | -1.06% | -1.38% |
| B1 |  | -0.96% |  | -1.65% |  | -1.36% |  | -0.60% | -1.29% | -0.63% |
| CfP SDR-A |  | -1.13% |  | -1.49% |  | -1.09% |  | -0.63% | -0.76% | -0.45% |
| CfP SDR-B |  | -1.15% |  | -1.22% |  | -1.77% |  | -0.57% | -1.05% | -1.02% |
| CfP HDR-A |  | -0.83% |  | -0.96% |  | -1.30% |  | -0.76% | -0.88% | -1.07% |
| CfP HDR-B |  | -0.52% |  | -0.81% |  | -0.36% |  | -0.30% | -0.53% | -0.34% |
| Overall |  | -1.04% |  | -1.36% |  | -1.24% |  | -0.64% | -1.06% | -0.80% |
| Enc Time [%] |  | 112% |  |  |  |  |  | 111% |  |  |
| Dec Time [%] |  | 103% |  |  |  |  |  | 101% |  |  |

The detailed results, the sequence composition of the classes and other classes can be found enclosed in JVET-J0038_RestrFilters_TempLayer0.

## Random Access, Additional Test In Merge Case, No Restrictions

The results in the table below in the merge case, the diffusion filter is tested on top of the chosen merge candidate. Restrictions 1.1.3 a) – d) are not employed.

|  | Random Access Main10 (High Tier) |  |  | Random Access Main10 (Main Tier) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| JVET A1 | -2.00% | -4.46% | -2.00% | -1.52% | -5.12% | -2.57% |
| JVET A2 | -1.32% | -1.15% | -0.98% | -0.79% | -0.46% | -0.62% |
| A3 | -1.04% | -0.44% | -0.38% | -0.96% | -0.18% | -0.25% |
| JVET B | -1.08% | -0.62% | -1.16% | -0.71% | -0.76% | -0.70% |
| B1 | -0.89% | -1.82% | -1.70% | -0.74% | -1.76% | -1.58% |
| CfP SDR-A | -1.18% | -1.22% | -0.93% | -0.81% | -0.54% | -0.47% |
| CfP SDR-B | -1.17% | -1.15% | -1.44% | -0.75% | -1.13% | -0.80% |
| CfP HDR-A | -1.19% | -1.08% | -1.67% | -1.17% | -1.04% | -1.86% |
| CfP HDR-B | -0.58% | -0.90% | -0.45% | -0.40% | -0.79% | -0.26% |
| Overall | -1.16% | -1.43% | -1.19% | -0.87% | -1.31% | -1.01% |
| Enc Time [%] | 133% |  |  | 130% |  |  |
| Dec Time [%] | 101% |  |  | 100% |  |  |

The detailed results, the sequence composition of the classes and other classes can be found enclosed JVET-J0038_XMerge.xlsm.

## Random Access, Additional Test In Merge Case, With Restrictions a)-d)

In the table below results are depicted that are also tested additionally in the merge case. Here, restrictions 1.1.3 a) – d) are employed.

|  |  | Random Access Main10 (High Tier) |  |  |  |  |  | Random Access Main10 (Main Tier) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  | Y BD-rate | U BD-rate | V BD-rate |
| JVET A1 |  | -1.69% |  | -3.43% |  | -1.77% |  | -1.31% | -4.12% | -2.30% |
| JVET A2 |  | -1.14% |  | -1.30% |  | -0.59% |  | -0.68% | -0.78% | -0.30% |
| A3 |  | -0.90% |  | -0.48% |  | -0.71% |  | -0.87% | -0.46% | -0.66% |
| JVET B |  | -0.99% |  | -0.51% |  | -1.31% |  | -0.67% | -0.83% | -1.27% |
| B1 |  | -0.87% |  | -1.30% |  | -1.45% |  | -0.75% | -0.69% | -0.91% |
| CfP SDR-A |  | -1.02% |  | -1.35% |  | -0.96% |  | -0.73% | -0.89% | -0.59% |
| CfP SDR-B |  | -1.10% |  | -0.82% |  | -1.50% |  | -0.74% | -0.78% | -1.04% |
| CfP HDR-A |  | -1.05% |  | -1.11% |  | -1.75% |  | -1.08% | -1.18% | -2.11% |
| CfP HDR-B |  | -0.57% |  | -0.40% |  | 0.08% |  | -0.46% | -0.27% | -0.30% |
| Overall |  | -1.04% |  | -1.19% |  | -1.10% |  | -0.81% | -1.11% | -1.05% |
| Enc Time [%] |  | 131% |  |  |  |  |  | 131% |  |  |
| Dec Time [%] |  | 106% |  |  |  |  |  | 105% |  |  |

The detailed results, the sequence composition of the classes and other classes can be found enclosed JVET-J0038_XMerge_RestrFilters_TempLayer0.xlsm.

As can be seen in sections 3.3 and 3.4 in comparison to section 3.2, the additional test in the merge case increases the gains in the main tier configuration by around -0.2% but also adds an additional encoder cost of around 20%. The gains for the high resolution sequences shown here remains more or less the same for high tier but considering the lower resolution classes the overall gain increases using the additional merge case by about -0.1% as can be seen in the detailed results in the excel sheets.

Comparing the results of sections 3.3 and 3.4, it can be seen that the restrictions decrease the overall gains of the high resolution classes in random access shown here by around -0.1% but seem to decrease the run times by only 1-2%. But considering the results depicted in the attached detailed results, the impact of the restrictions can be seen in the lower resolution sequences (the overall run time decreases by around 5-7%) and in low delay where the encoder run times decreases by around 10%. This behaviour can be explained by the fact that due to the restrictions, the filter is not considered on smaller blocks which has an impact on the run time especially for lower resolution sequences. In case of low delay the complexity in run reduction can be explained by the impact of restriction d).

# Patent rights declaration

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).