| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0042-v1 |
| --- | --- |

| Title: | Intra block copy for intra-frame coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuguang Zuo Li Wang Fangdong Chen Xiaoyang Wu | Tel: Email: | zuoxuguang@hikvision.com wangli7@hikvision.com chenfangdong@hikvision.com xiaoyangwu@hikvision.com |
| Source: | Hikvision Cooperation, No. 555, Qianmo Road, Hangzhou, China |  |  |

_____________________________

- Abstract

This contribution proposes to use intra block copy (IBC) method for the intra-frame coding of both natural video and screen content video. In comparison to the IBC in HEVC screen content coding extension, there are several changes: (1) Independent implementation of IBC for luma and chroma. (2) Syntax change for luma picture. (3) Integrate with FRUC technique.

Introduction

IBC is a block matching technique wherein the current block is predicted by a reference block located in the already reconstructed region in the same frame. Figure 1 gives a brief illustration of IBC technique. IBC can exploit the non-local similarity in the frame can especially improve the coding efficiency of screen content. That is why IBC has been adopted in the HEVC SCC extension standard.

Fig.1 Illustration of IBC

Joint Exploration Model (JEM) is the platform used to implement and evaluate new coding tools for the next generation of video coding standard currently. To improve the accuracy of intra prediction, JEM increases the number of intra modes from 35 in HEVC to 67. Also it uses an improved planar mode P-PDPC to better handle the prediction of blocks with non-directional textures. Thus the efficiency of intra frame coding is improved. However, for blocks with complex non-directional textures, the modes in JEM may still not provide a good prediction. If there are blocks in the reconstructed region sharing similar pattern with the coding blocks, IBC technique can help provide more accurate prediction. So in this proposal the IBC technique is implemented in JEM and evaluated with both natural and screen content videos. The details of the technique are descried in Section 2.

Discription of IBC

Independent implementation of IBC for luma and chroma

In HEVC SCC, IBC mode regarded as inter mode, and the original intra frame are changed to P frames when IBC is used. In JEM, the coding of luma and chroma in intra frame are independent. So the partition of luma and chroma is different. In this way the efficiency of intra frame can be improved. To align with the partition design in JEM, the type of intra frame does not change in this proposal. In result, the implementation of IBC for luma and chroma is different.

First, for luma, the IBC mode is also regarded as inter mode. So in luma picture there are intra blocks and inter blocks. So the luma picture can be regarded as inter picture and the coding method is basically inherit from the inter picture. Each block should be signaled to indicate its prediction type. Also for blocks predicted by IBC, the residual is transformed using only one transform set, which consists of DST-VII and DCT-VIII. This is just the same way that the inter prediction residual is transformed in JEM.

For chroma, the IBC mode is added into the mode candidate list as an extra mode. In original intra frame, all chroma blocks are regarded as intra blocks. An intra mode candidate list is first built for each block and the optimal mode is chosen from the list for chroma block. The intra mode candidate is composed of 6 cross component prediction modes (CCLMs) and 5 traditional intra modes. The IBC mode is inserted in the list as behind the CCLMs. The length of the list keeps unchanged.

Syntax change for luma

It is said in Section 2.1 that coding method of luma picture is basically inherit from the inter picture. However, considering the changed proportions of inter blocks and intra blocks, the syntax is changed. In the coding of inter picture, a skip_flag syntax is first coded for each block. If the value of the syntax is false, then the pred_mode_flag syntax is coded to indicate the prediction type of the block. However, for the luma picture in intra frame, it is no longer efficient to code the skip_flag syntax for each block because the proportion of blocks choosing skip mode is rather low. Therefore, in this proposal the skip_flag syntax is omitted when encoding luma picture. Then for each luma block, the pred_mode_flag syntax is first encoded. Correspondingly, for inter blocks with merge mode, the cbf flag needs to be encoded. Because the original skip mode blocks become inter blocks with merge mode. Since there is no residual, the value of cbf flag is false and coded.

Integrate with FRUC

FRUC is an inter coding tool which can help the coding of motion information. It works by refining the merge candidate list and AMVP list to provide more accurate motion prediction. Since there is also motion vector to be coded for IBC mode block, the FRUC technique is enabled when constructing the merge and amvp list. Besides, there is only one reference list for IBC mode, so only the Template Mode in FRUC is used.

Motion estimation for IBC

For blocks smaller than or equal to 32x32, the same full search in HEVC SCC is exploited.

Experimental Results

The IBC technique is implemented on JEM 7.1 and tested under all intra configuration. The results are shown in Table 1. (Currently only the results tested by one frame is available, the results of full sequence will be supplemented)

Table 1 Comparison of IBC with JEM 7.1

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | Over 7.0CTC |  |  |
|  | Y | U | V |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -1.26% | -1.58% | -1.12% |
| Class C | -0.47% | -0.21% | -1.03% |
| Class D | -0.45% | -0.96% | -0.67% |
| Class E | -1.47% | -1.29% | -1.55% |
| Overall | -0.90% | -1.03% | -1.07% |
| Class F (optional) | -22.51% | -22.42% | -23.05% |

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).