| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0047-v1 |
| --- | --- |

| Title: | Improvement on top of Tencent’s CfP response |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Xiaozhong Xu, Xin Zhao, Jing Ye, Liang Zhao, Shan Liu, Meng Xu, Guichun Li | Tel: Email: | {xlxiangli, xiaozhongxu, xinzzhao, jingye, leolzhao, shanl, mengxxu, guichunli} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution summarizes the recent work on top of Tencent’s CfP response on SDR contents. Three elements of the response, i.e., block structure, intra block copy and merge candidate list construction, are reportedly refined. One encoder only feature, i.e., adaptive chroma QP offset, is newly introduced. It is reported that xxx, xxx and xxx luma BD rate reduction over Tencent’s CfP response (JVET-J0029), JEM and HM anchor was obtained, respectively.

# Introduction

In this proposal, Tencent’s SDR CfP response [1] is further improved. First, three elements of the response, i.e., block structure, intra block copy and merge candidate list construction, are improved. Second, one encoder only feature on adaptive chroma QP offset is newly proposed. In the following sections, these four parts will be discussed in detail.

# Improvement of block structure

The coding efficiency of block structure is improved from two aspects. First, a new tree type called SplitToSquare is introduced. Second, the signaling cost of tree type is reduced.

## SplitToSquare

The block structure is firstly improved by introducing a tree type called SplitToSquare. With this tree type, a block is split into largest same-size square sub-blocks. That is, if the input block is a rectangular block with the size of 2M×2N (M≠N), after SplitToSquare, we will have 2M+N-2×min(M,N) sub-blocks whose size are 2min(M,N)×2min(M,N). If the input block is a square block, Split2ToSquare leads to four square same-size sub-blocks, which is the same as the current quad-tree split. Basically, SplitToSquare may be used to replace the current quad-tree split as it covers more cases.

On the other hand, SplitToSquare is also to improve the case that a narrow rectangular block needs more splitting signaling to split to into small blocks. For example, a 16×4 block may be split into four 4×4 blocks with this tree type.

To keep relatively low complexity, Split2Square can only be used when the width to height ratio of a input block is 1:1, 1:4, or 4:1.

## Signaling cost reduction

The signaling cost of tree types is reduced by avoiding particular bins under certain conditions. First, the bin indicating block split is inferred as true and not signaled when the current block is partially outside picture boundary. Second, the bin indicating vertical or horizontal split is inferred and not signaled if the current block is outside the current picture either vertically or horizontally but not both. The split bin is inferred as along the picture boundary, i.e., it is inferred as vertical split if the block is across vertical boundary, or is inferred as horizontal split if the block is across horizontal boundary.

# Improvement of intra block copy

Compared to the technology described in Tencent’s corresponding CfP response [1], the improvements on IBC are summarized as follows. More details may be found in [2].

- IBC merge candidate list and inter merge candidate list is unified into one single list

- Move IBC signaling flag from before merge flag to after merge flag, and explicitly signal this flag when merge flag is false (non-merge case).

- Adaptive IBC block vector resolution (switch between 1-pel resolution and 4-pel resolution) is added to reduce the signaling cost of block vector difference.

# Improvement of merge candidate list construction

Compared to the technology described in Tencent’s CfP response [1], the following aspects are improved for merge candidate list construction.

- The additional merge candidates are extended from the JEM merge candidates. The left, above, left bottom, above right, and top left candidates that are not immediately next to the current block are checked. The detailed positions that are checked are described in Figure 1.

- The maximum number of merge candidates is reduced from 23 to 10.

Figure 1 Additional merge candidates

In Figure 1 the top left corner of the reference block has an offset of (-96, -96) to the current block. Each candidate B (i, j) or C (i, j) has an offset of 16 in the vertical direction compared to its previous B or C candidates. Each candidate A (i, j) or D (i, j) has an offset of 16 in the horizontal direction compared to its previous A or D candidates. Each E (i, j) has an offset of 16 in both horizontal direction and vertical direction compared to its previous E candidates. The candidates are checked from inside to the outside. And the order of the candidates is A(i, j), B (i, j), C (i, j), D (i, j), and E (i, j).

# Adaptive chroma QP offset

It is observed that significant luma BD-rate saving can be obtained by sacrificing some chroma quality when chroma components have much details. Therefore, an encoder only method is proposed to adjust the Cb and Cr QP offset signaled in slice header. For each slice, on top of any existing chroma QP offset, additional chroma QP adjustment may be applied, based on the PSNR values in a previously coded picture. If the luma and chroma PSNR difference is small, the chroma QP offset may be increased in slice header. More details may be found in [3].

# Simulation results

The proposed improvements are tested under the same conditions as those of Tencent’s CfP response including QP settings [1]. The table below summarized the simulation results over [1], JEM-7.0 anchor and HM-16.6 anchor, respectively. On average, xxx, xxx, and xxx BD-rate reduction is achieved over [1], JEM-7.0 anchor and HM-16.6 anchor, respectively.

# Conclusions

In this contribution, the recent work on top of Tencent’s CfP are summarized. Three elements of the response, i.e., block structure, intra block copy and merge candidate list construction are improved. One encoder only feature, i.e., adaptive chroma QP offset, is introduced. It is reported that xxx, xxx and xxx luma BD rate reduction over Tencent’s CfP response, JEM and HM anchor was obtained, respectively.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- X. Li, X. Xu, X. Zhao, J. Ye, L. Zhao, S. Liu, “Description of SDR video coding technology proposal by Tencent”, doc. JVET-J0029, April 2018.

- X. Xu, X. Li, G. Li, S. Liu, “Intra block copy improvement on top of Tencent’s CfP response”, doc. JVET-J0050, April 2018.

- M. Xu, X. Li, S. Liu, “Adaptive chroma QP offset on top of Tencent’s CfP response”, doc. JVET-J0055, April 2018.