| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0055-v1 |
| --- | --- |

| Title: | Adaptive chroma QP offset on top of Tencent’s CfP response |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, and Shan Liu Tencent America LLC 661 Bryant St, Palo Alto, CA 94301 | Tel: Email: | +1 650-798-3300 mengxxu@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution describes an encoder only method, i.e., adaptive chroma QP offset on top of Tencent’s CfP response JVET-J0029 [4] and implemented in [6]. It is reported that the proposed method brings xx%, 0.01% luma BD rate change for SDR constraint set 1 and 2, respectively.

# Introduction

In HEVC[1] and the NEXT software [2], Cb and Cr QP offsets are signaled in slice header. Then  QP mapping is applied during chroma QP derivation process. For example, a function QpC=f(qPi) illustrated in Table 1 is used to determine the scaled chroma QP QpC from the offset adjusted QP qPi for 4:2:0 format.

Table 1 Specification of QpC as a function of qPi for 4:2:0 in HEVC

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

It is observed that for some sequences, bigger chroma QP offset leads to better tradeoff between luma BD-rate gain and chroma BD-rate loss. In this contribution, an encoder only method is proposed to adaptively adjust the chroma QP offset.

# Algorithm description

At each slice, the luma and chroma PSNR values of a previously coded picture is examined. If the PSNR difference between luma and chroma is below a certain threshold, the adaptive chroma QP offset is activated for the current slice.

When adaptive chroma QP offset is activated, a QP QpW is computed as the weighted average of qPi and QpC=f(qPi) for Cb and Cr. Then the chroma QP offset Dqp is determined as the minimum value satisfying f(qPi+Dqp)QpW. Dqp is combined with the existing chroma QP offset and signaled in the slice header.

This is an encoder only change and does not affect the decoding process. It does not change the chroma QP for qPi<30. For qPi30, the chroma QP may be increased and consequently decreases the bit rate when adaptive chroma QP offset is activated. Although big luma BD-rate saving is observed for several sequences, it is unclear whether the proposed method would cause any noticeable artifact, since the perceptual quality has not been evaluated.

# Simulations

The adaptive chroma QP offset in this contribution is implemented in Tencent’s CfP response improvement [6]. The BD rate changes are shown in Table 2 for CfP sequences. The testing QPs are the same as those in Tencent’s corresponding CfP response [4]. The anchor and test are [6] without and with the proposed method.

Table 2 Simulation Results for Cfp Sequences

| Sequences | RA (Y) | RA (Cb) | RA (Cr) | Enc Time | Dec Time | LB (Y) | LB (Cb) | LB (Cr) | Enc Time | Dec Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FoodMarket4 |  |  |  |  |  |  |  |  |  |  |
| CatRobot1 |  |  |  |  |  |  |  |  |  |  |
| DaylightRoad2 |  |  |  |  |  |  |  |  |  |  |
| ParkRunning3 |  |  |  |  |  |  |  |  |  |  |
| CampfireParty2 |  |  |  |  |  |  |  |  |  |  |
| UHD Average |  |  |  |  |  |  |  |  |  |  |
| BQTerrace | 0.04% | 0.03% | 0.03% | 100% | 101% |  |  |  |  |  |
| RitualDance | 0.01% | 0.30% | 0.22% | 100% | 100% |  |  |  |  |  |
| MarketPlace | 0.01% | 0.02% | 0.01% | 100% | 99% |  |  |  |  |  |
| BasketballDrive | 0.02% | 0.02% | 0.02% | 100% | 99% |  |  |  |  |  |
| Cactus | -0.03% | 1.11% | 1.27% | 100% | 100% |  |  |  |  |  |
| HD Average | 0.01% | 0.30% | 0.31% | 100% | 100% |  |  |  |  |  |

# Conclusions

This proposal describes an encoder only method adaptive chroma QP offset on top of Tencent’s CfP response improvement. Simulations show that, xx%, 0.01% luma BD rate change for SDR constraint set 1 and 2 was achieved, respectively. It is proposed to adopt this method into test model.

# Refences

- Recommendation ITU-T H.265 | International Standard ISO/IEC 23008-2, High efficiency video coding: https://www.itu.int/rec/T-REC-H.265

- Wieckowski, T. Hinz, V. George, J. Brandenburg, J. Ma, S. De-Luxán-Hernández, B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “AHG10: Updated NextSoftware as an alternative implementation of the Joint Exploration Model (JEM)”, JVET-I0034, Gwangju, KR, 20–26 Jan. 2018.

- JVET Reference software Ver. 7.0 (JEM-7.0):  https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.0

- Xiang Li, et al, “Description of video coding technology proposal by Tencent”, JVET-J0029, San Diego, US, April 2018

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, JVET-H1002, Macau, CN, Oct. 2017

- X. Li, X. Xu, X. Zhao, J. Ye, L. Zhao, S. Liu, M. Xu, G. Li, “Improvement on top of Tencent’s CfP response”, doc. JVET-J0047, April 2018.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.