| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 April 2018 | Document: JVET-J0057 |
| --- | --- |

| Title: | DMVR extension based on template matching |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen, Jicheng An, Jianhua Zheng | Tel: Email: | +86-13480990067 anderson.chen@hisilicon.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

- Abstract

A decoder-side motion vector refinement extension algorithm is proposed. The algorithm is based on template matching and aims to reduce the bit-rate of motion vector by refining the motion vectors at decoder side. Compared to JEM7.0, an average bit-rate savings of 2.02% gain for RA with 22% encoding time increase and 17% decoding time increase is achieved under a tools-off configuration and around 0.14% BD-rate gain with 11% encoding time increase and 1% decoding time increase is achieved under common test condition.

Introduction

In current JEM, DMVR only apply for merge mode of bi-prediction with one from the reference picture in the past and the other from reference picture in the future.

In this contribution, DMVR extension method is proposed and can be applied on uni or bi-prediction not low-delay case based on template matching.

Proposed DMVR extension

The motion vector (MV) of list0 or MV of list1 are further refined by a template matching process. As shown in Figure 1, template matching performs a distortion-based search between a template (top and/or left neighbouring blocks of the current CU) in the current picture and a block (same size to the template) in a reference picture in order to obtain a refined MV without the transmission of additional syntax element.

Fig. 1 Template matching

In current JEM, AMVP has two candidates. With template matching method, MV of each candidates is refined. For merge case, MV refined based on template matching is only applied on the first candidate of merge candidate list. When applied to AMVP or Merge mode, only CU level search is applied.

DMVR extension method also be applied on another new prediction mode (AM mode). AM mode is a new bi-prediction mode that is the combination mode of Merge and AMVP, which one direction using Merge method to get the prediction block and the other direction use AMVP. In this case, MV refined is only applied on the Merge direction and only CU level search is applied.

Simulation results

The proposed DMVR extension has been implemented on top of JEM7.0 [1]. Two configurations are tested. The first one is the common test condition (CTC) [2] but with some JEM7.0 tools disabled (with command line settings as --EMT=0 --NSST=0 --PDPC=0 --FRUC=0 --OBMC=0 --IlluCompEnable=0 --AFFINE=0 --ATMVP=0 --IMV=0 --BIO=0 –DMVR=0 –BilateralFilter=0 –ALF=0) for fast evaluation and eliminating the possible impact of new JEM7.0 tools. The second one is the CTC.

Table 1 shows the results of tools-off test. Table 2 shows the results of CTC test.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 ToolsOff |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class UHD1 | -2.29% | -2.19% | -2.37% | 119% | 118% |
| Class HD1 | -1.74% | -1.66% | -1.73% | 125% | 117% |
| Overall (Ref) | -2.02% | -1.92% | -2.05% | 122% | 117% |

Table 1. DMVR extension performance under the tools-off configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JEM7.0 AllToolsOn |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class UHD1 | -0.18% | -0.14% | -0.21% | 111% | 100% |
| Class HD1 | -0.11% | -0.10% | -0.10% | 112% | 102% |
| Overall (Ref) | -0.14% | -0.12% | -0.15% | 111% | 101% |

Table 2. DMVR extension performance under CTC

It can be seen that the proposed DMVR extension can achieves averagely 2.02% BD-rate gain for tools-off RA configuration with 22% encoding time increase and 17% decoding time increase. When tested under CTC RA configuration, it can provide 0.14% BD-rate gain with 11% encoding time and 1% decoding time increase.

Conclusion

This contribution proposes a new decoder-side motion vector refinement algorithm which is based on template matching and aims to reduce the bit-rate of motion vector by refining the motion vectors at decoder side. In the tools-off test, an average bit-rate savings of 2.02% with 22% encoding time increase and 17% decoding time increase is observed compared to JEM7.0. In CTC, it can provide 0.14% BD-rate gain with 11% encoding time and 1% decoding time increase.

References

[1] J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm Description of Joint Exploration Test Model 7”, JVET-G1001, July, 2017.

[2] K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-B1010, Feb, 2016.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).