| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0058-v1 |
| --- | --- |

| Title: | Merge mode modification on top of Tencent’s software in response to CfP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye, Xiang Li, and Shan Liu Tencent America LLC 661 Bryant St, Palo Alto, CA 94301 | Tel: Email: | +1 650-798-3300 jingye@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution describes the technical aspects of merge mode complexity reduction on top of Tencent’s CfP response [1]. First, the proposed method directly extends the spatial merge candidates from the nearest neighbor of current block to an outer reference region in NEXT software. Second, the proposed method reduces the maximum merge candidates number from 23 to 10. It is reported similar RA luma BD rate reduction as Tencent’s CfP response [1] for SDR constraint set 1.

# Introduction

In NEXT software [2], a merge mode for Inter-picture prediction is introduced. A merge candidate list of motion parameters from neighboring blocks is constructed. Then an index is signaled which identifies the candidates to be used. During the merge candidate list construction, the merge candidates from the nearest neighboring block of current block are inserted into the merge candidate list. In Tencent’s CfP response [1], the merge candidates from outer regions of the reference blocks are also inserted into the merge candidate list. Each 16x16 block’s motion parameters from the outer region are inserted into the merge candidate list after pruning. The maximum number of merge candidates is increased from 7 to 23. To reduce the maximum number of merge candidates while keeping coding performance, a new merge list construction method is proposed.

# Algorithm description

The implementation of merge mode modification in this contribution is based on Tencent’s corresponding CfP response [1]. After the submission of CfP, a few improvements have also been done. The following are a list of technical aspects for merge mode in this document.

- The additional merge candidates will be a direct extension of the NEXT merge candidates. The left, above, left bottom, above right, and top left candidates that are not immediately next to the current block are checked. The detailed positions that are checked are described in Figure 1.

- The maximum number of merge candidates is reduced to 10 from 23.

Figure 1 Proposed merge candidate from outer region

In Figure 1 the top left corner of the reference block has an offset of (-96, -96) to the current block. Each candidate B (i, j) or C (i, j) has an offset of 16 in the vertical direction compared to its previous B or C candidates. Each candidate A (i, j) or D (i, j) has an offset of 16 in the horizontal direction compared to its previous A or D candidates. Each E (i, j) has an offset of 16 in both horizontal direction and vertical direction compared to its previous E candidates. The candidates are checked from inside to the outside. And the order of the candidates is A (i, j), B (i, j), C (i, j), D (i, j), and E (i, j).

# Simulations

The proposed merge mode tool is implemented on top of [1]. The simulation results are summarized as follows. In Table 1, [1] is used as the anchor and the test is the proposed method. The same QP settings as in Tencent’s CfP response [1] are used for both anchor and test. From the results, it can be observed that -0.06% luma coding gain is obtained by the proposed method without adding many merge candidates. In Table 2, the proposed method off is used as the anchor and the test is the proposed method. The same QP settings as in Tencent’s CfP response [1] are used for both anchor and test. From the results, it can be observed that -0.92% luma coding gain is obtained by the proposed method.

- Simulation Results for CfP Sequences

|  | Over Tencent’s CfP response |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class UHD | -0.08% | -0.07% | -0.04% | 98% | 107% |
| Class HD | -0.05% | 0.06% | 0.12% | 97% | 107% |
| Overall (Ref) | -0.06% | -0.01% | 0.04% | 98% | 107% |

- Simulation Results for CfP Sequences

|  | Over Tencent’s CfP response without this tool |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class UHD | -0.91% | -1.04% | -1.22% | 109% | 101% |
| Class HD | -0.92% | -0.92% | -0.85% | 108% | 100% |
| Overall (Ref) | -0.92% | -0.98% | -1.03% | 108% | 101% |

# Conclusions

This proposal describes an improvement on merge candidate list construction. With this method, the maximal number of merge candidates is kept as low as 10. Compared to the anchor without this tool 0.92% luma BD rate reduction for SDR constraint set 1 was achieved.

# Refences

- Xiang Li, X. Xu, X. Zhao, J. Ye, L. Zhao, S. Liu, “Description of video coding technology proposal by Tencent”, JVET-J0029, San Diego, US, April 2018

- Wieckowski, T. Hinz, V. George, J. Brandenburg, J. Ma, S. De-Luxán-Hernández, B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “AHG10: Updated NextSoftware as an alternative implementation of the Joint Exploration Model (JEM)”, JVET-I0034, Gwangju, KR, 20–26 Jan. 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).