| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0065-v1 |
| --- | --- |

| Title: | Further investigations on multi-line intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, two methods are proposed on top of multi-line intra prediction. Firstly, the reference line index is only signaled for even directional intra modes. For the remaining directional modes together with Planar and DC, reference line index is not signaled and only the first reference line can be used for intra prediction. Secondly, the reference samples buffers are extended such that the reconstructed values of reference samples are used for 4-tap intra interpolation when they are available. Simulation results reportedly show that average XX BD-rate gain is achieved on top of Tencent’s CfP response JVET-J0029 for all intra (AI).

Introduction

Multi-line intra prediction [1], [2] was proposed to use more reference lines for intra prediction, and encoder decides and signals which reference line is used to generate the intra predictor. The reference line index is signaled before intra prediction modes, and Planar/DC modes are excluded from intra prediction modes in case a nonzero reference line index is signaled. In Figure 1, an example of 4 reference lines is depicted, where each reference line is composed of six segments, i.e., Segment A to F, together with the top-left reference sample. In addition, in [1], Segment A and F are padded with the closest samples from Segment B and E, respectively.

Figure 1 Four reference lines adjacent to a coding block unit

Proposed method

Mode-dependent reference line selection

In this contribution, it is proposed that the signaling of reference line index is dependent on the intra prediction modes. For directional intra prediction modes with an even mode index, e.g., mode 2, 4, 6… 66, each of the four reference lines can be used and the selection is signaled. However, for the remaining modes, i.e., Planar/DC and directional intra prediction modes with odd mode index, only the first reference line, i.e., reference line 0, can be used and reference line index is not signaled.

As depicted in Figure 2, the directional intra modes with even mode indices are indicated by black solid arrows, and the directional intra modes with odd mode indices are indicated by the red dotted arrows.

Figure 2 Intra prediction modes for next software [3]

Reference sample extension

In this contribution, instead of padding the closest sample from Segment B and E, the reconstructed values of reference samples in Segment A and F are used when they are available. In addition, the reference sample buffer is extended by two samples, as described by the samples highlighted with yellow in Figure 1, so that the reconstructed values of reference samples are always available for a 4-tap interpolation.

Experimental results

The proposed scheme is implemented on top of Tencent CfP software proposed in JVET-J0029 [5]. The simulations are performed following all intra (AI) configuration on Linux cluster, and the test sequences are those defined in the common test condition described in [4]. The anchor used in Table 1 is Tencent CfP software and the test is Tencent CfP software with proposed methods.

Table 1: Proposed method over Tencent CfP software.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -0.12% | -0.10% |  |  |
| Class C | -0.28% | -0.24% | -0.28% |  |  |
| Class D | -0.27% | -0.14% | -0.31% |  |  |
| Class E | -0.23% | -0.16% | -0.23% |  |  |
| Overall |  |  |  |  |  |

References

- Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, and C.-C. Lin, “Arbitrary reference tier for intra directional modes,” JVET-C0043, Geneva, CH, May 2016.

- J. Li, B. Li, J. Xu, R. Xiong, G. J. Sullivan, “Multiple line-based intra prediction,” JVET-C0071, Geneva, CH, May 2016.

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, and J. Boyce, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7),” JVET-G1001, Torino, IT, July 2017.

- K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-G1010, Jul. 2017.

- X. Li, X. Xu, X. Zhao, J. Ye, L. Zhao, S. Liu, “Description of SDR video coding technology proposal by Tencent,” JVET-J0029, San Diego, US, 10–20 Apr. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).