| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0066-v2 |
| --- | --- |

| Title: | Complexity Reduction for Adaptive Multiple Transforms (AMTs) using Adjustment Stages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi Egilmez, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents a new technique to reduce the complexity of AMT by providing an efficient alternative that approximates the AMT transforms using only a transform of the DCT-2 family and adjustment stages of low complexity.  The proposed adjustment stages are defined using sparse block-band orthogonal matrices, which provide good approximation for the set of AMTs used in JEM7. It is shown that employing matrices with not more than 4 nonzero elements per row results in very small changes in coding gains (on average less than 0.05% in BD-rate under CTC conditions).

# Introduction

In JEM7, five types of discrete cosine and sine transforms (DCTs and DSTs) are employed for primary separable transformation of residual blocks, as shown in Fig. 1. Supporting multiple transforms has several consequences related to complexity and computational costs. For instance, not all transforms can be implemented efficiently for all dimensions, and they may require different types of hardware designs. This proposal addresses these problems by using a unified (i.e., a single type of) transform implementation and additional processing stages of very low complexity, with practically no loss in coding performance.

Figure 1: Current JEM-7.1 Adaptive Multiple Transform.

# Proposed Method

This contribution exploits the fact that the DCT-3, DST-2, and DST-3 transforms (which we call the DCT-2 family of transforms) all can be interpreted as variants of the DCT-2 with changes that are computationally trivial.. The relationships between these trigonometric transform matrices are:

DCT-2:		T,

DCT-3:		Tt,

DST-2:		F Tt S,

DST-3:		S T F,

where matrices F and S, which perform vector reflection (“flipping”) and sign changes, respectively, are defined by [2, §4.2]

Fm,n=1,if n=N-1-m,0,otherwise,

Sm,n=(-1)m,if n=m,0,otherwise.

This proposal’s transform method approximates different types of DCTs and DSTs in JEM7 by applying adjustment stages (pre-processing and post-processing) to a transform in the DCT-2 family, as shown in Fig. 2. The type of transform used for each case is shown in Table I.

The adjustment stages are defined by sparse block-band orthogonal matrices (see Fig.3), which are easy to compute and very similar to filtering with small number of taps. Our experiments show that adjustment stages using “4-tap” sparse block-band matrices (as shown in Fig.1) provide a good coding gain and memory trade-off. Note that because each block-band matrix is orthogonal, its inverse is simply the transpose, which has identical properties. The adjustment is a pre-processing stage in the forward AMT transform (as shown in Fig. 2), and its inverse is a post-processing stage in the inverse AMT transform.

Figure 2: Proposed solution.

Table I: Transforms of the DCT-2 family that are employed for each of the AMT transforms.

| AMT transform | DCT-5 | DCT-8 | DST-1 | DST-7 |
| --- | --- | --- | --- | --- |
| Forward transform | DCT-2 | DCT-3 | DST-2 | DST-3 |
| Inverse transform | DCT-3 | DCT-2 | DST-3 | DST-2 |

The proposed technique only employs DCT-2, which is by far the most optimized transform among different DST/DCT types, and sparse block-band matrices have very little computational and memory complexity over DCT-2. So, the overall complexity is significantly lower than using full DCT/DST transforms as in JEM7. Moreover, the transform implementation is unified by means of using only DCT-2 with simple pre- and post-processing operations using sparse matrices.

Figure 3: Example of sparse block-band matrices with 4 “taps”, on a 16×16 matrix. White squares represent elements that must be zero, and gray squares represent element that can be different from zero.

In this contribution, we specifically apply the proposed approximation methods for all five types of DCT and DSTs in JEM7, with dimensions 16, 32, and 64.

Since the exact complexity of the overall transforms depends on the implementation of the DCT-2 related transforms, we just indicate the additional complexity. For a transform of dimension N, the complexity of each adjustment stage, using 9 bits to store each matrix element, and up to 4 non-zero elements per row, is

| Transform dimension | N = 16 | N = 32 | N = 64 |
| --- | --- | --- | --- |
| Memory | 576 bits | 1,152 bits | 2,304 bits |
| Multiplications | 64 | 128 | 256 |

# Simulation Results

Tables II and III show the simulation results obtained using the proposed method on JEM-7.1 using adjustment matrices with up to 4 nonzero elements per row, and Tables IV and V show the results with matrices with up to 6 nonzero elements per row. As can be seen in the tables the coding losses are in all cases very small, and for that reason the option with 4 nonzero elements is recommended.

In all tables the computation times are shown for completeness. The implementation was not optimized for the JEM-7 SW. In addition, to simplify the initial implementation of this proposal, the optimizations of JVET-D0077 were disabled (a lossless change that only alters execution time, as can be see in Tables VI and VII), and the anchor in Tables II to V is modified accordingly.

Table II: Results for AI tests, adjustment matrices with up to 4 non-zero entries per row.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM-16.6-JEM-7.1 - D0077 off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.18% | 0.19% | 101% | 97% |
| Class A2 | 0.06% | 0.03% | 0.10% | 102% | 97% |
| Class B | 0.03% | 0.09% | 0.13% | 102% | 98% |
| Class C | 0.01% | -0.01% | 0.11% | 105% | 105% |
| Class D | 0.01% | 0.05% | -0.01% | 107% | 109% |
| Class E | 0.03% | -0.06% | 0.11% | 109% | 106% |
| Overall | 0.03% | 0.05% | 0.11% | 104% | 101% |

Table III: Results for RA tests, adjustment matrices with up to 4 non-zero entries per row.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM-16.6-JEM-7.1 - D0077 off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.06% | 0.14% | 101% | 99% |
| Class A2 | 0.04% | 0.12% | 0.22% | 100% | 100% |
| Class B | 0.05% | -0.13% | 0.05% | 101% | 99% |
| Class C | 0.00% | 0.04% | 0.03% | 99% | 98% |
| Class D | 0.05% | 0.26% | 0.24% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 0.04% | 0.06% | 0.13% | 100% | 99% |

Table IV: Results for AI tests, adjustment matrices with up to 6 non-zero entries per row.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM-16.6-JEM-7.1 - D0077 off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.05% | 102% | 98% |
| Class A2 | 0.00% | -0.03% | -0.02% | 102% | 98% |
| Class B | 0.01% | -0.01% | 0.05% | 103% | 98% |
| Class C | 0.00% | 0.03% | 0.09% | 106% | 106% |
| Class D | 0.00% | 0.03% | 0.10% | 108% | 109% |
| Class E | 0.01% | -0.05% | 0.02% | 110% | 107% |
| Overall | 0.00% | 0.01% | 0.05% | 105% | 102% |

Table V: Results for RA tests, adjustment matrices with up to 6 non-zero entries per row.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM-16.6-JEM-7.1 - D0077 off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | 0.05% | 101% | 99% |
| Class A2 | 0.03% | 0.09% | 0.21% | 101% | 99% |
| Class B | 0.02% | -0.08% | -0.08% | 101% | 99% |
| Class C | -0.04% | 0.00% | 0.17% | 100% | 98% |
| Class D | 0.00% | -0.02% | 0.22% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 0.01% | -0.03% | 0.11% | 101% | 99% |

Table VI: Results for AI tests, JVET-D0077 macro off.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM-16.6-JEM-7.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 103% | 111% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 110% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 107% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 96% | 93% |
| Class E | 0.00% | 0.00% | 0.00% | 95% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 104% |

Table VII: Results for RA tests, JVET-D0077 macro off.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM-16.6-JEM-7.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 104% | 104% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 103% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall (Ref) | 0.00% | 0.00% | 0.00% | 103% | 102% |

# References

- JEM Software, https://jvet.hhi.fraunhofer.de/svn/svn_HMJEMSoftware/tags/HM-16.6-JEM-7.0.

- V. Britanak, P. C. Yip, and K. R. Rao, Discrete Cosine and Sine Transforms: General Properties, Fast Algorithms and Integer Approximations, Academic Press, 2007.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).