| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 31st Meeting: San Diego, US, 13–20 Apr. 2018 | Document: JVET-J0073-v1 |
| --- | --- |

| Title: | Dynamic viewports for 360° video CfP subjective testing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

The dynamic viewports used for the Call for Proposal subjective testing of the 360° video category were not made available in advance to the proponents. This document describes the dynamic viewports provided to the test coordinator for use in subjective tests. The attachment contains the dynamic viewport files used as inputs to the 360Lib software.

# Description

For each test sequence, a single dynamic viewport path was selected. Each dynamic viewport path contained two piece-wise linear segments. When the dynamic paths were selected, care was taken to avoid overly fast motion.

start

end

mid

Figure 1. Piece-wise linear illustration of start, mid, and end of dynamic viewport center

Table 1 shows (yaw, pitch) for the start, midpoint, and end point representing the two linear segments. The attachment contains the input files for the 360Lib software.

Table 1. Dynamic viewports

| Sequence | (yaw_start, pitch_start) | (yaw_mid, pitch_mid) | (yaw_end, pitch_end) |
| --- | --- | --- | --- |
| Trolley | ( 340, 0) | ( 310, 0) | ( 280, -10) |
| Chairlift | ( -100, -10) | ( -100, 0) | (-80, 20) |
| KiteFlite | ( -29, 8) | (-59, 8) | ( -89, 18) |
| Harbor | ( 40, -12) | (8, 6) | ( -20, 18) |
| Balboa | ( 30, 0) | (25, 3) | (25, 3) |