| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0085-v1 |
| --- | --- |

| Title: | BoG report on 360 video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on Apr 14 with the primary goal of preparing a survey of the proposed technologies includes in responses to the Call for Proposals in the 360° video category. A spreadsheet containing the prepared summary is attached.

Some questions were raised during the BoG meeting regarding the plans for the 360Lib software. Currently, the 360Lib software is integrated with the HM and JEM. When a Test Model is defined, should the 360Lib software or a variant of it be integrated with it as well? For experiments involving 360°-specific coding tools such as those included in some CfP responses, some sort of integration of projection mapping and the codec is desirable.

Several new projection formats were proposed by proponents for inclusion in 360Lib, but BoG did not discuss yet, since it depends on the general plans for 360Lib. It was suggested to define a CE on projection formats.

# Discussion

In addition to the summary include in the spreadsheet table, some additional points were captured when discussing some of the contributions, which are noted below.

## JVET-J0019

Proposes MCP projection format plus coding tools.

MCP Format:

- For top and bottom faces use EAC

- For other four faces, use EAC in horizontal dimension, and other projection in vertical dimension

- Proposing MCP be added to 360Lib

Coding tools:

- For inter, derive projected MV from spherical neighbor block

- For other cases, consider neighbors unavailable if across dis. boundary

Coding gains (not included in CfP response individually):

- 0.35% gain for MCP vs. EAC with HM

- 1.08% for inter

- 0.45% for intra

- 0.2% for loop filter (asserts bigger impact on subjective quality)

## JVET-J0021

Coding tools average 2.2% gain for Low Complexity

## JVET-J0024

RSP with content-adaptive spatial adjustment, signaled per sequence. Should really be signaled per RAP.

0.2% gain for RSP change.

## JVET-J0022

CU-adaptive QP based on spatial position.

## JVET-J0033

Proposes PAU projection format. Similar to J0019 in that one dimension’s projection function is changed, but new projection function differs.

No coding tool changes.

0.2% coding gain for PAU vs EAC.

Proposing PAU be added to 360Lib.

## J0015

Proposes HAC adaptive projection mapping function. Is a generalization of EAC.

Signal 2 parameters per face, 12 total. Can change for each RAP.

Conversion of reference frames between projection mapping formats is used (for open GOP.)

Proposing HAC be added to 360Lib.

From tool-on test, coding gains:

- 0.32% for HAC over EAC

- 0.54% for adaptive frame packing

## 0.33% for face discontinuity (but more subjective impact)

- 1.62% for geometry padding

## Other discussion

Question: Should there be some type of a “Test Model” for 360Lib type functionality? Should some 360Lib-like software have some new status?

Suggest to have a CE on projection formats, to study proposed new formats and existing formats.

The CE would bring experimental results using the CTC. Will need to define CTC for projection formats and for 360 coding tools using the new test Model.

Which codec to use for projection formats? New Test Model would require integration work

Consider removing unused formats from 360Lib.

Raise question about 360Lib status in track, and hold additional BoG session afterwards.