| hJoint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0086 |
| --- | --- |

| Title: | Proposal for starting point of Test Model development |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Marta Karczewicz (Qualcomm) Edouard Francois (Technicolor) Yan Ye (Interdigital) | Tel: Email: | +1-858-658-5673 wchien@qti.qualcomm.com martak@qti.qualcomm.com edouard.francois@technicolor.com yan.ye@interdigital.com |
| Source: | Qualcomm Inc., Technicolor, InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution evaluates two tool configurations on JVET-J0021 and JVET-J0035 software bases. Test 1 configuration offers lower complexity with 35.8% and34.3% bitrate reduction on the luminance component for JVET-J0021 and JVET-J0035, respectively. Test 2 configuration provides 37.2% and  36.5% bitrate reduction on the luminance component with added decoder side complexity.

# Introductions

In this contribution, a subset software of JVET-J0021[1]/JVET-J0022[2] is provided. Specifically, sign prediction, intra propagation, LM angular mode, SAO merge, multi-reference intra prediction, adaptive clipping, and motion compensated padding, are removed from the JVET-J0021 software. Two modifications were applied to adaptive loop filter and Bilateral matching, respectively. Instead of 2x2 classification, 4x4 classification is used in ALF. For Bilateral matching, bilinear filter is replacing DCT-IF in the refinement stage and search range is reduced to 2 from 8. The simplification of subblock merge candidate, Bilateral matching, BIO,  and OBMC proposed in JVET-J0015[3], are also integrated to reduce the computational complexity of the encoder and decoder. In addition, NextSoftware with JEM tools provided in JVET-J0035 [4] was used to study the coding performance and computation complexity of different settings on the coding tools.

# Tool settings

Two software were used in this contribution. One is based on JVET-J0021 and the other one is JVEG-J0035 NextSoftware, which contains JEM tools on extended QTBT partition structure. In all experiments, the partition configurations are set to QTBT + TT.  On coding tools, adaptive clipping is disabled in both software. In addition, sign prediction, intra propagation, LM angular mode, SAO merge, multi-reference intra prediction, and motion compensated padding are also disabled in JVET-J0021.  Table 1 summarizes all the tools enabled in each software and Table 2 lists additional tools enabled in the experiments. The primary goal of enabling the 9-tap deblocking filter in JVET-J0021 is to improve visual quality of the decoded pictures and to capture the commonality among several responses [1, 2, 6-9] to the joint Call for Proposal.

Table 1. Test 1 settings

| Tools enabled in JVET-J0021 | Tools enabled in JVET-J0035 |
| --- | --- |
| - QTBT+TT (CTU size 128) - Intra prediction - EMT+NSST - Entropy coding changes - Affine motion - GALF - Merge candidate changes - Integer motion vector precision - Bilateral motion refinement - Bilateral filter - 9 tap deblocking filter | - QTBT+TT (CTU size 128) - Intra prediction - EMT+NSST - Entropy coding changes - Affine motion - GALF - Subblock merge candidate - Integer motion vector precision - Decoder motion vector refinement - Bilateral filter |

Table 2. Test 2 setting

| Additional tools enabled in JVET-J0021 | Additional tools enabled in JVET-J0035 |
| --- | --- |
| - OBMC - BIO | - OBMC - BIO |

# Compression performance for constrained set 1

The following tables summarize the test results for Constraint Set 1 relative to HM anchor. The simulation was conducted with the test sequences in CfP[5] with JEM anchor QP settings. Note that the tables are updated with decoding time of concatenated bitstream. Two set of results are presented:

## Test 1

Table 2. JVET-J021

|  | HM Anchor |  |  | JEM Anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Average SDR-A | -39.3% | -41.9% | -42.8% | -5.9% | -0.4% | -2.0% |
| Average SDR-B | -32.4% | -45.6% | -45.8% | -3.0% | -3.6% | -2.6% |
| Average all | -35.8% | -43.8% | -44.3% | -4.4% | -2.0% | -2.3% |
| Enc Time[%] | 445% |  |  | 59% |  |  |
| Dec Time[%] | 190% |  |  | 21% |  |  |

Table 3. JVET-J035

|  | HM Anchor |  |  | JEM Anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Average SDR-A | -39.3% | -26.5% | -28.8% | -5.5% | 23.8% | 19.5% |
| Average SDR-B | -29.3% | -30.5% | -30.0% | 1.6% | 19.6% | 20.9% |
| Average all | -34.3% | -28.5% | -29.4% | -1.9% | 21.7% | 20.2% |
| Enc Time[%] | 478% |  |  | 63% |  |  |
| Dec Time[%] | 277% |  |  | 30% |  |  |

## Test 2

Table 4. JVET-J021

|  | HM Anchor |  |  | JEM Anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Average SDR-A | -40.6% | -42.9% | -44.1% | -8.0% | -2.1% | -4.0% |
| Average SDR-B | -33.9% | -46.7% | -46.9% | -5.1% | -5.8% | -4.6% |
| Average all | -37.2% | -44.8% | -45.5% | -6.5% | -3.9% | -4.3% |
| Enc Time[%] | 476% |  |  | 63% |  |  |
| Dec Time[%] | 277% |  |  | 31% |  |  |

Table 5. JVET-J035

|  | HM Anchor |  |  | JEM Anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Average SDR-A | -41.3% | -28.5% | -31.4% | -8.73% | 20.30% | 15.22% |
| Average SDR-B | -31.7% | -33.1% | -32.4% | -1.84% | 15.38% | 16.94% |
| Average all | -36.5% | -30.8% | -31.9% | -5.29% | 17.84% | 16.08% |
| Enc Time[%] | 689% |  |  | 85% |  |  |
| Dec Time[%] | 555% |  |  | 81% |  |  |

# Conclusions

This contribution evaluates two tool configurations on JVET-J0021 and JVET-J0035 software bases. Test 1 configuration offers lower complexity with 35.8% and 34.3% bitrate reduction on the luminance component for JVET-J0021 and JVET-J0035, respectively. Test 2 configuration provides 37.2% and 36.5% bitrate reduction on the luminance component with added decoder side complexity. The authors suggest Test 1 configuration as a starting point for standardization development.

# Reference

[1] Y.-W. Chen et al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, JVET-J0021, April 2018.

[2] P. Bordes et al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”, JVET-J0022, April 2018.

[3] X. Xiu et al., “Description of SDR, HDR and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, April 2018. https://ipbt.hhi.fraunhofer.de/svn/svn_hhi-next-software/tags/NextSoftware-JVET-J0035_plus_JEMTools

[4] M. Albrecht et al., “Quadtree plus binary tree with shifting”, JVET-J0035, April 2018.

[5] JVET, “Joint Call for Proposals on Video Compression with Capabilities beyond HEVC,” JVET-H1002, Oct. 2017.

[6] R. Sjöberg et al., “Description of SDR and HDR video coding technology proposal by Ericsson and Nokia”, JVET-J0012, April 2018.

[7] K. Kawamura et al., “Description of SDR video coding technology proposal by KDDI”, JVET-J0016, April 2018.

[8] C.-W. Hsu, et al., “Description of SDR video coding technology proposal by MediaTek”, JVET-J0018, April 2018.

[9] T. Suzuki, et al., “Description of SDR and HDR video coding technology proposal by Sony”, JVET-J0028, April 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).