| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0087 |
| --- | --- |

| Title: | A placeholder concept and a software development plan for the test model and the core experiment common ground software |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Wen Huang | Email: | yuwen.huang@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a placeholder concept and a software development plan for the test model and the core experiment (CE) common ground software. Quadtree plus binary tree (QTBT) with triple tree (TT) block partitioning structure and a number of JEM-7.0 tools are suggested to be included into the same software to facilitate core experiments.

There are two basic principles. First, QTBTTT, which is the most commonly used block partitioning structure concept among all the Call for Proposals (CfP) responses, is selected as the block partitioning structure in the test model. Second, a number of JEM-7.0 tools are suggested below and are integrated to the QTBTTT block partitioning structure. The suggested tools are treated as placeholders in the test model, and the placeholders-integrated software can be called the CE common ground software (CE-SW). Each placeholder shall be tested together with other proposed changes or replacements in a corresponding future core experiment (CE). That is, the CE anchor shall be generated by disabling the placeholder of the CE. In general, the CE anchor enables the rest of placeholders, but further investigations on the interaction between tools in different CEs may be planned in the CE. In addition, each placeholder, even if remains one of the best choices in the CE, is not adopted into the test model automatically after the CE. Whether a tool tested in a CE is adopted into the test model shall be decided by the JVET group.

A number of JEM-7.0 tools to be built on top of the QTBTTT block partitioning structure should be discussed and selected by the JVET group. An example of the initial placeholder tool list is shown as follows, and it is suggested that the JVET group should shorten the placeholder tool list, if desirable.

  - 128x128 coding tree unit (CTU) with maximum transform size equal to 128x128

  - Intra mode coding with 67 intra prediction modes and four-tap intra interpolation filter

  - Cross-component linear model (CCLM) prediction

  - Alternative temporal motion vector prediction (ATMVP)

  - Adaptive motion vector resolution (AMVR) with 1/16-sampel motion vector storage accuracy

  - Affine motion compensation prediction

  - Bi-directional optical flow (BIO)

  - Decoder-side motion vector refinement (DMVR)

  - Adaptive multiple core transform (AMT) and non-separable secondary transforms (NSST)

  - Adaptive loop filter (ALF)

  - Context-based adaptive binary arithmetic coding (CABAC) design modifications

After the placeholder tool list is approved by the JVET group, software associated to unselected tools shall be removed from the CE-SW. Each selected placeholder tool shall be enabled or disabled through a configuration.

A preliminary software has been tested. The purpose of the preliminary software is only to demonstrate the example initial placeholder tool list’s level of compression performance and runtimes, not to become the starting point of the test model or the CE-SW. The preliminary software achieves -34.74%/-42.48%/-42.94% Y/U/V BD-rates with 454% encoding time and 224% decoding time compared against HM-16.16 for constraint set 1 (CS1).

In addition, to significantly clean up the software, it is suggested to remove the prediction unit (PU) and transform unit (TU) concepts and the irrelevant high-level syntax (HLS) in HEVC from the CE-SW.

An example implementation of the CE-SW is to add the selected placeholder tools exactly the same as those in JEM-7.0 to JVET-J0075 (HM plus QTBTTT and nothing else) and then remove the PU and TU concepts and the irrelevant HLS in HEVC. JVET-J0035 (NextSoftware) can be modified to become the CE-SW as well. A Break-out Group (BoG) may be needed to discuss and suggest implementation details.

It is suggested to establish a test model and CE common ground software development Ad-Hoc Group (AHG) to develop the software according to the approved placeholder tool list. The test model and the CE-SW should be released in 3 or 4 weeks after the last day of the JVET-J meeting. It is also suggested that new proposals submitted to the JVET-K meeting are developed and tested on the CE-SW.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).