| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0088-v1 |
| --- | --- |

| Title: | Proposed Test Model toolset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jill Boyce, Sam Wong, Zhipin Deng | Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

A toolset is proposed for the Test Model, as summarized below:

- HEVC + TT

- JEM tools, with the following changes

  - Remove: FRUC, BIO, DMVR, OBMC, LIC

  - Simplify: Limit to 64x64 transform

Based on discussion with other meeting participants, and especially influenced by the concepts proposed in JVET-J0087, a second, less-preferred, alternative option is also proposed. In this alternative option, a two-tier system is proposed, defining both a Test Model and a Core Experiment Model. Both the Test Model and Core Experiment Model would be supported in the initial version of the reference software, with the Core Experiment Model on a separate branch. Only the Test Model would be described in the test model text description output document.

The toolsets for the alternative option models are summarized as follows:

- Test Model: HEVC + TT

- Core Experiment Model: All of the JEM tools listed above for the preferred singled-tier option

# Discussion of preferred solution

## TT tool

Numerous CfP responses included variations on this tool. Because the coding structure is a very important basis of the design, upon which other tools depend, it is important to make this decision early.

## Other new tools proposed in CfP responses

Many new tools not already present in the JEM have been proposed in the CfP responses. In most cases there are closely related tools proposed in several responses. It is recommended to define several CEs to study these proposals rather than attempting to decide among the variations during this meeting, because we currently lack good information directly comparing the performance of these tools, isolated from other tools in the CfP responses.

## JEM tools removal

It is proposed to not include in the test model several tools that are already included in the JEM: FRUC, BIO, DMVR, OBMC, LIC.

These tools have significant decoder complexity requirements. In several CfP responses, proposals were made to simplify these tools. We recommend that these tools be studied in Core Experiments before a decision is made whether or not to include them in the test model.

However, we propose to include a number of the JEM tools in the Test Model. For evaluation of future tool proposals, it is preferable to have some well-studied tools adopted as part of the anchor, so that tool interactions and additive nature of gains can be studied.

## JEM tools simplifications

It is proposed to limit the block transform size to 64x64, because of significant implementation complexity of larger transform sizes.

# Discussion of alternative solution

Because the CfP responses contained variants of many of the tools contained in JEM, some meeting participants prefer to postpone adoption of those tools into the Test Model until after those tools are studied in CEs.

Typically, new coding tools demonstrate higher gains when compared with a lower coding efficiency anchor than when compared against a higher coding efficiency anchor, because not all coding gains are additive. In order to expedite development of the new standard, it is proposed to use a higher coding efficiency anchor for CEs. When each CE is defined, coding tools contained in the Core Experiment Model that are not directly being studied in the CE will be enabled. Coding tools in the Core Experiment Model that are direct related to the tools being studied in the CE will not be included in the anchor for that CE.

When decisions are made by the group in the future to adopt tools to the Test Model based upon CE results, the presence of a particular variant of a tool in the Core Experiment Model is not intended to be an endorsement.

# Reference Software

The set of tools proposed for inclusion in the Test Model, and for the alternative solution in the Core Experiment Model, could be supported using reference software derived by modifying either the Qualcomm software in JVET-J0021 or the HHI software in JVET-J0035. In particular, for those tools present in both the JEM and the reference software, the reference software would match the JEM behavior, except for the minimum necessary changes to add support for TT. For example, bitstreams generated with this software with the TT tool off would be decodable by the JEM 7.0.