| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0089 |
| --- | --- |

| Title: | Suggestion on non-HEVC inherited approach |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina, Kiho Choi (Samsung) Jianle Chen, Ye-kui Wang, Haitao Yang (Huawei) Adeel Abbas, David Newman (GoPro) Jianhua Zheng (Hisilicon) | Tel: Email: Tel: Email: | +82-10-8855-2932 elena_a.alshina@samsung.com kiho14.choi@samsung.com +1-408-330-5334 jianle.chen@huawei.com yekui.wang@huawei.com haitao.yang@huawei.com +1-443-739-8055 aabbas@gopro.com +86-010-82829521 zhengjianhua@hisilicon.com |
| Source: | Samsung Electronics, Huawei, GoPro, HiSIlicon |  |  |

_____________________________

# Proposal

Two potential starting point for new standard development were suggested during Joint MPEG and VCEG discussion:

- HEVC + QTBT + TT,

- HEVC + QTBT + TT – some HEVC tools.

This proposal suggests specific clarification with the comparison of “option 2” and “option 1”. Suggested delta between two options is explained in Table 1. Suggested modification of HM is not the whole package of JVET-J0024/25 CfP response [1]. There is initial suggestion for HEVC clean-up which can be done quickly w/o significant impact to the performance and drastically code changes. We also believe that suggested modifications would make tools experiment easier. Example of implementation for suggested changes can be found in JVET-J0072 [2].

Only highlighted in yellow changes are suggested for HEVC clean-up at this point. More significant simplification is desired in a future standardization process.

| Module | HM + QT&BT&TT (“option 1”) | HM + QT&BT&TT– some HEVC tools (“option 2”) |
| --- | --- | --- |
| Block structure | - Binary-Tree/Triple-Tree, no luma and chroma separate tree - CU=PU= TU=88 ..., 128128 - Boundary processing | - Binary-Tree/Triple-Tree, no luma and chroma separate tree - CU=PU= TU=88 ..., 128128 - Boundary processing |
| Intra prediction | - 33 angular modes, with bilinear interpolation - DC and Planar - 3 MPM mode, naturally selection from left block, above block, Planar and DC - 6 bits fixed length coding for remaining modes - Reference samples smoothing - Boundary smoothing - Strong reference smoothing | - 33 angular modes, with bilinear interpolation - DC and Planar - 3 MPM mode, naturally selection from left block, above block, Planar and DC - 6 bits fixed length coding for remaining modes - None - None - None |
| Inter prediction | - 8-tap luma and 4-tap chroma DCT-IF interpolation filter - HEVC merge and AMVP coding method - TMVP (compressed MV field) | - 8-tap luma and 4-tap chroma DCT-IF interpolation filter - Straightforward motion merge and AMVP coding method - TMVP (compressed MV field) |
| Transform | - DCT-II 10-bits, 44, …, 128128 - DST-VII 8-bits, 44 | - DCT-II 8-bits, 44 ... , 6464 - None |
| Quantization | - AVC method | - AVC method |
| CABAC engine | - AVC CABAC | - Straightforward (textbook) implementation of high precision probability status (16-bits) binary arithmetic coding with single window uprate |
| Coefficient coding | - Coefficients group coding, Last X,Y - GT1, GT2, … - Sign data hiding - Mode dependent scan | - Run-length coding, more than 20 year conventional method - None - None - One scan (Zig-zag) |
| Loop filter | - HEVC de-blocking filter - SAO | - HEVC de-blocking filter - none |
| High level syntax and parallel coding functionality | - NAL unit header, - 32 VCL NAL unit types - 32 non-VCL NAL unit types - VPS/VPS VUI, - SPS/SPS VUI, - PPS, - Slices / Slice header, - Independent slice segment - Dependent slice segment - Tile - Wavefront - Reference picture set (RPS) based reference picture management | - Sequence Header, - Slices / Slice header - No support of dependent slice segment - MMCO based reference picture management |

#### Sequence header syntax

| sequence_header( ) { | Descriptor |
| --- | --- |
| chroma_format_idc | ue(v) |
| pic_width_in_luma_sample | u(16) |
| pic_height_in_luma_sample | u(16) |
| closed_gop_flag | u(1) |
| log2_ctu_size_minus1 | u(3) |
| QT_BT_TT_information | se(v) |
| seq_chroma_qp_offset | se(v) |
| slice_chroma_qp_offset_present_flag | u(1) |
| cu_delta_qp_enabled_flag | u(1) |
| chroma_qp_mapping_table_idc | u(1) |
| seq_chroma_qp_split_flag | u(1) |
| if( seq_chroma_qp_split_flag ) { |  |
| delta_cb_qp_offset | se(v) |
| delta_cr_qp_offset | se(v) |
| } |  |
| seq_tool_A_enable_flag | u(1) |
| seq_tool_..._enable_flag | u(1) |
| reserved_bits | f(12) |
| byte_align( ) |  |
| } |  |

#### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| dtr | u(10) |
| slice_type | u(3) |
| key_picture_flag | u(1) |
| slice_deblocking_filtering_flag | u(1) |
| if( seq_sao_flag ) { |  |
| slice_sao_flag | u(1) |
| } |  |
| update_exist | u(1) |
| slice_qp | u(6) |
| if( slice_type ! = I ) { |  |
| dptr | se(v) |
| } |  |
| temporal_layer_id | u(3) |
| mmco_flag | u(1) |
| if( mmco_flag ) { |  |
| for( cnt = 0; cnt < 16; cnt++ ) { |  |
| mmco_type | ue(v) |
| if( mmco_type = = 1) |  |
| break |  |
| mmco_data | ue(v) |
| } |  |
| } |  |
| rmpni_flag | u(1) |
| if( rmpni_flag ) { |  |
| for( cnt = 0; cnt < 16; cnt++ ) { |  |
| rmpni_type_list0 | ue(v) |
| if( rmpni_type_list0 = = 0 ) |  |
| break |  |
| rmpni_data_list0 | ue(v) |
| } |  |
| for( cnt = 0; cnt < 16; cnt++ ) { |  |
| rmpni_type_list1 | ue(v) |
| if( rmpni_type_list1 = = 0 ) |  |
| break |  |
| rmpni_data_list1 | ue(v) |
| } |  |
| } |  |
| byte_align( ) |  |
| } |  |

# Conclusion

Significant clean-up for HEVC could be convergence point between HEVC-based and non-HEVC inherited CfP responses. Suggested modification is just starting point for the discussion. Creation of a BoG for further clarification of all details is recommended.

# References

- JVET-G1001JVET-J0024, Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario,.

- JVET-J0072, SW for technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario (JVET-J0024).

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

GoPro, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hisilicon Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).