| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0090-v1 |
| --- | --- |

| Title: | AHG5: Measurement result of memory bandwidth comparison with JEM and HM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ryoji HASHIMOTO Seiji Mochizuki | Tel: Email: | ryoji.hashimoto.te@renesas.com seiji.mochizuki.zc@renesas.com |
| Source: | Renesas Electronics Corporation |  |  |

_____________________________

- Abstract

This contribution shows comparison between the memory bandwidth in JEM7.1 and that in HM16.16. At 9th JVET meeting in Gwangju, JVET-I0033 [1] shows the measurement results of memory bandwidth in JEM7.1. Based on comments to it, this contribution shows additional results. This can check memory bandwidth in the test model for next video standardization and provide comparison with HEVC.

Introduction

The video applications in various devices become much popular today. In some devices such as mobile devices, it is very important to save power consumption in order to expand battery life. The memory bandwidth is one of the biggest factors in power consumption. Moreover, rapid increase of bandwidth consumed by video applications makes system design difficult. Therefore, a contribution of AHG5 has checked memory bandwidth in JEM7.1 by implementing cache model [1]. However, at the 9th meeting in Gwangju, there are some comments to the measurement result. Based on comments, this contribution shows additional results such as memory bandwidth in HM.

Measurement result

We used the same way as JVET-I0033 in the measurement. Figure 1 shows the access model in this measurement. The test condition as shown in Table 1 and Table 2. This has no change from JVET-I0033. We have tested with anchor streams for CfP, JEM7.1 and HM16.16. To compare the difference between encoder options, we used Class SDR-B constraint set 1 and 2 (random access and low delay).

Figure 2 shows total access number to cache. It is clear that RA in JEM accesses to cache much more times than other configurations. As bitrate increases, the number of access decreases in JEM RA while that in HM increases as shown in Figure 2. On the other hand, the number of access increases in HM RA/LD configuration so that new tools adopted in JEM such as decoder side motion vector derivation may cause this. The maximum ratio of the number of access is 19.3 in RA “Cactus” sequence. As shown in Figure 3 - Figure 6, this tendency is also shown in the bandwidth. In all sequences, RA in JEM requires the highest bandwidth and LD in HM requires the lowest bandwidth. JEM requires 6.4 and 4.4 times higher bandwidth than HM in RA and LD configuration, respectively.

Figure 1 Decoder memory access model (with cache)

Table 1 cache configuration

| Parameter | Value |
| --- | --- |
| cache type | n-way set associative |
| cache replacement algorithm | Tree Pseudo-Least Recently Used (Tree PLRU) |
| cache line size | 128, 256 [Byte] |
| # of way | 4, 8 |
| # of cache line | 64, 128 |
| total cache size | 32, 64 [KByte] |

Table 2 cache configuration set

| Configuration set | Cache line size [Byte] | # of way | # of cache line | Total cache size [KByte] |
| --- | --- | --- | --- | --- |
| A | 128 | 4 | 64 | 32 |
| B | 128 | 4 | 128 | 64 |
| C | 128 | 8 | 64 | 64 |
| D | 256 | 4 | 64 | 64 |

Figure 2 total access number to cache

Figure 3 average bandwidth with configuration A

Figure 4 average bandwidth with configuration B

Figure 5 average bandwidth with configuration C

Figure 6 average bandwidth with configuration D

Conclusion

This contribution shows the measurement result of memory bandwidth with several cache configurations. On the measurement, JEM7.1, HM16.16 and anchor streams for CfP are used. Measurement result shows JEM7.1 requires 6.4 and 4.4 times higher bandwidth than HM16.16 in RA configuration and LD configuration, respectively. This cache implementation is useful to check the bandwidth in tool on/off test in the next test model and comparison with HEVC.

[1] JVET-I0033 AHG5: Measurement result of memory bandwidth with anchor streams “http://phenix.int-evry.fr/jvet/doc_end_user/current_document.php?id=3414”

Patent rights declaration(s)

Renesas Electronics Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).