| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0092 |
| --- | --- |

| Title: | Cross-check of JVET-J0090 Measurement result of memory bandwidth on HM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Tomohiro Ikai | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution presents a cross-check result of JVET-J0090 Measurement result of memory bandwidth on HM. Specifically the test was done using CfP HM anchor bitstreams in SDR CS1 and CS2 categories. The cross-checker confirmed that the difference between results provided by proponent and the results generated by the cross-checker are all within a range of 0.01 % and 0.1% in terms of Hit rate and Required bandwidth respectively. That means “matched”.

It is noted that in the development of the measurement software, the cross-checker (Sharp) also contributes several fixes on the base software developed by the proponent (Renesas Electronics).

Crosscheck result

The test condition as shown in Table 1.

Table 1 cache configuration set

| Configuration set | Cache line size [Byte] | # of way | # of cache line | Total cache size [KByte] |
| --- | --- | --- | --- | --- |
| A (config4) | 128 | 4 | 64 | 32 |
| B (config2) | 128 | 4 | 128 | 64 |
| C (config3) | 128 | 8 | 64 | 64 |
| D (config1) | 256 | 4 | 64 | 64 |

Table 2. shows the differences between Test of J0090 and Cross-check of J0092. It shows 0.01 % and 0.1 % in terms of Hit rate and Required bandwidth respectively.

Table 2  Differences between Test of J0090 and Cross-check of J0092

|  | RA | RA | RA | RA | RA | RA | RA | RA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | config1 (D) | config1 (D) | config2 (B) | config2 (B) | config3 (C) | config3 (C) | config4 (D) | config4 (D) |
|  | diff of Hit rate [%] | diff of Required bandwidth [MB/frame] | diff of Hit rate [%] | diff of Required bandwidth [MB/frame] | diff of Hit rate [%] | diff of Required bandwidth [MB/frame] | diff of Hit rate [%] | diff of Required bandwidth [MB/frame] |
| P01S10C1R1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0.1 |
| P01S10C1R2 | 0 | 0 | 0 | -0.1 | 0 | -0.1 | 0 | 0 |
| P01S10C1R3 | 0 | 0.1 | 0 | 0 | 0.01 | -0.1 | 0 | 0 |
| P01S10C1R4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| P01S11C1R1 | 0 | 0 | 0 | -0.1 | 0 | -0.1 | 0 | 0 |
| P01S11C1R2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| P01S11C1R3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| P01S11C1R4 | 0 | 0 | 0 | -0.1 | 0 | -0.1 | 0 | 0 |
| P01S12C1R1 | 0 | 0 | 0 | 0 | 0 | 0.1 | 0 | 0.1 |
| P01S12C1R2 | 0.01 | 0 | 0 | 0.1 | 0 | 0.1 | 0 | 0 |
| P01S12C1R3 | 0 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0.1 |
| P01S12C1R4 | 0 | 0 | 0 | 0.1 | 0 | 0.1 | 0 | 0.1 |
| P01S13C1R1 | 0 | 0 | 0 | -0.1 | 0 | -0.1 | 0 | 0 |
| P01S13C1R2 | 0 | 0 | 0 | 0 | 0 | -0.1 | 0 | 0 |
| P01S13C1R3 | 0 | 0 | 0 | 0.1 | 0 | 0.1 | 0 | 0 |
| P01S13C1R4 | 0 | -0.1 | 0 | -0.1 | 0 | -0.1 | 0 | 0 |
| P01S14C1R1 | 0.01 | -0.1 | 0 | 0 | 0 | 0 | 0 | 0 |
| P01S14C1R2 | 0 | 0 | 0 | 0.1 | 0 | 0 | 0 | 0 |
| P01S14C1R3 | 0 | 0 | 0 | 0.1 | 0 | 0 | 0 | 0 |
| P01S14C1R4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |