| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0093-v1 |
| --- | --- |

| Title: | Two tier test model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jill Boyce (Intel) Yan Ye (InterDigital) Yu-Wen Huang (Mediatek) Marta Karczewicz (Qualcomm) Edouard Francois (Technicolor) Walt Husack (Dolby) Justin Ridge (Nokia) | Email: | jill.boyce@intel.com Yan.Ye@InterDigital.com yuwen.huang@mediatek.com martak@qti.qualcomm.com Edouard.Francois@technicolor.com Husak, Walt <WJH@dolby.com> justin.ridge@nokia.com |
| Source: | Intel, InterDigital, Mediatek, Qualcomm, Technicolor, Dolby, Nokia |  |  |

_____________________________

# Abstract

A two tier model is proposed for codec development, including a Test Model and a Benchmark Model.

- Test Model:

  - HEVC +  QTBT + TT

  - Possibly remove some HEVC tools

- Benchmark Model:

  - Test Model

  - JEM tools except remove: FRUC, BIO, OBMC, LIC, transforms larger than 64x64

# Discussion

## Test Model

The Test Model will contain only limited tools above HEVC: QTBT and TT. These tools contain major structural changes that impact many additional coding tools.

## Benchmark Model

The Benchmark Model contains many placeholder tools previously studied in the JEM, but removes several JEM tools with significant decoder implementation complexity. The Benchmark Model is expected to provide significantly better coding efficiency than the Test Model.

Because the CfP responses contained variants of many of the tools contained in JEM, it is preferred to postpone adoption of those tools into the Test Model until after those tools are studied in CEs.

Typically, new coding tools demonstrate higher gains when compared with a lower coding efficiency anchor than when compared against a higher coding efficiency anchor, because not all coding gains are additive. In order to expedite development of the new standard, it is proposed to use a higher coding efficiency anchor for CEs. When each CE is defined, coding tools contained in the Benchmark Model that are not directly being studied in the CE will be enabled. Coding tools in the Benchmark Model that are direct related to the tools being studied in the CE will not be included in the anchor for that CE.

When decisions are made by the group in the future to adopt tools to the Test Model based upon CE results, the presence of a particular variant of a tool in the Benchmark Model is not intended to be an endorsement.

# Reference software

The reference software will support both the Test Model and the Benchmark Model, on separate branches.