| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0094-v1 |
| --- | --- |

| Title: | Suggested process to select the Benchmark Set |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Benjamin Bross Adam Wieckowski Heiko Schwarz Detlev Marpe Thomas Wiegand | Tel: Email: | +49 30 310020 firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document suggests a process to select the benchmark set (BMS).  The proposed method starts with a base configuration that includes HM + QTBT + TT.  On top of that, additional coding tools are tested individually and ranked according to their BD-rate vs. encoder/decoder runtime slope.  This ranking can provide guidance and an order for selecting tools for a test model for given coding efficiency and runtime expectations.  As an example, the method is applied to JEM tools.  Two different combinations of tools with a certain minimum BD-rate gain and a high slope are selected to generate two operation points. For example, the BD-rates and runtimes of SDR UHD sequences are as follows:

MTT with -15% BD-rate over the HM anchor at 113% encoder and 87% decoder runtime.

OP1 with -26% BD-rate over the HM anchor at 370% encoder and 109% decoder runtime.

The proposed method provides a systematic way to compile a benchmark set from a set of tools for given BD-rate and complexity conditions.

# Introduction

The majority of CfP proposals including subjectively good performing ones where using an advanced partitioning with a QTBT+TT.  The idea of this proposal is start to with the HM toolset plus QTBT+TT partitioning.  Having this as a basis, BD-rate and encoder/decoder runtimes of additional coding tools are measured on top of that.  Since the block structure interacts with all other coding tools, BD-rate and runtimes for each individual tool are measured compared to the HM + QTBT + TT anchor.

Based on the BD-rates and runtimes, a slope for each tool is calculated as described in section 2. Ordering the different tools by slope gives an indication of the coding efficiency gain for a certain tool for given encoder and decoder runtimes, i.e. the "biggest bang for the buck".  As an example, this method is applied to JEM tools and results are presented in section 3.  All of the tested JEM tools are integrated in the NextSoftware and listed in the Annex.  Using this software and the information from the slope analysis, two different example operation points having the tools with high slopes enabled are generated as presented in section 4.

In summary, the following process is suggested:

- Run the test software with advanced block partitioning and just one tool on

- Compute the slope for each tool as described in section 2

- Consider all tools for the benchmark set that

  - provide a BD rate reduction more than X % (X to be discussed)

  - meet other pre-defined criteria, e.g. no decoder search

- Start with an advanced block partitioning (MTT) and turn on considered tools with the steepest slope on until Y% overall BD rate reduction is reached (Y to be discussed)

# Slope calculation

The proposed coding efficiency / complexity analysis of individual tools is similar to that presented in JVET-B0044 [14].  As complexity measure, a weighted measure of encoder and decoder run time overhead is used

C=(Cenc-1)+w (Cdec-1)w+1,

where Cenc and Cdec represent the ratios of the geometric means of the run times for encoder and decoder, respectively.  The ratios Cenc and Cdec are the complexity measures that are commonly reported in evaluation of coding tools.  The weighting factor w=5 is used for an unequal weighting of encoder and decoder complexity.  Hence, we assumed that the decoder complexity is five times more important than the encoder complexity. However, this factor can be changed if agreed to do so.

The coding efficiency / complexity trade-off is measured as the slope angle given by the complexity measure C defined above and the luma Bjøntegaard delta rate BDR:

α=atan2C, -BDR

# Selection Example

As an illustrative example, the proposed process is applied using the JEM tools.  For all considered JEM tools, BD-rate and runtime data is generated by running simulations to calculate the slope for each individual tool.  Based on a minimum BD-rate, slope ranking and design considerations, two different set of tools are compiled and results are reported in the next section.

## Calculate slopes

The simulations for individual JEM tools were performed according to the JVET common test conditions [1] for the random-access scenario with the following modifications:

- test sequences are from the JVET common test set and the CfP test set [2];

- 49 frames of each sequence were encoded (for reducing the simulation time);

- QP values {27, 32, 37, 42} are used (for a better approximation of the QP values actually used for achieving the CfP bit rates).

As reference configuration, an encoder setting in which all JEM coding tools are disabled is used together with a modified block partitioning. e.g. the QT+BTS partitioning described in JVET-J0035 [3].  It is configured to only use block sizes for which both the block width and height represent integer powers of two. This setting ensures that only block sizes also available with QTBT are used and it provides an increased coding efficiency relative to QTBT. More detailed results can be found in JVET-J0035 [3].

Each of the JEM coding tools has been separately enabled in addition to the QT+BTS partitioning and the BD rates and run times have been measured against the QT+BTS reference.  The results are summarized in Table 1.  The coding tools are ordered according to the measured coding efficiency / complexity slopes introduced above.  The corresponding spreadsheet can be found enclosed to this contribution.

Table 1 BD-rates, runtimes and coding efficiency / complexity slopes of individual JEM tools

## Select tools

In this example, the following criteria are applied to select tools:

- luma BD-rate gain larger than 0.75%

- no decoder search as in InterDMVR, InterFRUC or InterBIO

All tools in Table 1 that meet these criteria have been highlighted in red. From this set of tools, the following operations points have been defined:

- MTT: This is the structure only base operation point without any additional tools.

- OP1: This operation point includes the first 5 tools with the highest slope

Table 2 Tested combinations of JEM tools

| Tool | MTT | OP1 |
| --- | --- | --- |
| MTT | 1 | 1 |
| IntraLMChroma | 0 | 1 |
| InterAMVDR | 0 | 1 |
| Intra65 | 0 | 1 |
| TrafoAMT+NSST | 0 | 1 |
| InterAffine* | 0 | 1 |

* requires InterHighPrecMV to be enabled

# Results

In the previous section, two operation points have been defined. Results for both operation points compared to the HM anchor are shown in Table 3 and Table 4 for the CfP SDR CS1 category (HD and UHD), in Table 5 for the CfP SDR CS2 category (HD) and in Table 6 and Table 7 for the CfP HDR CS1 category (HLG and PQ).

The simulations were performed according to the JVET common test conditions [1] for the random-access scenario with the following modifications:

- test sequences are from the the CfP test set (SDR and HDR);

- 49 frames of each sequence were encoded (for reducing the simulation time);

- QP values {27, 32, 37, 42} are used (for a better approximation of the QP values actually used for achieving the CfP bit rates).

The detailed spreadsheets can be found enclosed to this contribution.

Table 3 Results for two different operation points for UHD and the CfP SDR CS1 configuration

| UHD CfP SDR CS1 average all over HM16.16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OP | BD-rate(Y) | BD-rate(U) | BD-rate(V) | Enc Time (4 WPP threads) | Enc. Time (1 thread*) | Dec Time |
| MTT | -14.97% | -16.28% | -15.92% | 52% | 113% | 87% |
| OP1 | -25.86% | -35.63% | -33.15% | 170% | 370% | 109% |

Table 4 Results for two different operation points for HD and the CfP SDR CS1 configuration

| HD CfP SDR CS1 average all over HM16.16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OP | BD-rate(Y) | BD-rate(U) | BD-rate(V) | Enc Time (4 WPP threads) | Enc. Time (1 thread*) | Dec Time |
| MTT | -10.87% | -22.01% | -23.85% | 60% | 130% | 94% |
| OP1 | -20.75% | -40.86% | -40.30% | 214% | 465% | 118% |

Table 5 Results for two different operation points for HD and the CfP SDR CS2 configuration

| HD CfP SDR CS2 average all over HM16.16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OP | BD-rate(Y) | BD-rate(U) | BD-rate(V) | Enc Time (4 WPP threads) | Enc. Time (1 thread*) | Dec Time |
| MTT | -11.25% | -29.48% | -30.82% | 61% | 133% | 101% |
| OP1 | -18.98% | -42.57% | -42.84% | 220% | 478% | 129% |

Table 6 Results for two different operation points for the CfP HLG HDR CS2 configuration

| UHD CfP HLG HDR CS1 average all over HM16.16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OP | BD-rate(Y) | BD-rate(U) | BD-rate(V) | Enc Time (4 WPP threads) | Enc. Time (1 thread*) | Dec Time |
| MTT | -10.3% | -20.6% | -33.0% | 36% | 78% | 81% |
| OP1 | -17.2% | -49.1% | -53.0% | 122% | 265% | 93% |

Table 7 Results for two different operation points for the CfP PQ HDR CS2 configuration

| HD CfP PQ HDR CS1 average all over HM16.16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OP | BD-rate(Y) | BD-rate(U) | BD-rate(V) | Enc Time (4 WPP threads) | Enc. Time (1 thread*) | Dec Time |
| MTT | -9.4% | -16.3% | -20.7% | 38% | 83% | 93% |
| OP1 | -18.3% | -50.0% | -47.6% | 143% | 311% | 109% |

* Simulations have been performed using 4 wavefront parallel processing (WPP) threads as described in JVET-J0036 [4]. This approach reduces the encoder runtime to 46%. In order to approximate the single thread runtime, the 4 WPP threads runtime was divided by 0.46.

# References

- Suehring, Li, „JVET common test conditions and software reference configurations,” JVET-H1010, Oct. 2017.

- JVET, “Joint Call for Proposals on Video Compression with Capabilities beyond HEVC,” JVET-H1002, Oct. 2017

- Schwarz, Rudat, Siekmann, Bross, Marpe, Wiegand, „Coding Efficiency / Complexity Analysis of JEM 1.0 coding tools for the Random Access Configuration,” JVET-B004, Feb. 2016.

- Wieckowski, Hinz, George, Ma, Brandenburg, Lehmann, Schwarz, Marpe, Schierl, Wiegand, „Thread Parallel Encoding,“ JVET-J0036, Apr. 2018

# Annex: List of Tools

Table 8 shows the complete list of JEM tools that have been evaluated in this example/

Table 8 List of JEM tools plus MTT available in NextSoftware

| Block Partitioning |  |
| --- | --- |
| MTT | Multi-type tree (MTT) block partitioning as in JVET-D0117 |

| Inter |  |
| --- | --- |
| InterAMVDR | Adaptive motion vector difference resolution as in JEM |
| InterDMVR | Decoder motion vector refinement as in JEM |
| InterHighPrecMV | High precision motion vectors as in JEM |
| InterFRUC | Pattern matched motion vector derivation as in JEM |
| InterBIO | Bi-directional optical flow as in JEM |
| InterSubPU | Sub PU motion vector prediction as in JEM |
| InterOBMC | Overlapping block motion compensation as in JEM |
| InterAffine | Affine motion compensation as in JEM |
| InterLIC | Local illumination compensation as in JEM |

| Intra |  |
| --- | --- |
| Intra65 | 65 intra-picture prediction modes as in JEM |
| Intra4Tap | 4-tap filter for intra prediction as in JEM |
| IntraBF | Intra boundary filtering as in JEM |
| IntraPlanarPDPC | Planar PDPC as in JEM |
| IntraLMChroma | Cross-component linear model prediction as in JEM |
| IntraMDMS | Extended chroma multiple DM modes as in JEM |

| Transform |  |
| --- | --- |
| TrafoAMT | Adaptive Multiple Core Transform as in JEM |
| TrafoNSST | Mode-dependent non-separable secondary transforms (NSST) as in JEM |

| In-loop Filter |  |
| --- | --- |
| InloopBiFi | Bilateral in-loop filter as in JEM |
| InloopAClip | Adaptive clipping as in JEM |
| InloopGALF | Geometric adaptive loop filter as in JEM |

| Quantization, Scaling and Transform Coding |  |
| --- | --- |
| TCC-JEM | transform coefficient coding as in JEM |

| Entropy Coding |  |
| --- | --- |
| CabacMP | Multi-parameter CABAC as in JEM |
| CabacInitPrevFrame | CABAC initialization from previous frame as in JEM |