| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J0096-v1 |
| --- | --- |

| Title: | BoG report on Benchmark Set tool selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on Apr 18 from 2:30 pm to 4:00 pm and 5:30 pm to 6:00 pm to select which subset of the JEM tools should be included in the “benchmark set” (BMS) of tools with higher coding efficiency than the test model. The BoG was directed to exclude tools from the BMS that either had high decoder implementation complexity, or that had low demonstrated coding efficiency improvement.

A list of JEM tools was gathered from JVET-G1001 “Algorithm description of Joint Exploration Test Model 7 (JEM7).”  The attached spreadsheet was prepared during consideration of the JEM tools.

The BoG recommends and exclusion of 17 JEM tools, and inclusion of the following 9 JEM tools:

- 65 intra prediction modes - Coefficient coding - AMT + 4x4 NSST - Affine motion

- GALF

- Subblock merge candidate (ATMVP )

- Adaptive motion vector precision

- Decoder motion vector refinement

- LM Chroma mode

# Document presentation

## JVET-J0094, “Suggested process to select the Benchmark Set”

This document suggests a process to select the benchmark set (BMS).  The proposed method starts with a base configuration that includes HM + QTBT + TT.  On top of that, additional coding tools are tested individually and ranked according to their BD-rate vs. encoder/decoder runtime slope.  This ranking can provide guidance and an order for selecting tools for a test model for given coding efficiency and runtime expectations.  As an example, the method is applied to JEM tools.  Two different combinations of tools with a certain minimum BD-rate gain and a high slope are selected to generate two operation points. For example, the BD-rates and runtimes of SDR UHD sequences are as follows:

MTT with -15% BD-rate over the HM anchor at 113% encoder and 87% decoder runtime.

OP1 with -26% BD-rate over the HM anchor at 370% encoder and 109% decoder runtime.

The proposed method provides a way to compile a benchmark set from a set of tools for given BD-rate and complexity conditions.

The model calculates a slope of coding gain vs. the complexity metric. Models complexity by weighted average of 5* decoder run time plus encoder runtime, with the factor configurable. Some participants questioned if 5x weighting for decoder makes sense.

Used QPs (27, 32, 37, 42) to closer match the CfP bitrates. Suggestion to use lower QPs.

A table was provided of coding efficiency gains and runtimes for encoder and decoder for 49 frames.