| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J1003-v1 |
| --- | --- |

| Title: | Report of results from the Call for Proposals on Video Compression with Capability beyond HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Report |  |  |
| Authors | Vittorio Baroncini EVATech Via Pisanelli, 4 00196 – Rome, Italy Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen, Germany Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: Tel: Email: Tel: Email: | +39 3335474643 baroncini@gmx.com +49 241 80 27671 ohm@ient.rwth-aachen.de +1 425 703 5308 garysull@microsoft.com |
| Source: | JVET |  |  |

_____________________________

# Introduction

This document provides results and a description of the subjective testing (also with some objective measurement analysis) for the Joint Call for Proposals (CfP) on Video Compression Technology with Capability beyond HEVC [1]. This Call was issued jointly in final form in October 2017 by ISO/IEC JTC 1/SC 29/WG 11 (MPEG) and ITU-T SG16/Q6 (VCEG). The subsequent work to develop a new video coding standard known as Versatile Video Coding (VVC) will be conducted by the ITU-T/ISO/IEC Joint Video Experts Team (JVET), a joint collaborative team that has been established by the parent levels of both organizations.

# Test categories and test cases

Test categories for standard dynamic range (SDR), high dynamic range (HDR) and 360° content had been defined as described in the three sub-sections below. Proponents were encouraged (but not required) to submit results for all test categories. However, submitters were required to provide results for all test cases when submitting in a given test category.

## Standard dynamic range

### Video test sequence formats and frame rates

All test materials were progressively scanned and used 4:2:0 colour sampling with either 8 or 10 bits per sample.

The classes of video sequences are:

Class SDR-A:	3840×2160p 60 fps 10 bits: "FoodMarket4","CatRobot1","DaylightRoad2"
3840×2160p 50 fps 10 bits: "ParkRunning3"
3840×2160p 30 fps 10 bits: "Campfire"

Class SDR-B:	1920×1080p 50fps 8 bits: "BasketballDrive", "Cactus"
1920×1080p 60fps 8 bits: "BQTerrace"
1920×1080p 60fps 10 bits: "RitualDance","MarketPlace"

### Coding conditions of submissions

Constraint cases were defined as follows:

- Constraint Set 1 (a.k.a. Random Access): not more than 16 frames of structural delay, e.g. 16-picture “group of pictures (GOP)”, and random access intervals of 1.1 seconds or less. A random access interval of 1.1 seconds or less was defined as 32 pictures or less for a video sequence with a frame rate of 24, 25 or 30 frames per second, 48 pictures or less for a video sequence with a frame rate of 50 frames per second, 64 pictures or less for a video sequence with a frame rate of 60 frames per second, and 96 pictures or less for a video sequence with a frame rate of 100 frames per second.

- Constraint Set 2 (a.k.a. Low Delay): no picture reordering between decoder processing and output, with bit rate fluctuation characteristics and any frame-level multi-pass encoding techniques to be described with the proposal.

Submissions were required to include encodings for all video sequences in all classes, and decoding from the bitstream was required to produce the full specified number of pictures for the video sequence (no missing pictures).

Submissions were to be made for the test cases (combinations of classes and constraint sets) listed in Table 1.

Table 1 – Combinations of classes and constraint sets for the standard dynamic range category

|  | Class SDR-A | Class SDR-B |
| --- | --- | --- |
| Constraint Set 1 Random Access | X | X |
| Constraint Set 2 Low Delay | – | X |

Submissions for Class SDR-A using Constraint Set 1, and submissions for class SDR-B using both Constraint Set 1 and Constraint Set 2 were evaluated by means of a formal subjective assessment and also by BD PSNR and BD rate [2][3] criteria.

For Class SDR-A using Constraint Set 1 defined above, submissions to this Call were required to provide results for the target bit rates listed in Table 2. The submitted results were not allowed to exceed the target bit rate points.

Table 2 – Target bit rate points not to be exceeded for Class SDR-A

|  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| FoodMarket4 | 950 | 1400 | 2200 | 3800 |
| CatRobot1 | 1500 | 2400 | 3800 | 6000 |
| DaylightRoad2 | 1400 | 2200 | 3500 | 5500 |
| ParkRunning3 | 2000 | 3300 | 6000 | 10000 |
| Campfire | 2000 | 3300 | 6000 | 10000 |

For Class SDR-B using Constraint Set 1 (Random Access) and Constraint Set 2 (Low Delay) as defined above, submissions to this Call were required to provide results for the target rate points listed in Table 3. The submitted results were not allowed to exceed the target bit rate points.

Table 3 – Target bit rate points not to be exceeded for Class SDR-B

|  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| BQTerrace | 400 | 600 | 1000 | 1500 |
| RitualDance | 900 | 1500 | 2300 | 3800 |
| MarketPlace | 500 | 800 | 1200 | 2000 |
| BasketballDrive | 800 | 1200 | 2000 | 3500 |
| Cactus | 500 | 800 | 1200 | 2000 |

Submissions to this Call were required to obey the following additional constraints:

- No use of pre-processing.

- Only using post-processing if it was part of the decoding process, i.e., any processing that was applied to a picture prior to its use as a reference for inter-picture prediction of other pictures. Such processing could also be applied to non-reference pictures.

- It was requested that quantization settings should be kept static except that a one-time change of the quantization settings to meet the target bit rate was allowed. When any change of quantization was used, it was required to be described, including the one-time change.

- Proponents were discouraged from optimizing encoding parameters using non-automatic means as well as optimizing encoding parameters on a per sequence basis within a Class. When any optimization was employed, it was required to be described.

- Proponents were required to not use any part of the video coding test set as a training set for training large entropy coding tables, VQ codebooks, etc.

### Anchors

Two anchors were generated by encoding the above video sequences. The first anchor used the HM 16.16 software package. This anchor used static quantization parameter (QP) settings, except for a one-time change of the quantization parameter from value QP to value QP+1 which was used for some bitstreams in order to meet the defined target bit rates. The anchors obeyed the coding conditions described in section 2.1.2. Additionally, a higher bit rate bitstream was created for the purpose of calculating objective metrics.

The second anchor used the Joint Exploration Test Model (JEM 7.0) software package. The Joint Video Exploration Team (JVET) has developed and maintained the JEM software package in order to study proposed coding tools in a coordinated test model [6]. JEM anchor bitstreams were generated using this software package, and these bitstreams obeyed the coding conditions described in section 2.1.2 and used the same type of QP control as with the HM anchors.

The purpose of the HM and JEM anchors was to facilitate testing in accordance with the BT.500 double-stimulus impairment scale (DSIS) methodology [4] by providing useful reference points demonstrating the behaviour of well-understood configurations of current known technology while obeying the same constraints as were imposed on the proposals.

## High dynamic range

### Video test sequence formats and frame rates

All test material was progressively scanned and used 4:2:0 colour sampling with 10 bits per sample.

The classes of video sequences were:

Class HDR-A:	3840x2160p 60 fps HLG: “DayStreet”, “PeopleInShoppingCenter”,

“SunsetBeach”

Class HDR-B:	1920×1080p 50 fps PQ: "Market3", "Hurdles", "Starting"

1920×1080p 25 fps PQ: "ShowGirl2"

1920×1080p 24 fps PQ: "Cosmos1"

### Coding conditions of submissions

The coding conditions in section 2.1.2 were also applied to the HDR test category, with the following exceptions:

- The quantization settings did not need to be kept static. A static temporal quantizer structure was to be used. However, the quantizer could be adjusted within a frame as a function of the local average luma value and/or the local average chroma value. A one-time change of the temporal quantizer structure was also allowed to meet the target bit rate. If either a local adjustment or a one-time change was used, a description of the adjustment scheme was required to be provided in the descriptive document submission.

- A colour volume transform within the YCbCr 4:2:0 sample domain could be performed on the input video signal prior to coding, and an inverse of the colour volume transform could be performed on the signal after decoding. A colour volume transform modifies a luma or chroma sample at a relative location in the picture based only on the value of the luma or chroma sample and on the value of other component samples located at the same or closest relative location. It was requested that the colour volume transform should be kept static within a frame and within a sequence. The use of a colour volume transform for the purpose of bit rate allocation or denoising was discouraged. If a colour volume transform was used, a description of the transform was required to be provided in the descriptive document submission. Respondents were additionally asked to provide descriptions of any multi-pass processing and any dynamic changes in the colour volume transform.

Submissions were required to be made for the test cases (combinations of classes and constraint sets) listed in Table 4.

Table 4 – Combinations of classes and constraint sets for the High Dynamic Range category

|  | Class HDR-A | Class HDR-B |
| --- | --- | --- |
| Constraint set 1 Random Access | X | X |
| Constraint set 2 Low Delay | – | – |

Submissions for Class HDR-A and Class HDR-B were evaluated by means of a formal subjective assessment, and by the following metrics: PSNR, weighted PSNR, deltaE100 and PSNR-L100, as well as computing the Bjøntegaard Delta-Rate and Delta-Distortion for each metric.

For Class HDR-A using Constraint Set 1 (Random Access) as defined above, submissions were required to provide results for the target bit rate points listed in Table 5, for each of the test cases defined above. The submitted results were not allowed to exceed the target bit rate points.

Table 5 – Target bit rate points not to be exceeded for the HDR-A category

|  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| DayStreet | 680 | 1100 | 1750 | 2800 |
| PeopleInShoppingCenter | 640 | 960 | 1450 | 2150 |
| SunsetBeach | 1500 | 3000 | 6000 | 10000 |

For Class HDR-B using Constraint Set 1 (Random Access) as defined above, submissions were required to provide results for the target bit rate points listed in Table 6 for each of the test cases defined above. The submitted results were not allowed to exceed the target bit rate points.

Table 6 – Target bit rate points not to be exceeded for the HDR-B category

|  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| Market3 | 600 | 900 | 1500 | 2400 |
| ShowGirl2 | 350 | 500 | 750 | 1200 |
| Hurdles | 450 | 800 | 1500 | 2000 |
| Starting | 450 | 600 | 850 | 1400 |
| Cosmos1 | 500 | 900 | 1500 | 3000 |

### Anchors

The conditions and descriptions in section 2.1.3 were also applied to the HDR-A and HDR-B test categories, with the following change for HDR-B:

- The generation of the anchor did not use a static quantization parameter (QP) setting. Instead, the configuration allowed the QP value to vary spatially, where the variation was an explicit function of the average local luma value. A one-time change of the quantization parameter from value QP to value QP+1 could also be applied in some cases in order to meet the defined target bit rates.

## 360° video

### Video test sequence formats and frame rates

All test material was progressively scanned and used 4:2:0 colour sampling with 8 or 10 bits per sample.

The video sequences were:

Class 360:	8192×4096 30 fps 10 bits: "ChairliftRide"

8192×4096 30 fps 8 bits: "KiteFlite”, "Harbor", "Trolley"

6144×3072 60 fps 8 bits: "Balboa"

### Coding conditions of submissions

Submissions were to be made for the test cases (combinations of classes and constraint sets) listed in Table 7.

Table 7 – Combinations of classes and constraint sets for the 360° video category

|  | Class 360 |
| --- | --- |
| Constraint set 1 Random Access | X |
| Constraint set 2 Low Delay | – |

Submissions for Class 360 were evaluated by means of a formal subjective assessment, and by using the following objective metrics: end-to-end WS-PSNR and end-to-end S-PSNR-NN, as well as the Bjøntegaard Delta-Rate and Delta-Distortion for each metric.

Submissions were required to submit results for the target bit rate points (which were not to be exceeded) listed in Table 8 for each of the test cases defined above.

Table 8 – Target bit rate points not to be exceeded for the 360° video category

|  | Target bit rates [kbit/s] |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences | Rate 1 | Rate 2 | Rate 3 | Rate 4 |
| Balboa | 2000 | 3300 | 6000 | 10000 |
| Chairlift | 1500 | 2400 | 4000 | 7000 |
| KiteFlite | 1200 | 2400 | 4000 | 7000 |
| Harbor | 700 | 1200 | 2000 | 3300 |
| Trolley | 1500 | 2400 | 4000 | 7000 |

Submissions to this Call were also required to obey the constraints in section 2.1.2, with the following changes:

- The quantization settings did not need to be kept static. Instead, the quantization settings could be adjusted within a picture as a function of the geometric position. If local adjustment was used, a description of the adjustment scheme was required to be provided in the descriptive document submission.

- Pre-processing could be used to perform a projection mapping operation, and post-filtering could be used to perform an inverse projection mapping operation. The projection mapping algorithms could allow dynamic changes within a sequence if an automatic selection algorithm was used. The same projection mapping operation and inverse projection mapping operation were required to be used for all test sequences in the test case. If a projection mapping was used, a description of the projection mapping technique was required to be provided in the descriptive document submission. Respondents were requested to provide information regarding at least: (i) the coded resolution of the projection map, (ii) the use of padding and blending, (iii) the use of global rotation, (iv) the use of multi-pass projection mapping, and (v) end-to-end WS-PSNR values comparing each test sequence to the result of applying the projection mapping algorithm and then converting this result back to the equirectangular projection format without compression. The coded resolution, defined as the number of samples provided as input to the encoder, was required to be the same for all rate points in all test sequences with the same original, spatial resolution.

- Post-processing of seam artefacts near discontinuous edges after decoding was permitted in submissions but was required to be fully described. If post-processing of seam artefacts was performed, the post-processed pictures were used in computing the end-to-end objective metrics.

Decoder and/or projection format conversion binaries of submissions for Class 360 were required to decode and/or projection format convert each bitstream to a 4:2:0 YUV file of the same resolution as the input sequence, either 8192×4096 or 6144×3072. For subjective evaluation, a viewport using a previously undisclosed view path per sequence was extracted from this decoded video using the 360Lib software tool developed in JVET, as described in [8]. The viewport was generated by rectilinear projection using bilinear interpolation filters; it had a field of view of 78.1×49.1 degrees and a resolution of 1920×1080. The viewport used progressive scanning and 4:2:0 colour sampling with 8 bits per sample.

### Anchors

The anchor conditions and descriptions in section 2.1.3 were also applied to the 360° video category, with the following exception:

- The equirectangular projection input pictures (at 8192x4096 or 6144x3072 resolution) were spherically downsampled using the 360Lib software to 4096×2048 equirectangular projection format sequences representing the full 360°×180° sphere, and 8 luma samples of padding on the left and right sides of the picture were added by copying samples, yielding 4112×2048 padded equirectangular projection pictures, in which the yaw range exceeded 360°, for encoding by the HM and JEM anchors. After decoding, the 4112×2048 padded equirectangular projection pictures used linear position-based weighted blending of the overlapping region to convert to 4096×2048 equirectangular projection format without padding, representing 360° of yaw range. The 360Lib software was used for spherical upsampling to the input resolution of 8K or 6K, respectively. end-to-end objective metrics used the 4096×2048 pictures with blending in their calculations.

# Test design

The test procedure for the formal subjective evaluation was designed to consider three main requirements:

- To be, as much as possible, reliable and effective in ranking the proposals in terms of subjective quality;

- To take into account the evolution of technology and laboratory set-up oriented to the use of a suitable FPD (flat panel display) and video server as the video recording and playing equipment;

- To distribute the workload over different test sites to minimize the risk of failures due to missing or poorly measured results.

The above considerations led to the following decisions:

- To select a well established and reliable subjective testing method, i.e., the DSIS (double stimulus impairment scale; per ITU-R BT.500), also known as DCR (Degradation Category Rating; Rec. ITU-T P.910 [5]), which has already been proven in previous testing experiments to be effective when a precise ranking of coding algorithms was needed;

- To distribute the testing workload (more than 50 test sessions) over as many as possible test laboratories whose reliability was already demonstrated in past evaluation experiments; for this reason, 10 test laboratories were selected as described in section 4.

## DSIS methodology

This test method is commonly adopted when the material to be evaluated shows a range of visual impairment that is distributed well across the grading scale.

This method was used under the schema of evaluation of the impairment, using an impairment scale made of 11 levels, ranging from "0" (lowest quality) to "10" (highest quality). The tests were held in multiple laboratories located in countries speaking different languages: this implies that it is better not to use categorical adjectives (e.g. excellent, good, fair etc.) to avoid any bias due to a possible different interpretation by naïve subjects speaking different languages.

All the video material used for these tests consisted of video clips of 10 seconds duration each.

The structure of the basic test cell (BTC) of the DSIS method is made by two consecutive presentations of the video clip under test; at first the original version of the video clip (SRC, i.e., source reference content) is displayed, announced by the letter “A” on a mid-grey screen (for half a second); immediately afterwards, the coded version of the video clip (PVS, i.e., processed video sequence), announced by the letter “B” on a mid-grey screen (for half a second), is presented; then a message is displayed for 5 seconds asking the viewers to vote (see Figure C-1)

Figure 1 – DSIS BTC

## How to express the visual quality opinion with DSIS

The viewers were asked to express their vote by putting a number in a box on a scoring sheet.

The scoring sheet for the DSIS test was made with a section for each BTC; each section had a series of numbered boxes (see Figure C-2).

The viewers would write a number in the box that corresponded to the number of the message “Vote N” that was shown on the screen. For example, by writing "10", the subject could declare that he/she saw no difference between the SRC and the PVS, while by writing "0" the subject could declare that he/she saw a significant and clearly visible amount of difference between the SRC and the PVS across the entire observed image.

The vote was required to be written when the message "Vote N" appears on the screen, where the number "N" was a numerical progressive indication on the screen aiming to help the viewing subjects to use the appropriate box on the scoring sheet.

Figure 2 – Example of DSIS test method scoring sheet

## Training and stabilization phase

The outcome of such a subjective test is highly dependent on a proper training of the test subjects.

For this purpose, each subject was trained by means of a short practice (training) session.

The video materials used for the training session were carefully selected from those of the test, taking care to represent as much as possible the range of visual quality and the type of impairment expected to appear during the test.

The stabilization phase used the test material of a test session; three BTCs, containing one sample of best quality, one of the worst quality and one of medium quality, which were duplicated at the beginning of the test session. In this way, the test subjects were given an immediate impression of the quality range they were expected to evaluate during that session.

The scores of the stabilization phase were discarded. Consistency of the behaviour of the subjects was checked by inserting in the session a BTC in which the original was compared to the original.

## The laboratory setup

The laboratories for the subjective assessments were set up following, as much as possible, the more relevant ITU Recommendations (esp. ITU-R BT.500 and ITU-T P.910).

For the SDR and 360° video tests, high quality TV sets (OLED LG 55”) were used, taking care of disabling (as much as possible) all the internal local post-processing of the images (e.g., noise reduction, etc.).

For the HDR tests, appropriate displays were used (Sony BVM X300 for HDR-A, SIM2 for HDR-B)

The video play servers used to play the video clips on the displays were designed to support the presentation of both HD and UHD video formats, at 24, 30, 50 and 60 frames per second, without any limitation, or without introducing any temporal or visual artefacts.

A new version (2_0_2) of the MUP player was used for all three categories of tests (SDR, 360°, and HDR).

### Viewing distance, seats and monitor size

For the SDR and 360° tests, a 55” commercially available OLED TV set was used; this arrangement allowed the seating of three subjects at the same time in front of the display.

For the HDR-A test, a 30” Sony BVM 300 monitor was used; in this case, two subjects were seated in front of the display.

For the HDR-B test, a 47” monitor was used; in this case, three subjects were seated in front of the display.

The viewing distance was 2H, where H is equal to the height of the active part of the screen.

### Viewing environment

The test laboratory was carefully protected from any external visual or audio pollution.

Internal general, light was kept low (just enough to allow the viewing subjects to fill out the scoring sheets) and a uniform light was placed behind the monitor; this light had an intensity of maximum 30 nits. No light source was allowed to be directed to the screen or to create reflections; the ceiling, floor and walls of the laboratory were made of non-reflecting black or dark grey material (e.g., carpet or velvet).

# Logistics and conduct of the subjective tests

A total of 10 test labs, which were experienced in their respective fields, contributed to the test effort. Tests were partially overlapping (conducted in at least two labs) such that verification about the validity and comparability of the tests would be possible. In particular, the test labs were as follows:

- For SDR @ HD resolution: BBC (England), Queen Mary University of London (QMUL – England), University of the West of Scotland (UWS – Scotland);

- For SDR @UHD resolution: Centrum Wiskunde & Informatica (CWI – Netherlands)

- For 360°: University of Padova and GBTech (Italy);

- For HDR PQ: CRIT and Sisvel Technologies (Italy);

- For HDR HLG: DBW and EVATech (Italy).

GBTech, EVATech and CWI collaborated on general coordination and took care of contacts and administrative matters with the proponents; designing the test sessions and creating the scripts, as well as visually checking all files on the disks received. Furthermore, they created the dynamic viewports for the 360° video test cases (not known to proponents beforehand), which had been selected with help from Jill Boyce (of Intel) as an independent expert.

A large number of human viewing subjects were hired and participated in the testing at the above test sites; the number of subjects was chosen to be very high (around twice the theoretical number) to ensure a low level of fatigue and maintain a high degree of response and efficiency for the participants.

Following this approach, more than 2000 people were involved as test participants to perform a total of 58 test sessions, for a total testing time exceeding 500 hours including test execution and laboratory set-up time.

The design of the 58 test sessions was done by the test coordinator, with the support of a CWI expert.

This is an example of the schedule of the planned test activity for one day:

- A time slot of one hour was dedicated every morning and afternoon to welcome, to screen and to train the viewers.

- After the subjects’ screening for visual acuity and colour blindness, the subjects were grouped in testing groups.

The SDR testing effort required 36 test sessions; HDR tests were conducted with 14 test sessions; and 360° video required 8 test sessions. The medium duration of a test session never exceeded 20 minutes; this approach was chosen to ensure a low fatigue level and high efficiency for the viewers.

All scores were collected by paper sheets; a novel data acquisition technique was used (which had previously been successfully tested in other JVET testing experiments) assigning the data entry task to some of the viewers; this approach was considered very safe and efficient (two viewers dialled in the data, and another two checked the inputs) and allowed to lower the level of stress of the participants relative to alternatives that would involve changing the cognitive load of the test subjects during the viewing activities.

The exact test procedure was also described in the joint Call for Proposals [1].

# Summary of Responses

A total of 46 submissions were received in the three category groups, as follows:

- 22 SDR submissions with 36 test cases each,

- 12 HDR submissions with 32 test cases each,

- 12 360° submissions with 20 test cases each.

Additional partners were added to some proposals later, bringing the total number of submitting institutions to 32. All proponent groups properly completed the submission process.

## List of submissions

The submissions (including links to the JVET document site where the documents and associated data spreadsheets are available) are given below:

JVET-J0011 Description of SDR video coding technology proposal by DJI and Peking University [Z. Wang, X. Meng, C. Jia, J. Cui, S. H. Wang, S. S. Wang, S. Ma (Peking University), W. Li, Z. Miao, X. Zheng (DJI)]

JVET-J0012 Description of SDR and HDR video coding technology proposal by Ericsson and Nokia [R. Sjöberg, K. Andersson, R. Yu, Z. Zhang, P. Wennersten (Ericsson), J. Lainema, A. Hallapuro, A. Aminlou, M. Hannuksela, R. Ghaznavi-Youvalari, J. Ridge (Nokia)]

JVET-J0013 Description of SDR video coding technology proposal by ETRI and Sejong University [J. Kang, H. Lee, S.-C. Lim, J. Lee, H. Y. Kim (ETRI), N.-U. Kim, Y.-L. Lee (Sejong Univ.)]

JVET-J0014 Description of SDR, HDR and 360° video coding technology proposal by Fraunhofer HHI [M. Albrecht, C. Bartnik, S. Bosse, J. Brandenburg, B. Bross, J. Erfurt, V. George, P. Haase, P. Helle, C. Helmrich, A. Henkel, T. Hinz, S. de Luxan Hernandez, S. Kaltenstadler, H. Kirchhoffer, C. Lehmann, W.-Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, B. Stallenberger, J. Stegemann, K. Sühring, G. Tech, G. Venugopal, S. Walter, A. Wieckowski, T. Wiegand, M. Winken (Fraunhofer HHI)]

JVET-J0015 Description of SDR, HDR and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories [X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. Ye (InterDigital), T. Lu, F. Pu, P. Yin, W. Husak, T. Chen (Dolby)]

JVET-J0016 Description of SDR video coding technology proposal by KDDI [K. Kawamura, Y. Kidani, S. Naito (KDDI Corp.)]

JVET-J0017 Description of SDR video coding technology proposal by LG Electronics [M. Koo, J. Heo, J. Nam, N. Park, J. Lee, J. Choi, S. Yoo, H. Jang, L. Li, J. Lim, S. Paluri, M. Salehifar, S. Kim (LGE)]

JVET-J0018 Description of SDR video coding technology proposal by MediaTek [C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-J0019 Description of 360° video coding technology proposal by MediaTek [J.-L. Lin, Y.-H. Lee, C.-H. Shih, S.-Y. Lin, H.-C. Lin, S.-K. Chang, P. Wang, L. Liu, C.-C. Ju (MediaTek)]

JVET-J0020 Description of SDR video coding technology proposal by Panasonic [T. Toma, T. Nishi, K. Abe, R. Kanoh, C. Lim, J. Li, R. Liao, S. Pavan, H. Sun, H. Teo, V. Drugeon (Panasonic)]

JVET-J0021 Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions [Y.-W. Chen, W.-J. Chien, H.-C. Chuang, M. Coban, J. Dong, H. E. Egilmez, N. Hu, M. Karczewicz, A. Ramasubramonian, D. Rusanovskyy, A. Said, V. Seregin, G. Van Der Auwera, K. Zhang, L. Zhang (Qualcomm), P. Bordes, Y. Chen, C. Chevance, E. François, F. Galpin, M. Kerdranvat, F. Hiron, P. de Lagrange, F. Le Léannec, K. Naser, T. Poirier, F. Racapé, G. Rath, A. Robert, F. Urban, T. Viellard (Technicolor)]

JVET-J0022 Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version [P. Bordes, Y. Chen, C. Chevance, E. François, F. Galpin, M. Kerdranvat, F. Hiron, P. de Lagrange, F. Le Léannec, K. Naser, T. Poirier, F. Racapé, G. Rath, A. Robert, F. Urban, T. Viellard (Technicolor), Y. Chen, W.-J. Chien, H.-C. Chuang, M. Coban, J. Dong, H. E. Egilmez, N. Hu, M. Karczewicz, A. Ramasubramonian, D. Rusanovskyy, A. Said, V. Seregin, G. Van Der Auwera, K. Zhang, L. Zhang (Qualcomm)]

JVET-J0023 Description of SDR and 360° video coding technology proposal by RWTH Aachen University [M. Bläser, J. Sauer, M. Wien (RWTH Aachen Univ.)]

JVET-J0024 Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario [A. Alshin, E. Alshina, K. Choi, N. Choi, W. Choi, S. Jeong, C. Kim, J. Min, J. H. Park, M. Park, M. W. Park, Y. Piao, A. Tamse, A. Dsouza, C. Pujara (Samsung), H. Chen, J. Chen, R. Chernyak, S. Esenlik, A. Filippov, S. Gao, S. Ikonin, A. Karabutov, A. M. Kotra, X. Lu, X. Ma, V. Rufitskiy, T. Solovyev, V. Stepin, M. Sychev, T. Wang, Y.-K. Wang, W. Xu, H. Yang, V. Zakharchenko, H. Zhang, Y. Zhao, Z. Zhao, J. Zhou, C. Auyeung, H. Gao, I. Krasnov, R. Mullakhmetov, B. Wang, Y. F. Wong, G. Zhulikov (Huawei), A. Abbas, D. Newman (GoPro), J. An, X. Chen, Y. Lin, Q. Yu, J. Zheng (HiSilicon)]

JVET-J0025 Description of SDR, HDR and 360° video coding technology proposal by Huawei, GoPro, HiSilicon, and Samsung – general application scenario [H. Chen, J. Chen, R. Chernyak, S. Esenlik, A. Filippov, S. Gao, S. Ikonin, A. Karabutov, A. M. Kotra, X. Lu, X. Ma, V. Rufitskiy, T. Solovyev, V. Stepin, M. Sychev, T. Wang, Y.-K. Wang, W. Xu, H. Yang, V. Zakharchenko, H. Zhang, Y. Zhao, Z. Zhao, J. Zhou, C. Auyeung, H. Gao, I. Krasnov, R. Mullakhmetov, B. Wang, Y. F. Wong, G. Zhulikov (Huawei), A. Abbas, D. Newman (GoPro), J. An, X. Chen, Y. Lin, Q. Yu, J. Zheng (HiSilicon), A. Alshin, E. Alshina, K. Choi, N. Choi, W. Choi, S. Jeong, C. Kim, J. Min, J. Park, M. Park, M. W. Park, Y. Piao, A. Tamse, A. Dsouza, C. Pujara (Samsung)]

JVET-J0026 Description of SDR and HDR video coding technology proposal by Sharp and Foxconn [K. Misra, J. Zhao, A. Segall, W. Zhu, B. Choi, F. Bossen, M. Horowitz, P. Cowan, Y. Yasugi, T. Hashimoto, T. Zhou, T. Ikai, T. Chujoh, T. Aono (Sharp), Y.-J. Chang, H.-Y. Jiang, T.-H. Li, Y.-C. Yang (Foxconn Technology Group)]

JVET-J0027 Description of SDR and HDR video coding technology proposal by NHK and Sharp [S. Iwamura, S. Nemoto, K. Iguchi, A. Ichigaya (NHK), K. Misra, J. Zhao, A. Segall, W. Zhu, B. Choi, F. Bossen, M. Horowitz, P. Cowan, Y. Yasugi, T. Hashimoto, T. Zhou, T. Ikai, T. Chujoh, T. Aono (Sharp)]

JVET-J0028 Description of SDR and HDR video coding technology proposal by Sony [T. Suzuki, M. Ikeda, K. Sharman (Sony)]

JVET-J0029 Description of SDR video coding technology proposal by Tencent [X. Li, X. Xu, X. Zhao, J. Ye, L. Zhao, S. Liu (Tencent)]

JVET-J0030 Description of 360° video coding technology proposal by TNO [A. Gabriel, E. Thomas (TNO)]

JVET-J0031 Description of SDR video coding technology proposal by University of Bristol [D. Bull, F. Zhang, M. Afonso (Univ. of Bristol)]

JVET-J0032 Description of SDR video coding technology proposal by University of Science and Technology of China, Peking University, Harbin Institute of Technology, and Wuhan University (IEEE 1857.10 Study Group) [F. Wu, D. Liu, J. Xu, B. Li, H. Li, Z. Chen, L. Li, F. Chen, Y. Dai, L. Guo, Y. Li, Y. Li, J. Lin, C. Ma, N. Yan (USTC), W. Gao, S. Ma, R. Xiong, Y. Xu, J. Li (Peking Univ.), X. Fan, N. Zhang, Y. Wang, T. Zhang, M. Gao (Harbin Inst. Tech.), Z. Chen, Y. Zhou, X. Pan, Y. Li, F. Liu, Y. Wang (Wuhan Univ.)]

JVET-J0033 Description of 360° video coding technology proposal by Zhejiang University [Y. Sun, X. Huangfu, R. Zheng, B. Wang, L. Yu (Zhejiang Univ.)]

## Technology elements of submissions

A technical analysis indicated that in terms of overall coding architecture, all proposals were using the well-known “hybrid” video coding approach. The majority of proposals used the JEM and its software as an implementation basis, whereas a few proposals implemented their own software from scratch. At the individual coding tool level, a larger degree of diversification was observed. Some common elements of many proposals were:

- Partititioning: Quad/binary/ternary tree

- Intra prediction using directional modes, DC and planar, sample smoothing and some inheritance of chroma modes and sample prediction from luma

- Inter prediction using advanced motion vector prediction, affine models, sub-block partitioning

- Switchable primary and mode-adaptive secondary transforms

- Deblocking filter basically similar to HEVC/JEM, longer filter support for certain cases

- Adaptive loop filter similar to JEM, modified classification methods and filter support

- Template matching tools (decoder side) for purposes of mode/MV derivation and sample prediction both in intra and inter coding JEM

- Mostly conventional concepts for quantization and context-adaptive arithmetic coding, the latter with higher precision compared to HEVC

Some highlighted elements found in individual proposals were:

- Neural networks for prediction, loop filtering, upsampling

- Additional elements using template matching

- Intra block copy / current picture referencing

- Additional non-linear and statistics-based loop filters

- Additional linear and non-linear elements in prediction

- Finer partitioning: Asymmetric, geometric

For HDR, most proposals suggested using the same basic coding scheme as for SDR, with some additional out-of-loop processing and additional encoder optimization. However, some proposals also had in-loop HDR-specific tools such as:

- In-loop “reshaping”

- Inferred QP

- Internal bit-depth increase

- Modified loop filters

For 360° video, a substantial variety of projection formats was used, although several proposals used the equiangular cube map projection (EAP) with some individual modifications of face arrangements. 8 out of 12 proposals included aspects having a specific effect on the coding loop, in particular by proposing coding tools where the decoder requires knowledge about the positions and orientations of face boundaries. This was used to disable or modify coding tools at such positions, in order to avoid using "wrong" neighbors, or to use correct spherical neighbors (e.g., for intra prediction, loop filtering, CABAC context determination, etc.). The approach of "geometry padding" was also used in several proposals for motion compensation referencing across face boundaries.

A more detailed survey of proposal elements from the proposal contributions as listed in section 5.1 can be found in [9], [10], and [11].

# Test results and graphs

## Test results per sequence

The test results are shown in a set of graphs which depict the results for each test sequence. These were produced to show the relationship between the HEVC and JEM anchors and the various proposals for each sequence over the four bit rate points. The results are based on an 11-grade scale, where 0 stands for the worst and 10 stands for the best quality. Each mean opinion score (MOS) data point is shown together with a 95% confidence interval (CI) in addition to the mean MOS value. The presence or absence of overlapping confidence intervals was used as a basic measure of the statistical significance of differences between the MOS scores for different encodings (as detailed pairwise t-test analysis did not seem necessary for assessment of the overall results of the CfP).

Bitstream decoding capability (for at least some of the bitstreams) and the conformance of all encoded bit rates to the specified limits were checked for each proposal, and no significant problems were identified in this cross-checking process. The results for individual proposals are shown in the graphs without identification of the specific proposals (as is further explained in section 6.2), whereas the HEVC anchors are identified by red circles, and the JEM anchors are identified by blue circles. This way, it is possible, for example, to identify the quality of the best-performing proposals for a certain test sequence at a certain bit rate point in comparison to the anchor, and whether some proposals may even perform similarly or better than an anchor at a higher bit rate point.

In the graphs, proposals are ordered for each case of test sequence and bit rate point, ranked by their respective MOS performance (i.e., it is not necessarily the same proposal that appears in the same column for different bit rates and for different graphs). It can be observed that:

- Most of the proposals are clearly better (as indicated by non-overlapping confidence intervals) than the HM anchors at the same bit rate.

- The best-performing proposals are often better (as indicated by non-overlapping confidence intervals) than the JEM anchors at the same bit rate, and are also sometimes better than the HM anchors at the next higher bit rate, or in some cases event two bit rate steps higher.

- The best-performing proposals are often equivalent (by overlapping confidence intervals) in subjective quality to the HM anchors at two bit rate steps higher.

It must, however, be considered that different individual proposals were the “best performing” in the various cases shown below. Further, in the SDR and HDR categories, the benefit compared to HEVC and the JEM is slightly larger for UHD than it is for HD picture resolution.

### Results for the 360° Constraint Set 1 (Random Access) category

### Results for the SDR HD Constraint Set 1 (Random Access) category

### Results for the SDR HD Constraint Set 2 (Low Delay) category

### Results for the SDR UHD Constraint Set 1 (Random Access) category

### Results for the HDR HLG Constraint Set 1 (Random Access) category

### Results for the HDR PQ Constraint Set 1 (Random Access) category

## Overall results analysis

### Mean MOS

The tables below show the mean MOS values (on the aforementioned 0..10 scale), computed over all test cases of the three categories for each proposal and for the anchors. All proposal numbers have been “anonymized” to “Pnn” in the tables to avoid attempting to assess the performance of specific individual proposals, as the primary goals of the CfP were to assess the overall readiness of technology for the development of a new standard and to solicit contributions of suitable technology elements for further detailed study, rather than to rank individual specific proposals as complete packages.

Table 9 – Mean MOS values for proposals in the three categories

| SDR | Mean MOS |  | HDR | Mean MOS |  | 360° | Mean MOS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pnn | 6.53 |  | Pnn | 6.04 |  | Pnn | 6.20 |
| Pnn | 6.46 |  | Pnn | 6.00 |  | Pnn | 6.19 |
| Pnn | 6.41 |  | Pnn | 5.94 |  | Pnn | 6.06 |
| Pnn | 6.37 |  | Pnn | 5.93 |  | Pnn | 6.03 |
| Pnn | 6.33 |  | Pnn | 5.86 |  | Pnn | 5.99 |
| Pnn | 6.33 |  | Pnn | 5.85 |  | Pnn | 5.96 |
| Pnn | 6.26 |  | Pnn | 5.80 |  | Pnn | 5.86 |
| Pnn | 6.23 |  | Pnn | 5.67 |  | Pnn | 5.69 |
| Pnn | 6.17 |  | JEM | 5.62 |  | Pnn | 5.67 |
| Pnn | 6.15 |  | Pnn | 5.60 |  | Pnn | 5.51 |
| Pnn | 6.13 |  | Pnn | 5.59 |  | Pnn | 5.45 |
| Pnn | 6.11 |  | Pnn | 5.45 |  | JEM | 5.11 |
| Pnn | 6.04 |  | Pnn | 5.11 |  | HM | 3.79 |
| Pnn | 6.04 |  | HM | 4.14 |  | Pnn | 3.45 |
| Pnn | 6.03 |  |  |  |  |  |  |
| Pnn | 6.03 |  |  |  |  |  |  |
| Pnn | 6.01 |  |  |  |  |  |  |
| JEM | 6.01 |  |  |  |  |  |  |
| Pnn | 6.00 |  |  |  |  |  |  |
| Pnn | 5.96 |  |  |  |  |  |  |
| Pnn | 5.94 |  |  |  |  |  |  |
| Pnn | 5.88 |  |  |  |  |  |  |
| Pnn | 5.86 |  |  |  |  |  |  |
| HM | 4.57 |  |  |  |  |  |  |

The above results indicate that the average increase in subjective quality of the best-performing proposals compared to the HM anchors was around 2 steps on the scale; and compared to the JEM anchors, an improvement of approximately 0.5 steps was achieved. These gaps are larger for the case of the 360° video category (around 2.5 steps benefit over the HM and 1 step over the JEM). This is likely explainable by the fact that, in addition to better compression by use of proposed coding tools, other projection formats were used which provide further benefit in quality compared to the anchors (both anchors were using padded equirectangular projection).

### Analysis of subjective quality improvement compared to JEM anchors at the same bit rate points

The mean MOS analysis of section 6.2.1 does not indicate whether the subjective quality improvement suggested by the MOS results is statistically significant (as would be indicated by non-overlapping confidence intervals). The range of the confidence intervals is typically larger than the differences in mean values. Therefore, the following method has been used to obtain further evidence about the subjective benefit. For each test case, each proposal was compared against the JEM anchor. Then the overlap or lack of overlap of confidence intervals are considered as follows (see Figure below),

- If the lower limit of the proposal’s confidence interval is higher than the upper limit of the JEM’s confidence interval, +1 credit is given;

- If the upper limit of the proposal’s confidence interval is below the lower limit of the JEM’s confidence interval, 1 credit is given;

- Otherwise, 0 credits are given.

Finally, the credits are accumulated over all test cases. This means that in the SDR category (60 test cases), the maximum number of accumulated credits would be 60, in the HDR category it would be 32, and in the 360° category it would be 20.

Figure 3 – Illustration of scoring credit methodology

The results are shown in the tables below for the three test categories, where, in the cases of SDR (UHD Random Access, HD Random Access, HD Low Delay) and HDR (HLG and PQ in Random Access), also the results of sub-categories are shown. The proposals are ordered in the table by decreasing rank of credits, and it was found that different proposals were best performers in different sub-categories, and thus the Pnn codes in one row of the table may not necessarily refer to the same proposal; as a consequence, the overall credits in the leftmost column cannot be accumulated from the same rows of the sub-category columns.

Table 10 – Scoring credit values for SDR proposals

| SDR | credits |  | UHD (RA) | credits |  | HD-RA | credits |  | HD-LD | credits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pnn | 23 |  | Pnn | 6 |  | Pnn | 10 |  | Pnn | 9 |
| Pnn | 21 |  | Pnn | 6 |  | Pnn | 9 |  | Pnn | 7 |
| Pnn | 20 |  | Pnn | 6 |  | Pnn | 9 |  | Pnn | 7 |
| Pnn | 18 |  | Pnn | 6 |  | Pnn | 8 |  | Pnn | 6 |
| Pnn | 15 |  | Pnn | 4 |  | Pnn | 8 |  | Pnn | 5 |
| Pnn | 14 |  | Pnn | 4 |  | Pnn | 7 |  | Pnn | 5 |
| Pnn | 14 |  | Pnn | 4 |  | Pnn | 7 |  | Pnn | 4 |
| Pnn | 11 |  | Pnn | 4 |  | Pnn | 5 |  | Pnn | 4 |
| Pnn | 9 |  | Pnn | 3 |  | Pnn | 4 |  | Pnn | 4 |
| Pnn | 8 |  | Pnn | 3 |  | Pnn | 3 |  | Pnn | 4 |
| Pnn | 5 |  | Pnn | 3 |  | Pnn | 2 |  | Pnn | 2 |
| Pnn | 4 |  | Pnn | 2 |  | Pnn | 2 |  | Pnn | 2 |
| Pnn | 2 |  | Pnn | 1 |  | Pnn | 1 |  | Pnn | 1 |
| Pnn | 1 |  | JEM | 0 |  | Pnn | 1 |  | JEM | 0 |
| Pnn | 1 |  | Pnn | 0 |  | JEM | 0 |  | Pnn | 0 |
| Pnn | 1 |  | Pnn | 0 |  | Pnn | 0 |  | Pnn | 0 |
| JEM | 0 |  | Pnn | 0 |  | Pnn | 0 |  | Pnn | 0 |
| Pnn | 0 |  | Pnn | −1 |  | Pnn | 0 |  | Pnn | −1 |
| Pnn | −1 |  | Pnn | −3 |  | Pnn | −1 |  | Pnn | −2 |
| Pnn | −2 |  | Pnn | −3 |  | Pnn | −1 |  | Pnn | −2 |
| Pnn | −2 |  | Pnn | −3 |  | Pnn | −2 |  | Pnn | −2 |
| Pnn | −5 |  | Pnn | −3 |  | Pnn | −2 |  | Pnn | −2 |
| Pnn | −7 |  | Pnn | −4 |  | Pnn | −2 |  | Pnn | −4 |
| HM | −47 |  | HM | −15 |  | HM | −16 |  | HM | −16 |

Table 11 – Scoring credit values for HDR and 360° proposals

| HDR | credits |  | HLG | credits |  | PQ | credits |  | 360° | credits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pnn | 10 |  | Pnn | 5 |  | Pnn | 6 |  | Pnn | 9 |
| Pnn | 7 |  | Pnn | 3 |  | Pnn | 5 |  | Pnn | 9 |
| Pnn | 7 |  | Pnn | 2 |  | Pnn | 5 |  | Pnn | 8 |
| Pnn | 6 |  | Pnn | 1 |  | Pnn | 5 |  | Pnn | 7 |
| Pnn | 5 |  | Pnn | 1 |  | Pnn | 4 |  | Pnn | 7 |
| Pnn | 4 |  | Pnn | 1 |  | Pnn | 3 |  | Pnn | 6 |
| Pnn | 3 |  | Pnn | 1 |  | Pnn | 2 |  | Pnn | 5 |
| Pnn | 1 |  | Pnn | 1 |  | Pnn | 1 |  | Pnn | 4 |
| JEM | 0 |  | JEM | 0 |  | JEM | 0 |  | Pnn | 2 |
| Pnn | 0 |  | Pnn | 0 |  | Pnn | −1 |  | Pnn | 1 |
| Pnn | −1 |  | Pnn | −1 |  | Pnn | −1 |  | Pnn | 1 |
| Pnn | −5 |  | Pnn | −2 |  | Pnn | −4 |  | JEM | 0 |
| Pnn | −11 |  | Pnn | −2 |  | Pnn | −9 |  | HM | −9 |
| HM | −25 |  | HM | −11 |  | HM | −14 |  | Pnn | −12 |

Overall, it can be concluded that in all categories (and sub-categories) the best-performing proposals provided a subjective quality superior to the JEM anchors in around 1/3 of the test cases. In this context, it should, however, be considered that the JEM anchors typically had an almost transparent quality at least at the highest bit rate point, such that it would have been difficult to obtain a perceivable subjective improvement over that. Therefore, typically, cases with significant subjective improvement over the JEM were mainly found at the lower bit rate points.

### Analysis of subjective quality improvement compared to HM anchors at higher bit rate points

A similar comparison as performed in the previous section appeared meaningless with regard to the HM anchors, as the majority of proposals performed significantly better in terms of subjective quality at a given bit rate point, except for some cases of the highest bit rate, where the subjective quality of the HM anchors was approaching the transparent range, such that it became difficult to demonstrate subjective benefit. Therefore, an analysis was conducted that would unveil whether a proposal at a lower bit rate point would be performing even better than an HM anchor at a higher bit rate point. To simplify this analysis, for each bit rate point the best-performing proposal was picked. Such an approach would allow an approximate analysis of what the bit rate benefit of any currently-best technology over HEVC would be; moreover, as all best-performing proposals were conceptually very similar, it could be anticipated that beneficial elements from individual proposals could be straightforwardly combined during the VVC standard development process, such that similar gains would finally be achievable at a minimum.

As individual bit rate values were defined per sequence (see section 2), the average bit rate savings were computed over the entire set. For example, if R1 is 1 Mbit/s, and R2 is 1.6 Mbit/s, the bit rate reduction would be (1R1/R2) 100%.

The table below shows the percentage of cases where the best-performing proposal (which may not always be the same technology for a given sequence / rate point) is subjectively better than the HM anchor at a higher bit rate point for the same sequence. The percentage bit rate saving is also indicated, e.g., for almost half of the test cases a best proposal at R1 performed better than the HM anchors at R3, whereas it used 37.5% lower bit rate.

Table 12 – Percentage of cases with best-performing proposal better than HM at higher bit rate

| Pbest vs HM | R1 vs R2 | R1 vs R3 | R1 vs R4 | R2 vs R3 | R2 vs R4 | R3 vs R4 |
| --- | --- | --- | --- | --- | --- | --- |
| SDR UHD (RA) | 60% | 40% | 0% | 80% | 0% | 20% |
| SDR HD-RA | 40% | 0% | 0% | 20% | 0% | 20% |
| SDR HD-LD | 40% | 0% | 0% | 0% | 0% | 0% |
| HLG (RA) | 67% | 0% | 0% | 67% | 0% | 33% |
| PQ (RA) | 40% | 0% | 0% | 40% | 0% | 20% |
| 360° (RA) | 40% | 20% | 0% | 20% | 0% | 60% |
| Total | 46% | 11% | 0% | 36% | 0% | 25% |
| Rate saving | 37.5% | 65% | 78% | 43% | 35% | 39% |

The complementary analysis was also conducted, i.e. it was analysed whether the HEVC HM performed better at a higher bit rate than a best-performing proposal at a lower bit rate. Results are given in the table below.

Table 13 – Percentage of cases with HM at higher bit rate better than best-performing proposal

| HM vs Pbest | R1 vs R2 | R1 vs R3 | R1 vs R4 | R2 vs R3 | R2 vs R4 | R3 vs R4 |
| --- | --- | --- | --- | --- | --- | --- |
| SDR UHD (RA) | 0% | 0% | 60% | 0% | 0% | 0% |
| SDR HD-RA | 0% | 60% | 100% | 0% | 80% | 0% |
| SDR HD-LD | 0% | 60% | 80% | 0% | 80% | 0% |
| HLG (RA) | 0% | 0% | 100% | 0% | 67% | 0% |
| PQ (RA) | 0% | 60% | 100% | 0% | 60% | 0% |
| 360° (RA) | 0% | 40% | 80% | 0% | 40% | 0% |
| Total | 0% | 39% | 86% | 0% | 54% | 0% |
| Rate saving | 37.5% | 65% | 78% | 43% | 35% | 39% |

It can be observed that at the next higher bit rate, the HM anchor was never better than a best-performing proposal at a lower bit rate, across all categories and test cases. From this, together with the previous table, the conclusion can be drawn that, over a wide range of bit rates, technology exists that clearly saves approximately 40% rate compared to HM with at least equal quality, where approximately one third of all test cases was even judged better. By tendency, the number of test cases where a proposal at a lower bit rate is better than the HM at a higher bit rate increases towards lower bit rates; this is again due to the fact that for some test cases the HM already comes close to transparent quality at the highest bit rates.

It is further interesting to see that when comparing results over two steps of bit rate (R1 vs R3, R2 vs R4), in particular in the SDR-UHD subcategory, there is no test case where the HM performs better. From this, it can be concluded that technology exists for this specific category which provides equal or better quality with approximately 65% bit rate reduction. A similar tendency can be observed for the HDR-HLG subcategory (which also comes with UHD resolution), where at least the HM at R3 is never better than a best-performing proposal at R1. This is remarkable, as the need for higher compression is particularly relevant for ultra-high-resolution video.

# Conclusions

The subjective test results indicate that a clear quality improvement has been achieved by the majority of proposals, as compared to the quality of the HM anchors, and for some best-performing proposals still a significant quality improvement can be found in comparison to the JEM anchors. Furthermore, it was not possible to identify a “clear winner”, although some of the best-performing proposals used very similar sets of coding tools with different degrees and different techniques of encoder optimization (and encoding optimization is outside the anticipated scope of standardization). This similarly applies to all test categories (SDR, HDR and 360°), as well as both constraint set conditions (Random Access and Low Delay). For a considerable number of test points, the subjective quality of a proposal encoding was as good, for the best-performing proposals, as the quality of the anchors with roughly double the bit rate. Objective (PSNR based) bit rate savings compared to HM anchors were also reported as more than 40% compared to the HEVC HM anchors for some proposals, and 10–15% compared to the JEM anchors. Even when considering the fact that some proposals certainly used more advanced encoder optimization than the anchors (and also some needed significantly higher encoder and decoder run times), a substantial gain can be identified for the prospective starting point of the new generation of video coding standard to be developed in the VVC initiative. The analysis in section 6.2.3 shows that with 40% bit rate savings, proposals were already equal or better in terms of subjective quality compared to the HEVC HM anchors, and for the SDR UHD test cases, already 65% bit rate savings have been achieved relative to the HEVC HM anchors with at least equal quality.

The assessment of the proposed technology, as performed at the tenth JVET meeting held in San Diego, US, between 10 and 20 April 2018, revealed that all proposed algorithms were based on the traditional hybrid coding approach combining motion-compensated prediction between video frames together with intra-picture prediction, closed-loop operation with in-loop filtering, 2D transformation of the spatial residual signals, and advanced adaptive entropy coding. The results also indicate that the coding tools that provide benefit for SDR video provide additional benefit for the HDR and 360° categories, such that a high degree of versatility is achievable by using an identical core coding architecture for different application cases. One common element used in many proposals was the use of more granular block partitioning known as a combination of quad-tree (QT), binary tree (BT), and ternary tree (TT). As an initial step toward moving forward into collaborative work, a first draft of VVC [12] and a corresponding VVC test model (VTM) [13] were defined, starting from a subset of HEVC technical elements, where HEVC’s cascaded-QT approach was replaced by the aforementioned QT/BT/TT scheme and aspects of HEVC that seemed potentially unnecessary were omitted. The VTM is the basis of a software implementation, which will enable more rigorous assessment of other building blocks and coding tools by starting a process of Core Experiments for development of the VVC standard.

The very good results submitted in response to the CfP indicate that it should be feasible to reach or even outperform the goal of VVC to provide the same video quality as HEVC with substantially lower bit rate (e.g., half the bit rate), and to achieve this goal by the targeted date of 2020 for completing the first version of VVC.

# Acknowledgments

Below are listed the organizations and test managers that organized, managed and conducted the tests at each test laboratory.

- SDR tests

  - Queen Mary University of London (QMUL): Fiona Rivera

  - BBC: Marta Mrak

  - University of the West of Scotland (UWS): Stamos Katsigiannis

  - Centrum Wiskunde & Informatica (CWI): Pablo Cesar

- HDR tests

  - CRIT: Roberto Iacoviello

  - Sisvel Technologies: Alessandra Mosca, Giovanni Ballocca

  - DBW: Marco Cossu

  - EVATech: Vittorio Baroncini

- 360° tests

  - Università di Padova: Simone Milani, Sebastiano Verde

  - GBTech: Giacomo Baroncini

The analysis described in sections 6.2.2 and 6.2.3 was designed and performed by Mathias Wien.

Appreciation is also expressed to the 32 organizations (and the contributing individuals at those organizations) that submitted the proposals in response to the Call, as it is their efforts that have resulted in the positive outcome of the Call and their work that has become the basis for the subsequent collaborative process to develop the upcoming VVC standard.

# References

- “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, Doc. JVET-H1002, Macao, Oct. 2017

- G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6 VCEG 13th meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33 (available at http://wftp3.itu.int/av-arch/video-site/0104_Aus/).

- G. Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6 VCEG 35th meeting, Berlin, Germany, 16–18 July, 2008, Doc. VCEG-AI11 (available at http://wftp3.itu.int/av-arch/video-site/0807_Ber/).

- International Telecommunication Union – Radiocommunication Sector, Methodology for the subjective assessment of the quality of television pictures, Recommendation ITU-R BT.500-13, January 2012 (available at https://www.itu.int/rec/R-REC-BT.500).

- International Telecommunication Union – Telecommunication Sector, Subjective video quality assessment methods for multimedia applications, Recommendation ITU-T P.910, April 2008 (available at https://www.itu.int/rec/T-REC-P.910).

- “Algorithm Description of Joint Exploration Test Model 7 (JEM7)”, Joint Video Exploration Team (JVET) of ITU-T VCEG (Q6/16) and ISO/IEC MPEG (JTC 1/SC 29/WG 11), 7th Meeting, Torino, Italy, July 2017, Doc. JVET-G1001

- “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 4”, Joint Video Exploration Team (JVET) of ITU-T VCEG (Q6/16) and ISO/IEC MPEG (JTC 1/SC 29/WG 11), 7th Meeting, Torino, Italy, July 2017, Doc. JVET-G1003

- J. Boyce, “Dynamic viewports for 360° video CfP subjective testing”, Doc. JVET-J0073, San Diego, Apr. 2018

- M. Zhou, “BoG report on survey of proposed technology in SDR category”, Doc. JVET-J0082, San Diego, Apr. 2018

- A. Segall, “BoG report on survey of proposed technology in HDR category”, Doc. JVET-J0084, San Diego, Apr. 2018

- J. Boyce, “BoG report on 360° video”, Doc. JVET-J0085, San Diego, Apr. 2018

- “Versatile Video Coding (Draft 1)”, Doc. JVET-J1001, San Diego, Apr. 2018

- “Algorithm description for Versatile Video Coding and Test Model 1 (VTM 1)”, Doc. JVET-J1002, San Diego, Apr. 2018

JVET documents are available at http://phenix.it-sudparis.eu/jvet/

1 kbit/s means 103 bits per second, and 1 Mbit/s means 106 bits per second.

1 kbit/s means 103 bits per second, and 1 Mbit/s means 106 bits per second.

1 kbit/s means 103 bits per second, and 1 Mbit/s means 106 bits per second.

1 kbit/s means 103 bits per second, and 1 Mbit/s means 106 bits per second.