| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J1021-r5 |
| --- | --- |

| Title: | Description of Core Experiment: Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Jackie Ma Fraunhofer HHI Fabrice Le Léannec Technicolor Min Woo Park Samsung Electronics | Email: | jackie.ma@hhi.fraunhofer.de fabrice.leleannec@technicolor.com m.w.park@samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document defines and describes the Core Experiment on Partitioning with the results being presented at the upcoming meeting in Ljubljana in July 2018. The experiment aims to investigate the performance of different partitioners under general test conditions to explore the potential of each partitioning method. The general CE is divided into several subCEs that study the particular sub technologies such as the boundary handling. In total there will be 49 experiments for VTM and 49 for BMS configuration.

# Introduction

During the 10th Meeting in San Diego in April 2018 several partitioning methods have been proposed. A great number of proponents have proposed to use QTBT+TT, however, more general partitioning methods have been proposed during the meeting as well. Among them are generalized binary splits JVET-J0035 and Asymmetric Binary Tree (ABT, JVET-J0075). Although some of the proposed methods share similar elements, there are still differences that have to be studied in more detail. Further non-rectangular partitioning has been proposed in JVET-J0020 and JVET-J0023.

# Participant list

| # | Name | Affiliation | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
|  | Jackie Ma | Fraunhofer HHI | jackie.ma@hhi.fraunhofer.de | X | X |
|  | Adam Wieckowski | Fraunhofer HHI | adam.wieckowski@hhi.fraunhofer.de | X | X |
|  | Tobias Hinz | Fraunhofer HHI | tobias.hinz@hhi.fraunhofer.de |  |  |
|  | Robert Skupin | Fraunhofer HHI | robert.skupin@hhi.fraunhofer.de |  |  |
|  | Fabrice Le Léannec | Technicolor | fabrice.leleannec@technicolor.com | X | X |
|  | Tangi Poirier | Technicolor | tangi.poirier@technicolor.com | X | X |
|  | Franck Galpin | Technicolor | franck.galpin@technicolor.com |  |  |
|  | Edouard François | Technicolor | edouard.francois@technicolor.com |  |  |
|  | Michel Kerdranvat | Technicolor | michel.kerdranvat@technicolor.com |  |  |
|  | Yin Zhao | Huawei | yin.zhao@huawei.com | X | X |
|  | Jianle Chen | Huawei | jianle.chen@huawei.com |  |  |
|  | Haitao Yang | Huawei | haitao.yang@huawei.com |  |  |
|  | Han Gao | Huawei | han.gao@huawei.com | X | X |
|  | Xiang Li | Tencent | xlxiangli@tencent.com | X | X |
|  | Meng Xu | Tencent | mengxxu@tencent.com |  | X |
|  | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |  |  |
|  | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com |  |  |
|  | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com | X | X |
|  | Tzu-Der Chuang | MediaTek | Peter.Chuang@mediatek.com | X |  |
|  | Han Huang | MediaTek | h.huang@mediatek.com | X |  |
|  | Shih-Ta Hsiang | MediaTek | shih-ta.hsiang@mediatek.com | X | X |
|  | Shaw-Min Lei | MediaTek | shawmin.lei@mediatek.com |  |  |
|  | Chia-Ming Tsai | MediaTek | chia-ming.Tsai@mediatek.com | X | X |
|  | Andrew Segall | Sharp | asegall@sharplabs.com |  |  |
|  | Kiran Misra | Sharp | misrak@sharplabs.com | X | X |
|  | Weijia Zhu | Sharp | zhuw@sharplabs.com |  |  |
|  | Hsiao-Chiang Chuang | Qualcomm | hchuang@qti.qualcomm.com | X |  |
|  | Nan Hu | Qualcomm | nanh@qti.qualcomm.com | X |  |
|  | Vadim Seregin | Qualcomm | vseregin@qti.qualcomm.com | X |  |
|  | Wei-Jung Chien | Qualcomm | wchien@qti.qualcomm.com |  |  |
|  | Nikolay Shlyakhov | Qualcomm | nikolays@qti.qualcomm.com |  | X |
|  | Min Woo Park | Samsung Electronics | m.w.park@samsung.com | X | X |
|  | Yinji Pao | Samsung Electronics | yj1003.piao@samsung.com | X | X |
|  | Minsoo Park | Samsung Electronics | mss.park@samsung.com | X | X |
|  | Kiho Choi | Samsung Electronics | kiho14.choi@samsung.com |  |  |
|  | Virginie Drugeon | Panasonic | virginie.drugeon@eu.panasonic.com |  |  |
|  | Han Boon Teo | Panasonic | hanboon.teo@sg.panasonic.com | X | X |
|  | Chong Soon Lim | Panasonic | chongsoon.lim@sg.panasonic.com | X |  |
|  | Junghak Nam | LGE | junghak.nam@lge.com | X | X |
|  | Hyeongmun Jang | LGE | hm.jang@lge.com | X |  |
|  | Seung Hwan Kim | LGE | seunghwan3.kim@lge.com |  |  |
|  | Jaehyun Lim | LGE | jaehyun.lim@lge.com |  |  |
|  | Jung Won Kang | ETRI | jungwon@etri.re.kr |  |  |
|  | Sang-hyo Park | Yonsei University | spark@barunict.kr |  | X |
|  | Zhao Wang | Peking University | zhaowang@pku.edu.cn |  |  |
|  | Chris Rosewarne | Canon | chris.rosewarne@cisra.canon.com.au |  | X |
|  | Andrew Dorrell | Canon | andrew.dorrell@cisra.canon.com.au |  | X |
|  | Patrice Onno | Canon | patrice.onno@crf.canon.fr |  |  |
|  | Yan Ye | InterDigital | yan.ye@interdigital.com |  |  |
|  | Yuwen He | InterDigital | yuwen.he@Interdigital.com |  |  |
|  | Xiaoyu Xiu | InterDigital | xiaoyu.xiu@interdigital.com |  |  |
|  | Rahul Vanam | InterDigital | rahul.vanam@interdigital.com |  |  |
|  | Jaesob Shin | Pixtree | jsshin@pixtree.com |  |  |
|  | Yu-Chen Sun | Alibaba Group | yuchen.s@alibaba-inc.com |  |  |
|  | Kenji Kondo | Sony | kenji.kondo@sony.com | X |  |
|  | Jean-Marc Thiesse | Vitec | jean-marc.thiesse@vitec.com |  |  |
|  | Didier Nicholson | Vitec | didier.nicholson@vitec.com |  |  |
|  | Yue Li | USTC | lytt@mail.ustc.edu.cn | X |  |
|  | Dong Liu | USTC | dongeliu@ustc.edu.cn |  |  |
|  | Ankur Saxena | Nvidia | saxena.ankur0@gmail.com |  |  |
|  | Dan Grois | Comcast | groisz@yahoo.com |  |  |
|  | Jianqing Zhu | Fujitsu | zhujianqing@cn.fujitsu.com |  |  |
|  | Kimihiko Kazui | Fujitsu | kazui.kimihiko@jp.fujitsu.com |  |  |
|  | Seungwook Hong | Arris | seungwook.hong@arris.com |  |  |
|  | Yue Yu | Arris | yue.yu@arris.com |  |  |
|  | Limin Wang | Arris | limin.wang@arris.com |  |  |
|  | Quanhe Yu | HiSilicon | yuquanhe@hisilicon.com |  | X |
|  | Kei Kawamura | KDDI | ki-kawamura@kddi.com |  |  |
|  | Xiaozhen Zheng | DJI | xiaozhen.zheng@dji.com |  |  |
|  | Byeungwoo Jeon | SKKU | bjeon@skku.edu |  | X |
|  | Yongjo Ahn | Digital Insights Inc. | yjahn@digitalinsights.co.kr |  |  |
|  | Benben Niu | Tsinghua University | nbb16@mails.tsinghua.edu.cn |  |  |
|  | Minhua Zhou | Broadcom | minhua.zhou@broadcom.com |  |  |
|  | Wade Wan | Broadcom | wade.wan@broadcom.com |  |  |
|  | Peisong Chen | Broadcom | peisong.chen@broadcom.com |  |  |
|  | Yao-Jen Chang | Foxconn | yaojen.chang.foxconn@gmail.com |  |  |
|  | John Son | WILUS Inc. | john.son@wilusgroup.com |  |  |
|  | Xiaozhen Zheng | DJI | xiaozhen.zheng@dji.com |  | X |
|  | Amir Ahmad Mohammadi | Sharif University of Technology | amirahmad.mohamadi@ee.sharif.edu |  |  |
|  | Li Zhang | Bytedance Inc. | lizhang.idm@bytedance.com |  |  |

# SubCE1: Partitioning structure

The studied partitioning methods are block-based partitioning methods that go beyond QTBT and have been presented in a CfP during the Meeting. These are summarized in Table I.

Table I Block partitioning methods proposed during the Cfp meeting

| Doc | Brief description on Partitioning structure |  |
| --- | --- | --- |
|  | Allowed partitioning types | Coding tree structure |
| JVET-J0017 JVET-J0018 JVET-J0020 JVET-J0029 JVET-J0075 (configurable) JVET-J0021 (configurable) | QT, BT, TT (similar to JVET-D0117) | - Recursive QT from CTU to QT leaf; - Recursive BT or TT from QT leaf to whole-tree leaf - Separate trees for Luma and Chroma in I Slice (JVET-J0018) |
| JEVT-J0024/ JVET-J0025 | QT, BT, TT | - One forced QT split at CTU - Recursive BT or TT from QT leaf to whole-tree leaf - Define allowed CU ratio and CU size |
| JVET-J0047 | QT, BT, TT + SplitToSquare | - Recursive QT (or four squares in one line), BT or TT from CTU to whole-tree leaf - Allow SplitToSquare even after BT or TT split |
| JVET-J0021 JVET-J0026 JVET-J0027 | QT, BT, TT, ABT | - Recursive QT from CTU to QT leaf; - Recursive BT, TT or ABT from QT leaf to whole-tree leaf - Modified context modeling for BT/TT/ABT decision tree |
| JVET-J0022 | QT, BT, ABT | - Recursive QT from CTU to QT leaf; - Recursive BT or ABT from QT leaf to whole-tree leaf, allowing 1:1, 1:3, 1:2 BT partitioning |
| JVET-J0035 | QT, BTS | - Recursive QT from CTU to QT leaf; - Recursive BT from QT leaf to whole-tree leaf, allowing 1:1, 1:3, 3:5, 1:2, 1:4, 2:3 BT partitioning - Separate trees for Luma and Chroma in I Slice |
| JVET-J0014 | GBS | - Recursive BT from CTU leaf to whole-tree leaf, allowing 1:1, 1:3, 3:5, 1:2, 1:4, 2:3 BT partitioning - Separate trees for Luma and Chroma in I Slice |

## Partitioning methods

### QT+BTS (Quadtree plus binary tree with shifting)

Quadtree plus binary tree with shifting (QT+BTS) is proposed in JVET-J0035. QT+BTS extends the QTBT split modes by:

- 1/4 and 3/4: if the split side is 2n, n∈N and at least 16 in luma pixels,

- 3/8 and 5/8: if the split side is 2n, n∈N and at least  32 in luma pixels.

Figure 1: Possible splits for power of 2 sizes. First row: QTBT splits. Second row: 1:3 splits with different orientations. Third row: 3:5 splits with different orientations.

If the size of the side that is to be split is not 2n, n∈N, then it is either 3·2n, n∈N or 5·2n, n∈N. It is proposed to split these non-power of 2 sizes by either

- 1/2: if the side to be split is at least 8 in luma pixels,

- 1/3 and 2/3: if the split side is 3·2n,

- 1/5, 2/5, 3/5, 4/5: if the split side is 5·2n.

The split modes are visualized in Figure 1 and 2. The availability of splits is constrained by several high-level parameters and split optimizations such as the prohibition of redundant splitting patterns.

Figure 2: Possible shifted binary splits for non-power of 2 sizes. First row: 1:2 splits with different orientations. Second and third row: 1:4 and 2:3 splits with different orientations.

### QTBT+TT (Triple Tree)

Beyond QTBT framework, horizontal and vertical center-side triple-trees are introduced, as shown in Figure 3(d) and (e).

Figure 3. (a) quad-tree partitioning (b) vertical binary-tree partitioning (c) horizontal binary-tree partitioning (d) vertical center-side triple-tree partitioning (e) horizontal center-side triple-tree partitioning

### Binary and Triple Tree (BTT) in J0024

A coding tree unit (CTU) is the basic unit of the proposed coding scheme. A CTU is automatically partitioned by using quad tree structure, thus the maximum coding unit (CU) size is always less than the CTU size. After the quad tree split, the maximum CU can be further partitioned recursively by binary and triple tree (BTT) structure. For instance, if the CTU size is 256x256 then the CTU is split into four 128x128 CUs and each 128x128 CU becomes the maximum CU size and can be split further. By using proposed CU partitioning rule with BTT structure, both square and non-square shapes for the CU are possible. As per the decision that CTU size is 128x128, QT split on CTU can be skipped.

In this proposal, CU shapes are described by the ratio between width and height of a block. For instance, if width and height of a block are the same it can be represented as 1:1 ratio CU or a square CU, and if width is equal to 64 and height is equal to 16 it can be represented as 1:4 ratio CU. CU partitioning is conducted based on the allowed CU shapes and their allowed maximum and minimum sizes.

Figure 4: BTT split modes

Depending on the shape of the current CU, allowed split modes can be varied by sequence level signaling. In this proposal, for each shape of the CU the minimum and the maximum sizes are defined in the sequence header as below. (All the sizes are in log2 scale and minus2 implies original size in log2 scale minus two)

- log2_cu_11_ratio_max_minus2: the maximum allowed size of 1:1 ratio CU

- log2_cu_11_ratio_min_minus2: the minimum allowed size of 1:1 ratio CU

- log2_cu_12_ratio_max_minus2: the maximum allowed size of 1:2 or 2:1 ratio CU (for longer side)

- log2_cu_12_ratio_min_minus2: the minimum allowed size of 1:2 or 2:1 ratio CU (for longer side)

- log2_cu_14_ratio_max_minus2: the maximum allowed size of 1:4 or 4:1 ratio CU (for longer side)

- log2_cu_14_ratio_min_minus2: the minimum allowed size of 1:4 or 4:1 ratio CU (for longer side)

In addition to this, triple split modes can be configured by signaling the minimum and the maximum allowed CU size as below.

- log2_tri_split_max_ minus2: the maximum allowed size of tri-split (for longer side)

- log2_tri_split_min_ minus2: the minimum allowed size of tri-split (for longer side)

For example, if CU size is 64x64 and log2_cu_12_ratio_max_minus2 is larger than 4 (real size is 64), bi-vertical and bi-horizontal split modes can be applied. And if log2_tri_split_max_minus2 is larger than 4 (real size is 64) but log2_cu_14_ratio_max_minus2 is smaller than 4 (real size is 64), tri-splits are not allowed. Because tri-sub CU of the CU has shape of 64x16 or 16x64 CU but these shape are not allowed by log2_cu_14_ratio_max_minus2. This is shown in the following figure.

Figure 5: Example of allowed split modes

### ABT (Asymmetric Binary Tree)

Four new Binary Tree splitting modes were introduced into the QTBT framework, so as to allow new splitting configurations. So-called Asymmetric Binary Tree split modes (ABT) are employed in addition to the splitting modes already available in QTBT, as shown by Figure 3.

According to the added splitting modes, a coding unit with size S is divided into 2 sub-CU with sizes S/4 and 3.S/4, either in the horizontal or in the vertical direction. In practice the added available CU sizes are 6, 12, 24 and 48.

Overall, a CTU can be divided into square or rectangle CUs, according to various split options, as illustrated in Figure 4: quadtree (QT), symmetric binary tree (BT), and a new type of split, asymmetric binary tree (ABT).

Figure 4: Split modes (tree types) supported in the QTBT+ABT framework

A CU resulting from an asymmetric splitting may be further split in a recursive way, as shown on Figure 5 This includes binary symmetric split, asymmetric binary split along the direction which size is a power of two, and optionally asymmetric binary split in the orientation where the CU dimension is a multiple of 3. In the latest case, the divided CU horizontal or vertical sizes are respectively equal to s/3 and 2*s/3, where s is the parent CU horizontal or vertical size.

Figure 5: Spatial splitting optionally adaptive to the parent CU Size

### QTBT+TT+SplitToSquare (JVET-J0049)

A new tree type STS (SplitToSquare) is proposed in JVET-J0049. With this tree type, a block can be split into largest same-size square sub-blocks. When implemented on top of the QTBT+TT structure, STS can be applied when the block width to height ratio is 1:1, 1:4, or 4:1. For example, a 64x16 block can be split into 4 CUs with the size of 16x16.

## Signaling

#### QT+BTS signaling (JVET-J0035)

A shifted binary split is uniquely determined by its split direction and its split ratio. The direction of the binary splits are coded depending on the previous split, i.e. instead of being signaled as horizontal or vertical split, they are signaled as perpendicular split or parallel split which, however, can always be translated to a horizontal or vertical split. The first split has to be set always by convention. There are two binary flags perpend_split_flag and parallel_split_flag for the two possible directions (perpendicular and parallel). The split ratio then describes the location where to split binarily, see Figure 1 and Figure 2. It is coded using a binary decision tree. The full signaling tree for the quadtree plus binary splits with shifting (QT+BTS) without any restrictions is shown in Figure 6.

Figure 6: Full signaling tree, the perpendicular part is not displayed as it has the same sub tree as the parallel case. Branches or leafs of tree can be omitted, if the options are not available in a specific context. If a node only contains one child, the decision is not signaled.

#### ABT signaling (JVET-J0022)

In the QTBT+ABT framework up to 2 bins are added to the QTBT signaling a CU’s split mode, as illustrated on Figure 7. The first bin indicates if ABT is used, in case a CU is partitioned in non-quad-tree mode. If ABT is used for the considered coding tree node, then the second added bin indicates the asymmetric split type: 0 for left/top (vertical split), 1 for right/bottom (horizontal split). The two added bins are arithmetically with one single context.

Figure 7: ABT signaling

#### QTBT+TT+ABT (JVET-J0026)

|  | Binarization; Partitioning. 1; QT. 0 0; No Split. 0 1 0 0; Vertical TT. 0 1 0 1 0; Vertical BT 1/2. 0 1 0 1 10; Vertical BT 1/4. 0 1 0 1 11; Vertical BT 3/4. 0 1 1 0; Horizontal TT. 0 1 1 1 0; Horizontal BT 1/2. 0 1 1 1 10; Horizontal BT 1/4. 0 1 1 1 11; Horizontal BT 3/4. |
| --- | --- |

Figure 8: QT, BT, TT signaling

The binarization of the split and no-split indicators is shown in Figure 8.  As can be seen in the Figure, the second bin of the binarization signals if the current block will be further split. If the block will be split, the third bin indicates the splitting direction (horizontal or vertical). The fourth bin indicates whether a TT partition is selected. If not, the last one or two bins are used to represent the split offset for binary split.

#### BTT signaling in J0024

Split configuration parameters in the sequence header are defined as below.

- log2_cu_11_ratio_max_minus2: the maximum allowed size of 1:1 ratio CU.

- log2_cu_11_ratio_min_minus2: the minimum allowed size of 1:1 ratio CU.

- log2_cu_12_ratio_max_minus2: the maximum allowed size of 1:2 or 2:1 ratio CU (for longer side).

- log2_cu_12_ratio_min_minus2: the minimum allowed size of 1:2 or 2:1 ratio CU (for longer side).

- log2_cu_14_ratio_max_minus2: the maximum allowed size of 1:4 or 4:1 ratio CU (for longer side).

- log2_cu_14_ratio_min_minus2: the minimum allowed size of 1:4 or 4:1 ratio CU (for longer side).

- log2_tri_split_max_ minus2: the maximum allowed size of tri-split (for longer side).

- log2_tri_split_min_ minus2: the minimum allowed size of tri-split (for longer side).

Split modes in this proposal are defined as below.

- NO_SPLIT: Use current CU

- QUAD_SPLIT: Split current CU into four sub-CU as both vertical and horizontal direction in 1:1 ratio

- BI_VER_SPLIT: Split current CU into two sub-CU as vertical direction in 1:1 ratio

- BI_HOR_SPLIT: Split current CU into two sub-CU as horizontal direction in 1:1 ratio

- TRI_VER_SPLIT: Split current CU into three sub-CU as vertical direction in 1:2:1 ratio

- TRI_HOR_SPLIT: Split current CU into three sub-CU as horizontal direction in 1:2:1 ratio

Split mode for the current CU is derived as follows:

- Input: split configuration, split information of neighboring CUs, width and height of CU

- Output: split mode

- Process

  - If both width and height of CU are equal to CTU size

    - split mode is set to QUAD_SPLIT

  - Otherwise, if both width and height of CU are equal to the minimum CU size

    - split mode is set to NO_SPLIT

  - Otherwise, split mode is derived as follows (depending on the number of allowed split mode):

    - Parse one bit if there is at least one allowed split mode; otherwise, the bit is set to be zero

    - If the bit is equal to zero, split mode is set to NO_SPLIT

    - Otherwise, check a number of allowed split modes

      - If the number of allowed split modes is equal to four (Figure (a))

        - Parse two bits and select split mode

      - If the number of allowed split modes is equal to three (Figure (b))

        - Parse one bit.

        - Check the bit and if parsing one more bit is needed

        - If one more bit is needed

          - Parse one bit

          - Select split mode

        - Otherwise

          - Select split mode

      - If the number of allowed split modes is equal to two (Figure (c))

        - Parse one bit

        - Select split mode

      - If the number of allowed split modes is equal to one (Figure (d))

        - Select split mode

Allowed split modes for a CU are defined by split configuration as described in section 3.1.2. In other words, bits assignment of each split mode is conducted based on split configuration. All the possible combinations of split modes in this proposal are illustrated in the following Figure.

Figure 6: Bits assignment for split mode based on split configuration (i.e. allowed split modes)

#### QTBT+TT (JVET-J0017)

In the JVET-J0017, signaling of QT structure is identical with HEVC. Firstly, a syntax can be signalled to indicate whether a block is partitioned or not at QT leaf. If the block is not split, the second bin is not presented. When the block is split, another syntax is signaled to indicate the split type between BT or TT.

| Binarization | Block partitioning type |
| --- | --- |
| 0 | NO_SPLIT |
| 10 | BT |
| 11 | TT |

After determining either BT or TT, a syntax is further signaled to indicate split direction.

| Binarization | Direction |
| --- | --- |
| 0 | Horizontal |
| 1 | Vertical |

#### QTBT+TT+SplitToSquare (JVET-J0047)

In JVET-J0047, the binarization table of tree signaling is as follows. A particular bin (such as split direction) may be skipped if related tree types are not available.

| Binarization | Tree Type |
| --- | --- |
| 0 | no_split |
| 11 | split_to_square |
| 1000 | splti_bt_hor |
| 1001 | split_tt_hor |
| 1010 | split_bt_ver |
| 1011 | split_tt_hor |

### Non-power of 2 adaptations

#### QT+BTS (JVET-J0035)

The following topics are modified due to the occurrence of non-power of 2 block sizes (further described in JVET-J0014):

- Inclusion of further core transforms to support non-power of 2 block sizes

- Indication of last non-zero coefficient

- Quantization

#### QT+ABT (JVET-J0022)

The following topics changes are made due to the occurrence of non-power of 2 block sizes (further described in JVET-J0022):

- Additional core transforms to support non-power of 2 block sizes

- Changes in indication of last non-zero coefficient,

- Changes in scaling factor to support multiplication and bit shift only transform/quantization,

- Adaptation of the sampling step for LMChroma and LIC in BMS mode.

### Context modeling

Modified context modeling for BT/TT/ABT decision tree. (JVET-J0021).

Modified context modeling for BT/TT decision tree. (JVET-J0017).

Modified context modeling for BT/TT decision tree. (JVET-J0024).

Modified context modeling for signaling the decision tree. (JVET-J0018).

Modified context modeling for QT+BTS decision tree. (JVET-J0035).

## Tests to be conducted

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1.0.1 | F. Le Léannec (Technicolor) | - Configuration = QTBT+ABT - Allow CU sizes 12, 24 and 48 (luma) - Allow transform sizes 12, 24 and 48 (luma) - Disallow split mode ratio (1/3,2/3) | JVET-J0022 (CfP) JVET-J0075 | Y. Zhao (Huawei) A. Wieckowski (Fraunhofer HHI) |
| 1.0.2 | F. Le Léannec (Technicolor) | - Configuration = QTBT+ABT - Allow CU sizes 12 and 24 (luma) - Allow transform sizes 12 and 24 (luma) - Disallow split mode ratio (1/3,2/3) | JVET-J0022 (CfP) JVET-J0075 | S. H. Park (Yonsei University) |
| 1.0.3 | F. Le Léannec (Technicolor) | - Same test as 2, where split mode ratio (1/3,2/3) is allowed |  | S. H. Park (Yonsei University) |
| 1.0.4 | F. Le Léannec (Technicolor) | - Configuration = QTBT+ABT - Allow CU sizes 12, 24 and 48 (luma) - Disallow transform size not equal to a power of 2 - Code/Decode CU with size 12,24,48 either with null residual, or by dividing the CU into 2 transform blocks | JVET-J0022 (CfP) JVET-J0075 | S. H. Park (Yonsei University) |
| 1.0.5 | J. Ma (Fraunhofer HHI) | - Configuration = QT+BTS - Use quad split with binary 1:1, 1:2, 1:3 splits | JVET-J0035 | J. Nam (LGE) |
| 1.0.6 | J. Ma (Fraunhofer HHI) | - Configuration = QT+BTS - Use quad split with binary 1:1, 1:2, 1:3 splits - Force final power of 2 partitioning | JVET-J0035 | F. Le Léannec (Technicolor) |
| 1.0.7 | J. Ma (Fraunhofer HHI) | - Configuration = QT+BTS - Use quad split with binary 1:1, 1:2, 1:3, 3:5, 1:4, 2:3 splits | JVET-J0035 | Y. Zhao (Huawei) |
| 1.0.8 | J. Ma (Fraunhofer HHI) | - Configuration = QT+BTS - Use quad split with binary 1:1, 1:2, 1:3, 3:5, 1:4, 2:3 splits - Force final power of 2 partitioning | JVET-J0035 | M. W. Park (Samsung) |
| 1.0.9 | N. Hu (Qualcomm) | - Configuration = QT + BT + TT | JVET-J0021 | M. W. Park (Samsung) A. Wieckowski (Fraunhofer HHI) |
| 1.0.10 | K. Misra (Sharp) | - Configuration = QT+BT+TT+ABT | JVET-J0026 | J. Ma (Fraunhofer HHI) |
| 1.0.11 | K. Misra (Sharp) | - Configuration = QT+BT+TT (without ABT) - Remove bins corresponding to BT offset | JVET-J0026 | M. W. Park (Samsung) |
| 1.0.12 | M. W. Park (Samsung) | - Configuration = BTT (Use QT on the top) - Max. BT depth: 8 - Max. TT depth: 6 - Allowed CU ratio: 1:1, 1:2, 1:4 - Max CU size for 1:4 and TT: 64 | JVET-J0024 (CfP) | A. Dorrell (Canon) K. Rosewarne (Canon) |
| 1.0.13 | M. W. Park (Samsung) | - Configuration = BTT (Not use QT on the top) - Max. BT depth: 10 - Max. TT depth: 8 - Allowed CU ratio: 1:1, 1:2, 1:4 - Max CU size for 1:4 and TT: 64 | JVET-J0024 (CfP) | A. Dorrell (Canon) K. Rosewarne (Canon) |
| 1.0.14 | M. Park (Samsung) | - Configuration = QT+BT+TT (VTM) - Allowed CU ratio: 1:1, 1:2, 1:4 | JVET-J0024 (CfP) | Q. Yu (HiSilicon) |
| 1.0.15 | M. Park (Samsung) | - Configuration = QT+BT+TT (VTM) - Allowed CU ratio: 1:1, 1:2, 1:4 - Max CU size for 1:4 and TT: 64 | JVET-J0024 (CfP) | J. Ma (Fraunhofer HHI) |
| 1.0.16 | J. Nam (LGE) | - Configuration = QT + BT + TT | JVET-J0017 | J. Ma (Fraunhofer HHI) |
| 1.0.17 | Y. Zhao (Huawei) | - Configuration = QTBT+TT - context modeling | JVET-J0024 | J. Ma (Fraunhofer HHI) |
| 1.0.18 | X. Li (Tencent) | - Configuration = QTBT+TT+SplitToSquare | JVET-J0047 | J. Ma (Fraunhofer HHI) |
| 1.0.19 | S.-T. Hsiang (MediaTek) | - Configuration = QTBT+TT - context modeling | JVET-J0018 | Q. Yu (HiSilicon) |

# SubCE2: Picture boundary handling

## JVET-J0035: QT+BTS on edge

In QT+BTS, if only two corners of a current block extent beyond coded picture boundaries, the block can be additionally split using a binary split with ratio ½ and split direction parallel to the picture boundary the block extents over. The encoder can choose, based on RD-cost, if the QT or implicit BTS split is preferred. The decision has to be signaled using a single bin (see Figure 9). If the current block is a part of a QT+BTS rather than the QT tree, the QT will not be available and the split will be implicitly inferred as the BTS split with the ratio 1/2 parallel to the boundary edge the block extents over. If the current block extents over a coded picture boundary, QT+BTS-tree restriction like maximal tree depth are ignored.

Figure 9: Reduced split signaling for block within a QT tree, for which two of its corners lie outside of the coded picture boundaries.

If a different number of corners of a current block extent over the coded picture boundaries, a QT split is implicitly inferred and no further signaling is required.

## JVET-J0021/JVET-J0022: rate distortion optimized choice of the series of split modes that lead to block boundaries matching the picture border (bottom or right)

In JVET-J0021/JVET-J0022, the CU splitting rules on the picture bottom and right borders may apply to any of the coding tree configuration QTBT+TT, QTBT+ABT or QTBT+TT+ABT. They include the two following aspects:

- If a part of a given Coding Tree node (CU) is partially located outside the picture, then the binary symmetric splitting of the CU is always allowed, along the concerned border direction (horizontal split orientation along bottom border, vertical split orientation along right border). If the bottom-right corner of the current CU is outside the frame, then only the quad-tree splitting of the CU is allowed. In addition, if the current binary tree depth is greater than the maximum binary tree depth and current CU is on the frame border, then the binary split is enabled to ensure the frame border is reached.

- With respect to the triple tree splitting process, the triple tree split is allowed in case the first or the second boundary between resulting sub-CU exactly lies on the border of the picture. The asymmetric binary tree splitting is allowed if a splitting line (boundary between two sub-CU resulting from the split) exactly matches the picture border.

A typical picture block partitioning resulting from this new block partitioning policy on the picture borders is illustrated on Figure 10.

Figure 10: Block partitioning on the picture border, in JVET-J0022.

## QT or BT split on boundary CUs (JVET-J0018)

When a CU corresponds to a region across a current frame boundary, it is forced to be further split into smaller-size child-CUs using QT split or BT split. QT split is inferred when the current CU is across both the right picture boundary and the bottom picture boundary or the current CU size is greater than the maximum allowed CU size for BT split. BT split is inferred when the BT/TT depth of the current CU is greater than zero or the QT depth of the current CU is not less than the maximum allowed QT depth. When BT split is selected, horizontal split is inferred if the current CU is across the bottom picture boundary and vertical split is inferred if the current CU is across the right picture boundary.

## Picture boundary handling as in JVET-J0026

The CTUs spanning across picture boundary undergo implicit partitioning until all the samples at the leaf of the partitioning tree created using implicit splits do not span across the picture boundary. The implicit splits can include both quad tree and binary tree splits.  The quad-tree splitting can be further controlled by receiving a minimum number of required quad-tree splits of the CTU.  After the minimum number of quad tree splits have been performed, the decoder performs a series of implicit binary splits according to the following process.  First, for both the horizontal and vertical directions, the binary tree split that results in the split edge being closest to the picture boundary is selected. Then, between the two split direction candidates, the direction that does not partition the block of samples inside the picture boundary is chosen – or if both directions partition the block of samples inside the picture boundaries, the horizontal partition is simply chosen. This process is performed recursively. For boundary CTU’s depth restrictions are adjusted. Extensions to ternary tree would be tested as well.

## Picture boundary handling as in JVET-J0032

A block partitioning method for frame boundary nodes is presented in JVET-J0032. For non-boundary CTU, the maximum BT/TT depth is set as uiMaxBTD = 3 as JVET CTC. For CUs which are located in boundary CTU, uiMaxBTD is set according to the following equation:

uiMaxBTD = max(3, ForceSplitLevel – ForceQTLevel + Extension)

Where the ForceSplitLevel is needed split level to ensure the boundary CU is fully located within the frame boundary, the ForceQTLevel is the needed QT split level before forced BT/TT, the Extension controls BT/TT partition freedom.

For instance, in HD sequences, 4 level (56 = 32+16+8, 8=128>>4 and the forced level is 4) forced splitting (QT/BT) are needed until the boundary CU is fully located within the frame boundary. When the ForceQTLevel and Extension are 0 and 2, the uiMaxBTD is 6 (4 – 0 + 2), as shown in Fig. 11, the yellow CU has used 2 levels BT split, the maximal number of possible BT/TT levels is 4; the green CU has used 3 levels BT//TT split, 3 levels BT/TT partition is possible; the blue CU has used 4 levels BT/TT split, 2 levels BT/TT is possible.

Figure 7: Example of ForceQTLevel is 0 and Extension is 2

## Picture boundary handling as in JVET-J0024/JVET-J0072

A block partitioning method for frame boundaries is used in JVET-J0024 and JVET-J0025, and is disclosed in the software in JVET-J0072. This partitioning scheme uses QT and BT partitioning for handling frame boundary, which includes the following rules:

- QT or horizontal BT for bottom boundary, QT or vertical BT for right boundary, for bottom-right corner, only QT is used

- First N level of QT is performed, the level N is adaptive selected

## Picture boundary handling as in JVET-J0047

The signaling cost of tree types is reduced by avoiding particular bins under certain conditions. First, the bin indicating block split is inferred as true and not signaled when the current block is partially outside picture boundary. Second, the bin indicating vertical or horizontal split is inferred and not signaled if the current block is outside the current picture either vertically or horizontally but not both. The split bin is inferred as along the picture boundary, i.e., it is inferred as vertical split if the block is across vertical boundary, or is inferred as horizontal split if the block is across horizontal boundary.

## Tests to be conducted

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 2.0.1 | T.Poirier (Technicolor) | - Configuration = QTBT+TT - Allow binary/triple split if a CU partially lies outside the picture. - RDO between successions of split modes leading to block boundaries matching the picture border - Allow only quad-tree split if the bottom right corner of the block lies outside the picture - Allow triple tree in case the first or the second boundary between resulting sub-CU exactly matches the picture border. | JVET-J0021 (CfP) JVET-J0075 | H. Gao (Huawei) |
| 2.0.2 | T.Poirier (Technicolor) | - Configuration = QTBT+ABT - Allow binary/triple split if a CU partially lies outside the picture. - RDO between successions of split modes leading to block boundaries matching the picture border - Allow only quad-tree split if the bottom right corner of the block lies outside the picture Allow asymmetric binary tree in case the first or the second boundary between resulting sub-CU exactly matches the picture border. | JVET-C0022 (CfP) JVET-J0075 | Y. Zhao (Huawei) |
| 2.0.3 | J. Ma (Fraunhofer HHI) | - Configuration = QTBT - Allow QT and BT at picture boundaries depending on the block positioning outside the coding area - with boundary handling | JVET-J0035 | H. Gao (Huawei) |
| 2.0.4 | J. Ma (Fraunhofer HHI) | - Configuration = QTBT - Allow QT and BT at picture boundaries depending on the block positioning outside the coding area - without boundary handling | JVET-J0035 | H. Gao (Huawei) |
| 2.0.5 | J. Ma (Fraunhofer HHI) | - Configuration = QTBT+TT - Allow QT and BT at picture boundaries depending on the block positioning outside the coding area - with boundary handling | JVET-J0035 | Y. Zhao (Huawei) |
| 2.0.6 | J. Ma (Fraunhofer HHI) | - Configuration = QTBT+TT - Allow QT and BT at picture boundaries depending on the block positioning outside the coding area - without boundary handling | JVET-J0035 | H. Gao (Huawei) |
| 2.0.7 | J. Ma (Fraunhofer HHI) | - Configuration = QT+BTS - Allow QT and BT at picture boundaries depending on the block positioning outside the coding area - with boundary handling | JVET-J0035 | H. Gao (Huawei) |
| 2.0.8 | J. Ma (Fraunhofer HHI) | - Configuration = QT+BTS - Allow QT and BT at picture boundaries depending on the block positioning outside the coding area - without boundary handling | JVET-J0035 | H. Gao (Huawei) |
| 2.0.9 | Y. Li (USTC) | - Configuration = QTBT+TT - Allow binary/quad split if a CU partially lies outside the picture. - Use QT first to meet the MaxBTTSize restriction - With MaxBTTDepth being extended for nodes in boundary CTU | JVET-J0032 | X. Zheng (DJI) |
| 2.0.10 | S.-T. Hsiang (MediaTek) | - Configuration = QTBT+TT - Allow BT/QT split if a CU partially lies outside the picture. | JVET-J0018 | M. W. Park (Samsung) |
| 2.0.11 | H. Gao (Huawei) | - Configuration = QTBT+TT - Allow binary/quad split if a CU partially lies outside the picture. | JVET-J0024 | A. Wieckowski (Fraunhofer HHI) |
| 2.0.12 | M. W. Park (Samsung) | - Configuration = QTBT+TT (VTM) - Allow BT/QT if a CU partially lies outside the picture. | JVET-J0024 | K. Misran (Sharp) |
| 2.0.13 | X. Li (Tencent) | - Configuration = QT+BT+TT+STS - infer split if a block is partially outside the current picture - infer split direction if a block is partially outside the current picture | JVET-J0047 | H. Gao (Huawei) X. Zheng (DJI) |
| 2.0.14 | K. Misra (Sharp) | - Configuration = QT+BT+TT Allow BT/QT split if a CU partially lies outside the picture. | JVET-J0026 | F. Le Léannec (Technicolor) |
| 2.0.15 | K. Misra (Sharp) | - Configuration = QT+BT+TT - Allow BT/QT/TT split if a CU partially lies outside the picture. | JVET-J0026 | F. Le Léannec (Technicolor) |

# SubCE3: Split restriction

### Redundancy removal

### JVET-J0022: Non-redundancy between successions of splits

As the splitting process is recursive and given the multiple supported CU split modes, several recursive splitting paths may lead to the same partitioning of a CTU or of a part of a CTU.

Some normative restrictions are introduced to ensure that any topology in the representation of a CTU can be reached through at most one series of quad-tree plus binary-tree/triple tree recursive splitting steps.

These restrictions consist in forbidding some splitting modes for a CU, given the split modes associated to the parent CU (Figure 12) or the neighboring sub-CU already processed in the same parent CU (Figure 13).

- First, no succession of 2 asymmetric binary splits can be used, if it produces a topology than can be obtained with the binary tree (see figure below). The non-desired successions of binary tree are forbidden in a normative way, i.e. all syntax elements that lead to a non-admissible case are omitted from the bitstream, and their value is inferred by the decoder.

Figure 12: Restriction to avoid redundancies between asymmetric and symmetric binary trees.

- Second, another type of normative restriction extends the concept of anti-emulation already present in JEM7, which prevents successive symmetric binary tree splits from emulating the quad-tree. This extension is shown on figure below. The basic principle is to forbid some horizontal Triple Tree or Asymmetric Binary Tree splitting of the second sub-CU of a parent CU, when the parent CU has undergone a vertical TT or binary split. If a triple or binary split mode is assigned to the first sub-CU, then the same split mode cannot be given to the second sub-CU. This prevents from producing topologies that can be reached through a horizontal triple/binary split of the parent CU, followed by two identical vertical splits of its sub-CUs.

Figure 13: Restriction of split modes allowed for a CU, given the split mode of the parent CU and of the neighboring sub-CU already processed in the considered parent CU.

### Redundant CU partition removal in JVET-J0018

Two kinds of redundant CU partitions are removed. The first kind of redundant is that the partitioning structures and CU coding orders are both the same with another partition method. For example, the following partitions are not allowed.

The second kind of redundant is that the partitioning structure is the same with another partition method but the CU coding orders are different. For example, the following partitions are not allowed.

### JVET-J0020: Split restrictions

If current block uses TT partitioning, to avoid redundancy with BT partitioning, the middle block of a TT split block will not perform BT partition of the same splitting direction as TT split.

TT vertical partitioning is invalid when the block width is smaller than the larger value between MinBTTTSize and 16, or when the block height is greater than twice the block width. When TT vertical partitioning is invalid, TT_HOR (TT horizontal partitioning) is automatically chosen. TT horizontal partitioning is invalid when the block height is smaller than the larger value between MinBTTTSize and 16 or when the block width is greater than twice the block height. When TT horizontal partitioning is invalid, TT_VER (TT vertical partitioning) is automatically chosen.

TT partitioning is invalid when both TT horizontal partitioning and TT vertical partitioning are invalid, or when the block width and height are larger than MaxBTTTSize. When TT partitioning is invalid, BT partitioning is automatically chosen. Please note that both BT and TT partitioning increase the depth of the BT/TT common tree, so that there is only one parameter MaxBTTTDepth to limit the depth of the two partitioning methods. When the common depth for BT/TT partitioning reaches MaxBTTTDepth, both BT and TT partitioning are invalid, which stops the partitioning process. MinBTTTSize equals to MinBTSize, MaxBTTTDepth equals to MaxBTDepth, and MaxBTTTSize equals to MaxBTSize. MinBTSize, MaxBTDepth and MaxBTSize have the same values as in JEM7.0.

### JVET-J0017: Redundant CU partition removal

In J0017, there are four split constraints for partitioning by QTBT+TT. Split constraint is applied to prevent partitioning of quad-tree shape using the binary tree as shown in (a) and (b). In addition, split constraint is applied to prevent redundant partitioning of stripe style using the binary tree as shown in (c) and (d).

(a)                    (b)                     (c)                    (d)

### Redundant CU partition removal in JVET-J0024/J0072

A restriction on BT split is used to remove one type of partitioning redundancy introduced by recursive BT and TT, as disclosed in the software in J0072. In Fig. 14, the left-side split combination (i.e., VBT+VBT+VBT) is not allowed, to prevent from generating the same partitioning pattern by the right-side split combination. Similarly, split combination of HBT+HBT+HBT is not allowed.

In addition, the allowed split mode of the parent node is checked to test whether the partitioning redundancy really exists. For example, if one node (i.e., partitioned by split 1 in the figure) is allowed to use VBT split but not allowed to use VTT, the partitioning pattern of four nodes in Fig. X can only be generated from the left-side split combination. If partition redundancy does not exist, the split combination of VBT+VBT+VBT is not restricted.

Fig. 14 Two ways to generate a same partitioning pattern by using BT and TT splits

### Tests to be conducted

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 3.0.1 | T.Poirier (Technicolor) | - Configuration = QTBT+TT - Normative prevention of emulating BT splits with TT then BT splits. - Restriction to reach a quaternary block division through at most one succession of BT/TT splits | JVET-J0021/JVET-J0022 (CfP) JVET-J0075 | K. Misran (Sharp) |
| 3.0.2 | T.Poirier (Technicolor) | - Configuration = QTBT+ABT - Normative prevention of emulating BT splits with ABT - Restriction to reach a ternary block division through at most one series of splits - Restriction to reach a quaternary block division through at most one succession of BT/ABT splits | JVET-C0022 (CfP) JVET-J0075 | Q. Yu (HiSilicon) |
| 3.0.3 | H. B. Teo (Panasonic) | - Configuration = QTBT+TT - Normative prevention of emulating BT splits with TT. - Restriction to reach very narrow block for TT. | JVET-J0020 | C.-Y. Chen (MediaTek) |
| 3.0.4 | J. Nam (LGE) | - Configuration = QTBT+TT - Normative prevention of emulating QT splits with BT (ex: a, b) - Normative prevention of redundant partitioning with BT (ex: c, d) | JVET-J0017 (CfP) | Q. Yu (HiSilicon) |
| 3.0.5 | H. Huang (MediaTek) | - Configuration = QTBT+TT - Normative prevention of redundant partitions in JVET-J0018 | JVET-J0018 | Y. Zhao (Huawei) |
| 3.0.6 | Y. Zhao (Huawei) | - Configuration = QTBT+TT - Normative prevention of redundant partitioning with BT | JVET-J0024 JVET-J0072 | J. Nam (LGE) X. Li, M. Xu (Tencent) |
| 3.0.7 | V. Seregin (Qualcomm) | - VTM - Encoder only prevention of redundant split partitioning |  | J. Ma (Fraunhofer HHI) |

### Configuration of the segmentation tree

To configure the partition tree to be of different complexity, some split restriction is proposed in JVET-J0024 and the software of JVET-J0072.

- If the aspect ratio of a node is 1:4 (or higher), the node cannot be partitioned by some split modes which result in child nodes of aspect ratio higher than 1:4, which is called split restriction on node aspect ratio.

- All child nodes from a TT split cannot further split, i.e., TT split used only at leaf nodes.

The maximum TT split depth can be restricted or controlled separately, as proposed in JVET-J0018:

- The TT partition is allowed for splitting the current node when the current BTTDepth is smaller than MaxBTTDepth minus a threshold (> 0). The threshold is indicated by a SPS syntax. Besides, the syntax for selecting the TT partition type is absent when the current BTTDepth is equal to or larger than MaxBTTDepth minus a threshold.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 3.0.8 | Y. Zhao (Huawei) | - Configuration = QTBT+TT - Split restriction on node aspect ratio 1:4 (with MaxBTTDepth = 3) | JVET-J0024 (CfP) | C.-M. Tsai (MediaTek) |
| 3.0.9 | Y. Zhao (Huawei) | - Configuration = QTBT+TT - TT split used at leaf nodes | JVET-J0072 | C.-M. Tsai (MediaTek) |
| 3.0.10 | C.-M. Tsai (MediaTek) | - Configuration = QTBT + TT - MaxBTTDepth = 3 - Maximum TT split depth = MaxBTTDepth – threshold (threshold = 2 for this test) - Set a SPS syntax to signal the threshold | JVET-J0018 (CfP) | Y. Zhao (Huawei) |

# SubCE4: Split unit coding order

Split Unit Coding Order (SUCO) in J0024 enables more flexible coding order, such as left to right (L2R) and right to left (R2L), to allow intra prediction from right reference pixels and inter prediction with right motion vector predictors. If a SU is partitioned vertically (vertical splitting), a flag is signaled to indicate L2R or R2L coding order of partition units. If a SU is partitioned by quad tree structure, a flag is shared for above two units and bottom two units. If no flag is signaled for coding order of an SU, the coding order follows its parent’s SU coding order. Due to more flexible coding order in SU level, the neighboring availability of a leaf CU become more diverse than common left and above neighbors in HEVC. There are four different availability cases if only left and right neighboring blocks are considered, i.e., LR_10, LR_01, LR_11 and LR_00. Above block is always available unless the current CU locates on the top boundary of a slice. Availability of the above left or above right corner block depends on the corresponding left or right neighbor availability. Intra prediction and most probable mode list is modified accordingly based on the left and right availability. Derivation for motion vector predictor in inter prediction is also modified accordingly based on the left and right availability.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 4.0.1 | Y. Piao (Samsung) | - VTM + SUCO | JVET-J0024 (CfP) | F. Le Léannec (Technicolor) |
| 4.0.2 | Y. Piao (Samsung) | - BMS + SUCO | JVET-J0024 (CfP) | N. Shlyakhov (Qualcomm) |

# SubCE5: Separate Trees for Luma and Chroma

## SubCE5.1 Separate tree partitioning in JVET-J0018

Chroma can have a separate tree partitioning structure in I-slice, with its own configurations of maximum size for BT/TT partitioning, maximum depth of BT/TT partitioning, minimum QT size, et al. The test including the additional coding tools related to separate tree partitioning such as the Multi-DM mode will be further conducted.

A chroma CU size constraint is further applied and tested. The chroma CU size is constrained not to be smaller than the corresponding luma CU size and smaller than the neighboring chroma CU size.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 5.1.1 | S.-T. Hsiang (MediaTek) | - Configuration = QTBT+TT Allow separate trees for luma and chroma in I slice. | JVET-J0018 | H. B. Teo (Panasonic) |
| 5.1.2 | S.-T. Hsiang (MediaTek) | - Configuration = QTBT+TT - Allow separate trees for luma and chroma in I slice - Multi-DM - TU64 for chroma - Chroma size constraint | JVET-J0018 | K. Kondo (Sony) |

## SubCE5.2 Separate Trees for Luma and Chroma

A separate tree implementation for I slices is available. Separate depths can be assigned for subtrees. As the dual tree has influence on other tools it is tested in a SubCE with regard to the BMS.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 5.2.1 | J. Ma (Fraunhofer HHI) | - Configuration = QT+BTS - Separate tree for Luma and Chroma for I Slices - BMS tools on | JVET-J0035 | Q. Yu (HiSilicon) |
| 5.2.2 | J. Ma (Fraunhofer HHI) | - Configuration = QTBT+TT - Separate tree for Luma and Chroma for I Slices - BMS tools on | JVET-J0035 | B. Jeon (SKKU) |
| 5.2.3 | F. Le Léannec (Technicolor) | - Configuration = QTBT+ABT - Allow separate trees for luma and chroma in I slice. | JVET-J0022 | Q. Yu (HiSilicon) |
| 5.2.4. | V. Seregin (Qualcomm) | - Configuration = QTBT+TT - Allow separate trees for luma and chroma in I slice | JVET-J0021 | Q. Yu (HiSilicon) |
| 5.2.5. | V. Seregin (Qualcomm) | - Configuration = QTBT+TT - Allow separate trees for luma and chroma in I slice - Multi-DM | JVET-J0021 | K. Misran (Sharp) |

## SubCE5.3 CU-level shared-separate partitioning trees in JVET-J0026

This test evaluates adaptively switching between a shared or separate tree structure for intra and inter slices. The switching is signaled on a coding unit basis, and it is only available for intra coded regions. The selection is received by first receiving the partition information for the CTU, and then, receiving the prediction type for each resulting coding unit.  In the case that the prediction type is intra, a flag is received indicating if the intra region in the coding unit will be further refined with a separate or shared tree structure. The test includes inferring shared or separate tree flag value based on block size.

Note: In this test, compared to SubCE5.3 below, CU-level shared or separate tree decision is made for both inter and intra slice types

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 5.3.1 | K. Misra (Sharp) | - Configuration = QT+BT | JVET-J0026 | J. Kang (ETRI) |
| 5.3.2 | K. Misra (Sharp) | - Configuration = QT+BT+TT | JVET-J0026 | N. Shlyakhov (Qualcomm) |

## SubCE 5.4 Combining shared-separate partitioning trees in JVET-J0026 and JVET-J0018

This test evaluates adaptively switching between a shared or separate tree structure for intra and inter slices. The switching is signaled on a coding unit basis, and it is only available for intra coded regions. CU-level selection is enabled by first receiving the partition information for the CTU, and then, receiving the prediction type for each resulting coding unit.  In the case that the prediction type is intra, a flag is received indicating if the intra region in the coding unit will be further refined with a separate or shared tree structure. The test includes inferring shared or separate tree flag value based on block size. Separate luma and chroma trees are always used for intra-slices.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 5.4.1 | K. Misra (Sharp) | - Configuration = QT+BT | JVET-J0026 | J. Kang (ETRI) |
| 5.4.2 | K. Misra (Sharp) | - Configuration = QT+BT+TT | JVET-J0026 | N. Shlyakhov (Qualcomm) |

# SubCE6: Large CTU

## SubCE6.1 Processing CUs of width or height greater than the max transform size in JVET-J0018.

In Test 1, the CU with width or height greater than the maximum transform size 64 are inferred to be further partitioned by quadtree split (without explicitly signaling).

In Test 2, the CBF values are inferred to be equal to 0 for the CUs with width or height greater than 64 in B or P slice. In non-low-delay inter slices, for CUs with luma width or height larger than 64, skip mode is inferred and always applied. In low-delay inter slices (all reference frames are prior to the current frame in display order), for CUs with luma width or height larger than 128, skip mode and advanced motion vector prediction (AMVP) mode with all-zero residuals are supported, where the coded block flag (CBF) of the AMVP mode is not signaled and inferred as 0.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 6.1.1 | S.-T. Hsiang (MediaTek) | - Configuration = QTBT+TT - Set CTU size larger than maximum transform size - Inferred QT split if CU size larger than maximum transform size | JVET-J0018 | M. W. Park (Samsung) |
| 6.1.2 | S.-T. Hsiang (MediaTek) | - Configuration = QTBT+TT - Set CTU size larger than maximum transform size - Infer that a CU is coded with zero residual or in skip mode if the CU width or height is larger than maximum transform size | JVET-J0018 | M. W. Park (Samsung) |

## SubCE6.2 Processing to support large CUs in JVET-0028.

In this test, if the width or height size of the CU is larger than 64 for an inter prediction mode, the residual coding is implicitly disabled to avoid usage of large transforms beyond 64. The skip mode is used or the CBF values are inferred to be equal to 0.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 6.2.1 | K. Kondo (Sony) | - Configuration = QTBT+TT - Set CTU size larger than maximum transform size - Infer that a CU is coded with zero residual or in skip mode and an inter prediction mode if the CU width or height is larger than maximum transform size | JVET-J0028 | S.-T. Hsiang (MediaTek) |

# SubCE7: Partitioning configuration parameters (e.g., adaptive on frame basis)

In JVET-J0018, the maximum CU size for BT/TT partitioning (MaxCTSize) is signaled in the slice header. The encoder can automatically choose the value based on block partitioning statistics of previously coded slices.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 7.0.1 | S.-T. Hsiang (MediaTek) | - Configuration = QTBT+TT - Encoder adaptively chooses the maximum CU size for BT/TT split | JVET-J0018 | X. Li (Tencent) |

# SubCE8: Encoder Optimization

## JVET-J0022: ABT operating points (Random Access)

This section describes the encoder configuration that will be used in the ABT-related tests of Section 3.3. (tests 1 to 4).

The rate distortion search for the block partitioning with ABT uses the following fast methods.

- For a given CU, the first evaluated split mode is NO_SPLIT. Then symmetric BT splitting is evaluated, then ABT splitting is evaluated. The ABT splitting is not evaluated if the BT splitting already evaluated in the same orientation is not competitive enough compared to the NO_SPLIT split mode. This takes the following form.

| - If( RDCost(HOR_SPLIT) > α×RDCost(NO_SPLIT) ) | then disable asymmetric horizontal |
| --- | --- |
| - If( RDCost(VER_SPLIT) > α×RDCost(NO_SPLIT) ) | then disable asymmetric vertical |
| - If( RDCost(HOR_SPLIT) > β×RDCost(VER_SPLIT) ) | then disable asymmetric horizontal |
| - If( RDCost(VER_SPLIT) > β×RDCost(HOR_SPLIT) ) | then disable asymmetric vertical |

In practice the values α and β are equal to 1.1.

- Some caching mechanisms re-use coding parameters already determined when visiting a block in the past, having the same shape and position as current block, as illustrated in Figure 15.

Figure 15: Re-using already optimized coding parameter for same block (shape and position)

If the current block has already been processed before, with exactly the same causal environment in terms of surround reconstructed pixels and motion vectors, then the already found split mode is chosen without any further RD search.

- The caching mechanism is also used to limit the number of tested binary tree split modes, when a block with same position and size as current block has already been processed in the past. In that case, only the M best split modes found in the past are considered as candidate split modes for current block. Thus, the caching process stores the evaluated split modes for each block position and size, together with the RD cost associated to each evaluated split mode.

Coding tree configuration

The used coding tree configuration varies according to the slice type and the temporal layer. Some exemplary maximum QT and BT depths allowed for each picture of a random-access sequence are shown on table below. The maximum allowed QT depth varies according to the slice type, temporal level and component. Moreover, the maximum allowed BT depth varies according to the slice type, slice temporal layer and QT depth level.

Table 1: coding tree configuration used ABT tests 1 to 4 (section 3.3)

According to another “mid-complexity” trade-off between encoding time and coding efficiency, the coding tree configuration of the following table is employed.

Table 2: mid-complexity coding tree configuration

According to a last “low-complexity” trade-off, the encoder configuration will be set as follows.

- The ABT rate distortion search is further accelerated compared to Section 3.3.: the speed-up factors α and β are equal to 1.05.

- The ABT tool is deactivated in B slices with a temporal layer strictly higher than a certain value.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 8.0.1 | F. Le Léannec (Technicolor) | - Configuration = QTBT+ABT - Test 1 of section 3.3 with mid-complexity encoder configuration above | JVET-J0022 (CfP) | J. Ma (Fraunhofer HHI) |
| 8.0.2 | F. Le Léannec (Technicolor) | - Configuration = QTBT+ABT - Test 1 of section 3.3 with low-complexity encoder configuration above | JVET-J0022 (CfP) | J. Ma (Fraunhofer HHI) |

# Core experiment tasks

The tasks for the Core Experiment are as follows:

- Implement the partitioning into the code basis

- Evaluate bit rate savings in luma and chroma for different resolution classes

- Collect runtime data for encoder and decoder side for different resolution classes

- Report the results and conclusions of these experiments to CE coordinators

# Test conditions, configurations and anchor

The CE results are to be produced under the common test conditions. Further, the following two configurations are to be used for all CE experiments:

- Configuration 1:
VVC software with Benchmark set (BMS) configuration as provided in the repository.

- Configuration 2:
Same software as in Configuration 1 but with disabling the BMS tools at config level.

No additional tool should be added to these two configurations.

Two anchors are to be generated using the BMS 1.0 software for Configuration 1 and Configuration 2, respectively. Config files for both configurations are provided and labeled by “xxx_bms1.cfg” and “xxx_vtm1.cfg”.

Parallelization has to be switched off for anchors and all final results.

# Interdependencies with other CEs

- SubCE4 is expected to have dependencies with CE3 and CE4.

# Timeline

- May 11: Finalization of the CE document

- June 1: Finished implementation of the technologies into the code basis. Initial study by cross-checkers can begin. Proponents may continue to modify the software in this branch until June 19.

- June 19: Report results to CE coordinators

- July 2: Upload document for the next meeting

# Reporting of results

Results should be reported using the common spreadsheets as it is done in previous contributions. These spreadsheets contain four-point BD-rate and BD-PSNR metrics to evaluate the coding efficiency. Additionally, encoder and decoder run times for all tests shall be provided.

Further, proponents should provide the following fundamental information in case they differ from the VTM 1.0:

- Minimum and maximum CTU size

- Minimum and maximum TU size

- Minimum and maximum PU size

- Additional transform types and sizes

Further changes/extensions from the VTM should be highlighted, for instance, entropy coding, quantization, etc.

Proponents are encouraged to provide further analysis of data, such as memory usage.