| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, USA, 10–20 Apr. 2018 | Document: JVET-J1023_r2 |
| --- | --- |

| Title: | Description of Core Experiment 3: Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Jin Heo Alexey Filippov | Email: | geertv@qti.qualcomm.com jin78.heo@lge.com Alexey.Filippov@huawei.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

The goal of this Core Experiment (CE3) is to study intra prediction tools including mode coding. There are 7 sub-tests defined. In CE3.1 tests are included targeting DC, planar, directional, and additional modes. CE3.2 tests reference sample filtering, interpolation, across boundary filtering and prediction sample filtering. CE3.3 targets intra mode coding, such as most probable mode list variations. In CE3.4 cross-component linear model and variations are tested. CE3.5 focuses on multi-reference line intra prediction tests and CE3.6 on neural network technologies for intra prediction. In CE3.7 tests are performed regarding bidirectional intra prediction.

The following is the list of defined sub-tests in CE3:

- CE3.1: Intra modes

- CE3.2: Intra filtering

- CE3.3: Intra mode coding

- CE3.4: Cross-component linear model (CCLM)

- CE3.5: Multi-reference line intra prediction

- CE3.6: Neural network technologies for intra

- CE3.7: Bidirectional intra prediction

# Participants

The following table contains a list of participants with tools in the core experiment as well as participating cross-checkers.

| Name | Company | Email | CE3.x Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Geert Van der Auwera Vadim Seregin Nan Hu Ted Hsieh Adarsh Krishnan Ramasubramonian | Qualcomm | geertv@qti.qualcomm.com vseregin@qti.qualcomm.com nanh@qti.qualcomm.com thsieh@qti.qualcomm.com aramasub@qti.qualcomm.com | 1, 2, 3, 4 | X |
| Xin Zhao Liang Zhao | Tencent | xinzzhao@tencent.com leolzhao@tencent.com | 5 | X |
| Fabien Racapé Gagan Rath Fabrice Urban Franck Galpin | Technicolor | Fabien.Racape@technicolor.com Gagan.rath@technicolor.com Fabrice.urban@technicolor.com Franck.galpin@technicolor.com | 5, 7 | X |
| Jani Lainema | Nokia | jani.lainema@nokia.com | 1 | X |
| Virginie Drugeon | Panasonic | Virginie.Drugeon@eu.panasonic.com | 1, 2 | X |
| Jaehyun Lim SeungHawn Kim Jin Heo Jangwon Choi Sunmi Yoo Ling Li | LGE | jaehyun.lim@lge.com seunghwan3.kim@lge.com jin78.heo@lge.com jangwon84.choi@lge.com sunmi.yoo@lge.com l.li@lge.com | 2, 3, 4, 7 | X |
| Jungwon Kang Jinho Lee | ETRI | jungwon@etri.re.kr jinosoul@etri.re.kr | 2, 5 | X |
| Jonathan Pfaff Heiko Schwarz Benjamin Bross Philipp Merkle Santiago de Luxán Hernández Philipp Helle | HHI | jonathan.pfaff@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de benjamin.bross@hhi.fraunhofer.de philipp.merkle@hhi.fraunhofer.de santiago.de.luxan@hhi.fraunhofer.de philipp.helle@hhi.fraunhofer.de | 1, 2, 5, 6 | X |
| Krit Panusopone Yue Yu Seungwook Hong Limin Wang | ARRIS | Krit.Panusopone@arris.com Yue.Yu@arris.com Seungwook.Hong@arris.com Limin.Wang@arris.com | 1 | X |
| Yao-Jen Chang Teresa Jiang | Foxconn | morriss.yj.chang@foxconn.com teresa.hy.jiang@foxconn.com | 4 | X |
| Alexey Filippov Sergey Ikonin Jianle Chen Anand Meher Kotra Xiang Ma Vasily Rufitskiy Timofey Solovyev Maxim Sychev | Huawei | Alexey.Filippov@huawei.com Sergey.Ikonin@huawei.com jianle.chen@huawei.com anand.meher.kotra@huawei.com maxiang6@huawei.com Vasily.Rufitskiy@huawei.com solovyev.timofey@huawei.com Sychev.Maxim@huawei.com | 2, 3, 4, 7 | X |
| Kei Kawamura Yoshitaka Kidani Sei Naito | KDDI | ki-kawamura@kddi.com yo-kidani@kddi.com se-naitou@kddi.com | 4 | X |
| Peter Chuang, ChingYeh Chen, Yu-Wen Huang, Chih-Wei Hsu, Chia-Ming Tsai, Zhi-Yi Lin, Mohammed Sarwer | MediaTek | peter.chuang@MediaTek.com chingyeh.chen@MediaTek.com yuwen.huang@MediaTek.com cw.hsu@MediaTek.com Chia-ming.Tsai@MediaTek.com Zhi-Yi.Lin@MediaTek.com Mohammed.Sarwer@MediaTek.com | 2, 4 | X |
| Yinji Piao Narae Choi Kiho Choi Jie Chen | Samsung | yj1003.piao@samsung.com narae1.choi@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com | 2, 3 | X |
| Ching-Chieh Lin Po-Han Lin | ITRI | jacklin@itri.com pohan@itri.com | 5 | X |
| Steve Keating | Sony | Steve.Keating@sony.com | 5 | X |
| Shimpei Nemoto | NHK | nemoto.s-fy@nhk.or.jp | 2 | X |
| Andre Seixas Dias | BBC | Andre.SeixasDias@bbc.co.uk |  | X |
| Andrew Segall Kiran Misra Yukinobu Yasugi Takeshi Chujoh Tomohiro Ikai | Sharp | asegall@sharplabs.com misrak@sharplabs.com yasugi.yukinobu@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp | 1 | X |
| Zhi Zhang Rickard Sjoberg | Ericsson | zhi.a.zhang@ericsson.com rickard.sjoberg@ericsson.com |  | X |
| Yongbing Lin Quanhe Yu Jianhua Zheng | HiSilicon | linyongbing@hisilicon.com yuquanhe@hisilicon.com zhengjianhua@hisilicon.com |  | X |
| Dongcheol Kim | WILUS | dongcheol.kim@wilusgroup.com |  | X |
| Xingchun Qi Xuguang Zuo Shurui Ye Liying Xu | Hikvision | qixingchun@hikvision.com zuoxuguang@hikvision.com yeshurui@hikvision.com xuliying5@hikvision.com |  | X |
| D. Y. Kim | Chips& Media | brad.kim@chipsnmedia.com |  | X |

# Test Conditions

CTC conditions as agreed at the San Diego meeting for SDR will be used for the evaluation of objective performance (JVET-J1010). The first anchor is the ‘VVC Test Model’ or VTM. The second anchor is the ‘Benchmark Model Set’ or BMS. Both the ‘All Intra’ and ‘Random Access’ test configurations shall be used and providing the results for each of these configurations is mandatory. The BMS version 1.0 software shall be used with ‘bms’ configuration files to produce the BMS anchor, while ‘vtm’ configuration files shall be used to produce the VTM anchor. Tools are to be implemented into the BMS version 1.0 software.

Proposals will primarily be compared with respect to objective compression efficiency while complexity will also be assessed.

BD-rates, encoding and decoding time comparisons, shall be reported by all proponents for at least the following two cases:

- #2 vs. #1 (VTM case)

- #4 vs. #5 (BMS case)

Additional test results may be provided, for example: #4 vs. #3, #2 vs. #6.

Numbering is defined as follows for the case where ‘ToolA’ is replacing an existing ‘ToolB’ in BMS:

1.           VTM,			VTM (as a reference)

2.           VTM + ToolA,		VTM + ToolA (as a “tool on” test)

3.           BMS - ToolB,		BMS with similar ToolB turned off (as a “tool off” case)

4.           BMS - ToolB + ToolA,	BMS with similar ToolB replaced with ToolA under test

5.           BMS                         	BMS (as a reference)

6.           VTM + ToolB            	VTM with similar ToolB

Note: If ‘ToolB’ does not exist, then #3 = #5, and #6 = #1.

# Timeline

| T1 | May 11, 2018 | CE document finalized |
| --- | --- | --- |
| T2 | June 1, 2018 | Source codes to be released by proponents and begin cross-checking |
| T3 | June 19, 2018 | Cross-checking completed and report results to CE group |
| T4 | July 2, 2018 | Finalize and upload CE report |

# CE3.1: Intra modes

Modified or newly proposed intra modes are studied. Compression efficiency and complexity aspects are investigated. The following table lists the defined tests:

| Test# | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.1.1 | Use 129 directional modes for all blocks | Geert Van der Auwera (Qualcomm) | Steve Keating (Sony) |
| 1.1.2 | Use variable number of directional modes (33, 65, or 129) depending on block size | Geert Van der Auwera (Qualcomm) | Yukinobu Yasugi (Sharp) |
| 1.2.1 | DC mode with only shift operators | Virginie Drugeon (Panasonic) | Zhi Zhang (Ericsson) |
| 1.3.1 | Wide-angle prediction | Jani Lainema (Nokia) | Geert Van der Auwera (Qualcomm), Andre Dias (BBC), Alexey Filippov (Huawei) |
| 1.4.1 | Usage of line-based intra prediction mode | Santiago de Luxán Hernández (HHI) | Andre Dias (BBC) |
| 1.4.2 | Fast Line-based intra prediction mode | Santiago de Luxán Hernández (HHI) | Andre Dias (BBC), D. Y. Kim (Chips&Media) |
| 1.4.3 | Constrained Line-based intra prediction mode | Santiago de Luxán Hernández (HHI) | Andre Dias (BBC), D. Y. Kim (Chips&Media) |
| 1.5.1 | Unequal weighted planar prediction (UWP) | Krit Panusopone (ARRIS), Andrew Segall (Sharp) | Geert Van der Auwera (Qualcomm) |

## Tool 1: Variable directional intra modes (JVET-J0021 Qualcomm/Technicolor)

To capture the arbitrary edge directions presented in natural video, the number of directional intra prediction modes is increased. In this experiment, 33, 65 and 129 directional intra modes are tested. When 33 or 65 angular modes are used, they are respectively identical to the modes defined in HEVC and JEM. Included in this experiment is the case where the number of modes increases with the block size and are equal to 33, 65 or 129 depending on threshold values. These denser directional intra prediction modes apply for both luma and chroma intra predictions. The planar and DC modes remain the same as in HEVC. The directional mode increase from 33 to 65 is depicted as red dotted arrows in Figure 1.

Figure 1. 67 intra modes, including 65 directional modes + planar + DC

### Bitstream organization

For variable directional intra modes, there are two block size threshold values, which are signaled in the SPS as log2 values minus 4 and minus 6, respectively. The first threshold indicates the largest block area with 33 angular directions, and the second threshold indicates the largest block area with 65 angular directions, all other blocks use 129 angular directions.

| Location | Description |
| --- | --- |
| Sequence Header | A threshold indicates the largest block area with 33 angular modes. |
| Sequence header | A threshold indicates the largest block area with 65 angular modes. |

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.1.1 | Use 129 directional modes for all blocks | Geert Van der Auwera (Qualcomm) | Steve Keating (Sony) |
| 1.1.2 | Use variable number of directional modes (33, 65, or 129) depending on block size | Geert Van der Auwera (Qualcomm) | Yukinobu Yasugi (Sharp) |

## Tool 2: DC mode without division (JVET-J0020 Panasonic)

JVET-J0020: Two separate DC values for the reference samples above the block DCup and the reference samples left of the block DCleft are calculated and combined together by calculating (DCup + DCleft) >> 1, which enables using a shift operator for all the DC values calculation even when the sum of block width and height is not equal to a power of two. This is applied to both luma and chroma DC prediction.

### Bit stream organization

N/A.

### Test scenario

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.2.1 | DC mode with only shift operators (related CE3.2 Tool 5) | Virginie Drugeon (Panasonic) | Zhi Zhang (Ericsson) |

## Tool 3: Wide-angle intra prediction (JVET-J0012 Nokia/Ericsson)

This test is set up to verify performance of the wide-angle intra prediction tool proposed in JVET-J0012. In this test intra prediction from angles extending beyond the traditional 45-degree top-right and bottom-left directions is supported. In order to keep signaling overhead and computational complexity low, these angles can only be used for non-square blocks and following angle groups:

- Angles going beyond 45 degrees in top-right direction if block width is larger than block height

- Angles going beyond 45 degrees in bottom-left direction if block height is larger than block width

Signaling for the tool is implemented by sending a 1-bin context coded identifier for those directions that have an available wide-angle alternative. In the case of the 65-direction angular intra prediction, the availability of the wide-angle modes is limited to the 10 directional modes closest to the 45-degree diagonal top-right and bottom-left modes. In the case of 33 angular directions, wide-angle modes are limited similarly to 5 directions closest to the diagonal ones.

The sample prediction process follows the one in reference software, but the effective prediction direction for the wide-angle modes is calculated by inverting the signaled prediction direction (using calculations identical to what is being done when projecting the side reference in the case of negative prediction angles) and switching the reference arrays - using top instead of left or left instead of top references.

Further details:

- Reference samples outside of the existing reference sample arrays are padded

- Context based arithmetic coding of the flag uses one context

- Side smoothing is enabled for wide-angle modes as in JVET-J0012 software

- If intra slices have their own chroma coding tree, chroma blocks have their own wide-angle mode signaling. In the case of inter slices (and if there is common coding tree for all components) chroma uses wide-angle mode of the corresponding luma block.

Expected results: around 0.5% coding efficiency improvement on the test model for AI configuration. Around 2% encoder runtime increase and no impact on decoder runtimes.

### Bit stream organization

Signaling for the tool is implemented by sending a 1-bin context coded identifier for those directions that have an available wide-angle alternative.

### Test Scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.3.1 | Wide-angle prediction | Jani Lainema (Nokia) | Geert Van der Auwera (Qualcomm), Andre Dias (BBC), Alexey Filippov (Huawei) |

## Tool 4: Line-based intra coding mode (JVET-J0014 HHI)

The luma block of a coding unit is divided into 1d horizontal or vertical stripes. The chroma blocks are coded using conventional spatial intra prediction.

Figure 2.

If the line-based intra coding mode is used, the  luma coding block will comprise  one-dimensional prediction and transform blocks, where  will be defined as the width or the height of the luma coding block, according to the type of split used.

Like any other 2d block, each of the 1d partitions will be predicted and the corresponding residual signal will be transformed, quantized and coded individually in a sequential manner. Therefore, the reconstructed values of each line will be available for generating the prediction signal for the next line. This process is continued until all 1d partitions of the luma signal are coded. Hence, each line will be predicted using neighboring samples that are located at the shortest possible distance.

The line-based intra coding mode is used in combination with any of the conventional spatial intra prediction modes. To avoid an excessive overhead in signaling, all 1d partitions within a CU must use the same spatial intra prediction mode. Therefore, the intra mode signalization is left unchanged (a single luma intra prediction mode is coded per coding unit). The prediction samples of each line are calculated in the same way as for the first row/column of samples in the 2d case. An exception is the Planar mode, whose equation has been slightly modified and it is described next:

Let  be the prediction signal of a line, where the domains of  and  are defined according to the type of split and  points to the top-left neighboring sample of the line. Then, for a horizontal split type the planar prediction equation is specify by

where  is the width of the CU and the horizontal and vertical components  and  are defined as

The vertical split equation for the planar intra mode is obtained by swapping  with  and the vertical component with the horizontal one. Besides,  needs to be replaced with , the height of the CU. Finally, if the line-based intra coding mode is used for a CU, then no reference samples or prediction signal filters will be used on the block.

Based on the intra mode and the split utilized, the two different classes of processing orders are used, which are referred to as normal and reversed order.

In normal order, the first line to be processed is the one containing the top-left sample of the CU and then continuing line-wise downwards (horizontal split) or rightwards (vertical split). As a result, reference samples used to generate the 1d prediction signals are only located at the left and above sides of the lines.

On the other hand, the reverse processing order either starts with the line containing the bottom-left sample of the CU and continues upwards or starts with line containing the top-right sample of the CU and continues leftwards.

The figure below shows an example of both processing orders for three different intra modes, namely the diagonal (DIAG, 18 in HEVC and 34 in JEM), the horizontal-diagonal (HDIAG, 2 in HEVC and JEM) and the vertical-diagonal (VDIAG, 34 in HEVC and 66 in JEM) modes. The DIAG mode applies the normal processing order for both the horizontal and vertical splits. On the other hand, the HDIAG mode uses the reverse order when the split is horizontal, so that it can utilize reference samples located at the left and below sides of each line. In the vertical split case, however, it uses the normal order. Analogously, VDIAG employs the normal order when the split is horizontal and the reverse one when it is vertical, so that samples located at the top and right sides of each line can be used to generate the 1d prediction signals. The table below shows the complete list of processing orders according to the intra mode and the split type.

Figure 3.

Processing order according to the intra mode and the split type.

| Intra Mode | Split Type | Processing Order |
| --- | --- | --- |
| [HDIAG, HOR-1] | Horizontal | Reversed |
| [HDIAG, HOR-1] | Vertical | Normal |
| [VER+1, VDIAG] | Horizontal | Normal |
| [VER+1, VDIAG] | Vertical | Reversed |
| All remaining cases | Both | Normal |

### Bitstream organization

For the line-based intra prediction mode, a CU level syntax element indicates the usage of this mode (0: not used, 1: horizontal lines, 2: vertical lines). The syntax element is coded using two bins. The first bin signals whether the line-based intra prediction mode is used. If the first bin is equal to 1, the second bin signals whether the luma signal of the CU is split into horizontal or vertical lines.

| Location | Description |
| --- | --- |
| CU level | Syntax element indicating the usage of the line-based intra prediction mode (0: not used, 1: horizontal lines, 2: vertical lines) |

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.4.1 | Usage of line-based intra prediction mode | Santiago de Luxán Hernández (HHI) | Andre Dias (BBC) |
| 1.4.2 | Fast Line-based intra prediction mode | Santiago de Luxán Hernández (HHI) | Andre Dias (BBC), D. Y. Kim (Chips&Media) |
| 1.4.3 | Constrained Line-based intra prediction mode | Santiago de Luxán Hernández (HHI) | Andre Dias (BBC), D. Y. Kim (Chips&Media) |

## Tool 5: Unequal weight planar prediction (UWP) (JVET-J0026 Sharp/Foxconn; JVET-J0018 MediaTek)

UWP was proposed to JVET as a replacement of Planar mode. Compared to Planar mode, UWP introduces bottom-right pixel adjustment and employs unequal weight assignment for final predictor calculation. Specifically, UWP first computes the intensity value of the bottom right neighboring pixel, as described in equation (1), and then computes the intensity values of the bottom row and the right column neighboring pixels, using the derived intensity value of the bottom right neighboring pixel along with the intensity values of other corner neighboring pixels, i.e., the top right neighboring pixel, the bottom left neighboring pixel, as described in equations (2) and (3).

(1)

(2)

(3)

where R(x,y) denotes the intensity value of the reconstructed neighboring pixel at (x,y) coordinate, W is block width, and H is block height.

Once the neighboring pixels surrounding a current CU are defined, the horizontal and the vertical predictors ( and , respectively) for each pixel within the CU are determined according to equations (4) and (5) below.

(4)

(5)

Final predictor calculation with unequal weight assignment is described in equation (6).

(6)

### Bit stream organization

N/A.

### Test Scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.5.1 | UWP | Krit Panusopone (ARRIS), Andrew Segall (Sharp) | Geert Van der Auwera (Qualcomm) |

# CE3.2: Intra filtering

Following algorithms are studied: reference sample filtering, interpolation, across boundary filtering, intra prediction block filtering. Coding efficiency and complexity aspects are investigated. The following table lists the defined tests:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.1.1 | Bilateral reference sample filter | Geert Van der Auwera (Qualcomm) | Philipp Merkle (HHI) |
| 2.2.1 | 4-tap cubic filter for extending reference samples | Geert Van der Auwera (Qualcomm) | Steve Keating (Sony) |
| 2.2.2 | Bilinear interpolation not including smoothing | Virginie Drugeon (Panasonic) | Philipp Merkle (HHI) |
| 2.3.1 | 6-tap cubic filter prediction from reference samples | Geert Van der Auwera (Qualcomm) | Sunmi Yoo (LGE) |
| 2.3.2 | Combine tests 2.2.1 + 2.3.1 | Geert Van der Auwera (Qualcomm) | Sunmi Yoo (LGE) |
| 2.3.3 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size | Sunmi Yoo (LGE) | Geert Van der Auwera (Qualcomm) |
| 2.3.4 | Interpolation filter selection between 6-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size | Sunmi Yoo (LGE) | Geert Van der Auwera (Qualcomm) |
| 2.4.1 | PDPC planar, DC, HOR/VER, diagonal, adjacent diagonal modes | Geert Van der Auwera (Qualcomm) | Jani Lainema (Nokia) |
| 2.5.1 | Bilinear interpolation for projection and smoothing after projection | Virginie Drugeon (Panasonic) | Geert Van der Auwera (Qualcomm), Philipp Merkle (HHI) |
| 2.6.1 | 6-tap combined filter without reference sample smoothing | Jinho Lee (ETRI) | Virginie Drugeon (Panasonic), Geert Van der Auwera (Qualcomm) |
| 2.7.1 | Bilateral reference sample filter | Philipp Merkle (HHI) | Sergey Ikonin (Huawei) |
| 2.8.2 | Mode dependent de-ringing filter based on bitstream flag | Sergey Ikonin (Huawei) | Jonathan Pfaff (HHI), Geert Van der Auwera (Qualcomm) |
| 2.9.1 | Intra boundary filters | Chih-Wei Hsu (MediaTek) | Xiang Ma (Huawei) |
| 2.10.1 | Multiple 4-tap filter | Narae Choi (Samsung) | Virginie Drugeon (Panasonic) |
| 2.11.1 | Multi-combined intra prediction | Narae Choi (Samsung) | Geert Van der Auwera (Qualcomm) |

## Tool 1: Bilateral filter (JVET-J0021 Qualcomm/Technicolor)

The bilateral filter reduces noise while preserving edges. The experiment evaluates the bilateral filter application to the luma reference samples for intra prediction. The 5-tap bilateral filter taps are given by:

f = {f-2, f-1, f0, f1, f2}/( f-2+f-1+f0+f1+f2), where

with , the unfiltered reference at position , ,   and .

To save computational complexity, values of  and  are stored in look-up tables. The decision whether to use filtering is identical to JEM, except for planar mode which uses the nearest filtered reference line when the block area is larger than 32.

### Bit stream organization

N/A.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.1.1 | Bilateral reference sample filter | Geert Van der Auwera (Qualcomm) | Philipp Merkle (HHI) |

## Tool 2: 4-tap cubic filter for extending top reference samples (JVET-J0021 Qualcomm/Technicolor)

Interpolation filters are used for angular intra prediction in two cases. The first case is the derivation of the extended intra reference samples, and the second case is the derivation of the predicted samples from the references samples.

To derive the extended luma and chroma intra reference samples, the 4-tap cubic interpolation filter is used instead of the nearest neighbor rounding. As shown in the example in Figure 4, to derive the value of reference sample P, the 4-tap cubic interpolation filter is used, while in JEM or HEVC, the P sample is directly set as X1.

Figure 4. Example of interpolation to derive extended intra reference sample

### Bit stream organization

N/A.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.2.1 | 4-tap cubic filter for extending reference samples | Geert Van der Auwera (Qualcomm) | Steve Keating (Sony) |
| 2.2.2 | Bilinear interpolation not including smoothing (see CE3.2 Tool 5) | Virginie Drugeon (Panasonic) | Philipp Merkle (HHI) |

## Tool 3: Interpolation for prediction from reference samples (JVET-J0021 Qualcomm/Technicolor; JVET-J0017 LGE)

JVET-J0021: To generate a prediction sample from luma and chroma reference samples, two interpolation filters can be used. If a block’s width or height is larger than 8 pixels, the 4-tap Gaussian filter from JEM is used; otherwise, a 6-tap cubic interpolation filter is used which is the extension of the 4-tap cubic filter in JEM. The interpolation filter accuracy is equal to 1/64 fractional pixels.

In JVET-J0017, two interpolation filters can be used based on the block size and the intra prediction mode.

- Test 2.3.3: Interpolation filter based on intra prediction mode and block size is tested. Specifically, 4-tap cubic filter and 4-tap Gaussian filter is adaptively selected.

- Test 2.3.4: Interpolation filter based on intra prediction mode and block size is tested. Specifically, 6-tap cubic filter and 4-tap Gaussian filter is adaptively selected.

### Bit stream organization

N/A.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.3.1 | 6-tap cubic filter prediction from reference samples | Geert Van der Auwera (Qualcomm) | Sunmi Yoo (LGE) |
| 2.3.2 | Combine tests 2.2.1 + 2.3.1 | Geert Van der Auwera (Qualcomm) | Sunmi Yoo (LGE) |
| 2.3.3 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size | Sunmi Yoo (LGE) | Geert Van der Auwera (Qualcomm) |
| 2.3.4 | Interpolation filter selection between 6-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size | Sunmi Yoo (LGE) | Geert Van der Auwera (Qualcomm) |

## Tool 4: Position dependent intra prediction combination (JVET-J0021 Qualcomm/Technicolor)

Position dependent intra prediction combination (PDPC) is applied to planar, DC, horizontal mode, vertical mode, bottom-left angular mode and its x adjacent angular modes, and top-right angular mode and its x adjacent angular modes without signaling. The value x depends on the number of angular modes and is equal to 4, 8, or 16, respectively for 33, 65, or 129 angular modes.

### PDPC for planar, DC, HOR/VER modes (JVET-J0021 Qualcomm/Technicolor; JVET-J0069 Qualcomm)

The prediction sample  located at (x, y) is calculated as follows:

where Rx,-1, R-1,y represents the reference samples located at top and left of the current sample (x, y), and R-1,-1 represents the reference sample located in the top-left corner of the current block. The DC mode weights are calculated as follows:

where

For planar mode wTL = 0, while for the horizontal mode wTL = wT and for vertical mode wTL = wL. In Figure 5, DC mode PDPC weights (wL, wT, wTL) for (0, 0) and (1, 0) positions inside one 4x4 block are shown.

Figure 5: DC mode PDPC weights for (0, 0) and (1, 0) positions inside one 4x4 block.

### PDPC for diagonal and adjacent modes (JVET-J0069 Qualcomm)

Figure 6 (a) illustrates the definition of reference samples Rx,-1, R-1,y and R-1,-1 for the extension of PDPC to the top-right diagonal mode. The prediction sample pred(x’, y’) is located at (x’, y’) within the prediction block. The coordinate x of the reference sample Rx,-1 is given by: x = x’ + y’ + 1, and the coordinate y of the reference sample R-1,y is similarly given by: y = x’ + y’ + 1.

The PDPC weights for the top-right diagonal mode are:

wT = 16 >> ( ( y’<<1 ) >> shift ), wL = 16 >> ( ( x’<<1 ) >> shift ), wTL = 0.

| - Diagonal top-right mode | - Diagonal bottom-left mode |
| --- | --- |
| - Adjacent diagonal top-right mode | - Adjacent diagonal bottom-left mode |

Figure 6: Definition of samples used by PDPC extension to diagonal and adjacent angular intra modes.

Similarly, Figure 6 (b) illustrates the definition of reference samples Rx,-1, R-1,y and R-1,-1 for the extension of PDPC to the bottom-left diagonal mode. The coordinate x of the reference sample Rx,-1 is given by: x = x’ + y’ + 1, and the coordinate y of the reference sample R-1,y is: y = x’ + y’ + 1. The PDPC weights for the top-right diagonal mode are: wT = 16 >> ( ( y’<<1 ) >> shift ), wL = 16 >> ( ( x’<<1 ) >> shift ), wTL = 0. The case of an adjacent top-right diagonal mode is illustrated in Figure 6 (c). The PDPC weights for an adjacent top-right diagonal mode are: wT = 32 >> ( ( y’<<1 ) >> shift ), wL = 0, wTL = 0. Similarly, the case of an adjacent bottom-left diagonal mode is illustrated in Figure 6 (d). The PDPC weights for an adjacent bottom-left diagonal mode are: wL = 32 >> ( ( x’<<1 ) >> shift ), wT =0, wTL = 0. The reference sample coordinates for the last two cases are computed using the tables that are already used for angular mode intra prediction. Linear interpolation of the reference samples is used if fractional reference sample coordinates are calculated.

### Bit stream organization

N/A.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.4.1 | PDPC planar, DC, HOR/VER, diagonal, adjacent diagonal modes | Geert Van der Auwera (Qualcomm) | Jani Lainema (Nokia) |

## Tool 5: Modifications to intra prediction filtering (JVET-J0020 Panasonic)

In HEVC, reference samples for angular intra prediction that are located to the left of and above the block are sometimes smoothed with filter [1 2 1]/4. The smoothing filter is applied or not based on the angular intra prediction mode used (MDIS). After this smoothing operation, and depending on the angular prediction mode used, an array of reference samples is prepared that is subsequently used to calculate the prediction signal for the block. For vertical prediction modes, the samples located left of the block are projected in the above row (see Figure 7). For horizontal prediction modes, the samples located above the block are projected in the left column.

Figure 7: Projection of reference samples for a vertical mode

Two modifications are proposed: the projection process of reference samples is improved to be more accurate, and the smoothing filter is applied after projection rather than before. Specifically, projection of reference samples is performed by using bilinear interpolation to calculate the value of a projected sample. This is applied to both luma and chroma intra prediction.

The same smoothing filter as in HEVC is then applied to the projected array of reference samples, thereby smoothing only the samples used for prediction (see Figure 8). Smoothing of the array of reference samples is applied according to the same rules as MDIS in HEVC and applies only to luma intra prediction.

Figure 8: Improved projection and smoothing after projection.

### Bit stream organization

N/A.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.5.1 | Bilinear interpolation for projection and smoothing after projection | Virginie Drugeon (Panasonic) | Geert Van der Auwera (Qualcomm), Philipp Merkle (HHI) |

## Tool 6: Combined filter (JVET-J0013 ETRI/Sejong Univ.)

Two filters which are 3-tap reference sample smoothing filter and 4-tap interpolation filter are combined and represented as 6-tap interpolation filter. So, in the proposed method, only 6-tap interpolation filter is applied to the luma reference samples for angular intra prediction mode without additional filtering process. Like 4-tap interpolation filter defined in JEM7.0, there are two sets of 6-tap interpolation filter. For chroma angular prediction, bi-linear interpolation filter of HEVC is used.

### Bit stream organization

N/A.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.6.1 | 6-tap combined filter without reference sample smoothing | Jinho Lee (ETRI) | Virginie Drugeon (Panasonic), Geert Van der Auwera (Qualcomm) |

## Tool 7: Bilateral filter (JVET-J0014 HHI)

Test the bilateral reference sample filter as proposed in JVET-J0014. For intra reference samples , the Gaussian bilateral filter is defined as:

The bilateral filter is applied to all reference sample positions . For avoiding complex calculations (exponential function) and floating-point arithmetic, filter weights  are pre-calculated as integers and stored in a lookup table.

Figure 9: Reference samples of a W×H intra block and arrangement for bilateral filtering

The filter parameters are:

- for filter size

- for Gaussian range kernel (corresponds to level of smoothing)

- for Gaussian spatial kernel (corresponds to size of smoothing)

In JVET-J0014, the bilateral filter is applied to the reference samples of intra blocks that meet the following conditions (assuming that JEM tools like 65 angular modes and Planar PDPC are available and enabled):

- luma channel

- block size W×H ≥ 16×16

- angular prediction mode  with

The bilateral filter is applied in two configurations depending on the block size, using different values for parameters , , , and number of iterations  (see JVET-J0014).

### Bit stream organization

N/A.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.7.1 | Bilateral reference sample filter JVET-J0014 | Philipp Merkle (HHI) | Sergey Ikonin (Huawei) |

## Tool 8: De-ringing filter (JVET-J0025 Huawei)

The de-ringing filter (paragraphs 2.1.8.4 and 3.1.3.6 in JVET-J0025) reduces ringing artefacts and increases sharpness of edges. The experiment evaluates the appliance of gradient based non-linear de-ringing filter to the reference samples of intra prediction. As first step filter calculates edge pattern of reference samples then performs warping of reference samples based on edge pattern. The filter is applied to both luma and chroma if prediction mode is angular.

### Bit stream organization

1 bin flag with context if prediction mode is angular.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.8.2 | Mode dependent de-ringing filter based on bitstream flag | Sergey Ikonin (Huawei) | Jonathan Pfaff (HHI), Geert Van der Auwera (Qualcomm) |

## Tool 9: Intra boundary filter (JVET-J0018 MediaTek)

In JVET-J0018, intra gradient filter and intra bi-boundary are used to further improve the predictor quality of intra mode. Intra gradient filter is applied on horizontal and vertical prediction mode as the same as that in HEVC. Intra bi-boundary filter is similar to intra boundary filter in JEM as shown in the following figure. Along intra prediction direction, the final predictor is generated by using the reference samples in the other boundary and the original intra predictor.

Figure 10: Examples of boundary prediction filters for intra modes 30–34

In this CE, intra gradient filter and intra bi-boundary filter are added and the filter design including the selected intra modes, the number of sample lines or sample columns, the predictor generation, and the corresponding weights are evaluated.

### Bit stream organization

N/A.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.9.1 | Intra boundary filters | Chih-Wei Hsu (MediaTek) | Xiang Ma (Huawei) |

## Tool 10: Multiple 4-tap filter (JVET-J0024 Samsung)

Four-tap linear prediction is composed with interpolation filter and smoothing filter. Smoothing filter makes a prediction block natural and transform friendly but removes detail information especially near the reference pixels. Filter type is alternatingly chosen with the block size but still same filter is used in the same block. Multiple filters which have different degrees of smoothness in the same block can improve both accuracy and naturalness of intra prediction block. Multiple 4-tap filter is applied to luma block for angular prediction.

### Bit stream organization

N/A.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.10.1 | Multiple 4-tap filter | Narae Choi (Samsung) | Virginie Drugeon (Panasonic) |

## Tool 11: Multi-combined intra prediction (JVET-J0024 Samsung)

MIP modes (paragraph 3.1.3.3 in JVET-J0024) generate more natural patterns by combining filtered and original reference pixels. Filtered reference is calculated with weighted summation of original reference pixels. Reference filtering direction is determined by intra direction mode. This is applied to luma intra prediction.

### Bit stream organization

N/A.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.11.1 | Multi-combined intra prediction | Narae Choi (Samsung) | Geert Van der Auwera (Qualcomm) |

# CE3.3: Intra mode coding

The following experiments investigate the prediction and coding of intra modes:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3.1.1 | Intra 65, 6 modes in primary MPM, secondary MPM, shape dependency | Adarsh Krishnan Ramasubramonian (Qualcomm) | Anand Meher Kotra (Huawei) |
| 3.2.1 | Priority based list with primary MPM, secondary MPM and first few default modes prioritized (method 1 from Samsung): TM intra modes | Narae Choi (Samsung) | Zhi Zhang (Ericsson), Jonathan Pfaff (HHI) |
| 3.2.2 | Priority based list with primary MPM, secondary MPM and first few default modes prioritized (method 2 from Huawei): 65 intra modes | Anand Meher Kotra (Huawei) | Yue Yu (ARRIS), Zhi Zhang (Ericsson), Jonathan Pfaff (HHI) |
| 3.2.3 | 6 MPM + Selected + Non-selected modes list (JEM macro JVET_B0051_NON_MPM_MODE), 65 intra modes | Anand Meher Kotra (Huawei) | Shimpei Nemoto (NHK) |
| 3.3.1 | MPM list construction based on dependency between neighboring intra modes | Ling Li (LGE) | Anand Meher Kotra (Huawei) |

## Tool 1: Two most probable mode lists with shape-dependency (JVET-J0021 Qualcomm/Technicolor)

To signal a luma intra mode, two most probable mode (MPM) lists are derived for a block. The MPM lists include intra modes of the neighboring blocks and modes derived from those intra modes. The process of the MPM list derivation is like JEM with additional modifications proposed in JVET-D0113, JVET-D0114, JVET-D0115, JVET-D0119, and JVET-G0060.

Figure 12. Neighboring blocks for MPMs

Available neighboring intra modes relative to the current block at predefined locations are added to the MPM list, and only unique modes are included. Those predefined five locations are the same as in JEM and depicted in Figure 12. The MPM list size is set to six or seven.

The initial MPM list is derived by adding the intra modes in the following order: left (L), above (A), planar, DC, below-left (BL), above-right (AR), and above-left (AL).

If the initial MPM list is not complete, then the intra modes are derived by adding 1 (2 for 67 modes, 4 for 35 modes) when there are 131 modes, and 1 for 67 modes when there are 67 modes, to the angular most probable modes already included into the MPM list.

Finally, if the MPM list is not complete, the following default modes: VER_IDX, HOR_IDX, 2, DIA_IDX, VDIA_IDX, 10, 26 are added to the list in that order.

At the end of this derivation process, the MPM list of size 6 or 7 containing only unique intra modes is obtained.

As proposed in JVET-D0114, a secondary MPM list is constructed based on the primary MPM list. The size of the secondary MPM list depends on the total number of intra modes used in the current block, and it can be 32, 16, or 8, for 131, 67, 35 total number modes, respectively.

The secondary MPM modes are derived from the angular modes that are already included into the MPM list and are coming from a neighboring block. This process is not applied for the derived or default primary MPM modes. Derivation of the secondary MPM modes is done by adding an offset 3, … to the MPM modes and only unique modes are added into the secondary MPM list. This process is similar to the derivation of MPM modes in the primary MPM list. Number of derived modes depends on the MPM index as {x>>2, (x>>2)-1, (x>>2)-1, x>>3, x>>3, 1, 1}, where x is the length of the secondary MPM list, where the first number in the list indicates the maximum value of the offset that is used during the secondary MPM derivation, which is applied to the first mode in the primary MPM list.

Similarly, to the primary MPM list construction, if secondary MPM list is not complete, default modes are added. The default mode list is defined as {2, 18, 34, 50, 66; 10, 26, 42, 58; 6, 14, 22, 30, 38, 46, 54, 62; 17, 19, 49, 51}.

Differently from HEVC, rectangular shaped blocks can be used for intra prediction, and depending on the shape of a block some intra modes can be preferred. For a rectangular shape block, if the block is vertically oriented, i.e., when the height is greater than the width, the left (L) and above (A) modes are swapped in the MPM list.

Similarly, the neighboring intra mode close to horizontal intra direction may be preferable for the horizontally oriented rectangular blocks, and intra mode close to vertical intra direction may be preferable for vertically oriented rectangular blocks. In such cases, neighboring intra mode from left (L) and above (A) locations, which is closer to vertical or horizontal intra directions is inserted first into the MPM list.

### Bit stream organization

First, a context coded flag is signalled to indicate whether the intra mode is from the primary MPM list; this process is the same as in JEM.

If the mode is MPM, then the MPM index is signaled using truncated unary binarization, where the first three bins are context coded. Like JEM, the contexts are dependent on the intra mode index and 4 contexts are used in total.

If the mode is not from the primary MPM list, a context coded flag is signaled to indicate whether the mode is from the secondary MPM list. If the mode is from the secondary MPM list, the secondary MPM index is signaled in bypass mode, otherwise the intra mode index is signalled using truncated binary (TB) binarization with maximum value depending on the total number of intra modes as follows.

| Number of available intra mode | 35 | 67 | 131 |
| --- | --- | --- | --- |
| MaxVal | 20(19*) | 44(43) | 92(91) |

(*value for 7 MPMs).

Due to the symmetry in horizontal and vertical intra modes, during intra mode signalling the remaining mode index of the vertical intra modes is mapped to horizontal mode indices for the rectangular blocks with height greater or equal to the width as follows:

new_mode_index = MaxVal – mode_index.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3.1.1 | Intra 65, 6 modes in primary MPM, secondary MPM, shape dependency | Adarsh Krishnan Ramasubramonian (Qualcomm) | Anand Meher Kotra (Huawei) |

## Tool 2: Luma intra mode coding using a priority based list with Primary MPM, Secondary MPM and first few default modes prioritized (JVET-J0024 Samsung/Huawei)

Currently, MPM (most probable mode) is increased with increasing intra mode however this simple extension of MPM dilutes the purpose of MPM. As described in JVET-J0024 where the intra direction mode set is composed with 49 angular modes and 3 non-angular modes (DC, Plane, and Bilinear mode), two MPM and additional probable intra modes are used. It is called extended MPM and is aimed at reducing mode bits by using additional intra mode set.

The idea of prioritizing “remaining intra modes” is very similar to the proposal JVET-H0051 where a single priority list is created and distributed among Primary MPM, Secondary MPM, and remaining modes.

### Bit stream organization

Two MPMs are determined with intra mode of left, upper and right blocks. If information doesn’t exist or is not enough, DC or Planar mode is used for MPM. If MPM_flag=0, MPM_EXT_flag is checked. If MPM_flag=1, one bit is used for signaling the index of MPM.

Extended MPM is a secondary intra mode set composed with eight intra modes. Extended MPM is selected after MPM is determined. Neighbor block modes and MPM are considered to determine extended MPM set. If candidates are not enough to compose the mode set, statistically selected default candidate list is used. Default candidate is adaptively composed with reference availability and MPM mode. MPM_EXT flag is check when the MPM flag is 0. If MPM_EXT_flag=0, five or six bits are used to code intra mode. If MPM_EXT_flag=1, three bits are used for signaling the index of extended MPM.

The “remaining intra modes” apart from the intra modes in the MPM list and extended MPM list are signaled using a “truncated binary” code. The truncated binary code can be five bits or six bits long. The signaled truncated binary code can be treated as an index into the “remaining intra modes list”. First nine modes in the “remaining intra modes list” are filled from a “default mode list” which is ordered based on the “intra mode probability” occurrence.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3.2.1 | Priority based list with Primary MPM, Secondary MPM and first few default modes prioritized (method 1 from Samsung): TM intra modes | Narae Choi (Samsung) | Zhi Zhang (Ericsson), Jonathan Pfaff (HHI) |
| 3.2.2 | Priority based list with Primary MPM, Secondary MPM and first few default modes prioritized (method 2 from Huawei): 65 intra modes | Anand Meher Kotra (Huawei) | Yue Yu (ARRIS), Zhi Zhang (Ericsson), Jonathan Pfaff (HHI) |
| 3.2.3 | 6 MPM + Selected + Non-selected modes list (JEM macro JVET_B0051_NON_MPM_MODE), 65 intra modes | Anand Meher Kotra (Huawei) | Shimpei Nemoto (NHK) |

## Tool 3: Adaptive six MPM (JVET-J0017 LGE)

Figure 13. Neighboring blocks for MPMs

To accommodate the increased number of directional intra modes, an intra mode coding with 6 most probable modes (MPMs) is used in JEM. The changes proposed in this proposal differ in the order of constructing MPM list based on the dependency between neighboring intra modes.

### Bit stream organization

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3.3.1 | MPM list construction based on dependency between neighboring intra modes | Ling Li (LGE) | Anand Meher Kotra (Huawei) |

# CE3.4: Cross-component linear model (CCLM)

Technologies that are studied: cross component chroma from luma prediction with single model (LM) or multiple models (MMLM), multiple downsampling filters (MFLM), combining LM with intra prediction mode (LM-Angular). The following table lists the defined tests:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 4.1.1 | LM + MMLM + MFLM + LM-Angular | Adarsh Ramasubramonian (Qualcomm) | Jangwon Choi (LGE) |
| 4.1.2 | MMLM + MFLM + LM-angular | Adarsh Ramasubramonian (Qualcomm) | Jangwon Choi (LGE) |
| 4.1.3 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.4 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.5 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) + MMLM/MFLM (A, B, E, G) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.6 | LM + MMLM + multi filter LM + extended LM-Angular | Jangwon Choi (LGE) | Teresa Jiang (Foxconn) |
| 4.1.7 | LM + MMLM + extended LM-Angular | Jangwon Choi (LGE) | Teresa Jiang (Foxconn) |
| 4.1.8 | LM only (or single model CCLM) | Xiang Ma (Huawei) | D. Y. Kim (Chips&Media) |
| 4.1.9 | LM only + CCLM Cb-to-Cr | Xiang Ma (Huawei) | D. Y. Kim (Chips&Media) |
| 4.1.10 | LM+MMLM | Xiang Ma (Huawei) | Jinho Lee (ETRI) |
| 4.1.11 | LM+MMLM+MFLM | Xiang Ma (Huawei) | Jinho Lee (ETRI) |
| 4.2.1 | MDLM | Xiang Ma (Huawei) | Kei Kawamura (KDDI) Jonathan Pfaff (HHI) |
| 4.2.2 | LM + MDLM | Xiang Ma (Huawei) | Kei Kawamura (KDDI) Jonathan Pfaff (HHI) |
| 4.2.3 | LM+MDLM+MMLM | Xiang Ma (Huawei) | Chih-Wei Hsu (MediaTek) |
| 4.2.4 | LM+MDLM+MMLM+MFLM | Xiang Ma (Huawei) | Chih-Wei Hsu (MediaTek) |
| 4.3.1 | Inter-color reference prediction | Kei Kawamura (KDDI) | Xiang Ma (Huawei) |
| 4.3.2 | Adaptive inter-residual prediction with fast RDO | Kei Kawamura (KDDI) | Xiang Ma (Huawei) |
| 4.4.1 | LM + LM-left + LM-top | Chia-Ming Tsai (MediaTek) | Xingchun Qi (Hikvision), D.C. Kim (WILUS) |
| 4.4.2 | LM + LM-CbCr | Chia-Ming Tsai (MediaTek) | Xingchun Qi (Hikvision) |
| 4.4.3 | LM + LM fusion | Chia-Ming Tsai (MediaTek) | Xingchun Qi (Hikvision) |
| 4.4.4 | LM + LM-left + LM-top + LM-CbCr + LM fusion | Chia-Ming Tsai (MediaTek) | Xingchun Qi (Hikvision) |

## Tool 1: LM, MMLM, MFLM, LM-Angular, extended LM-Angular (JVET-J0021 Qualcomm/Technicolor; JVET-J0026 Foxconn/Sharp; JVET-J0017 LGE)

### LM

To reduce the cross-component redundancy, a cross-component linear model (CCLM) prediction mode is used in the JEM, for which the chroma samples are predicted based on the reconstructed luma samples of the same CU by using a linear model as follows:

where  represents the predicted chroma samples in a CU and  represents the downsampled reconstructed luma samples of the same CU. Parameters  and  are derived by minimizing the regression error between the neighbouring reconstructed luma and chroma samples around the current block as follows:

where  represents the down-sampled top and left neighbouring reconstructed luma samples,  represents the top and left neighbouring reconstructed chroma samples, and value of is equal to twice of the minimum of width and height of the current chroma coding block. For a coding block with a square shape, the above two equations are applied directly. For a non-square coding block, the neighbouring samples of the longer boundary are first subsampled to have the same number of samples as for the shorter boundary. Figure 14 shows the location of the left and above causal samples and the sample of the current block involved in the CCLM mode.

This regression error minimization computation is performed as part of the decoding process, not just as an encoder search operation, so no syntax is used to convey the α and β values.

Figure 14: Locations of the samples used for the derivation of α and β

The CCLM prediction mode also includes prediction between the two chroma components, i.e., the Cr component is predicted from the Cb component. Instead of using the reconstructed sample signal, the CCLM Cb-to-Cr prediction is applied in residual domain. This is implemented by adding a weighted reconstructed Cb residual to the original Cr intra prediction to form the final Cr prediction:

The scaling factor is derived in a similar way as in the CCLM luma-to-chroma prediction. The only difference is an addition of a regression cost relative to a default  value in the error function so that the derived scaling factor is biased towards a default value of −0.5 as follows:

where  represents the neighbouring reconstructed Cb samples,  represents the neighbouring reconstructed Cr samples, and  is equal to .

The CCLM luma-to-chroma prediction mode is added as one additional chroma intra prediction mode. At the encoder side, one more RD cost check for the chroma components is added for selecting the chroma intra prediction mode. When intra prediction modes other than the CCLM luma-to-chroma prediction mode is used for the chroma components of a CU, CCLM Cb-to-Cr prediction is used for Cr component prediction.

### Multiple model CCLM

In the JEM, there are two CCLM modes: the single model CCLM mode and the multiple model CCLM mode (MMLM). As indicated by the name, the single model CCLM mode employs one linear model for predicting the chroma samples from the luma samples for the whole CU, while in MMLM, there can be two models. In MMLM, neighbouring luma samples and neighbouring chroma samples of the current block are classified into two groups, each group is used as a training set to derive a linear model (i.e., a particular α and β are derived for a particular group). Furthermore, the samples of the current luma block are also classified based on the same rule for the classification of neighbouring luma samples.

Figure 15 shows an example of classifying the neighbouring samples into two groups. Threshold is calculated as the average value of the neighbouring reconstructed luma samples. A neighbouring sample with Rec′L[x,y] <= Threshold is classified into group 1; while a neighbouring sample with Rec′L[x,y] > Threshold is classified into group 2.

Figure 15: An example of classifying the neighbouring samples into two groups

### Multiple downsampling filters in CCLM mode

To perform cross-component prediction, for the 4:2:0 chroma format, the reconstructed luma block needs to be downsampled to match the size of the chroma signal. The default downsampling filter used in CCLM mode is as follows.

Note that this downsampling assumes the “type 0” phase relationship for the positions of the chroma samples relative to the positions of the luma samples – i.e., collocated sampling horizontally and interstitial sampling vertically.

The above 6-tap downsampling filter is used as the default filter for both the single model CCLM mode and the multiple model CCLM mode.

For the MMLM mode, the encoder can alternatively select one of four additional luma downsampling filters to be applied for prediction in a CU, and send a filter index to indicate which of these is used. The four selectable luma downsampling filters for the MMLM mode are as follows:

### Multiple neighbor LM mode (MNLM)

A cross component linear model prediction process is used in the JEM to predict chroma samples based on the reconstructed luma samples of the same CU. This includes a simple linear model (LM), multiple model LM (MMLM), and multiple filter LM (MFLM). The parameters of these linear models are derived by minimizing the regression error using the reconstructed luma samples and chroma samples in the neighbouring data set.

In JEM, the neighbouring data set for LM is composed of the pairs of luma and chroma samples in line areas B and C as shown in Figure 16, and the neighbouring data set for MMLM and MFLM is composed of the pairs of luma and chroma samples in line area B, C, E and F. However, it might not be able to fully reflect the linear relationship between the neighbouring luma and chroma samples to the linear relationships in CU. Therefore, multiple neighbouring data sets for CCLM (MNLM) is proposed in JVET-J0026 to use three neighbouring data sets for LM, MMLM and MFLM to cover various linear relationships between luma samples and chroma samples in CU.

Figure 16: Multiple neighbouring data sets are required for various linear relationship between luma and chroma samples in CU

Three neighbouring data sets are defined as follows:

T0:  B, C

T1:  B, C, E, F

T2:  A, B, C, D

T3:  C, D, F, H

T4:  A, B, E, G

One example for the neighbouring data sets and the corresponding binarization for different cross component predictions is shown in the below table:

|  | Binarization | Training data set |
| --- | --- | --- |
| MMLM1 | 1 | T1 |
| LM | 01 | T0 |
| MMLM2 | 001 | T2 |
| MFLM1 | 00001 | T1 |
| MFLM2 | 00010 | T1 |
| MFLM3 | 00000 | T2 |
| MFLM4 | 00011 | T2 |

More various neighbouring data sets are asserted to be able to cover more various luma-chroma linear relationships in CU.

### LM-Angular

CCLM included in JEM is extended by adding LM-Angular mode as proposed in JVET-D0110. In this mode final chroma prediction is a combination of LM prediction, , and prediction, , obtained using DC, planar or angular mode:

### Extended LM-Angular

In this mode, two CCLM modes (single model CCLM mode or multiple model CCLM mode) are used for LM-Angular prediction. If the current chroma block is coded with the LM-Angular mode, a flag is signaled to indicate corresponding CCLM mode (single model CCLM or multi model CCLM) used for LM-Angular prediction.

### Bit stream organization

1st Bin: 0 (if 1, DM mode)

2nd Bin: 1: MMLM (stop), 0: others

3rd Bin: 1: LM(stop), 0: continue for filter type

4-5 Bins: filter type

Extended LM-Angular method:  N/A

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 4.1.1 | LM + MMLM + MFLM + LM-Angular | Adarsh Ramasubramonian (Qualcomm) | Jangwon Choi (LGE) |
| 4.1.2 | MMLM + MFLM + LM-angular | Adarsh Ramasubramonian (Qualcomm) | Jangwon Choi (LGE) |
| 4.1.3 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.4 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.5 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) + MMLM/MFLM (A, B, E, G) | Yao-Jen Chang (Foxconn) | Adarsh Ramasubramonian (Qualcomm) |
| 4.1.6 | LM + MMLM + multi filter LM + extended LM-Angular | Jangwon Choi (LGE) | Teresa Jiang (Foxconn) |
| 4.1.7 | LM + MMLM + extended LM-Angular | Jangwon Choi (LGE) | Teresa Jiang (Foxconn) |
| 4.1.8 | LM only (or single model CCLM) | Xiang Ma (Huawei) | D. Y. Kim (Chips&Media) |
| 4.1.9 | LM only + CCLM Cb-to-Cr | Xiang Ma (Huawei) | D. Y. Kim (Chips&Media) |
| 4.1.10 | LM+MMLM | Xiang Ma (Huawei) | Jinho Lee (ETRI) |
| 4.1.11 | LM+MMLM+MFLM | Xiang Ma (Huawei) | Jinho Lee (ETRI) |

## Tool 2: MDLM (JVET-J0025 Huawei/Samsung)

### Multi-directional LM (MDLM)

Besides the above template and left template can be used to calculate the linear model coefficients together, they also can be used alternatively in the other 2 LM modes, called LM_A, and LM_L modes.

In LM_A mode, only the above template are used to calculate the linear model coefficients. To get more samples, the above template are extended to (W+H).

Figure 17: LM_A mode

In LM_L mode, only left template are used to calculate the linear model coefficients. To get more samples, the left template are extended to (H+W).

Figure 18: LM_L mode

For a non-square coding block, another (N-W-H) samples will be sampled from (W+H) samples with equal steps to meet the condition of power of 2. Here N is the number of reference samples (to calculate the linear model coefficient). Here note that, N is the smallest integer value that is not smaller than (W+H), and N is a value of power of 2.

### Bit stream organization

lm_flag

if( lm_flag ) {

mdlm_flag

if( mdlm_flag )

mdlm_l_flag

}

else

/*other chroma modes*/

### Test Scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 4.2.1 | MDLM | Xiang Ma (Huawei) | Kei Kawamura (KDDI) Jonathan Pfaff (HHI) |
| 4.2.2 | LM + MDLM | Xiang Ma (Huawei) | Kei Kawamura (KDDI) Jonathan Pfaff (HHI) |
| 4.2.3 | LM+MDLM+MMLM | Xiang Ma (Huawei) | Chih-Wei Hsu (MediaTek) |
| 4.2.4 | LM+MDLM+MMLM+MFLM | Xiang Ma (Huawei) | Chih-Wei Hsu (MediaTek) |

## Tool 3: Inter-color chroma prediction in (JVET-J0016 KDDI)

### Inter-color reference prediction (JVET-J0016 KDDI)

JVET-J0016 is stated as an additional chroma-intra-prediction mode. The reference samples of chroma component is replaced by that of luma component with a linear model. The linear model is borrowed from the existing CCLM tool in the benchmark set. After the refinement of the reference samples, the existing chroma intra-prediction is performed. The prediction mode of chroma is borrowed from the mode of luma.

For the interaction testing, alternative parameter derivation of CCLM prediction need to be tested.

### Adaptive inter-residual prediction in (JVET-J0016 KDDI)

CCLM Cb-to-Cr prediction exists in the benchmark set. The adaptive inter-residual prediction introduces CCLM Cr-to-Cb prediction. Reference component is signaled by an additional flag. The remaining parts like prediction and scaling factor derivation are the same as in the benchmark set. One flag for each chroma CU is added to identify the reference chroma component. This is implemented by adding a weighted reconstructed Cb residual to the original Cr intra prediction to form the final Cr prediction:

The scaling factor  is derived in the same way of CCLM Cb-to-Cr prediction.

When the intra prediction modes other than CCLM luma-to-chroma prediction mode is used for chroma component of a CU, CCLM Cb-to-Cr or Cr-to-Cb prediction is used to enhance the Cr or Cb component prediction.

Two tests are proposed. One is test with full RDO and another is test with fast RDO. The details are shown in the following section.

For a fast encoding, a reduction of the number of RD cost check is proposed. The key idea is to decide the reference chroma component by only one time RD cost check. To realize the idea, the decision is based on the RD cost of DM_Chroma since a probability of DM_Chroma is highest. At the same time, DM_Chroma is moved to the first position in chroma RD-cost-check order. It is noted that a signal processing based decision without RD cost check does not work well in our experiments.

### Bit stream organization

### Test Scenarios

| # | Test | Tester | Cross-checkers |
| --- | --- | --- | --- |
| 4.3.1 | Inter-color reference prediction | Kei Kawamura (KDDI) | Xiang Ma (Huawei) |
| 4.3.2 | Adaptive inter-residual prediction with fast RDO | Kei Kawamura (KDDI) | Xiang Ma (Huawei) |

## Tool 4: Linear model modes for intra chroma prediction (JVET-J0018 MediaTek)

### Additional LM-based modes for intra chroma prediction

In addition to existing LM-based modes in JEM, some new LM-based modes are presented, including LM-left and LM-top modes, LM-CbCr mode, and LM fusion mode. The proposed LM-left and LM-top modes, and LM-CbCr mode provide alternative methods for cross-component linear model parameters derivation. For the LM-left and LM-top modes, 2 left columns or 2 above rows neighboring CU reconstructed samples are used to determine the linear model parameters between the neighboring reconstructed luma and Cb components. Besides, LM-left or LM-top mode is also implicitly used when the above or left neighboring reconstructed samples of the current CU are not accessible. For the LM-CbCr mode, two separate linear models are derived. One is between the neighboring reconstructed luma and Cb components, and the other is between the neighboring reconstructed Cb and Cr components. Finally, LM fusion mode is presented for combining the predictions from LM and DM as a new prediction.

### Sample selection method for model derivation by discarding samples at the longer side

A division-free derivation process of linear model parameters is used in JEM. The number of samples for model derivation needs to be power of 2. Therefore, for a non-square chroma CB to select samples for model derivation, samples at the longer side are uniformly sub-sampled to match the sample number at the shorter side. In the proposed method, the samples that are at the longer side and far most from the shorter side are discarded, and the number of discarded samples at the longer side is equal to the samples at the shorter side. An example is shown in Figure 19.

| (a) | (b) |
| --- | --- |

Figure 19: For a 16x4 chroma CB, only the yellow samples at the longer side and the blue samples at the shorter side are used in model derivation process. (a) The chosen samples in JEM-7.0. (b) The chosen samples in this proposal with 4 samples discarded at the longer side.

### Bit stream organization

1st Bin: 0 (if 1, then DC, planar or angular mode)

2rd Bin: 1: LM(stop), 0: others

3rd Bin: 1: LM-left(stop), 0: others

4rd Bin: 1: LM-top(stop), 0: LM-CbCr(stop)

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 4.4.1 | LM + LM-left + LM-top | Chia-Ming Tsai (MediaTek) | Xingchun Qi (Hikvision), D.C. Kim (WILUS) |
| 4.4.2 | LM + LM-CbCr | Chia-Ming Tsai (MediaTek) | Xingchun Qi (Hikvision) |
| 4.4.3 | LM + LM fusion | Chia-Ming Tsai (MediaTek) | Xingchun Qi (Hikvision) |
| 4.4.4 | LM + LM-left + LM-top + LM-CbCr + LM fusion | Chia-Ming Tsai (MediaTek) | Xingchun Qi (Hikvision) |

# CE3.5: Multi-reference line intra prediction

The following table lists the defined tests:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 5.1.1 | Mode dependent reference line selection | Liang Zhao (Tencent) | Po-Han Lin(ITRI), Benjamin Bross (HHI) |
| 5.1.2 | Reference sample extension for multiline intra prediction | Liang Zhao (Tencent) | Po-Han Lin(ITRI), Benjamin Bross (HHI) |
| 5.2.1 | Multiple reference lines | Fabien Racape (Technicolor) | Ling Li (LGE) |
| 5.2.2 | Multiple reference lines + boundary filtering | Fabien Racape (Technicolor) | Ling Li (LGE) |
| 5.2.3 | Multiple reference lines and not used for top line of CTU | Steve Keating (Sony) | Po-Han Lin (ITRI) |
| 5.2.4 | Multiple reference lines with 50:50 weighting | Steve Keating (Sony) | Ling Li (LGE) |
| 5.2.5 | Multiple reference lines with 50:50 weighting (multiple reference lines not used for 4xN and Nx4) | Steve Keating (Sony) | Po-Han Lin (ITRI) |
| 5.3.1 | r1: 6-tap combined filter without reference sample smoothing, r2: bi-linear | Jinho Lee (ETRI) | Fabien Racape (Technicolor) |
| 5.3.2 | r1: 4-tap filter, r2: bi-linear | Jinho Lee (ETRI) | Fabien Racape (Technicolor) |
| 5.3.3 | r1: bi-linear, r2: bi-linear | Jinho Lee (ETRI) | Fabien Racape (Technicolor) |
| 5.4.1 | MRL all block sizes | Benjamin Bross (HHI) | Liang Zhao (Tencent) |
| 5.4.2 | MRL restricted block sizes (encoder only) | Benjamin Bross (HHI) | Liang Zhao (Tencent) |
| 5.4.3 | MRL restricted block sizes (restr. signalling) | Benjamin Bross (HHI) | Liang Zhao (Tencent) |
| 5.4.4 | MRL + Mode dependent reference line selection (5.1.1) | Benjamin Bross (HHI) | Maxim Sychev (Huawei) |
| 5.5.1 | Use two extended reference lines | Po-Han Lin (ITRI) | Liang Zhao (Tencent) |
| 5.5.2 | Use three extended reference lines | Po-Han Lin (ITRI) | Liang Zhao (Tencent) |

## Tool 1: Mode dependent reference line selection (JVET-J0029/JVET-J0069 Tencent)

It is proposed that the signaling of reference line index is dependent on the intra prediction modes. For one set of intra prediction modes, e.g., directional intra prediction modes with an even mode index, e.g., mode 2, 4, 6… 66, each of the four reference lines can be used and the selection is signaled. However, for the remaining modes, e.g., Planar/DC and directional intra prediction modes with odd mode index, only the first reference line, i.e., reference line 0, can be used and reference line index is not signaled.

### Reference sample extension

Instead of padding the closest sample from Segment B and E, as shown in Figure 20, the reconstructed values of reference samples in Segment A and F are used when they are available. In addition, the reference sample buffer is extended by two samples, as indicated by yellow samples in Figure 20.

Figure 20: Four reference lines adjacent to a coding block unit

### Bit stream organization

For multi-line index signaling, unary code is used and up to 3 contexts are used.

### Test Scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 5.1.1 | Mode dependent reference line selection | Liang Zhao (Tencent) | Po-Han Lin(ITRI), Benjamin Bross (HHI) |
| 5.1.2 | Reference sample extension for multiline intra prediction | Liang Zhao (Tencent) | Po-Han Lin(ITRI), Benjamin Bross (HHI) |

## Tool 2: Multi-Reference intra prediction (MRIP) (JVET-J0022 Technicolor/Qualcomm; JVET-J0028 Sony)

Two reference layers are used for intra prediction, where each reference layer consists of a left reference array and a top reference array (Figure 21). Each reference layer is constructed by the reference sample substitution and then pre-filtered.

Using each reference layer, a prediction for the block is constructed. The final prediction is formed as a weighted average of the predictions made from the two reference layers. The prediction from the closest reference layer is given higher weight than the prediction from the farthest layer. The used weights are 3 and 1. Multi-reference intra prediction is not activated when for luma blocks coded as Planar or DC, or for chroma blocks coded as LMChroma. In the other cases, it is activated.

Figure 21. Intra prediction with 2 reference layers. The layer #1 consists of the usual reference array, the layer #2 consists of the next reference array on top and on left of layer #1.

To avoid the cases of sharp changes in intensity at the block boundaries, multiple reference layers are used to smoothen the boundary samples for certain prediction modes. The process is mode dependent. For example,

- For DC prediction mode, the boundary filtering remains the same as in JEM.

- For the four modes closest to the purely vertical/horizontal modes, the predicted pixel values on the first column/line of a block are filtered as:

Vertical modes – for y = 0 to H – 1

P(0,y) = Clip(P(0,y) + (3*((P(-1,y)-P(-1,-1)) >> 2) + ((P(-2,y)-P(-2,-2)) >> 2))/(4))

Horizontal modes – for x = 0 to W – 1

P(x,0) = Clip(P(x,0) + (3*((P(x,-1)-P(-1,-1)) >> 2) + ((P(x,-2)-P(-2,-2)) >> 2))/(4))

This filtering is applied to luma blocks having their width and height smaller than 32.

- For vertical/horizontal modes, a gradient based filtering is applied to the predicted samples on the left/top boundary as follows:

Down-left modes – for x = 0 to W – 1, y = 0 to H – 1

P(x,y) = Clip(P(x,y) + (y)* L(x) + 32)/64)

where: D(y) = P(-1,y)-((3*P(s1,-1)+P(s2,-2)+2) >> 2)

and:    L(x) = (32 >> ((x << 1) >> 2))

Up-right modes– for x = 0 to W – 1, y = 0 to H – 1

P(x,y) = Clip(P(x,y) + (x)* L(y) + 32)/64)

where: D(x) = P(x,-1)-((3*P(-1,s1)+P(-2,s2)+2) >> 2)

and:     L(y) = (32 >> ((y << 1) >> 2))

where D(y)/D(x) correspond to the gradient at the first left/top reference array at distance y/x from the top of the block (Figure 22). s1 and s2 are the reference samples in the first and second reference layers, respectively, of the sample P(x,y), for the considered directional mode.

Figure 22. Gradient based boundary filtering for a vertical mode. The gradient is computed at the left reference array by taking the difference of the actual sample value and its prediction using the reference arrays on the top.

### Bit stream organization

In the solution provided in J0022, a flag is coded in the SPS to enable the tool to enable the tool at sequence level.

### Test Scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 5.2.1 | Multiple reference lines | Fabien Racape (Technicolor) | Ling Li (LGE) |
| 5.2.2 | Multiple reference lines + boundary filtering | Fabien Racape (Technicolor) | Ling Li (LGE) |
| 5.2.3 | Multiple reference lines and not used for top line of CTU | Steve Keating (Sony) | Po-Han Lin (ITRI) |
| 5.2.4 | Multiple reference lines with 50:50 weighting | Steve Keating (Sony) | Ling Li (LGE) |
| 5.2.5 | Multiple reference lines with 50:50 weighting (multiple reference lines not used for 4xN and Nx4) | Steve Keating (Sony) | Po-Han Lin (ITRI) |

## Tool 3: Multi-line based Intra Prediction (MIP) (JVET-J0013 ETRI/Sejong Univ.)

MIP uses weighted combination of two reference lines, the nearest reference line and the second nearest reference line to the CU, for intra prediction. Weighting factor 3/4 is used for the nearest reference line and weighting factor 1/4 is used for the second nearest reference line as shown in Figure 8. The MIP is applied when intra prediction mode is horizontal, vertical or angular mode for luma signal.

For angular mode where interpolation filter applies, CF filter, 4-tap or bilinear is applied to the nearest reference line and bi-linear interpolation filter defined in HEVC is applied to the second nearest reference line.

Figure 23. Multi-line based intra prediction when using CF filter and bi-linear filter

### Bit stream organization

N/A.

### Test Scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 5.3.1 | r1: 6-tap combined filter without reference sample smoothing, r2: bi-linear | Jinho Lee (ETRI) | Fabien Racape (Technicolor) |
| 5.3.2 | r1: 4-tap filter, r2: bi-linear | Jinho Lee (ETRI) | Fabien Racape (Technicolor) |
| 5.3.3 | r1: bi-linear, r2: bi-linear | Jinho Lee (ETRI) | Fabien Racape (Technicolor) |

## Tool 4: MRL with explicit signaling (JVET-J0014 HHI)

Test MRL as proposed in JVET-J0014. Conventional intra picture prediction always uses the directly neighboring samples as reference samples for prediction. MRL extends the conventional luma prediction to use neighboring samples with one or three samples distance to the left and top of the current prediction block. The conventional directly neighboring reference sample lines and the extended reference sample lines are illustrated in the figure below. The mrl_idx indicates which line to use in intra-picture prediction of a CU.

Figure 24. Directly neighboring (nearest) reference samples in blue and extended reference samples in green used in MRL.

### Bit stream organization

The usage of multiple-reference line intra prediction (MRL) is indicated by the syntax element mrl_idx. The value of mrl_idx indicates which reference line is used in intra. Allowed values are 0, 1 and 3 which are coded using truncated unary binarization with 0 for mrl_idx = 0, 10 for mrl_idx = 1, 11 for mrl_idx = 3 and a separate context model for each bin.

### Test Scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 5.4.1 | MRL all block sizes | Benjamin Bross (HHI) | Liang Zhao (Tencent) |
| 5.4.2 | MRL restricted block sizes (encoder only) | Benjamin Bross (HHI) | Liang Zhao (Tencent) |
| 5.4.3 | MRL restricted block sizes (restr. signalling) | Benjamin Bross (HHI) | Liang Zhao (Tencent) |
| 5.4.4 | MRL + Mode dependent reference line selection (5.1.1) | Benjamin Bross (HHI) | Maxim Sychev (Huawei) |

## Tool 5: Number of extended reference line for intra prediction (JVET-J0070 ITRI)

Not only the nearest(first) reference line but also extended reference lines with single line increment are utilized in intra prediction. The prediction generated from the nearest and extended reference lines will compete together to choose the best prediction for each block. DC and Planar modes are not used to generate prediction from the extended reference lines. Extended reference lines are applied only for luma prediction.

### Bit stream organization

Use two extended reference lines:

1st Bin: 0 (if 1, use the first reference line)

2nd Bin: 1: use the second reference line, 0: use the third reference line

Use three extended reference lines:

1st Bin: 0 (if 1, use the first reference line)

2nd Bin: 1: use the second reference line(stop), 0: others

3rd Bin: 1: use the third reference line, 0: use the fourth reference line

### Test Scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 5.5.1 | Use two extended reference lines | Po-Han Lin (ITRI) | Liang Zhao (Tencent) |
| 5.5.2 | Use three extended reference lines | Po-Han Lin (ITRI) | Liang Zhao (Tencent) |

# CE3.6: Neural Network technologies for intra

## Tool 1: Neural network intra prediction (JVET-J0014, JVET-J037 HHI)

The basic idea of the neural network based intra predictions is to generate an intra prediction signal on a given block by processing the reference samples trough a neural network. This network has three hidden layers and one output layer. In order to reduce the number of operations, it is suggested that the neural network based intra predictors predict into the frequency domain where a lot of frequencies in the prediction signal are constrained to a constant value. An example of the neural network based intra predictors is described in the documents JVET-J0014 and JVET-J0037.

Generation of the prediction signal

On a given MxN block with , the generation of a proposed luma prediction signal  is performed by processing a set of reference samples  through a neural network. The reference samples  consist of K rows of size N+K above and K columns of size M left of the block, see Figure 1. The number K depends on M and N.

Fig. 1: Prediction of MxN intra block from reconstructed samples using a neural network

The proposed neural network first extracts a vector  of features from the reconstructed samples  as follows. If d0=K*(N +M+K) denotes the number of samples of , then  is regarded as a vector in the real vector space of dimension d0. For fixed integral square-matrices  and  which have d0 rows resp. columns and for fixed integral bias vectors b1 and b2 of dimension d0 one first computes

Here,  denotes the ordinary matrix-vector product. Moreover, the function ρ is an integer-approximation of the ELU function , where the latter function is defined on a p-dimensional vector  by putting

where  and  denote the i-th component of the vectors. One applies similar operations to  and computes

Finally, for a fixed integer d1 with , there is a predefined integral matrix  with d1 rows and d0 columns and there is a predefined integral bias vector  of dimension d1 such that one computes the feature vectoras

The value of d1 depends on M and N.

Now one differentiates two cases. First, the case N=4 and M=4 is treated. Here, one directly predicts into the image domain. Thus, there is a predefined matrix  with M*N rows and d1 columns and a predefined bias vector  of dimension M*N such that one computes

where is the standard clipping operation that depends on the bit-depth.

Now, the case N>4 or M>4 is treated. Here, one predicts into the frequency domain. Thus, there is a predefined matrix  with M*N rows and d1 columns and a predefined bias vector  of dimension M*N such that one computes

If  is the matrix of the integral approximation of the Inverse Discrete Cosine Transform (DCT II) that is already present in the codecit is proposed that the actual prediction signal  is then obtained as

It is proposed that most of the frequency components of the prediction signal  are constrained to a constant value. If the latter constraint is true for a specific frequency component, then the row of the matrix  corresponding to that frequency component is identically zero and no multiplications need to be performed for the corresponding frequency component. The latter effect is illustrated by the fact that for example on square blocks of size 32x32, it is reported that at least 813 and up to 927 rows of the 1024 rows of the matrices  are identically zero for the 11 proposed intra prediction modes.

Signalization of the specific proposed intra mode

We proposed  different intra prediction modes, where  is set to 35 for max(M,N)<32 and to 11 else. Thus, an index  with is to be signaled by an encoder and to be parsed by a decoder and the following syntax is proposed. One has

where if max(M,N)=32 and , else. In a first step, an index  with is signaled using the following code. First, one bin encodes whether  or not. If , a second bin encodes if  or not, and, if , another bin encodes whether  is equal to 1 or 2. If  then the value of  is signaled in the canonical way using  bins. This syntax for the signalization of the index  is delineated in Table 1.   	
From the index , the actual index  is derived using a fully connected neural network with one hidden layer that has the reconstructed samples  on the two rows of size N+2 above and the two columns of size M left of the block as input, see Figure 2.

Fig. 2: Prediction of mode probabilities from reconstructed samples using a neural network

The reconstructed samples  are regarded as a vector in the real vector space of dimension 2*(M+N+2). There is a fixed square-matrix A1ʹ which has 2*(M+N+2) rows resp. columns and there is a fixed bias vector b1ʹ in the real vector space of dimension 2*(M+N+2) such that one computes

Next, there exists a matrix A2ʹ which has  rows and 2*(M+N+2) columns and there is a fixed bias vector b2ʹ in the real vector space of dimension n such that one computes

The index  is now derived as being the position of the -th largest component of . Here, if two components  and  are equal for k≠l , is regarded as larger than   if  and   is regarded as larger than, else.	
To explain the above procedure, it is remarked that the -th entry of the vector is to be interpreted, up to normalization, as the logarithm (‘logit’) of the conditional probability of the -th proposed intra prediction mode given the reference samples and thus the index indicates that the -th most probable mode is selected. 	
The derivation of the index  proposed in this section is depicted on the left hand side of Figure 3 below.

|  |  | flag | unary_max |  | fixed_length |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | predIdx | CTX | EP |  | LSB |  | EP | MSB |  |
| MPM | 0 | 1 | 0 |  |  |  |  |  |  |
|  | 1 | 1 | 1 | 0 |  |  |  |  |  |
|  | 2 | 1 | 1 | 1 |  |  |  |  |  |
|  | 3 | 0 |  |  | 0 | 0 | 0 | 0 | 0 |
|  | 4 | 0 |  |  | 1 | 0 | 0 | 0 | 0 |
|  | 5 | 0 |  |  | 0 | 1 | 0 | 0 | 0 |
|  | 6 | 0 |  |  | 1 | 1 | 0 | 0 | 0 |
|  | 7 | 0 |  |  | 0 | 0 | 1 | 0 | 0 |
|  | 8 | 0 |  |  | 1 | 0 | 1 | 0 | 0 |
|  | 9 | 0 |  |  | 0 | 1 | 1 | 0 | 0 |
|  | 10 | 0 |  |  | 1 | 1 | 1 | 0 | 0 |
| if width < 32 and height < 32 | 11 | 0 |  |  | 0 | 0 | 0 | 1 | 0 |
|  | 12 | 0 |  |  | 1 | 0 | 0 | 1 | 0 |
|  | 13 | 0 |  |  | 0 | 1 | 0 | 1 | 0 |
|  | 14 | 0 |  |  | 1 | 1 | 0 | 1 | 0 |
|  | 15 | 0 |  |  | 0 | 0 | 1 | 1 | 0 |
|  | 16 | 0 |  |  | 1 | 0 | 1 | 1 | 0 |
|  | 17 | 0 |  |  | 0 | 1 | 1 | 1 | 0 |
|  | 18 | 0 |  |  | 1 | 1 | 1 | 1 | 0 |
|  | 19 | 0 |  |  | 0 | 0 | 0 | 0 | 1 |
|  | 20 | 0 |  |  | 1 | 0 | 0 | 0 | 1 |
|  | 21 | 0 |  |  | 0 | 1 | 0 | 0 | 1 |
|  | 22 | 0 |  |  | 1 | 1 | 0 | 0 | 1 |
|  | 23 | 0 |  |  | 0 | 0 | 1 | 0 | 1 |
|  | 24 | 0 |  |  | 1 | 0 | 1 | 0 | 1 |
|  | 25 | 0 |  |  | 0 | 1 | 1 | 0 | 1 |
|  | 26 | 0 |  |  | 1 | 1 | 1 | 0 | 1 |
|  | 27 | 0 |  |  | 0 | 0 | 0 | 1 | 1 |
|  | 28 | 0 |  |  | 1 | 0 | 0 | 1 | 1 |
|  | 29 | 0 |  |  | 0 | 1 | 0 | 1 | 1 |
|  | 30 | 0 |  |  | 1 | 1 | 0 | 1 | 1 |
|  | 31 | 0 |  |  | 0 | 0 | 1 | 1 | 1 |
|  | 32 | 0 |  |  | 1 | 0 | 1 | 1 | 1 |
|  | 33 | 0 |  |  | 0 | 1 | 1 | 1 | 1 |
|  | 34 | 0 |  |  | 1 | 1 | 1 | 1 | 1 |

Table 1: Binarization of the index j used to generate the prediction mode index predIdx

Extension to larger blocks

For blocks of size 64x64, the proposed intra prediction modes for blocks of size 32x32 are reused. For that purpose, for K=2 one first takes 2K rows of size 64+2K above the block and 2K columns of size 64 left of the block with reconstructed samples . Then, one applies a normalized one-two-one-filter in each direction followed by a factor two downsampling in each direction. This yields reconstructed samples that can be regarded as an input for the previously proposed prediction modes on blocks of size 32x32. Let  be a prediction signal of size 32x32 generated by the proposed method using the input rds. Then it is proposed that is upsampled by bilinear interpolation to obtain a prediction signal  on the actual block of size 64x64. Using a similar operation, the signalization of the proposed intra prediction modes for blocks of size 32x32 induces a signalization for the proposed intra prediction modes for blocks of size 64x64. 	
A similar procedure to reuse the proposed intra prediction modes for blocks of size 16x16 to generate prediction signals on blocks of size 32x32 is proposed.

Variation of the network structure and additional transforms

It is proposed to conduct several core experiments in order to study the impact of a variation of the network architecture both on coding performance and on complexity. This might include for example a study of the variation of the number of layers of the network, the replacement of the exponential linear unit by the rectified linear unit, a sharing of the weights of the networks between different blocksizes, a variation of the number of proposed modes for each blocksize and a variation of the binarization scheme for the mode index of the neural network based intra predictor. Also, mode dependent (secondary) transforms for the proposed neural network based intra predictions may be studied.

### Bit stream organization

A first syntax element on CU level indicates the usage of the neural network mode. Another syntax element indicates the mode index.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.1 | Intra prediction using neural networks (JVET-J0037) | Jonathan Pfaff (HHI) | F. Racape (Technicolor), Sergey Ikonin (Huawei), Yongbing Lin (HiSilicon) |

# CE3.7: Bidirectional intra prediction

The following table lists the defined tests:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 7.1.1 | Bi-directional Intra prediction (BDIP) | Fabien Racape (Technicolor) | Vasily Rufitskiy (Huawei), Jin Heo (LGE) |
| 7.2.1 | Distance-Weighted Directional Intra Prediction | Alexey Filippov (Huawei) | Fabien Racape (Technicolor), Jonathan Pfaff (HHI) |
| 7.3.1 | Linear interpolation intra prediction (LIP) based on MPM flag following LIP flag signaling | Jin Heo (LGE) | Alexey Filippov (Huawei) |
| 7.3.2 | LIP based on LIP flag following MPM flag signaling | Jin Heo (LGE) | Alexey Filippov (Huawei) |

## Tool 1: Bi-directional Intra prediction (BDIP) (JVET-J0022 Technicolor/Qualcomm)

In bi-directional prediction a target pixel value is predicted using two reference samples on both ends of a positive prediction direction, as shown in the Figure 25. A CU-level flag (BDIPflag) indicates the application of bi-directional intra prediction. The flag is signaled only for luma blocks with prediction modes which cross both left and above reference sample lines. For the remaining modes, the traditional uni-directional prediction is performed.

Figure 25. In bi-directional prediction, two reference samples are used for computing the prediction for a target pixel. The second reference sample is located at the intersection of the prediction direction with the second reference array.

For a positive vertical prediction direction, the first reference sample is at displacement  from x-coordinate of the target pixel at (x,y). As specified in JEM,

where A denotes the angle associated with the prediction mode. The second reference sample is at displacement  from the y-coordinate of the target pixel at (x,y).  can be computed as:

The values of   are pre-computed for all positive A and x = 0, and stored in a table. For any value of x, the value of  is computed by incrementing this stored value by its magnitude x times. Denoting , the tabulated values for different positive A values are given in Table 1 in case of using 65 modes as in JEM.

Table 1. Parameter Ay for different positive values of A in JEM.

| A | 1 | 2 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 23 | 26 | 29 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ay | 1024 | 512 | 341 | 205 | 146 | 114 | 93 | 79 | 68 | 60 | 54 | 49 | 45 | 39 | 35 | 32 |

If the coordinate of the second reference sample lies beyond the length of the left reference array, the last reference sample on the left reference array is taken to be the second reference sample value.

Using the first and the second reference samples, an update term is computed and added to the initial prediction (which is the first reference sample value):

Here  and  are the first and the second reference sample values for a target pixel,  is the update, and  is the final prediction value. For positive horizontal prediction directions, the computations are similar.

### Bit stream organization

The flag for CU-level activation is coded with a CABAC context.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 7.1.1 | Bi-directional Intra prediction (BDIP) | Fabien Racape (Technicolor) | Vasily Rufitskiy (Huawei), Jin Heo (LGE) |

## Tool 2: Distance-Weighted Directional Intra Prediction (DWDIP) (JVET-J0025 Huawei/Samsung)

Distance-weighted directional intra prediction (DWDIP) described in JVET-J0024 and JVET-J0025 is a generalization of bidirectional intra-prediction that uses two opposing reference samples for any direction. Unavailable set of reference samples (secondary reference samples) is generated by taking the available ones (primary reference samples). If DWDIP is enabled, a bi-directional prediction is involved using either two primary reference samples (when both corresponding references belong to available neighbor blocks) or primary and secondary reference samples (otherwise, when one of the references belongs to neighbor blocks that is not available).

Figure 26 illustrates the process of obtaining predicted sample value using distance-weighting. A predicted block generated using DWDIP is smoother in such a direction where there is no reliable information on reference samples. The predicted block is adaptable to the difference between the primary and secondary reference samples along a selected direction.

Figure 26 Distance-weighting process

Secondary reference samples are derived by a weighted sum of linear interpolation between two corner-positioned primary reference samples and directional interpolation from primary reference samples using selected intra prediction mode.

Predicted pixel value is calculated:

Weight  representing distance ration is derived from tabulated values. In the case when primary and secondary reference samples are used, this weight compensates for directional interpolation from primary reference samples using selected intra prediction mode, so that  comprises only linear-interpolated part.

### Bit stream organization

DWDIP flag is signaled after intra prediction mode is parsed. The DWDIP flag is disabled and not signaled for non-directional intra prediction mode and non-medium sized CUs (e.g., for 4x4 or having a length of any side that exceeds 32 pixels).

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 7.2.1 | Distance-Weighted Directional Intra Prediction | Alexey Filippov (Huawei) | Fabien Racape (Technicolor), Jonathan Pfaff (HHI) |

## Tool 3: Linear interpolation intra prediction (JVET-J0017 LGE)

Linear interpolation intra prediction (LIP) based on bidirectional prediction is tested. The tested bidirectional interpolation is based on linear weight between two reference samples newly generated from bottom-right reference samples as well as conventional reference samples. LIP mode is applied only for MPM candidate mode, and LIP flag is signaled to indicate that LIP mode is applied.

### Bit stream organization

Test 1: In this test LIP is performed based on MPM flag following LIP flag signaling.

Test 2: In this test LIP is performed based on LIP flag following MPM flag signaling.

### Test scenarios

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 7.3.1 | LIP based on MPM flag following LIP flag signaling | Jin Heo (LGE) | Alexey Filippov (Huawei) |
| 7.3.2 | LIP based on LIP flag following MPM flag signaling | Jin Heo (LGE) | Alexey Filippov (Huawei) |