| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J1025-v1 |
| --- | --- |

| Title: | Core Experiment 5: Arithmetic Coding Engine |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Tung Nguyen Fraunhofer HHI Amir Said Qualcomm | Email: | tung.nguyen@hhi.fraunhofer.de asaid@qti.qualcomm.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This Core Experiment (CE) investigates the proposed modifications to the binary arithmetic coding engine for the next video coding standard effort by the JVET. The experiments conducted in this CE address different aspects of the entropy coding engine design, including the probability estimation and parameterization, the derivation of the sub-interval range for the LPS, and the context model initialization. Compression efficiency, memory requirement, and computational complexity are addressed in this CE with the latter being either by experimental results or by calculations.

# Introduction

The proposed modifications to the CABAC engine target different aspects of the design. This Core Experiment (CE) categorizes the techniques into three sub-categories when possible. The main category includes proposals that include modifications to almost all aspect of the engine. The first category, Subset A, comprises of modifications to the probability estimation and its parameterization only. The second category, Subset B, includes changes to the derivation of the sub-interval range for the LPS, whereas the third category, Subset C, addresses modifications to the context model initialization. Note that the used categorization approach is in terms of best-effort.

# Participants List

| Participant | Person, Email address | P | C |
| --- | --- | --- | --- |
| HHI | Tung Nguyen (tung.nguyen@hhi.fraunhofer.de) Heiner Kirchhoffer (heiner.kirchhoffer@hhi.fraunhofer.de) Jan Stegemann (jan.stegemann@hhi.fraunhofer.de) | x | x |
| Qualcomm | Amir Said (asaid@qti.qualcomm.com) H. E. Egilmez (hegilmez@qti.qualcomm.com) | x | x |
| DJI Peking University | Xiaozhen Zheng (xiaozhen.zheng@dji.com) Shanshe Wang (sswang@pku.edu.cn) | x | x |
| Samsung | Yinji Piao (yj1003.piao@samsung.com) Kiho Choi (kiho14.choi@samsung.com) | x | x |
| Technicolor | Fabrice Leleannec (fabrice.leleannec@technicolor.com) Ya Chen (ya.chen@technicolor.com) |  |  |
| Sharp | Kiran Misra (misrak@sharplabs.com) |  |  |
| Sony | Karl Sharman (karl.sharman@sony.com) |  |  |
| Mediatek | Tzu-Der Chuang (peter.chuang@mediatek.com) Yuwen Huang (yuwen.huang@mediatek.com) Chih-Wei Hsu (cw.hsu@mediatek.com) Ching-Yeh Chen (chingyeh.chen@mediatek.com) |  |  |
| Nvidia | Ankur Saxena (saxena.ankur0@gmail.com) |  |  |
| Comcast | Dan Grois (groisz@yahoo.com) |  |  |
| Huawei | Jianle Chen (jianle.chen@huawei.com) |  |  |
| Canon | Chris Rosewarne (chris.rosewarne@cisra.canon.com.au) Andrew Dorrell (andrew.dorrell@cisra.canon.com.au) |  |  |
| Hulu | Wenhao Zhang (wenhao.zhang@hulu.com) |  |  |
| Expert | Zhu Li (zhu.li@ieee.org) |  |  |
| Intel | Samuel Wong (samuel.wong@intel.com) |  |  |
| InterDigital | Yan Ye (yan.ye@interdigital.com) Yuwen He (yuwen.he@interdigital.com) Xiaoyu Xiu (xiaoyu.xiu@interdigital.com) Rahul Vanam (rahul.vanam@interdigital.com) |  |  |
| Alibaba | Yuchen Sun (yuchen.s@alibaba-inc.com) |  |  |
| HiSilicon | Yuquan He (yuquanhe@hisilicon.com) |  |  |
| Tencent | Sehoon Yea (sehoonyea@tencent.com) |  |  |
| Arris | Yue Yu (yue.yu@arris.com) Seungwook Hong (seungwook.hong@arris.com) Limin Wang (limin.wang@arris.com) |  |  |

# Description of Experiments and Coding Tools

This CE describes 3 experiments in the main category, 5 experiments in Subset A, 3 experiments in Subset B, and 2 experiments in Subset C, resulting in a total number of experiments equal to 13.

### Experiment 1 (Qualcomm, from JVET-J0021)

The proposed implementation preserves all features of the arithmetic coding main engine used JEM-7.1 software with exception of bin probability estimation, which follows proposal JVET-G0112. As an option submission also includes reduced precision engine described in JVET-E0119.

[TN: probability estimation together with JEM engine, additional engine may be tested in Subset B, need feedback from the proponent]

### Experiment 2 (HHI, from JVET-J0014)

All low-level syntax elements (slice data and below) are coded using context-based adaptive arithmetic coding (CABAC). There are two modifications relative to CABAC in HEVC:

Coding engine: Similar to JEM, the entropy coding stage employs a counter-based probability estimator, which uses two custom window sizes for each context model, initialized at the beginning of each slice. For coding a binary symbol, the counters are converted to the MPS/LPS representation and used to address a 32x8x8-bit range table for the interval subdivision of the M Coder.

Context initialization: Similar to JEM, the probability models of inter-coded slices can be initialized using stored probability models for previous slices. But in contrast to JEM, separate sets of probability models are stored for the individual CTU lines of a picture.

[TN: probability estimator, engine, and context initialization, update TBD.]

### Experiment 3 (Samsung, from JVET-J0024)

Two-probabilities update model / Probability estimation process

[TN: probability estimator, engine, context initialization, clear text description from the proponents for all the three aspects.]

## Subset A: Probability Estimation

## [TN: short description + exact parameterization necessary]

### Experiment A1 (Qualcomm, from JVET-G0112, and JVET-H0061)

Per-context CABAC initialization with double-windows: This approach extends the probability estimation technique in JEM 7 to allow different pairs of window parameters for each coding context. Since those parameters are set only during context initialization, this modification does not affect the encoding and decoding computational complexity.

### Experiment A2 (HHI, from JVET-J0014)

Counter-based probability estimator: Similar to JEM, the entropy coding stage employs a counter-based probability estimator, which uses two custom window sizes for each context model, initialized at the beginning of each slice.

### Experiment A3 (DJI and Peking University, from JVET-J0011)

The basic concept is to propose a new context adaptive counter-based probability estimation scheme.

For the given context, one context counter c is included in the context buffer. The counter c is increased when the given context is used in the CABAC process. Two sliding window parameter a and b is assigned as the following:

If c<Thr,(a1,b1) = (a0,b0)

If c≥Thr, (a1,b1) = (a0+1,b0+1)

Where a0 and b0 are the initialized values. The a1 and b1 are the adjusted value when the proposed scheme is used. Thr is the counter threshold, which can be set as 32, 64, 128 and 512. The default value is set to 512 in J0011.

### Experiment A4 (Samsung, from JVET-J0024)

TBD

## Subset B: Derivation of Sub-Interval Range for LPS

### Experiment B1 (Qualcomm, from JVET-J0019)

The interval subdivision is realized using a smaller look-up table (16×16 range table with 8 bit values). For addressing this look-up table the counts of the probability estimator are converted to the MPS/LPS representation.

### Experiment B2 (HHI, from JVET-J0014):

The interval subdivision is realized using a smaller look-up table (32×8 range table with 8 bit values). For addressing this look-up table the counts of the probability estimator are converted to the MPS/LPS representation.

### Experiment B3 (Samsung, from JVET-J0024)

Probability estimation with double-window and bin counter for changing the window sizes after a threshold is reached.

## Subset C: Context Initialization and Parameterization

### Experiment C1 (Qualcomm, from VCEG-AZ07)

CABAC probability initialization from previous inter frames: In VCEG-AZ07, for inter B and P slices, probability initialization can be made based on a previous B or P slice by using the probability estimate obtained after coding certain number of CTUs. VCEG-AZ07 proposes a location-based method where the probability estimate is obtained after coding CTUs up to the middle of the slice.

### Experiment C2 (HHI, from JVET-J0014)

Separate sets of probability models for the individual CTU lines of a picture: Similar to JEM, the probability models of inter-coded slices can be initialized using stored probability models for previous slices. But in contrast to JEM, separate sets of probability models are stored for the individual CTU lines of a picture.

# Test Conditions

This section describes the test conditions that shall be used for all experiments in this CE.

## Software

NextSoftware x.x shall be used for the experiments, as specified in JVET-XXXX.

## Test Configurations

The test configurations as specified in JVET-XXXX shall be used for all experiments in this CE. Additionally, the experiments shall be run for the Intra High-Efficiency, Intra Low-Complexity, Random Access High-Efficiency, and Random Access Low-Complexity coding configurations.

## Test sequences

The test sequences as described in JVET-XXXX shall be used for the experiments.

# Results Reporting

Proponents and cross-checkers shall report the compression efficiency together with the measured encoder and decoder run times. Additional experimental results are encouraged.

## Compression Efficiency

To obtain objective rate-distortion measurements, four-point BD-rate and BD-PSNR metrics shall be used to evaluate coding efficiency. Rates and PSNR values obtained for all test points shall be provided as well. Compression efficiency, the encoder, and decoder run-time results shall also be reported with the common spreadsheet template that is being used for most JVET contributions. The references for these computations shall be generated from the unmodified version of the NextSoftware used for implementing the proposed techniques.

## Computational complexity

Encoder and decoder run times for the tests in shall be provided as part of the compression efficiency report. In addition to that, proponents are encouraged to provide complexity analysis voluntarily using measures such as arithmetic operations, memory requirements, hardware considerations.

# Timeline

| T1: 2018, May-XX: | Release of the reference software implementation |
| --- | --- |
| T2: 2018, May-XX: | Final CE description uploaded |
| T3: 2018, June-XX: | Proponents release implementation for the experiments |
| T4: 2018, June-XX: | Proponents and cross-checkers report results to the CE coordinators |
| T5: 2018, July-XX: | Input documents uploaded |

Proponent

Cross-Checker