| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J1025-v3 |
| --- | --- |

| Title: | Core Experiment 5: Arithmetic Coding Engine |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Tung Nguyen Fraunhofer HHI Amir Said Qualcomm | Email: | tung.nguyen@hhi.fraunhofer.de asaid@qti.qualcomm.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This Core Experiment (CE) investigates the proposed modifications to the binary arithmetic coding engine for the Versatile Video Coding (VVC) standard effort by the JVET. The experiments conducted in this CE address different aspects of the entropy coding engine design, including the probability estimation and parameterization, the derivation of the sub-interval range for the LPS, and the context model initialization. Compression efficiency, memory requirement, and computational complexity are addressed in this CE with the latter being either by experimental results or by calculations.

# Introduction

The proposed modifications to the CABAC engine target different aspects of the design. This Core Experiment (CE) categorizes the techniques into three sub-categories when possible. The main category covers proposals that include modifications to almost all aspect of the engine. The first category, Subset A, comprises of modifications to the probability estimation and its parameterization only. The second category, Subset B, includes changes to the derivation of the sub-interval range for the LPS, whereas the third category, Subset C, addresses modifications to the context model initialization. Note that the used categorization approach is in terms of best-effort.

# Participants List

| Affiliation | Participants | P | C |
| --- | --- | --- | --- |
| HHI | Heiner Kirchhoffer (heiner.kirchhoffer@hhi.fraunhofer.de) Jan Stegemann (jan.stegemann@hhi.fraunhofer.de) Paul Haase (paul.haase@hhi.fraunhofer.de) | x | x |
| Qualcomm | Amir Said (asaid@qti.qualcomm.com) H. E. Egilmez (hegilmez@qti.qualcomm.com) | x | x |
| DJI Peking University | Xiaozhen Zheng (xiaozhen.zheng@dji.com) Shanshe Wang (sswang@pku.edu.cn) | x | x |
| Samsung | Yinji Piao (yj1003.piao@samsung.com) Kiho Choi (kiho14.choi@samsung.com) Jie Chen (jie1222.chen@samsung.com) | x | x |
| Technicolor | Fabrice Leleannec (fabrice.leleannec@technicolor.com) Ya Chen (ya.chen@technicolor.com) |  |  |
| Sharp | Kiran Misra (misrak@sharplabs.com) |  |  |
| Sony | Karl Sharman (karl.sharman@sony.com) |  |  |
| Mediatek | Tzu-Der Chuang (peter.chuang@mediatek.com) Yuwen Huang (yuwen.huang@mediatek.com) Chih-Wei Hsu (cw.hsu@mediatek.com) Ching-Yeh Chen (chingyeh.chen@mediatek.com) | x | x |
| Nvidia | Ankur Saxena (saxena.ankur0@gmail.com) |  |  |
| Comcast | Dan Grois (groisz@yahoo.com) |  |  |
| Huawei | Jianle Chen (jianle.chen@huawei.com) |  |  |
| Canon | Chris Rosewarne (chris.rosewarne@cisra.canon.com.au) Andrew Dorrell (andrew.dorrell@cisra.canon.com.au) |  |  |
| Hulu | Wenhao Zhang (wenhao.zhang@hulu.com) |  |  |
| Expert | Zhu Li (zhu.li@ieee.org) |  |  |
| Intel | Samuel Wong (samuel.wong@intel.com) |  |  |
| InterDigital | Yan Ye (yan.ye@interdigital.com) Yuwen He (yuwen.he@interdigital.com) Xiaoyu Xiu (xiaoyu.xiu@interdigital.com) Rahul Vanam (rahul.vanam@interdigital.com) |  |  |
| Alibaba | Yuchen Sun (yuchen.s@alibaba-inc.com) |  |  |
| HiSilicon | Yuquan He (yuquanhe@hisilicon.com) |  |  |
| Tencent | Sehoon Yea (sehoonyea@tencent.com) Xin Zhao (xinzzhao@tencent.com) Xiang Li (xlxiangli@tencent.com) |  |  |
| Arris | Yue Yu (yue.yu@arris.com) Seungwook Hong (seungwook.hong@arris.com) Limin Wang (limin.wang@arris.com) |  |  |
| Hikvision | Xiaobo Li (lixiaobo5@hikvision.com) |  | x |

# Description of Experiments and Coding Tools

This CE describes 2 experiments in the main category, 4 experiments in Subset A, 4 experiments in Subset B, and 2 experiments in Subset C, resulting in a total number of experiments equal to 12.

### Experiment CE5.1 (Qualcomm, from JVET-J0021)

Summary: Qualcomm’s response to the Call for Proposal (CFP) includes a modified probability estimator, and a modified entropy coding engine. The proposed implementation preserves all features of the arithmetic coding main engine used JEM-7.1 software with exception of bin probability estimation, which follows proposal JVET-G0112. As an option submission also includes reduced precision engine described in JVET-E0119.

Cross-checkers: Hikvision, HHI

### Experiment CE5.2 (HHI, from JVET-J0014)

Summary: HHI’s response to the CfP includes a modified probability estimator, a modified entropy coding engine, and a modified context initialization.

Coding engine: The entropy coding stage employs a counter-based probability estimator, which uses two custom window sizes for each context model, initialized at the beginning of each slice. During context model initialization the derivation of the binary logarithm of the window sizes nq uses the current slice type and the value of cabac_init_flag. After decoding or encoding a binary symbol xt the counts q=a,b of the probability estimator are updated as follows:

qt+1=qt+2BITSq-qt≫nq,  if xt=1qt-qt≫nq                      ,  if xt=0,

where BITSq defines the number of bits used to represent each counter and is set to BITSq=15 in this experiment. For coding a binary symbol, the counters are weighted ct=at+bt≫1 and the result is used to calculate an index for addressing the table containing the pre-calculated 32x8x8bit range values:

pStateIdx=ct-2(BITSres-1)XORvalMPS-1≫BITSdiff,

where valMPS=ct≫14 and  BITSdiff=9 in this experiment.

Context initialization: Similar to JEM, the probability models of inter-coded slices can be initialized using stored probability models for previous slices. But in contrast to JEM, separate sets of probability models are stored for the individual CTU lines of a picture.

Cross-checkers: Qualcomm

## Subset A: Probability Estimation

### Experiment CE5.A1 (Qualcomm, from JVET-J0021)

Per-context CABAC initialization with double-windows: This approach extends the probability estimation technique in JEM 7 to allow different pairs of window parameters for each coding context. Since those parameters are set only during context initialization, this modification does not affect the encoding and decoding computational complexity.

Cross-checkers: Samsung

### Experiment CE5.A2 (HHI, from JVET-J0014)

Counter-based probability estimator: As described in section CE5.2, the entropy coding stage employs a counter-based probability estimator, which uses two custom window sizes for each context model, initialized at the beginning of each slice. While the binary logarithm of the associated window sizes from experiment CE5.2 are in the range [2,11], inclusive, this experiment contains a subset of the allowed pairs. The number of bits, used to represent the individual counters, is reduced so the memory consumption of each context model does not exceed 32 bits including counters and window size pairs. Therefore, the resolution of each counter is set to BITSq≤15. Before addressing the table containing the pre-calculated range values, the counters q'=a',b' are shifted to the same resolution: q't=qt≫BITSq-BITSmin,  where BITSmin= minBITSa, BITSb.  As a result, the weighting operation is changed to c't=a't+b't≫1 and the table containing the range values is addressed by:

pStateIdx=c't-2(BITSmin-1)XORvalMPS-1≫BITSmin-BITSRTAB,

with valMPS=c't≫(BITSmin-1) and BITSRTAB defining the resolution of the table containing the range values.

Cross-checkers: DJI

### Experiment CE5.A3 (DJI and Peking University, from JVET-J0011)

The basic concept is to propose a new context adaptive counter-based probability estimation scheme whose initialization takes place once per slice.

For the given context, one context counter c is included in the context buffer. The counter c is increased when the given context is used in the CABAC process. Two sliding window parameter a and b is assigned as the following:

If c<Thr,(a1,b1) = (a0,b0)

If c≥Thr, (a1,b1) = (a0+1,b0+1)

Where a0 and b0 are the initialized values. The a1 and b1 are the adjusted value when the proposed scheme is used. Thr is the counter threshold, which can be set as 32, 64, 128 and 512. The default value is set to 512 in J0011.

Cross-checkers: Qualcomm

### Experiment CE5.A4 (Samsung, from JVET-J0024)

Similar to JEM, two-probability update model is employed in J0024. The probability is updated with linear combination for probability estimation that consists of only two summands corresponding N0=16 (short window) and N1=128 (long window):

P0 = (Y >> 4) + P0 – (P0 >> 4)

P1 = (Y >> 7) + P1 – ( P1>> 7)

P = (P0 + P1 + 1) >> 1

where Y = 2^15.

To start probability estimation it is necessary to initialize probability. Different from JEM, only short window size model is updated until the counter reaches 31. When the counter reaches 31, long window model starts updating with initial value borrowed from short window model. This approach significantly reduces negative effect from wrong initialization and also reduces the complexity due to less update of long window size model.

The probability estimation process is as follows,

- bFastProbUpdate = counter < 31

- counter++

- MPS pass:

  - P0mps = P0 – (P0 >> 4)

  - P1mps = bFastProbUpdate ? P0mps: P1 – (P1 >> 7)

- LPS pass

  - P0lps = P0 + ((2^15 – P0) >> 4)

  - P1lps = bFastProbUpdate ? P0lps : P1 + ((2^15 – P1) >> 7)

  - If(P0 + P1 > 2^15)

    - mps = 1 – mps

    - P1lps = 2^15 – P1lps

    - P0lps = 2^15 – P0lps

- P= (P0 + P1) >> 1

Cross-checkers: HHI

Subset B: Derivation of Sub-Interval Range for LPS

### Experiment CE5.B1 (Qualcomm, from JVET-E0119)

The interval subdivision is realized using a smaller look-up table (16×16 range table with 8 bit values). For addressing this look-up table the counts of the probability estimator are converted to the MPS/LPS representation.

Cross-checkers: HHI

### Experiment CE5.B2 (HHI, from JVET-J0014):

In JEM, the interval subdivision is realized using a look-up table containing 32768x9bit values (512x64 table dimensions). These values cover the whole probability range (0,1). Instead, JVET-J0014 uses a smaller look-up table containing 256x8bit values (32×8 table dimensions) which covers the LPS range (0,0.5]. Therefore, the counters of the probability estimator are converted to the MPS/LPS representation, as described in sections CE5.2 and CE5.A2, before the table is addressed.

Cross-checkers: MediaTek

### Experiment CE5.B3 (MediaTek, from JVET-J0018)

The interval subdivision is realized using a smaller look-up table (32×8 range table with 8-bit values). The range interval is divided into the rangeOne and the rangeZero. The rangeOne and the rangeZero derivation is modified as follows.

probLPS   = ( P >= 16384 ) ? 32767 – P : P

rangeLPS  = LUT[ probLPS >> 9 ][ ( range >> 5 ) & 7 ]

rangeOne  = ( P >= 16384 ) ? range – rangeLPS : rangeLPS

rangeZero = ( P >= 16384 ) ? rangeLPS : range – rangeLPS

where the P is the probability of a context and the range is the current range in binary arithmetic coder (BAC).

Cross-checker: Qualcomm

### Experiment CE5.B4 (MediaTek, from JVET-J0018)

The interval subdivision is realized using a multiplier. The range interval is divided into the rangeOne and the rangeZero. The rangeOne and the rangeZero derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 64 – (P>>M) : 1+ (P>>M)

rangeLPS  = (probIdx * ( range >> N )) >> K

rangeOne  = ( P >= 16384 ) ? range – rangeLPS : rangeLPS

rangeZero = ( P >= 16384 ) ? rangeLPS : range – rangeLPS

where the P is the probability of a context and the range is the current range in binary arithmetic coder (BAC). M = 9, N = 5, K = 1.

Cross-checker: Qualcomm

## Subset C: Context Initialization and Parameterization

### Experiment CE5.C1 (Qualcomm, from JVET-J0021)

CABAC probability initialization from previous inter frames: In JVET-J0021, for inter B and P slices, probability initialization can be made based on a previous B or P slice by using the probability estimate obtained after coding certain number of CTUs. JVET-J0021 proposes a location-based method where the probability estimate is obtained after coding CTUs up to a pre-defined position in the slice.

Cross-checker: HHI

### Experiment CE5.C2 (HHI, from JVET-J0014)

Separate sets of probability models for the individual CTU lines of a picture: Similar to JEM, the probability models of inter-coded slices can be initialized using stored probability models for previous slices. But in contrast to JEM, separate sets of probability models are stored for the individual CTU lines of a picture.

Cross-checker: Qualcomm

# Test Conditions

This section describes the test conditions that shall be used for all experiments in this CE.

## Software

VTM 1.0 and BMS 1.0 shall be used for the experiments, as described in JVET-J1010. To minimize the working load, the proponents shall implement their coding tools into the BMS software.

## Test Configurations

The test configurations as specified in JVET-J1010 shall be used for all experiments in this CE. Additional test configurations reflecting the relevant operation points are encouraged, and the following special case should be considered: Proposals introducing modifications that include changes to context probability initialization tables shall also report results with the context probability initialization tables in the BMS. For the VTM environment, the proponents and cross-checkers shall use the BMS software together with the provided VTM configuration files so that the coding tool configurations are aligned with the VTM software setup.

## Test sequences

The test sequences as described in JVET-J1010 shall be used for the experiments.

# Results Reporting

Proponents and cross-checkers shall report the compression efficiency together with the measured encoder and decoder run times. Additional experimental results are encouraged.

## Compression Efficiency

To obtain objective rate-distortion measurements, four-point BD-rate and BD-PSNR metrics shall be used to evaluate coding efficiency. Rates and PSNR values obtained for all test points shall be provided as well. Compression efficiency, the encoder, and decoder run-time results shall also be reported with the common spreadsheet template that is being used for most JVET contributions. The references for these computations shall be generated from the unmodified version of the VTM used for implementing the proposed techniques.

## Computational complexity

Encoder and decoder run times for the tests in shall be provided as part of the compression efficiency report. In addition to that, proponents are encouraged to provide complexity analysis voluntarily using measures such as arithmetic operations, memory requirements, hardware considerations.

Proponents shall report amount of memory used:

- Per-context state (RAM): State is the information that may change after a bin is encoded/decoded.

- Per-context initializers (ROM): e.g., table-based probability and window initializations.

- Per-slice state initialization (RAM): e.g., temporal initialization updates.

# Timeline

General timeline for core experiments (as agreed by JVET).

| T1 | May 15, 2018 | - Final CE description uploaded |
| --- | --- | --- |
| T2 | June 1, 2018 | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | June 19, 2018 | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

Proponent

Cross-Checker