| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: San Diego, US, 10–20 Apr. 2018 | Document: JVET-J1026-v3 |
| --- | --- |

| Title: | Description of Core Experiment 6: Transforms and transform signalling |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Amir Said, Xin Zhao | Email: | asaid@qti.qualcomm.com xinzzhao@tencent.com |
| Source: | CE coordinators |  |  |

____________________________

# Introduction

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signaling on top of the Test Model. In this CE, the tradeoff between compression efficiency and complexity of different transformation tools will be investigated. In addition to encoder and decoder run-times, other complexity metrics, e.g., arithmetic operation counts, storage, transform core bit-precision and internal bit-precision will be also evaluated. The list of tools and experiments to be performed are detailed in this document.

# Participant list

| Name | Affiliation | Email | P/C |
| --- | --- | --- | --- |
| Amir Said Hilmi E. Egilmez Yung-Hsuan Chao Vadim Seregin Ted Hsieh | Qualcomm | asaid@qti.qualcomm.com hegilmez@qti.qualcomm.com yunghsua@qti.qualcomm.com vseregin@qti.qualcomm.com thsieh@qti.qualcomm.com | P/C |
| Xin Zhao Xiang Li Liang Zhao | Tencent | xinzzhao@tencent.com xlxiangli@tencent.com leolzhao@tencent.com | P/C |
| Mischa Siekmann Heiko Schwarz Santiago de Luxán Hernández | HHI | mischa.siekmann@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de santiago.de.luxan@hhi.fraunhofer.de | P/C |
| Pierrick Philippe | Orange | pierrick.philippe@orange.com | C |
| Fabrice Le Léannec Karam Naser Eduard Francois | Technicolor | fabrice.leleannec@technicolor.com karam.naser@technicolor.com Edouard.Francois@technicolor.com | P/C |
| Kiho Choi | Samsung | kiho14.choi@samsung.com | P/C |
| Kiyofumi Abe Kanoh Ryuichi | Panasonic | abe.kiyo@jp.panasonic.com kanoh.ryuichi@jp.panasonic.com | P/C |
| Jungwon Kang Sung-Chang Lim | ETRI | jungwon@etri.re.kr sclim@etri.re.kr | P/C |
| Yung-Lyul Lee Nam-Uk Kim | Sejong Univ. | yllee@sejong.ac.kr nukim@sju.ac.kr | P/C |
| Takeshi Tsukuba Masaru Ikeda | Sony | takeshi.tsukuba@sony.com masaru.ikeda@sony.com | P/C |
| Kei Kawamura Yoshitaka Kidani Sei Naito | KDDI | ki-kawamura@kddi.com yo-kidani@kddi.com se-naitou@kddi.com | P/C |
| Chih-Wei Hsu Yu-Wen Huang Ching-Yeh Chen Han Huang Tzu-Der Chuang Man-Shu Chiang Shih-Ta Hsiang | MediaTek | cw.hsu@mediatek.com yuwen.huang@mediatek.com chingyeh.chen@mediatek.com h.huang@mediatek.com peter.chuang@mediatek.com man-shu.chiang@mediatek.com shih-ta.hsiang@mediatek.com | P/C |
| Moonmo Koo Mehdi Salehifar Seung Hwan Kim Jaehyun Lim | LG Electronics | moonmo.koo@lge.com mehdi.salehifar@lge.com seunghwan3.kim@lge.com jaehyun.lim@lge.com | P/C |
| Yin Zhao Jianle Chen Haitao Yang Sergey Ikonin Alexey Filippov Vasily Rufitskiy Alexander Karabutov | Huawei | yin.zhao@huawei.com jianle.chen@huawei.com haitao.yang@huawei.com Sergey.Ikonin@huawei.com Alexey.Filippov@huawei.com Vasily.Rufitskiy@huawei.com karabutov.alexander@huawei.com | P/C |
| Yongbing Lin Quanhe Yu Jianhua Zheng Xuguang Zuo | HiSilicon | linyongbing@hisilicon.com yuquanhe@hisilicon.com zhengjianhua@hisilicon.com zuoxuguang@hikvision.com | P/C |
| Yukinobu Yasugi Takeshi Chujoh Tomohiro Ikai | Sharp | yasugi.yukinobu@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |  |
| Kiran Misra | Sharp Labs | misrak@sharplabs.com |  |
| Yuriy Reznik | Brightcove | yreznik@brightcove.com |  |
| Yu-Chen Sun | Alibaba | yuchen.s@alibaba-inc.com |  |
| Yue Yu Seungwook Hong Limin Wang | Arris | Yue.Yu@arris.com Seungwook.Hong@arris.com Limin.Wang@arris.com |  |
| Minhua Zhou Peisong Chen Wade Wan | Broadcom | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |  |
| Chris Rosewarne Andrew Dorrell | Canon | Chris.Rosewarne@cisra.canon.com.au Andrew.Dorrell@cisra.canon.com.au |  |
| Dan Grois | Comcast | groisz@yahoo.com |  |
| Pankaj Topiwala | FastVDO | pankajtva@gmail.com |  |
| Hari Kalva | Florida Atlantic Univ. | hari.kalva@fau.edu |  |
| Yan Ye Yuwen He Xiaoyu Xiu Rahul Vanam | InterDigital | yan.ye@interdigital.com yuwen.he@interdigital.com xiaoyu.xiu@interdigital.com rahul.vanam@interdigital.com |  |
| Samuel Wong | Intel | samuel.wong@intel.com |  |
| Andrey Norkin | Netflix | anorkin@netflix.com |  |
| Shunsuke Iwamura | NHK | iwamura.s-gc@nhk.or.jp |  |
| Ankur Saxena | Nvidia | ankur81saxena@gmail.com |  |
| Byeungwoo Jeon | SKKU | bjeon@skku.edu |  |
| Benben Niu Liqiang Wang | Tsinghua Univ. | nbb16@mails.tsinghua.edu.cn wlq15@mails.tsinghua.edu.cn |  |
| John Son | WILUS Inc. | john.son@wilusgroup.com |  |
| D.Y. Kim | Chips & Media | brad.kim@chipsnmedia.com | P/C |
| Victorien Lorcy | b<>com | victorien.lorcy@b-com.com | C |
| John Son | Wilus | john.son@wilusgroup.com | C |
| Yao-Jen Chang | Foxconn | yaojen.chang.foxconn@gmail.com |  |

# Core Experiment tasks

The following tasks shall be performed in this Core Experiment (CE):

- Evaluate compression efficiency, computational complexity and memory of transforms studied in this CE;

- Investigate interactions between primary and secondary transform schemes;

- Report the results and conclusions of these experiments.

Core experiments (CEs) are organized according to the following three categories:

- Primary transformation: CEs 6.1.1-6.1.14 test proposals that introduce changes for primary transforms only.

- Secondary transformation: CEs 6.2.1-6.2.7 test proposals that introduce changes for secondary transforms only.

- Combined primary and secondary transformation: CEs 6.3.1-6.3.3 test proposals that introduce changes for both primary and secondary transforms.

Additional requirements for core experiments in each category are listed in Section 5.3.

# Description of tools

The table below summarizes the tools to be considered in this CE and is followed by additional information for each case.

| CE 6.1 – Primary Transforms |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.1.1 | A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm) | JVET-J0021 (CfP) JVET-C0022 JVET-D0126 | Primary transformation: In addition to transform types of AMT in JEM7, DST-4 and Identity transform are also used for intra prediction residual. For inter prediction residual, KLT-1 and KLT-2 are used to replace AMT for small block sizes. | 1. Orange 2. Samsung 3. Brightcove 4. Wilus |
| 6.1.2 | A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm) | JVET-J0066 | AMT complexity reduction: The AMT transforms are implemented using the DCT-2 family and adjustment stages. | 1. Orange 2. Technicolor 3. ETRI 4. Samsung 5. Sony 6. Tencent 7. Brightcove |
| 6.1.3 | Technicolor | JVET-J0022 (CfP) | Transform Reduction: It is proposed to reduce transform types in AMT to DCT-8, DST-4 and DST-7. DST-7 is implemented based on the DCT-8 with flipping and sign changes. | 1. Brightcove 2. FastVDO |
| 6.1.4 | ETRI/Sejong Univ. | JVET-J0013 (CfP) | Residual flipping: It is proposed to replace the AMT with DCT-II and DST-VII using four types of residual flipping. | 1. Qualcomm 2. 3. |
| 6.1.5 | Samsung | JVET-J0024, JVET-J0025 | Reduced AMT transform types: The AMT transforms in JEM7 are replaced with a reduced set of transforms using DCT-8 and DST-7. | 1. Qualcomm 2. Brightcove 3. |
| 6.1.6 | Samsung | JVET-J0024, JVET-J0025 | Primary transformation restriction: When block width or height is over 32 pixels, AMT is not used or signaled. | 1. Qualcomm 2. Tencent 3. |
| 6.1.7 | T. Tsukuba (Sony) | JVET-J0028 | Adaptive Multiple core Transforms for Luma and Chroma: It is proposed to apply AMT, which includes transform matrix replacement, for both luma and chroma. | 1.Tencent 2. 3. |
| 6.1.8 | T. Tsukuba (Sony) | JVET-J0028 | Transform Matrix Replacement: It is proposed to replace transform matrix of DCT-8, DST-1 and DCT-5 used in JEM with that of alternate transforms as below: - Flipped DST-7, which is derived by flipping DST-7 without sign changes, is used instead of DCT8. - DST-6, which is derived by transposing DST-7, is used instead of DST-1. DCT-2 is used instead of DCT-5. | 1.Brightcove 2. Sejong Univ. 3. |
| 6.1.9 | KDDI | JVET-J0016 | Shrink Transform: When the transform block size is equal or larger than 64, inverse transform is realized by two steps: inverse transform with half size and up-sampling. | 1. MediaTek 2. Panasonic 3. |
| 6.1.10 | MediaTek | JVET-J0018 | Adaptive multi-transform (AMT) with constraints: AMT transform type is reduced to DCT-II, DST-VII and flipped DST-VII. AMT is only allowed when both CU width and height are smaller than or equal to 64. For some block sizes, the number of AMT candidates is reduced. | 1.Brightcove 2. 3. |
| 6.1.11 | LG Electronics | JVET-J0017(CfP) | Primary transform: Only DST7 and DCT8 are used for AMT. AMT is applied up to 32. DST7 and DCT8 are implemented using FFT | 1. Brightcove |
| 6.1.12 | Huawei | JVET-J0024 | Spatial varying transform: For an inter-predicted CU, a TU of half size of the CU may be used, and in this case the TU shape and position information are signaled. | 1. MediaTek 2. bcom |
| 6.1.13 | Y. Lin, M. Mao, S. Song, J. Zheng, J. An, C. Zhu, (HiSilicon) | JVET-J0064 | Prediction dependent Transform - Intra prediction mode dependent transform for luma and chroma Position dependent transform for residual of HEVC merge mode | 1. Brightcove |
| 6.1.14 | X. Zhao, X. Li, S. Liu (Tencent) | JVET-J0029 (CfP) | Block size dependent zero-out transform: When 64-point, 128-point and 256-point inverse primary transform is applied, only coefficients of the first 32, 16 and 4 are used and the remaining coefficients are not used and considered as 0. | 1. Sony |
| CE 6.2 – Secondary Transforms |  |  |  |  |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.2.1 | Technicolor | JVET-J0021 (CfP) JVET-D0120 | Secondary transformation: NSST can be applied only when AMT transform flag or index is equal to 0. NSST transform choice is separately signaled for Cb and Cr chroma components. | 1. Samsung 2. HHI 3. |
| 6.2.2 | A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm) | JVET-J0062 | Hierarchically Structured Matrix-based Transforms: HyGT based NSST is replaced by hierarchically structured matrix-based transforms (HSMTs). | 1. Technicolor 2. Samsung 3. HHI 4. Tencent 5. Brightcove 6. bcom |
| 6.2.3 | Samsung | JVET-J0024, JVET-J0025 | Secondary transformation: NSST is modified with reduced sets based on grouping intra prediction modes and signaling with on/off flag with 4 modes. | 1. Qualcomm 2. HHI 3. Tencent 4. |
| 6.2.4 | X. Zhao, X. Li, S. Liu (Tencent) | JVET-J0029 (CfP) | Matrix multiplication based NSST: It is proposed to use matrix multiplication based NSST. | 1. LG Electronics 2. |
| 6.2.5 | LG Electronics | JVET-J0017(CfP) | 4x4 Secondary transform: 4x4 Layered Givens Transform (LGT) is tested. | 1. Brightcove |
| 6.2.6 | LG Electronics | JVET-J0017(CfP) | 8x8 Secondary transform: 8x8 Reduced Secondary Transform (RST) keeping 16 bases of the 64-dimensional space (16x64 matrix) with NSST index modification based on RST modification. | 1. Huawei 2. bcom |
| CE 6.3 – Combinations and Signalling |  |  |  |  |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.3.1 | X. Zhao, X. Li, S. Liu (Tencent) | JVET-J0054 | Coupled primary and secondary transform: AMT and NSST is coupled and signaled by only one transform index and applied for both luma and chroma components. | 1. Technicolor 2. Samsung 3. Sony 4. Brightcove |
| 6.3.2 | HHI | JVET-J0014, JVET-J0040 | Set of transforms: A set of 5 predefined transform candidates, and each candidate specifies a primary horizontal transform, a primary vertical transform, and a non-separable secondary transform. The transform candidates depend on the block size and the prediction mode. | 1. Qualcomm 2.Brightcove 3. |
| 6.3.3 | Panasonic | JVET-J0020 | AMT and NSST complexity reduction: 1) NSST is used only when DCT2 is used as primary transform. 2) DST7 is used as the horizontal and vertical transform without signaling when width or height is less than or equal to 4. | 1. KDDI 2. 3. |
| 6.3.4 | MediaTek | JVET-J0018 | AMT and NSST syntax signaling: 1) The NSST index signaling depends on luma/chroma, number of nonzero DC/AC coefficient of upper-left 8x8 or 4x4. | 1. Bcom |

Detailed description on each coding tool tested in this CE are provided below.

## Core Experiments on Primary Transformation

### CE 6.1.1: Extended AMT Transforms [JVET-J0021 (CfP), JVET-C0022, JVET D0126]

The AMT transforms in JEM7 are replaced with a new set of transforms. In addition to transforms used in JEM, which are DCT-2, DCT-5, DCT-8, DST-1, and DST-7, two more cores are added for each intra and inter coded blocks. Namely, DST-4 and identity transform are added to intra, KLT-1 and KLT-2 are added to inter. It was proposed in JVET-C0022 and JVET-D0126, respectively.

To accommodate more flexible partitioning, where a block size is not power of 2, such as 4x24 and 8x48, corresponding transform cores are included.

In intra coding, the following 7 transform sets, where each set has 4 transform pairs, are defined:

T0, intra = { (DST-4, DST-4), (DST-7, DST-7), (DST-4, DCT-8), (DCT-8, DST-4) }

T1, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-5, DST-7), (DST-1, DCT-5) }

T2, intra = { (DST-7, DST-7), (DST-7, DCT-8), (DCT-8, DST-7), (DCT-5, DCT-5) }

T3, intra = { (DST-4, DST-4), (DST-4, DCT-5), (DCT-8, DST-4), (DST-1, DST-7) }

T4, intra = { (DST-4, DST-7), (DST-7, DCT-5), (DCT-8, DST-7), (DST-1, DST-7) }

T5, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-8, DST-7), (DST-1, DST-7) }

T6, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-5, DST-7), (DST-1, DST-7) }.

The first entry in the transform pair determines vertical transform, and the second entry specifies the horizontal transform.

A look-up table is defined to assign each of the 7 transform sets based on different intra prediction modes and block sizes.

Identity transform is applied for blocks that are not exceeding 16x16 and have intra modes within the proximity of horizontal and vertical intra directions, the proximity is defined by a block size-based threshold. If transform index is equal to 3 and block satisfies the above condition, the horizontal and/or vertical identity transform is applied.

In inter coding, the following 2 transform sets are used, where each set has 4 transform pairs defined as follows:

T0, inter = { (DCT-8, DCT-8), (DCT-8, DST-7), (DST-7, DCT-8), (DST-7, DST-7) }

T1, inter = { (KLT-1, KLT-1), (KLT-1, KLT-2), (KLT-2, KLT-1), (KLT-2, KLT-2) }.

The transform set with KLTs (i.e., T1, inter) is used for blocks that are smaller than or equal to 16x16, and for larger blocks the transforms from T0, inter set are used.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.1.1 | Extended AMT Transforms (Intra only) | Qualcomm | Orange Samsung Brightcove Wilus |
| 6.1.1.2 | Extended AMT Transforms (Inter only) | Qualcomm | Orange Samsung Brightcove Wilus |

### CE 6.1.2: AMT complexity reduction [JVET-J0066]

This is a technique proposed to reduce the complexity of AMT by providing an efficient alternative that approximates the AMT transforms using only a transform of the DCT-2 family and adjustment stages of low complexity. The proposed adjustment stages are defined using sparse block-band orthogonal matrices, which provide good approximation for the set of AMTs used in JEM7.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.2.1 | AMT complexity reduction | Qualcomm | Orange Technicolor ETRI Samsung Sony Tencent Brightcove |

### CE 6.1.3: Transform Reduction [JVET-J0022 (CfP)]

In JVET-J022, it is proposed to reduced the set of primary transforms to DCT-2, DCT-8, DST-4, DST-7. The AMT set thus is reduced to DCT-8, DST-4 and DST-7. Moreover, DST-7 is implemented based on the DCT-8 transform matrix, on which flipping, and sign changes operations are applied.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.3.1 | Transform Reduction | Technicolor | Brightcove FastVDO |

This transform design is a lowered complexity version of the design in JVET-J021. For inter coding, the set of 2D transforms (horizontal and vertical transform) are unchanged. However, for intra coding, the following design of 2D transforms is used:

| TrIdx predModIdx | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 0 | DST4,DST4 | DST7,DST7 | DST4,DCT8 | DCT8,DST4 |
| 1 | DST7,DST7 | DST7,DCT2 | DCT2,DST7 | DCT2,DCT8 |
| 2 | DST7,DST7 | DST7,DCT8 | DCT8,DST7 | DCT2,DST7 |
| 3 | DST4,DST4 | DST4,DCT2 | DCT8,DST4 | DCT2,DST7 |
| 4 | DST4,DST7 | DST7,DCT2 | DCT8,DST7 | DCT2,DST7 |
| 5 | DST7,DST7 | DST7,DCT2 | DCT8,DST7 | DCT2,DST7 |
| 6 | DST7,DST7 | DST7,DCT2 | DCT2,DST7 | DCT2,DST7 |

Table 1: The distribution of vertical and horizontal transform, for each prediction mode and transform index.

### CE 6.1.4: Transform Replacement [JVET-J0013 (CfP)]

It is proposed to replace the adaptive multiple transform (AMT) with DCT-II and DST-VII with four types of residual flipping in order to reduce the number of transform matrix compared to AMT. The four types of residual flipping candidate are as follows: No-Flip (no flipping), H-Flip (horizontal flipping), V-Flip (vertical flipping), HV-Flip (horizontal & vertical flipping). For intra coding, one of the residual flipping candidates in the pre-defined residual flipping candidate set for the given intra prediction mode is selected based on the signalled index. For inter coding, one of four residual flipping candidates is selected based on the signalled index. While four additional transforms are needed for AMT as primary transformation, one additional transform and residual flipping are needed in the proposed method. On residual flipping, inverse residual flipping can be implemented by simply reordering the output of inverse DST-VII transform without any additional process.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.4.1 | Residual flipping | ETRI/Sejong Univ. | Qualcomm |

### CE 6.1.5: Transform Replacement [JVET-J0024, JVET-J0025]

The AMT transforms in JEM7 are replaced with a reduced set of transforms using DCT-8 and DST-7.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.5.1 | Reduced AMT transform types | Samsung | Qualcomm Brightcove |

### CE 6.1.6: Transform Restriction [JVET-J0024, JVET-J0025]

For AMT transforms, if width or height is over 32 pixels in length, the block is not considered to be applied for AMT and not signaled to decoder

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.6.1 | Primary transform restriction | Samsung | Qualcomm Tencent |

### CE 6.1.7: AMT for Luma and Chroma [JVET-J0028]

It is proposed to apply AMT, which includes transform matrix replacement described in 4.1.8, for both luma and chroma.

A transform type for luma is determined based on a mode-dependent transform set and an index explicitlyexplicitly signaled, which indicates horizontal and vertical transform.

For chroma/intra case, a transform type is determined in the same way as luma, but the number of transform candidates is reduced than luma’s.

For chroma/inter case, a transform type is determined by one bit flag indicating whether the index is the same as in the collocated luma block or default transform type (DCT-2xDC-T2).

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.7.1 | Chroma AMT | Sony | Tencent |

### CE 6.1.8: Transform Replacement [JVET-J0028]

It is proposed to replace transform matrix of DCT-8, DST-1 and DCT-5 used in JEM with that of alternate transforms as below:

- Flipped DST-7 is used instead of DCT-8.

- DST-6 is used instead of DST-1.

DCT-2 is used instead of DCT-5.

Transform matrices of Flipped DST-7 and DST-6 are derived from DST-7:

Flipped DST-7:  FlipSN7[l,k]=SN7[l, N-k-1]   - (A1)

DST-6:  SN6[l,k]=SN7[k,l]    - (A2)

, where SN7[k,l] represents the l-th element of the k-th basis vector in NxN transform matrix of DST-7.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.8.1 | Transform Matrix Replacement | Sony | Brightcove |

### CE 6.1.9: Shrink Transform [JVET-J0016]

When the transform block size is equal or larger than 64, inverse transform is realized by two steps; inverse transform with half size and up-sampling. The up-sampling ratio is twice with DCT based coefficient, which is the same as in SHVC. Otherwise (the transform block size is less than 64), inverse transform is the same as in the test model. The following aspects are required to evaluate the proposal. Maximum transform size is configured to 64x64 and 128x128. QP range is set as normal (22, 27, 32, and 37) and lower (like 12 or 17) to confirm the effect by the upsampling filter. Proposal is turned on and off. Complexity in hardware implementation or software implementation (SIMD implementation) is also required to be studied.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.9.1 | Shrink Transform | KDDI | MediaTek Panasonic |

### CE 6.1.10: Adaptive multi-transform (AMT) with speed-up [JVET-J0018]

In J0018, the adaptive multi-core transform (AMT) can select the horizontal transform and vertical transform from DCT-II, DST-VII and flipped DST-VII (FDST-VII). An AMT flag is defined. AMT flag equal to “0” indicates that DCT-II is used for both horizontal transform and vertical transform, AMT flag equal to “1” indicates that another transform type is used depending on an indicator, AMT index. In J0018, AMT is only allowed when both CU width and height are smaller than or equal to 64. AMT flag can be indicated by the intra prediction modes. Even Intra prediction modes are implicitly assigned with AMT flag 1 and odd Intra prediction modes are implicitly assigned with AMT flag 0. When AMT flag is equal to 1, different numbers of AMT modes will be tested according to the CU size.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.10.1 | Proposed AMT set | MediaTek | Brightcove |
| 6.1.10.2 | Proposed AMT set with complexity reduction | MediaTek |  |

### CE 6.1.11: AMT replacement and restriction [JVET-J0017(CfP)]

For core transform, simplified Adaptive multiple core transform is utilized by adding two transforms (DST7 and DCT8) on top of HEVC, and maximum AMT is restricted to a size of 32. A forward N×N DST7 with 2N + 1 length Discrete Fourier Transform (DFT) is implemented to achieve the N×N DST7. 2N+1 FFT has been reformulated to a two dimensional FFT. DCT8 can be derived from DST7 with sign changes and reordering just before and after the DST7 computation. Hence, DST7 is reused in this implementation for DCT8.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.11.1 | AMT using only DST7 and DCT8 | LG Electronics |  |

### CE 6.1.12: Spatial varying transform[JVET-J0024]

For an inter-predicted CU with cbf_Y equal to 1, a svt_flag is signaled to indicate whether the whole residual block or a subpart of the residual block is decoded. The latter case is called a spatial varying transform (SVT), in which a part of the residual block is coded and the rest of the residual block is forced to be zero.

When SVT is used for a CU, SVT type and SVT position information are further decoded from the bitstream. There are two SVT types and three SVT positions, as indicated in Fig 1. For SVT-V (or SVT-H), the TU width (or height) equals to half of the CU width (or height), and three TU positions are equally spaced as (i/4) of the CU width (or height) to the top-left corner of the CU, where i=0, 1 and 2. The three positions are hence denoted as position 0, 1 and 2.

Both TU and CU boundaries are filtered in de-blocking filtering. SVT-V (or SVTH) is allowed when CU width (or height) is in the range of [8, 64]. To ensure the gap between two filtered boundaries are not smaller than 4 pixels, Position 1 of SVT-V (or SVT-H) is not allowed when CU width (or height) equals to 8.

Position-dependent core transform is applied in SVT. The three positions are associated with different core transforms. More specifically, DCT-8 is used for position 0 while DST-7 is used for position 1 and 2.

Fig. 1 Two types of SVT TU

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.12.1 | Spatially varying transform | Huawei | MediaTek bcom |

### CE 6.1.13: prediction dependent transforms [JVET-J0064]

An implicit transform solution is used for Intra and Inter frame coding. The following 3 aspects will be tested in the CE

- Intra prediction mode dependent transform for luma and chroma

The proposed Intra mode dependent transform mapping tables are shown in the following tables.

Table 1a. Transform mapping table for luma residual

| Intra prediction mode | Horizontal T | Vertical T | Block Size constraint |
| --- | --- | --- | --- |
| Planar Ang. 31,32,34,36,37 | DST-7 | DST-7 | Width <= 64 && Height <= 64 |
| DC Ang. 33, 35 | DCT-2 | DCT-2 | Width <= 64 && Height <= 64 |
| Ang. 2, 4, 6…28,30 Ang. 39,41,43…63,65 | DST-7 | DCT-2 | Width <= 64 && Height <= 64 |
| Ang. 3,5,7…27,29 Ang. 38,40,42…64,66 | DCT-2 | DST-7 | Width <= 64 && Height <= 64 |

Table 1b. Transform mapping table for chroma residual

| Intra prediction mode | Horizontal T | Vertical T | Block Size constraint |
| --- | --- | --- | --- |
| LM modes | DST-7 | DST-7 | Width <= 8 && Height <= 8 |
| Planar | DST-7 | DST-7 | Width <= 16 && Height <= 16 |
| Hor | DST-7 | DCT-2 | Width <= 16 && Height <= 32 |
| Ver, VDIA | DCT-2 | DST-7 | Width <= 32 && Height <= 16 |

- Position dependent transform for residual of HEVC merge mode

Transform for HEVC merge mode residual is dependent on position of the spatial MVP candidate used for MCP of current block.

As shown in Table 2, for left MVP candidate, DST-7 and DCT-2 are used as horizontal and vertical transforms respectively. For above MVP candidate, DCT-2 and DST-7 are used as horizontal and vertical transforms respectively. For other cases, DCT-2 is used as default transform.

Table 2 Mapping between MVP position and transform

| MVP position | Horizontal T | Vertical T | Block Size constraint |
| --- | --- | --- | --- |
| L (left) | DST-7 | DCT-2 | Width <= 32 && Height <= 32 |
| A (above) | DCT-2 | DST-7 | Width <= 32 && Height <= 32 |

- DST-7 transform for residual of template matching mode.

This test should be deleted since there is no template matching mode in both TM and BMS reference software.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.13.1a | Intra prediction mode dependent transform for luma only (Table 1a) | HiSilicon | Brightcove |
| 6.1.13.1b | Intra prediction mode dependent transform for chroma only (Table 1b) | HiSilicon | Brightcove |
| 6.1.13.1c | Intra prediction mode dependent transform for luma and chroma (Table 1a + 1b) | HiSilicon | Brightcove |
| 6.1.13.2 | Position dependent transform (Table 2) | HiSilicon | Brightcove |
| 6.1.13.3 | Combination of Intra prediction mode dependent transform and position dependent transform (Table 1a+ 1b + Table 2 ) | HiSilicon | Brightcove |

### CE 6.1.14: Block size dependent zero-out transform [JVET-J0029]

When 64-point, 128-point and 256-point inverse primary horizontal transform is applied, only coefficients of the first 32, 16 and 4 columns are used and the remaining coefficients are not used and considered as 0. When 64-point, 128-point and 256-point inverse primary vertical transform is applied, only coefficients of the first 32, 16 and 4 rows are used and the remaining are not used and considered as 0.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.14.1 | Block size dependent zero-out transform | Tencent | Sony |

## Core Experiments on Secondary Transformation

### CE 6.2.1: Secondary Transformation: NSST Signaling [JVET-J0021 (CfP), JVET-D0120]

The non-separable secondary transform (NSST) is the same as in JEM7, including the JEM NSST signaling, with the following three modifications:

- To support additional intra modes, which can be up to 131 directions in JVET-J0021, the look-up table that maps intra mode to NSST transform was extended, where each entry in the table was simply duplicated.

- To lower the complexity, the restriction is imposed on AMT and NSST use that NSST can be applied and signaled only when AMT transform index is equal to 0, it also happens when AMT flag is equal to 0.

- NSST transform choice is separately signaled for Cb and Cr chroma components.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.1.1 | Conditional NSST signaling | Technicolor | Samsung HHI |

### CE 6.2.2: Hierarchically Structured Matrix-based Transforms [JVET-J0062]

This contribution presents hierarchically structured matrix-based transforms (HSMTs) for non-separable secondary transformation (NSST) as alternatives to HyGT-based NSST implementations. The proposed set of HSMTs reduces the NSST memory use and provides very similar coding gains when the AMTs in JEM7 are replaced with HSMTs.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.2.1 | Hierarchically Structured Matrix-based Transforms | Qualcomm | Technicolor Samsung HHI Tencent Brightcove bcom |

### CE 6.2.3: Transform Signaling [JVET-J0024, JVET-J0025]

The non-separable secondary transform (NSST) with the following two modifications:

- Reduced angle sets based on grouping intra prediction mode

- Signaling with 1 on/off flag with 4 modes

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.3.1 | Modified NSST sets and signaling | Samsung | Qualcomm HHI Tencent |

### CE 6.2.4: Matrix multiplication based NSST [JVET-J0029 (CfP)]

It is proposed to use matrix multiplication based NSST.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.4.1 | Matrix multiplication based NSST | Tencent | LG Electronics |

### CE 6.2.5: Layered Givens Transform (LGT) [JVET-J0017(CfP)]

For smaller blocks, a Givens rotation based transform called LGT is applied on 4×4 blocks at the top-left 4×4, 4×8, or 8×4 region of a coded block. This consists of 5 Givens rotation layers (GRLs) and 6 permutations. Each GRL is composed of 8 Givens rotations and each permutation shuffles transform inputs or outputs of the previous stage.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.5.1 | Layered Givens Transform (LGT) | LG Electronics | Brightcove |

### CE 6.2.6: Reduced Secondary Transform (RST) [JVET-J0017 (CfP)]

An 8x8 reduced secondary transform (RST) is proposed. The main idea of a Reduced Transform (RT) is to map an N dimensional vector to an R dimensional vector in a different space, where R/N (R < N) is the reduction factor. In this contribution, the 8×8 RST with a reduction factor of 1/4 is applied. The forward 8x8 RST uses 16×64 matrices so that it produces non-zero coefficients only in the lower 4×4 region within the given 8×8 region. In other words, if RST is applied (i.e. ST_idx is greater than zero) then the lower 8×8 region except the lower 4×4 region (this region is called ROI region) will have to generate only zero coefficients.

As a result, the ST_idx coding method based on the distribution of non-zero coefficients in the ROI region is introduced. Specifically, ST_idx is not coded when RST is used as a secondary transform and any non-zero element is detected within the ROI region because it implies that RST was not applied. In such a case, ST_idx is inferred to be zero.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.6.1 | Reduced Secondary Transform (RST) | LG Electronics | Huawei bcom |

## Core Experiments on Combined Primary and Secondary Transformation

### CE 6.3.1: Coupled primary and secondary transform [JVET-J0054]

It is proposed to couple AMT and NSST, the usage of AMT and NSST is indicated by a single transform index. Instead of signaling the indices of primary and secondary transform independently, the primary and secondary transform is coupled and signaled by only one transform index. In addition, the proposed method is applied for both luma and chroma components.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.1.1 | Coupled primary and secondary transform for luma and chroma | Tencent | Technicolor Samsung Sony Brightcove |
| 6.3.1.2 | Coupled primary and secondary transform for luma only | Tencent | Technicolor Samsung Sony Brightcove |

### CE 6.3.2: Set of transforms [JVET-J0014, JVET-J0040]

The transform is selected among a set of 5 predefined transform candidates for each CU. Each of the transform candidates specifies a primary horizontal transform, a primary secondary transform, and a non-separable secondary transform (which may also be equal to the identity transform). The ordered list of transform candidates depends on the block size and the prediction mode. The transform chosen is signaled as follows: When cbf is equal to 1, a flag is transmitted which specifies whether the first transform in the candidate list is used. When this flag is equal to 0, the following applies: If the number of non-zero transform coefficient levels is greater than a threshold, another syntax element is transmitted which indicates the transform candidate used; otherwise, the second transform candidate of the list is used.

The set of predefined transforms comprises AMT and NSST transforms as defined in JEM and a set of additionally proposed transforms. In order to investigate the effect of both aspects, the joint candidate derivation of primary and secondary transforms and signaling scheme on the one hand, and the additionally proposed transforms on the other hand, two configurations are tested. In the first test the candidate lists comprise AMT and NSST transforms as defined in JEM only, while in the second test also the additionally proposed transforms are included.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.2.1 | Set of transforms – comprising AMT and NSST transforms as defined in JEM only. | HHI | Qualcomm Brightcove |
| 6.3.2.2 | Set of transforms – comprising AMT and NSST transforms as defined in JEM and additionally proposed transforms. | HHI | Qualcomm Brightcove |

### CE 6.3.3 AMT and NSST Complexity reduction [JVET-J0020]

This contribution proposes to reduce the complexity with the following two modifications. 1) NSST is used only when DCT2 is used as primary transform.  2) AMT (Hor or Ver) selects DST7 without signaling when width or height is less than or equal to 4 independently.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.3.1 | AMT and NSST complexity reduction | Panasonic | KDDI |

### CE 6.3.4: AMT and NSST syntax signaling [JVET-J0018]

AMT flag is signaled when the number of non-zero coefficients is larger than a threshold. For inter CUs, the threshold is set to 2. For intra CUs, it is set to 0. AMT index is signaled if the number of non-zero coefficients is larger than 2; otherwise, it is inferred as 0. For NSST, for an intra luma CU, for an intra luma CU, an NSST index is signaled when the number of non-zero coefficients of upper-left 8x8 or 4x4 luma plus the number of non-zero AC coefficients of upper-left 8x8 or 4x4 chroma components is larger than 2.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.4.1 | AMT and NSST syntax signaling | MediaTek | Bcom |

# Test conditions

The following test conditions shall be used for this CE:

## Software

The Test Model [1] shall be used for these experiments, as specified in JVET-J1010 [2].

## Test configurations

This CE shall use the common test conditions as described in JVET-J1010 [2]. Experimental results shall be reported under two configurations: Test Mode (VTM) and Benchmark Set (BMS) configurations, as provided in JVET-J1010 [2]. The VTM and BMS configurations are obtained by using BMS 1.0 software with VTM configuration files (encoder_xxx_vtm1.cfg) and BMS configuration files (encoder_xxx_bms1.cfg), respectively.

The following requirements apply to all CEs:

- As in CTC, all experiments should report results with transform dimensions not larger than 64.

- If transform dimensions larger than 64 are applied, experiment results using anchors (VTM and BMS) with the same max transform dimensions, if applicable, should be reported.

- Any encoder or decoder optimizations (e.g., SIMD or similar implementations, RD optimization changes) that are not in the test model shall be described, and individual experiment results on the optimizations shall be reported.

- For experiments which have impact on the transform precision, i.e., modifications on shift operations in transform, modifications on bit-depth of intermediate data representation, results using low-QP configurations, i.e., QP 5, 10, 15 and 20 shall be provided as supplemental information.

- If new transforms are proposed for both intra and inter residuals, individual results of applying the proposed methods on intra and inter residuals should be reported separately.

## Additional requirements per CE

The following test requirements are additional required, according to the CE. The anchors are described below for each additional test, and the test method is the anchor plus the proposed method. In the following tests, primary (AMT) and secondary transform (NSST) should be enabled/disabled by the configuring the related parameter in encoder configuration files.

### Additional requirements for CE 6.1 – primary transforms

- Experimental results with secondary transform disabled (BMS configuration, NSST disabled) shall be reported.

- Experimental results with secondary transform enabled (VTM configuration, NSST enabled) shall be reported as supplemental information.

### Additional requirements for CE 6.2 – secondary transforms

- Experimental results using the primary transforms specified in the Test Model [1] with Benchmark Set (BMS) configuration (VTM configuration, AMT enabled) shall be reported.

### Additional requirements for CE 6.3 – combinations and signaling

- If new primary transforms are also proposed, then experimental results with secondary transform disabled (BMS configuration, NSST disabled) and enabled (VTM configuration, NSST enabled) shall be both reported.

## Test sequences

The test sequences as described in JVET-J1010 [2] shall be used for experiments.

# Evaluation Criteria

## Quantitative metrics

To obtain objective rate-distortion measurements, the Test Model (TM) [1] and the Benchmark Set (BMS) [1] will be used as two separate anchors. In the BMS anchor, all the coding tools in the BMS are enabled. Proposals will be evaluated using BD rates, encoder and decoder run-time, and experiments will be performed under Common Test Conditions (CTC) as described in JVET-J1010[2].

## Qualitative evaluation

If sequences coded using a proposed alternative transform exhibit subjective improvements or degradations, proponents shall describe these subjective differences.

## Complexity metrics

For the understanding of complexity, the following shall be provided:

- Operation counts (number of multiplications, additions, and bit-shift operations) for individual transforms. If encoder and decoder use different number of transform operations, operation counts shall be reported for both encoder and decoder separately.

- Memory requirements for storing transform cores

  - Total memory for sets of several transforms (minimum amount, in bits)

  - Amount of memory needed to be accessed to compute one transform

- Specify the minimum bit-precision required by the transforms

  - To represent transform coefficients

  - Precision of arithmetic operations needed during transform computation

  - For storing intermediate data representation (e.g., between vertical and horizontal transformations)

- Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel.

- Other operations and memory requirements relevant to the proposed tool, e.g., scaling matrix modification in quantization

- Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided.

- List of all combinations of transform types (e.g., DCT-2 and DST-7) and transforms block sizes (e.g., 4x4, 4x8, 8x4 and 8x8) used for primary transformation.

- List of all combinations of transforms block size and transform type used for secondary transformation.

- Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations).

- If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics.

# Participant task checklist

Participants shall:

- Write source code or receive source code for their corresponding tool. Typically, proponents write the source code and cross-verifiers receive it.

- Examine the software to determine if it is implementing the proposed technique;

- Compile the software and run the tests described in this CE;

- Cross-verifiers shall compare their results to those provided by the proponent;

- Comment on the proposed technique and on the results.

# Timeline

General timeline for core experiments (as agreed by JVET).

| T1 | May 11, 2018 | - Final CE description uploaded |
| --- | --- | --- |
| T2 | June 1, 2018 | - Proponents complete initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | June 19, 2018 | - Proponents freeze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

# References

[1] Test Model software

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET_J1010, San Diego, US, 10–20 Apr. 2018.