| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0007 |
| --- | --- |

| Title: | JVET AHG report: Coding of HDR/WCG Material (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall E. Francois D. Rusanovskyy | Email: | asegall@sharplbas.com Edouard.Francois@technicolor.com dmytror@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 10th meeting in San Diego, US (10 – 20 Apr. 2018) and the 11th meeting in Ljubljana, SI (10 – 18 July 2018).

# Mandates

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content, including reducing the number of frames in the HLG sequences.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results of the CfP responses.

- Evaluate transfer function conversion methods.

- Prepare and coordinate expert viewing of HDR content at the 11th JVET meeting.

- Coordinate implementation of HDR anchor aspects in the test model software with AHG3.

- Study additional aspects of coding HDR/WCG content.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with an [AHG7] indication on message headers.  The primary activity of the AhG was related to the mandates of (i) coordinating the implantation of HDR anchor aspects in the test software and (ii) studying and evaluating available HDR/WCG test content with the goal of reducing the number of frames in the HLG sequences.  This work is described in the following subsections.

## HDR Anchor Aspects in the Test Software

During the AHG study period, it was reported that versions 1.0 of the VTM and BMS software encoder generated significant visual artifacts for PQ content.  After study, it was determined that the artifact could be removed by defining the WCG_EXT macro in the software.  This macro had the additional benefit in providing improved wPSNR performance, as it activates the weighted RDO using weights corresponding to the wPSNR metrics calculation.

Also during the AhG study period, a patch was provided for the VTM and BMS software.  This patch is commit id 38 in the repository, and it was included in version 1.1 of the VTM and BMS software.  After study, it was determined that the patch did not change the encoder performance when the WCG_EXT macro was enabled.  Furthermore, it was determined that the enabling the WCG_EXT macro provided more coding gain in the wPSNR metrics than did enabling commit id 38.

As a result of the study, the CTC was updated to clarify that the WCG_EXT macro should be enabled when testing HDR content, and anchors were generated using this macro setting.

It was further noted that it may be desirable to have this configuration supported in a future version of the VTM and BMS software.

## Study and Evaluate Available HDR Test Content

The group also had the mandate to study and evaluate available HDR test content, with one goal being the reduction of the number of frames used for testing in the HLG sequences.  To make progress on the mandate, the AhG used the GammutTest software that is part of the HDRTools repository to compute the dynamic range of each sequence as a function of time.  The results are provided as part of the AhG report.

To assist in the selection of frames included in the test conditions, plots of the dynamic range were also computed.  These are also provided in the XLS, and an example is shown in Figure 1.

Figure 1 – Characteristics of the DayStreet sequence.  The GammutTest tool was used to compute the minimum, maximum and dynamic range of the code values in the sequence on a per frame basis.  The full data is provided in the attached Excel file, and it includes both data and plots for the four HLG sequences.

Figure 1 – Characteristics of the FlyingBird sequence.  The GammutTest tool was used to compute the minimum, maximum and dynamic range of the code values in the sequence on a per frame basis.  The full data is provided in the attached Excel file, and it includes both data and plots for the four HLG sequences.

Figure 3 – Characteristics of the PeopleInShoppingCenter sequence.  The GammutTest tool was used to compute the minimum, maximum and dynamic range of the code values in the sequence on a per frame basis.  The full data is provided in the attached Excel file, and it includes both data and plots for the four HLG sequences.

Figure 4 – Characteristics of the SunSetBeach sequence.  The GammutTest tool was used to compute the minimum, maximum and dynamic range of the code values in the sequence on a per frame basis.  The full data is provided in the attached Excel file, and it includes both data and plots for the four HLG sequences.

It is recommended that the group selects 300 frame versions of the HLG sequences using the provided data.

# Contributions

There are 3 contributions related to HDR video coding:

| JVET-K0032 | m43229 | CE12: Summary report on HDR coding | E. Francois, D. Rusanovskyy, P. Yin |
| --- | --- | --- | --- |
| JVET-K0298 | m43231 | CE12: Report of Dynamic Range Adaptation (DRA) and DRA refinement | E. Francois (Technicolor), D. Rusanovskyy (Qualcomm) |
| JVET-K0308 | m43241 | CE12: HDR In-loop Reshaping (CE12-5, 12-6, 12-7 and 12-8) | T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen (Dolby) |

# Recommendations

The AHG recommends the following:

- Enable the WCG_EXT macro in the next version of the VTM/BMS software

- Select 300 frame versions of the HLG sequences during the meeting

- Review all input contributions