| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0012-v1 |
| --- | --- |

| Title: | Report of AHG12 on high-level parallelism |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Muhammed Coban Hyeongmun Jang Robert Skupin Ye-Kui Wang | Tel: Email: | ikai.tomohiro@sharp.co.jp mcoban@qti.qualcomm.com hm.jang@lge.com robert.skupin@hhi.fraunhofer.de yekui.wang@huawei.com |
| Source: | AHG |  |  |

_____________________________

- Abstract

The document summarizes activities of the AHG on high-level parallelism between the 10th and the 11th JVET meetings.

Introduction

The mandates of this AHG are:

- Study high-level parallelism techniques.

- Prepare software and configuration in TM to facilitate parallel processing test.

- Study the coding efficiency impact of parallel processing.

A kick-off message was sent on May 24, 2018. There has been no email discussions in the main reflector.

Related contributions

Three contributions, listed below, are on tiles and related to this AHG. All the three contributions contain aspects related to the first mandate. Some of the three contributions also contain aspects related to the third mandate.

The three contributions are:

| JVET-K0155 | m43082 | AHG12: Flexible Tile Partitioning | Y. Yasugi, T. Ikai (Sharp) |
| --- | --- | --- | --- |
| JVET-K0260 | m43189 | Flexible Tiles | R. Sjöberg, M. Damghanian, M. Pettersson, J. Enhorn (Ericsson) |
| JVET-K0300 | m43233 | Design goals for tiles | M. M. Hannuksela, A. Zare, M. Homayouni, R. Ghaznavi-Youvalari, A. Aminlou (Nokia) |

Below is a summary of the three contributions:

- JVET-K0155 proposes flexible size tile, which enables more flexible tiling segmentation than in HEVC, such that the tile size is in units of an explicitly signalled tile size unit, which can be other than in the units of CTUs, and provides experimental results for conventional and proposed tile in BMS (VTS config).

- JVET-K0260 proposes flexible tile segmentation that allows

  - rectangular tiles of varying sizes (wherein the number of tiles and their sizes are explicitly signalled); and

  - efficient tile structure signaling with tile sizes in subtile units and “use a previous tile size” flag.

- JVET-K0300 proposes some design goals to be used in evaluating merits of technical contributions on tiles and to be included as mandates of an appropriate JVET AHG covering tiling aspects. The proposed design goals are to make VVC tiles more suited for viewport-dependent 360° video streaming, including:

  - Encoding of motion-constrained tile sets (MCTSs) that are more efficient than HEVC MCTSs in terms of rate-distortion penalty;

  - Avoiding visible MCTS boundaries with as small processing cost as possible;

  - Intra block copy across tiles for enabling prediction from one constituent frame to another for frame-packed stereoscopic video, provided that intra block copy is adopted as a tool in VVC;

  - Extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without VCL NAL unit modifications.

Recommendations

- Review the related contributions.

- Continue to study high-level parallelism techniques.