| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0023-v3 |
| --- | --- |

| Title: | CE3: Summary Report on Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Jin Heo Alexey Filippov | Email: | geertv@qti.qualcomm.com jin78.heo@lge.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Introduction

This is the summary report of the third Core Experiment (CE3) [1]. The goal of CE3 is to study intra prediction tools including mode coding for the VVC standard. There are 7 sub-tests defined. In CE3.1 tests are included targeting DC, planar, directional, and additional modes. CE3.2 tests reference sample filtering, interpolation, across boundary filtering and prediction sample filtering. CE3.3 targets intra mode coding, such as most probable mode list variations. In CE3.4 cross-component linear model and variations are tested. CE3.5 focuses on multi-reference line intra prediction tests and CE3.6 on non-linear intra prediction. In CE3.7 tests are performed regarding bidirectional intra prediction.

The following is the list of defined sub-tests in CE3:

- CE3.1: Intra modes

- CE3.2: Intra filtering and interpolation

- CE3.3: Intra mode coding

- CE3.4: Cross-component linear model (CCLM)

- CE3.5: Multi-reference line intra prediction

- CE3.6: Non-linear intra prediction

- CE3.7: Bidirectional intra prediction

The CE3 description [1] originally defined 75 tests of which 7 were withdrawn and one redefined. This document summarizes the objective results (BD-rates, runtimes), cross-check reports and related input contributions.

# Test conditions and guidelines

CTC conditions as agreed at the San Diego meeting for SDR are followed for the evaluation of objective performance (JVET-J1010). In CE3 it was agreed that it is mandatory to report results with both the ‘All Intra’ and ‘Random Access’ test configurations. Two anchors are used to study the performance of the proposed technologies. The first anchor is the ‘VVC Test Model’ or VTM. The second anchor is the ‘Benchmark Model Set’ or BMS. The BMS version 1.0 software is used with ‘bms’ configuration files to produce the BMS anchor, while ‘vtm’ configuration files are used to produce the VTM anchor. Tools are implemented into the BMS version 1.0 software. Proposals are compared with respect to objective compression efficiency while complexity is also assessed.

The source codes of the tests were uploaded by proponents into the CE3 repository:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE3/

# Cross-check reporting

Each test is covered by at least one cross-checker and the cross-check reports are included within the respective sub-test sections. Within CE3 it was agreed that cross-checkers answer following questions:

- Did the cross-checker study the software? If so, did the cross-checker find any significant inconsistencies between the description and the provided software?

- Do the cross-checked results closely match the proponent results for AI and RA conditions (VTM and BMS)?

- Were additional results cross-checked?

- Additional comments that may be helpful to the group?

This report summarizes the answers in tables reporting whether the objective test results match in one table column and all other comments in a second table column.

Several cross-checkers have reported 4th decimal average PSNR differences between proponent results and cross-checked results for RA conditions. One cause is running RA in parallel versus sequential while PSNR numbers are available in the encoder log with 4-digit accuracy. Another reason may be due to using separate PSNR measurement tools with different accuracies. One suggested solution is increasing the encoder log accuracy for the next software version of VTM and BMS.

# CE3.1 on ‘Intra modes’

## Defined tests

| Test# | Short description | Doc. # |
| --- | --- | --- |
| 1.1.1 | Use 129 directional modes for all blocks | JVET-K0060 (Qualcomm) |
| 1.1.2 | Use variable number of directional modes (33, 65, or 129) depending on block size comparison with two SPS thresholds |  |
| 1.2.1 | DC mode with only shift operators | JVET-K0211 (Panasonic) |
| 1.3.1 | Wide-angle prediction | JVET-K0046 (Nokia) |
| 1.4.1 | Usage of line-based intra prediction mode | JVET-K0049 (HHI) |
| 1.4.2 | Fast Line-based intra prediction mode |  |
| 1.4.3 | Constrained Line-based intra prediction mode |  |
| 1.5.1 | Unequal weighted planar prediction (UWP) | JVET-K0055 (ARRIS, Sharp) |

## CE3.1: Test results

### CE3.1: ‘All Intra Main10’ results

|  |  | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1.1 | Use 129 directional modes for all blocks | -1.30% | -1.43% | -1.42% | 119% | 101% | 0.04% | -0.10% | -0.13% | 102% | 100% |
| 1.1.2 | Use variable number of directional modes (33, 65, or 129) depending on block size | -1.44% | -1.45% | -1.48% | 116% | 101% | -0.14% | -0.13% | -0.18% | 101% | 101% |
| 1.2.1 | DC mode with only shift operators | 0.01% | 0.04% | 0.01% | 100% | 99% | 0.01% | 0.09% | 0.02% | 100% | 99% |
| 1.3.1 | Wide-angle prediction | -0.28% | -0.38% | -0.42% | 106% | 101% | -0.23% | -0.23% | -0.30% | 102% | 98% |
| 1.4.1 | Usage of line-based intra prediction mode | -2.34% | -2.15% | -2.47% | 293% | 120% | -0.70% | -0.99% | -1.15% | 125% | 105% |
| 1.4.2 | Fast Line-based intra prediction mode | -2.00% | -1.83% | -2.09% | 164% | 112% | -0.35% | -0.55% | -0.57% | 106% | 102% |
| 1.4.3 | Constrained Line-based intra prediction mode | -1.58% | -1.74% | -1.98% | 146% | 110% | -0.26% | -0.39% | -0.49% | 105% | 102% |
| 1.5.1 | Unequal weighted planar prediction (UWP) | -0.20% | -0.03% | -0.06% | 101% | 103% | -0.53% | -0.53% | -0.53% | 100% | 102% |

### CE3.1: ‘Random Access Main10’ results

|  |  | Random Access Main10 - Over VTM1.0 |  |  |  |  | Random Access Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1.1 | Use 129 directional modes for all blocks | -0.60% | -0.55% | -0.56% | 104% | 101% | 0.03% | -0.05% | -0.13% | 101% | 99% |
| 1.1.2 | Use variable number of directional modes (33, 65, or 129) depending on block size | -0.64% | -0.51% | -0.54% | 103% | 101% | -0.04% | -0.03% | -0.15% | 100% | 100% |
| 1.2.1 | DC mode with only shift operators | 0.01% | 0.08% | -0.01% | 100% | 101% | 0.02% | 0.13% | -0.05% | 100% | 101% |
| 1.3.1 | Wide-angle prediction | -0.18% | -0.23% | -0.41% | 102% | 101% | -0.13% | -0.15% | -0.26% | 103% | 100% |
| 1.4.1 | Usage of line-based intra prediction mode | -0.98% | -0.86% | -1.10% | 128% | 105% | -0.30% | -0.49% | -0.61% | 109% | 101% |
| 1.4.2 | Fast Line-based intra prediction mode | -0.82% | -0.69% | -0.89% | 110% | 102% | -0.15% | -0.20% | -0.36% | 102% | 101% |
| 1.4.3 | Constrained Line-based intra prediction mode | -0.63% | -0.70% | -0.95% | 107% | 103% | -0.08% | -0.19% | -0.30% | 102% | 101% |
| 1.5.1 | Unequal weighted planar prediction (UWP) | -0.06% | 0.21% | 0.03% | 99% | 103% | -0.27% | -0.23% | -0.47% | 100% | 102% |

### CE3.1: Additional test results

NA

### CE3.1: Cross-check reports

| Test# | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 1.1.1 | Steve Keating (Sony) | y | It seems that using 129 directional modes is no overall improvement on 65 directional modes but switching the number of angles does show an improvement, particularly for class F. |
| 1.1.2 | Yukinobu Yasugi (Sharp) | y | 521 lines except empty lines and comments were added to BMS-1.0. We think the changes are acceptable. |
| 1.2.1 | Zhi Zhang (Ericsson) | y | RA parallel job should be considered to generate more accurate PSNR result to avoid the fourth decimal mismatch. For example, in JEM we use the precise log which is 6 decimal accuracy. |
| 1.3.1 | Geert Van der Auwera (Qualcomm) | y |  |
|  | Andre Dias (BBC) | y |  |
|  | Alexey Filippov (Huawei) | y |  |
| 1.4.1 | Andre Dias (BBC) | y (~encT) | On top of VTM, the crosschecks report around 30% higher encoding time increase for AI on top of VTM than the encoding time increase reported by the proponent. Partial tests on different machines resulted in an encoding time increase different both from what was reported by the proponent and by the initial crosscheck. It seems the machines used to run the tests have an impact on the encoding time of the tool. |
| 1.4.2 | Andre Dias (BBC) | y (~encT) | On top of VTM, the crosschecks report around 10% higher encoding times for AI on top of VTM than the encoding times reported by the proponent. It looks like the same issue as in CE3-1.4.1 but less accentuated as the tool is used less in this test. |
|  | D. Y. Kim (Chips&Media) | y |  |
| 1.4.3 | Andre Dias (BBC), | y (~encT) | On top of VTM, the crosschecks report around 10% higher encoding times for AI on top of VTM than the encoding times reported by the proponent. It looks like the same issue as in CE3-1.4.1 but less accentuated as the tool is used less in this test. |
|  | D. Y. Kim (Chips&Media) | y |  |
| 1.5.1 | Geert Van der Auwera (Qualcomm) | y | Table with 254 entries is used in computation. |

## CE3.1: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-K0059 (Huawei) | 1.3.1 | CE3-related: Reference sample processing for wide-angle intra-prediction |
| JVET-K0122 (Huawei) | 1.2.1 | CE3-related: Alternative techniques for DC mode without division |
| JVET-K0169 (Technicolor) |  | CE3-related: Block Shape Adaptive Intra Prediction Directions |
| JVET-K0242 (MediaTek) | 1.5.1 | CE3-related: Intra planar mode prediction |
| JVET-K0289 (Tencent) | 1.3.1 | CE3-related: Wide angular intra prediction for non-square blocks |
| JVET-K0400 (WILUS) | 1.2.1 | CE3-related: DC mode with only shift operators based on sub-sampling |

# CE3.2 on ‘Intra filtering and interpolation’

## Defined tests

| Test # | Short Description | Doc. # |
| --- | --- | --- |
| 2.1.1 | Bilateral reference sample filter | JVET-K0061 (Qualcomm) |
| 2.2.1 | 4-tap cubic filter for extending reference samples | JVET-K0062 (Qualcomm) |
| 2.2.2 | Bilinear interpolation for extending reference samples | JVET-K0211 (Panasonic) |
| 2.3.1 | Interpolation filter selection between 6-tap cubic and 4-tap Gaussian filter based on block size | JVET-K0062 (Qualcomm) |
| 2.3.2 | Combine tests 2.2.1 + 2.3.1 |  |
| 2.3.3 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size | JVET-K0097 (LGE) |
| 2.3.4 | Interpolation filter selection between 6-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size |  |
| 2.4.1 | Simplified position dependent intra prediction combination (PDPC) | JVET-K0063 (Qualcomm) |
| 2.5.1 | Bilinear interpolation for projection and smoothing after projection | JVET-K0211 (Panasonic) |
| 2.6.1 | 6-tap combined filter without reference sample smoothing | JVET-K0165 (ETRI) |
| 2.7.1 | Bilateral reference sample filter | JVET-K0043 (HHI) |
| 2.8.2 | Mode dependent de-ringing filter based on bitstream flag | JVET-K0066 (Huawei) |
| 2.9.1 | Intra boundary filters | JVET-K0240 (MediaTek) |
| 2.10.1 | Multiple 4-tap filter | JVET-K0179 (Samsung) |
| 2.11.1 | Multi-combined intra prediction | JVET-K0180 (Samsung) |

## CE3.2: Test results

### CE3.2: ‘All Intra Main10’

|  |  | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.1.1 | Bilateral reference sample filter | 0.03% | -0.02% | -0.04% | 100% | 101% | -0.13% | -0.16% | -0.17% | 100% | 99% |
| 2.2.1 | 4-tap cubic filter for extending reference samples | -0.05% | -0.08% | -0.11% | 102% | 100% | -0.01% | -0.04% | -0.03% | 100% | 99% |
| 2.2.2 | Bilinear interpolation for extending reference samples | -0.05% | -0.08% | -0.08% | 101% | 100% | 0.00% | 0.01% | -0.03% | 100% | 99% |
| 2.3.1 | Interpolation filter selection between 6-tap cubic and 4-tap Gaussian filter based on block size | -0.57% | -0.39% | -0.43% | 107% | 103% | -0.44% | -0.48% | -0.53% | 101% | 101% |
| 2.3.2 | Combine tests 2.2.1 + 2.3.1 | -0.63% | -0.49% | -0.48% | 107% | 103% | -0.46% | -0.58% | -0.58% | 102% | 100% |
| 2.3.3 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size | -0.54% | -0.41% | -0.46% | 106% | 103% | -0.40% | -0.41% | -0.43% | 106% | 104% |
| 2.3.4 | Interpolation filter selection between 6-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size | -0.58% | -0.39% | -0.43% | 107% | 102% | -0.44% | -0.57% | -0.57% | 101% | 101% |
| 2.4.1 | Simplified position dependent intra prediction combination (PDPC) | -0.97% | -0.14% | -0.01% | 109% | 107% | -1.35% | -0.94% | -0.81% | 103% | 105% |
| 2.5.1 | Bilinear interpolation for projection and smoothing after projection | -0.01% | -0.11% | -0.13% | 102% | 100% | 0.02% | 0.01% | -0.04% | 100% | 99% |
| 2.6.1 | 6-tap combined filter without reference sample smoothing | -0.55% | -0.01% | 0.02% | 108% | 102% | -0.39% | -0.11% | -0.16% | 101% | 101% |
| 2.7.1 | Bilateral reference sample filter | -0.22% | -0.11% | -0.14% | 101% | 101% | -0.24% | -0.20% | -0.24% | 101% | 102% |
| 2.8.2 | Mode dependent de-ringing filter based on bitstream flag | -0.40% | -0.26% | -0.32% | 116% | 100% | -0.40% | -0.37% | -0.39% | 115% | 100% |
| 2.9.1 | Intra boundary filters | -0.75% | -0.69% | -0.75% | 104% | 102% | -0.87% | -0.55% | -0.59% | 103% | 106% |
| 2.10.1 | Multiple 4-tap filter | -0.41% | -0.19% | -0.16% | 112% | 103% | -0.13% | -0.11% | -0.16% | 103% | 102% |
| 2.11.1 | Multi-combined intra prediction | -0.12% | -0.04% | -0.06% | 196% | 100% | 0.01% | 0.04% | 0.00% | 212% | 101% |

### CE3.2: ‘Random Access Main10’

|  |  | Random Access Main10 - Over VTM1.0 |  |  |  |  | Random Access Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.1.1 | Bilateral reference sample filter | 0.00% | 0.10% | 0.06% | 100% | 101% | -0.04% | 0.09% | -0.06% | 100% | 99% |
| 2.2.1 | 4-tap cubic filter for extending reference samples | -0.04% | -0.01% | -0.02% | 101% | 100% | 0.00% | 0.00% | -0.07% | 100% | 99% |
| 2.2.2 | Bilinear interpolation for extending reference samples | -0.04% | 0.01% | -0.08% | 101% | 102% | 0.01% | 0.00% | -0.03% | 100% | 101% |
| 2.3.1 | Interpolation filter selection between 6-tap cubic and 4-tap Gaussian filter based on block size | -0.29% | -0.64% | -0.75% | 101% | 100% | -0.18% | -0.42% | -0.47% | 101% | 99% |
| 2.3.2 | Combine tests 2.2.1 + 2.3.1 | -0.32% | -0.71% | -0.81% | 102% | 101% | -0.19% | -0.47% | -0.51% | 102% | 100% |
| 2.3.3 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size | -0.28% | -0.52% | -0.61% | 102% | 102% | -0.16% | -0.36% | -0.36% | 102% | 100% |
| 2.3.4 | Interpolation filter selection between 6-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size | -0.30% | -0.70% | -0.74% | 102% | 100% | -0.19% | -0.44% | -0.50% | 101% | 100% |
| 2.4.1 | Simplified position dependent intra prediction combination (PDPC) | -0.48% | 0.39% | 0.46% | 101% | 101% | -0.70% | -0.41% | -0.56% | 102% | 101% |
| 2.5.1 | Bilinear interpolation for projection and smoothing after projection | 0.00% | -0.08% | -0.03% | 98% | 99% | 0.03% | 0.02% | -0.12% | 100% | 100% |
| 2.6.1 | 6-tap combined filter without reference sample smoothing | -0.29% | 0.13% | 0.13% | 102% | 100% | -0.14% | 0.12% | -0.07% | 100% | 100% |
| 2.7.1 | Bilateral reference sample filter | -0.13% | -0.02% | -0.03% | 100% | 102% | -0.11% | -0.10% | -0.24% | 101% | 100% |
| 2.8.2 | Mode dependent de-ringing filter based on bitstream flag | -0.22% | -0.18% | -0.28% | 103% | 100% | -0.17% | -0.16% | -0.29% | 105% | 100% |
| 2.9.1 | Intra boundary filters | -0.38% | -0.21% | -0.24% | 99% | 94% | -0.41% | -0.31% | -0.46% | 104% | 100% |
| 2.10.1 | Multiple 4-tap filter | -0.21% | -0.14% | -0.12% | 103% | 100% | -0.07% | -0.03% | -0.14% | 99% | 98% |
| 2.11.1 | Multi-combined intra prediction | -0.02% | -0.09% | -0.17% | 113% | 100% | 0.05% | -0.02% | -0.13% | 141% | 100% |

### CE3.2: Additional test results

|  |  |  | Intra Main10 - Over VTM1.0 |  |  |  |  |  |  | Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Add. Result | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.4.1, 2.7.1, 5.4.4 | 2.4.1 + 2.7.1 + 5.4.4 + Intra65 (JVET-K0348) | -3.2% | -1.8% | -1.7% | 188% | 107% | -2.0% | -1.3% | -1.2% | 120% | 100% |

|  |  |  | Random Access Main10 - Over VTM1.0 |  |  |  |  |  | Random Access Main10 - Over BMS1.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # |  | Add. Result |  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| 2.4.1, 2.7.1, 5.4.4 |  | 2.4.1 + 2.7.1 + 5.4.4 + Intra65 (JVET-K0348 |  | -1.6% | -0.3% | -0.4% | 116% | 100% |  | -1.1% | -0.6% | -0.6% | 112% | 101% |

|  |  | Intra Main10 - Over VTM1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test # | Add. Result | Y | U | V | EncT | DecT |
| 2.4.1, 5.2.5 | 2.4.1 + 5.2.5 + Intra65 (JVET-K0377) | -2.59% | -1.75% | -1.62% | 134% | 114% |

|  |  | Intra Main10 - Over VTM1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test # | Add. Result | Y | U | V | EncT | DecT |
| 2.4.1, 5.2.5 | 2.4.1 + 5.2.5 + Intra65 + JVET-K0064 (JVET-K0377) | -3.35% | -2.03% | -1.92% | 149% | 121% |

### CE3.2: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 2.1.1 | Philipp Merkle (HHI) | y |  |
| 2.2.1 | Steve Keating (Sony) | y |  |
| 2.2.2 | Philipp Merkle (HHI) | y |  |
| 2.3.1 | Sunmi Yoo (LGE) | y |  |
| 2.3.2 | Sunmi Yoo (LGE) | y |  |
| 2.3.3 | Geert Van der Auwera (Qualcomm) | y | There is some content dependency (gain of class C sequence BasketballDrill is 2.33% over VTM) |
| 2.3.4 | Geert Van der Auwera (Qualcomm) | y | There is some content dependency (gain of class C sequence BasketballDrill improved to 3.13% over VTM by using 6-tap cubic over 4-tap) |
| 2.4.1 | Jani Lainema (Nokia) | y | Only able to do the mandatory AI+RA set during this round due to cluster constraints. In addition to the mandatory classes of sequences, tested the method also on Class F, where some degradation of coding efficiency was noticed. |
| 2.5.1 | Geert Van der Auwera (Qualcomm) | y | Performing intra smoothing after projection allows for combination with interpolation in single step. |
|  | Philipp Merkle (HHI) | y | One minor mismatch (BMS_RandomAccess_MarketPlace_QP37) |
| 2.6.1 | Virginie Drugeon (Panasonic) | y | This is a fixed implementation of the technology in the software using a macro, i.e. there is no configuration file parameter to turn the tool off, which means that it is intended as a replacement of existing code, which makes sense in my opinion given the technology proposed. The smoothing of reference samples prior to the angular intra prediction function is turned off (since the smoothing is applied together with the 6-tap interpolation filter as described), but the code and memory allocation for it is still present in the code. In my opinion, I believe that it makes sense to combine the different intra filtering processes together within the angular intra prediction function, so that it would make sense to adopt either this technology or another similar one. I believe that CE3-2.5.1 and CE3-2.10.1 are trying to address similar issues. |
|  | Geert Van der Auwera (Qualcomm) | y | Combining smoothing with interpolation may save memory to store the filtered reference samples. |
| 2.7.1 | Sergey Ikonin (Huawei) | y | In this test filter with one iteration (N=1) is applied. |
| 2.8.2 | Jonathan Pfaff (HHI) | y |  |
|  | Geert Van der Auwera (Qualcomm) | y | There are many lines of source code added. Visual quality improvement is claimed, so there may be more subjective gain. |
| 2.9.1 | Xiang Ma (Huawei) | y | DC filter is also used in this CE test. |
| 2.10.1 | Virginie Drugeon (Panasonic) | y | The description states that smoothing is applied together with the interpolation filters, so it seems to suggest that it is not useful to apply the MDIS smoothing filter prior to the angular intra prediction function. However, it seems that MDIS smoothing prior to the angular intra prediction function is still applied in the software, which I find strange at first sight because it means that smoothing can be applied several times for prediction of some of the pixels. This is a fixed implementation of the technology in the software using a macro, i.e. there is no configuration file parameter to turn the tool off, which means that it is intended as a replacement of existing code, which makes sense in my opinion given the technology proposed. In my opinion, I believe that it makes sense to combine the different intra filtering processes together within the angular intra prediction function, so that it would make sense to adopt either this technology or another similar one. I believe that CE3-2.5.1 and CE3-2.6.1 are trying to address similar issues. One advantage of CE3-2.10.1 is that it uses 4-tap filters instead of 6-tap filters in CE3-2.6.1, which is better from a hardware complexity point of view. However, the issue with applying MDIS smoothing prior to the angular intra prediction function needs to be clarified. |
| 2.11.1 | Geert Van der Auwera (Qualcomm) | y | Encoder runtimes are high. |

## CE3.2: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-K0064 (Qualcomm) | 2.6.1 | CE3-related: On MDIS and intra interpolation filter switching |
| JVET-K0172 (Technicolor) |  | CE3-related: Gradient-Based Boundary Filtering in Intra Prediction |
| JVET-K0348 (Qualcomm, HHI) | 2.4.1, 2.7.1, 5.4.4 | CE3-related: On the combination of multiple reference lines, bilateral reference line filtering, PDPC and 65 directional modes for intra prediction |
| JVET-K0377 (Sony, Qualcomm) | 2.4.1, 5.2.5 | CE3-related: Combining CE3-5.2.5 on using two rows of reference lines for prediction, CE3-2.4.1 on PDPC and 65 angular intra modes |

# CE3.3 on ‘Intra mode coding’

## Defined tests

| Test # | Short Description | Doc. # |
| --- | --- | --- |
| 3.1.1 | Intra 65, 6 modes in primary MPM, secondary MPM, shape dependency | JVET-K0081 (Qualcomm) |
| 3.2.1 | Priority based list with primary MPM, secondary MPM and first few default modes prioritized (method 1 from Samsung): TM intra modes | JVET-K0181 (Samsung) |
| 3.2.2 | Priority based list with primary MPM, secondary MPM and first few default modes prioritized (method 2 from Huawei): 65 intra modes | JVET-K0365 (Huawei) |
| 3.2.3 | 6 MPM + Selected + Non-selected modes list (JEM macro JVET_B0051_NON_MPM_MODE), 65 intra modes | JVET-K0368 (Huawei) |
| 3.3.1 | MPM list construction based on dependency between neighboring intra modes | JVET-K0087 (LGE) |

## CE3.3: Test results

### CE3.3: ‘All Intra Main10’

| Test # | Description | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.1.1 | Intra 65, 6 modes in primary MPM, secondary MPM, shape dependency | -1.41% | -1.33% | -1.35% | 109% | 101% | -0.16% | -0.07% | -0.08% | 99% | 101% |
| 3.2.1 | Priority based list with primary MPM, secondary MPM and first few default modes prioritized (method 1 from Samsung): TM intra modes | -0.07% | -0.07% | -0.08% | 104% | 101% | -0.08% | 0.07% | 0.01% | 102% | 101% |
| 3.2.2 | Priority based list with primary MPM, secondary MPM and first few default modes prioritized (method 2 from Huawei): 65 intra modes | -1.48% | -1.29% | -1.28% | 112% | 104% | -0.25% | -0.03% | -0.03% | 102% | 102% |
| 3.2.3 | 6 MPM + Selected + Non-selected modes list (JEM macro JVET_B0051_NON_MPM_MODE), 65 intra modes | -1.25% | -1.26% | -1.29% | 114% | 102% | 0.01% | 0.02% | -0.01% | 100% | 102% |
| 3.3.1 | MPM list construction based on dependency between neighboring intra modes | -1.26% | -1.22% | -1.23% | 115% | 101% | -0.04% | 0.08% | 0.01% | 101% | 102% |

### CE3.3: ‘Random Access Main10’

| Test # | Description | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.1.1 | Intra 65, 6 modes in primary MPM, secondary MPM, shape dependency | -0.65% | -0.51% | -0.45% | 101% | 99% | -0.05% | -0.02% | -0.13% | 100% | 100% |
| 3.2.1 | Priority based list with primary MPM, secondary MPM and first few default modes prioritized (method 1 from Samsung): TM intra modes | -0.04% | 0.04% | -0.01% | 100% | 100% | -0.01% | 0.11% | 0.00% | 97% | 99% |
| 3.2.2 | Priority based list with primary MPM, secondary MPM and first few default modes prioritized (method 2 from Huawei): 65 intra modes | -0.68% | -0.35% | -0.36% | 103% | 100% | -0.11% | 0.14% | 0.04% | 100% | 100% |
| 3.2.3 | 6 MPM + Selected + Non-selected modes list (JEM macro JVET_B0051_NON_MPM_MODE), 65 intra modes | -0.58% | -0.42% | -0.46% | 104% | 100% | 0.01% | 0.07% | -0.11% | 101% | 101% |
| 3.3.1 | MPM list construction based on dependency between neighboring intra modes | -0.58% | -0.42% | -0.39% | 102% | 100% | 0.01% | 0.08% | -0.11% | 101% | 101% |

### CE3.3: Additional test results

| Test # | Description | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.2.3 | 6 MPM + Selected + Non-selected modes list (JEM macro JVET_B0051_NON_MPM_MODE), 65 intra modes + encoder optim. | -1.25% | -1.26% | -1.29% | 108% | 102% | 0.01% | 0.02% | -0.01% | 100% | 102% |

| Test # | Description | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.2.3 | 6 MPM + Selected + Non-selected modes list (JEM macro JVET_B0051_NON_MPM_MODE), 65 intra modes + encoder optim. | -0.58% | -0.42% | -0.46% | 100% | 100% | 0.01% | 0.07% | -0.11% | 99% | 101% |

### CE3.3: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3.1.1 | Anand Meher Kotra (Huawei) | y | Intra65Ang needs to be switched on for VTM results. Encoding and decoding times closely match when compared to the results provided by the proponent. Our simulations showed around 1 to 2% increase in run times in few configurations, when compared to the results provided by the proponent. |
| 3.2.1 | Zhi Zhang (Ericsson) | y | AI results match. For RA, there is little mismatch at the fourth decimal. I agree that the fourth decimal mismatch is small and due to GOP split parallel simulation. But this issue should be bring up to the CE3 and discuss during the meeting. RA parallel job should be considered to generate more accurate PSNR result to avoid the fourth decimal mismatch. For example, in JEM we use the precise log which is 6 decimal accuracy. |
| 3.2.1 | Jonathan Pfaff (HHI) | y |  |
| 3.2.2 | Yue Yu (ARRIS) | y |  |
| 3.2.2 | Zhi Zhang (Ericsson) | y | The cross-checker reports that significant inconsistencies between the description and the provided software were found: “The provided software contains two tools are not described in the CE3 documents. These two tools are: second tier MPM and replace DIA with VDIA.” AI results match. For RA, there is little mismatch at the fourth decimal. I agree that the fourth decimal mismatch is small and due to GOP split parallel simulation. But this issue should be bring up to the CE3 and discuss during the meeting. RA parallel job should be considered to generate more accurate PSNR result to avoid the fourth decimal mismatch. For example, in JEM we use the precise log which is 6 decimal accuracy. |
| 3.2.2 | Jonathan Pfaff (HHI) | y | Not everything was mentioned in the CE document. Also, for the VTM results, Intra65Ang needs to be switched on, otherwise, the tool does not work. |
| 3.2.3 | Shimpei Nemoto (NHK) | y |  |
| 3.3.1 | Anand Meher Kotra (Huawei) | y | Regarding the BMS results, the bitrate mismatch of fourth digit after the decimal also occur to another crosscheck I take, it can be deduced that it is the difference of script we use causes this bitrate mismatch. |

## CE3.3: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-K0058 (Arris) | 3.2.1, 3.2.2 | EE3-Related: Priority List Based Intra Mode Coding with 5 MPM |
| JVET-K0175 (Foxconn) |  | CE3-related: Advanced MPM based on intra reference line selection scheme |
| JVET-K0243 (MediaTek) |  | CE3-related: Intra mode coding |
| JVET-K0293 (Tencent) |  | CE3-related: Simplifications for chroma intra coding |

# CE3.4 on ‘Cross-component linear model (CCLM)’

## Defined tests

| Test # | Short Description | Doc. # |
| --- | --- | --- |
| 4.1.1 | LM + MMLM + MFLM + LM-Angular | JVET-K0082 (Qualcomm) |
| 4.1.2 | MMLM + MFLM + LM-angular |  |
| 4.1.3 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) | JVET-K0073 (Foxconn) |
| 4.1.4 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) |  |
| 4.1.5 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) + MMLM/MFLM (A, B, E, G) |  |
| 4.1.6 | LM + MMLM + multi filter LM + extended LM-Angular | JVET-K0092 (LGE) |
| 4.1.7 | LM + MMLM + extended LM-Angular |  |
| 4.1.8 | LM only (or single model CCLM) | JVET-K0190 (Huawei) |
| 4.1.9 | LM only + CCLM Cb-to-Cr |  |
| 4.1.10 | LM+MMLM |  |
| 4.1.11 | LM+MMLM+MFLM |  |
| 4.2.1 | MDLM | JVET-K0191 (Huawei) |
| 4.2.2 | LM + MDLM |  |
| 4.2.3 | LM+MDLM+MMLM |  |
| 4.2.4 | LM+MDLM+MMLM+MFLM |  |
| 4.3.1 | Inter-color reference prediction | JVET-K0395 (KDDI) |
| 4.3.2 | Adaptive inter-residual prediction with fast RDO |  |
| 4.4.1 | LM + LM-left + LM-top | JVET-K0241 (MediaTek) |
| 4.4.2 | LM + LM-CbCr |  |
| 4.4.3 | LM + LM fusion |  |
| 4.4.4 | LM + LM-left + LM-top + LM-CbCr + LM fusion |  |

## CE3.4: Test results

### CE3.4: ‘All Intra Main10’

| Test # | Description | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.1 | LM + MMLM + MFLM + LM-Angular | -1.71% | -11.19% | -12.01% | 158% | 109% | -0.03% | -0.56% | -0.60% | 111% | 102% |
| 4.1.2 | MMLM + MFLM + LM-angular | -1.68% | -9.77% | -10.52% | 149% | 109% | -0.02% | 0.84% | 0.67% | 106% | 102% |
| 4.1.3 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) | -1.74% | -9.78% | -10.73% | 102% | 102% | -0.08% | 0.64% | 0.48% | 76% | 100% |
| 4.1.4 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) | -1.74% | -10.07% | -11.03% | 106% | 102% | -0.07% | 0.29% | 0.11% | 79% | 100% |
| 4.1.5 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) + MMLM/MFLM (A, B, E, G) | -1.74% | -10.20% | -11.17% | 110% | 102% | -0.06% | 0.24% | 0.05% | 81% | 100% |
| 4.1.6 | LM + MMLM + multi filter LM + extended LM-Angular | -1.71% | -11.01% | -11.82% | 141% | 106% | -0.03% | -0.30% | -0.23% | 101% | 101% |
| 4.1.7 | LM + MMLM + extended LM-Angular | -1.70% | -10.80% | -11.58% | 122% | 103% | -0.01% | -0.01% | 0.02% | 90% | 100% |
| 4.1.8 | LM only (or single model CCLM) | -1.19% | -9.01% | -8.00% | 110% | 102% | 0.47% | 1.88% | 4.39% | 82% | 99% |
| 4.1.9 | LM only + CCLM Cb-to-Cr | -1.53% | -9.80% | -10.48% | 113% | 104% | 0.14% | 1.09% | 1.42% | 83% | 101% |
| 4.1.10 | LM+MMLM | -1.66% | -10.36% | -11.38% | 122% | 104% | 0.01% | 0.32% | 0.32% | 0.32% | 0.32% |
| 4.1.11 | LM+MMLM+MFLM | -1.68% | -10.65% | -11.59% | 142% | 106% | 0.00% | 0.00% | 0.00% | 101% | 102% |
| 4.2.1 | MDLM | -1.20% | -9.50% | -8.54% | 121% | 103% | 0.48% | 1.26% | 3.63% | 88% | 100% |
| 4.2.2 | LM + MDLM | -1.61% | -11.11% | -12.07% | 135% | 106% | 0.07% | -0.33% | -0.23% | 97% | 102% |
| 4.2.3 | LM+MDLM+MMLM | -1.67% | -11.40% | -12.46% | 145% | 106% | 0.02% | -0.76% | -0.84% | 102% | 102% |
| 4.2.4 | LM+MDLM+MMLM+MFLM | -1.70% | -11.65% | -12.68% | 165% | 108% | -0.01% | -1.05% | -1.09% | 114% | 103% |
| 4.3.1 | Inter-color reference prediction | 0.07% | -1.62% | -1.62% | 110% | 100% | 0.08% | -0.23% | -0.27% | 110% | 100% |
| 4.3.2 | Adaptive inter-residual prediction with fast RDO | -0.42% | -2.70% | -3.35% | 128% | 102% | 0.02% | 0.02% | -0.07% | 117% | 99% |
| 4.4.1 | LM + LM-left + LM-top | -1.54% | -10.75% | -11.64% | 121% | 102% | 0.14% | -0.01% | 0.20% | 88% | 98% |
| 4.4.2 | LM + LM-CbCr | -1.50% | -10.16% | -11.23% | 119% | 102% | 0.18% | 0.69% | 0.73% | 87% | 98% |
| 4.4.3 | LM + LM fusion | -1.18% | -11.17% | -11.37% | 118% | 103% | 0.52% | -0.52% | 0.21% | 87% | 101% |
| 4.4.4 | LM + LM-left + LM-top + LM-CbCr + LM fusion | -1.19% | -12.09% | -12.62% | 136% | 103% | 0.53% | -1.59% | -1.09% | 98% | 101% |

### CE3.4: ‘Random Access Main10’

| Test # | Description | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.1 | LM + MMLM + MFLM + LM-Angular | -1.03% | -12.85% | -13.59% | 110% | 102% | 0.01% | -0.56% | -0.75% | 108% | 102% |
| 4.1.2 | MMLM + MFLM + LM-angular | -1.01% | -11.25% | -12.24% | 108% | 101% | 0.02% | 1.05% | 0.54% | 105% | 100% |
| 4.1.3 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) | -1.06% | -11.74% | -12.74% | 101% | 101% | -0.05% | 0.66% | 0.20% | 92% | 100% |
| 4.1.4 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) | -1.06% | -12.09% | -13.12% | 100% | 100% | -0.05% | 0.21% | -0.13% | 92% | 100% |
| 4.1.5 | MNLM: LM + MMLM/MFLM (B, C, E, F) + MMLM/MFLM (A, B, C, D) + MMLM/MFLM (C, D, F, H) + MMLM/MFLM (A, B, E, G) | -1.05% | -12.18% | -13.32% | 101% | 101% | -0.04% | 0.08% | -0.34% | 93% | 100% |
| 4.1.6 | LM + MMLM + multi filter LM + extended LM-Angular | -1.03% | -12.70% | -13.39% | 107% | 103% | 0.01% | -0.38% | -0.33% | 103% | 101% |
| 4.1.7 | LM + MMLM + extended LM-Angular | -1.01% | -12.40% | -13.20% | 105% | 102% | 0.03% | -0.07% | -0.19% | 98% | 100% |
| 4.1.8 | LM only (or single model CCLM) | -0.76% | -10.39% | -9.24% | 102% | 100% | 0.28% | 2.40% | 4.76% | 94% | 100% |
| 4.1.9 | LM only + CCLM Cb-to-Cr | -0.88% | -11.01% | -11.67% | 102% | 101% | 0.16% | 1.58% | 1.87% | 95% | 100% |
| 4.1.10 | LM+MMLM | -1.00% | -12.00% | -13.05% | 104% | 101% | 0.03% | 0.42% | 0.14% | 97% | 100% |
| 4.1.11 | LM+MMLM+MFLM | -1.01% | -12.46% | -13.26% | 106% | 101% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| 4.2.1 | MDLM | -0.74% | -11.04% | -9.93% | 103% | 101% | 0.30% | 1.79% | 4.00% | 96% | 100% |
| 4.2.2 | LM + MDLM | -0.93% | -12.75% | -13.43% | 107% | 101% | 0.13% | -0.26% | -0.14% | 100% | 100% |
| 4.2.3 | LM+MDLM+MMLM | -0.99% | -13.24% | -14.16% | 108% | 101% | 0.07% | -0.83% | -1.00% | 102% | 100% |
| 4.2.4 | LM+MDLM+MMLM+MFLM | -1.00% | -13.59% | -14.38% | 111% | 102% | 0.04% | -1.20% | -1.23% | 106% | 100% |
| 4.3.1 | Inter-color reference prediction | 0.04% | -2.02% | -1.89% | 102% | 100% | 0.09% | -0.22% | -0.31% | 102% | 100% |
| 4.3.2 | Adaptive inter-residual prediction with fast RDO | -0.14% | -2.67% | -3.37% | 24% | 100% | 0.04% | -0.02% | -0.11% | 104% | 99% |
| 4.4.1 | LM + LM-left + LM-top | -0.87% | -12.26% | -12.87% | 103% | 101% | 0.17% | 0.16% | 0.45% | 97% | 100% |
| 4.4.2 | LM + LM-CbCr | -0.84% | -11.39% | -12.36% | 102% | 100% | 0.22% | 1.12% | 1.08% | 96% | 100% |
| 4.4.3 | LM + LM fusion | -0.73% | -12.00% | -12.12% | 102% | 99% | 0.33% | 0.11% | 0.77% | 96% | 100% |
| 4.4.4 | LM + LM-left + LM-top + LM-CbCr + LM fusion | -0.74% | -13.17% | -13.61% | 105% | 100% | 0.35% | -1.22% | -0.79% | 100% | 100% |

### CE3.4: Additional test results

| Test # | Add. Result | All Intra Main10 - Over VTM1.0 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |
| 4.1.3 | 4.1.3 + 4.1.10 | -1.73% | -9.65% | -10.60% | 96% | 102% |
| 4.1.4 | 4.1.4 + 4.1.10 | -1.74% | -9.87% | -10.92% | 97% | 102% |
| 4.1.5 | 4.1.5 + 4.1.10 | -1.74% | -10.02% | -11.09% | 101% | 102% |

|  |  | Intra Main10 - Over VTM1.0 |  |  |  |  | Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Add. Result | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.1 | 4.1.1 + separate tree | -3.10% | -15.52% | -15.43% | 144% | 108% | -0.12% | -1.09% | -1.03% | 106% | 101% |
| 4.1.2 | 4.1.2 + separate tree | -3.07% | -14.27% | -14.05% | 138% | 107% | -0.09% | 0.30% | 0.29% | 105% | 101% |

|  |  | Random Access Main10 - Over VTM1.0 |  |  |  |  | Random Access Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Add. Result | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.1 | 4.1.1 + separate tree | -1.33% | -15.75% | -15.29% | 110% | 103% | -0.02% | -0.91% | -0.87% | 106% | 100% |
| 4.1.2 | 4.1.2 + separate tree | -1.32% | -14.20% | -13.94% | 109% | 102% | -0.02% | 0.75% | 0.50% | 104% | 100% |

### CE3.4: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 4.1.1 | Jangwon Choi (LGE) | y |  |
| 4.1.2 | Jangwon Choi (LGE) | y |  |
| 4.1.3 | Adarsh Ramasubramonian (Qualcomm) | y | Mismatch only in RA configuration in the 4th/5th decimal point for some bitrates and almost all PSNRs. Following observations are made from the software: - MFLM method includes four filters; these tests only use one out of the four filters and disables the other three. The 6-tap filter used by MMLM is unchanged. - The encoder RD decision has been changed as follows: Anchor/BMS only computes SATD to choose between one of the MFLM filters, followed by full-RD for a set number of chroma modes. The proposal computes SATD-cost for all modes and performs full RD only for the best mode (SATD-cost-wise) and DM mode. - The mode-list used for signaling the chroma mode has updated to reflect the changes (multiple neighbours and reduced number of filters), and some conditions to include non-LM modes based on neighbouring block LM-modes have been removed. |
| 4.1.4 | Adarsh Ramasubramonian (Qualcomm) | y | Mismatch only in RA configuration in the 4th/5th decimal point for some bitrates and almost all PSNRs. Following observations are made from the software: - MFLM method includes four filters; these tests only use one out of the four filters and disables the other three. The 6-tap filter used by MMLM is unchanged. - The encoder RD decision has been changed as follows: Anchor/BMS only computes SATD to choose between one of the MFLM filters, followed by full-RD for a set number of chroma modes. The proposal computes SATD-cost for all modes and performs full RD only for the best mode (SATD-cost-wise) and DM mode. - The mode-list used for signaling the chroma mode has updated to reflect the changes (multiple neighbours and reduced number of filters), and some conditions to include non-LM modes based on neighbouring block LM-modes have been removed. |
| 4.1.5 | Adarsh Ramasubramonian (Qualcomm) | y | Mismatch only in RA configuration in the 4th/5th decimal point for some bitrates and almost all PSNRs. Following observations are made from the software: - MFLM method includes four filters; these tests only use one out of the four filters and disables the other three. The 6-tap filter used by MMLM is unchanged. - The encoder RD decision has been changed as follows: Anchor/BMS only computes SATD to choose between one of the MFLM filters, followed by full-RD for a set number of chroma modes. The proposal computes SATD-cost for all modes and performs full RD only for the best mode (SATD-cost-wise) and DM mode. - The mode-list used for signaling the chroma mode has updated to reflect the changes (multiple neighbours and reduced number of filters), and some conditions to include non-LM modes based on neighbouring block LM-modes have been removed. |
| 4.1.6 | Teresa Jiang (Foxconn) | y |  |
| 4.1.7 | Teresa Jiang (Foxconn) | y |  |
| 4.1.8 | D. Y. Kim (Chips&Media) | y |  |
| 4.1.9 | D. Y. Kim (Chips&Media) | y |  |
| 4.1.10 | Jinho Lee (ETRI) | y |  |
| 4.1.11 | Jinho Lee (ETRI) | y |  |
| 4.2.1 | Kei Kawamura (KDDI) | y | Code is different from the bitstream organization in the output CE document MDLM_L and MDLM_T is chosen by RDO. |
| 4.2.1 | Jonathan Pfaff (HHI) | y |  |
| 4.2.2 | Kei Kawamura (KDDI) | y | Code is different from the bitstream organization in the output CE document MDLM_L and MDLM_T is chosen by RDO. |
| 4.2.2 | Jonathan Pfaff (HHI) | y |  |
| 4.2.3 | Chih-Wei Hsu (MediaTek) | y |  |
| 4.2.4 | Chih-Wei Hsu (MediaTek) | y |  |
| 4.3.1 | Xiang Ma (Huawei) | y | Only will be used under DM mode |
| 4.3.2 | Xiang Ma (Huawei) | y | No fast RDO |
| 4.4.1 | Xingchun Qi (Hikvision) | y |  |
| 4.4.1 | D.C. Kim (WILUS) | y |  |
| 4.4.2 | Xingchun Qi (Hikvision) | y |  |
| 4.4.3 | Xingchun Qi (Hikvision) | y |  |
| 4.4.4 | Xingchun Qi (Hikvision) | y |  |

## CE3.4: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-K0074 (Foxconn) | 4.1.1, 4.1.3, 4.1.4, 4.1.5 | CE3-related: Combination of LAM (Test 4.1.1) and MNLM (Tests 4.1.3, 4.1.4, and 4.1.5) |
| JVET-K0204 (Canon) |  | Non-CE3: On cross-component linear model simplification |
| JVET-K0313 (Kwangwoon University) |  | Non-CE3: CCLM simplification for reference sample line reduction |
| JVET-K0336 (Huawei) |  | CE3-related: Improved multi-directional LM |

# CE3.5 on ‘Multi-reference line intra prediction’

## Defined tests

| Test# | Short description | Doc. # |
| --- | --- | --- |
| 5.1.1 | Mode dependent reference line selection | JVET-K0284 (Tencent) |
| 5.1.2 | Reference sample extension for multiline intra prediction |  |
| 5.2.1 | Multiple reference lines | JVET-K0162 (Technicolor) |
| 5.2.2 | Multiple reference lines + boundary filtering |  |
| 5.2.3 | Multiple reference lines and not used for top line of CTU | JVET-K0221 (Sony) |
| 5.2.4 | Multiple reference lines with 50:50 weighting |  |
| 5.2.5 | Multiple reference lines with 50:50 weighting (multiple reference lines not used for 4xN and Nx4) |  |
| 5.3.1 | r1: 6-tap combined filter without reference sample smoothing, r2: bi-linear | JVET-K0166 (ETRI) |
| 5.3.2 | r1: 4-tap filter, r2: bi-linear |  |
| 5.3.3 | r1: bi-linear, r2: bi-linear |  |
| 5.4.1 | MRL all block sizes | JVET-K0051 (HHI) |
| 5.4.2 | MRL restricted block sizes (encoder only) |  |
| 5.4.3 | MRL restricted block sizes (restr. signalling) |  |
| 5.4.4 | MRL + Mode dependent reference line selection |  |
| 5.5.1 | Use two extended reference lines | JVET-K0277 (ITRI) |
| 5.5.2 | Use three extended reference lines |  |

## CE3.5: Test results

### CE3.5: ‘All Intra Main10’ results

|  |  | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 5.1.1 | Mode dependent reference line selection | -0.86% | -0.15% | -0.09% | 203% | 99% | -0.88% | -0.42% | -0.54% | 179% | 101% |
| 5.1.2 | Reference sample extension for multiline intra prediction | -0.86% | -0.13% | -0.12% | 209% | 105% | -0.88% | -0.45% | -0.53% | 181% | 102% |
| 5.2.1 | Multiple reference lines | -0.31% | -0.51% | -0.54% | 123% | 105% | -0.21% | -0.21% | -0.24% | 105% | 101% |
| 5.2.2 | Multiple reference lines + boundary filtering | -0.91% | -1.40% | -1.49% | 134% | 109% | -0.97% | -1.29% | -1.43% | 108% | 104% |
| 5.2.3 | Multiple reference lines and not used for top line of CTU | -0.26% | -0.42% | -0.44% | 116% | 108% | -0.18% | -0.20% | -0.21% | 105% | 104% |
| 5.2.4 | Multiple reference lines with 50:50 weighting | -0.11% | -0.39% | -0.48% | 115% | 107% | -0.03% | -0.21% | -0.19% | 104% | 103% |
| 5.2.5 | Multiple reference lines with 50:50 weighting (multiple reference lines not used for 4xN and Nx4) | -0.42% | -0.34% | -0.34% | 113% | 107% | -0.28% | -0.05% | -0.02% | 104% | 103% |
| 5.3.1 | r1: 6-tap combined filter without reference sample smoothing, r2: bi-linear | -0.75% | -0.63% | -0.72% | 119% | 107% | -0.57% | -0.60% | -0.60% | 105% | 102% |
| 5.3.2 | r1: 4-tap filter, r2: bi-linear | -0.76% | -0.64% | -0.68% | 116% | 106% | -0.56% | -0.51% | -0.54% | 104% | 102% |
| 5.3.3 | r1: bi-linear, r2: bi-linear | -0.29% | -0.15% | -0.13% | 110% | 104% | -0.19% | 0.00% | -0.03% | 103% | 101% |
| 5.4.1 | MRL all block sizes | -0.9% | -0.4% | -0.4% | 216% | 100% | -0.5% | -0.2% | -0.3% | 136% | 100% |
| 5.4.2 | MRL restricted block sizes (encoder only) | -0.4% | -0.2% | -0.3% | 145% | 98% | -0.3% | -0.2% | -0.2% | 116% | 99% |
| 5.4.3 | MRL restricted block sizes (restr. signalling) | -0.7% | -0.2% | -0.2% | 147% | 99% | -0.4% | -0.1% | -0.1% | 116% | 100% |
| 5.4.4 | MRL + Mode dependent reference line selection | -0.7% | -0.3% | -0.3% | 147% | 101% | -0.5% | -0.1% | -0.1% | 117% | 100% |
| 5.5.1 | Use two extended reference lines | -0.89% | -0.18% | -0.15% | 176% | 100% | -0.79% | -0.42% | -0.49% | 146% | 99% |
| 5.5.2 | Use three extended reference lines | -1.05% | -0.19% | -0.16% | 208% | 99% | -0.93% | -0.52% | -0.59% | 161% | 100% |

### CE3.5: ‘Random Access Main10’ results

|  |  | Random Access Main10 - Over VTM1.0 |  |  |  |  | Random Access Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | U | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 5.1.1 | Mode dependent reference line selection | -0.27% | -0.10% | -0.09% | 104% | 98% | -0.31% | -0.26% | -0.36% | 128% | 101% |
| 5.1.2 | Reference sample extension for multiline intra prediction | -0.28% | -0.09% | -0.16% | 105% | 101% | -0.32% | -0.29% | -0.41% | 128% | 101% |
| 5.2.1 | Multiple reference lines | -0.18% | -0.29% | -0.37% | 106% | 103% | -0.10% | -0.18% | -0.32% | 102% | 101% |
| 5.2.2 | Multiple reference lines + boundary filtering | -0.44% | -0.97% | -1.02% | 107% | 106% | -0.49% | -1.03% | -1.19% | 104% | 101% |
| 5.2.3 | Multiple reference lines and not used for top line of CTU | -0.15% | -0.26% | -0.21% | 101% | 100% | -0.08% | -0.11% | -0.22% | 103% | 100% |
| 5.2.4 | Multiple reference lines with 50:50 weighting | -0.08% | -0.29% | -0.31% | 119% | 139% | -0.03% | -0.18% | -0.32% | 102% | 100% |
| 5.2.5 | Multiple reference lines with 50:50 weighting (multiple reference lines not used for 4xN and Nx4) | -0.23% | -0.24% | -0.25% | 101% | 100% | -0.13% | 0.09% | -0.14% | 103% | 100% |
| 5.3.1 | r1: 6-tap combined filter without reference sample smoothing, r2: bi-linear | -0.38% | -1.01% | -1.08% | 104% | 101% | -0.26% | -0.53% | -0.52% | 102% | 101% |
| 5.3.2 | r1: 4-tap filter, r2: bi-linear | -0.40% | -0.80% | -1.00% | 103% | 101% | -0.27% | -0.47% | -0.42% | 102% | 100% |
| 5.3.3 | r1: bi-linear, r2: bi-linear | -0.16% | -0.08% | -0.16% | 102% | 100% | -0.09% | 0.06% | -0.12% | 101% | 100% |
| 5.4.1 | MRL all block sizes | -0.5% | -0.4% | -0.4% | 117% | 100% | -0.3% | -0.1% | -0.2% | 117% | 100% |
| 5.4.2 | MRL restricted block sizes (encoder only) | -0.2% | -0.2% | -0.2% | 107% | 101% | -0.1% | -0.1% | -0.2% | 109% | 100% |
| 5.4.3 | MRL restricted block sizes (restr. signalling) | -0.3% | -0.2% | -0.2% | 107% | 100% | -0.2% | 0.0% | 0.0% | 109% | 100% |
| 5.4.4 | MRL + Mode dependent reference line selection | -0.4% | -0.2% | -0.3% | 108% | 101% | -0.3% | 0.0% | -0.1% | 110% | 100% |
| 5.5.1 | Use two extended reference lines | -0.53% | -0.19% | -0.18% | 113% | 99% | -0.46% | -0.29% | -0.49% | 131% | 100% |
| 5.5.2 | Use three extended reference lines | -0.62% | -0.19% | -0.23% | 119% | 98% | -0.56% | -0.47% | -0.56% | 144% | 100% |

### CE3.5: Additional test results

NA

### CE3.5: Cross-check reports

| Test# | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 5.1.1 | Po-Han Lin (ITRI) | Y |  |
|  | Benjamin Bross (HHI) | Y | The provided source code was analyzed and the proposed method can be summarized as follows: - Always use 3 additional reference lines for intra-picture prediction (ref idx=1,2,3) - For first additional reference lines (ref idx 1): allow and test NSST (BMS only) - For all additional reference lines (ref idx 1,2,3) Use one more sample to the left and below for the first ref line, and two more for the additional ref lines No DC / PLANAR mode No odd angular modes for VTM (3,5,7,...,35) and BMS (3,5,7,...,67) Not allowed for P/B slices Syntax: - ref idx syntax after intra luma pred mode syntax - ref idx syntax restricted on intra luma pred mode, i.e. not present when mode is DC, PLANAR or odd angular mode, MPM list derivation has to be performed in order to parse ref idx syntax Encoder search: - Luma fast SATD-based search for all 4 reference indices 0,1,2,3 - Luma RD-cost based search for N*number of reference lines best modes from fast search - Chroma mode searched once for best luma mode |
| 5.1.2 | Po-Han Lin (ITRI) | Y |  |
|  | Benjamin Bross (HHI) | Y | See 5.1.1 Benjamin Bross’s comment |
| 5.2.1 | Ling Li (LGE) | Y |  |
| 5.2.2 | Ling Li (LGE) |  |  |
| 5.2.3 | Po-Han Lin (ITRI) | Y |  |
| 5.2.4 | Ling Li (LGE) | Y |  |
| 5.2.5 | Po-Han Lin (ITRI) | Y |  |
| 5.3.1 | Fabien Racape (Technicolor) | Y | The method is not applied when the intra direction is vertical and the current block is at the top border of CTU. |
| 5.3.2 | Fabien Racape (Technicolor) | Y | See 5.3.1 Fabien Racape’ comment |
| 5.3.3 | Fabien Racape (Technicolor) | Y | See 5.3.1 Fabien Racape’ comment |
| 5.4.1 | Liang Zhao (Tencent) | Y |  |
| 5.4.2 | Liang Zhao (Tencent) | Y |  |
| 5.4.3 | Liang Zhao (Tencent) | Y |  |
| 5.4.4 | Maxim Sychev (Huawei) | Y |  |
| 5.5.1 | Liang Zhao (Tencent) | Y | No |
| 5.5.2 | Liang Zhao (Tencent) | Y | No |

## CE3.5: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-K0221 (SONY (Sony) | 5.2.3, 5.2.4, and 5.2.5 | CE3-related: Additional results of JVET-J1023 Core Experiments 5.2.3, 5.2.4 and 5.2.5 |
| JVET-K0276 (HHI) | 5.4.4 | CE3-related: Mode-dependent multiple reference line intra prediction with bi-prediction |
| JVET-K0348 (Qualcomm, HHI) | 2.4.1, 2.7.1, 5.4.4 | CE3-related: On the combination of multiple reference lines, bilateral reference line filtering, PDPC and 65 directional modes for intra prediction |

# CE3.6 on ‘Non-linear intra prediction’

## Defined tests

| Test# | Short description | Doc. # |
| --- | --- | --- |
| 6.1.1 | Intra prediction using neural networks | JVET-K0266 (HHI) |

## CE3.6: Test results

### CE3.6: ‘All Intra Main10’ results

|  |  | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 6.1.1 | neural networks | -2.81% | -1.42% | -1.46% | 265% | 145% | -2.02% | -1.91% | -1.95% | 145% | 121% |

### CE3.6: ‘Random Access Main10’ results

|  |  | Random Access Main10 - Over VTM1.0 |  |  |  |  | Random Access Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | U | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 6.1.1 | neural networks | -1.28% | 0.27% | 0.18% | 123% | 111% | -1.04% | -1.14% | -1.28% | 115% | 104% |

### CE3.6: Additional test results

NA

### CE3.6: Cross-check reports

| Test# | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 6.1.1 | F. Racape (Technicolor), | Y | The simulations for cross check were performed using handcrafted optimized SSE4 code (same as proponent’s simulations). For information, the nn structure contains approximately 1.3M coefficients, corresponding to 7.1MB of ROM. |
|  | Sergey Ikonin (Huawei), | Y | The software in the main matches to the description given in sec 2.1.9.2 of J0014. |
|  | Yongbing Lin (HiSilicon) | Y | The encoding time ratio for VTM AI test case is much higher (~20%) than that provided by proponent. The main reason may be usage of different compiler and simulation set up. Cross-checker used VS2015 as compiler and set SIMD=AVX2. |

## CE3.6: Related contribution

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-K0196 (HHI) | 6.1.1 | CE3-related: Non-linear weighted intra prediction (cross-check report in JVET-K0262). |

# CE3.7 on ‘Bidirectional intra prediction’

## Defined tests

| Test# | Short description | Doc. # |
| --- | --- | --- |
| 7.1.1 | Bi-directional Intra prediction (BDIP) | JVET-K0163 (Technicolor) |
| 7.2.1 | Distance-Weighted Directional Intra Prediction (DWDIP) | JVET-K0045 (Huawei) |
| 7.3.1 | Linear interpolation intra prediction (LIP) based on MPM flag following LIP flag signaling | JVET-K0090 (LGE) |
| 7.3.2 | LIP based on LIP flag following MPM flag signaling | JVET-K0090 (LGE) |

## CE3.7: Test results

### CE3.7: ‘All Intra Main10’ results

|  |  | All Intra Main10 - Over VTM1.0 |  |  |  |  | All Intra Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 7.1.1 | BDIP | -0.26% | -0.29% | -0.30% | 111% | 97% | -0.38% | -0.62% | -0.65% | 102% | 96% |
| 7.2.1 | DWDIP | -0.28% | -0.33% | -0.29% | 106% | 100% | -0.35% | -0.42% | -0.44% | 105% | 99% |
| 7.3.1 | LIP (LIPMPM) | -0.55% | -0.52% | -0.53% | 106% | 102% | -0.67% | -1.01% | -1.04% | 105% | 103% |
| 7.3.2 | LIP (MPMLIP) | -0.57% | -0.53% | -0.51% | 105% | 102% | -0.68% | -1.02% | -1.03% | 105% | 104% |

### CE3.7: ‘Random Access Main10’ results

|  |  | Random Access Main10 - Over VTM1.0 |  |  |  |  | Random Access Main10 - Over BMS1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | U | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 7.1.1 | BDIP | -0.20% | -0.30% | -0.33% | 102% | 102% | -0.27% | -0.56% | -0.68% | 102% | 100% |
| 7.2.1 | DWDIP | -0.28% | -0.33% | -0.29% | 106% | 100% | -0.23% | -0.36% | -0.49% | 101% | 100% |
| 7.3.1 | LIP (LIPMPM) | -0.27% | -0.36% | -0.47% | 102% | 100% | -0.43% | -0.86% | -1.09% | 104% | 102% |
| 7.3.2 | LIP (MPMLIP) | -0.28% | -0.38% | -0.49% | 102% | 100% | -0.44% | -0.92% | -1.12% | 104% | 102% |

### CE3.7: Additional results

NA

### CE3.7: Cross-check reports

| Test# | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 7.1.1 | Vasily Rufitskiy (Huawei) | Y | No |
|  | Jin Heo (LGE) | Y | We have some comments on CE3-7.1.1 source code: 1. You have used the division operation, and we wonder if you can remove the division operation in the source code. If possible, we wonder how it will affect the coding performance. 2. When BDIP flag is parsed, parsing dependency occurs. We think it would be better to remove the parsing dependency. 3. Since BDIP is used under the same condition as boundary filtering (BF), we wonder how much coding gain could be achieved when both BDIP and BF are enabled. |
| 7.2.1 | Fabien Racape (Technicolor) | Y | The method uses stored weights instead of computing on the fly. It will be interesting to study and check opinions on memory footprint vs. runtime complexity. |
|  | Jonathan Pfaff (HHI) | Y | No |
| 7.3.1 | Alexey Filippov (Huawei) | Y | We recommend to separately study every aspect included into these tests: 1. the LIP technology for directional modes within a design having a single MPM list 2. the LIP technology for planar mode within a design having a single MPM list 3. the LIP technology for directional modes based on LIP flag following MPM flag signaling 4. the LIP technology for Planar mode based on LIP flag following MPM flag signaling 5. the LIP technology for directional modes based on MPM flag following LIP flag signaling 6. the LIP technology for Planar mode based on MPM flag following LIP flag signaling |
| 7.3.2 | Alexey Filippov (Huawei) | Y |  |

## CE3.7: Related contributions

NA

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 10th JVET Meeting, San Diego, USA, April 2018, JVET-J1023